University Valley, Antarctica

Hobo instrumentation deployed in December 2009

SENSOR DEPLOYMENT OVERVIEW

Shallow Ground Ice Site - S77° 51.946’    E160° 43.720’; elevation: ~1697m

• 4-channel external (3 temperature and 1 Echo sensor)

• Hobo Pro v2 (temperature and RH)

Medium Depth Ground Ice Site - S77° 51.773’    E160° 43.527’; elevation: ~1661m

• 4-channel external (4 temperature sensors)

• Hobo Pro v2 (temperature and RH) in soil

• Hobo Pro v2 (temperature and RH) under rock with salt

Deep Ground Ice Site - S77° 51.584’    E160° 43.019’; elevation: ~1684m

• 4-channel external (4 temperature sensors)

• Hobo Pro v2 (temperature and RH) in soil

• Hobo Pro v2 (temperature and RH) under rock with salt

Loggers

4-Channel External Loggers

Logger is placed in a water-resistant plastic box (tupperwear), covered with rocks. Leads of sensors come out of plastic box.

Hobo Pro v2 Temperature & RH Loggers

Loggers are all set at 1 hour data collection intervals, have long white tube with sensor cord.

Shallow Ground Ice Site - S77° 51.946’    E160° 43.720’; elevation: ~1697m

depth to ice-cemented ground 8 cm

4 channel external logger:  3 temperature sensors and 1 Echo probe

logger name = UniversityValley_closest2snowpack_shalow

1 hour data collection interval, serial # 1122968

New battery installed 8 Dec 2009, Hobo = non-industrial, placed inside Tupperware container

Sensor #1 (temperature) = surface

Sensor #2 (temperature) = at ground ice boundary (ice table; 8 cm depth)

Sensor #3 (temperature) = 4 cm into soil **this should be 15cm In Ice

Sensor #4 (Echo) = between ice table and soil, ~4 cm depth

Hobo Pro v2 Temperature & RH Loggers:

UniversityV_closest2snowpack_shalow_TRH, Serial # 2429439

Deployed at 4 cm depth in dry soil (same location as Echo probe)

Medium Depth Ground Ice Site - S77° 51.773’    E160° 43.527’; elevation: ~1661m

Ground ice ~20 cm depth, located midway down valley.

4 channel external industrial logger:  4 temperature probes

1 temperature probe = “HD” = sensor 2

logger name = UniversityValley_Middle_20cmpermafrost

1 hour data collection interval, serial #2432937

4-channel external deployed near salt rock, measuring subsurface temperatures.

Sensor #1 = 10 cm deep.

Sensor #2 = 9 cm into drilled hole into ground ice in diagonal hole.

Sensor #3 = just below surface.

Sensor #4 = ground ice table.

Sensors 1 and 3 are in dry permafrost.

For deployment, a pit was dug at the site.  Drilled a hole diagonally into ground ice to insert sensor into ground ice.  Remaining probes inserted horizontally into side of pit, sensor 1 inserted 12 cm deep into side of pit.

Hobo Pro v2 Temperature & RH Loggers

1) UniversityV_med_20cmpermafrost_rock_TRH, this sensor deployed under a rock with salt, serial # 2429436

2) UniversityV_med_20cmpermafrost_soil_TRH, this sensor deployed where no rocks, just soil, serial # 2429434

Salt rock is a dolorite rock.  Hobo (T and RH) placed 7 cm below bottom of rock, at 15 cm from soil surface, therefore rock is 8 cm into soil.

Another Hobo Pro v2 (T and RH) placed in soil next to the rock, 15 cm deep into soil.

Depth to permafrost at rock/soil location = 22 cm.

Deep Ground Ice Site - S77° 51.584’  E160° 43.019’; elevation: ~1684m

Permafrost depth = 36 cm

4-channel external industrial logger:  4 temperature probes

logger name = UniversityValley_deep_40cmpermafrost

1 hour data collection interval, serial # 2432936

Sensor #1 = surface

Sensor #2 = 10 cm depth, 8 cm into side of pit

Sensor #3 = at ground ice boundary (ice table), ~4 cm into side of pit

Sensor #4 = in ground ice, 11 cm diagonal

Hobo Pro v2 Temperature & RH Loggers

1) UniversityV_deep_40cmPermafrost_soil_TRH, place under rock with salt, serial # 2429437

2) Placed Hobo Pro v2 (T and RH) under rock with salt, 5 cm below rock bottom, 11 3) cm below (soil) surface.

Probe also placed 15 cm below soil surface, no large overlying rocks.