University Valley weather station data

Campbell Weather Station

Summary

| Data | Status/any issues? |
| --- | --- |
| BattV |  |
| T107_air |  |
| RHwrtH2O | This is the raw data from the sensor – do NOT use |
| RHwrtIce |  |
| SlrW |  |
| WS_ms |  |
| WindDir |  |
| T107_surf |  |
| T107_10cm |  |
| T107_ice42cm |  |
| T107_InIce49cm |  |
| water_int |  |
| PA_mS_int |  |
| water20cm |  |
| PA_mS20cm |  |
| LW_kOhm |  |
| Echo |  |

Campbell Sensors:

- BattV – dropped as low as 12.2 V in the first winter, but recovered well every summer. In following winters dropped to 12.36V in each winter; up to 14.5 V

- T107_air – looks good throughout

- Air_RH – have throughout

- SlrW – full set

- WS – full set

- WindDir – all good

- T surf – a bunch of missing data. Have Days 344-706.5, 1211-1440 – a bit of data afterwards but some unreasonable values and data is not consistent. ** We use this as our surface temperature!

- T10cm – full dataset

- Tice – full dataset, 1 errant point

- TinIce – full dataset

Hobo sensors:

- Surf T – 343-702; a tiny bit of data at the end of the data set

- Surf RH – 343-702; a tiny bit of data at the end of the data set

- Surf Batt – between 3.21 and 3.56V for the period during which we have good data. No battV at the end where we can sporadic data – weird

- 20cm T – full dataset

- 20cm RH – full dataset

- 20cm Batt – only have the first year, no data after – weird

- Ice 42cm T – full dataset

- Ice 42cm RH – full dataset. RH wrt water goes to 80%, but once corrected for ice it about flatlines at 100%

- Ice42cm Battery – only have the first year

Based on the battery voltages, no power-related issues are expected in the data.

Air temperature data looks good for all years

Surface temperature

This is where the sensor did not make a good connection all the time, and we lost much of the data between Jan 2011 and Jan 2013. When the sensor was not making a good contact, the data was recorded as “NaN”, thus effectively automatically removing bad data points from the plots. But there are a few places where there are erroneous values.

2009 through 2011 are good. But in 2012, I will remove these large spikes. Removed data points are:

Days: 346.2083 to 346.3542 (8 data points)

Days: 353 to 366.98 (end of year; 719 data points, but more than 1/3 of these were already NaN)

This is done in the Matlab script used to do the calibration of the data (University_Valley_Met_Station_Calibration_all.m) – the original data file is not edited.

Humidity looks fine – nothing all that peculiar. Max RH, before correction, is 105%.

Wind speed distributions are similar for each of the years, though 2012 is definitely windier than 2010, which is winder than 2011

Wind directions look consistent, nothing peculiar.

Warm air temperatures occurred when the wind was coming from any of the directions, but the coldest air temperatures occurred when the wind was from the south, for all years, by at least 5 deg (small outliers notwithstanding).

Distributions of wind speed vs air temperature do look a bit different between the years. May be worth investigating further.

2012 definitely has the warmest winter, by almost 10C – for most of the 2012 winter the winds were higher in the winter than for 2010 and 2011 by almost 1 m/s! Catabatic warming! Month 10 does not follow the trend though.

There is some weird missing data in Oct 2012. Battery is fine, temperatures are not especially warm, but there is just missing data. Interestingly, the Echo and water20cm jump up in the measurement right before the data drops out. Don’t see anything on the water_int.

8 data points (4hrs) missing.

Thoughts on what happened here??

I added in the missing times, and put NaN’s for all the sensors for that time period – Screws up getting means for that month and year!! Also for now put in NaN for the two high points on Echo and water20cm right before the dropout.

Weird 1-point jump in ice-interface temperature (42cm depth).

Interpolated the temperature since it is a single point.