BOKU CORE FACILITY MASS SPECTROMETRY

Daniel Maresch, Rudolf Figl, Dr Clemens Grünwald-Gruber

Vienna, May 10, 2022

# Analysis report

ESI-MS analysis

# Aim

Intact protein analysis

# Samples

11 intact protein samples, heme derivates

# Analyses performed

LC-ESI-MS/MS analysis of intact proteins
MATERIAL AND METHODS

Protein analysis

5 µL of the protein solution was directly injected to a LC-ESI-MS system (LC: Agilent 1290 Infinity II UPLC). A gradient from 15 to 80 % acetonitrile in 0.1 % formic acid (using a Waters BioResolve column (2.1 x 5 mm)) at a flow rate of 400 µL/min was applied (9 minute gradient time). Detection was performed with a Q-TOF instrument (Agilent Series 6560 LC-IMS-QTOFMS) equipped with the Jetstream ESI source in positive ion, MS mode (range: 100-3200 Da). Instrument calibration was performed using ESI calibration mixture (Agilent). Data was processed using MassHunter BioConfirm B.08.00 (Agilent) and the spectrum was deconvoluted by MaxEnt.

Sample information:

| Sample | LmCpfC variant | Porphyrin type | MW |
| --- | --- | --- | --- |
| 0 | CdCpfC truncated | Coproporphyrin III or Fe-Coproheme | 39219.17 Da 655 Da 708.2 Da |
| 1 | LmCpfC E271Q + Fe2+ | Coproporphyrin III or Fe-Coproheme | 37788.61 Da 655 Da 708.2 Da |
| 2 | LmCpfC H87A + Fe2+ | Coproporphyrin III or Fe-Coproheme | 37723.53 Da 655 Da 708.2 Da |
| 3 | LmCpfC H182A + Fe2+ glovebox | Coproporphyrin III or Fe-Coproheme | 37723.53 Da 655 Da 708.2 Da |
| 4 | LmCpfC H182A + Fe2+ cary | Coproporphyrin III or Fe-Coproheme | 37723.53 Da 655 Da 708.2 Da |
| 5 | LmCpfC H182A + Fe2+ | Protoporphyrin IX or Fe-Protoporphyrin IX | 37723.53 Da 562.66 Da 616.49 Da |
| 6 | LmCpfC WT + Fe2+ | Protoporphyrin IX or Fe-Protoporphyrin IX | 37789.59 Da 562.66 Da 616.49 Da |
| 7 | CdCpfC C127R C353D + Fe2+ Fraction 1 | Coproporphyrin III or Fe-Coproheme | 43557.07 Da 655 Da 708.2 Da |
| 8 | CdCpfC C127R C353D + Fe2+ Fraction 2 | Coproporphyrin III or Fe-Coproheme | 43557.07 Da 655 Da 708.2 Da |
| 9 | No protein | Coproheme; MMD; Heme b | 708.2 Da 678 Da 662.2 Da 616.2 Da |
| 10 | CdChdC Y135A | Coproheme; MMD; Heme b | 27164.90 708.2 Da 678 Da 662.2 Da 616.2 Da |

Results:

Intact protein

In the following the deconvoluted MS spectra of the intact proteins are shown (one mass range – others could be added on request); note that the third panel is sample 10

Heme derivates:

1 to 4 heme derivates were identified in all samples. The relative proportions were calculated by comparing the Areas of the EICs of the monoisotopic masses. A table showing the relative proportions is provided (see Excel file: “20220510_Report_quantification_Heme_SH_TG.xlsx”) while the chromatograms are presented in the following.