Mw = 6.6 Kos earthquake of July 20, 2017 - Modelling of the GPS and InSAR data

- Data

We processed continuous GPS data from NOA, TREE, METRICA, HEPOS, and three CORS-Turkey (DATC, DIDI, MUG1) with GIPSY v6.4. The other GPS vectors (Tiryakioğlu et al., 2017) are from data acquired at GEOTEKNIK and other CORS-Turkey permanent stations and campaign stations.

| Code | Array | UTM_E | UTM_N | dE | dN | dU | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | km | km | mm | mm | Mm |  |
| 086A | HEPOS | 513.18 | 4071.18 | -9 ± 3 | -10 ± 3 | 16 ± 9 |  |
| 087A | HEPOS | 590.15 | 4024.85 | 3 ± 3 | -5 ± 3 | -8 ± 9 |  |
| ASTY | Metrica | 442.12 | 4044.61 | -2 ± 4 | -1 ± 4 | 2 ± 12 |  |
| AYD1 | CORS-TR | 573.72 | 4188.47 | 2 ± 4 | 4 ± 4 | -7 ± 15 | a |
| BODR | Campaign | 535.67 | 4097.06 | -38 ± 9 | 160 ± 9 | 119 ± 22 | a |
| CAMK | Campain | 574.18 | 4117.00 | 2 ± 5 | 28 ± 6 | 28 ± 21 | a |
| CESM | CORS-TR | 445.14 | 4239.71 | 0 ± 3 | 1 ± 3 | -6 ± 11 | a |
| DATC | CORS TR | 561.79 | 4062.77 | 10 ± 5 | -32 ± 5 | 8 ± 15 |  |
| DIDI | CORS TR | 523.79 | 4136.19 | -5 ± 5 | 19 ± 5 | 2 ± 15 |  |
| FETH | CORS-TR | 689.90 | 4055.51 | -1 ± 3 | 2 ± 3 | -5 ± 13 | a |
| IKAR | Tree | 435.86 | 4162.28 | -5 ± 6 | -1 ± 6 |  |  |
| IZMI | CORS-TR | 507.14 | 4249.62 | 1 ± 3 | 1 ± 3 | -2 ± 10 | a |
| KALY | Tree | 496.59 | 4090.70 | -7 ± 6 | -1 ± 6 |  |  |
| KALY | Metrica | 497.88 | 4089.97 | -3 ± 4 | 0 ± 4 | 11 ± 12 |  |
| KNID |  | 535.19 | 4059.69 | -20 ± 4 | -50 ± 4 | -2 ± 16 | a |
| KRP1 | Metrica | 510.91 | 3926.34 | 0 ± 4 | -4 ± 4 | 5 ± 12 |  |
| KYCZ | Campaign | 650.10 | 4093.85 | 12 ± 9 | 6 ± 9 | 10 ± 30 | a |
| MARM | Campaign | 585.92 | 4070.08 | 6 ± 3 | -2 ± 3 | 6 ± 10 | a |
| MUG1 | CORS TR | 620.29 | 4119.51 | 0 ± 5 | -4 ± 5 | 6 ± 15 |  |
| MUMC | GeoTeknik | 554.98 | 4110.44 | 23 ± 2 | 69 ± 3 | 4 ± 10 | a |
| ORTA | GeoTeknik | 531.00 | 4100.56 | -39 ± 2 | 100 ± 3 | 15 ± 9 | a |
| ROD0 | Tree | 607.18 | 4029.23 | -1 ± 6 | -11 ± 6 |  |  |
| ROD2 |  | 583.18 | 3986.66 | 2 ± 2 | -6 ± 2 | 6 ± 10 | a |
| SAM0 | Tree | 497.66 | 4178.91 | -9 ± 6 | 1 ± 6 |  |  |
| SAM0 | Metrica | 474.06 | 4182.86 | -4 ± 4 | 1 ± 4 | 6 ± 12 |  |
| TGRT | GeoTeknik | 522.85 | 4095.69 | -9 ± 3 | 25 ± 3 | 1 ± 11 | a |
| TRKB | GeoTeknik | 528.66 | 4107.56 | -25 ± 2 | 65 ± 2 | 3 ± 9 | a |
| YALI | GeoTeknik | 547.63 | 4094.45 | 7 ± 3 | 153 ± 3 | 7 ± 11 | a |

Table 1: Co-seismic GPS displacements and uncertainties. Source (a) is from Tiryakioğlu et al. (2017). Stations coordinates are in UTM 35. Data from various networks, NOA, HEPOS, METRICA, TREE, GEOTEKNIK, CORS-TR.

InSAR data is from ESA Sentinel 1A and 1B satellites processed using the ESA SNAP package.

- Co-seismic model parameters

Here we make the assumption that the earthquake was produced by the homogenous slip of a rectangular fault buried in an elastic half-space. In such case, the fault is described by nine parameters (Okada, 1985), the three coordinates of the centre of the upper edge of the fault, the length and width of the fault, the azimuth, dip and rake angles, and the slip.

A priori values of the three angles are provided by seismology through the centroid moment tensors (CMT) along with values of the seismic moment. Tab. 2 gives a list of various determinations.

| Source | Mw (*E+18) | Depth (km) | Strike (°) | Dip (°) | Rake (°) | Strike (°) | Dip (°) | Rake (°) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| GCMT | 9.7 | 12 | 275 | 36 | -85 | 89 | 54 | -94 |
| CPPT | 14.2 | 10 | 290 | 26 | -58 | 76 | 68 | -104 |
| IPGP | 9.7 | 9 | 284 | 32 | -71 | 82 | 60 | -102 |
| GFZ | 9.7 | 11 | 270 | 56 | -94 | 98 | 35 | -82 |
| USGS | 11.3 | 11.5 | 281 | 34 | -78 | 87 | 56 | -97 |
| KOERI | 11.3 | 6 | 286 | 53 | -72 | 78 | 40 | -112 |
| INGV | 13 | 11.3 | 296 | 49 | -55 | 68 | 52 | -124 |
| NOA | 8.9 | 6 | 265 | 43 | -102 | 102 | 48 | -79 |
| AUTH | 8.9 | 7 | 275 | 41 | -88 | 93 | 49 | -91 |
| NKUA | 8.9 | 11 | 267 | 38 | -110 | 112 | 55 | -75 |
| Average | 10.6 | 9.5 | 279 | 41 | -81 | 89 | 52 | -96 |
| r.m.s. scatter | 1.8 | 2.2 | 10 | 9 | 17 | 13 | 9 | 14 |

Table 2: Centroid moment tensors from various seismological centres

In the following we do not invert for the azimuth and dip angles and use the average values from seismology. Thus we seek for seven parameters only. For the comparison of seismologic and geodetic moment tensors, we assume that the rigidity of the crust is 3 1010 N m.

- Inversion of the GPS data

InSAR and GPS data have different status and importance for constraining the fault parameters. For that earthquake, GPS vectors are available in all azimuths, at different distances, and they are large at several stations. They will be efficient to constrain all fault parameters but not to discriminate between south and north dipping candidate planes. This discrimination will be performed using the InSAR data in a second step. Indeed the observed InSAR fringes cover only a small part of the azimuth range and at small distance from the fault. The only parameters InSAR will constrain efficiently are the depth of the top of the fault and the selection among south and north dipping. The inversion of the GPS data is performed for both south and north dipping hypothesis. The best fitting models are in Tab. 3.

| Dip | E | N | U | L | W | Slip | Az. | Dip | Rake | Moment | Centr. depth | r.m.s. hor. | r.m.s. vert. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | km | km | km | km | km | m | ° | ° | ° | 1018 Nm | km |  |  |
| North | 542.3 | 4084.7 | 3 | 16.6 | 6.7 | 2.91 | 279 | 41 | -80 | 9.7 | 5.2 | 3.2 | 12.3 |
| South | 543.3 | 4090.2 | 2.9 | 19.2 | 8.6 | 2.07 | 89 | 52 | -93 | 10.3 | 6.3 | 3.3 | 13.9 |

Table 3: Best fitting models when using the GPS data only

The inversion process was stable for both north and south dipping cases, and the same minimum was found when starting from different initial parameters. The predicted geodetic moment and the rake fit well with the seismological ones. The north case, predicts a significant component of left lateral strike slip on the fault plane. The root mean square (r.m.s.) scatters for the horizontal and vertical components of the GPS vectors are small and consistent with  the a priori values used for the inversion (4 and 10 mm). The predicted slip is large but not unusual for this magnitude and the size of the fault is therefore relatively small, especially in the north dipping case. In the inversion there is a trade off between the slip on fault and the fault width and we found that solutions with fault width larger by up to 30% (i.e. 8.7 km) and corresponding slip lower by up to 30% (i.e. 2.24 m) were giving residuals not much larger that the optimal one. The fault did not reach the surface, its modelled top being at ~3 km depth.

Fig. 1 and 2 below show the observed and predicted interferograms for both ascending (track 131) and descending (track 36) orbits, in both south and north dipping cases. Those interferograms, predicted by GPS, will be used to assess the a priori rank of the values picked on the fringes and injected in the forthcoming joint inversion. We see that the fit of the south dipping model is not very good. There are too many predicted fringes in the Karaada island. The most efficient way to reduce this number would be to move the fault towards south and/or to move it down. Also the azimuth and bending of the fringes predicted by the south dipping model are less consistent with the observations that the azimuths predicted in the north dipping case. We see also that the south dipping model predicts fringes along the west coast of the Kos island, for the ascending view, that are not observed in the interferograms.

- Joint inversion of GPS and InSAR

We picked 67 points on the ascending interferograms track 131 (using the four pairs June 30 - July 30, July 12 – July 24, July 12 - July 30, and July 18 - July 24). The picked points belong to eight different fringes corresponding to displacements along the line of sight ranging from +28 mm to -168 mm. We added eight points arbitrarily picked along the main road of Kos and gave a zero value to those points as no fringe is visible there, this is order to better constrain the western side of the fault.

We picked 86 points on the descending interferograms track 36 (using the four pairs July 6 - July 24, July 6 – July 30, July 18 - July 24, and July 18 - July 30). The picked points belong to seven different fringes corresponding to displacements along the line of sight ranging from 0 mm to -168 mm. We added eight points arbitrarily picked along the main road of Kos and gave a zero value to those points as no fringe is visible there, this is order to better constrain the western side of the fault.

We produced many other interferograms but the four and four used for the above picking were the most coherent ones and with short time window after the main shock, so as to minimize the possible contribution of post-seismic motions. We found also this number of four convenient to verify that the differential tropospheric effect, from one to another interferogram, was not affecting the picking. The zero value of the pickings was established based on the prediction of the GPS and further controlled after inversion. The use of the prediction from GPS was particularly useful to assess the continuity of the fringes between Karaada island and the mainland.

The a priori uncertainties used for the inversion are 4 mm for the horizontal GPS components and 10 mm for the vertical GPS component and for InSAR. We checked that our solutions remained almost the same when modifying those values in the range 3-5 mm and 8-20 mm respectively. The ratio between the number of picked points (153 + 8 in Kos) and the number of GPS points (28), weighted by the a priori uncertainties, gives approximately the same weight to InSAR and horizontal GPS in the inversions.

Figure 1: ascending interferogram and models with north and south dip

Figure 2: descending interferogram and models with north and south dip

The best fitting models determined by the joint GPS and InSAR inversions are summarized in Tab. 4 below.

| Dip | E | N | U | L | W | Slip | Rake | Moment | Centr. depth | r.m.s. hor. | r.m.s. vert. | r.m.s. asc. | r.m.s. desc |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | km | km | km | km | km | m | ° | 1018 Nm | km | mm | mm | mm | mm |
| North | 542.9 | 4081.6 | 2.4 | 11.8 | 10.3 | 2.94 | -79 | 10.7 | 5.8 | 4.6 | 15.0 | 5.0 | 11.0 |
| South | 542.7 | 4089.0 | 4.6 | 21.2 | 7.0 | 2.12 | -105 | 9.4 | 7.4 | 12.7 | 12.0 | 8.8 | 12.1 |

Table 4: Best fitting models from the joint inversion GPS & InSAR (28 GPS points, 67 ascending pickings, 86 descending pickings). Azimuth and dip angles are the same as in Tab. 3.

Tab. 5 and 6 below contain the observed and modelled GPS displacements in both north dipping and south dipping cased. Fig. 3 and Fig. 4 show the fit of the InSAR data (ascending data are in the left and descending in the right).

| Code | UTM_E | UTM_N | dE (mm) | dN (mm) | dU (mm) | modE (mm) | modN (mm) | modU (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 086A | 513.18 | 4071.18 | -9 | -10 | 16 | -4 | -10 | 7 |
| 087A | 590.15 | 4024.85 | 3 | -5 | -8 | 2 | -7 | 0 |
| ASTY | 442.12 | 4044.61 | -2 | -1 | 2 | 0 | -1 | 1 |
| AYD1 | 573.72 | 4188.47 | 2 | 4 | -7 | 1 | 6 | -1 |
| BODR | 535.67 | 4097.06 | -38 | 160 | 119 | -43 | 139 | 39 |
| CAMK | 574.18 | 4117 | 2 | 28 | 28 | 8 | 17 | 2 |
| CESM | 445.14 | 4239.71 | 0 | 1 | -6 | 0 | 1 | 0 |
| DATC | 561.79 | 4062.77 | 10 | -32 | 8 | 12 | -34 | 5 |
| DIDI | 523.79 | 4136.19 | -5 | 19 | 2 | -5 | 20 | 0 |
| FETH | 689.9 | 4055.51 | -1 | 2 | -5 | 0 | 0 | 1 |
| IKAR | 435.86 | 4162.28 | -5 | -1 | 0 | -1 | 1 | 0 |
| IZMI | 507.14 | 4249.62 | 1 | 1 | -2 | 0 | 2 | 0 |
| KALY | 496.59 | 4090.7 | -7 | -1 | 0 | 1 | 1 | 5 |
| KALY | 497.88 | 4089.97 | -3 | 0 | 11 | 1 | 1 | 5 |
| KNID | 535.19 | 4059.69 | -20 | -50 | -2 | -15 | -72 | 4 |
| KRP1 | 510.91 | 3926.34 | 0 | -4 | 5 | 0 | -3 | 0 |
| KYCZ | 650.1 | 4093.85 | 12 | 6 | 10 | 0 | 0 | 1 |
| MARM | 585.92 | 4070.08 | 6 | -2 | 6 | -2 | -3 | 4 |
| MUG1 | 620.29 | 4119.51 | 0 | -4 | 6 | 1 | 2 | 1 |
| MUMC | 554.98 | 4110.44 | 23 | 69 | 4 | 20 | 67 | 7 |
| ORTA | 531 | 4100.56 | -39 | 100 | 15 | -36 | 82 | 22 |
| ROD0 | 607.18 | 4029.23 | -1 | -11 | 0 | 1 | -4 | 1 |
| ROD2 | 583.18 | 3986.66 | 2 | -6 | 6 | 1 | -5 | -1 |
| SAM0 | 497.66 | 4178.91 | -9 | 1 | 0 | -1 | 5 | 0 |
| SAM0 | 474.06 | 4182.86 | -4 | 1 | 6 | -1 | 3 | 0 |
| TGRT | 522.85 | 4095.69 | -9 | 25 | 1 | -9 | 21 | 13 |
| TRKB | 528.66 | 4107.56 | -25 | 65 | 3 | -24 | 60 | 10 |
| YALI | 547.63 | 4094.45 | 7 | 153 | 7 | 13 | 154 | 7 |

Table 5: Observed and modelled GPS co-seismic displacements for the best fitting north dipping model from the joint inversion GPS & InSAR

| Code | UTM_E | UTM_N | dE (mm) | dN (mm) | dU (mm) | modE (mm) | modN (mm) | modU (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 086A | 513.18 | 4071.18 | -9 | -10 | 16 | -11 | -15 | 5 |
| 087A | 590.15 | 4024.85 | 3 | -5 | -8 | 1 | -4 | 0 |
| ASTY | 442.12 | 4044.61 | -2 | -1 | 2 | -1 | -1 | 0 |
| AYD1 | 573.72 | 4188.47 | 2 | 4 | -7 | 1 | 5 | -1 |
| BODR | 535.67 | 4097.06 | -38 | 160 | 119 | -8 | 102 | 57 |
| CAMK | 574.18 | 4117 | 2 | 28 | 28 | 11 | 18 | 2 |
| CESM | 445.14 | 4239.71 | 0 | 1 | -6 | 0 | 1 | 0 |
| DATC | 561.79 | 4062.77 | 10 | -32 | 8 | -1 | -15 | 3 |
| DIDI | 523.79 | 4136.19 | -5 | 19 | 2 | -3 | 17 | 1 |
| FETH | 689.9 | 4055.51 | -1 | 2 | -5 | 0 | 0 | 1 |
| IKAR | 435.86 | 4162.28 | -5 | -1 | 0 | 0 | 1 | 0 |
| IZMI | 507.14 | 4249.62 | 1 | 1 | -2 | 0 | 2 | 0 |
| KALY | 496.59 | 4090.7 | -7 | -1 | 0 | 1 | -1 | 4 |
| KALY | 497.88 | 4089.97 | -3 | 0 | 11 | 1 | -1 | 4 |
| KNID | 535.19 | 4059.69 | -20 | -50 | -2 | -11 | -50 | 5 |
| KRP1 | 510.91 | 3926.34 | 0 | -4 | 5 | 0 | -2 | 0 |
| KYCZ | 650.1 | 4093.85 | 12 | 6 | 10 | 0 | 0 | 1 |
| MARM | 585.92 | 4070.08 | 6 | -2 | 6 | -5 | 0 | 3 |
| MUG1 | 620.29 | 4119.51 | 0 | -4 | 6 | 1 | 2 | 1 |
| MUMC | 554.98 | 4110.44 | 23 | 69 | 4 | 23 | 65 | 10 |
| ORTA | 531 | 4100.56 | -39 | 100 | 15 | -18 | 70 | 32 |
| ROD0 | 607.18 | 4029.23 | -1 | -11 | 0 | 0 | -2 | 1 |
| ROD2 | 583.18 | 3986.66 | 2 | -6 | 6 | 0 | -3 | 0 |
| SAM0 | 497.66 | 4178.91 | -9 | 1 | 0 | -1 | 4 | 0 |
| SAM0 | 474.06 | 4182.86 | -4 | 1 | 6 | -1 | 2 | 0 |
| TGRT | 522.85 | 4095.69 | -9 | 25 | 1 | 1 | 9 | 14 |
| TRKB | 528.66 | 4107.56 | -25 | 65 | 3 | -14 | 50 | 15 |
| YALI | 547.63 | 4094.45 | 7 | 153 | 7 | 11 | 71 | 27 |

Table 6: Observed and modelled GPS co-seismic displacements for the best fitting south dipping model from the joint inversion GPS & InSAR

In the joint inversion for the north dipping case, the inversion is stable with all parameters free (except azimuth and dip angles). The main evolutions with respect to the GPS only case are a decrease of the fault length and an increase of its width. The top of the fault is now at 2.4 km depth and its bottom at 9.2 km depth. The fit of the horizontal GPS is 46% less good that with the GPS only inversion. The fit of the vertical GPS is 22% less good that with the GPS only inversion. The fit of the ascending and descending picked fringes (Fig. 3) is 8 mm in average.

In the joint inversion for the south dipping case, the inversion is not stable for the fault width which decreases to the unrealistic value of 3.5 km. When fixing the width, the inversion becomes stable for all other parameters. After some trials we fixed the width at 7 km, e.g. 20% less that for the GPS only case. The main evolutions with respect to the GPS only case are an increase of the fault length at 21.2 km and a deepening of the fault tip at 4.6 km. This deepening is needed to reduce the modelled fringes density in the Karaada island. In the best fitting model, the fit of the horizontal GPS is four times less good than with the GPS only inversion. This is due to the fact that the fault is not efficient to predict the north component of the vector at the GPS station located to the north, in particular YALI and ORTA. On the opposite, the fit of the vertical is slightly improved. The fit of the ascending and descending picked fringes (Fig. 3) is 10 mm in average. The ascending model (Fig. 5) predicts two fringes in the west of the Kos island that are not observed. For both south and north dipping the GPS vector at BODR is questionable.

Clearly the joint south dipping model is less stable and less coherent (for the two families of observations) than the north dipping one. This south dipping model has several inconsistencies. Therefore we believe that the joint inversion of GPS and InSAR strongly indicates that the right fault plane for the Kos earthquake is the north dipping one.

Figure 3: fit of the InSAR data in the north dipping model (ascending data are in the left and descending in the right).

Figure 4: fit of the InSAR data in the south dipping model (ascending data are in the left and descending in the right).

Figure 5: ascending interferogram and models with north and south dip

Figure 6: descending interferogram and models with north and south dip

- Joint inversion GPS & InSAR with all parameters free

In this last inversion, we leave all the parameters free and the azimuth and dip angle rotate slightly by 3° each. The parameters of the model are in Tab. 7. The best fitting dip angle is 38°. The component of left lateral motion is also increased in this model. In that model the fault is almost a square of ~11.5 x 11.5 km. This model thus suggests a small blind fault (buried at 2.4 km depth with bottom at 9.6 km depth) with a relatively large slip of 2.67 m. The value of the vertical r.m.s. is calculated without the station BODR that is outlier, otherwise the r.m.s. becomes 13.7 mm.

| E | N | U | L | W | Slip | Azi. | Dip | Rake | M | Centr. depth | r.m.s. hor. | r.m.s. vert. | r.m.s. InSAR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| km | km | km | km | km | m | ° | ° | ° | 1018 Nm | km | mm | mm | mm |
| 542.3 | 4081.2 | 2.4 | 11.2 | 11.7 | 2.67 | 282 | 38 | -75 | 10.5 | 6.0 | 4.2 | 5.1 | 7.2 |
|  |  | 0.2 | 0.8 | 1.0 | 0.3 | 3 | 3 | 3 | 0.4 | 0.3 |  |  |  |

Table 7: Best fitting models, and uncertainties, from the joint inversion GPS & InSAR (28 GPS points, 67 ascending pickings, 86 descending pickings) with all parameters free.

- Test of the best fitting model for various amplitudes of slip

Finally we computed the best fitting solution for various amplitudes of slip arbitrarily fixed in the range 1.5 - 3 m. Although fitting best InSAR, the solution with slip 1.58 m fits less the data as well as the CTM parameters. Also the solution with slip 3.57 m fits less the data. All solutions in the range 2.07 - 3.07 m are possible with the best overall fit found for slip 2.67 m as indicated in the previous section.

| Slip | X | Y | Depth | Az. | Dip | Rake | Length | Width | Moment | Centroid depth | fit GPS horiz. | fit GPS vert. | fit InSAR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | km | km | km | ° | ° | ° | km | km | 1018 Nm | km | mm | mm | mm |
| 1.58 | 541.3 | 4079.8 | 1.5 | 284 | 31 | -72 | 12.6 | 15.3 | 9.1 | 5.4 | 4.4 | 5.4 | 6.9 |
| 2.07 | 542.1 | 4080.6 | 2.1 | 282 | 35 | -75 | 12 | 13.3 | 9.9 | 5.9 | 4.3 | 5.2 | 6.9 |
| 2.57 | 542.4 | 4081.1 | 2.4 | 280 | 37 | -77 | 11.4 | 11.9 | 10.5 | 6 | 4.0 | 5.2 | 7.2 |
| 3.07 | 542.6 | 4081.3 | 2.6 | 279 | 39 | -78 | 10.6 | 11 | 10.7 | 6.1 | 4.3 | 5.1 | 7.6 |
| 3.57 | 542.7 | 4081.5 | 2.7 | 279 | 40 | -79 | 10 | 10.3 | 11 | 6 | 4.8 | 5.0 | 7.7 |

Table 8: Fault parameters and quality of fit as a function of the amplitude of slip