IJESRT

International Journal of Engineering Sciences & Research Technology

ANALYSIS & OPTIMIZATION OF HEAVY SHELL TILTING FIXTURE BY USING FINITE ELEMENT METHOD

Kartik Upadhyay*1, Vikas Sharma2 & KrishnaWaghe3

*1M.Tech Scholar, Dept. of Mechanical Engineering MIT Indore , (M.P.), India

2& 3Assistant Professor, Dept. of Mechanical Engineering MIT Indore , (M.P.), India

DOI: 10.5281/zenodo.1147530

ABSTRACT

Heavy Engineering is related to manufacturing of process plant equipment related to refinery and fertilizer, ship building etc.Manufacturing of reactor is related to heavy engineering industries. Reactor is made of two or more shell. Shell is an empty vessel which is manufactured by bending the plate. During welding of the vessels, vessels should be in a particular position for appropriate welding. For particular position of vessels, tilting is necessary. Tilting of vessel is an important part of heavy engineering industries. Tilting of vessel is done by use of two cranes, they have some limitation. Because of these reasons new mechanism required for tilting of heavy shell or vessel. Heavy shell tilting fixture is mainly used for tilting the shell from horizontal to vertical and vice versa. For certain types of application shell have to move vertical to horizontal at that time this fixture is used. During the time of assembly of Dished End and shell, both vessels should be in a horizontal position. At that time shell tilting will necessary. Second time during the heat treatment, vessel should be in a horizontal position for proper adjustment into furnace.

Design and analysis of heavy shell tilting fixture for weight capacity 120 MT will use in this work. The design includes design of fixture, design of support stool, design of saddle, selection of wire rope sling. Dimensions obtained from design, have been used for G.A (General Assembly) drawing, 2-D manufacturing drawings. Modeling of 3-D components and assembly of tilting fixture have been done with the help of CREO software and analysis of 3-D structure and analysis with ANSYS

KEYWORDS: Manufacturing, Bending, Tilting fixtures, Pressure vessel, Ansys.

- INTRODUCTION

A fixture is a device for locating, holding and supporting a work piece. Fixtures are essential elements of production processes as they are required in most of the automated manufacturing, inspection, and assembly operations. Fixtures must correctly locate a work piece in a given orientation with respect to measuring device, or with respect to another component, as for instance in assembly or welding. Such location must be invariant in the sense that the devices must clamp and secure the work piece in that location for the particular processing operation. Heavy shell tilting fixture is mainly used for tilting the shell from horizontal to vertical and vice versa. For certain types of application shell have to move vertical to horizontal at that time this fixture is used.

- METHODOLOGY

Material and Constructional detail of fixture

Structural steel - IS 2062 (Grade B)

E = 210 GPa

Density = 7033 Kg/m3

Yield Strength = 240 MPa

Poisson's Ratio = 0.3

Factor of Safety = 2

Weight of fixture - 50.737 MT [18]

Structure - Box section

Bottom curvature plate - 30 mm thick

Thick side plate - 30 mm thick

Inner plate - 12 mm thick

Upper (top) plate - 20 mm thick

3D model of tilting Fixture

Figure 1 Tilting Fixture

Weight Optimization Of Tilting Fixture

Optimization is the act of obtaining best result under given circumstances. In weight optimization, weight is reducing at optimum level. There are different schemes are given for reduction (optimization) of weight of tilting fixture but here in this paper we shows the safe scheme.

Weight reduction scheme

Weight reduction : 4.85 MT

Structure : box section

Extra plates are cut from both sides

Upper plate - 20 mm

Side plates - 12 mm

Bottom curvature plate - 30 mm

Inner gusset plate -12 mm

Figure 2: Weight reduction scheme

- ANALYSIS CONDITION: NEUTRAL POSITION

Element : 4 noded rectangular

Meshing : shell mesh

Material : structural steel

Load applied by gravity

Constraint : at ground level

12 mm side plates

30 mm plates for more strength in critical zone

Figure 3: Mesh model

Figure 4: Stress generated in tilting fixture

Deformation produced in tilting fixture

Figure 5: Deformation produced in tilting fixture

- ANALYSIS CONDITION: STATIC POSITION

Figure 6: Mesh model

Stress generated in tilting fixture

Figure 7: Stress generated in tilting fixture

Deformation produced in tilting fixture

Figure 8: Deformation produced in tilting fixture

Analysis condition: Neutral position

Maximum stress is 277.61 MPa which is in local area.

Deflection of fixture is 1-3 mm

Result of analysis is within limit, so for particular loading condition in neutral position fixture is safe.

Analysis condition: Static position

Maximum stress is 288..64 MPa which is in local area.

Deflection of fixture is 1-3 mm.

Result of analysis is within limit, so for particular loading condition in static position fixture is safe.

- CONCLUSION

As define in problem, design and analysis of tilting fixture has been done for weight capacity of 125 MT. After comparing the results of original and modified tilting fixture, some observations were made. These observations are as below:

In weight reduction scheme,Results of structural analysis for neutral position and static position were safe.

- The weight reduction of fixture was 4.85 MT.

- The maximum stress produced in fixture was 146 MPa.

- The displacement of fixture was 1-3 mm.

- In critical region of fixture, stresses were inacceptable level.

- FUTURE SCOPE

- Automation of tilting fixture

- Design of tilting mechanism for automation

- Chain drive through reduction gearbox and idler rollers

- Loading / Unloading : By crane

- Operation : By gearbox and motor

##### REFERENCES

- L. P. Zick “Stresses in Large Horizontal Cylindrical Pressure Vessels on Two Saddle Supports”, The Welding Journal Research Supplement, Pages: 959 –970, Sept 1951.

- Shafique M.A. Khan “Stress distributions in a horizontal pressure vesseland the saddle supports”, International Journal of Pressure Vessels and Piping,V, Pages: 239 –244, Oct 2016.

- Mr. Amarnath Y. Zore “Design and Optimization of Saddle For Horizontal Pressure Vessel”, International Engineering Research Journal,Special Issue, Issue 2, Pages: 4201 –4203, 2015.

- Limbasiya Vimal, “Design and Analysis of Fixture for Heavy Shell Tilting ”, Institute Of Technology, Nirma University, Ahmedabad,Pages: 1 –5, Dec 2010.

- Brian Dorchik “Design of a 5-Axis Fixture System”, Worcester polytechnic institute,Aug2006.

- .Shailesh S.Pachbhai “A Review on Design of Fixtures”, International Journal of Engineering Research and General Science,Volume 2, Issue 2, Pages: 126 –146, Feb 2014.

- Shailesh S.Pachbhai “Design And Development Of Hydraulic Fixture For Machining Hydraulic Lift Housing”, International Journal of Mechanical Engineering and Robotics Research, Volume 3, Issue 3, Pages: 204 –214, July 2014

CITE AN ARTICLE

It will get done by IJESRT Team