Pseudo-Direct Drive for Blade Pitch Control Actuation

Radu DRAGAN, Robert BARRETT, Gregg WILSON, Robert KINGHORN, David POWELL.

Magnomatics Limited, Park House, Bernard Road, Sheffield, UK, S2 5BQ

The full paper is published in the proceedings of the Recent Advances in Aerospace Actuation Systems and Components Conference, September 25-27, 2023, Toulouse, France. Full copies are available from:

Mrs. Patricia JARRY, R3ASC'23 Organization secretary
INSA SAIC, 135 Avenue de Rangueil
31077 Toulouse Cedex 4, France
Phone +33 (0)5 6155 9531, fax +33 (0)5 6155 9538
email patricia.jarry@insa-toulouse.fr

ABSTRACT

This paper presents an investigation into the design and control of a fault tolerant magnetically geared Pseudo Direct Drive (PDD®[1]) motor as part of a propeller blade Pitch Control Mechanism (PCM) of an Open Rotor Engine (ORE).  This type of engine offers significant fuel savings over incumbent technology and is the subject of significant research. As the blades are rotating, the actuation mechanism must by necessity also rotate. This presents challenges in the transfer of electrical power and results in highly mechanically stressed electrical and electromagnetic components. It was found that the PDD motor has significant advantages for this application, where continuously high levels of holding torque to react the aerodynamic loads require high torque density to meet mass targets, whilst the intrinsic magnetic gear provides a passive torque fuse to protect the actuator mechanism in the case of a fault.

KEYWORDS

Actuation, brushless machine, Direct drive, fault-tolerant, magnetic gear, PDD, pitch control

I INTRODUCTION

Open Rotor Engines have been the subject of significant research and development (Cleansky projects OREAT I and II[2], GREENBARRELS[3], CFM RISE programme) where the goal is to produce an engine with 20% fuel reduction compared with incumbent technology. The PCM in this paper is based on transfer of electrical power through a high-frequency rotating transformer [4] to a rotating actuator featuring a PDD motor, roller-screw, and kinematic mechanism as shown in Figure 1. The challenging overall mass target for the PCM resulted in significant research in to lightweighting of all the components. This resulted in the adoption of a relatively high pole number and gear ratio for the PDD. A high pole number results in reduced core mass whilst a high gear ratio results in reduced copper mass due to the reduced high-speed rotor (HSR) torque requirements (but is a trade-off with mechanical stress and speed-related losses).

II Pseudo Direct-Drive electrical machine

A highly innovative, fault-tolerant (FT) electromechanical actuation system is proposed for the PCM, centred around the PDD. The PDD topology shown in Figure 2 represents a synchronous PM surface mount electrical machine which is magnetically and mechanically integrated with a magnetic gear (MG). The MG element consists of the inner high-speed rotor (HSR), ferromagnetic pole-piece output rotor (PPR) and stationary magnet array mounted on the stator bore. The PM brushless machine element consists of the inner HSR which is synchronously coupled to the fundamental component of the stator field produced by the winding. The electromagnetic torque produced on the HSR is transferred to the PDD output rotor due to the interaction of the HSR field with the asynchronous space harmonic field created by the PPR modulation of the stator magnet array PM field. By employing a PDD, the mechanical gear stages can be completely removed, with the HSR of the PDD being connected to the load through the magnetic gear element. Due to the inherent compliance and torque limiting characteristics of the magnetic gear element, the mechanical drive train is effectively isolated from the negative effects of the kinetic energy stored in the high-speed rotor. This embedded torque fuse characteristic of the PDD can reliably protect against jamming of the mechanical actuation system due to load side overload.

2.1 Fault-tolerant PDD slot-pole-gear ratio selection

In order to select a suitable fault tolerant winding configuration, a hazard rate analysis of the different winding topologies and associated electronics was conducted by the authors of [5]. The redundant winding topologies considered can be separated into 3-phase winding topologies with duplex, triplex or quadruplex configurations (with each 3-phase lane supplied from independent 6-switch inverters) and configurations with 3, 4, 5 and 6 independent phases with each phase supplied from independent H-bridge inverters. Figure 3 shows the hazard rate of the different configurations with the assumptions that out of a total of N independent channels of each winding topology, only M are required for normal operation of the fault tolerant PDD.

It is shown in Figure 3 that the duplex 3-phase winding configuration results in the lowest hazard rate, with the capability of accepting 1 faulted lane. In order to achieve a fault tolerant (FT) PDD design, the principles of magnetic, electrical, physical and thermal isolation must be respected [6], [7]. This is enabled by selecting a single layer concentrated PDD winding topology which can be arranged in a duplex 3-phase

configuration. This allows the PDD to operate with one faulty 3-phase lane. Furthermore, each slot only contains the conductors of one phase, with any two adjacent phases being separated by a stator tooth. This ensures physical and thermal isolation between the different phases. Magnetic isolation is also implemented due to the reduced mutual coupling between adjacent phases, such that the fault current in one phase does not induce large voltages in the adjacent phases.

The slot-pole combinations compatible with the duplex winding configurations are given in Table 1. A parametric optimization routine was used in 2D electromagnetic finite element analysis to investigate the effects of the leading design parameters for the slot-pole-gear ratio variants in Table 1. This was achieved by varying the various geometric features of the PDD.

Table 1. Slot-pole combinations compatible with duplex winding configurations

Optimum designs were selected based on the minimum referred output rotor inertia and active length, due to dynamic response and space envelope constraints. Figure 4 shows the variation of the referred load side inertia with active length, while the PDD active weight and efficiency variations are shown in Figure 5 and 6. For a 12-slot FT configuration, the inertia increases with the magnetic gear ratio while the active weight is constant with gear ratio and increases linearly with active length. The 24s16p variant of the same magnetic gear ratio has a higher referred inertia but significantly lower active weight compared to the 12-slot variant due to the higher torque density. The higher pole number results in a reduced stator back-iron thickness which allows for both airgap diameters to be increased, resulting in an improved torque output for a fixed stator outer diameter.

2.2 FT PDD Nominal Operation

Several operating modes can be defined for the fault tolerant PDD: The open-circuit mode (OCOC) assumes both 3-phase lanes are open circuit. In this scenario, the performance of the embedded magnetic gear element can be characterised. The active-passive (APOC) mode assumes one 3-phase lane is active and supplying rated power to the load and the second 3-phase lane is passive (OC). The active-active (AA) mode assumes each lane supplies half rated power to the load. This has the advantage of having half the copper loss of the APOC mode. For APOC operation, Figure 7 shows the variation of the average output torque with phase input current and magnetic gear load angle, for the 24S16P topology. For the average output torque values, Figure 8 shows that the 24S16P exhibits a similar torque ripple to the 12S8P variant.

The major difference between the 24S16P and 12S8P variants under healthy operation is related to the unbalanced magnetic pull (UMP) acting on the HSR (Figure 9) and PPR (Figure 10). A low UMP is important to reduce noise and vibration and bearing size and prolong bearing life.

The UMP of the 12S8P is substantially higher when compared to the 24S16P variant under APOC operation. This is because the duplex 12S8P topology has only 1 coil per phase per 3-phase lane compared to the 2 coils per phase per 3-phase lane of the 24S16P variant. Moreover, the coils of the 24-slot variant can be grouped such that each lane has phase coils which are diametrically opposed around the stator core. This enables the 24S16P topology to effectively suppress UMP under APOC mode.

The peak-to-peak UMP amplitude of the two slot-pole combinations, is shown in Table 2 for the various operating modes. This comparison includes a fault case (OCSC), where one lane is under open-circuit and the other under symmetrical 3-phase short circuit. It can be observed that where both 3-phase lanes are equally excited, the 12S8P has low UMP, comparable to the 24S16P variant. This shows that the high UMP of the FT 12S8P topology is not a function of the magnetic gear ratio and is introduced by the single coil per phase per lane coupled with utilising only one lane in APOC mode. This unwanted UMP behaviour is completely avoided in the 24S16P topology.

Table 2. UMP variation of 24S16P and 12S8P with operation mode

2.3 FT PDD Faulty Operation

The duplex 3-phase winding topology enables the PDD to operate with one faulty 3-phase lane. For any of the short circuit cases, it is important to ensure that the short circuit current does not exceed the value of the rated current. So copper losses remain unchanged, enabling the PDD to operate with symmetrical 3-phase short circuit without exceeding the thermal limits. The independent inverter of each lane can be used to trigger a symmetrical 3-phase short circuit in order to supress the large fault currents associated with phase-to-phase faults or inter-turn short circuit.

Since the 2D cross-section of the PDD is fixed as a result of the parametric scan for reduced output rotor inertia and active weight, the phase inductance was adjusted to limit the short circuit current by adjusting the slot leakage inductance component. The short circuit transient response at the rated PPR speed is shown in Figure 11 with the steady state lane 2 total s/c current being limited to the rated value of the healthy lane 1 phase current.

III Thermal Model

A finite element (FE) thermal model was developed to understand the operating temperatures of the PDD under various conditions. Figure 12 shows the FE model and Table 3 shows the current densities at various operating conditions. The coolant temperature is assumed to be 120˚C with a mass flow rate of 0.14kg/s. The material properties are shown in Table 4.

Table 3. Lane 1 and 2 current densities

| Operating conditions | Lane 1 current density (ARMS/m2) | Lane 2 current density (ARMS/m2) |
| --- | --- | --- |
| AA | 1.23x106 | 1.23x106 |
| APOC | 2.46x106 | 0 |
| APSC | 2.46x106 | 2.45x106 |

Table 4. Material properties for the thermal analysis

| Part Description | Density (kg/m3) | Specific Heat Capacity (J/kg.K) | Thermal Conductivity Transverse (W/m.K) | Thermal Conductivity Axial (W/m.K) |
| --- | --- | --- | --- | --- |
| Windings | 4,500 | 385 | 4 | 190 |
| Stator Laminations | 8,150 | 465 | 28 | 3 |
| Slot Liner | 1,000 | 1,000 | 0.15 | 0.15 |
| Outer Magnets | 7,500 | 450 | 8 | 8 |
| Casing and Frame | 2,700 | 890 | 180 | 180 |

The PDD motor is air cooled with aluminium finned heat sinks on the outer surface of the laminations. The fins were represented by thermal convection at the outer diameter of the casing assuming that the fins exist (not modelled). The thermal convection of the fins is calculated from the Nusselt number, thermal conductivity of the fin material and length of the fins. The Nusselt number is calculated from the Reynolds number. A practical fin design must also be manufacturable so optimum fin geometry was not chosen. An effective fin thermal convection coefficient of 500W/m2K was calculated. The worst-case condition in terms of temperature was determined to be the holding condition where peak torque is held with minimal rotor movement, hence the speed related electromagnetic losses were neglected. Figure 13 shows results from the steady-state model for this worst-case condition. It can be seen that increasing the fin thermal convection above 200W/m2K results in diminishing returns. The APSC operating mode resulted in the highest temperatures (see Figure 14 which shows temperature distribution in the stator in APSC condition,) and all results from the FE analysis are summarised in Table 5.

Table 5. Summary of predicted temperatures for active passive short-circuit (APSC)

| Region | Temp (֯C) @h=200W/m2K | Temp (֯C) @h=400W/m2K | Temp (֯C) @ h=500W/m2K |
| --- | --- | --- | --- |
| Copper | 204 | 173 | 167 |
| Outer Magnets | 193 | 163 | 157 |
| PPR | 188 | 159 | 153 |
| HSR | 188 | 159 | 153 |
| Fins | 175 | 146 | 141 |

IV PCM TEST APPARATUS

The test apparatus is designed to replicate the rotating environment of the ORE (see Figure 15 for reference). Counterweights provide speed related blade torque load (approx. 3000 Nm per blade) which is transferred through the linkage system to the roller-screw and PDD motor. The blade angular position is measured using an encoder located at the blade root. This also allows the overall system stiffness and dynamic response to be measured. The non-contacting rotating transformer provides the power required for the actuator.  The power electronics are located at the front of the rotating section. The available cooling air from the engine (120C) is routed into the rotating section at the rear, cooling the PDD motor and the power electronics before exiting at the rear. Temperature and vibration on the rotating PCM will be monitored via wireless connection, whereas more critical control signals use a slip-ring located at the rear.  The test program will consist of several discreet sub-assembly tests before the main PCM assembly is tested in Q4 2023.

V PCM Simulink Model

A dynamic Simulink model of the PCM was developed to define the torque requirements of the PDD. The PCM model is made up of subsystems including the blades, linkages, screw and PDD.

## 5.1 Mathematical model

Equations (1) and (2) are the fundamental mechanical equations of a PDD. Equation (1) represents the HSR and Equation (2) represents the PPR.

|  | (1) |
| --- | --- |
|  | (2) |

Where Jh is the HSR inertia in kg m-2; h is the HSR angular position in rads; TEM is the electromagnetic torque applied to HSR in Nm; TG  is the maximum gear torque in Nm; ph is the number of pole pairs on the HSR; np is the number of pole pieces in the PPR; p is the PPR angular position; Jp  is the PPR inertia in kg.m2 and TL is the load torque on the PPR.

The torque and position relationship between blade roots and the PDD were calculated from the simple mechanics of the linkages in between the blade roots and the roller screw. Efficiency of the roller screw was considered through a look-up table in the model (forward and back-driven operation).

Various machine designs have been considered, with shorter, larger OD PDDs resulting in a higher HSR inertia than the longer, smaller OD PDDs. Due to the gearing between the HSR and the blade, the HSR inertia dominated the total inertia, resulting in a significant dynamic torque requirement, in turn resulting in a larger motor with higher inertia.

## 5.2 Controller design

A cascade controller was implemented for the system (Figure 16). A position control loop on the blade root position feeds a demand to a speed control loop on the HSR. This produces the PDD torque demand. A feedforward controller is used in conjunction with the feedback control to compensate for the known disturbance from the blade load and the position PI controller is gain scheduled. The controller incorporates physical limits of the designed system for two reasons: 1) So that the controller doesn’t force the system to exceed them and 2) to apply preventative control on these constraints. These constraints include a power limit imposed by the engine which is limited using the speed of the system to calculate a dynamic torque limit.

Max torque limit on the HSR was defined by the maximum drive current. The speed of the HSR is limited by setting a saturation limit on the demand of the speed control loop. The gain-scheduled feedback controller was tuned by running a multi-step profile, stepping up and down in angle across the range. The terms for each step were tuned to give vectors of P and I terms for the position controller. These vectors formed lookup tables based on position. For small angle changes, the real position should be within prescribed bounds of the demand. For larger steps, a settling time was defined. Figure 17 shows simulated demand and actual blade pitch angle which settles within the specified 2 seconds.

Conclusions

A fully electric blade pitch control mechanism (PCM) has been designed for an Open Rotor Engine. The PCM is based on motive power from a Pseudo-Direct Drive (PDD) motor which is a mechanical and magnetic integration of a magnetic gear and a permanent magnet motor. A high torque density PDD with 2x3-phase fault tolerant windings has been designed with low unbalanced magnetic pull. The PDD was found to be significantly lighter than an equivalent direct drive permanent magnet motor. The thermal environment and the requirement to provide rated torque with a 3-phase short-circuit in 50% of the windings required relatively low current density to remain within material temperature limits. A Simulink model of the full PCM that included a mechanical model of the PDD has been developed and used to investigate dynamic performance and provide a necessary input to the PDD design in the trade-off between inertia and overall mass. The developed cascade controller was able to provide effective control throughout the full duty cycle.

The full-scale PCM is currently in construction and results will be made available in Q1 2024.

References

[1] K. Atallah, J. Rens, S. Mezani, D. Howe, "A Novel “Pseudo” Direct Drive Brushless Permanent Magnet Machine," IEEE Transactions on Magnetics, vol. 44, no. 11, pp. 4349-4352, Nov. 2008.

[2] https://cordis.europa.eu/project/id/296515/reporting

[3] https://cordis.europa.eu/project/id/306997/reporting

[4] H. Vardhan, M. Odavic, K. Atallah, “Design Optimization Methodology for High-Frequency Rotary Transformers for Contactless Power Transfer Systems,” IEEE ECCE, October 2022.

[5] E. K. Hussain, K. Atallah, M. Odavic, R. S. Dragan, and R. E. Clark, “Pseudo Direct Drive electrical machines for flight control surface actuation,” in 8th IET International Conference on Power Electronics, Machines and Drives (PEMD 2016), 2016, pp. 1–6.

[6] B. C. Mecrow, A. G. Jack, J. A. Haylock, and J. Coles, “Fault-tolerant permanent magnet machine drives,” IEE Proc. - Electr. Power Appl., vol. 143, no. 6, p. 437, 1996.

[7] J. W. Bennett, G. J. Atkinson, B. C. Mecrow, and D. J. Atkinson, “Fault-Tolerant Design Considerations and Control Strategies for Aerospace Drives,” IEEE Trans. Ind. Electronics., vol. 59, no. 5, pp. 2049–2058, May 2012.

Acknowledgements

The work is supported by CleanSky2 Joint Undertaking JTI-CS2-2019-CfP10-LPA-01-77: Advanced Pitch Control Mechanism TRL4 Demonstration. The JU receives support from the European Union’s Horizon 2020 research and innovation programme and the Clean Sky 2 JU members other than the Union. The views and results communicated in this paper reflect only the author's view. The EU Joint Undertaking is not responsible for any use that may be made of the information it contains.

Figure 1. PCM simplified diagram 			Figure 2. Fault-tolerant PDD topology

Figure 3. Hazard rate of fault tolerant PDD winding/inverter combinations

Figure 4. Variation of referred load side inertia with active length - optimum designs

Figure 5. Variation of PDD active weight with active length - optimum designs

Figure 6. Variation of efficiency (2D FEA) with active length - optimum designs

Figure 7. Variation of PDD output torque and phase current with PDD load angle - APOC 24S16P

Figure 8. Variation of PDD output torque ripple with PDD load angle – APOC

Figure 9. HSR UMP APOC

Figure 10. PPR UMP - APOC

Figure 11. Symmetrical 3-phase short circuit current response at rated output speed

Figure 12. FE model of the fault tolerant PD.

Figure 13. Maximum winding temperature with variation in thermal convection from the fins

Figure 14. Thermal Analysis Results (APSC at 500W/m2K)

Figure 15. PCM test rig arrangement (cut-through)

Figure 16. Cascade controller developed for the PCM

Figure 17. Demand and actual (predicted) pitch angle.