# FLEXPART-CTM User’s Guide

Version 1.1.0

Stephan Henne

2018-05-12

# History

## Changes since version 1.0.0

- New module for nudging simulations towards observations

- Improved NetCDF grid and totals output

- Replaced binary totals output by netcdf output

# Overview

It is recommended to get acquainted with the FLEXPART user guide of the main FLEXPART release before reading this guide (https://www.flexpart.eu).

The FLEXPART CTM (chemistry transport model) extension utilises a global domain filled with particles that are constantly advected throughout the atmosphere. Each particle can carry different chemical species and clocks that give the time since when a particle has left certain atmospheric regions. Emissions of a species are not simulated as particle releases in this concept, as it would be done in standard FLEXPART, but are taken up by particles situated in the vicinity of the emission source.

FLEXPART_CTM builds on FLEXPART version 8.0. Several bug fixes were reported up to the current version 9.01. These were included were influencing the particle transport. Deposition is currently not considered in the CTM version and routines were therefore not kept up to date.

# Elements of the extension

## SPECIES definition

The species definition form standard FLEXPART had to be extended. See appendix for an example.

## Clocks and region definition

In the CTM extension every particle carries several clocks according to defined regions in the atmosphere. The clocks count the time since a particle entered (negative count) or left (positive count) a certain region. Regions can be of three different vertical representation types:

- Boundary layer regions

- Stratospheric regions

- Total column regions

The domain and resolution of the region masks has to agree with the dimensions of the meteo fields. The folder containing the region files is given in the COMMAND file. The files have to follow the naming convention: region.mask.<nnn>, where nnn is the region number with preceding zeros. The format of the gridded region files is “FORMATED” ASCII and has the following structure:

‘(2F12.3)’	smallest and largest longitude covered in degree east (grid centre)

‘(2F12.3)’	smallest and largest latitude covered in degree north (grid centre)

‘(F12.3)’	resolution in x-direction

‘(F12.3)’	resolution in y-direction

‘(I5)’	Indicator of vertical criterion type

‘(nxI4)’	mask values (0: outside or 1: inside region), one line for each latitude, starting in the south

The vertical criterion type corresponds to three possible atmospheric layers: (1) boundary layer regions, (2) stratospheric regions, (3) total column regions.

Every species can be connected to 1 clock. Output of this species is split into ageclasses using the standard ageclass definition of output field, but according to the particle clock and not the release time of the particle. The age class definition was modified in comparison to the original version to allow for negative ages (within region). An example is given in the Appendix.

In the previous versions of the CTM extensions the region clocks added an additional dimension to the gridded and receptor output for which the available dimension for separate output for different release points was used (maxpointspec_act). However, this proved to be too storage intensive.

## RELEASES setup

Although the CTM version of FLEXPART does not create any particle releases after the initial domain filling, a release file needs to be set-up to give the domain that should initially be filled with particles and set the species carried by each particle. A single release point needs to be specified. Usually this would be the global domain. Furthermore, the total number of particles is set in the RELEASE file. Note that for every species that should be carried in the simulation the index of the species file needs to be given in the first block of the file (same as in FLEXPART, see Appendix). In addition, the field “Total emitted mass” (again for each species) can be used to set initial concentrations (see section 2.4). The first species in the RELEASE file needs to be a passive air mass tracer. This is assumed during initialisation, but no check on the species settings is performed. The passive mass tracer can later be used to convert concentration output to mass mixing ratios or mole fractions.

## Initialisation

Three different possibilities for initialising particle masses are provided. Most of the time simulations will start based on previous particle dumps and will therefore not require initialisation (see section 2.5). Initialisation is only done if the IPIN switch in the COMMAND file is set to 0 and the domain filling switch MDOMAINFILL is set to 1. Depending on the availability of input files containing initial mixing ratios in the specified output directory the different initialisation methods are carried out.

It is assumed that the first species selected in the RELEASES file contains a passive air mass tracer. It is initialised by dividing the atmosphere in vertical columns of 1° by 1° degree and assigning a certain number of particles per grid column so that their total mass matches the grid column dry air mass.

All three types of initialisation use an initial dry air mole fraction interpolated to the initial particle position, i. The dry air mole fraction is then converted to a mass on the particle, Mi, proportional to the air tracer mass on the particle (species 1), Ma. Since the air tracer mass represents moist air, dry and moist air densities at particle position have to be taken into account in the conversion. The initial mass of species i on a particle is then given by

,

where d and m are the densities of dry and moist air, respectively, and i and d are the molar weights of the species i and dry air, respectively.

### Globally constant concentration

The simplest approach is to set all particles masses according to a global average mixing ratio for each species. This option does not require any additional initialisation files in the output directory, but average mole fractions need to be provided in units ppbv (referred to moles of dry air) in the RELEASES file as an entry for each species in the line entitled “Total emitted mass” (see Appendix).

### Latitudinal profile

For each species for which a file latitude_profile_nnn, with nnn being the species run index including preceding zeros, exists in the output directory, the initialisation will be based on this profile and the “Total emitted mass” given in the RELEASES file. The product of both values should give mole fractions in units ppbv referred against moles of dry air. The profile files have to be header free, fixed format ASCII files containing three floating point columns with a width of 13 characters. The first giving the latitude, the second the latitude dependency of the mixing ratio and the third an additional floating point parameter that is currently not used. The latitudes have to be the same as in the wind fields used for the simulation.  An example is given below.

-90.000000  1726.722719     0.968752
  -89.000000  1726.738067     0.968760
  -88.000000  1726.754810     0.968770
...

### Gridded fields

The third possible initialisation uses gridded input data. It is carried out only if gridded data files are available in the output directory for all species (except for species 1 carrying the air mass tracer). The gridded initialisation data have to have the same dimensions as the output grid as given in file OUTGRID. Gridded initialisation data have to reside in the output directory following the naming convention grid_ppbv_<YYYYMMDDhhmmss>_<nnn>.bin with <YYYYMMDDhhmmss> being the time stamp of the simulation start and <nnn> the species run index including preceding zeros. The grid output has to be provided as mole fractions in ppbv referred against moles of dry air. The format of the gridded output is “UNFORMATTED” binary (no record end bytes) and has the following structure:

integer	number of points in longitudinal direction
integer	number of points in latitudinal direction
integer	number of vertical levels
float x2	smallest and largest longitude covered
float x2    smallest and largest latitude covered
float x2	lowest and highest vertical level
float x nz  level tops, starting with the lowest level

float x nx*ny*nz	mixing ratios in units ppbv in order x,y,z

## Restart

For a restart from a previous run the header file and a particle dump file (partposit_end) from the previous run must exist. These need to reside in the current output directory and need to be renamed to header_previous and partposit_end_previous. In addition, the IPIN switch in the COMMAND file has to be set to 1. Finally it is possible to carry over the emission residual field (see below) to a restarted run. To do so the file emission_residuals from the previous run needs to be copied to the output directory of the current run.

## Emission handling

For each species emissions can be specified. Emissions are not handled as particle releases (as in standard FLEXPART) but are picked up by particles close to the surface. If a species is only emitted from a certain region it is possible to define a region clock for the same region and link the species to this clock. This allows studying the age of emitted species by region along its transport path.

Emissions are added to particles residing below the diagnosed boundary layer height of each grid cell. There is no further vertical distribution of emissions. If at a certain time step no particles are present in a certain grid box, the current emissions are added to an emission residual field that will determine the emissions in the next time step when particles are present in a grid box. In order to prevent extreme loading of individual particles arriving in the boundary layer of a grid cell with large stored emissions, only 20% of the latter are released per time step. The emission time step corresponds to the synchronisation time step defined in the COMMAND file.

Emission fields can be time-dependent. The same concept followed by FLEXPART for wind fields is followed for emission fields, meaning that available emission files are listed in an AVAILABLE file in the case of emissions residing in the same directory as the emission files. The folder containing the emission data is given in the COMMAND file. The emission files don’t have to follow a specific naming convention as long as they are listed correctly (time) in the AVAILABLE file. However, it is recommended to give the time stamp in the emission file as well. Emissions of all species for one time are stored in one file. The domain and resolution of the emission field has to agree with the dimensions of the meteo fields.

The file format of the gridded emissions is either “UNFORMATTED”, “DIRECT” access binary (no record end bytes) with the following structure

Integer (size 4)	number of points in longitudinal direction (nx)

integer (size 4)	number of points in latitudinal direction (ny)

integer (size 4)	number of species in file (nspec), has to agree with number of species in RELEASES file

float 2 (size 4)	smallest and largest longitude covered

float 2 (size 4)	smallest and largest latitude covered

for each species

integer (size 4)        if 0 no emissions for the current species in file

(float nx*ny (size 4)	emissions in units kg/s/grid cell in order y,x)

Or emissions can be read from netcdf files with dimensions lat, lon and time, where time only contains one entry corresponding to the one specified in the AVAILABLE files. For a species with emissions a variable SPECIES_<nnn>, where nnn indicates the species number, has to be present in the NetCDF file. If no variable with this name is present emissions for this species are set to zero. To switch to NetCDF input the NCDF_IN_EM variable has to be set to .true. in the par_mod.f file.

Emissions should be given in units kg/s/grid cell.

## OH reaction

The destruction of gaseous species due to reaction with OH is assumed to be a pseudo first order loss process following:

,

where OH concentrations are assumed to be constant during an integration step, so that

.

Five different functional relationships of k on atmospheric temperature, T, and molecule number concentration, M, are included in the model:

These can be listed in the REACTIONS definition files (see Appendix) and the parameters a1 to a3 have to be given there as well. Finally an entry in each SPECIES file (see Appendix) links to the required reaction. The resulting rate constant, k, needs to be in units cm3/N/s.

### Time variable OH input

OH number concentrations in untis N/cm3 are used in the model. OH input data have to reside in the folder given in the COMMAND file and follow the names given in an AVAILABLE file residing in this folder. The dimensions and grid spacing do not need to correspond to the underlying wind fields. The vertical grid spacing should be equidistant. The format of the gridded OH field is either “UNFORMATTED”, “DIRECT” access binary (no record end bytes) with the following structure

integer	number of points in longitudinal direction
integer	number of points in latitudinal direction
integer	number of vertical levels
float x2	smallest and largest longitude covered
float x2    smallest and largest latitude covered
float x2	lowest and highest vertical level
float x nz  level tops, starting with the lowest level

float x nx*ny*nz	OH number concentrations in units N/cm3 in order x,y,z

or OH number concentrations can be read from netcdf files with dimensions lat, lon, lev and time, where time only contains one entry corresponding to the one specified in the AVAILABLE files. OH number concentrations should be saved in the variable OH. Note, that the NCDF_IN_OH switch in par_mod.f needs to be set to .true. if NetCDF input should be used.

### Monthly OH climatology

If no AVAILABLE file exists in the OH directory monthly OH fields are looked for. These should have the name OH_<MM>.bin or OH_<MM>.nc depending on the chosen input file format, MM being the month of the year. These fields are assumed to represent a climatology and are used for any year. OH monthly fields should have the same structure as time variable fields.

## Chemical loss rates (CLR)

In addition to the OH reaction, other first order chemical loss reactions can be described by

where clr specifies a loss rate that can be provided as a three dimension field.

### Climatological CLR input

Climatological CLR fields in untis 1/s are used in the model. CLR input data have to reside in the folder given in the COMMAND file and follow the naming convention lossrates_<MM>.bin or lossrates_<MM>.nc, where MM is the 2 digit representation of the month of the year with preceding zeros. The dimensions and grid spacing do not need to correspond to the underlying wind fields. The vertical grid spacing should be equidistant. The format of the gridded CLR field is either “UNFORMATTED”, “DIRECT” access binary (no record end bytes) with the following structure

integer	number of points in longitudinal direction
integer	number of points in latitudinal direction
integer	number of vertical levels
float x2	smallest and largest longitude covered
float x2    smallest and largest latitude covered
float x2	lowest and highest vertical level
float x nz  level tops, starting with the lowest level

float x nx*ny*nz	chemical loss rate in units 1/s in order x,y,z

or chemical loss rates can be read from netcdf files with dimensions lat, lon, lev and time, where time only contains one entry corresponding to the month of the year. Chemical loss rates should be saved in the variable CLR. Note, that the NCDF_IN_CLR switch in par_mod.f needs to be set to .true. if NetCDF input should be used.

## Receptor output

The standard receptor output of FLEXPART was modified to incorporate receptors above model ground and receptor output by age class. Furthermore, the uncertainty of the receptor output can be assessed using the same concept as implemented for grid output, namely sub-sampling of tagged particle classes.

Receptor mass concentrations are calculated with a kernel method

,

where x,y,z are the coordinates of the receptor, hx,hy,hz are kernel widths and rx,ry,rz are defined by

,

with X,Y,Z being the coordinates of an individual particle.

The volume integral of the kernel function K has to be equal to 1. The following kernel function is implemented in FLEXPART

.

The factor w depends on the relative elevation of the receptor above model ground

.

The factor f equals 2 and 1 for receptors at the model surface (previous implementation in standard FLEXPART) and at elevations larger than hz, respectively.

For each receptor hx, hy, hz can be set individually in the RECEPTORS file. See the example in the appendix.

## Mass shift

In addition to clocks a second way to trace the age of emissions was implemented. This concept utilised a cascade of species, each species holding emissions from a certain period only. To use this function a set of species has to be set up using the mass shift index in each SPECIES files and a mass shift interval has to be defined in the COMMAND file.

In the SPECIES file the parameter “species from which mass is shifted (0: none)” can be set to a certain species with a smaller index in the RELEASE file as the current index. Assume the species appear in the same order in the RELEASES file as given by their indices and  say that SPECIES_001 should shift all its mass to SPECIES_002, this would be defined in the SPECIES_002 file by using index 1. Furthermore, emissions are only defined for SPECIES_001. During the first mass shift interval emissions would only accumulate in SPECIES_001. At the end of the interval all mass from SPECIES_001 is then shifted to SPECIES_002. SPECIES_002 now contains the mass emitted during the previous mass shift interval. Adding more species to this chain one can trace the time of emission. The mass finally accumulates in the terminating species of this chain (basically carrying the background mass).

To reduce the model output particle clocks should not be used in combination with mass shift.

## Nudging

The particle masses can be relaxed towards observations using a simple nudging technique. If activated, at each synchronisation time step an additional mass tendency due to nudging is calculated for each particle within the kernel of an observation. The kernel weight of each observation is defined by its spatial and temporal extend. For the spatial kernel weight, ws, an Epanechnikov function is used (similar to the receptor kernel, see above) for each particle-observation pair (i,j)

,

where rij is calculated from the kernel extent in each room direction (hx,hy,hz)

and the factor fs is the same as used for the calculation of receptor concentrations above. Its value only depends on the relative position of the observation location relative to the ground and the value of hz.

For the temporal kernel weight, wt, a tricubic weight function was chosen, resulting in a somewhat broader kernel shape and a smoother transition to zero

,

where ht is the temporal kernel width, tk the current model time and tl the time of the closest valid observation.

The nudging tendency for a particle is then calculated as

,
where Mj is the observed mass, which is calculated from observed mole fractions by , Mair being the mass of the air tracer that is assigned as the first species in the simulation. n is the nudging relaxation time scale and t the model synchronisation time step. The latter should be smaller than the former to assure numerical stability.

If more than on observation lies within the temporal kernel width only the closest is used.

Kernel widths and nudging relaxation times can be given individually for each observation location.

Two input files are required in order to switch on nudging: the NUDGING control file and a netcdf file holding all the observations to be used in the current run. Observation locations are identified by the names given in the NUDGING control file. These names have to match the names given in the observation file. If a name defined in the NUDGING control file is not present in the netcdf observation file the program will continue to run but, obviously, not be able to assign any observations for this observation location. The format of the nudging control and observation files is given in the appendix.

# Running FLEXPART_CTM

## Compiling FLEXPART_CTM

For compiling FLEXPART_CTM on IPAZIA a run job exists in the FLEXPART_CTM main directory. Submit it to the queuing system using qsub compile. Make sure to modify the Makefile in the sub-folder src according to your setup. The following libraries are required: grib_api_f90, grib_api and jasper. If netcdf input is used netcdf libraries (netcdff, netcdf) and include files need to linked as well.

The Makefile contains machine specific settings. For compiling on a new machine a new block of settings should be added. Use make clean to remove previous object files. The file Makefile.depends contains the projects dependencies. When changing the program structure (adding new files/subrountines/functions; using different project subroutines/functions within others) you will need to run the make depends command before recompiling.

## Run script

A script (run_CTM) running the FLEXPART_CTM step by step for a longer period with defined restart possibilities can be found in the scripts sub-folder. The script creates its own run jobs that are send to the queuing system. Therefore this script has to be run on the IPAZIA frontend and should not be submitted to the queuing system itself.

Settings within the run_CTM script:

###########################################################################

#	SETTINGS

###########################################################################

# directory where executable is located

set EXE_DIR =      "/home/hes134/FLEXPART80CTM"

# directory where output should be stored in sub-folders for each month

set MAIN_OUT_DIR =  "/mnt/workspc/hes134/FLEXPART80CTM/output/URMEL_INI_11"

# obsolete

set MAIN_BACK_DIR = "/mnt/storage/hes134/FLEXPART80CTM/output/URMEL_INI_11"

# directory where input files are stored

set OPTIONS_DIR =  "/home/hes134/FLEXPART80CTM/input/URMEL_INI"

# directory where wind fields reside

set FIELDS_DIR =   "/mnt/storage/hes134/ecmwf_data/en/"

# path to wind fields AVAILABLE file

set AVAILABLE_FN = "/mnt/storage/hes134/ecmwf_data/en/AVAILABLE"

#	WHEN STARTING WITH THE FIRST MONTH RESTART SHOULD BE 0 (so that an

# initialisation is started)  AND 1 FOR RESTARTED RUNS

set RESTART = 1

#	TO WHOM EMAIL MESSAGES SHOULD BE SEND

set RECIPIENT = "stephan.henne@empa.ch"

#	start time of simulation (YYYYMMDDhhmmss)

# (possible restart conditions or initial conditions are taken from output

# sub-folder for this month)

set START_DTM = 20040401000000

#	end time of simulation (YYYYMMDDhhmmss)

set END_DTM =   20050101000000

#	Define how the simulation is divided into different FLEXPART runs. If BY_MONTH equal

# to 1 each month is treated in a new FLEXPART run and the parameter STEP is

# overwritten with the number of seconds in the current month

set BY_MONTH = 1

#	If BY_MONTH is set to 1 $STEP will be overwritten and contain the length of the

# current month in seconds

set STEP = 86400

#	interval (s) for checking queue status

set SLEEP_INT = 30

#	settings for the queuing system

set WALLTIME = "72:00:00"

set MEM = "3072mb"

set QUEUE = "serial"

###########################################################################

#	END OF SETTINGS

############################################################################

## Output

The model was set up to output dry air mole fractions (ppt). This requires setting the IND_RECEPTOR option to 2 and the IOUT setting to 2. The IND_SOURCE option should be on 1 for mass emissions. Note, that this is a parameter combination that is not allowed in standard FLEXPART. Here an adjusted version of the output handling was introduced, but all other combinations of the above parameters were not tested.

It was intended that the model would directly yield mole fraction output. However, when analysing the output it became clear that the noise in the particle distribution was too large and added too much noise to the derived mole fraction fields and comparisons to measurements improved when mole fractions were recalculated from the mole fractions of the tracer of interest and the mole fractions of the air mass tracer. This is currently the recommended procedure.

Mole fractions (grid or receptor) are calculated by summation of tracer masses on particles and referencing these to the grid scale density at particle position.

, for all j within current output volume.

When referring a tracer against dry air the dry air density is used and otherwise the moist air density. When using the air tracer for an additional scaling of the obtained tracer mole fractions. The air tracer should be referred to moist air mole fractions (see SPECIES file) but given the molar weight of dry air. With this setup and a simulation with a sufficiently large number of particles one should obtain air tracer mole fractions of unity everywhere.

### Gridded output

The gridded output format was not modified in comparison to the original FLEXPART version > 8. For each defined grid output time step an individual output file is created for each species. In the previous version of the CTM extension the output dimension usually used for separating contributions from different releases (maxpointspec_act) was used for the different region clocks. In the current version this is not the case anymore and since every species can only be influenced by one clock the releases dimension in the output is only 1.

Compared to standard FLEXPART the way how mass mixing ratios are calculated was changed. In the CTM Version if IND_RECEPTOR=2 the atmospheric density in the outgrid is considered at every sampling time and not only at the output time. This seemed necessary for the use of long time averages (1 month) of the output field.

Setting the NCDF_OUT switch in par_mod.f to .true. outputs one single NetCDF file at every output time step containing information from all species. Next to the mean mole fractions the NetCDF output also contains the uncertainty of these mole fractions, based on the particle tagging method already available in FLEXPART. Note that these uncertainties are not included in the standard output and the switch luncout in outg_mod.f90 has to be set to .true..

### Extended receptor output

A single receptor output file is created during one run (receptor_pptv). The format of the file is either FORMATTED binary or NetCDF (depending on compiler settings) and has the following structure.

FORMATTED binary:

integer	number of receptor output uncertainty classes

character 16*nrec		receptor names, 16 byte strings each

float 6*nrec	receptor positions and bandwidths for each receptor in the following order: x, y, z, hx, hy, hz

For each time step

integer	time since start of simulation in seconds

For each region clock

For each age class

For each species

float nrec	mixing ratio or concentration for each receptor

if (number of receptor output uncertainty classes larger than 1)

float nrec	mixing ratio or concentration uncertainty

end of species loop

float nrec	average number of particles in each sub-sampling

float nrec	average sum of kernel weights in each sub-sampling

NetCDF:

dimensions:

time = UNLIMITED ; // (0 currently)

rec = 347 ;

pnt = 1 ;

age = 1 ;

nchar = 16 ;

variables:

int time(time) ;

time:units = "seconds since 1989-01-01 00:00" ;

time:calendar = "proleptic_gregorian" ;

float rec(rec) ;

rec:longname = "receptors" ;

rec:units = "indexed" ;

float pnt(pnt) ;

pnt:longname = "release points" ;

pnt:units = "indexed" ;

float age(age) ;

age:longname = "particle age" ;

age:units = "seconds" ;

char receptorname(rec, nchar) ;

receptorname:longname = "receptor name" ;

float lon(rec) ;

lon:longname = "receptor longitude" ;

lon:units = "degrees_east" ;

float lat(rec) ;

lat:longname = "receptor latitude" ;

lat:units = "degrees_north" ;

float lev(rec) ;

lev:longname = "receptor altitude" ;

lev:units = "meters" ;

float hx(rec) ;

hx:longname = "receptor kernel width in longitudinal direction" ;

hx:units = "degrees" ;

float hy(rec) ;

hy:longname = "receptor kernel width in latitudinal direction" ;

hy:units = "degrees" ;

float hz(rec) ;

hz:longname = "receptor kernel width in vertical direction" ;

hz:units = "meters" ;

For each species the VMR, the average (time) count of particles and the average sum of kernel weights is stored in the ncdf file.

float Air\ tracer(time, age, pnt, rec) ;

Air\ tracer:units = "pptv" ;

Air\ tracer:longname = "mean recepotor VMR" ;

float nn_Air\ tracer(time, age, pnt, rec) ;

nn_Air\ tracer:units = "counts" ;

nn_Air\ tracer:longname = "number of particles at receptor" ;

float xk_Air\ tracer(time, age, pnt, rec) ;

xk_Air\ tracer:units = "" ;

xk_Air\ tracer:longname = "average kernel weight at receptor" ;

In terms of output units the same as for the grid output applies to the receptor output: while the output is already given in mole fractions, smoother results are obtained by recalculating the mole fractions from the target tracer and the air mass tracer.

### Total mass and flux output

A single output file is created during one run (totals.nc) containing information on the total masses and fluxes due to emissions and OH reaction for each species and for each synchronisation step in FLEXPART (see position 13 in COMMAND file). The format of this file is netcdf and the following variables are contained in the file  (example output):

dimensions:

species = 2 ;

time = UNLIMITED ; // (2976 currently)

nchar = 18 ;

variables:

int time(time) ;

time:units = "seconds since 2013-01-01 00:00" ;

time:calendar = "proleptic_gregorian" ;

char species(species, nchar) ;

species:long_name = "Species names" ;

double mass(time, species) ;

mass:units = "kg" ;

mass:long_name = "Total global mass" ;

double emissions(time, species) ;

emissions:units = "kg s-1" ;

emissions:long_name = "Actual emission flux" ;

double em_fld(time, species) ;

em_fld:units = "kg s-1" ;

em_fld:long_name = "Emission flux in fields" ;

double em_res(time, species) ;

em_res:units = "kg s-1" ;

em_res:long_name = "Emission residuals" ;

double OH_loss(time, species) ;

OH_loss:units = "kg s-1" ;

OH_loss:long_name = "Mass loss through OH reaction" ;

double OH_gain(time, species) ;

OH_gain:units = "kg s-1" ;

OH_gain:long_name = "Mass gain from OH reaction" ;

double CLR_loss(time, species) ;

CLR_loss:units = "kg s-1" ;

CLR_loss:long_name = "Mass loss through other reactions" ;

double nudging(time, species) ;

nudging:units = "kg s-1" ;

nudging:long_name = "Mass flux due to nudging" ;

# Appendix

## Extended COMMAND file

An example of a COMMAND file is given below. The following switches should not be changed for a CTM run, since they might conflict with some of the adjustments:

1. should always be 1 for forward runs

12. no particle splitting in CTM runs

16. should either be 1,2 or 3, but was only tested for 2

22. should be 0

24. should be 1 for domain filling, even for restarted runs!

28. some changes for nested output are not implemented yet. So it should be switched off.

* ********************************************************************************

*                                                                              *

*      Input file for the Lagrangian particle dispersion model FLEXPART        *

*                           Please select your options                         *

*                                                                              *

********************************************************************************

1. __                3X, I2

1

LDIRECT           1 FOR FORWARD SIMULATION, -1 FOR BACKWARD SIMULATION

2. ________ ______   3X, I8, 1X, I6

20040401 000000

YYYYMMDD HHMISS   BEGINNING DATE OF SIMULATION

3. ________ ______   3X, I8, 1X, I6

20040501 000000

YYYYMMDD HHMISS   ENDING DATE OF SIMULATION

4. _____             3X, I5

2592000

SSSSS             GRID OUTPUT EVERY SSSSS SECONDS

5. _____             3X, I5

2592000

SSSSS             TIME AVERAGE OF GRID OUTPUT (IN SSSSS SECONDS)

6. _____             3X, I5

900

SSSSS             SAMPLING RATE OF GRID OUTPUT (IN SSSSS SECONDS)

7. _____             3X, I5

1

Number of UNCERTAINTY CLASSES OF GRID OUTPUT

8. _____             3X, I5

86400

SSSSS             RECEPTOR OUTPUT EVERY SSSSS SECONDS

9. _____             3X, I5

86400

SSSSS             TIME AVERAGE OF RECEPTOR OUTPUT (IN SSSSS SECONDS)

10._____             3X, I5

900

SSSSS             SAMPLING RATE OF RECEPTOR OUTPUT (IN SSSSS SECONDS)

11._____             3X, I5

9

Number of UNCERTAINTY CLASSES OF RECEPTOR OUTPUT

12._________         3X, I9

999999999

SSSSSSSSS         TIME CONSTANT FOR PARTICLE SPLITTING (IN SECONDS)

13._____             3X, I5

900

SSSSS             SYNCHRONISATION INTERVAL OF FLEXPART (IN SECONDS)

14. ---.--           4X, F6.4

-5.0

CTL              FACTOR, BY WHICH TIME STEP MUST BE SMALLER THAN TL

15. ---              4X, I3

4

IFINE            DECREASE OF TIME STEP FOR VERTICAL MOTION BY FACTOR IFINE

16. -                4X, I1

2

IOUT             1 CONCENTRATION (RESIDENCE TIME FOR BACKWARD RUNS) OUTPUT, 2 MIXING RATIO OUTPUT, 3 BOTH,4 PLUME TRAJECT., 5=1+4

17. -                4X, I1

2

IPOUT            PARTICLE DUMP: 0 NO, 1 EVERY OUTPUT INTERVAL, 2 ONLY AT END

18. _                4X, I1

1

LSUBGRID         SUBGRID TERRAIN EFFECT PARAMETERIZATION: 1 YES, 0 NO

19. _                4X, I1

1

LCONVECTION      CONVECTION: 1 YES, 0 NO

20. _                4X, I1

0

LAGESPECTRA      AGE SPECTRA: 1 YES, 0 NO

21. _                4X, I1

1

IPIN             CONTINUE SIMULATION WITH DUMPED PARTICLE DATA: 1 YES, 0 NO

22. _

0                4X,I1

IOFR             IOUTPUTFOREACHREL CREATE AN OUPUT FILE FOR EACH RELEASE LOCATION: 1 YES, 0 NO

23. _                4X, I1

0

IFLUX            CALCULATE FLUXES: 1 YES, 0 NO

24. _                4X, I1

1

MDOMAINFILL      DOMAIN-FILLING TRAJECTORY OPTION: 1 YES, 0 NO, 2 STRAT. O3 TRACER

25. _                4X, I1

1

IND_SOURCE       1=MASS UNIT , 2=MASS MIXING RATIO UNIT

26. _                4X, I1

2

IND_RECEPTOR     1=MASS UNIT , 2=MASS MIXING RATIO UNIT

27. _                4X, I1

0

MQUASILAG        QUASILAGRANGIAN MODE TO TRACK INDIVIDUAL PARTICLES: 1 YES, 0 NO

28. _                4X, I1

0

NESTED_OUTPUT    SHALL NESTED OUTPUT BE USED? 1 YES, 0 NO

29. __________       4X, I10

2592000

SSSSSSSSSS       Interval for mass shift (0: no mass shift)

30. _                A

/mnt/storage/spc134/FLEXPART80CTM/input/URMEL_INI/emissions/

EM_PATH          PATH TO EMISSION FIELDS, IF EMPTY EMISSION SWITCHED OFF, (include / at the end)

31. _                A

/mnt/storage/spc134/FLEXPART80CTM/input/URMEL_INI/regions/

REG_PATH          PATH TO REGION MASKS (include / at the end)

32. _                A

/mnt/storage/spc134/FLEXPART80CTM/input/URMEL_INI/OH/

OH_PATH          PATH TO OH fields (include / at the end)

33._                 A

/mnt/storage/spc134/FLEXPART80CTM/input/URMEL_INI/CLR/

CLR_PATH         PATH TO add. chemical loss rate fields (include / at the end)

1. Simulation direction, 1 for forward, -1 for backward in time

(consult Seibert and Frank, 2004 for backward runs)

2. Beginning date and time of simulation. Must be given in format

YYYYMMDD HHMISS, where YYYY is YEAR, MM is MONTH, DD is DAY, HH is HOUR,

MI is MINUTE and SS is SECOND. Current version utilizes UTC.

3. Ending date and time of simulation. Same format as 3.

4. Average concentrations are calculated every SSSSS seconds.

5. The average concentrations are time averages of SSSSS seconds

duration. If SSSSS is 0, instantaneous concentrations are outputted.

6. The concentrations are sampled every SSSSS seconds to calculate the time

average concentration. This period must be shorter than the averaging time.

7. Number of uncertainty classes used for grid output. Has to be equal to, a multiple of, or

a divisor of the number of uncertainty classes for receptor output.

8. Average receptor concentrations are calculated every SSSSS seconds.

9. The average receptor concentrations are time averages of SSSSS seconds

duration. If SSSSS is 0, instantaneous concentrations are outputted.

10. The receptor concentrations are sampled every SSSSS seconds to calculate the time

average concentration. This period must be shorter than the averaging time.

11. Number of uncertainty classes used for receptor output. Has to be equal to, a multiple of, or

a divisor of the number of uncertainty classes for grid output.

12. Time constant for particle splitting. Particles are split into two

after SSSSS seconds, 2xSSSSS seconds, 4xSSSSS seconds, and so on.

13. All processes are synchronized with this time interval (lsynctime).

Therefore, all other time constants must be multiples of this value.

Output interval and time average of output must be at least twice lsynctime.

14. CTL must be >1 for time steps shorter than the  Lagrangian time scale

If CTL<0, a purely random walk simulation is done

15.IFINE=Reduction factor for time step used for vertical wind

16.IOUT determines how the output shall be made: concentration

(ng/m3, Bq/m3), mixing ratio (pptv), or both, or plume trajectory mode,

or concentration + plume trajectory mode.

In plume trajectory mode, output is in the form of average trajectories.

17.IPOUT determines whether particle positions are outputted (in addition

to the gridded concentrations or mixing ratios) or not.

0=no output, 1 output every output interval, 2 only at end of the

simulation

18.Switch on/off subgridscale terrain parameterization (increase of

mixing heights due to subgridscale orographic variations)

19.Switch on/off the convection parameterization

20.Switch on/off the calculation of age spectra: if yes, the file AGECLASSES

must be available

21. If IPIN=1, a file "partposit_end" from a previous run must be available in

the output directory. Particle positions are read in and previous simulation

is continued. If IPIN=0, no particles from a previous run are used

22. IF IOUTPUTFOREACHRELEASE is set to 1, one output field for each location

in the RLEASE file is created. For backward calculation this should be

set to 1. For forward calculation both possibilities are applicable.

23. If IFLUX is set to 1, fluxes of each species through each of the output

boxes are calculated. Six fluxes, corresponding to northward, southward,

eastward, westward, upward and downward are calculated for each grid cell of

the output grid. The control surfaces are placed in the middle of each

output grid cell. If IFLUX is set to 0, no fluxes are determined.

24. If MDOMAINFILL is set to 1, the first box specified in file RELEASES is used

as the domain where domain-filling trajectory calculations are to be done.

Particles are initialized uniformly distributed (according to the air mass

distribution) in that domain at the beginning of the simulation, and are

created at the boundaries throughout the simulation period.

25. IND_SOURCE switches between different units for concentrations at the source

NOTE that in backward simulations the release of computational particles

takes place at the "receptor" and the sampling of particles at the "source".

1=mass units (for bwd-runs = concentration)

2=mass mixing ratio units

26. IND_RECEPTOR switches between different units for concentrations at the receptor

1=mass units (concentrations)

2=mass mixing ratio units

27. MQUASILAG indicates whether particles shall be numbered consecutively (1) or

with their release location number (0). The first option allows tracking of

individual particles using the partposit output files

28. NESTED_OUTPUT decides whether model output shall be made also for a nested

output field (normally with higher resolution)

29. Interval for mass shifts in seconds.

30. PATH TO EMISSION FIELDS, IF EMPTY EMISSION SWITCHED OFF, (include "/" at the end of path name)

31. PATH TO REGION MASKS, (include "/" at the end of path name)

32. PATH TO OH fields, (include "/" at the end of path name)

33. PATH TO CLR fields, (include "/" at the end of path name)

## RELEASES file

Example of the RELEASES file for the LORAT setup.

*************************************************************************

*                                                                       *

*                                                                       *

*                                                                       *

*   Input file for the Lagrangian particle dispersion model FLEXPART    *

*                        Please select your options                     *

*                                                                       *

*                                                                       *

*                                                                       *

*************************************************************************

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

15

___                        i3    Total number of species emitted

1

___                        i3    Index of species in file SPECIES

2

___                        i3    Index of species in file SPECIES

3

___                        i3    Index of species in file SPECIES

4

___                        i3    Index of species in file SPECIES

5

___                        i3    Index of species in file SPECIES

6

___                        i3    Index of species in file SPECIES

7

___                        i3    Index of species in file SPECIES

8

___                        i3    Index of species in file SPECIES

9

___                        i3    Index of species in file SPECIES

10

___                        i3    Index of species in file SPECIES

11

___                        i3    Index of species in file SPECIES

12

___                        i3    Index of species in file SPECIES

13

___                        i3    Index of species in file SPECIES

14

___                        i3    Index of species in file SPECIES

15

___                        i3    Index of species in file SPECIES

=========================================================================

20050101 000000

________ ______            i8,1x,i6 Beginning date and time of release

20050101 000000

________ ______            i8,1x,i6 Ending date and time of release

-179.0000

____.____                  f9.4  Longitude [DEG] of lower left corner

-90.0000

____.____                  f9.4  Latitude [DEG] of lower left corner

180.0000

____.____                  f9.4  Longitude [DEG] of upper right corner

90.0000

____.____                  f9.4  Latitude [DEG] of upper right corner

1

_________                  i9    1 for m above ground, 2 for m above sea level

0.000

_____.___                  f10.3 Lower z-level (in m agl or m asl)

50000.000

_____.___                  f10.3 Upper z-level (in m agl or m asl)

3000000

_________                  i9    Total number of particles to be released

1.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.0000E00

_.____E__                  e9.4  Total mass emitted

0.110E00

_.____E__                  e9.4  Total mass emitted

0.340E00

_.____E__                  e9.4  Total mass emitted

0.167E00

_.____E__                  e9.4  Total mass emitted

0.128E00

_.____E__                  e9.4  Total mass emitted

0.081E00

_.____E__                  e9.4  Total mass emitted

0.079E00

_.____E__                  e9.4  Total mass emitted

0.096E00

_.____E__                  e9.4  Total mass emitted

DOMAINFILL

________________________________________   character*40 comment

+++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++++

## REACTIONS definition file

Reactions with OH can be defined in the REACTIONS file. The type parameter is associated with the functional character of the rate constant (see above). The parameter fields correspond to the parameters for the rate constant calculation. The index and comment fields are only for documentation. The example given here is for CH4 and CO reactions with OH. Within a SPECIES file an OH reaction can be specified according to the position in the REACTIONS file.

INDX TYPE Para 1     Para 2     Para 3     Comment

---- ---- ---------- ---------- ---------- -------------------------------------

1    3   9.65E-20    1.082E3     2.58E0 CH4 + OH

2    5    1.3E-13  7.541E-23        0.0 CO + OH

## Extended SPECIES files

The standard species definition files for FLEXPART had to be extended to accommodate various features of the extended version. Listed below is an example of a species definition file for a methane tracer undergoing OH loss and additional chemical loss due to other reactions. The given OH reaction refers to the reactions listed in REACTIONS (see above for details). The associated species of the OH degradation is species 2 with a molecular yield of 86 %: species therefore receives 86 % of the mass lost due to OH reaction from the CH4 tracer.

****************************************************************************

*                                                                          *

*     Input file for the Lagrangian particle dispersion model FLEXPART     *

*               Definition file of chemical species/radionuclides          *

*                                                                          *

****************************************************************************

CH4_1                Tracer name

-999.9             Species half life

-9.9E-09           Wet deposition - A

Wet deposition - B

-9.9               Dry deposition (gases) - D

Dry deposition (gases) - Henrys const.

Dry deposition (gases) - f0 (reactivity)

-9.9E09            Dry deposition (particles) - rho

Dry deposition (particles) - dquer

Dry deposition (particles) - dsig

-9.99              Alternative: dry deposition velocity

16.00             molweight

1                  switch on additional chemical loss reactions

1                  OH reaction (0 for none)

-9                 number of associated specias (neg. none)

-99.99             KOA - organic matter air partitioning

1                  Clock index, <0 no clock used

1                  Use dry (1) or moist (0) air density when needed for output

2                  index of associated species for OH-reaction

0.86               molar yield of associated species for OH-reaction

Parameters new (or modified) in the extension are:

- Switch of additional chemical loss reactions: should be 0 for a species without additional loss reactions and 1 otherwise.

- OH reaction: 0 for no OH reaction. Otherwise the index of the reaction defined in the REACTIONS file.

- Species from which mass is shifted: 0 no mass shift. Otherwise the index of the species (in the order occurring in the RELEASE file) from which is mass is transferred to the current species.

- Number of associated species: has to do with aerosol/gas partitioning (not required for gases).

- KOA: aerosol/gas partitioning coefficient (not required for gases)

- Clock index: 0 no clock used, positive number will try to locate a region mask with the same index.

- Use dry or moist air density for output: When producing mass mixing ratios or mole fractions these can either be referred to the dry atmospheric mass or the moist atmospheric mass. Standard procedure would be to output the moist VMR for the air tracer and the dry VMR for any other trace species.

- Index of associated species for OH-reaction: Index of species (in the order occurring in the RELEASE file) receiving mass lost from the current species due to OH degradation

- Molar yield of associated species for OH-reaction: Molar yield of the amount of the current species lost through OH degradation that is then added to the associated species.

It is actually possible to enter more than one associated species by adding other pairs of index and molar yield. No attempt is made to verify that the total molar yield is not larger 1.

The next example is that of CO. No further species carrying the OH reaction product is associated with the CO tracer, therefore the definition file is missing the last two columns as compared to the CH4 tracer.

****************************************************************************

*                                                                          *

*     Input file for the Lagrangian particle dispersion model FLEXPART     *

*               Definition file of chemical species/radionuclides          *

*                                                                          *

****************************************************************************

CO_1               Tracer name

-999.9             Species half life

-9.9E-09           Wet deposition - A

Wet deposition - B

-9.9               Dry deposition (gases) - D

Dry deposition (gases) - Henrys const.

Dry deposition (gases) - f0 (reactivity)

-9.9E09            Dry deposition (particles) - rho

Dry deposition (particles) - dquer

Dry deposition (particles) - dsig

-9.99              Alternative: dry deposition velocity

28.00             molweight

0                  switch on additional chemical loss reactions

1                  OH reaction (0 for none)

-9                 number of associated specias (neg. none)

-99.99             KOA - organic matter air partitioning

1                  Clock index, <0 no clock used

0                  Use dry (1) or moist (0) air density when needed for output

For the passive air mass tracer which should be the first species include in the RELEASES file the following settings should be used

****************************************************************************

*                                                                          *

*     Input file for the Lagrangian particle dispersion model FLEXPART     *

*               Definition file of chemical species/radionuclides          *

*                                                                          *

****************************************************************************

Air tracer         Tracer name

-999.9             Species half life

-9.9E-09           Wet deposition - A

Wet deposition - B

-9.9               Dry deposition (gases) - D

Dry deposition (gases) - Henrys const.

Dry deposition (gases) - f0 (reactivity)

-9.9E09            Dry deposition (particles) - rho

Dry deposition (particles) - dquer

Dry deposition (particles) - dsig

-9.99              Alternative: dry deposition velocity

28.97             molweight

0                  switch on additional chemical loss reactions

0                  OH reaction (0 if none)

0                  species from which mass is shifted (0: none)

-9                 number of associated specias (neg. none)

-99.99             KOA - organic matter air partitioning

-9                 Clock index, <0 no clock used

1                  Use dry (1) or moist (0) air density when needed for output

## Extended RECEPTORS file

In contrast to original FLEXPART each receptor point requires 4 additional parameters in the CTM setup: the height of the receptor above model ground and the three bandwidths of the receptor kernel.

********************************************************************************

*                                                                              *

*      Input file for the Lagrangian particle dispersion model FLEXPART        *

*                       Please specify your receptor points                    *

*     For the receptor points, ground level concentrations are calculated      *

*                                                                              *

********************************************************************************

1.  ----------------  4X,A16

DMV 0           NAME OF RECEPTOR POINT

RECEPTORNAME

2.  ------.----       4X,F11.4

117.8436        GEOGRAFICAL LONGITUDE

XRECEPTOR

3.  ------.----       4X,F11.4

4.9814       GEOGRAFICAL LATITUDE

YRECEPTOR

4.  ------.----       4X,F11.4

0.0000       altiude in meters above ground

ZRECEPTOR

5.  ------.----       4X,F11.4

1.0000       kernel width in east-west direction in degree

HXMAX

6.  ------.----       4X,F11.4

1.0000       kernel width in north-south direction in degree

HYMAX

7.  ------.----       4X,F11.4

150.0000       kernel width in vertical direction in metres

HZMAX

================================================================================

...

## AGECLASSES file

Example of an ageclass file. The given ages in seconds correspond to the lower bound of the ageclass. Negative values correspond to times spend within a region, while positive numbers correspond to times after leaving a region. The class 0 stand for particle clocks there were not initialised yet. In this example the first ageclass contains particles that resided with a region for longer than a day, the second age class is for partilces that resided in a region for less than a day, the third is the class for non initialised particles, the forth class contains particles that have left a region less than a day ago, the fifths contains particles that left a region between 1 and 2 days ago, etc. In contrast to original FLEXPART particles are not removed after reaching the maximum age specified!

************************************************

*                                              *

*Lagrangian particle dispersion model FLEXPART *

*         Please select your options           *

*                                              *

*This file determines the ageclasses to be used*

*                                              *

*Ages are given in seconds. The first class    *

*starts at age zero and goes up to the first   *

*age specified. The last age gives the maximum *

*time a particle is carried in the simulation. *

*                                              *

************************************************

11          Integer        Number of age classes

-86400

-1

0

1

86400

172800

345600

604800

1296000

2592000

5184000

## NUDING control file

The nudging control file is similar to previous FLEXPART input files, providing the required information in a custom ASCII file format. Next to some general parameters for the nudging set-up each observation site is defined in the file. An example structure is given below:

********************************************************************************

*                                                                              *

*      Input file for the Lagrangian particle dispersion model FLEXPART        *

*                       Please specify your receptor points                    *

*     For the receptor points, ground level concentrations are calculated      *

*                                                                              *

********************************************************************************

/mnt/project/hes/MAIOLICA2/input/observations/wdcgg_for_FLEXPART_201212_201402.nc

# location of observation file (complete path)

1

# number of nudged species

CH4

# names of nudged species (one per line)

1

# maximum number of FLEXPART species contributing to a nudged species

2

# FLEXPART species contributing to nudged species CH4
================================================================================

1.  ----------------  4X,A16

ALT             NAME OF OBSERVATION POINT

NAME

2.  ------.----       4X,F11.4

-62.5200        GEOGRAFICAL LONGITUDE

XOBSERVATION

3.  ------.----       4X,F11.4

82.4500       GEOGRAFICAL LATITUDE

YOBSERVATION

4.  ------.----       4X,F11.4

0.0000       altiude in meters above ground

ZOBSERVATION

5.  ------.----       4X,F11.4

15.2217 kernel width in east-west direction in degree

HXMAX

6.  ------.----       4X,F11.4

2.0000     kernel width in north-south direction in degree

HYMAX

7.  ------.----       4X,F11.4

500.0000     kernel width in vertical direction in metres

HZMAX

8.  ---------.-       4X,F11.1

172800.0     temporal nudging kernel width in seconds

WIDTH_TEMP

9.  ---------.-       4X,F11.1

7200.0     temporal nudging relaxation timescale seconds

TAU

=============================================================================

The first line (after the header) specifies the location of the observation file used for the current simulation.

The second information line (separated by a comment line #) gives the number of observed species that should be considered.

The third line gives the names of the observed species (if more than one species, each should be given on a new line but without separating comment line). These are not the names of the FLEXPART SPECIES but the names given in the observation file.

The fourth block then gives the maximum number of FLEXPART species that are connected with an observed species. More than one FLEXPART species can be connected to the same observed species assuming that the sum of the simulated species corresponds to the observed species. In this way tagged tracers (for different regions or source types) can be used in the simulation and nudged towards the same observation.

For each observed species the indices of the connected FLEXPART species in the simulation (not the SPECIES index)  is then given as separate block with each FLEXPART species on a new line and a comment line (#) separating block for different observed species. In this way it is also possible to have different numbers of FLEXPART species for each observed species.

After a further comment line (===== this time) blocks for each observation location are given. It is important that the observation names given here match those in the observation file name. All other parameters have been explained in the nudging section.

## Observation file

Observations for the nudging module have to be given in a single netcdf file covering the whole simulation period. The file structure should be as given in the example below:

dimensions:

time = 10249 ;

nameobs = 81 ;

nchar = 16 ;

variables:

double time(time) ;

time:units = "days since 1970-01-01" ;

time:long_name = "time" ;

time:calendar = "proleptic_gregorian" ;

char nameobs(nameobs, nchar) ;

nameobs:long_name = "Observation site name" ;

float CH4(nameobs, time) ;

CH4:units = "ppb" ;

CH4:_FillValue = -9999.9f ;

CH4:long_name = "atmospheric mole fraction of CH4" ;

Here time needs to refer to the elapsed time since 1970-01-01 in days and nameobs should be names of the observation locations as given in the NUDGING definition file (trailing spaces are ignored, but preceding spaces should be avoided). For each observation variable a two-dimensional array (dimensions site and time) needs to be given. Missing values have to indicated by a negative value. The observations should be given as dry air mole fractions given in units ppb. Using this format all observations have to be brought onto the same temporal dimension resulting in a large number of missing values when flask sampling data is mixed with continuous observations aggregated to hourly means.