// -*- mode: c++; coding: utf-8; tab-width: 4; indent-tabs-mode: nil; c-basic-offset: 4; show-trailing-whitespace: t -*- vim:fenc=utf-8:ft=cpp:et:sw=4:ts=4:sts=4

namespace Feel {

/** \page HeatSink Heat Sink

\author Christophe Prud'homme

\author Baptiste Morin

\date 2011-06-28

\tableofcontents

<br>

<hr>

<br>

This problem considers the performance of a heat sink designed for the thermal management of high-density electronic components. The heat sink is comprised of a base/spreader which in turn supports a number of plate fins exposed to flowing air. We model the flowing air through a simple convection heat transfer coefficient. From the engineering point of view, this problem illustrates the application of conduction analysis to an important class of cooling problems: electronic components and systems.

<br>

<br>

Our interest is in the conduction temperature distribution at the base of the spreader. The target is to study how the heat transfer occurs with different parameters on our heat sink. The heat generated by high-density electronic components is such that it's very expensive to cool large structures (data center). The cooling optimization is consequent in the run for decreasing operating costs.

A classical thermal CPU cooler looks like this

<center>

<table border=0px>

<tr>

<td>\image html heatsink/complete_cooler.png</td>

</tr>

<tr>

<td><center>Mesh of a classical CPU cooler</center></td>

</tr>

</table>

</center>

We are here going to describe how it is theoretically working and how it is implemented with \feel.

\section HeatSink_Description Description

\subsection Description_Domain Domain

We consider here a classical "radiator" which is a CPU heat sink. Those types of coolers are composed with a certain number of plate fins exposed to flowing air or exposed to a ventilator. Regarding the periodicity and geometry of our concern, we can make our study on a characteristic element of the problem : a half cell of the heat sink single thermal fin with its spreader at the basis. Let's take a look at the geometry of our problem :

<center>

<table border=0px>

<tr>

<td>\image html heatsink/sink_geom.png</td>

</tr>

<tr>

<td><center>Geometry of heat sink</center></td>

</tr>

</table>

</center>

Our study is available in 2 or 3 dimensions, depending on the application's parameters. You'll see later how to work with it. Let's see on which meshes we are working on :

<center>

<table border=0px>

<tr>

<td>\image html heatsink/mesh_2d.png</td>

<td>\image html heatsink/mesh_3d.png</td>

</tr>

<tr>

<td><center>2D mesh</center></td>

<td><center>3D mesh</center></td>

</tr>

</table>

</center>

\subsection Description_Inputs Inputs

The implementation of these parameters is described in the section \ref Implementation_Parameters.<br>

\subsubsection Inputs_Material Material

Here the material parameter can be described with furthers parameters. We have, with \f$i=1\f$ for the fin and \f$i=2\f$ for the base :

\li the thermal conductivity \f$\kappa_i\f$

\li the material's density \f$\rho_i\f$

\li the heat capacity of the material \f$C_i\f$

The term \f$\rho_i C_i\f$ corresponds to the heat volumetric capacity. In that way, we make possible the construction of a heat sink with \f$2\f$ different materials. Here is a list of the well-known ones, \f$\rho\f$ and \f$C\f$ are gave at \f$298 K\f$ :

<table class="manual">

<tr><th>Material</th><th>Thermal conductivity<br>\f$\kappa$ in $W.m^{-1}.K^{-1}\f$</th><th>Density<br>\f$\rho$ in $kg.m^{-3}\f$</th><th>Heat Capacity <br>\f$C\f$ in \f$J.kg^{-1}.K^{-1}\f$</th></tr>

<tr><td>Aluminium <td></td>180 (alloys)<br> or 290 (pure) <td></td> 2700 <td></td> 897 </td></tr>

<tr><td>Copper  <td></td>386 <td></td>8940 <td></td>385 </td></tr>

<tr><td>Gold <td></td>314 <td></td>19320 <td></td>129 </td></tr>

<tr><td>Silver <td></td>406 <td></td>10500 <td></td>233 </td></tr>

</table>

\subsubsection Inputs_Physical Physical

\li Depth<br>

This parameter is only to take into account for the 3D simulation. It represents the depth of the characteristic heat sink and is called \c depth in the application.

\li Length<br>

You can also parameterize the length of the fin. This one is called \c L in the application's parameters, its dimension is the meter.

\li Width<br>

Typically, this parameter is linked with constructor's standards. This parameter is called \c width in the application's implementation.

\subsubsection Inputs_Thermal Thermal

\li Heat flux<br>

It represents the heat flux brought by the electronic component at the bottom of the base. Here it's typically the heat brought by the processor.

\li Thermal coefficient<br>

The thermal coefficient \f$h\f$ named \f$therm_coeff\f$ in the application is representative of the heat transfer between the fin and the air flow.

\li Ambient temperature}<br>

This parameter called \f$T_{amb}\f$ represents the temperature around the heat sink at the beginning. That means the ambient temperature before the computer is turned on.

\subsubsection Inputs_Summary Summary table

The following table displays the various fixed and variables parameters of this application.

<table class="manual">

<tr><th>Name <th></th> Description <th></th> Nominal Value <th></th> Range <th></th> Units</th></tr>

<tr><th>BDF parameters </th></tr>

<tr><td>\f$time-initial\f$ <td></td> beginning <td></td> \f$0\f$ <td></td> <td></td>  </td></tr>

<tr><td>\f$time-final\f$ <td></td> end <td></td> \f$50\f$ <td></td> \f$]0, 1500]\f$ <td></td>   </td></tr>

<tr><td>\f$time-step\f$ <td></td> time step <td></td> \f$0.1\f$ <td></td> \f$]0,1[\f$ <td></td>   </td></tr>

<tr><td>\f$steady\f$ <td></td> steady state <td></td> \f$0\f$ <td></td> \f$\{0,1\}\f$ <td></td>   </td></tr>

<tr><td>\f$order\f$ <td></td> order <td></td> \f$2\f$ <td></td> \f$[0, 4]\f$ <td></td>  </td></tr>

<tr><th>Physical parameters </th></tr>

<tr><td>\f$L\f$ <td></td> fin's length <td></td> \f$2\cdot 10^{-2}\f$ <td></td> \f$[0.02, 0.05]\f$ <td></td> \f$m\f$ </td></tr>

<tr><td>\f$width\f$ <td></td> fin's width <td></td> \f$5\cdot 10^{-4}\f$ <td></td> \f$[10^{-5}, 10^{-4}]\f$ <td></td> \f$m\f$ </td></tr>

<tr><td>\f$deep\f$ <td></td> heat sink depth <td></td> \f$0\f$ <td></td> \f$[0, 7\cdot 10^{-2}]\f$ <td></td> \f$m\f$ </td></tr>

<tr><th>Mesh parameter </th></tr>

<tr><td>\f$hsize\f$ <td></td> mesh's size <td></td> \f$10^{-4}\f$ <td></td> \f$[10^{-5},10^{-3} ]\f$ <td></td> </td></tr>

<tr><th>Fin Parameters </th></tr>

<tr><td>\f$\kappa_f\f$ <td></td> thermal conductivity <td></td> \f$386\f$ <td></td> \f$[100,500]\f$ <td></td> \f$W \cdot m^{-1} \cdot K^{-1}\f$</td></tr>

<tr><td>\f$\rho_f\f$ <td></td> material density <td></td> \f$8940\f$ <td></td> \f$[10^3,12\cdot 10^3 ]\f$ <td></td> \f$kg\cdot m^{-3}\f$</td></tr>

<tr><td>\f$C_f\f$ <td></td> heat capacity <td></td> \f$385\f$ <td></td> \f$[10^2,10^3]\f$ <td></td> \f$J\cdot kg^{-1} \cdot K^{-1}\f$</td></tr>

<tr><th>Base/spreader Parameters </th></tr>

<tr><td>\f$\kappa_s\f$ <td></td> thermal conductivity <td></td> \f$386\f$ <td></td> \f$[100,500]\f$ <td></td> \f$W \cdot m^{-1} \cdot K^{-1}\f$</td></tr>

<tr><td>\f$\rho_s\f$ <td></td> material density <td></td> \f$8940\f$ <td></td> \f$[10^3,12\cdot 10^3 ]\f$ <td></td> \f$kg\cdot m^{-3}\f$</td></tr>

<tr><td>\f$C_s\f$ <td></td> heat capacity <td></td> \f$385\f$ <td></td> \f$[10^2,10^3]\f$ <td></td> \f$J\cdot kg^{-1} \cdot K^{-1}\f$</td></tr>

<tr><th>Heat Parameters </th></tr>

<tr><td>\f$T_{amb}\f$ <td></td> ambient temperature <td></td> \f$300\f$ <td></td> \f$[300,310]\f$ <td></td> \f$K\f$ </td></tr>

<tr><td>\f$heat\_flux\f$ <td></td> heat flux \f$Q\f$ <td></td> \f$10^6\f$ <td></td> \f$[0 ,10^{6}]\f$ <td></td> \f$W \cdot m^{-3}\f$</td></tr>

<tr><td>\f$therm\_coeff\f$ <td></td> thermal coefficient \f$h\f$ <td></td> \f$10^3\f$ <td></td> \f$[0,10^3]\f$ <td></td> \f$W\cdot m^{-2} \cdot K^{-1}\f$ </td></tr>

</table>

\section HeatSink_Theory Theory

\subsection Theory_Figure Figure

The global domain is \f$\varOmega = \varOmega_1 \cup \varOmega_2 \f$ where \f$\varOmega_1\f$ is the fin's domain and \f$\varOmega_2\f$ the spreader's domain. We note \f$\partial\varOmega\f$ the border of the domain \f$\varOmega\f$. The physical lines we are using will be noted as \f$\Gamma_i\f$ such as described above. The following figure describes the parameters and the geometry we are using in the equations to solve our 3D issue :

The following figures describe the parameters and the geometry we are using in the equations to solve our 2D or 3D issue :

<center>

<table border=0px>

<tr>

<td>\image html heatsink/figure_2d.png</td>

<td>\image html heatsink/figure_3d.png</td>

</tr>

<tr>

<td><center>2D geometry details</center></td>

<td><center>3D geometry details</center></td>

</tr>

</table>

</center>

\subsection Theory_Equations Equations

Our concern satisfies the heat equation which reads

\f{eqnarray}

\sum_{i=1}^{2} \kappa_i \Delta T - \rho_i C_i \frac{ \partial T}{\partial t} & =& 0& \\

\kappa_1 \frac{\partial T}{\partial n} &= & 0 & \text{on }  \Gamma_2 \text{ and }  \Gamma_6\quad\quad \\

\kappa_2 \frac{\partial T}{\partial n} &= & 0 & \text{on } \Gamma_5 , \Gamma_7 \text{ and }  \Gamma_8\quad\quad \\

\kappa_1 \frac{\partial T}{\partial n} &= &- h( T - T_{amb}) & \text{on } \Gamma_1\quad\quad \\

\kappa_2 \frac{\partial T}{\partial n} &= & Q(1-e^{-t}) & \text{on } \Gamma_4\quad\quad \\

T_{|\varOmega_1} &= &T_{| \varOmega_2} & \text{on } \Gamma_3\quad\quad \\

\kappa_1 \nabla T \cdot n &=& \kappa_2 \nabla T \cdot n & \text{on } \Gamma_3\quad\quad

\f}

with \f$i=1\f$ for the fin and \f$i=2\f$ for the base and where \f$\kappa_i\f$ is the thermal conductivity, \f$\rho_i\f$ is the material's density (\f$kg.m^{-3}\f$ in the SI unit), \f$C_i\f$ the heat capacity and \f$T\f$ the temperature at a precise point (in 2D or 3D). To see how it has been coded, you can read \ref Implementation .

\subsection Theory_Boundary Boundary Conditions

The problem requires that the temperature and heat flux are continute on \f$\Gamma_3\f$. Considering the problem's geometry, we also impose zero heat flux on the vertical surfaces of the spreader. Let's detail the conditions we have imposed :

\li Homogeneous Neumann condition ( \f$(2)\f$ ) and ( \f$(3)\f$ ) : it represents the fact that the heat flux is only vertical (for \f$\Gamma_6\f$ and \f$\Gamma_7\f$) or the fact that the heat flux is only provided by \f$\Gamma_4\f$ (for \f$\Gamma_2\f$ and \f$\Gamma_5\f$).

\li Homogeneous Neumann condition ( \f$(4)\f$ ) : it imposes that the heat flux is brought by this surface (it mathematically represents that the heat sink is placed on the heat source).

\li Non-homogeneous Neumann condition ( \f$(5)\f$ ) :  this boundary condition represents the transient state for the heat transfer calculation.

\li Temperature continuity ( \f$(6)\f$ ) : it imposes that the temperature is continute at the interface between the two materials (if there are two materials, we can also have the same one for the two pieces).

\li Heat flux continuity ( \f$(7)\f$ ) : it represents that the heat flux is continute at the interface between the two materials. Literally, it means that the two flows offset each other.

These conditions have been coded as explained in the section \ref Implementation_Equations.

\subsection Theory_FEM Finite Element Method

Let's apply the method to our concern, we introduce the test function \f$v\f$ and we integrate the main equation, which reads now as :

\f{equation*}

\sum_{i=1}^{2} \rho_i C_i \int_{\varOmega_i} v\frac{ \partial T}{\partial t} - \kappa_i \int_{\varOmega_i} v\Delta T   & = 0

\f}

We integrate by parts, which leads to :

\f{equation*}

\sum_{i=1}^{2} \rho_i C_i \int_{\varOmega_i} v\frac{ \partial T}{\partial t} +  \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T} - \kappa_i \int_{\partial \varOmega_i} {(\nabla T \cdot n) v}   & = 0

\f}

then, by decomposing the borders \f$\partial\varOmega_i\f$, we obtain :

\f{multline*}

\displaystyle{- \kappa_1 \int_{\Gamma_1}{(\nabla T \cdot n) v} - \kappa_2 \int_{\Gamma_4} {(\nabla T \cdot n) v} - \kappa_1 \int_{\Gamma_{2,6}}{(\nabla T \cdot n) v}  - \kappa_2 \int_{\Gamma_{5,7,8}}{(\nabla T \cdot n) v} } \quad + \\ \displaystyle{ \sum_{i=1}^{2}  \rho_i C_i \int_{\varOmega_i} v\frac{ \partial T}{\partial t} + \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T} - \kappa_i \int_{\partial\varOmega_i \cap \Gamma_3}{(\nabla T \cdot n) v}  } =   0

\f}

Now, we apply the conditions \f$(2)\f$, \f$(3)\f$, \f$(4)\f$ and \f$(5)\f$ which brings us to :

\f{equation*}

\int_{\Gamma_1}{hv(T-T_{amb})} - \int_{\Gamma_4} {vQ(1-e^{-t})} + \sum_{i=1}^{2}  \rho_i C_i \int_{\varOmega_i} v\frac{ \partial T}{\partial t} + \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T}  - \underbrace{\kappa_i \int_{\partial\varOmega_i \cap \Gamma_3}{(\nabla T \cdot n) v}}_{\text{=0}}   & =   0

\f}

Now we apply the boundary conditions ( \f$(7)\f$ ) which results in :

\f{equation*}

\begin{split}

\displaystyle{ h \int_{\Gamma_1}{v(T-T_{amb})} - \int_{\Gamma_4} {vQ(1-e^{-t})}  + \sum_{i=1}^{2} \rho_i C_i \int_{\varOmega_i} v\frac{ \partial T}{\partial t} + \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T} } & =   0

\end{split}

\f}

We can now start to transform the equation by putting in the right hand the known terms :

\f{equation*}

\begin{split}

\displaystyle{ h \int_{\Gamma_1}{v T}  + \sum_{i=1}^{2} \rho_i C_i \int_{\varOmega_i} v\frac{ \partial T}{\partial t} + \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T} } & =

\int_{\Gamma_4} {vQ(1-e^{-t})}	 +  hT_{amb}\int_{\Gamma_1}{v}

\end{split}

\f}

We discretize \f$\displaystyle{\frac{\partial T}{\partial t}}\f$ where \f$\delta t\f$ is the time step, such as:

\f{equation*}

\begin{split}

\displaystyle{ h \int_{\Gamma_1}{v T} + \sum_{i=1}^{2} \rho_i C_i \int_{\varOmega_i} v\frac{T^{n+1} - T^n}{\delta t} + \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T} } & =

\int_{\Gamma_4} {vQ(1-e^{-t})}	 +  hT_{amb}\int_{\Gamma_1}{v}

\end{split}

\f}

Finally we obtain :

\f{equation*}

\color{red}

\begin{split}

\displaystyle{ h \int_{\Gamma_1}{v T} + \sum_{i=1}^{2} \rho_i C_i \int_{\varOmega_i} v\frac{T^{n+1}}{\delta t} + \kappa_i \int_{\varOmega_i} {\nabla v \cdot \nabla T}} & =

\displaystyle{ \int_{\Gamma_4} {vQ(1-e^{-t})} +  hT_{amb}\int_{\Gamma_1}{v}	+  \sum_{i=1}^{2}  \rho_i C_i \int_{\varOmega_i} v \frac{T^n}{\delta t}  }

\end{split}

\f}

This is that equation which is implemented in the application \c feel_heatsink.

\section HeatSink_Implementation Implementation

\subsection Implementation_Parameters Application Parameters

The parameters of the application are implemented such as

\snippet heatsink.cpp marker1

\subsection Implementation_Surfaces Surfaces

To be able to calculate the surfaces in further dimension without changing the code, we have given the same names for the faces we were interested in. In 2D \f$\Gamma_i\f$ represents a line whereas in 3D it represents a surface. The calculation of those surfaces which makes possible the calculation of averages temperature is as follow :

\snippet heatsink.cpp marker2

\subsection Implementation_Equations Equations

First we start by calculate the non-steady state which means that we integrate all the time-independent terms, which is done with :

\snippet heatsink.cpp marker3

Then, to compute the transient state, which means time dependant terms, you have to initialize the temperature (which is initialized as \f$T_{amb}\f$ on \f$X_h\f$ space) and create a new vector \f$F_t\f$ which corresponds to the time dependent term. The code is as follow :

\snippet heatsink.cpp marker4

\section HeatSink_Results Results

As you can see in the equation's implementation above, there are two outputs :

\li \c Gmsh format : this file contains the entire mesh and the temperatures associated to each degrees of freedom of the mesh. To open it, you juste have to do as you always do with \c Gmsh : \c gmsh heatsink-1_0.msh. You will obtain the figure with the different temperatures, you are now able to click on "play" with its significative logo and admire the evolution

\li \c averages file : this file is completed at each time step, each line contains the current time, the average temperature on \f$\Gamma_4\f$ (surface where is the contact between the heat sink and the heat source) and the average temperature on \f$\Gamma_1\f$. To analyze this file, we recommend you to work with \c Octave  which is an open-source software similar to \c Matlab.<br>

If it is installed, open a command line and go to

\verbatim

~/feel/heatsink/Simplex_'.'.'/0.000'/

\endverbatim

and try :

\verbatim

> octave

octave:1> M=load('averages');

octave:2> plot(M(:,1),M(:,2))

octave:3> plot(M(:,1),M(:,3))

octave:4> plot(M(1:70,1),M(1:70,2))

octave:5> plot(M(1:70,1),M(1:70,3))

\endverbatim

The \f$ 4^{th}\f$ and \f$ 5^{th}\f$ lines are here to observe the transient state.

\subsection Results_How How to use it ?

To make easier the use of this application, we recommend you to use the configurations files. This is the fastest way : to do it, you juste have to create the file \c heatsink.cfg and place it in the same directory that your application's executable. <br>

We have created \f$3\f$ typical \c cfg files such as :

\co

# file heatsink_1.cfg

# spreader and fin in copper

# 2D simulation

hsize=1e-4

kappa_s=386 # W/m/K

c_s=385

rho_s=8940

kappa_f=386 # W/m/K

c_f=385 #J/kg/K

rho_f=8940

L=15e-3

width=5e-4

therm_coeff=1000  #W/(m2K)

heat_flux=1e6

[bdf]

order=2

time-step=0.05

time-final=100

steady=0

[exporter]

format=gmsh

\eco

\co

# file heatsink_3.cfg

# spreader in copper

# fin in aluminium

# 3D simulation

hsize=3e-4

kappa_s=386 # W/m/K

c_s=385

rho_s=8940

kappa_f=386 # W/m/K

c_f=385 #J/kg/K

ho_f=8940

L=15e-3

width=5e-4

deep=4e-2

therm_coeff=1000  #W/(m2K)

heat_flux=1e6

[bdf]

order=2

time-step=0.05

time-final=100

steady=0

[exporter]

format=gmsh

\eco

This file is the only modification you will have to bring to the application, in that way you won't have to compile each time the files (except for \c heatsink.cpp if you want to increase the order and/or the dimension, in that case you'ill have to modify this parameter at then end of the file in the \c main method).

\subsection Results_Figures Figures

\subsubsection Results_2D 2D cases

Here are some results of the 2D simulations considering different configurations files. The figures have been extracted thanks to \c Gmsh and \c Octave :

<center>

<table border=0px>

<tr>

<td>\image html heatsink/heatsink_1.png</td>

</tr>

<tr>

<td><center>\c heatsink_1.cfg : steady state, spreader and fin in copper, \f$Q=1e6\f$ and \f$h=1e3\f$</center></td>

</tr>

</table>

</center>

<center>

<table border=0px>

<tr>

<td>\image html heatsink/heatsink_1_gamma4.png</td>

<td>\image html heatsink/heatsink_1_gamma1.png</td>

</tr>

<tr>

<td><center>\c heatsink_1.cfg : transient state on \f$\Gamma_4\f$</center></td>

<td><center>\c heatsink_1.cfg : transient state on \f$\Gamma_1\f$</center></td>

</tr>

</table>

</center>

\subsubsection Results_3D 3D cases

Here is the result of 3D simulations considering the following configurations :

<center>

<table border=0px>

<tr>

<td>\image html heatsink/heatsink_3.png</td>

</tr>

<tr>

<td><center>\c heatsink_3.cfg : spreader and fin in copper, \f$Q=1e6\f$ and \f$h=1e3\f$</center></td>

</tr>

</table>

</center>

<center>

<table border=0px>

<tr>

<td>\image html heatsink/heatsink_3_gamma4.png</td>

<td>\image html heatsink/heatsink_3_gamma1.png</td>

</tr>

<tr>

<td><center>\c heatsink_3.cfg : transient state on \f$\Gamma_4\f$</center></td>

<td><center>\c heatsink_3.cfg : transient state on \f$\Gamma_1\f$</center></td>

</tr>

</table>

</center>

*/

}