Global Journal of Engineering Science and Researches

ADVANCEMENT IN PLANT MONITORING USING ARDUINO AND INTERNET OF THINGS

Prof. Ashwini R.Patil*1, Prof.N.B.Bhawarkar2 & Nishigandha Unhale3

*1,2Assistant Professor, 3PG Student, Electronics & Telecommunication Department,

Mauli College of Engineering and Technology, Shegaon , India

ABSTRACT

Advancement in farm activities can lead the agricultural domain to transform into intelligent and dynamic sector from being manual resulting in higher production with lesser human supervision. This paper describes an advanced irrigation system which monitors and maintains the desired soil moisture content via automatic watering depending upon the necessity. Microcontroller Arduino Uno platform is used to implement the control unit. The setup uses different sensors that measure the various parameters such as soil moisture, water level, temperature, presence of light. These values enable the system to use appropriate quantity of water which avoids under/over irrigation. The farmers can get the status of the field at small distance using some indicators and remote place also using IoT.Information from the sensors is regularly updated on a webpage using network of IoT through which a farmer can check whether the water sprinklers are ON/OFF at any given time. The information from the sensor is also transmitted to a Thing speak channel to generate graphs for analysis.

Keywords: IoT, Arduino, ESP266, ThingSpeak, Cloud, LM35

- INTRODUCTION

People always forget to water their plants becauseof their busy life routine. They are  usually  come home  late  at night  and  tend  to  skip  watering  their plants. Furthermore, people need to supply water to  their  plants  every  day  to prevent  it  from  dying due  to lack  of  nutrients  in  the  soil. Usually, people will use water pipes  or  rainwater  to  water  their  plants.

Moreover it happens in nurseries and gardens that when people get some important  work while watering plants so  they leave the water  pump ONfor a long time and forgetto makeit OFF so the water is  provided  in  huge  amount whichis not necessary and thus  the  water  get  wasted.  Hence to overcome  such  a  problem this system  has  been designed while will automatically sense  the  parameters  of  plants  and  water  it  accordingly.

Internet of Things is nowadays becoming a reality.Internet of Things enables the network devices to sense and collect data from the world around us, and then share that data across the Internet where it can be processed and utilized for various purposes. This implemented system consists of Sensing unit, Controller unit and the Output unit. In sensing unit different sensors are used for sensing the conditions of the field that are necessary for the monitoring. Controller section consists of Arduino Uno as a controlling element. It will sense the parameters from the sensing unit and will compare it with the threshold values set for the particular parameter. Then according to the conditions present, it will generate the output in the form of the indications. Various indicators are used for getting the status of the field in nearby location and using IoT the status can be send over the internet.

Internet of things

The Internet of things (IoT) is the network of physical devices, vehicles, home appliances, and other items embedded with electronics, software, sensors, actuators, and connectivity which enables these things to connect, collect and exchange data.

Fig.(1) Internet of Things

IoT involves extending Internet connectivity beyond standard devices, such as desktops, laptops, smartphones and tablets, to any range of traditionally dumb or non-internet-enabled physical devices and everyday objects. Embedded with technology, these devices can communicate and interact over the Internet, and they can be remotely monitored and controlled.

- METHODOLOGY

The system is divided in two sections as Transmitter section and Receiver sections.

The transmitter sectionincludes sensing module, controller module and output module. Sensing module includes the soil moisture sensor and temperature sensor that will sink in the soil to measure the humidity level and temperature level. The controller module is an important part.In this system, arduino UNO board is used as the controller module to manage the system.The purpose of regulated powersupply is to supply the required power for the arduino UNO board to functions well.Besides that, output module is the module where to produce an output from the microcontroller.

Fig (a).Block Diagram for Transmitter SectionFig (b).Block Diagram for Receiver Section

In this system, the output is taken through different ways. Such as

- Output is given to the motor pump through driver circuit. The motor pump is activated to provide the necessary water to the plantsbased  on  the  plant’s condition.   .

- The status is indicated on the LCD display and LED indicators so as to get the status at small distance.

- Also depending upon the status of the plants the message is send over long distance at remote place using ESP8266 prototype i.e. IoT.

Receiver Section

As shown in Figure (b), the block diagramfor receiver section includessensing module, controller   module   and output module. Sensing module includes the RF receiver module to receive themessages thatare transmitted from transmitter module toknow the status to client Depending uponthe message received the differentindicators will be activated.

The  LED  indicator  is  used  to  indicate  the  users  the  soil’s  condition either  dry,  moderate  or  wet  condition.  Different color of LED’s are used todifferentiate the plant’s condition.  The  buzzer  also  act  as  alert  system  indicating  that  the  soil  is  in dry condition.

- COMPONENTS DESCRIPTION

A. Arduino Uno

Fig(c).Arduino Uno

The  Arduino  UNO is a control  device  which  incorporates  a  microprocessor  and prototype  board  that  can  be  program  by using Arduino IDEsoftware. It  can  be interfaced  to thereal  world  and  virtual  world  by  connecting  the Arduino UNO board to  the  Internet,  either  sensing  data  to  the  internet  or responding to data on the  internet  or  both.Arduino  uses  ATMEGA 328  microcontroller  that  consist  of  14  input  and  output  for  analog  and  digital pins.It is used for monitoring sensors including soil moisture, temperature, sound, light.

B. Soil Moisture Sensor

Fig(d).Soil Moisture Sensor

The  soil  moisture  sensor  was  used  to  measure  soil  humidity  level  by  immersed the  probes  into  the  soil.  When  the  soil  was  dried,  the  soil  moisture  sensor output is  at  high  level,  else  the  output  is  at  low  level.  Water is one of good conductor. So,  the  conductivity is  increased  if  it sense  more  water  content from  the  soil  moisture  sensor.  Thus, the resistance will be decrease.  The conductivity  of  water  is  decreased  if  it  is  sense  less  water  content.  Then, the resistance will be increase.  The  soil  moisture  sensor  has  two  probes  and  need  to  sink  into  the  soil.  The  soil  moisture  sensor  gives  an  analog  input  values ranging  from  0V  to  5V  for  the  difference  soil  humidity  levels.

C. Temperature Sensor

Fig(e).Temperature Sensor LM35

The LM35   is an integrated circuit sensor that can be used to measure temperature with an electrical output proportional to the temperature (in 0C). If the temperature is high then the fan will on and vice versa.

D. RF Module

The RF module, as the name suggests, operates at Radio Frequency. The corresponding frequency range varies between 30 kHz & 300 GHz. In this RF system, the digital data is represented as variations in the amplitude of carrier wave. This kind of modulation is known as Amplitude Shift Keying (ASK).

Fig (f): RF Transmitter and Receiver module

Transmission through RF is better than IR (infrared) because of many reasons. Firstly, signals through RF can travel through larger distances making it suitable for long range applications. Also, while IR mostly operates in line-of-sight mode, RF signals can travel even when there is an obstruction between transmitter & receiver. Next, RF transmission is more strong and reliable than IR transmission. RF communication uses a specific frequency unlike IR signals which are affected by other IR emitting sources.

This RF module comprises of an RF Transmitter and an RF Receiver. The transmitter/receiver (Tx/Rx) pair operates at a frequency of 434 MHz..An RF transmitter receives serial data and transmits it wirelessly through RF through its antenna connected at pin4. The transmission occurs at the rate of 1Kbps - 10Kbps.The transmitted data is received by an RF receiver operating at the same frequency as that of the transmitter.

E. ThingSpeak

ThingSpeak is an open source Internet of Things (IoT) application and API to store and retrieve data from things using the HTTP protocol over the Internet or via a Local Area Network. ThingSpeak enables the creation of sensor logging applications, location tracking applications, and a social network of things with status updates.

F. ESP8266

The ESP8266 is a low-cost Wi-Fi microchip with full TCP/IP stack and microcontroller capability Systems.

The ESP8285 is an ESP8266 with 1 MB of built-in flash, allowing for single-chip devices capable of connecting to Wi-Fi.The successor to these microcontroller chips is the ESP32.This small module allows microcontrollers to connect to a Wi-Fi network and make simple TCP/IP connections using Hayes-style commands. The very low price and the fact that there were very few external components on the module, which suggested that it could eventually be very inexpensive in volume makes it very reliable.

- FLOWCHART

The overall module flowchartshows the overall working of the advanced plant monitoringsystem.  Based  on  the  humidity section,  if  the  soil  moisture   sensor sense  dry  condition of  the soil then the  humidity level is less  than  the dry threshold  values. Thus, Red LED indicator is ignitedfor self-watering  mode to give  an  alert  to the  users for watering  purpose. After red LED indicator is blinked about 1 minutes, the system is triggered to the auto watering  mode  and  water  pump  will  turn  ON for watering process about 1 minutes.

When  the  soil  moisture  sensor sense  moderate humidity condition when the humidity level  is  between  moderate  threshold value  and  dry threshold  value. Yellow LED indicator is  blink in reduction  mode  to  indicate  the  users  that  their  plant  in good  condition.

Fig (g): Overall Module Flow for Transmitter Section

On  the  other  hand,  when  the  humidity  level  is  greater  than  the  wet  threshold value  means  that  the  plants  in  wet  condition  and  green  LED  indicator  is  blink in  reduction  mode  and  the  lighting  system  is  triggered  to  supply  some  lighting to  the  plants.

- RESULTS AND DISCUSSION

The plotted graph in Figure (h) shows the average of humidity level in the plant for ten days. The reading of soil humidity level was taken every six hours per days.The plant was watered by constant volume for every six hours per days. Based on the graph, the reading of soil humidity level was not constant due to weather conditions was different for every day.

Fig(h).Graph showing humidity in soil

User Indicators

As mentioned before, this system comes with three  modes  that  are  self-watering mode,  auto  watering  mode  and  lighting  mode. Hence  a  user  indicator  for  the self-watering  mode  is  needed  to  enable  the  user  to    their  plants. Table 1shows the descriptionabout the  display  module.

Table 1: Description of display module

| Parameter | Description |
| --- | --- |
| White LED | For indicating partial wet mode of plant |
| Red LED | For indicating dry condition |
| Green LED | For indicating wet condition |
| Reset button | Reset system |

The  self-watering  mode  is triggered  when  the  buzzer  is  activated  and  red  LED  indicator is  blinked  about one  minute  to  give  an  alert  o  the  user  that  their  plant  in  the dry  condition  and  needed  to  water  it  before  auto  watering  mode  was  triggered. This  shows  that  the  system  is  very  user  friendly.  Auto  watering  mode  and lighting  mode  are  activated  according  to  the  program  set  up  in  the  system. Basically,  auto  watering  mode  is  triggered  about  one  minute  when  the  system  or  sensor  senses  the plants  in  dry  condition.  This  happened  when  it  is  does  not have  any  feedback  from  the  user  in  self-watering  mode.  Lighting  mode  is triggered  about  1  minutes  when  the  system  or  sensor  sense  wet  condition  on  the plants.  Figure below shows  user indicator  used i.e LED  and  buzzer.

Fig (i): User indicator for dry soilFig (j): User indicatorpartial wet soilFig (k):User indicator for wet soil

Figure (l)illustrates the readings to monitorthe  humidity  and  temperature  level.  Digital  thermometer  is  used  for  testing  only  but  for  the actual   project uses  temperature  sensor  that  vary  precise  in  term  of  their  values.  The  method issame  as  the  soil  moisture  sensor  by s inks  it  into  the  plants  and  monitor  it through  LCD  display.  The  test  is  to  know  the  relationship  between  the humidity  level  and  temperature  level.

Three conditions  were  investigated  for  the  humidity level  and  temperature  level.  These  are  dry  mode,  partial  wet  mode  and  fully wet  mode  as  shown  in fig(l). Dry  mode  is  mode  where  the soil  is  fully  dried  and  plant  can’t  stand  more  if  this condition  continued.  Partial  wet  mode  is  mode  where  soil  was  watered  with some  amount  of  water.  So  that,  the  plant  becomes  partial  wet.  Besides  that, wet  mode  is  mode  where  soil  was  watered  with  huge  amount  ofwater.  So that,  the  plant  became  very  wet.  Wet  mode  was  observed  which  is  responsible for  wastage  of  water  hence  we  can  save  water  before  wasting  it,  by  controlling  watering  automatically.  The  experimental  phase  was  conducted  about few  days  to  see  the  relationship  between  the  temperature  level  and  humidity level  during  the  dry  mode,  partial  wet  mode  and  wet  mode.

Fig (l).Dry,partial wet  and fully wet soil readings

- CONCLUSION

Nowadays  people have  an  interest  for  doing  planting  in  residential  area  but  because many  people  lives  at  storeyed building or apartment they sometimes  neglects  to water  their  plants  which  causes  harm  to  the  plants.  Also due to the long working hours  in  the  office and busy schedule  with their works, they  tend  to  forget  to water their  own  plant. Thus, this system  plays a vital  role  for  the  user  to monitor  their  plant’s  condition  and  for watering  purpose. Also it is the fact that manyplants  can  withstand  for some  days  under direct sunlight  but  it  cannot sustain under lack of water. The  characteristics  of  plant only  growth  at wet environment means  that  plant  really  consume  a  lot  of  water but not  after  fully  wet  condition. The  system  can  performed  automatic   watering the  plants  based  on  the  humidity  level  and  temperature  level detection. This technique  also  reduces  the  water  wastage  by giving only the needed  amount  of water  hence  water  conservation  is  achieve.

REFERENCES

- HemantKuruva, BalumuriSravani,“Remote Plant Watering and Monitoring System Based on IoT ”, International Journal For Technological Research In Engineering  Volume 4, Issue 4, December-2016,PP.668-671

- Srinidhi Siddagangaiah, “A Novel Approach to IoT Based Plant Health Monitoring System”, International Research Journal of Engineering and Technology (IRJET), Volume: 03 Issue: 11 |Nov -2016,PP.880-886.

- K. Lakshmisudha, Swathi Hegde, Neha Kale , Shruti Iyer  “Smart Precision based Agriculture using   Sensors” International Journal of Computer Applications (0975 – 8887) Volume 146 – No.11, July 2016,PP.36-38

- Bhagyashree K.Chate , Prof.J.G.Rana, “Smart Irrigation System Using Raspberry Pi”, International Research Journal of Engineering and Technology (IRJET) , Volume: 03 Issue: 05  May-2016,PP.247-249.

- Er. Vineet Biswal, Er. Hari M. Singh, Dr. W. Jeberson, Er. Anchit S. Dhar, “A Smart Houseplant Watering and Monitoring System”, International Journal of Science, Engineering and Technology Research (IJSETR), Volume 4, Issue 7, July 2015,PP.2499-2507

- Er. Anshoo Sarswat, Hitendra Pratap Singh, Shaynak Saxena, Yash Chaug,  Sanu Mandal, “Remote Irrigation and Monitoring System Using GSM Technology”, International Journal of Electrical and Electronics Research, ISSN 2348-6988 (online), Vol. 3, Issue 2, pp: (277-280), Month: April - June 2015, Available at: www.researchpublish.com Page 277

- Prof. Pranit P. Kathale,JyotiMankari, Payal Shire, “A Review on Monitoring and Controlling System for the Operation of Greenhouse Environment”, International Journal of Advanced Research in Computer Science and Software Engineering Research Volume 5, Issue 4, 2015 ISSN: 2277 12

- ChangboJi ,HongyueLu, ChangqingJi,Jingguo Yan, “An IoT and Mobile Cloud based Architecture for Smart Planting”, 3rd International Conference on Machinery, Materials and Information Technology Applications (ICMMITA 2015),PP.1001-1005.

- Nileshkuchekar and Prof. Pagare R.A. “ Microcontroller Based Advanced Water Deployment System for Irrigation using a WSN & GSM module”, International Journal of Emerging Trends in Engineering and Development Issue 5, Vol. 4 (June.-July. 2015) ISSN 2249-6149 ,Page 513.

- Pavithra D. S , M. S .Srinath, “GSM based Automatic Irrigation Control System for Efficient Use of Resources and Crop Planning by Using an Android Mobile”, IOSR Journal of Mechanical and Civil Engineering (IOSR-JMCE) Volume 11, Issue 4 Ver. I (Jul- Aug. 2014), PP 49-55.

- Akash Jain , Suraj Kudre , Mahesh Giri “A Rewiev on Smart Sensors Based Monitoring System for Agriculture”, International Journal of Engineering & Science Research, ISSN 2277-2685, p-ISSN 2320-976 IJESR/May 2014/ Vol-4/Issue-5/352

- Jia Uddin, S.M. Taslim Reza, Qader Newaz, Jamal Uddin, Touhidul Islam, and Jong-Myon Kim,“Automated Irrigation System Using Solar Power”,  7th International conference on Electrical and computer engineering, 20-22 Dec 2012 ,IEEE

- Abhishek Barve Pragnesh shah “Android based Remote Monitoring System”, International Conference in Recent Trends in Information Technology and Computer Science (ICRTITCS - 2012)

- Pooja Kharat,, Ameya Salvi, Omkar Sawant, Nilesh Rathod, “ Research per on Vegetation Health Monitoring Using Agricultural IOT” International Journal on Recent and Innovation Trends in Computing and Communication  Volume: 4 Issue: 11,PP.200-202.

- Dursun M, Ozden S. “A Wireless Application of Drip Irrigation Automation Supported by Soil Moisture Sensors”, Scientific Research and Essays, April 2011,1573–1582.

- Shruti A Jaishetty, Rekha Patil, “ IOT Sensor Network based Approach for Agricultural Field Monitoring and Control”, IJRET: International Journal of Research in Engineering and Technology,PP.45-48.

- V. Kacholia, S. Pandit, S. Chakrabarti, S. Sudarshan, R. Desai, and H. Karambelkar, “Bidirectional Expansion For Keyword Search on Graph Databases,” in Proceedings of the 31st International Conference on Very Large Data Bases, ser. VLDB ’05, August 2005, pp. 505.