Automated Solutions for Sustainable and Circular Construction and Demolition Waste Management

ENVISAN-GEM a.s.

D4.3 Methodology for improved logistics concepts for collection and automated management of off-site CDW treatment

Contract No: 101058580 
January, 2025

Document Control Sheet

| Project | Automated solutions for sustainable and circular construction and demolition waste management |
| --- | --- |
| Call identifier | HORIZON-CL4-2021-TWIN-TRANSITION-01 |
| Grant Agreement N° | 101058580 |
| Coordinator | CESKE VYSOKE UCENI TECHNICKE V PRAZE ¦Czech Technical University in Prague |
| Work package | WP 4. Automated off-site CDW management and treatment |
| Work package leader | CVUT |
| Related tasks | Task 4.3 |
| Deliverable title | Methodology for improved logistics concepts regarding collection and automated management of off-site CDW treatment |
| Deliverable nature | Report |
| Dissemination level | PU |
| Lead Beneficiary | ENVISAN-GEM, a.s. |
| Contributing partners | RECSO, ANAKEM, STRABAG |
| Authors | Václav Filištein, ENVISAN-GEM, a.s. |
| Reviewer(s) | Jan Valentin, Czech Technical University in Prague Vaclav Nezerka, Czech Technical University in Prague Jan Trejbal, Czech Technical University in Prague RECSO ANAKEM STRABAG |
| Version | 1.1 |
| Total number of pages | 54 |
| Due date | 31.01.2025 |
| Submission date | 31.01.2025 |

All rights reserved: The document is proprietary of the consortium members. No copying or distributing, in any form or by any means, is allowed without the prior written agreement of the owner of the property rights. This document reflects only the authors’ view. The European Community is not liable for any use that may be made of the information contained herein.

Table of Contents

Document Control Sheet	2

Table of Contents	3

List of Tables	4

List of Figures	4

List of Acronyms	5

Executive Summary	6

1	Introduction	7

1.1	Purpose of the document	7

1.2	Structure of the document	8

2	Legislative obligation for on-site pre-sorting	9

3	Logistics concepts for waste collection and treatment	14

3.1	CDW collection centres and waste redistribution network	15

3.2	On-site collection	15

3.2.1	Containers	16

3.2.2	Development of a smart device for containers	18

3.3	Off-site treatment	21

3.3.1	Waste reception and inspection	21

3.3.2	Pre-treatment	22

3.3.3	Pre-screening	22

3.3.4	Crushing, shredding and size reduction	23

3.3.5	Screening	24

3.3.6	Sorting	25

3.3.7	Output quality control	27

3.4	Primary CDW Stream from Buildings	28

3.4.1	Concrete	29

3.4.2	Bricks	31

3.4.1	Wood	33

3.4.1	Gypsum	35

3.4.1	Glass	37

3.4.1	Mineral insulation	38

3.4.1	Expanded polystyrene (EPS)	40

3.4.2	Ceramic and tiles	41

3.4.3	Other	42

3.5	Mixed CDW	43

3.5.1	Concept of a recycling plant for mixed CDW	44

4	Conclusions	48

References	49

List of Tables

Table 1 Legal requirements for on-site pre-sorting	9

Table 2 Theoretical composition of CDW from building based on MFA modelling	28

List of Figures

Figure 1 Skip (Laudon GmbH & Co. KG 2024a)	17

Figure 2 roller container (Laudon GmbH & Co. KG 2024b)	18

Figure 3 Smart device for monitoring CDW containers	20

Figure 4 Container Dashboard: Top-left: current location. Top-Right: real-time content data. Graphs: fill level, temperature (top row); humidity, battery level (bottom row)	21

Figure 5 Schematic image of a) impact crusher b) jaw crusher c) cone crusher	24

Figure 6 Concrete waste processing flow-chart	32

Figure 7  Bricks waste processing flow-chart	34

Figure 8 Wood waste processing flow-chart	36

Figure 9 Plasterboard waste processing flow-chart	37

Figure 10 Glass waste processing flow-chart	39

Figure 11 Mineral insulation waste processing flow-chart	41

Figure 12 EPS waste processing flow-chart	42

Figure 14 Concept of a recycling plant for mixed CDW	48

List of Acronyms

| Abbreviation / acronym | Description |
| --- | --- |
| AAC | Autoclaved aerated concrete |
| AI | Artificial Intelligence |
| CDW | Construction and demolition waste |
| EC | European Commission |
| EoW | End of Waste |
| EPS | Expanded polystyrene |
| ETICS | External thermal insulation composite systems |
| EU | European Union |
| HBCD | Hexabromocyclododecane |
| HDF | High density fiberboard |
| IoT | Internet of things |
| LDPE | Low-Density polyethylene |
| MDF | Medium density fiberboard |
| MFA | Material flow analysis |
| MS | Member state |
| OSB | Oriented strand board |
| PAHs | Polycyclic aromatic hydrocarbons |
| POP | Persistent organic pollutant |
| pp | Polypropylene |
| PVC | Polyvinyl chloride |
| RA | Recycled aggregates |
| RCA | Recycled concrete aggregates |
| WP | Work Package |

Executive Summary

This document address significant challenges posed by construction and demolition waste (CDW) in Europe, with a focus on current legislative obligations, practical realities of on-site sorting, and opportunities for advanced off-site treatment. It offers insights into how CDW is collected and processed, highlighting both shortcomings in current practices and the potential of new technologies to enhance efficiency and recycling rates. Readers will learn about barriers — such as contamination, insufficient on-site sorting, and economic disincentives — and discover how improvements in regulation, digital monitoring, and robotic sorting can support the transition to a circular economy.

The report underscores that advanced off-site sorting solutions are necessary when on-site separation is not effectively carried out. It examines the role of Internet of Things (IoT) devices, such as GPS tracking, fill-level sensors, and digital tagging, in optimizing container use and improving manual oversight, as well as the value of deep learning and robotic systems in reducing contamination and generating higher-quality secondary materials. A conceptual design for a modern recycling plant demonstrates how multi-stage sorting, including heavy-duty robots and sensor-based ejectors, can yield high-value recycled aggregates, wood, metals, and plastics.

Key takeaways include the importance of legislation that reflects technological progress and encourages stakeholders to invest in cutting-edge sorting infrastructure. Clear, harmonized end-of-waste criteria and fair thresholds for contaminants will further enable consistent, safe recycling of CDW, while also boosting market confidence in secondary raw materials. Ultimately, the combination on-site sorting requirements, transparent documentation, clear legislation and advanced off-site technologies can help the construction industry move from a largely linear model toward a more sustainable, circular approach.

# Introduction

## Purpose of the document

CDW represents one of the most significant waste streams within the European Union. According to data from 2022, the EU's total waste production, encompassing all economic sectors and households, reached 2 233 million tons. Of this total, the construction industry was responsible for approximately 38% (Eurostat 2022). The construction and building sector has a significant impact on the economy, local employment, and quality of life. It accounts for approximately 50% of all extracted materials. Greenhouse gas emissions from material extraction, production of construction products, building activities, and renovation are estimated to contribute 5–12% of national emissions, with 80% of these emissions potentially avoidable through improved material efficiency. To address these challenges, the European Commission has introduced a comprehensive strategy to promote circular economy principles throughout the lifecycle of buildings, focusing on sustainability, resource efficiency, and waste management (European Commission 2020). The European Parliament and the Council, in accordance with the Directive 2008/98/EC, set a target for the recovery and reuse of CDW to reach 70% by 2020 (European Parliament and of the Council 2008).

By 2020, the EU achieved an 89% recovery rate for CDW, surpassing other waste streams. However, most recovered materials are repurposed for low-value applications, such as road fillers or backfilling, rather than high-value reuse or recycled materials. These materials often lack the technical quality needed to replicate their original functions, leading to reduced market value and missed opportunities for energy and resource savings. While the recovery target is widely met, enhancing material reuse or recycling in line with circular economy goals remains a major challenge (Caro et al. 2024).

This report focuses on the collection process of CDW and its treatment at off-site collection centres. A collection centre is a facility that receives waste from generators, sorts and stores the material, and processes it before transferring it to specialized facilities for specific material treatment or directly to end users.

## Structure of the document

Chapter 2

provides an overview of the legal frameworks in various EU Member States regarding mandatory on-site pre-sorting of CDW. It identifies common elements of these regulations, highlights significant differences among countries, and potential exemptions.

Chapter 3

This chapter examines the operational aspects of CDW management in a structured manner. It begins by describing how waste collection centres and redistribution networks function, discussing both on-site and off-site practices. The text then outlines various on-site waste collection methods, including the use of container systems, and highlights emerging IoT solutions that can improve logistics and efficiency.

Subsequent sections focus on off-site treatment processes, including pre-treatment, screening, crushing, and sorting. These procedures range from basic separation methods to advanced solutions driven by artificial intelligence and robotic technologies. Additionally, the chapter evaluates different management pathways for key waste fractions (for example, concrete, bricks, gypsum, and mineral insulation), emphasizing both recovery and high-value recycling. Finally, a conceptual design for a recycling plant dedicated to mixed CDW is presented, illustrating how sophisticated sorting strategies can yield higher-quality secondary materials for use in the construction industry.

# Legislative obligation for on-site pre-sorting

A review of national legislation has identified legal requirements for the on-site pre-sorting of CDW. The specific regulations for each country are detailed in Table 1.

Table 1: Legal requirements for on-site pre-sorting

| Country | Obligation on source separation | Non-hazardous waste categories | Possibility for off-site separation | Further information |
| --- | --- | --- | --- | --- |
| - Austria | YES, Exception: if the combined treatment of specified waste is permitted for producing specific recycled construction materials. | Excavated material; mineral waste; recycled asphalt; wood waste; metal waste; plastic waste; municipal waste. | YES | BGBl. II Nr. 181/2015, §6 |
| - Belgium | Flanders YES, Wallonia YES, Brussels-Capital Region YES | Flanders 29 categories, for CDW (glass; inert rubble; wood; metals; recyclable hard plastics; polystyrene; asphalt rubble; foundation materials; aerated concrete; plasterboard and gypsum) Wallonia 15 categories, for CDW (metals; wood) Brussels-Capital Region 16 categories, for CDW (metals; wood; hard plastics; polystyrene; rubble) | Flanders YES Wallonia YES Brussels-Capital Region YES | Flanders - Vlarema 9 Wallonia - Décret relatif aux déchets, à la circularité des matières et à la propreté publique, AGW 2015-03-05 Brussels-Capital Region - 1 DECEMBRE 2016. - Arrêté du Gouvernement de la Région de Bruxelles-Capitale relatif à la gestion des déchets |
| - Bulgaria | GenerallyYES | Sorting according to the standard waste classification, including code 170904 mixed CDW | YES | НАРЕДБА ЗА УПРАВЛЕНИЕ НА СТРОИТЕЛНИТЕ ОТПАДЪЦИ И ЗА ВЛАГАНЕ НА РЕЦИКЛИРАНИ СТРОИТЕЛНИ МАТЕРИАЛИ Приета с ПМС № 267 от 05.12.2017 г. Чл. 4. |
| - Croatia | Not found in the legislation | - | - | - |
| - Cyprus | YES above 12 m3 generally, above 300 m3 defined | Wood; inorganic materials (concrete, bricks, tiles, ceramics, stone); metals; glass; plastics; gypsum | N/A | Κ.Δ.Π. 112/2023 |
| - Czech Republic | YES | Stationary machines; electrical appliances; flooring structures; sanitary fixtures; various types of glass (e.g., flat, insulating, construction glass); wood and wood products not contaminated with hazardous substances; doors and windows; plants; concrete; bricks; roof tiles; ceramics; non-hazardous asphalt mixtures; soil and stones free of hazardous substances; uncontaminated gypsum-based materials; metallic building components; plastic products (PVC and others); EPS; mineral wool; mixtures of concrete; bricks; tiles and ceramics without hazardous substances. | NO | § 42 vyhlášky č. 273/2021 Sb. o podrobnostech nakládání s odpady |
| - Denmark | YES above 1 ton | Double-glazed windows; natural stone (e.g., granite and flint); unglazed brick (wall and roof tiles); concrete; mixtures of natural stone; unglazed brick; concrete; iron and metal; gypsum; rock wool; soil; asphalt mixtures; mixtures of concrete and asphalt. | YES | BEK nr 573 af 23/05/2024 § 63 |
| - Estonia | Generally YES to highest possible extend, but specific conditions are regulated by municipalities. | e.g. Tallinn Wood (treated and untreated separately); uncoated paper and cardboard; metals (ferrous and non-ferrous metals separately); mineral waste (stones, construction stones, plaster, etc.); reinforced concrete and concrete components; gypsum; glass (including flat glass); bricks; asphalt without tar; plastic (film, polystyrene, plastic windows, and other construction plastics); ceramics and tiles; reinforced concrete and concrete parts; tar-free asphalt mixtures; plastic (film, styrofoam, plastic windows and other construction plastic). | - | Jäätmeseadus RT I 2004, 9, 52, §29 Tallinn - https://www.tallinn.ee/et/keskkond/ehitusjaatmed |
| - Finland | YES | Concrete; bricks; mineral tiles and ceramics; asphalt mix; bitumen and roofing felt; gypsum; unimpregnated wood; metal; glass; plastic; paper and cardboard; mineral wool insulation; soil and stones. | YES | Government Decree on Waste (978/2021) Section 26 |
| - France | YES, and an ERP system is also in place | Paper; metal; plastic; glass; textiles; wood; mineral fractions; plaster. | YES | Code de l'environnement Article D543-281 |
| - Germany | YES above 10 m³ | Glass; plastic; metals; wood; insulation material; bituminous mixtures; gypsum-based construction materials; concrete; bricks; tiles and ceramics. | YES | Gewerbeabfallverordnung |
| - Greece | YES | Wood; inorganic fractions (concrete, bricks, tiles and ceramics, stone); metals; glass; plastics; gypsum | YES | Article 30 of Law 4819/2021 |
| - Hungary | YES, thresholds defined in the regulation | Excavated soil; concrete; asphalt; wood waste; metal waste; plastic waste; mixed CDW; mineral-based construction materials. | YES | 45/2004. (VII. 26.) BM–KvVM együttes rendelet az építési és bontási hulladék kezelésének részletes szabályairól §3. |
| - Ireland | Not found in the legislation – only Guidelines | - | - | BEST PRACTICE GUIDELINES for the preparation of resource & waste management plans for construction & demolition projects |
| - Italy | Not found in the legislation | - | - | - |
| - Latvia | YES | Wood; mineral fractions (concrete, bricks, tiles and ceramics, stones); metal; glass; plastic; plaster. | N/A | Atkritumu apsaimniekošanas likums. 20.pants (7) https://likumi.lv/ta/id/221378 |
| - Lithuania | YES | Municipal waste; inert waste (concrete, bricks, tiles and ceramics, stones); wood waste; gypsum waste; recyclable and reusable materials (packaging, paper, glass, metal, plastic, polystyrene foam); hazardous waste (solvents, paints, adhesives, resins, etc.); non-recyclable waste (insulation materials, mineral wool); foam panels containing ozone-depleting substances or fluorinated greenhouse gases. | NO | ĮSAKYMAS DĖL STATYBINIŲ ATLIEKŲ TVARKYMO TAISYKLIŲ PATVIRTINIMO D1-637 (7) |
| - Luxembourg | YES | Wood; mineral fractions (concrete, bricks, tiles, ceramics, stones); metal; glass; cardboard; plastics; gypsum. | YES | Loi du 21 mars 2012 relative aux déchets. Art. 26 |
| - Malta | YES | Mineral fractions; metal; glass; wood; plastic waste. | YES | S.L. 549.161 Construction and Demolition Waste Framework Regulations |
| - Netherlands | YES, NO if the quantity of waste for a particular fraction is less than 1 m³ | Bituminous roofing materials (with or without roof decking); asphalt without tar; flat glass (with or without frame); gypsum blocks and plasterboard materials; roof gravel; fixtures. | YES | Besluit bouwwerken leefomgeving Artikel 7.26. |
| - Poland | YES from 1.1.2025 | Wood; metals; glass; plastics; gypsum; mineral waste (including concrete, bricks, tiles, ceramic materials, and stones). | YES | Ustawa z dnia 14 grudnia 2012 r. o odpadach, Art. 101a |
| - Portugal | YES | Wood; mineral fractions (including concrete, bricks, tiles, ceramics, stone); metal; glass; plastic; plaster. | YES | Decreto-Lei n.º 102-D/2020 Artigo 51.º |
| - Romania | YES if economically feasible | Wood; mineral materials (concrete, brick, tiles, ceramics, stone); metal; glass; plastic; gypsum. | N/A | Ordonanță de urgență Nr. 92 din 19 august 2021 privind regimul deșeurilor 17(4) |
| - Slovakia | YES | Flooring structures; sanitary equipment; glass (including flat glass, insulating glass, and construction glass); wood (uncontaminated by hazardous substances); doors and windows; concrete structures; bricks; aerated concrete; roofing tiles; ceramic and other tiling elements; sanitary ceramics; uncontaminated asphalt mixtures; uncontaminated soil and aggregates; uncontaminated railway ballast; uncontaminated drywall; metal construction components; PVC products (window profiles, flooring, waterproofing films, roofing materials); other plastic materials; uncontaminated polystyrene foam; uncontaminated mineral wool. | NO | § 1 vyhlášky č. 344/2022 Z. z. o stavebných odpadoch a odpadoch z demolácií |
| - Slovenia | YES | Wood; mineral fractions (concrete, bricks, tiles and ceramics, stones); metals; glass; plastics; gypsum. | NO |  |
| - Spain | YES | Wood; mineral fractions (concrete, bricks, tiles, ceramics, and stone); metals; glass; plastic; gypsum; reusable elements (e.g. roof tiles, sanitary elements, structural components). | YES | Ley 7/2022, de 8 de abril, de residuos y suelos contaminados para una economía circular. Artículo 30 |
| - Sweden | YES | Wood; minerals (concrete, bricks, tiles, ceramics, stones); metal; glass; plastic; gypsum. | NO | Avfallsförordning (2020:614) 10 § |

The requirements for on-site sorting of non-hazardous CDW in EU member states (MS) have been identified based on national legislation. While most MS mandate on-site sorting, their approaches vary significantly. Some countries allow exemptions based on the weight or volume of CDW generated (e.g., Denmark, Germany) or, as in Austria, based on approved waste processing methods for producing specific recycled construction materials. In most cases, CDW separation is regulated at the national level, though some countries, such as Belgium and Estonia, have regional differences in obligations.

The range of materials required for sorting also differs. Some countries, like the Czech Republic and Slovakia, provide detailed lists including sanitary equipment, polystyrene, and PVC. Others, such as Lithuania, focus on broader categories like concrete, wood, metals, and plastics.

Most countries permit off-site sorting, but this is often subject to conditions such as limited space at the construction or demolition site.

Overall, legislative frameworks aim to encourage on-site waste sorting while allowing flexibility. Off-site separation remains necessary in many cases despite the benefits of on-site sorting.

In the Czech Republic, where the ENVISAN-GEM recycling centre operates, monitoring of incoming waste shipments exposes a significant gap between regulatory requirements and actual practices. Despite strict legal obligations, CDW is frequently not sorted as mandated, and different waste categories are often mixed.

# Logistics concepts for waste collection and treatment

The basic logistics chain for CDW consists of the following key nodes: CDW generation, CDW collection, CDW treatment, and the final stage, which is either reuse, recycling, other material recovery, energy recovery, backfilling for some waste streams or disposal.

At the generation point (construction, renovation, or demolition), waste is generated and segregated. Collection centres are locations where waste from multiple generation points is gathered. At these places, activities such as sorting and modifications to improve transportability and handling, including crushing and compacting, take place.

From collection centres, the waste is either processed internally — common for mineral fractions of CDW — or transferred to specialized treatment facilities for further sorting and processing. From there, the waste or materials are passed through the chain to their final use or disposal.

To encompass the entire flow of materials, including those that do not formally enter the waste regime, the basic logistics chain must be expanded to include recovery operations at the generation point. These operations may involve backfilling, material reuse, or direct reuse of materials in subsequent construction projects. Additionally, it is essential to address illegal dumping, which can disrupt the proper flow of materials and pose environmental risks.

The redistribution network for CDW enables the efficient transfer of materials from their generation points to their final use and should ideally operate in accordance with the principles of the circular economy, keeping materials in a closed-loop system.

CDW is highly variable, and its components require different recycling and recovery methods, resulting in diverse redistribution opportunities. Several factors influence these processes, including legal requirements for waste management and the demolition or construction methods used at the site. Collection methods also play a role, as they affect the quality and purity of waste streams. Local conditions, such as project size and the availability of space for handling and storing waste, further shape the logistics of waste redistribution. Economic factors, including the prices of secondary materials, virgin materials, and energy, determine the feasibility of different treatment options. Landfill taxes provide financial incentives or disincentives that affect disposal rates. Additionally, the level of development of treatment infrastructure varies between countries.

## CDW collection centres and waste redistribution network

The pathway of the main CDW components is described in the following chapters. Although each component follows a different pathway, there are common principles that contribute to more effective management.

Documentation: The entire waste management process must be documented to ensure proper handling and minimize environmental risks. Accurate identification of waste at the beginning of its cycle ensures that only appropriate materials enter the management process and prevents contamination.

Separation of hazardous and non-hazardous waste: Proper documentation helps keep hazardous waste separate from non-hazardous waste. If hazardous waste is not separated at the source, it can contaminate the entire process.

Separation close to the generation: The closer waste is separated to its generation point, the more efficient the subsequent processes become and the lower is the problem of materials contaminating each other and reducing the level of material stream purity.

Waste/material acceptance criteria: Each stage of the chain should have clear waste acceptance criteria or secondary material criteria to ensure consistency in handling and processing.

Sorting over processing: Prioritizing sorting over processing improves efficiency. Each stage before final processing should focus on concentrating materials into single waste streams rather than processing mixed waste.

Proper storage: Materials must be stored correctly to prevent degradation and maintain their quality throughout the waste management chain.

## On-site collection

In the waste management at construction sites, selecting appropriate waste collecting and handling methods is important for ensuring efficient and safe treatment. The choice of method depends on factors such as available space, waste type, and project requirements.

During construction, renovation, or demolition projects — whether through conventional demolition, selective demolition, or deconstruction — waste collection methods can be classified into three main categories:

stockpiling (bulk storage at the construction site);

use of handling equipment such as containers and pallets;

direct treatment without interim storage (e.g., asphalt milling and mixing with new asphalt for reuse).

On-site stockpiling is used in locations where there is sufficient space and immediate waste removal is not required. This method allows for the temporary accumulation of large waste volumes, which are either processed on-site or transported by trucks for further treatment. In contrast, containers, especially large-capacity ones, are preferred for efficient waste loading and transport to disposal or recycling facilities. The type and size of containers are selected based on the waste characteristics and volume, ensuring compatibility with handling equipment such as loaders, cranes, or other handling equipment.

### Containers

The primary container types for CDW include standard containers (skips) for skip loader vehicles and roller containers for hook-lift vehicles.

Skips Figure 1, containers with a capacity ranging usually from 2 to 20 cubic meters and a maximum load capacity of 12 tons are available. A hydraulic arm is used to lift the skip, ensuring it remains in a horizontal position throughout the lifting process. Various skip designs exist, including open-top, enclosed containers, and versions with or without doors, to accommodate different waste management needs.

Figure 1: Skip (Laudon GmbH & Co. KG 2024a)

Roller containers Figure 2 have a capacity ranging from 3 to 40 cubic meters and a load capacity up to 15 tons. The size of the container is selected based on the type and expected volume of waste. Unlike other container systems, roller containers are loaded using a hook mechanism and can tilt up to 45 degrees during the loading process.

These containers are available in various configurations, including covered top, different door types, and side-loading options. Compared to standard skip containers, roller containers offer a higher degree of customization, allowing for tailored solutions based on specific waste management needs.

Figure 2: Roller container (Laudon GmbH & Co. KG 2024b)

A specialized subgroup of roller containers includes compaction containers, which integrate a built-in compaction system. This system enhances transport efficiency by reducing the volume of compressible waste.

A more flexible alternative for small-scale waste collection is the Flexible Intermediate Bulk Container (FIBC). Made from woven polypropylene, these lightweight yet durable containers have a capacity of up to 2,000 kg and are equipped with lifting loops for easy transport using forklifts or cranes. In cases where only a small amount of separable waste is generated, it may not be cost-effective for contractors to maintain an on-site separate skip, as transportation costs can exceed disposal costs. As a result, waste is often mixed despite being selectively generated. In such situations, FIBCs offer a practical solution, allowing for more efficient waste handling and cost management.

Recommendations for effective on-site waste management to promote sorting

Establish a dedicated waste area: Evaluate the types, quantities, and timing of waste generation based on a waste audit or project assessment. Select appropriate container types and quantities to ensure efficient waste separation and collection. The waste area should be easily accessible for both workers and waste removal vehicles to support smooth operations. For reusable and recyclable materials vulnerable to weather damage or material degradation due to weather conditions, provide sheltered containers or a covered storage area to prevent such degradation. To prevent recycling containers from being used for general waste, place a general waste container next to recycling containers.

Label waste area and containers: Label designated waste area. Each container or storage location for separated waste should be clearly labelled, preferably with images or pictograms to indicate the specific type of waste it is intended for IoT container accessories.

Digital tagging: Digital tagging technologies, such as QR codes or RFID tags, offer potential improvements in managing CDW containers. By integrating these tags into waste management systems, several benefits can be achieved.

- linking digital tags to internal dispatch systems can streamline collection requests. When a container is full, a quick scan can notify the waste facility, enabling automation in pickup scheduling.

- Second, digital tagging enables accurate waste tracking. Each container has a unique identifier, which allows waste facilities to record its movements, contents, and current status. This improves tracking and traceability of waste.

- Third, digital tags can give easy access to information about the waste type and proper usage guidelines. By scanning a code, users can verify what materials are allowed, reducing contamination and ensuring correct handling.

GPS: Similarly to digital tagging, GPS technology improves logistics, tracking, and traceability of waste. It provides detailed information on when a container was delivered, when it was picked up, and how long it remained on-site. When integrated into a dispatch system, it allows for automated route optimization, enabling waste facilities to select the most efficient collection routes.

Fill level monitoring: Automated fill-level monitoring has the potential to greatly improve the logistics of construction waste management. If sensors were used to measure container capacity, collection orders could be automatically generated whenever a container reaches its limit. This approach could help prevent overfilling, reduce the need for manual checks, and improve overall efficiency in the collection process. A Norwegian technology startup, SENSORITA, has developed a device that uses radar sensors and machine learning to measure fill levels (“Sensorita” 2025).

### Development of a smart device for containers

The Czech Technical University in Prague (CTU) developed the first version of a smart device for monitoring CDW containers Figure 3. This device is designed to provide a detailed overview of the container’s current location, fill level, and contents in real-time.

The technology includes a camera for capturing images of the container’s interior, a depth sensor to measure fill level, an integrated GPS module for accurate position tracking, battery power with solar panel recharging capability, and LTE data transmission to a CTU server. The collected data is displayed on a user-friendly dashboard accessible to ENVISAN-GEM’s dispatch team via a web interface Figure 4.

Figure 3: Smart device for monitoring CDW containers

Field Testing the Prototype

The first version of the device was tested in the field by ENVISAN-GEM on a 3 m³ roller container designated for construction debris. The goal was to verify the technology’s functionality under real conditions and identify potential technical or operational shortcomings.

During the testing phase, several key limitations became apparent. The solar panel was not optimally placed, leading to insufficient battery recharging. The GPS module experienced signal dropouts in densely built-up areas, adversely affecting the container’s location accuracy. Additionally, the depth sensor proved unsuitable for open containers, failing to provide reliable fill level data.

Figure 4: Container Dashboard: Top-left: current location. Top-Right: real-time content data. Graphs: fill level, temperature (top row); humidity, battery level (bottom row)

Practical Application and Limitations

Although the current version of the device does not yet support automated waste classification or autonomous fill-level measurement, it has significantly improved manual oversight. Real-time image data allowed ENVISAN-GEM’s dispatchers to visually assess both the container’s fill level and contents. In cases of incorrect loading, such as the presence of contaminants, the responsible party on-site was contacted to address the issue immediately. This approach minimized the risk of transporting inappropriate material to the processing facility, thereby enhancing overall operational efficiency.

## Off-site treatment

Efficient treatment of CDW is essential for sustainable resource management, reducing landfill dependency, and conserving primary materials. (Symonds Group 1999) categorized CDW recycling technologies into three levels of complexity.

Level 1: Basic systems using mobile or stationary crushers and simple screening, suited for inert materials like concrete and bricks. Economical and flexible, these are viable in regions with low landfill costs and cheap primary aggregates.

Level 2: Intermediate systems with enhanced sorting, including metal removal and refined sieving, capable of handling mixed but predominantly inert waste. They balance cost-efficiency with improved material recovery.

Level 3: Advanced facilities incorporating comprehensive sorting, washing, and handling of diverse waste streams (e.g., wood, plastics). These systems achieve high recovery rates but require significant investment, prevalent in areas with strict regulations and high landfill fees.

With ongoing advancements, Level 4 can be introduced, incorporating Industry 4.0 technologies into waste treatment (Davies 2015)(Santos et al. 2017). These technologies include Artificial Intelligence (AI), Machine Learning, Deep Learning, Computer Vision and Robotics (Iyiola et al. 2024)(Dodampegama et al. 2024)  This level enables the separation of individual waste streams, achieving high purity levels while providing real-time material flow characteristics to specify the composition of waste streams.

This tiered approach shows how CDW recycling can adjust to different needs. Better sorting on-site can make it possible to use simpler recycling technologies. Different waste streams may need different levels of technology, each with its own costs. It is important to match the method to the specific situation and requirements.

Offsite treatment includes several steps aimed at the processing, sorting, and treatment of waste.

### Waste reception and inspection

Every facility receiving waste should establish a waste acceptance protocol to ensure compliance with legal, environmental, and quality standards. The waste acceptance process begins with the submission of relevant waste information from the construction and demolition site or the transport company. This information includes legal requirements, material certificates, and additional waste-related data, which serve as the basis for further verification.

Upon physical reception of waste, an initial inspection is conducted to verify whether the provided information accurately reflects the actual waste composition. This step ensures that the incoming materials comply with specific regulatory, environmental, and quality requirements before being accepted for further processing, recycling, or disposal. A robust quality assurance framework is essential for proper waste management and helps mitigate the risk of contaminating the treatment process due to the acceptance of non-compliant waste materials.

Furthermore, the properties of incoming waste must align with the facility’s capability to process and refine the material into a final product that meets specified standards and criteria for further utilization.

Following the initial inspection, the accepted waste is weighed and directed into the appropriate processing stream based on its physical and chemical properties, in accordance with the facility’s internal logistics and operational framework. In most cases, the waste undergoes temporary storage (intermediate stockpiling), where it is grouped with other batches of the same classification.

Enhanced

To enhance the waste acceptance process, sensor-based technologies with deep learning algorithms can replace visual inspection at entry. These systems can analyse waste composition in real time, enabling more accurate classification and contaminant identification. This automation can then direct each waste batch to the correct intermediate storage and treatment stream based on its type, properties, and contamination level. Moreover, this technology opens the door to new business models, such as dynamic pricing of waste batches based on their impurity content. This would motivate better on-site sorting.

### Pre-treatment

Certain types of CDW require pre-processing, particularly large structural components such as panels and columns. These materials must be reduced to a manageable size before entering the next stages of processing.

Pre-treatment involves mechanical fragmentation using hydraulic shears, hydraulic hammers to break down oversized materials. Additionally, large foreign objects must be removed to ensure efficient downstream processing.

### Pre-screening

Pre-screening is a critical step in the recycling process of CDW, designed to separate oversized and fine particles from the main material flow before further processing. This stage enhances the efficiency of downstream operations, ensuring that only appropriately sized materials enter subsequent crushing or sorting units (Bilitewski 2010)(Barbudo et al. 2020).

Pre-screening units, such as grizzlies, bar screens, and disc screens, are coarse separation devices that remove oversized materials from the waste stream. These units are generally positioned at the beginning of the processing line to prevent large materials from overloading crushers or secondary screens. (Müller et al. 2022b)

Additionally, horizontal or inclined screening systems often integrated into feed hoppers further refine the material flow. Particles larger than a predefined size (e.g. 30 mm) are directed to crushing or further sorting, while finer fractions (such as soil, plaster, and small debris) are removed. These fine materials may undergo further processing or be excluded from additional treatment, depending on their composition and quality (“Hrubotřídiče” 2024).

### Crushing, shredding and size reduction

Crushers play a fundamental role especially in the recycling of mineral CDW by reducing particle size for further processing. The most commonly used crushers for mineral CDW include jaw crushers, impact crushers, and cone crushers Figure 5, each with specific advantages suited for different crushing stages and material requirements (Silva et al. 2017)(Pacheco et al. 2021).

Figure 5 Schematic image of a) impact crusher b) jaw crusher c) cone crusher(Bilitewski 2010)(Lindqvist et al. 2006)

Jaw crushers operate with one fixed and one oscillating plate that crushes material into smaller particles. This type is particularly effective for primary crushing, as it can handle large waste fragments, including reinforced concrete, without clogging. However, the resulting material often has an elongated shape, which may be less suitable for certain applications (Silva et al. 2017).

Impact crushers use a high-speed rotating rotor with hammers that propel material against fixed impact plates. This mechanism ensures more uniform particle distribution and improved shape. They are also less sensitive to materials that are difficult to crush, such as steel reinforcement, but tend to generate a higher proportion of fine particles than other crushers (Silva et al. 2017) (Bilitewski 2010).

Cone crushers crush material between an inner fixed cone and an outer cone that moves eccentrically. This design is suitable for secondary and tertiary crushing, particularly when a more refined particle shape is required. However, cone crushers are not ideal for processing large pieces due to the risk of clogging. Their main advantage lies in lower energy consumption and precise control over particle size distribution (Bilitewski 2010) (Silva et al. 2017).

For optimal recycled aggregate (RA) quality, a multi-stage crushing process is typically recommended. Primary crushing is usually performed with jaw or impact crushers, while secondary and tertiary stages often involve impact or cone crushers. This approach improves particle size distribution and enhances the properties of recycled material for various construction applications.

Other CDW fractions, such as wood and plastics, require different size reduction machinery, including shaft shredders, which operate with one or multiple rotating shafts equipped with blades. These machines efficiently process both rigid and elastic materials, producing output suitable for further recycling (Vatskicheva et al. 2017).

The selection of an appropriate crusher depends on the material type and its specific properties.

Enhanced

Particle size distribution is an important parameter for effective use of secondary materials, influenced by the high variability of input waste properties. AI techniques can estimate particle size distribution in real-time (Olivier et al. 2020) at the crusher output or, in combination with material composition estimation, at the crusher input. This information could be used for operational crusher adjustments, such as modifying motor speed, to control particle size distribution (Chimwani et al. 2021). Implementing such adjustments would help achieve a more uniform final product.

### Screening

Screen classification involves sorting a mixture of particles with varying sizes into distinct groups based on their geometric dimensions. This is achieved using a screening equipped with uniformly sized openings that determine the separation threshold. The material being processed is divided into two main fractions: smaller particles that pass through the openings (referred to as undersize or fines) and larger particles that remain above the screen (referred to as oversize or coarse material).

Screening is a critical process in the recycling of mineral CDW. The choice of screening equipment depends on the material's properties, desired output, and process requirements. The main types of screening equipment include trommel screens, vibrating screens, and grizzlies or grates, each serving distinct roles in the separation process.(Müller et al. 2022b)(Coelho et al. 2013)

Trommel screens consist of an inclined, rotating cylindrical drum that serves as the screening surface. Material is fed into the drum's higher end and moves through due to the rotational motion, which continuously loosens and mixes the material. Fine particles pass through the drum's openings, while coarse particles exit at the lower end. Trommel screens are ideal for separating medium-sized particles (10–80 mm) and are widely used for materials like light waste and mineral construction waste. Their simple design and vibration-free operation make them a reliable choice, though they require additional cleaning mechanisms for optimal performance (Müller et al. 2022b)(Bilitewski 2010).

Vibrating screens operate by inducing vibrations in the screening deck, facilitating the separation of materials based on size. They are versatile and can process a range of materials, including crushed rubble and fine particles. Vibrations can be linear, circular, or elliptical, depending on the design, and enhance particle movement and stratification. These screens are durable and suitable for heavy-duty applications, offering efficient loosening and classification. Variations like flip-flop screens include self-cleaning features for improved efficiency (Müller et al. 2022b).

### Sorting

Sorting is a process in the treatment of CDW that separates materials based on their physical and chemical properties. Sorting methods include dry methods, wet methods, ferromagnetic separation, eddy current separation, manual sorting (hand-picking) and advanced sorting technics. These methods are used to divide waste into different material fractions for further processing or disposal (Müller et al. 2022b)(Bilitewski 2010). Waste sorting can be classified into two categories: conventional direct sorting and advanced indirect sorting. Direct sorting uses physical forces or manual picking to separate materials by properties like weight, density, and electromagnetic behaviour. Indirect sorting, on the other hand, employs sensors and AI systems to identify materials based on chemical, optical, or physical features (Dodampegama et al. 2024).

Direct sorting: Dry sorting methods include air classification, ballistic separation, and density-based techniques. Air classifiers, such as zigzag or rotary air classifiers, separate materials by aerodynamic velocity, effectively distinguishing light fractions (e.g. paper, plastics) from heavy fractions (e.g. metals, stones). Ballistic separators exploit particle size, density, and rigidity by using inclined vibrating decks to segregate light, heavy, and fine fractions (Pacheco et al. 2021)(Silva et al. 2017)(Coelho et al. 2013).

Wet sorting methods utilize water as a medium to differentiate materials by density and weight. Techniques include swim-sink sorting (with static water), upstream separation, jigging or film layer sorting (Müller et al. 2022b)(Bilitewski 2010). Wet methods, like dry methods, separate materials but also help remove clay and some water-soluble contaminants (Pacheco et al. 2021). However, the water used in these processes must be treated.

Magnetic sorting isolates ferrous metals using overbelt magnets or drum separators. These methods efficiently extract reinforcing steel and other magnetic components. Eddy current sorting separates non-ferrous metals (e.g. aluminium, copper) by inducing eddy currents that repel conductive materials. This technique is effective for lightweight, high-conductivity metals but less so for dense or low-conductivity materials like lead (Müller et al. 2022b)(Bilitewski 2010)(Pacheco et al. 2021).

Manual sorting (hand-picking) remains relevant for coarse, heterogeneous waste streams. Operators manually extract large contaminants (e.g. wood, plastic, steel) to ensure cleaner downstream processes. While labor-intensive, this method is effective for non-standard or complex waste compositions. The manual sorting stations can be arranged either before or after the comminution (Bilitewski 2010)(Müller et al. 2022b).

Indirect sorting: Indirect sorting utilizes sensor technologies to analyse and classify materials based on their physical or chemical properties without direct physical intervention. This advanced method employs various sensors, such as optical cameras for colour and brightness detection, near-infrared spectrometers for identifying molecular composition, and X-ray detectors for distinguishing materials by density. By processing data from these sensors, the system controls sorting mechanisms like air jets or robotic grippers to separate materials efficiently (Müller and Martins 2022a).

These systems are driven by software-based algorithms, often referred to as traditional AI. This approach integrates sensors with manually designed classifiers.

Despite their effectiveness, traditional AI systems are constrained by their reliance on predefined rules and manually engineered classifiers. Machine learning and deep learning, a specialized domain within AI, represents a radical improvement in sorting technologies. Unlike traditional AI, which requires explicit programming, deep learning utilizes neural networks trained on large datasets to autonomously identify complex patterns and subtle distinctions in materials. Through the analysis of extensive image datasets, these systems acquire a detailed understanding of material properties, enabling the recovery of items that were previously indistinguishable (Kroell et al. 2022) (Rhoad 2023).

This multi-step process begins with waste identification, where sensors capture data such as chemical, optical, or physical attributes tailored to specific waste stream. The collected data is processed using deep learning techniques to accurately identify a classify waste fragments and locations.

In the next phase, robots or machinery equipped with intelligent systems handle the sorting. These systems use algorithms to manoeuvre robotic effectors, ensuring precise material handling and placement. Challenges such as irregular shapes, stacking, and varying material sizes are addressed during this stage. Indirect sorting provides enhanced accuracy and efficiency compared to direct methods (Dodampegama et al. 2024).

After the classification of material fragments, physical sorting is performed using robots equipped with various types of end-effectors, such as two-finger grippers, multi-finger grippers, or vacuum grippers (Kiyokawa et al. 2024). These robots operate under different configurations and speeds, depending on the material characteristics and system requirements.

For example, heavy-duty robots like the ZenRobotics Heavy Picker can handle large fractions up to 40 kg and dimensions of 1.5 m at a rate of 2,300 picks per hour. (“Heavy Picker | ZenRobotics” 2025). In contrast, light-duty robots, such as the ZenRobotics Fast Picker, excel at sorting lightweight materials with speeds reaching 80 picks per minute (“Fast Picker | ZenRobotics” 2025).

The type of robot directly influences its placement in the sorting process. Heavy-duty robots are better suited for handling large-size particles and are typically positioned after the primary crushing stage. This enables efficient sorting of coarse fractions. On the other hand, light-duty robots are optimized for finer fractions and are often deployed after the secondary crushing stage, where smaller particle sizes require higher precision and speed(Ku et al. 2021).

For smaller waste particles, additional technologies, such as ejectors, are commonly used to separate desired fractions (positive sorting) or to remove non-desired fractions from the waste stream (negative sorting). Various ejection mechanisms are employed in these systems, including mechanical flaps and water jets. However, air ejectors, which use compressed air to eject pieces from the stream, have become the standard in this application due to their efficiency and reliability. These devices offer high throughput, achieving up to 2,000 ejections per minute (“GAINnext: Waste Sorting Machine | TOMRA” 2025).

### Output quality control

Output quality control is the final stage of the quality management process in CDW treatment. This process begins with demolition, followed by waste acceptance, processing control, and final testing. Each of these steps ensures that recycled materials meet environmental and technical requirements, whether for regulatory compliance or the needs of further processing.

One of the most common barriers to the effective use of recycled materials is their variable composition, especially in mineral waste fractions. This variability can impact the consistency and reliability of the materials, making output quality control an essential part of the process.

Output quality control is conducted intermittently, with varying frequencies of sampling and testing.

Enhanced

Sensor-based technologies combined with deep learning algorithms can improve output quality monitoring by analysing the composition of processed waste in real time. These systems provide continuous data on the composition of recycled materials, allowing for more precise quality assessment. Real-time monitoring not only provides a comprehensive overview of the final product but also enables adaptive process control, adjusting processing parameters to maintain consistent material properties.

## Primary CDW stream from buildings

The generation of CDW in the EU was analysed by (Damgaard et al. 2022) through an extensive literature review. Their study compiled data on the quantity and composition of CDW for individual MS. The composition of CDW was found to be heavily influenced by demolition methods, separation processes, and waste management practices, as evidenced by the high proportion of mixed CDW. To estimate the potential of individual material fractions, the authors conducted material flow analysis (MFA) modelling.

The theoretical composition of materials originating from building construction and renovation activities was further refined and presented in an adapted form by (Garcia et al. 2024) as illustrated in Table 2. This approach provides a more detailed and standardized perspective on the material fractions within CDW, which is important for assessing recovery potential and advancing sustainable construction practices across the EU.

Table 2: Theoretical composition of CDW from building based on MFA modelling (Garcia et al. 2024)

| Material fraction | Year 2020 (kt) | Year 2020 (%) |
| --- | --- | --- |
| Aluminium | 2,323 | 1.8% |
| Brick | 8,583 | 6.5% |
| Cardboard and Paper | 9 | 0.01% |
| Ceramics | 7,34 | 5.6% |
| Concrete | 74,169 | 56.2% |
| Copper | 97 | 0.07% |
| Electronics | 3 | 0.002% |
| Glass | 5,328 | 4.0% |
| Gypsum | 750 | 0.6% |
| Insulation | 839 | 0.6% |
| Other Construction Minerals | 1,809 | 13.7% |
| Other Metal | 184 | 0.1% |
| Paint and Glue | 1,148 | 0.9% |
| Plastic | 1,889 | 1.4% |
| Sand | 1,195 | 0.9% |
| Steel | 6,174 | 4.7% |
| Wood | 3,835 | 2.9% |
| Total | 131,956 | 100% |

### Concrete

Waste specification: Concrete and reinforced concrete waste occurs as large blocks (panels, columns) or fragments depending on the demolition method. Common contaminants include reinforcing steel, soil, and other mixed CDW components, adhered tar or bitumen waterproofing.

Waste management pathways: Concrete is commonly recycled into recycled aggregate (RA), which is often downcycled and primarily used for road construction, subbase layers, foundation structures (product specification according to EN 13242), and backfilling. Higher-value recycling involves the use of recycled concrete aggregate RCA in the production of new concrete and prefabricated elements. In road construction limited use is even in asphalt mixtures where such recycled aggregate shall fulfil requirements of EN 13043.

RCA for concrete is regulated by the harmonized European standard EN 12620, which defines the requirements for its quality and properties. However, the use of RA in concrete production is governed by various national standards, which typically classify RCA into high-quality and moderate-quality categories based on the proportion of concrete fraction in the aggregate. These classifications determine the maximum allowable percentage of RCA that can be incorporated into different classes of structural and non-structural concrete.

In most national regulations, only coarse RCA (>4 mm) is permitted for use in concrete, while fine RA is generally excluded due to its negative impact on concrete properties. The inclusion of fines tends to reduce the mechanical and physical properties of concrete; however, some countries allow limited use under specific conditions. In general, only coarse RCA is used, and it is primarily utilized in lower strength classes and lower exposure classes of concrete.

Barriers to high-value applications: The usability of RA depends on its composition, particularly the proportion of concrete and natural stone waste. Apart from chemical contamination, the presence of other CDW materials, especially non-inert contaminants or floating inert particles, significantly reduces its applicability. Additionally, other inert impurities, including ceramics, bricks, and glass, further limit its potential uses.

The quality of RA is directly linked to the composition of the input materials and the chosen processing technology.

On-site collection and separation methods: The collection and segregation of concrete waste depend on the demolition method. It can either be separated from other CDW or collected as part of the mineral waste fraction, which may contain varying amounts of other CDW materials.

Concrete waste is typically stockpiled on-site or collected in open containers for further processing. Selective demolition and separate on-site collection significantly enhance the applicability of RA.

Figure 6: Concrete waste processing flow-chart

Off-site sorting and treatment: Concrete waste is typically sorted as part of the recycling process at regional collection centres, depending on the level of technological equipment available. The sorting process can be categorized into different levels:

Level 1 – Pre-sorting or no sorting: The output material retains the same composition as the input material and is only pre-crushed without further processing, additionally sieving.

Level 2 – Magnetic separation: Metal reinforcements (such as steel rebar) are removed using magnetic separation.

Level 3 – Comprehensive separation process: In recycling plants, a combination of methods is used to separate different materials:

    - Magnetic separation removes metal reinforcements.

    - Pre-sieving eliminates fine materials such as soil.

    - Lightweight fractions (such as wood and plastics) are removed using dry or wet sorting methods, including air sifters and flotation tanks, or manual sorting.

Difficult-to-separate impurities, such as bricks and gypsum-based materials, present challenges in the sorting process. However, advanced direct sorting techniques are developed to improve their separation (Ambrós et al. 2017).

The use of advanced robotic sorting technologies allows for the identification and separation of both non-inert fractions and inert materials with similar physical properties, which are difficult to sort using direct sorting methods. This advanced sorting improves the quality of recycled aggregate and enables its use in high-value applications.

### Bricks

Waste specification: Brick waste consists of various types of bricks, often bonded with mortar and commonly mixed with other mineral fractions of CDW.

Common contaminants: These typically include mortar residues, ceramic tiles, aerated concrete, gypsum-based materials, and soil. Additionally, non-mineral contaminants such as wood and wood-based materials, plastics, metals, and paper are frequently present. Other contaminants may include tar, sealants, adhesives, paints, and coatings, as well as thermal insulation materials.

Waste management pathways: Bricks, either alone or as part of other inert CDW, are mostly used in backfilling or as aggregates for subbase layers in roads and construction after being crushed. When processed into brick recycled aggregate, they can be incorporated into concrete mixes. According to EN 206, up to 30% by mass of coarse aggregates in lower-strength structural concrete can be replaced by recycled brick aggregate. Additionally, crushed bricks can be used as aggregates for masonry elements, providing a sustainable alternative to natural aggregates.

Beyond these applications, bricks also have potential for more advanced uses. They can serve as a supplementary cementitious material, partially replacing cement (Schackow et al. 2015) (He et al. 2021), or can be used as a raw material for alkali-activated binders and can be utilized in the production of building elements (Rivera et al. 2017).

Bricks can also be reused after mortar removal for masonry or cut into cladding for architectural applications (“Gamle Mursten” 2025).

Barriers to high-value applications: The utilization of brick waste faces several challenges, primarily contamination with hazardous substances, such as PAHs, whose concentrations may exceed regulatory limits, rendering the material unsuitable for further use. Another limitation is the presence of undesirable impurities, particularly other inert materials such as glass, bituminous materials, or (AAC), which can negatively impact the properties of the recycled aggregate and restrict its application in high-value products. Additionally, the high variability of this waste stream complicates standardization and the ability to ensure consistent quality in recycled materials.

On-site collection and separation methods: Bricks are typically mixed with other inert materials and either stockpiled or loaded into open containers during demolition and waste collection. Improvement measures include preparing bricks for reuse by cleaning and sorting them during selective demolition. Properly cleaned bricks can be separated and stacked on pallets, facilitating handling and transportation. It is also essential to ensure that brick waste is not mixed with materials exceeding regulatory limits for hazardous substances, such as PAHs, to maintain compliance and enable further use.

Figure 7:  Bricks waste processing flow-chart

Off-site sorting and treatment: Brick waste is typically sorted as part of the recycling process at regional recycling facility to produce RA, depending on their level of technological equipment. It is processed together with other inert waste, with non-inert impurities removed when necessary.

Level 1 – Presorting or no sorting: The output maintains the same composition as the input material, with only crushing additionally sieving.

Level 2 – Magnetic separation: Metallic materials are removed using magnetic separation.

Level 3 – Advanced sorting: In recycling facilities, metal materials are removed through magnetic separation, while soil and other fine materials are separated using pre-sieving. Lightweight fractions, such as wood and plastics, are extracted using dry or wet sorting techniques (e.g., air sifters, flotation tanks) or manual sorting.

The use of advanced robotic sorting technologies can replace direct sorting methods, enabling more precise removal of impurities and the separation of other inert fractions based on the desired output. This results in a material with lower variability, making it more suitable for high-value applications.

### Wood/timber

Waste specification: Wood waste includes solid wood materials such as beams, planks, columns, plywood, particle boards, OSB boards, and other wood-based materials. It can be found in the form of construction elements, formwork, flooring, cladding, or building components.

Common contaminants: Wood waste may contain contaminants such as coatings with heavy metals, chemical treatments (e.g., impregnation, pesticides, fire retardants), and metal fasteners (nails, screws, joints). Additionally, plastics, adhesives (e.g., asphalt-bonded parquet), mortar or concrete residues, and other surface treatments can further affect its recyclability. Some older plywood products might contain also hazardous adhesives and related chemicals.

Waste management pathways: Energy recovery remains a widespread practice, as well as recycling methods. Structural components, such as timber beams, have the potential for reuse in construction. If reuse is not possible, wood should be processed following a hierarchical recycling approach, where solid wood is first converted into chip-based products (e.g. OSB) and subsequently further refined into fiber-based materials, such as MDF or HDF, for applications in the construction and furniture industries.

Barriers to high-value applications: The recycling of wood is often limited by the presence of contaminants and unwanted impurities. Coatings and impregnations containing heavy metals, pesticides, or fire retardants prevent the material from being processed into new wood-based products. Additionally, impurities such, other CDW, low-quality wood waste (e.g. MDF), and metal components further restrict recycling potential. However, metal impurities are generally not a significant issue for particle board manufacturing, as they can be effectively removed during processing.

On-site collection and separation methods: Wood waste is often collected as part of mixed construction waste without thorough sorting or is categorized solely as hazardous or non-hazardous waste. However, in certain countries, such as Germany, Finland, and the Netherlands, a more detailed classification system based on quality criteria is implemented, categorizing wood into three to five distinct groups, including clean wood, non-hazardous I, non-hazardous II, hazardous I, and hazardous II.

To enhance on-site collection and segregation practices, the adoption of a standardized classification system aligned with quality criteria and priority use is recommended. Additionally, the use of covered containers is recommended to prevent weather-related degradation, ensuring that the wood remains suitable for further processing.

Off-site sorting and treatment: Upon arrival at waste collection centres, wood is sorted into two main categories: wood for material recovery and wood for energy recovery. Initially, manual pre-sorting is conducted to remove larger impurities, ensuring that the material meets processing requirements. Magnetic separation is also commonly used to extract metal contaminants.

The application of robotic vision technology enables the automated identification and removal of impurities. Additionally, it facilitates the classification of wood based on quality, allowing for the separation of mixed wood streams into different quality grades. This process distinguishes low-value wood (e.g., MDF, chipboard, chemically treated wood), which is suitable for energy recovery, from higher-quality wood that can be recycled into new materials.

Figure 8: Wood waste processing flow-chart

### Gypsum

Waste specification: gypsum blocks, plasterboard, and gypsum fibre boards.

Common contaminants: Common impurities found in plasterboard waste include plastics, foils, insulation materials (mineral and glass wool), steel rails and bars, wooden pieces, AAC and other CDW. On the other side, most of these impurities are easily separable.

Waste management pathways: Although gypsum waste is fully recyclable, post-demolition gypsum waste is still mostly landfilled in separate cell. In addition to closed-loop recycling into new gypsum-based products, gypsum can also be used in agriculture to improve soil quality and as an additive in cement production.

Barriers to high-value applications: New construction materials are more easily recycled, whereas demolition and renovation waste pose greater challenges due to increased contamination and difficulty in separation. The level of contamination plays a crucial role in determining market acceptance.

On-site collection and separation methods: In most MS, the separate collection of gypsum waste is required. However, it is often contaminated with other CDW fractions, reducing its recyclability. When stored in open containers or stockpiled on-site, gypsum waste is exposed to moisture contamination, which further diminishes its quality and value. To ensure its suitability for recycling, clean plasterboard should be properly separated and stored in covered storage areas.

Off-site sorting and treatment: Off-site sorting is generally not performed. The acceptance of gypsum waste for recycling applications is determined based on specific acceptance criteria. Batches that do not meet these criteria are rejected as non-recyclable and are subsequently landfilled. Advanced off-site sorting utilizing deep learning methods and automated sorting technologies can enhance the separation of plasterboard from contaminants, ensuring compliance with the required acceptance criteria for recycling.

Figure 9: Plasterboard waste processing flow-chart

### Glass

Waste specification: Glass from CDW primarily consists of flat glass, including clean float glass, wired glass, mirror glass, coated glass, ceramic frit glass, and laminated glass. It is often found as part of mixed CDW from conventional demolition processes or as a component of windows and doors.

Common contaminants: Cross-contamination with other CDW, laminated interlayers, sealants, paint or coatings applied to glass.

Waste management pathways: In mixed CDW, glass is typically processed into low-quality RA or sent to landfill. Recycling options include reprocessing into new float glass or other glass products. Alternative material recovery pathways involve using glass for foam glass production (incl. light-weight aggregates form foamed glass), glass wool insulation, geopolymers, composites, and abrasives.

Barriers to high-value applications: The recyclability of waste glass depends on the quality of the cullet. For new float glass production, any contamination or the presence of different glass types is a major barrier. Foam glass, glass wool, and other material recovery processes have lower quality requirements for cullet, allowing for a broader range of recycled glass inputs.

Figure 10: Glass waste processing flow-chart

On-site collection and separation methods: Many MS have a legal requirement for the separate collection of glass. However, on construction sites, glass is often smashed out of window frames, becoming contaminated with other CDW fractions and typically not sorted by type. To improve on-site collection and segregation, glass should be deglazed and separated into appropriate transport containers. Alternatively, entire windows can be placed into dedicated window holders for safer handling and transportation.

Off-site sorting and treatment: Due to low volumes in CDW and the difficulty of separating glass from other mineral fractions, off-site sorting at collection centres is generally not performed. In some cases, handpicking may be used to remove larger impurities. During incoming quality control, glass is assessed based on cullet quality, glass type, and contamination levels, then collected separately in storage locations before being transferred to a glass recycler. In selective demolition, entire windows are often separated and further processed. Advanced sorting technologies are not commonly used in collection centres, as they are not economically viable. However, glass recycling companies utilize optical sorters to further refine and separate glass cullet.

### Mineral insulation

Waste specification: This category includes mineral wool (glass and rock). Mineral insulation waste can be divided into three categories: (i) production waste, (ii) application waste, and (iii) construction and demolition waste. These materials, in the case of conventional demolition, end up in mixed CDW or are often part of the light mixed fraction of CDW containing plastic, wood, and other CDW.

Common contaminants: Other CDW streams/materials, (especially construction adhesives, plasters, etc.). Another common contaminant is surface hydrophobic and antistatic treatments (applied always during manufacture processes), organic pollution, including plants, animal droppings, etc. Additionally, it may be mixed with incorrectly identified asbestos insulation.

Waste management pathways: Production waste is easily recoverable—either it is processed on-site outside the waste management system or transferred as a by-product to another production process (e.g., for manufacturing acoustic ceiling panels). Waste from on-site applications is most often recycled as a by-product (also for ceiling panels) or used as a raw material in the production of new insulation. The most problematic is demolition waste. If no technical data sheets are available for the insulation, it is classified as hazardous waste and must be landfilled according to the relevant regulations. Additionally, mineral insulation is often found in mixed construction and demolition waste. Recycling options for this material are still being investigated.

Conventional waste management practices: Mineral insulation is, in the vast majority of cases, landfilled. Some producers of mineral wool offer a leftover material take-back scheme for their product (“Rockcycle®” 2025). Waste generated during insulation production or on-site application is efficiently reused for other purposes. In contrast, waste from building demolition—where the insulation is assumed to be several decades old—almost always ends up in landfills.

High-value applications: Recycling of mineral wool offers several possibilities, including reintegration into production of new mineral insulation, use in ceramic composites, ceiling panels, cementitious materials or hydroponic substrates (Väntsi et al. 2014). Current efforts focus on utilizing waste as so-called blown insulation. Wasted mineral wool at the form of boards or rolls is milled into fine particles, contaminants are removed, and the resulting material is reused as thermal insulation through blowing or free deposition.

Barriers to high-value applications: Post demolition mineral wool often contains impurities such as binders, plasters, lubricants, aluminium foils, organic pollution, and other construction materials, which complicate the recycling process. Due to its low density and high volume, transportation costs are considerable, making logistical handling challenging. During reintegration into production, fine mineral wool particles tend to clog furnaces, disrupting the manufacturing process. Given its high volume but relatively low weight, landfill disposal remains the more cost-effective option in most cases (Väntsi et al. 2014).

Common on-site collection and separation methods: Efforts are currently underway to develop a mobile recycling line capable of processing, cleaning, and treating waste mineral insulation on-site. The resulting material is then used as blown insulation. Several stationary recycling lines for this waste already exist in Europe. However, due to high transport costs and the associated environmental burden, their large-scale deployment remains limited.

Improved on-site collection and separation methods: Mineral insulation waste should be removed separately and collected in designated containers to prevent contamination with other materials. Shredders can be used to reduce the volume of mineral insulation waste, thereby improving its transportability.

Off-site sorting and treatment: Off-site sorting is not commonly conducted to recover mineral insulation. Instead, mineral wool is separated from mixed CDW to improve the quality of recycled aggregate. Once removed as a contaminant, it is typically disposed.

Mineral wool recovered from selective demolition can be sorted by type, cleaned, and prepared for further processing. This may involve volume reduction through shredding or briquetting, enabling its direct use as a raw material in the production of new mineral insulation.

Figure 11: Mineral insulation waste processing flow-chart

### Expanded polystyrene (EPS)

Waste specification: EPS in form of insulation board and EPS as part of External Thermal Insulation Composite Systems (ETICS).

Common contaminants: Include layers of ETICS, such as adhesives, plastering, fixings, reinforcement materials, and coatings, along with other CDW. EPS from buildings constructed before 2017 may contain the flame retardant HBCD, which is classified as a persistent organic pollutant.

Conventional waste management practices: As long as waste EPS does not contain toxic HBCD flame retardants, it is typically compressed and reused in the production of new polystyrene. A significant portion is also used for energy recovery, i.e., incineration. If HBCD retardants are present, the polystyrene is almost exclusively incinerated. In Europe, a solution has already been developed (called as PSLoop) to separate these retardants; however, it is a complex and chemically demanding process. As a result, large-scale implementation has not yet occurred (Garcia et al. 2024).

High-value applications: Recycling to produce new EPS by mechanical recycling or solvent based recycling. Solvent based recycling allowing separation of HBCD (Schleier et al. 2022). Recycling into raw material for lightweight concrete if not containing HBCD.

Barriers to High-Value applications: Material contamination, HBCD presence, high transport and handling costs due low density (Schleier et al. 2022)(“PSLoop” 2025).

Common on-site collection and separation methods: Typically separated and compressed. Such a material is then used for production of new EPS or incinerated. Smaller parts are often mixed with other lightweight CDW or segregated as an entire ETICS system.

Improved on-site collection and separation methods: Selective demolition should be carried out to facilitate the proper management of EPS-based insulation waste. It is necessary to determine the presence of HBCD before further processing. Additionally, waste collection must be conducted separately to prevent contamination and enable appropriate treatment.

Off-site sorting and treatment: Off-site sorting is not generally conducted (EUMEPS 2021), the separation of EPS contaminated with materials such as plastics, concrete, and bricks remains a challenge, as effective separation technologies have not yet been fully established (Schleier et al. 2022). However, fluid bed separators hold potential for this purpose (“EPSolutel” 2025). In contrast, pre-treatment methods for homogeneous EPS, such as compaction, are well-developed, significantly improving handling and transport efficiency (Schleier et al. 2022).

Figure 12: EPS waste processing flow-chart

### Ceramics and tiles

Waste specification: This category includes various types of ceramic tiles such as polished tiles, glazed tiles, rectified tiles, and vitrified tiles. These materials are durable and widely used in construction for flooring, walls, and decorative applications. However, they frequently end up mixed with other CDW during renovation, demolition, or construction processes.

Common contaminants: Ceramic and tile waste often becomes cross-contaminated with other types of CDW. Additionally, they contain adhesives, grout, and other bonding agents.

Waste management pathways: Ceramic and tile waste is typically managed as part of low-quality recycled aggregates or used for backfilling.

High-value applications: Ceramic tiles can be recycled as raw materials for new ceramic products or other construction materials (Fu and Lee 2024) or processed into supplementary cementitious materials for use in concrete production (de Matos et al. 2021). However, these methods are not adopted on a commercial scale (J et al. 2024).

Barriers to high-value applications: Ceramic tiles are often part of mixed CDW, making separation and recycling difficult. Their diverse properties limit suitability for advanced processing. Additionally, there is a lack of dedicated infrastructure for recycling ceramic tiles (Cerame-Unie 2020).

Common on-site collection and separation methods: Tiles are typically mixed with other mineral-based CDW.

Improved on-site collection and separation methods: Ceramic tiles should be removed separately through selective demolition and collected in dedicated containers to maintain material purity.

Off-site sorting and treatment: Off-site sorting is not commonly performed to obtain a high-purity single tile stream. Tiles can be separated from other mineral waste using inclined belt separators, which separate flat fragments (glass, tiles) from more cubic or round ones (Müller et al. 2022b). With advanced off-site sorting (Trotta et al. 2021) demonstrated the recognition of tiles, and if there is demand from the recycling industry, it is possible to separate this stream for further processing.

### Other materials in CDW

Plastic

Other plastic waste from CDW consists of various polymer types, including PVC, HDPE, PP, LDPE, PS, and others. The most preferred method for utilization is close-loop recycling, either through mechanical or chemical recycling. Due to the widespread use of plastics across different industries, the plastic recycling industry is well-established throughout Europe. If CDW plastics are sorted according to their resin type, they can be effectively recycled. However, due to small volumes and the difficulty of identification, CDW plastics are rarely sorted by resin type.

Additional recovery options for plastic waste processing include pyrolysis or incineration, although burning certain plastic types, such as PVC, poses challenges due to chlorine content. Other possibilities include the use of plastics in various composite materials, but this approach significantly complicates their recyclability at the end of life.

Whenever possible, plastics should be sorted on-site, at least for those that are easily recognizable and occur in large quantities, such as PVC window frames. However, off-site sorting of plastics is becoming increasingly feasible thanks to advancements in sensor-based sorting technologies, which enable the identification of plastics by resin type. This technological progress opens up new opportunities for higher recycling rates.

At a regional level, a potential strategy could involve separating and concentrating the mixed rigid plastic waste stream, which could then be sent to specialized recycling facilities for further processing and sorting.

Metals

The sorting and recycling of metals are well-established processes across the European Union, primarily due to the high recognizability of CDW metal waste and its economic value as a secondary material. Additionally, metals are easily separable in recycling processes using magnetic separation for magnetic metals and eddy current separation for non-ferrous metals.

## Mixed CDW

Similar to the management of single waste streams, it is essential to segregate waste containing hazardous substances, such as PAHs, POPs, heavy metals or asbestos, at the point of waste generation based on safe management on (pre-demolition audit, analyses, material or product knowledge). Early identification and separation of such materials are crucial in preventing the contamination of secondary raw materials, ensuring that the recovered fractions meet the necessary quality standards for effective recycling or recovery processes.

Mixed CDW can be categorized based on its source. Waste from new construction mainly consists of new materials, offcuts, a higher share of wood and plastics, and packaging materials. Waste from renovations includes partition walls and non-load-bearing structures, while demolition waste contains load-bearing structures. The composition of CDW varies depending on climate, insulation requirements, architectural styles, and material availability. It varies across Europe, sometimes even within regions. Seasonal changes also influence the volume and composition of CDW.

Non-hazardous mixed CDW consists of various waste fractions, generally divided into three categories: (1) mixed inert CDW; (2) mixed non-inert CDW and (3) mixed non-inert and inert CDW.

Without sorting, mixed CDW is either landfilled or, in the case of mixed inert waste, used as backfilling material. The choice of sorting technology and target output determines further processing.

Mixed CDW treatment focuses on achieving recovery rate targets by sorting waste fractions to reach the required purity and usability for recovery. Valuable and easily recyclable materials such as iron and other metals are separated, while other fractions, such as mineral waste, are classified based on quality for recycling or are kept as waste suitable for backfilling. Wood and plastics are sorted for energy recovery. The inert fraction used for backfill or low-grade recovery, such as recycled aggregates in road sub-bases or subgrades of infrastructural structures, has lower quality requirements. Non-inert waste intended for energy recovery is either sent to incineration facilities as waste or processed into solid recovered fuel. The European standard EN ISO 21640 defines quality specifications for solid recovered fuel based on properties such as calorific value, chlorine content, and mercury content, with Class 1 representing the highest quality and Class 5 the minimum acceptable standard.

For high-value recycling, output from sorting or treatment plants must meet more strict criteria, either defined by standards such as EN 12620 for aggregates in concrete or by specific waste/material acceptance criteria set by recycling facilities or product manufacturers. These criteria are not harmonized and vary between facilities depending on the products and recycling technologies used.

A sorting facility handling mixed waste and aiming to increase the recycling rate must focus on producing high-quality materials suitable for industrial use. To achieve this, it must define output criteria based on the requirements of industrial partners.

### Concept of a recycling plant for mixed CDW

This subchapter presents an example of a recycling plant designed to process mixed CDW using advanced sorting technologies, including robotic sorting and air ejector sensor sorting. For an efficient design, it is essential to understand the composition of the input waste stream and the average distribution of different waste fractions. Based on an analysis of waste streams and industry demand for recycled raw materials, key recyclable materials can be identified for extraction from mixed CDW.

The concept of recycling plant is designed to handle waste that consists primarily of inert materials such as concrete, bricks, and ceramics, while also containing high levels of impurities, including wood, plastics, and metals. The recyclable output material streams include mixed RA in various fractions, RCA, wood, mixed rigid plastics for further processing, and metals.

A key difference between robotic sorting and manual sorting lies in the preparation of waste for sorting. In conventional manual sorting systems, primary crushing is required to reduce the material size to a level manageable for human workers. In contrast, robotic sorting can handle larger waste pieces (up to 40 kg), reducing or even eliminating the need for initial crushing. Instead, the material is pre-sorted, with large pieces either sorted separately or selectively broken down into smaller fragments. When primary crushing is used in robotic sorting lines, the maximum particle size is adjusted to fit the robotic system’s capabilities. The throughput of robotic sorting is limited by the number of picks per hour, making it more efficient when handling fewer but larger waste pieces.

The recycling process begins with an initial inspection of the incoming waste, during which the material is evaluated to determine whether it meets environmental and technological requirements. This assessment ensures that the waste does not contain hazardous substances, complies with regulatory standards, and is suitable for further processing. Additionally, the waste documentation is reviewed to verify its origin and composition. Based on this evaluation, the waste is either directed straight into the main processing line, diverted to pre-treatment for further preparation before sorting, or rejected if it fails to meet the necessary criteria. The pre-treatment phase involves breaking down large waste fragments into smaller pieces using hydraulic hammers or other mechanical equipment. This step ensures that oversized materials are reduced to a manageable size for subsequent processing. Additionally, large impurities may be removed at this stage.

Once pre-treated, the waste is temporarily stored in the intermediate storage area, where it is prepared for entry into the main processing line. Here, additional sorting may take place, removing oversized or contaminant materials that require further fragmentation.

The waste then moves through grates, which help to screen out larger pieces before reaching the primary crusher. The primary crusher breaks down fragments into a manageable size. The maximum allowable particle size at this stage is determined by the actuator specifications of the robotic sorting system downstream.

After crushing, an overbelt magnet is used to extract ferrous metals from the material stream, ensuring that metallic contaminants do not interfere with subsequent processing steps.

Next, the material undergoes screening, where it is classified into two fractions:

- <45 mm fraction, which primarily consists of soil, mortar residues, sand, and small impurities such as wood and plastics. This fraction can undergo further processing or be used for low-value applications.

- >45 mm fraction, which includes concrete, ceramics, and non-inert impurities such as wood, plastics, and metals.

The >45 mm fraction is directed to air separation, where lightweight materials such as plastic foils or EPS are removed. It is then directed into a bunker with controlled discharge, which ensures a continuous and regulated material flow into the robotic sorting system. This bunker allows for multi-shift operation of the robotic sorting line, maximizing its potential capacity. In contrast, earlier processing steps, such as waste intake and pre-treatment, typically operate in a single-shift mode.

From the bunker, the material is conveyed through a vibratory feeder, which evenly spreads and isolates individual fragments before robotic sorting. This process prevents material from piling up, ensuring that each piece is clearly distinguishable for optimal robotic processing. The robotic sorting line then identifies and separates recyclable materials or contaminants from the waste stream. The system detects and removes metals, wood, rigid plastics, other non-inert contaminants such as various plastic materials, gypsum, and unsuitable inert fractions, depending on the intended application of the recycled product (e.g. AAC). The sorted fractions are then directed into designated chutes, each leading to separate containers.

Following robotic sorting, the material may either pass through a storage bunker or proceed directly to secondary crushing, typically performed by an impact crusher.

The impact crusher processes the purified material, breaking it down further. It then passes through another magnetic separator, which extracts any remaining iron, such as reinforcement steel released from crushed concrete. The processed material is then screened into different size fractions, such as 0-4mm, 4-16 mm, 16-32 mm, and >32 mm. The fraction exceeding 32 mm is recirculated back into the secondary crusher for further reduction.

The fine fraction of 0-4 mm can be used for backfilling purposes. The 4-16 mm and 16-32 mm fractions, which contain concrete, ceramics, and other inert materials that were not sorted through robotic processing, can either be utilized as mixed RA or further refined to obtain high-quality RCA.

To achieve a higher purity of RCA, advanced sorting technologies such as sensor-based sorting with air ejectors are employed. In this process, concrete fragments are identified and selectively removed from the mixed material stream using compressed air. This method ensures the separation of high-purity RCA from other inert materials. The remaining material can still be used as RA or subjected to further sorting using the same technology. For example, clay brick fragments can be separated in a similar manner, allowing them to be repurposed for more suitable applications.

Figure 14 Concept of a recycling plant for mixed CDW

Further treatment of pre-screened 0-45 mm material may involve additional screening to refine the fraction and improve material quality. Various direct sorting methods can also be applied, including wet sorting, which uses water to separate materials based on density, and air classifiers, which use airflow to separate lighter particles from heavier ones. These methods help remove remaining impurities and enhance the usability of the material for further processing or specific applications.

# Conclusions

This report examined the collection of CDW. It focused on the regulatory framework for on-site sorting, on sorting practices at collection and recycling centres, and on challenges related to the processing of CDW.

Information was gathered about the obligation for pre-sorting CDW at construction sites in different EU MS. Besides the general requirement to separate hazardous waste and not mix it with other waste, there are many differences in national rules for on-site sorting. Most countries require on-site sorting, but exceptions may apply based on factors such as the volume or weight of the waste or approved treatment methods. The range of materials that must be sorted also differs: some MS have detailed lists, while others have more general categories. Off-site sorting is allowed in many countries but often depends on the specific conditions of the construction site.

Even though regulations encourage on-site sorting, CDW is often mixed in practice. For example, monitoring at the ENVISAN-GEM recycling centre in the Czech Republic shows that actual practice does not always follow legal requirements. This highlights the ongoing challenge of aligning policy goals with real-world waste management and shows the importance of improving off-site sorting technologies and processes.

The main obstacles for better on-site sorting are economic factors and time constraints. Handling separated waste, especially in small quantities, can be more expensive than dealing with mixed waste. Poor management of construction and demolition waste and a lack of knowledge about waste handling also contribute to the problem. To improve this situation, it is important to introduce measures such as pre-demolition audits, waste management plans, and better organization of waste management on construction and demolition sites.

This also includes factors such as the language barrier among manual workers. Many do not speak the local language as their first language, making clear instructions on construction sites essential. Additionally, well-defined waste acceptance criteria from treatment facilities are needed, along with transparent communication. Unclear rules and poor communication among different stakeholders frequently lead to inefficient waste handling.

The report also described various collection methods and the role of the IoT in improving sorting and logistics. Different collection tools are chosen based on project needs and site conditions. Containers are commonly used for handling and transporting CDW. Good practice includes allocating proper space for waste and clearly labelling all collection containers. IoT features in containers—such as QR codes, RFID tags, GPS tracking, fill-level monitoring, and object classification—can help optimize logistics and sorting on-site.

Development and field testing of a smart device for CDW container monitoring, carried out by the Czech Technical University (CTU) together with ENVISAN-GEM, showed the potential of real-time waste tracking. Early tests revealed technical challenges, such as low efficiency of solar panels and sensor limitations, which need further improvements. Despite these drawbacks, digital monitoring already supports manual supervision and waste identification, underlining the importance of technological progress in modern waste management.

Several potential treatments for the main fractions of CDW were identified. Common handling methods and typical contaminants can limit high-value recycling. Although recycling options exist for most construction and demolition waste, contamination, market distrust, and economic barriers hold back higher recycling rates. Recycled raw materials must compete in price with virgin materials. Technological advances in sorting and classification can increase recycling rates. A concept for a recycling plant for mixed CDW has been developed, showing how advanced sorting can produce high-quality recycled materials. Robotic sorting can raise efficiency, accuracy, and flexibility when processing various waste fractions.

Sensor-based sorters not only help with the sorting process but also collect valuable data. Such data can improve the performance of the sorter itself, as well as support optimization of the entire process and value chain. Data transparency is crucial for shifting from a linear to a circular economy, and digitalization helps make data accessible.

A key requirement for effective CDW management is stable and clear legislation. This legislation must keep pace with new technology and give investors and recycling centre operators the confidence to plan for the long term. Pre-demolition audits are in the overall process also essential. They help identify the quality and any dangerous components of the waste before demolition, preventing contamination of large volumes of recyclables. Too much regulation may raise costs and lead to rule-breaking, so a balanced approach is needed. Monitoring waste flows and having transparent documentation are also important for building trust in recycled materials. Modern digital tools and technologies can speed up and improve recycling, but they must be recognized in legislation. If regulations are too rigid, innovation may be slowed and private investors discouraged.

Clear, unified European End of Waste criteria, along with harmonized standards for recyclable materials, would be helpful. These steps would support the use of recycled materials, simplify cross-border transport, and increase trust in recycled materials as a replacement for virgin raw materials.

At the same time, it is important to balance chemical regulations with circular economy goals. Fair thresholds for chemical substances (e.g. PAHs, heavy metals, POPs) and manageable compliance rules can ensure both safety and profitability in CDW recycling.

In summary, CDW policy should promote market demand for recycled materials, ensure effective supervision with reasonable administrative requirements, and help turn circular principles into actual practices.

References

Ambrós, Weslei Monteiro, Carlos Hoffmann Sampaio, Bogdan Grigore Cazacliu, Gerson Luis Miltzarek, and Leonardo R. Miranda. 2017. “Usage of Air Jigging for Multi-Component Separation of Construction and Demolition Waste.” Waste Management 60 (February):75–83. https://doi.org/10.1016/J.WASMAN.2016.11.029.

Barbudo, Auxi, Jesús Ayuso, Angélica Lozano, Manuel Cabrera, and Antonio López-Uceda. 2020. “Recommendations for the Management of Construction and Demolition Waste in Treatment Plants.” Environmental Science and Pollution Research 27 (1): 125–32. https://doi.org/10.1007/s11356-019-05578-0.

Bilitewski, Bernd. 2010. “Mechanical Treatment: Unit Processes.” In Solid Waste Technology & Management, 319–48. Chichester, UK: John Wiley & Sons, Ltd.

https://doi.org/10.1002/9780470666883.ch24.

Caro, D., C. Lodato, A. Damgaard, J. Cristóbal, G. Foster, F. Flachenecker, and D. Tonini. 2024. “Environmental and Socio-Economic Effects of Construction and Demolition Waste Recycling in the European Union.” Science of the Total Environment 908 (January). https://doi.org/10.1016/j.scitotenv.2023.168295.

Centre, European Commission: Joint Research, A Damgaard, C Lodato, S Butera, M Kamps, L Corbin, T Astrup, T Fruergaard, and D Tonini. 2022. Background Data Collection and Life Cycle Assessment for Construction and Demolition Waste (CDW) Management. Publications Office of the European Union. https://doi.org/doi/10.2760/772724.

Cerame-Unie. 2020. “Circular Economy in the Ceramic Industry.” https://cerameunie.eu/topics/construction-sustainability/circular-economy/circular-economy-in-the-ceramic-industry-position-paper/.

Chimwani, Ngonidzashe, and Murray M. Bwalya. 2021. “Milling Studies in an Impact Crusher I: Kinetics Modelling Based on Population Balance Modelling.” Minerals 2021, Vol. 11, Page 470 11 (5): 470. https://doi.org/10.3390/MIN11050470.

Coelho, A., and J. De Brito. 2013. “Preparation of Concrete Aggregates from Construction and Demolition Waste (CDW).” In Handbook of Recycled Concrete and Demolition Waste, 210–45. Elsevier Inc. https://doi.org/10.1533/9780857096906.2.210.

Davies, Ron. 2015. “Industry 4.0 Digitalisation for Productivity and Growth.” European Parliament PE 568.337, Eur. Parliamentary Res. Service, Vol. 1.

https://www.europarl.europa.eu/RegData/etudes/BRIE/2015/568337/EPRS_BRI(2015)568337_EN.pdf.

Dodampegama, Shanuka, Lei Hou, Ehsan Asadi, Guomin Zhang, and Sujeeva Setunge. 2024. “Revolutionizing Construction and Demolition Waste Sorting: Insights from Artificial Intelligence and Robotic Applications.” Resources, Conservation and Recycling. Elsevier B.V. https://doi.org/10.1016/j.resconrec.2023.107375.

“EPSolutel.” 2025. 2025. https://epsolutely.at/.

EUMEPS. 2021. “THE EPS-INDUSTRY’S JOURNEY TOWARDS CIRCULARITY Progress Report.” https://eumeps.eu/eumeps-newsroom/publications/conversio-study-on-the-eps-industrys-industry-to-sustainability.

European Commission. 2020. “A New Circular Economy Action Plan for a Cleaner and More Competitive Europe.” 2020.

https://eur-lex.europa.eu/legal-content/EN/TXT/?qid=1583933814386&uri=COM:2020:98:FIN.

European Parliament and of the Council. 2008. DIRECTIVE 2008/98/EC. European Parliament and of the Council.

https://eur-lex.europa.eu/legal-content/EN/TXT/HTML/?uri=CELEX:32008L0098.

Eurostat. 2022. “Waste Statistics - Statistics Explained.” 2022.

https://ec.europa.eu/eurostat/statistics-explained/index.php?title=Waste_statistics#Total_waste_generation.

“Fast Picker | ZenRobotics.” 2025. 2025. https://www.terex.com/zenrobotics/robots/fast-picker.

“Gamle Mursten.” 2025. 2025. https://gamlemursten.dk/.

Fu, Shuting, and Jechan Lee. 2024. “Recycling of Ceramic Tile Waste into Construction Materials.” Developments in the Built Environment 18 (April):100431. https://doi.org/10.1016/J.DIBE.2024.100431.

“GAINnext: Waste Sorting Machine | TOMRA.” 2025. 2025. https://www.tomra.com/en/waste-metal-recycling/products/machines/gainnext.

“Heavy Picker | ZenRobotics.” 2025. 2025. https://www.terex.com/zenrobotics/robots/heavy-picker.

“PSLoop.” 2025. 2025. https://www.psloop.eu/.

“Hrubotřídiče.” 2024. 2024. https://www.resta.cz/hrubotridice-i.

Iyiola, Comfort Olubukola, Winston Shakantu, and Emmanuel Itodo Daniel. 2024. “Digital Technologies for Promoting Construction and Demolition Waste Management: A Systematic Review.” Buildings. Multidisciplinary Digital Publishing Institute (MDPI). https://doi.org/10.3390/buildings14103234.

J, Cristobal Garcia, Caro D, Foster G, Pristera G, Gallo F, and Tonini D. 2024. “Techno-Economic and Environmental Assessment of Construction and Demolition Waste Management in the European Union,” no. KJ-NA-31-784-EN-N (online). https://doi.org/10.2760/721895 (online).

Kiyokawa, Takuya, Jun Takamatsu, and Shigeki Koyanaka. 2024. “Challenges for Future Robotic Sorters of Mixed Industrial Waste: A Survey.” IEEE Transactions on Automation Science and Engineering 21 (1): 1023–40. https://doi.org/10.1109/TASE.2022.3221969.

Ku, Yuedong, Jianhong Yang, Huaiying Fang, Wen Xiao, and Jiangteng Zhuang. 2021. “Deep Learning of Grasping Detection for a Robot Used in Sorting Construction and Demolition Waste.” Journal of Material Cycles and Waste Management 23 (1): 84–95. https://doi.org/10.1007/s10163-020-01098-z.

Laudon GmbH & Co. KG. 2024a. “P3 - Standard.” 2024. https://www.laudon.de/unternehmensbereiche/containerbau/absetzmulden/p3-standard/.

———. 2024b. “Spantenabroller.” 2024.

https://www.laudon.de/unternehmensbereiche/containerbau/abrollcontainer/spantenabroller/.

Lindqvist, M., and C.M. Evertsson. 2006. “Development of Wear Model for Cone Crushers.” Wear 261 (3–4): 435–42. https://doi.org/10.1016/j.wear.2005.12.010.

Matos, Paulo R. de, Rafael D. Sakata, Lucas Onghero, Vinicius G. Uliano, Jorge de Brito, Carlos E.M. Campos, and Philippe J.P. Gleize. 2021. “Utilization of Ceramic Tile Demolition Waste as Supplementary Cementitious Material: An Early-Age Investigation.” Journal of Building Engineering 38 (June):102187. https://doi.org/10.1016/J.JOBE.2021.102187.

Müller, Anette, and Isabel Martins. 2022a. “Advanced Recycling.” Recycling of Building Materials, 309–43. https://doi.org/10.1007/978-3-658-34609-6_10.

———. 2022b. “Processing of Construction and Demolition Waste.” In Recycling of Building Materials: Generation - Processing - Utilization, edited by Anette Müller and Isabel Martins, 65–126. Wiesbaden: Springer Fachmedien Wiesbaden. https://doi.org/10.1007/978-3-658-34609-6_4.

Olivier, Laurentz E., Michael G. Maritz, and Ian K. Craig. 2020. “Estimating Ore Particle Size Distribution Using a Deep Convolutional Neural Network.” IFAC-PapersOnLine 53 (2): 12038–43. https://doi.org/10.1016/J.IFACOL.2020.12.740.

Pacheco, João, and Jorge de Brito. 2021. “Recycled Aggregates Produced from Construction and Demolition Waste for Structural Concrete: Constituents, Properties and Production.” Materials 14 (19). https://doi.org/10.3390/ma14195748.

Rhoad, Ty. 2023. “Understanding How AI Can Help the Sortation Process.” December 2023. https://resource-recycling.com/recycling/2024/02/19/understanding-how-ai-can-help-the-sortation-process/.

Robayo-Salazar, Rafael Andrés, Jhonathan Fernando Rivera, and Ruby Mejía de Gutiérrez. 2017. “Alkali-Activated Building Materials Made with Recycled Construction and Demolition Wastes.” Construction and Building Materials 149 (September):130–38.

https://doi.org/10.1016/J.CONBUILDMAT.2017.05.122.

“Rockcycle®.” 2025. 2025.

https://www.rockwool.com/group/about-us/sustainability/environment/rockcycle/.

Santos, C., A. Mehrsai, A. C. Barros, M. Araújo, and E. Ares. 2017. “Towards Industry 4.0: An Overview of European Strategic Roadmaps.” In Procedia Manufacturing, 13:972–79. Elsevier B.V. https://doi.org/10.1016/j.promfg.2017.09.093.

Schackow, A., D. Stringari, L. Senff, S. L. Correia, and A. M. Segadães. 2015. “Influence of Fired Clay Brick Waste Additions on the Durability of Mortars.” Cement and Concrete Composites 62 (September):82–89. https://doi.org/10.1016/J.CEMCONCOMP.2015.04.019.

Schleier, Julia, Martin Simons, Kathrin Greiff, and Grit Walther. 2022. “End-of-Life Treatment of EPS-Based Building Insulation Material – An Estimation of Future Waste and Review of Treatment Options.” Resources, Conservation and Recycling 187 (December):106603. https://doi.org/10.1016/J.RESCONREC.2022.106603.

“Sensorita.” 2025. 2025. https://www.sensorita.com/.

Silva, R. V., J. de Brito, and R. K. Dhir. 2017. “Availability and Processing of Recycled Aggregates within the Construction and Demolition Supply Chain: A Review.” Journal of Cleaner Production. Elsevier Ltd. https://doi.org/10.1016/j.jclepro.2016.12.070.

Symonds Group, Ltd. 1999. “Report to DGXI, European Commission CONSTRUCTION AND DEMOLITION WASTE MANAGEMENT PRACTICES, AND THEIR ECONOMIC IMPACTS Final Report.”

Trotta, Oriana, Giuseppe Bonifazi, Giuseppe Capobianco, and Silvia Serranti. 2021. “Recycling-Oriented Characterization of Post-Earthquake Building Waste by Different Sensing Techniques.” Journal of Imaging 2021, Vol. 7, Page 182 7 (9): 182.

https://doi.org/10.3390/JIMAGING7090182.

Väntsi, Olli, and Timo Kärki. 2014. “Mineral Wool Waste in Europe: A Review of Mineral Wool Waste Quantity, Quality, and Current Recycling Methods.” Journal of Material Cycles and Waste Management 16 (1): 62–72. https://doi.org/10.1007/S10163-013-0170-5/METRICS.

Vatskicheva, Malina, and Irena Grigorova. 2017. “STUDY OF TWO-SHAFT SHREDDER FOR CRUSHING OF CONCRETE, RUBBER, PLASTIC AND WOOD.” Materials, Methods & Technologies ISSN. Vol. 11. www.scientific-publications.net.