HARDWARE OF FPGA   BASED CONTROL OF BLDC DRIVE FOR  MINIMUM TORQUE RIPPLE

# Neethu Alexander*1, Unnikrishnan2 L

# *1P.G  Scholar in Industrial Drives, Department of Electrical And Electronics engineering, RSET Kochi

# 2Assistant Professor, Department of Electrical And Electronics engineering, RSET Kochi

# DOI: 10.5281/zenodo.157170

ABSTRACT

Brushless DC motors have a trapezoidal back emf waveform and are fed with rectangular stator currents. On these conditions the torque produced is,theoretically, constant. However, in practice, torque ripple may exist, due to the machine itself but also to the feeding system. The causes of ripple coming from the machine are cogging torque and emf waveform imperfections. In this paper we discusses on the torque ripples minimisation by average torque control technique. Fpga  is used for switching the Bldc motor and control. Coding on FPGA is done using the software Xilinx. The simulation is done using MATLAB. This method  is not designed to reduce the torque ripple caused by high speed switching.

INTRODUCTION

BLDC motor  are used in common now a  days because of features like they  have high efficiency, low maintenance and long life, low noise, control simplicity, low weight, and compact construction. That’s why the Bldc motor has become a very popular and viable product in the market. Actually  the Bldc motor has more advantages compared with other types of AC motors in the market [1].Fig. 1 shows the ideal and actual measured three-phase back EMF. The electromagnetic torque may be written as:

where ωm is the mechanical angular velocity, ea, eb, ec the three phase back EMFs and ia, ib, ic the three phase currents.

FIG 1: a) Ideal back EMF

Fig 1:b :Actual measured back emf

According to (eqn 1), with ideal rectangular current, the electromagnetic torque is constant if the machine has an ideal trapezoidal back EMF, as shown in Fig. 1 (a). However, for the machine with non-ideal trapezoidal back EMF, such as the waveforms shown in Fig. 1 (b), the electromagnetic torque is not constant when the phase-current is still the rectangular waveform. Fig. 1(b) shows the torque waveform and the frequency of the torque ripple is six times the fundamental component of the back EMF.

WORKING OF PROPOSED  ALGORITHM

The energy flowing into the system in one switching cycle dWe is equal to the summation of energy change in the air-gap dWm, the mechanical output energy dWmech, and the system losses dWloss:

dWe = dWm + dWmech + dWloss

Fig 2: MATLAB simulation of the proposed system

dWe= dWmech + dWloss= 1η  dWmech

where η is the efficiency of the system. The relationship between the input energy  and the average torque Tav in one switching cycle can be expressed as:

dWe=1η  dWmech  =Δθη Tav =kTav

Δθ is the mechanical angular position change in one switching cycle and k the coefficient defined as Δθ/η which is constant under the steady state. The electric machine torque variation will reflect as the DC input energy fluctuation. The average torque Tav may be computed by using the energy flowing into the system for each control cycle. The DC bus voltage and current are monitored by sensors. The input energy in one control cycle is calculated by integrating the product of the DC bus voltage and current.

FPGA

A field programmable gate array (FPGA) is a general-purpose integrated circuit that is programmed by the designer rather than the device manufacturer .FPGAs are high performance data processing devices. The raw memory bandwidth of a large FPGA running at a clock rate of 150 MHz can be hundreds of terabytes per second. An FPGA can be reprogrammed, even after it has been deployed into a system. An FPGA is programmed by downloading a configuration program called a bitstream into static on-chip random-access memory. Much like the object code for a microprocessor, this bitstream is the product of compilation tools that translate the high-level abstractions produced by a designer into something equivalent but low-level and executable. Xilinx System Generator is a powerful tool for compiling an FPGA program from a high-level Simulink model based design .

Fig 3: Coding of FPGA

STEP BY STEP PROCESS OF PROGRAMMING IN FPGA

  - Architectural Design

  - Choice of language (Verilog,VHDL)

  - Editing Programes.

  - Compiling Programes.

  - Synthesizing programs(.EDIF)

  - Placing and routing programs (.VO, .SDF, .TTF)

  - Loading programs to FPGA

  - Debugging FPGA programs

  - Documenting programs

  - Delivering programs

Fig 4:Hardware Schemat

Fig 5:PCB Model

Fig 6:Electromagnetic torque of the proposed system

Fig 7:Electromagnetic torque from Conventional closed loop current control

Using the traditional control, the current is controlled to follow a square reference waveform and the torque shows the arc-shape ripple. The one-cycle torque control method ensures that the average torque in each switching cycle follows the reference. Thus to the controller can make sure the product of current and back EMF is constant. Arc shaped ripples are reduced. The current is not rectangular waveform and the torque is flat without the arc-shaped ripples.The proposed average torque control method is not designed to reduce the torque ripple caused by high speed switching.

CONCLUSION

# The torque ripple spectrum analysis demonstrates the one-cycle average torque control algorithm is able to reduce the torque harmonic components by more than 70%. The  approximate cost of hardware is  twelve thousand rupees including the motor. This method does not require the information of back EMF and accurate rotor position. The one cycle torque control method ensures that the average torque in each switching cycle follows the reference. Thus to the controller can make sure the product of current and back EMF is constant. arc shaped ripples are reduced.

# ACKNOWLEDGEMENT

I would like to thank Rajagiri School of Engineering ,Kochi for giving me the opportunity to complete the project successfully

REFERENCES

- J.F. Gieras and SpringerLink. (2008, Advancements in Electric Machines. Available: http://dx.doi.org/10.1007/978-1-4020-9007-3T.

- M. Jahns and W. L. Soong, "Pulsating torque minimization techniques for permanent  magnet AC motor drives-a review", IEEE Trans. On Industrial Electronics, vol. 43, 321-330, 1996.

- B. Singh, "Recent advances in permanent magnet brushless DC motors" Sadhana - Academy Proceedings in Engineering Sciences, vol. 22, 837-853, 1997.

- J. Holtz and L. Springob, "Identification and compensation of torque ripple in high-precision permanent magnet motor drives", IEEE Trans. On Industrial Electronics, vol. 43, 309-320, 1996