RFC XXXX — RS485MM: A Distributed Multi-Master Extension for RS485 with Token Ring Arbitration, Multi-Slot Join, and Modbus RTU Backward Compatibility

Network Working Group
Request for Comments: XXXX
Category: Informational
Author: Cristian Lorenzetto
Date: 2025

Abstract

This document specifies RS485MM, a distributed multi-master protocol operating on top of the RS485 physical layer. RS485MM introduces deterministic token-based arbitration, multi-slot join procedures, dynamic priorities, urgent preemption, and distributed topology consensus, while maintaining full backward compatibility with Modbus RTU slave devices.

The protocol enables RS485 networks to operate in a collision-free multi-master mode using existing hardware, making it suitable for residential HVAC systems, home automation, and retrofits.

Status of This Memo

This memo provides information for the community. It does not specify an Internet standard of any kind. Distribution of this memo is unlimited.

RS485MM is published as defensive prior art to prevent proprietary patent restrictions on multi-master RS485 protocols.

Requirements Language

The key words MUST, MUST NOT, REQUIRED, SHALL, SHALL NOT,
SHOULD, SHOULD NOT, RECOMMENDED, MAY, and OPTIONAL
in this document are to be interpreted as described in RFC 2119.

1. Introduction

RS485 is a widely used differential serial bus in industrial and residential environments. The majority of residential HVAC and home-automation systems deploy RS485 with Modbus RTU due to its low cost, robustness, and interoperability.

However, traditional RS485 deployments restrict communication to a single master, which is incompatible with modern distributed control requirements.

RS485MM extends RS485 to support multiple masters in a deterministic, collision-free manner. The protocol requires no hardware changes to existing Modbus RTU slaves.

2. Motivation

2.1 Prevalence of RS485 in Residential Systems

RS485 is commonly found in:

- dehumidifiers

- heat pumps

- ventilation systems (VMC)

- fan-coils

- thermostatic valves

- circulation pumps

- energy meters

- sensor networks

- photovoltaic inverters

These devices communicate over Modbus RTU due to:

- extremely low transceiver cost,

- ubiquitous support,

- existing cabling in buildings,

- tolerance for non-ideal wiring (e.g., star-like layouts).

2.2 Limitations of Single-Master RS485

Increasingly complex residential systems require:

- room-level controllers,

- distributed HVAC logic,

- redundant supervision nodes,

- real-time reactions to humidity, dew point, and pressure events,

- cooperative ventilation and air handling logic.

A single-master RS485 network cannot satisfy these needs.

2.3 Why CAN Is Not an Option

Although CAN is a native multi-master bus, it is rarely used in residential environments due to:

- lack of device vendor support,

- incompatible wiring expectations,

- cost structure,

- absence of legacy installed base,

- absence of CAN-ready residential conduits.

Replacing RS485 is economically impractical.

RS485MM solves this by extending RS485 to behave as a multi-master bus while preserving compatibility.

3. Architectural Overview

An RS485MM network consists of:

- one or more RS485MM masters,

- zero or more Modbus RTU slaves,

- a shared RS485 physical medium.

RS485 BUS

│

├── Master M1 (RS485MM)

├── Master M2 (RS485MM)

├── Master M3 (RS485MM)

│

├── Slave S1 (Modbus)

├── Slave S2 (Modbus)

└── Slave S3 (Modbus)

Modbus RTU slaves MUST NOT interpret RS485MM control frames.

4. Protocol Overview

RS485MM introduces:

- Distributed Token Ring Arbitration
Ensures only one master transmits Modbus operations at a time.

- Preemptive Window
Allows urgent nodes to request priority access.

- Multi-Slot Join Procedure
Enables new master nodes to join the network without collisions.

- Distributed Consensus via Table Intersection
Ensures all masters share a consistent view of network membership.

- Automatic Pruning (Autopruning)
Removes unresponsive masters consistently across the network.

- Backward Compatibility
All RS485MM frames are broadcast and ignored by Modbus slaves.

5. Token Ring Arbitration

5.1 Token Frame

[FF][80][NextMaster][CRC]

- The frame SHALL be sent as broadcast (0xFF).

- Only the designated NextMaster SHALL take control of the bus.

- All other masters MUST remain silent.

5.2 Turn Execution

Upon receiving the token, a master MUST:

- Transmit pending Modbus RTU requests.

- Wait for Modbus slave responses.

- NOT perform heavy computations during the turn.

- Enter the Preemptive Window.

- Forward the token according to the selection rules.

6. Preemptive Window

A fixed idle interval of 1–2 ms SHALL occur after each token turn.

During this interval, urgent masters MAY transmit:

[FF][95][Address][CRC]

Masters SHALL record urgent addresses in UrgentSet.
Urgent masters SHALL be served before non-urgent masters.

7. Master Selection and Priority Rules

The next token destination MUST be selected in the following order:

- Highest-priority node in UrgentSet.

- Highest-priority non-urgent node.

- Lowest address in case of ties.

A master MAY explicitly skip its turn.

8. Multi-Slot Join Procedure

Joining occurs after a full token cycle.

8.1 Join Slots

The Join Window is composed of multiple micro-slots of fixed duration:

JOIN-SLOT1 (1 ms)

JOIN-SLOT2 (1 ms)

...

JOIN-SLOTN

The window SHALL terminate upon detection of a silent slot.

8.2 Join Frame

[FF][90][Address][PriorityHint][CRC]

If multiple nodes collide, CRC failure SHALL cause the slot to be discarded.

9. OK Phase (Table Exchange)

After join completion, each master SHALL broadcast:

[FF][91][MasterTable][CRC]

All masters SHALL store the tables received from all others.

10. Distributed Consensus

At the end of the OK Phase, each master SHALL compute:

FinalTable = INTERSECTION( RemoteTables[] )

Only masters appearing in all tables SHALL remain valid.

This yields deterministic global agreement.

11. Autopruning

If a master fails to respond to its token assignment:

SuspectCount[M] += 1

If SuspectCount[M] exceeds threshold K (RECOMMENDED value: 2 or 3):

MasterTable.remove(M)

All masters MUST remove the node.

12. Backward Compatibility with Modbus RTU

RS485MM frames:

- SHALL use broadcast addressing, and

- MUST NOT match any valid Modbus RTU slave address.

Thus Modbus RTU slaves SHALL ignore RS485MM frames.

Modbus RTU traffic remains unmodified and fully functional.

13. Frame Summary

| Purpose | Opcode | Format |
| --- | --- | --- |
| Token | 0x80 | FF 80 NEXT CRC |
| Join | 0x90 | FF 90 ADDR PRIO CRC |
| Preemptive | 0x95 | FF 95 ADDR CRC |
| OK/Sync | 0x91 | FF 91 TABLE CRC |

14. Security Considerations

RS485MM assumes a trusted physical environment.
No authentication is defined.
Physical access to the bus implies control of the network.

Implementers MAY add cryptographic signing as an additional layer.

15. IANA Considerations

This document does not require any IANA actions.

16. References

RFC 2119 – Key words for use in RFCs to Indicate Requirement Levels.

Modbus Application Protocol v1.1b3 – Modbus Organization.

Author’s Address

Cristian Lorenzetto
Email: cristian.lorenzetto@gmail.com