Supplementary Note 1

Although NaH2PO4 and Na2SO4 were added to replace the PO4 and SO4 ions lost as a result of depletion of KH2PO4 and MgSO4, the experimentally determined phosphorus concentration in media along the potassium perturbation curve increased linearly with increasing potassium while the Sulphur concentration along the magnesium concentration curve remained relatively stable. Therefore, this series either represents a combination of potassium and Phosphorus excess and a potassium depletion along with sodium excess or potassium and phosphorus perturbation along the entire gradient. In the magnesium perturbation series, there were no detectable changes in sulphur concentrations upon magnesium depletion. However, when magnesium concentrations were changed above 5-times those in control media, the sulphur concentration also increased). Therefore, samples from both the potassium and magnesium perturbation series were not included in the statistical analysis to discover metal responsive genes and processes. However, they were retained for all analyses where global protein profiles were used to discover novel protein - protein connections or functions. For all other media, the concentration of elements that were measured, other than the perturbed metal, remained unchanged.

Supplementary Note 2

Given the ubiquitous presence of trace concentrations of some metals in solvents and lab materials, simply omitting these during media preparation does not guarantee complete depletion. One commonly employed strategy to deplete trace metals is the application of metal chelators (Shakoury-Elizeh et al. 2010; Navarrete-Perea et al. 2021)).However, these chelators, intended to reducing the availability of a specific metal impact a broad spectrum of molecular interactions, affecting metabolism and other biological processes (Kontoghiorghes et al. 2008). As a preliminary proteomic analysis validated this concern -  various commonly used Fe and Cu chelators we induced a distinct proteomic response- we devised a different strategy. We speculated that a cell in nature unlikely encounters metal ion concentrations that are lower than their concentration in ultra-high purity grade chemical solvents.

Supplementary Note M1: ICP-MS Acquisition Method

ICP-MS Acquisition parameters

Acq Mode Spectrum

Peak Pattern 3 points

Replicates 2

Sweeps/Replicate 100

Monitored Masses:

Mass Element Name

6 Li

10 B

23 Na

209 Bi

Tune Mode #1: No Gas

Quick Scan OFF

Stabilization Time 0 sec

Number of Masses 5

Mass Element Name IntegTime/Mass [sec]

6 Li 5.0001

10 B 2.0001

23 Na 0.0999

115 In 0.0999

209 Bi 0.0999

Tune Mode #2: H2

Quick Scan OFF

Stabilization Time 5 sec

Number of Masses 6

Mass Element Name IntegTime/Mass [sec]

39 K 0.0999

40 Ca 0.8001

45 Sc 0.3000

56 Fe 0.9999

72 Ge 0.2001

115 In 0.0999

Tune Mode #3: He

Quick Scan OFF

Stabilization Time 5 sec

Number of Masses 13

Mass Element Name IntegTime/Mass [sec]

6 Li 5.0001

11 B 2.0001

23 Na 0.0999

24 Mg 0.2001

45 Sc 0.3000

55 Mn 0.3999

59 Co 0.8001

63 Cu 0.8001

66 Zn 0.9999

72 Ge 0.2001

95 Mo 0.3000

115 In 0.0999

209 Bi 0.2001

Tune Mode #4: HEHe

Quick Scan OFF

Stabilization Time 5 sec

Number of Masses 10

Mass Element Name IntegTime/Mass [sec]

24 Mg 0.2001

31 P 1.5000

34 S 2.0001

39 K 0.0999

44 Ca 0.8001

45 Sc 0.3000

56 Fe 0.9999

72 Ge 0.2001

115 In 0.0999

209 Bi 0.2001

Peristaltic Pump Settings

Sample Introduction General

PeriPump/ISIS Settings

Pre Run

Uptake Speed (Nebulizer Pump) 0.3 rps

Uptake Time 29 sec

Stabilize 27 sec

Post Run (Probe Rinse)

Rinse Speed (Nebulizer Pump) 0.1 rps

Rinse at Rinse Port (Sample) 12 sec

Rinse at Rinse Port (Std) 20 sec

Post Run (Rinse)

Rinse Vial 1 <Manual>

Rinse Speed (Nebulizer Pump) 0.1 rps

Rinse at Rinse Vial (Step 1) 29 sec

Rinse at Rinse Port (Step 1) sec

Rinse Vial 2 <Manual>

Rinse Speed (Nebulizer Pump) 0.3 rps

Rinse at Rinse Vial (Step 2) 22 sec

Rinse at Rinse Port (Step 2) sec

Rinse Vial 3 <Manual>

Rinse Speed (Nebulizer Pump) 0.25 rps

Rinse at Rinse Vial (Step 3) 40 sec

Rinse at Rinse Port (Step 3) sec

Rinse Vial 4 <Manual>

Rinse Speed (Nebulizer Pump) 0.02 rps

Rinse at Rinse Vial (Step 4) 10 sec

Rinse at Rinse Port (Step 4) sec

Intelligent Rinse

Intelligent Rinse Off

Preemptive Rinse

Preemptive Rinse On

Start rinse before acquisition

complete

44 sec

Move to the next rinse step at

acquisition complete

Yes

Tune Settings

Tune Way Custom Tune

Tune Mode #1: No Gas

(Plasma)

RF Power 1550 W

RF Matching 1.80 V

Smpl Depth 8.0 mm

Nebulizer Gas 0.95 l/min

Option Gas 0.0 %

Nebulizer Pump 0.10 rps

S/C Temp 2 °C

Gas Switch Makeup Gas

Makeup/Dilution Gas 0.27 l/min

(Lenses)

Extract 1 0.0 V

Extract 2 -175.0 V

Omega Bias -65 V

Omega Lens 7.4 V

Cell Entrance -30 V

Cell Exit -49 V

Deflect 11.6 V

Plate Bias -34 V

(Cell)

Use Gas No

He Flow Off

He Flow Rate 0.0 ml/min

H2 Flow Off

H2 Flow Rate 0.0 ml/min

3rd Gas Flow Off

3rd Gas Flow Rate 0 %

OctP Bias -8.0 V

OctP RF 170 V

Energy Discrimination 5.0 V

Tune Mode #2: H2

(Plasma)

RF Power 1550 W

RF Matching 1.80 V

Smpl Depth 8.0 mm

Nebulizer Gas 1.03 l/min

Option Gas 0.0 %

Nebulizer Pump 0.10 rps

S/C Temp 2 °C

Gas Switch Makeup Gas

Makeup/Dilution Gas 0.14 l/min

(Lenses)

Extract 1 0.0 V

Extract 2 -185.0 V

Omega Bias -70 V

Omega Lens 8.0 V

Cell Entrance -40 V

Cell Exit -60 V

Deflect 0.6 V

Plate Bias -60 V

(Cell)

Use Gas Yes

He Flow Off

He Flow Rate 0.0 ml/min

H2 Flow On

H2 Flow Rate 6.0 ml/min

3rd Gas Flow Off

3rd Gas Flow Rate 0 %

OctP Bias -18.0 V

OctP RF 200 V

Energy Discrimination 5.0 V

Tune Mode #3: He

(Plasma)

RF Power 1550 W

RF Matching 1.80 V

Smpl Depth 8.0 mm

Nebulizer Gas 1.03 l/min

Option Gas 0.0 %

Nebulizer Pump 0.10 rps

S/C Temp 2 °C

Gas Switch Makeup Gas

Makeup/Dilution Gas 0.14 l/min

(Lenses)

Extract 1 0.0 V

Extract 2 -225.0 V

Omega Bias -90 V

Omega Lens 8.0 V

Cell Entrance -41 V

Cell Exit -60 V

Deflect 2.0 V

Plate Bias -60 V

(Cell)

Use Gas Yes

He Flow On

He Flow Rate 4.8 ml/min

H2 Flow Off

H2 Flow Rate 0.0 ml/min

3rd Gas Flow Off

3rd Gas Flow Rate 0 %

OctP Bias -18.0 V

OctP RF 200 V

Energy Discrimination 5.0 V

Tune Mode #4: HEHe

(Plasma)

RF Power 1550 W

RF Matching 1.80 V

Smpl Depth 8.0 mm

Nebulizer Gas 1.03 l/min

Option Gas 0.0 %

Nebulizer Pump 0.10 rps

S/C Temp 2 °C

Gas Switch Makeup Gas

Makeup/Dilution Gas 0.14 l/min

(Lenses)

Extract 1 0.0 V

Extract 2 -225.0 V

Omega Bias -110 V

Omega Lens 8.4 V

Cell Entrance -130 V

Cell Exit -150 V

Deflect -75.0 V

Plate Bias -150 V

(Cell)

Use Gas Yes

He Flow On

He Flow Rate 10.0 ml/min

H2 Flow Off

H2 Flow Rate 6.0 ml/min

3rd Gas Flow Off

3rd Gas Flow Rate 0 %

OctP Bias -100.0 V

OctP RF 200 V

Energy Discrimination 7.0 V

Supplementary Note M2

For the processing of all raw proteomics data described in this work, the following DIA-NN parameters were used:
 Logical CPU cores: 28
Number of threads : 14

Threshold for filtering output : 0.01 FDR ( precursors filtered using run-specific q-values, protein groups filtered using global q-values).
FASTA file for annotating protein names : yeast-uniprot-proteome-3AUP000002311- canonical.fasta (downloaded from uniprot.org (https://ftp.uniprot.org/pub/databases/uniprot/current_release/knowledgebase/pan_proteomes/UP000002311.fasta.gz ) , latest version of the genome as of 27th April 2022 )
Experimental spectral library : 20210225 yeastspeclib.tsv.speclib
Library-free search : enabled

In silico spectral library generation using : switched on
MBR : enabled
Min fragment m/z : 200
Max fragment m/z : 1800
N-terminal methionine excision : enabled
In silico digestion : cuts at K*,R*
Maximum number of missed cleavages : 1
Min peptide length : 7
Max peptide length : 30
Min precursor m/z : 300
Max precursor m/z : 1800
Min precursor charge : 1
Max precursor charge : 4
Cysteine carbamidomethylation as a fixed modification : enabled
Normalisation : Retention Time & Median-based signal-dependent local normalisation enabled
Mass accuracy : Optimised by DIA-NN automatically using the first run in the experiment.

Since a spectral library file generated using experimental data and the in silico library generation option was switched on, DIA-NN compared the in silico predicted spectra with those from the experimentally generated library and decided for each spectra which of the two was more reliable. The spectra from all samples were then re-analysed by DIA-NN using the more reliable of the two spectra.