Calculation methodology for (i) e-fuel yields from 217.61 Mt CO₂/year, (ii) carbon credit revenues, and (iii) global / removal percentage benchmarks

Overview

This document describes the arithmetic and assumptions used to derive (a) maximum theoretical e-fuel outputs from a total CO₂ feedstock of 217.61 MtCO₂/year (Tables 13 and 14), (b) carbon credit revenue estimates from the country breakdowns in Table 15, and (c) percentage shares relative to global fossil CO₂ emissions and to projected CO₂ removal needs for Paris-aligned pathways.

Inputs & assumptions

Primary input

- Total captured CO₂ available for e-fuels (combined Nigeria+Kenya+South Africa):
CO2_total = 217.61 MtCO₂/year (i.e., 217.61 × 1000000 tCO₂/year).

Stoichiometric CO₂ requirements (chemical minima) (used to compute theoretical maxima;  respective references are cited in our paper)

- E-kerosene: s_ker = 3.15 tCO₂ / t fuel

- Methanol (e-MeOH): s_meoh = 1.37 tCO₂ / t fuel

- Methane (synthetic CH₄ / SNG): s_ch4 = 2.75 tCO₂ / t fuel

- Ethanol (e-EtOH): s_etoh = 1.91 tCO₂ / t fuel

- Diesel (e-diesel via FT): s_diesel ≈ 3.15 – 3.20 tCO₂ / t fuel

Carbon credit price ranges (as used in our Table 15)

- Point-source CO₂ (gas flare/power/cement): $75 – $100 per tCO₂

- Direct Air Capture (DAC): $200 – $600 per tCO₂

Global baselines used for percentage calculations

- Global fossil CO₂ emissions (approx.): E_global_fossil = 37 GtCO₂/year = 37,000 MtCO₂/year  [117].

- Global CO₂ removal need (IAM/IEA benchmark for 2050): R_min = 5 GtCO₂/yr to R_max = 10 GtCO₂/year (i.e., 5,000–10,000 Mt/yr). IPPC [118], IEA [3].

1. Step-by-step: From 217.61 MtCO₂ → theoretical e-fuel outputs

Formula (per fuel):

Fuel_max (Mt/year) = CO2_total(Mt/year)Stoichiometric requirement(tCO2/tfuel)

(Units: CO2_total in Mt; stoichiometric requirement in tCO₂/t_fuel → result in Mt fuel/year.)

Numeric calculations (showing arithmetic):

E-kerosene

E-kerosine= 217.613.15=69.0825Mt/year=approx 69.08(rounded to 69Mt)

Methanol (e-MeOH)

Methanol= 217.611.37=158.8394Mt/year=approx 158.84(rounded to 159Mt)

Methane (synthetic CH₄ / SNG)

Methane= 217.612.75=79.1327Mt/year=approx 79.13(rounded to 79Mt)

Ethanol (e-EtOH)

Ethanol= 217.611.91=113.9372Mt/year=approx 113.94(rounded to 114Mt)

Diesel (e-diesel, FT) — show range for stoichiometric uncertainty:

Diesellow= 217.613.20=67.997Mt/year=approx 68.00(rounded to 68Mt)

Dieselhigh= 217.613.15=69.0825Mt/year=approx 69.08(rounded to 69Mt)

→ Reported as approx. 68–69 Mt/year.

Notes: these are theoretical chemical minima (stoichiometric), as real plant outputs will be lower after accounting for process inefficiencies, by-products, hydrogen consumption and energy penalties.

2. Step-by-step: Carbon credit revenue calculation (country breakdown → totals)

Method (per capture stream):

- For each capture stream in each country (CO₂ captured in Mt/year and price range $/t), we compute:

Revenue($)=CO2_Captured(Mt)*1000000(t/Mt)*Price($/t)

- Convert USD to billions (divide by 10^9) for readability.

- For each country sum the low and high revenues across all capture streams to give a country low–high revenue band. Sum country bands to get Nigeria, Kenya and South Africa total band.

Applied numeric example (Nigeria) — using data from your Table 15:

- Gas flare: 60.01 Mt; price range $75–100/t

  - Low: 60.01 × 10^6 × 75 = 4,500,750,000 USD = 4.50075 BUSD

  - High: 60.01 × 10^6 × 100 = 6,001,000,000 USD = 6.00100 BUSD

- Power plant: 0.42 Mt; price range $75–100/t

  - Low: 0.42 ×10^6 ×75 = 31,500,000 USD = 0.03150 BUSD

  - High: 0.42 ×10^6 ×100 = 42,000,000 USD = 0.04200 BUSD

- Cement: 19.01 Mt; $75–100/t

  - Low: 19.01×10^6×75 = 1,425,750,000 USD = 1.42575 BUSD

  - High: 19.01×10^6×100 = 1,901,000,000 USD = 1.90100 BUSD

- DAC: 4.09 Mt; $200–600/t

  - Low: 4.09×10^6×200 = 818,000,000 USD = 0.81800 BUSD

  - High: 4.09×10^6×600 = 2,454,000,000 USD = 2.45400 BUSD

Country totals (Nigeria)

- Low total: 4.50075 + 0.03150 + 1.42575 + 0.81800 = 6.77600 BUSD ≈ 6.78 BUSD

- High total: 6.00100 + 0.04200 + 1.90100 + 2.45400 = 10.39800 BUSD ≈ 10.39 BUSD

Repeat the same arithmetic for Kenya and South Africa using the CO₂ values and price ranges in Table 15. (i.e., multiply each stream’s Mt by 1e6 and by the two price endpoints, sum per country, then sum countries.)

Nigeria, Kenya and South Africa’s  totals (sum of country totals)

- Low (sum of Nigeria, Kenya, South Africa) ≈ 17.192 BUSD → reported rounded ~$17.2 B/year

- High ≈ 25.246 BUSD → reported rounded ~$25.3 B/year

Notes: The per-stream price ranges ($/t) are literature based from Tables 15.

3. Step-by-step: Global / removal percentages

A. Percentage of global fossil CO₂ emissions

- Convert global fossil CO₂ estimate to Mt: E_global_fossil = 37 GtCO₂/year = 37,000 MtCO₂/year.

- Compute:

% Global= CO2_total(Mt)E_global_fossil(Mt)*100

= 217.6137000*100=0.5883=approx 0.6%

B. Percentage of global CO₂ removal need (IAM / IEA benchmark)

- Convert removal benchmarks to Mt: Removal_min = 5 Gt = 5,000 Mt; Removal_max = 10 Gt = 10,000 Mt.

- Compute lower and upper percentages:

% Removal_max= 217.615000*100 =4.3522% =approx 4.35%

% Removal_min= 217.6110000*100 =2.1761% =approx 2.18%

So the capture potential corresponds to approx. 2.2–4.4% of the projected 2050 CO₂ removal requirement range (5–10 GtCO₂/year). We reported this rounded range as ~2–4% .

Reproducible Python snippet (copy/paste into Spyder / Jupyter)

We have provided the below python script that can be used to reproduce the numbers with different parameters:

# Reproducible calculations for SI

# Python 3

# Inputs

CO2_total_Mt = 217.61  # Mt CO2/yr

# Stoichiometric factors (tCO2 / t fuel)

s_ker = 3.15

s_meoh = 1.37

s_ch4 = 2.75

s_etoh = 1.91

s_diesel_low = 3.20

s_diesel_high = 3.15

# 1) Fuel theoretical maxima

fuel_outputs = {

"E-kerosene_Mt": CO2_total_Mt / s_ker,

"Methanol_Mt": CO2_total_Mt / s_meoh,

"Methane_Mt": CO2_total_Mt / s_ch4,

"Ethanol_Mt": CO2_total_Mt / s_etoh,

"E-diesel_low_Mt": CO2_total_Mt / s_diesel_low,

"E-diesel_high_Mt": CO2_total_Mt / s_diesel_high

}

for k, v in fuel_outputs.items():

print(f"{k}: {v:.4f} Mt/yr")

# 2) Carbon credit revenue calc

# Country capture breakdown (Mt/yr), and price ranges ($/t)

countries = {

"Nigeria": [

("Gas flare", 60.01, 75, 100),

("Power plant", 0.42, 75, 100),

("Cement", 19.01, 75, 100),

("DAC", 4.09, 200, 600)

],

"Kenya": [

("Gas flare", 0.0, 75, 100),

("Power plant", 1.46, 75, 100),

("Cement", 4.45, 75, 100),

("DAC", 1.71, 200, 600)

],

"South Africa": [

("Gas flare", 0.0, 75, 100),

("Power plant", 120.59, 75, 100),

("Cement", 4.70, 75, 100),

("DAC", 1.17, 200, 600)

]

}

total_low = 0.0

total_high = 0.0

for country, rows in countries.items():

low = 0.0

high = 0.0

for name, mt, p_low, p_high in rows:

low += mt * 1e6 * p_low

high += mt * 1e6 * p_high

total_low += low

total_high += high

print(f"{country}: Low ${low/1e9:.3f} B  High ${high/1e9:.3f} B")

print(f"SSA total: Low ${total_low/1e9:.3f} B  High ${total_high/1e9:.3f} B")

# 3) Global percentages

E_global_fossil_Mt = 37000.0  # 37 Gt = 37000 Mt

pct_global = CO2_total_Mt / E_global_fossil_Mt * 100

R_min_Mt = 5000.0  # 5 Gt -> 5000 Mt

R_max_Mt = 10000.0 # 10 Gt -> 10000 Mt

pct_removal_min = CO2_total_Mt / R_max_Mt * 100

pct_removal_max = CO2_total_Mt / R_min_Mt * 100

print(f"CO2_total share of global fossil CO2: {pct_global:.3f}%")

print(f"CO2_total share of removals (5-10 Gt): {pct_removal_min:.3f}% - {pct_removal_max:.3f}%")

Compact worked example

E-kerosene: 217.61 / 3.15 = 69.08 Mt/year (reported as 69 Mt/year).
Methanol: 217.61 / 1.37 = 158.84 Mt/year (≈159 Mt/year).
Methane: 217.61 / 2.75 = 79.13 Mt/year (≈79 Mt/year).
Ethanol: 217.61 / 1.91 = 113.94 Mt/year (≈114 Mt/year).
Diesel: 217.61 / 3.20 = 68.00 Mt/year to 217.61 / 3.15 = 69.08 Mt/year (≈68–69 Mt/year).

Carbon credits (Countries’ totals):

- Nigeria: low ≈ $6.776 B, high ≈ $10.398 B

- Kenya: low ≈ $0.785 B, high ≈ $1.617 B

- South Africa: low ≈ $9.631 B, high ≈ $13.231 B

- Countries’ total low ≈ $17.19 B; SSA total high ≈ $25.25 B (reported rounded as $17.2–25.3 B/year).

Global share calculations:

- Share of global fossil CO₂ (37 Gt): 217.61 / 37,000 × 100% = 0.588% ≈ 0.6%.

- Share of global removal need (5–10 Gt):
217.61 / 5,000 × 100% = 4.35% and 217.61 / 10,000 × 100% = 2.18% → ~2.2–4.4% (reported as ~2–4%)

Notes and caveats

- All fuel outputs are stoichiometric theoretical maxima (chemical lower bounds). Real plant-level outputs will be lower due to reaction conversions <100%, recycle, separation energy, auxiliary CO₂ demand, H₂ supply constraints, and other inefficiencies.

- Carbon credit revenue estimates are price-range based. They should be interpreted as indicative revenue potentials contingent on market rules and actual credit prices realised.

- Global baseline numbers are drawn from major datasets; small differences in baseline (e.g., using BP vs IEA vs GCP) will slightly change percentage results, however the qualitative conclusion (that SSA potential is regionally significant and globally meaningful in removal terms) is robust.