Step-by-step Procedure for Calculating Global and Regional Context of CO2 Capture and E-Fuel Production Potentials

Data sources (inputs)

Global jet fuel (kerosene) volume, 2023: 348.75 billion litres. Source: ATAG, Facts & Figures 2023.

Available at: https://atag.org/facts-figures (Accessed: 30 September 2025).

EU jet fuel demand (policy / projection baseline): 46 Mt (ReFuelEU / policy discussions). Source: Green Air summary of ReFuelEU.

Available at: https://www.greenairnews.com/?p=5227 (Accessed: 30 September 2025).

Available at: https://alternative-fuels-observatory.ec.europa.eu/transport-mode/aviation/general-information-and-context (Accessed: 30 September 2025).

Global methanol demand (2023): 110 Mt (IEA-Bioenergy / sector reporting).

Available at: https://www.ieabioenergy.com/wp-content/uploads/2024/06/Annual-Report-2023.pdf (Accessed: 30 September 2025).

Global ethanol production (2023): 29.5 billion US gallons (RFA).

Available at: https://ethanolrfa.org/file/2680/global_brief.pdf (Accessed: 30 September 2025).

Global diesel / middle-distillate demand (2023): 30.6 million barrels per day (BP / statistical reporting).

Available at: https://www.bp.com/en/global/corporate/energy-economics/statistical-review-of-world-energy.html (Accessed: 30 September 2025).

Global natural gas (used as methane proxy): ~4,000 bcm (billion cubic metres) (IEA reporting / 2023). EU ~350 bcm (IEA).

Available at: https://www.iea.org/reports/world-energy-outlook-2023 (Accessed: 30 September 2025).

Available at: https://www.iea.org/energy-system/transport/aviation (Accessed: 30 September 2025).

Assumptions / conversion constants

Mass / volume conversions

1 US gallon = 3.785411784 L. (standard)

1 oil barrel = 42 US gallons = 158.987294928 L. (EIA)

https://www.eia.gov/tools/faqs/faq.php?id=33&t=6

Densities used (typical values, stated explicitly)

Jet fuel / kerosene density: 0.80 kg·L⁻¹ (typical).

Ethanol density: 0.789 kg·L⁻¹.

Diesel / distillate density: 0.84 kg·L⁻¹ (assumed typical).

Methane gas density (STP, approximate from molar mass and ideal gas): computed from molecular weight 16.04 g·mol⁻¹ and molar volume 22.414 L·mol⁻¹ ⇒ ~0.7159 kg·m⁻³ (therefore 1 bcm ≈ 0.7159 Mt CH₄).

(Calculation method shown below.)

Mass unit conventions

1 tonne = 1,000 kg.

1 Mt (megatonne) = 1×10⁶ tonnes = 1×10⁹ kg.

Note: densities are stated; if we use slightly different densities (e.g. 0.78–0.85 kg·L⁻¹) and re-run the arithmetic, percentages will change slightly but not the qualitative conclusions.

Calculation steps

A. E-kerosene — global comparison

Input: global jet fuel = 348.75 × 10⁹ L (ATAG).

Density assumed: 0.80 kg·L⁻¹.

Convert litres → mass (kg):

mass_kg = 348.75e9 L × 0.80 kg·L⁻¹ = 279.000000000 × 10^9 kg

→ numeric: 279,000,000,000 kg

Convert kg → Mt:

mass_Mt = 279,000,000,000 kg ÷ 1e9 = 279.0 Mt

Compute % of our e-kerosene (69.0 Mt) that is of global jet fuel:

percentage = (69.0 ÷ 279.0) × 100% = 0.24731182796 × 100% = 24.731%

→ ≈ 24.7% (rounded  to 1 decimal place )

EU comparison: EU jet fuel baseline = 46 Mt (policy projection).

percentage_EU = (69.0 ÷ 46.0) × 100% = 1.5 × 100% = 150% → 69 Mt ≈ 150% of that EU baseline.

Citations: ATAG (2023); ReFuelEU / Green Air (2023).

B. Methanol — global comparison

Input: global methanol demand = 110 Mt (IEA-Bioenergy).

percentage_methanol = (159.0 ÷ 110.0) × 100% = 1.445454545 × 100% = 144.545%

→ ≈ 144.5% (i.e., theoretical 159 Mt > current global demand)

Citation: IEA-Bioenergy (2023).

C. Ethanol — global comparison

Inputs: RFA: 29.5 × 10⁹ US gallons (global production 2023).

Conversion constants: 1 US gal = 3.785411784 L; ethanol density = 0.789 kg·L⁻¹.

Convert gallons → litres:

ethanol_L = 29.5e9 gal × 3.785411784 L/gal = 111,658,948,528 L

→ numeric: 111,658,948,528 L

Convert litres → mass (kg):

ethanol_kg = 111,658,948,528 L × 0.789 kg/L = 88,107,351,978.492 kg

→ 88,107,351,978.5 kg

Convert kg → Mt:

ethanol_Mt = 88,107,351,978.5 kg ÷ 1e9 = 88.1073519785 Mt

→ ≈ 88.11 Mt

Percentage: our ethanol (114.0 Mt) as % of global ethanol:

percentage_ethanol = (114.0 ÷ 88.1073519785) × 100% = 1.2938761345 × 100% = 129.388%

→ ≈ 129.4%

Citation: RFA (2023).

D. Diesel (e-diesel / FT diesel) — global comparison

Inputs: BP report: middle distillate (diesel/gasoil) ≈ 30.6 million barrels per day (mb/d) (2023).

Constants: 1 barrel = 158.987294928 L; diesel density = 0.84 kg·L⁻¹.

Convert barrels/day → barrels/year:

barrels_year = 30.6e6 bpd × 365 d/year = 11,169,000,000 barrels/year

→ 11,169,000,000 barrels/year

Convert barrels → litres:

litres_year = 11,169,000,000 × 158.987294928 L/barrel = 1,775,184,877,200 L

→ ≈ 1.7751848772 × 10^12 L

Convert litres → mass (kg):

mass_kg = litres_year × 0.84 kg/L = 1,491,612,489,048 kg

→ ≈ 1.491612489048 × 10^12 kg

Convert kg → Mt:

diesel_global_Mt = 1,491,612,489,048 kg ÷ 1e9 = 1,491.612489048 Mt

→ ≈ 1,491.61 Mt

Percentage: our e-diesel (range 68–69 Mt):

Low: 68 ÷ 1,491.612489048 × 100% = 4.5588% → ≈ 4.56%

High: 69 ÷ 1,491.612489048 × 100% = 4.6259% → ≈ 4.63%

EU diesel (example calculation) — if EU diesel is taken as 6.0 mb/d (for example):

EU_barrels_year = 6.0e6 × 365 = 2,190,000,000 barrels

Convert to Mt as above → EU_diesel_Mt ≈ 292.47 Mt

68 ÷ 292.47 × 100% = 23.25% → ≈ 23.3% (note: use an authoritative EU diesel figure if you want a different EU %; Eurostat / FuelEurope / IEA provide region figures).

Citation: BP Statistical Review (2024); EIA conversion constants (barrel → litres).

E. Methane (synthetic CH₄ / SNG) — global comparison

Inputs: IEA global natural gas consumption ≈ 4,000 bcm (billion m³). EU ≈ 350 bcm.

CH₄ density computation (molar approach):

CH₄ molar mass = 16.04 g·mol⁻¹ = 0.01604 kg·mol⁻¹.

Molar volume (ideal gas, 0 °C) = 22.414 L·mol⁻¹ = 0.022414 m³·mol⁻¹.

Density ≈ 0.01604 kg/mol ÷ 0.022414 m³/mol = 0.7159 kg·m⁻³.

⇒ 1 bcm CH₄ ≈ 0.7159 Mt CH₄

Global methane mass (Mt):

methane_global_Mt = 4,000 bcm × 0.7159 Mt/bcm = 2,862.50 Mt

→ ≈ 2,862.50 Mt

Percentage: our synthetic methane (79 Mt):

percentage_methane = 79 ÷ 2,862.50 × 100% = 2.7598% → ≈ 2.76%

EU methane (350 bcm): EU_methane_Mt = 350 × 0.7159 = 250.47 Mt

79 ÷ 250.47 × 100% = 31.54% → ≈ 31.5%

Citation: IEA (global natural gas), and basic chemistry constants (molar volume; standard).

Compact table of results (numbers used & outputs)

E-kerosene: 69.0 Mt → 24.7% of global jet fuel (279 Mt); 150% of EU baseline (46 Mt). (ATAG; ReFuelEU)

Methanol: 159.0 Mt → 144.5% of global methanol (110 Mt). (IEA-Bioenergy)

Ethanol: 114.0 Mt → 129.4% of global ethanol (≈88.11 Mt). (RFA)

Diesel (e-diesel): 68–69 Mt → 4.56–4.63% of global diesel (≈1,491.61 Mt). (BP, conversions shown above)

Methane (SNG): 79.0 Mt → 2.76% of global natural gas (≈2,862.50 Mt); 31.5% of EU natural gas (≈250.47 Mt). (IEA, molar conversion)

Reproducible Python snippet (paste into Spyder / Jupyter / script)

We have provided the below python script that can be used to reproduce the numbers with different parameters:

# Reproducible calculations for e-fuel shares (Python 3)

import math

# Given outputs from 217.61 MtCO2 (Mt)

outputs = {

"E-kerosene": 69.0,

"Methanol": 159.0,

"Methane": 79.0,

"Ethanol": 114.0,

"E-diesel_low": 68.0,

"E-diesel_high": 69.0

}

# 1) Jet fuel (ATAG)

jet_liters = 348.75e9         # L

density_jet = 0.80            # kg/L

jet_mass_Mt = (jet_liters * density_jet) / 1e9   # Mt

pct_ek_global = outputs["E-kerosene"] / jet_mass_Mt * 100

# 2) Methanol (IEA-Bioenergy)

methanol_global_Mt = 110.0

pct_methanol = outputs["Methanol"] / methanol_global_Mt * 100

# 3) Ethanol (RFA)

gal_us = 29.5e9

liters_per_gallon = 3.785411784

ethanol_liters = gal_us * liters_per_gallon

density_ethanol = 0.789  # kg/L

ethanol_mass_Mt = (ethanol_liters * density_ethanol) / 1e9

pct_ethanol = outputs["Ethanol"] / ethanol_mass_Mt * 100

# 4) Diesel (BP)

bpd_diesel_global = 30.6e6

barrels_per_year = bpd_diesel_global * 365

liters_per_barrel = 158.987294928

density_diesel = 0.84  # kg/L

diesel_global_Mt = (barrels_per_year * liters_per_barrel * density_diesel) / 1e9

pct_ediesel_low = outputs["E-diesel_low"] / diesel_global_Mt * 100

pct_ediesel_high = outputs ["E-diesel_high"] / diesel_global_Mt * 100

# 5) Methane (natural gas)

bcm_global = 4000.0  # billion m3

# density via molar mass / molar volume (kg/m3)

density_ch4_kg_m3 = 16.04e-3 / 0.022414

methane_global_Mt = bcm_global * 1e9 * density_ch4_kg_m3 / 1e9

pct_methane = outputs["Methane"] / methane_global_Mt * 100

# Print results

print(f"Jet fuel global mass (Mt): {jet_mass_Mt:.2f}, E-kerosene share: {pct_ek_global:.2f}%")

print(f"Methanol share: {pct_methanol:.2f}% (global base {methanol_global_Mt} Mt)")

print(f"Ethanol global base (Mt): {ethanol_mass_Mt:.2f}, share: {pct_ethanol:.2f}%")

print(f"Diesel global (Mt): {diesel_global_Mt:.2f}, e-diesel share: {pct_ediesel_low:.2f}% - {pct_ediesel_high:.2f}%")

print(f"Methane global (Mt): {methane_global_Mt:.2f}, e-methane share: {pct_methane:.2f}%")

Notes, caveats and reproducibility

Stoichiometric maxima vs. real output: The e-fuel numbers (69, 159, 79, 114, 68–69 Mt) are theoretical chemical maxima derived from stoichiometry.  Real plant yields will be lower due to process inefficiencies, hydrogen availability, recycle streams, energy penalties and CO₂ purification needs. Treat the percentages as upper bounds on potential market share.

Data vintage / uncertainties: Some global/regional demand numbers are reported in volumes (L, barrels, bcm) and converted to mass using density assumptions. Different sources may use slightly different density assumptions or product definitions (e.g., “middle distillates”, “jet fuel” vs. “kerosene”). We list the source and the conversion constants used above but, if alternative baselines (IEA, BP, Eurostat) is preferred,  please re-run the small Python script with those values.

Citations and  access dates: We referenced source documents above with URLs and access date