Appendix B.3 – Estimation of Power Generation, Fuel Energy, and CO₂ Availability

For Kenya and South Africa, since Table 3 and Table 4 contain Plant_Capacity (MW) data, the following equations were used to estimate Power_Generation, Fuel_Energy, and CO₂_Availability.

(1) Power Generation

Power_Generation (MWh)  =Plant_Capacity (MW) *8760 *Capacity_Factor   (1)

where

Plant_Capacity (MW) = installed generation capacity

Capacity_Factor = average operational capacity factor

8760 = number of hours in one year

(2) Fuel Energy

Fuel_Energy (GJ)  =Power_Generation(MWh) *8760                                                   (2)

where

3.6 = conversion factor from MWh to GJ

(3) CO₂ Availability

CO2_Availability (Mt/year)  =  Fuel_Energy (GJ)*Emission_Factor                      (3)

where

Emission_Factor = fuel-specific CO₂ emission factor (MtCO₂ per GJ)

Sentence to reference this in your paper

For Kenya and South Africa, Power_Generation, Fuel_Energy, and CO₂_Availability were estimated from Plant_Capacity using Equations (1)–(3) provided in Appendix B.3 of the supplementary dataset.