Developing clean cooking strategies for Lao PDR; a modelling-based analysis

Research Report

Submitted to:

Climate Compatible Growth (CCG) Programme

Submitted by:

Xangpheuak Inthavideth and Xayalak Vilaida

National University of Laos (NUOL)

March 2026

# Table of Contents

Table of Contents	2

Abstract	4

1. Introduction	5

2. Cooking Technologies and Model Parameterisation	6

2.1 Cooking Technology Landscape in Lao PDR	6

2.1.1 Traditional Stoves	6

2.1.2 Improved Cookstoves (ICS)	7

2.1.3 Modern Energy Technologies	7

2.2 Socioeconomic Parameters	7

2.3 Technology Specification Parameters	8

3. Methodology and Data	10

3.1 The OnStove Modelling Framework	10

3.2 Geospatial Data Inputs	10

3.2.1 Administrative Boundaries	10

3.2.2 Population Data	10

3.2.3 Electricity Infrastructure	10

3.2.4 Biomass Availability	10

3.2.5 Base Layer	10

3.3 Urban and Rural Classification	11

3.4 Program Workflow and Simulation Logic	11

3.5 Core Calculation Formulas	11

3.6 Technology Specifications	12

3.7 Socioeconomic Parameters	12

3.8 Scenario Design	12

4. Baseline Cooking Technology Distribution	14

4.1 National Technology Mix	14

4.2 Urban–Rural Technology Divide	14

5. Scenario Analysis: Clean Cooking Transition 2025–2030	16

5.1 Evolution of Technology Distribution	16

5.1.1 Scenario 2026	16

5.1.2 Scenario 2027	16

5.1.3 Scenario 2028	17

5.1.4 Scenario 2029	18

5.1.5 Scenario 2030	19

5.2 Key Outcome Metrics	20

5.3 Investment Requirements	21

5.4 Energy Consumption Trends	21

5.5 CO₂ Emission Reductions	22

5.6 Net Economic Gains	23

6. Discussion and Policy Implications	25

6.1 Implications for National Policy	25

6.2 Carbon Finance Opportunities	25

6.3 Limitations and Uncertainties	25

7. Conclusions and Recommendations	26

References	27

Appendix A. Model File Structure	28

# Abstract

This report presents the development and application of the OnStove geospatial modelling framework for Lao PDR, aimed at informing evidence-based policy and investment decisions in the clean cooking sector. Laos currently relies heavily on solid biomass for household cooking, with tripod cook stoves, using firewood dominating at 51.9% national share, posing significant health, environmental, and economic costs. Using household-level geospatial data, technology specifications, and socioeconomic parameters, the OnStove model was calibrated for Lao conditions and used to simulate a clean cooking transition scenario from 2025 to 2030.

The baseline analysis, presenting the current situation, reveals a stark urban–rural divide: while urban households show higher uptake of improved cookstoves (ICS) and electricity-based cooking, rural communities remain predominantly dependent on traditional solid-fuel technologies. The scenario results demonstrate that a structured intervention programmed, targeting progressive replacement of traditional stoves with improved cookstoves (ICS WS-3 and ICS PPT-3) and electric cooking technologies, can deliver substantial co-benefits. By 2030, the modelled scenario projects a cumulative investment of USD 6.71 million, a reduction in annual energy consumption from ~12,000 TJ to ~9000 TJ, CO₂ emission reductions reaching ~133.17 million tons, and net economic gains of ~USD 1.3 billion, primarily driven by avoided health costs and time savings from reduced fuel collection.

These findings underscore the economic viability and development impact of scaling clean cooking in Laos, providing a quantitative foundation for national policy, development partner engagement, and carbon finance mechanisms.

Keywords: OnStove, clean cooking, Lao PDR, geospatial modelling, household energy, biomass, improved cookstoves, health impacts, CO₂ reduction

# 1. Introduction

Access to clean cooking remains one of the most pressing yet underappreciated energy challenges in Southeast Asia. In Lao PDR, the vast majority of rural households rely on traditional solid biomass — firewood and charcoal — for daily cooking and heating, primarily using open fires and rudimentary stoves such as the tripod stove. This dependence on unprocessed biomass is deeply rooted in cultural practice, limited market access, and the near-zero direct cost of collecting fuelwood in forested regions.

The consequences, however, are far-reaching. Household air pollution (HAP) from solid fuel combustion is one of the leading environmental health risks globally, responsible for approximately 3.2 million deaths per year (WHO, 2023). In Laos, women and children bear a disproportionate burden of this exposure, as they spend the most time near cooking fires. Beyond health, biomass dependence contributes to deforestation, greenhouse gas emissions (primarily black carbon and CO₂), and significant time poverty — particularly for women who collect fuelwood (IEA, 2023).

Despite these challenges, Laos has made policy commitments toward a clean energy transition. The National Socio-Economic Development Plan (NSEDP) and the country’s Nationally Determined Contributions (NDC) under the Paris Agreement identify household energy as a priority area. The 9th NSEDP (2021–2025) explicitly targets improved energy access and reduced biomass dependence as components of green growth (Government of Lao PDR, 2021). Lao PDR’s updated NDC (2021) sets a target of distributing 100,000 electric cookstoves by 2030 and commits to reducing emissions from the energy sector through fuel switching in households (Government of Lao PDR / UNFCCC, 2021). However, translating these policy intentions into targeted interventions requires a robust analytical foundation — spatial data, technology cost curves, and quantified co-benefit assessment.

The OnStove framework (Korkovelos et al., 2022) was developed to address this need. OnStove is an open-source geospatial modelling tool that simulates the net-benefit of transitioning households from traditional to cleaner cooking technologies. It integrates spatial population data, energy costs, technology efficiencies, health impact parameters, and time-saving estimates to identify optimal technology transitions at the household level across a country's geography. The model has been applied in multiple sub-Saharan African countries (Khavari et al., 2024) and offers a replicable methodology for Southeast Asian contexts.

This study presents the first application of the OnStove framework to Lao PDR. The primary objectives are: (1) to develop a calibrated baseline model representing the current distribution of cooking technologies across Lao households; (2) to run a forward-looking scenario exploring a clean cooking transition from 2025 to 2030; and (3) to quantify the investment needs, energy, health, and environmental co-benefits of this transition.

The report is structured as follows: Section 2 describes the cooking technology landscape and model parameterisation; Section 3 presents the methodology and data; Section 4 presents the baseline results; Section 5 details the scenario analysis and results; Section 6 discusses policy implications; and Section 7 concludes with recommendations.

# 2. Cooking Technologies and Model Parameterisation

## 2.1 Cooking Technology Landscape in Lao PDR

Lao PDR households rely on a diverse portfolio of cooking technologies, ranging from rudimentary open-fire stoves to modern electric appliances. These technologies vary substantially in their fuel requirements, thermal efficiency, pollutant emissions, cost, and practical availability across urban and rural settings. Understanding this landscape is essential to designing effective transition pathways and calibrating the OnStove model.

Figure 1 illustrates the six primary stove types used in Lao PDR households, organised by fuel type and thermal efficiency. Figure 2 shows the national fuel mix, which forms the foundation for baseline model calibration.

Figure 1. Primary Cooking Stove Types Used in Lao PDR Households. Thermal efficiencies range from 10–12% (Tripod) to 25–35% (Improved Cookstoves). Sources: GERES/SNV Lao PDR ICS Program; GERES/CTC Cookstove Testing Results.

### 2.1.1 Traditional Stoves

The Tripod stove is the most widely used cooking device in rural Laos, burning firewood at an efficiency of approximately 10–12%. Its extremely low capital cost (approximately USD 1) and the availability of free fuelwood from surrounding forests explain its persistence despite high PM2.5 emissions (656 µg/m³ emission factor). The TaoCement stove is a locally manufactured clay-and-cement construction with slightly improved efficiency (15–20%) and a wider grate that accommodates larger pieces of firewood. It is particularly prevalent in the central and southern lowlands. The TaoDam, a fired-clay design also known as the "black stove," operates on both firewood and charcoal at 15–20% efficiency. The TaoPayat is a charcoal-burning bucket stove (15–20% efficiency) more common in peri-urban areas where charcoal markets are accessible.

### 2.1.2 Improved Cookstoves (ICS)

Two ICS models are included in the analysis. The ICS WS-3 (Wood-burning, Tier 3) achieves 25–35% thermal efficiency through improved combustion chamber geometry and insulation, reducing both fuel consumption and PM2.5 exposure. It uses freely collected firewood, making it viable in rural areas with low fuel costs. Investment cost is approximately USD 22.50, with an operational lifespan of 2 years. The ICS PPT-3 (Processed-fuel, Tier 3) is designed for charcoal or pellet fuels, achieving efficiency equivalent to the WS-3 (25–35%) but with somewhat lower PM2.5 emissions (308 µg/m³ vs 361 µg/m³ for WS-3) due to more homogeneous fuel characteristics. Its investment cost is higher at USD 47, reflecting the more complex combustion chamber.

### 2.1.3 Modern Energy Technologies

LPG stoves achieve 60% thermal efficiency with near-negligible PM2.5 emissions (1.8 g/GJ) and zero time spent on fuel collection. However, the retail price of USD 1.65/kg (Ministry of Industry and Commerce, 2024) and the need for cylinder infrastructure make LPG economically inaccessible to most rural households. Additionally, all of the LPG in the country is imported. Electric cooking (induction or infrared hotplates) achieves 85% efficiency with zero point-of-use emissions. The electricity retail tariff of USD 0.065/kWh (Electricité du Laos, 2024) makes electric cooking cost-competitive where grid access is reliable. Investment cost for electric hotplates is approximately USD 35–55.

Figure 2. National Fuel Mix for Cooking in Lao PDR. Firewood dominates at 67%, followed by charcoal (24%), electricity (4%), other (3%), and LPG (2%). Source: Lao Social Indicator Survey (LSIS III, 2023).

The fuel mix data (Figure 2) confirms the overwhelming reliance on solid biomass: firewood accounts for 67% and charcoal 24% of cooking fuel nationally, totaling 91% biomass dependence. Modern fuels, electricity (4%), LPG (2%), and others (3%) collectively serve only 9% of households. This baseline fuel mix is consistent with the technology distribution modelled in OnStove, where biomass-burning stoves (Tripod, TaoCement, TaoDam, TaoPayat) collectively account for approximately 89.8% of national household cooking in the baseline year.

## 2.2 Socioeconomic Parameters

The socioeconomic specification file (soc_specs_lao.csv) encodes the demographic, epidemiological, and economic parameters used by OnStove to compute net benefits. Table 1 summarises the key parameter values and their sources.

Table 1. Key Socioeconomic Parameters for the Lao OnStove Model

| Parameter | Value | Source |
| --- | --- | --- |
| Population (2025) | 7,873,000 | UN World Population Prospects 2024 |
| Urban share | 38% | LSIS III (2023) |
| Electricity access (national) | 95% | World Bank WDI / MEM |
| Rural electrification rate | 90% | World Bank WDI / MEM |
| Urban electrification rate | 99% | World Bank WDI / MEM |
| Rural household size | 5.8 persons | LSIS III (2023) |
| Urban household size | 5.5 persons | LSIS III (2023) |
| Meals per day | 3 | LSB National Statistics |
| Minimum wage | USD 126/month | LSB / MoLSW 2024 |
| Value of Statistical Life (VSL) | USD 134,765 | Calculated (income-adjusted) |
| Discount rate | 15% | LSB / World Bank |
| Non-renewable biomass fraction (fnrb) | 0.15 | UNFCCC CDM TOOL33 v3.0 |
| Carbon cost | USD 51/tonne CO₂ | Social cost of carbon |
| Mortality rate — COPD (per 100k) | 15.2 | IHME GBD 2019 |
| Mortality rate — IHD (per 100k) | 85.5 | IHME GBD 2019 |
| Mortality rate — ALRI (per 100k) | 45.2 | IHME GBD 2019 |
| Mortality rate — Stroke (per 100k) | 95.4 | IHME GBD 2019 |
| Mortality rate — Lung Cancer (per 100k) | 12.4 | IHME GBD 2019 |

Table 1. Socioeconomic parameters encoded in soc_specs_lao.csv. COPD = Chronic Obstructive Pulmonary Disease; IHD = Ischaemic Heart Disease; ALRI = Acute Lower Respiratory Infection; GBD = Global Burden of Disease.

The health impact parameters deserve particular attention, as they are the primary driver of the net-benefit calculation. The mortality rates from IHME's Global Burden of Disease 2019 study reflect Lao PDR's disease burden attributable to household air pollution. The Stroke mortality rate (95.4 per 100,000) and IHD rate (85.5 per 100,000) are notably high, reflecting the compounding effects of HAP exposure, smoking prevalence, and limited access to cardiovascular care. The ALRI rate (45.2 per 100,000) predominantly affects children under five, a demographic particularly vulnerable to cooking smoke exposure.

The Value of Statistical Life (VSL) of USD 134,765 was calculated using an income-elasticity adjustment from the OECD base VSL, reflecting Lao PDR's per capita income level. This is substantially lower than VSL values used in high-income country analyses (typically USD 3–10 million), which explains why the net economic gains appear moderate relative to some comparable studies — the model applies conservative, country-appropriate health valuations.

## 2.3 Technology Specification Parameters

The technology specification file (tech_specs_lao.csv) encodes the physical, economic, and emission characteristics of all nine technologies in the model. Table 2 summarises the key parameters for each technology.

Table 2. Technology Specification Parameters for the Lao OnStove Model

| Technology | Eff. (%) | Inv. Cost (USD) | Tech Life (yr) | PM2.5 (µg/m³) | CO₂ (kg/GJ) | Fuel Cost (USD/unit) | Source |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Tripod stove | 12 | 1.00 | 5 | 656 | 112 | 0.02/kg | GACC; IPCC 2006 |
| TaoCement | 17.5 | 5.00 | 3 | 656 | 112 | 0.00 | GERES/CTC |
| TaoDam | 17.5 | 2.50 | 2.5 | 656 | 112 | 0.00 | GERES/SNV 2017 |
| TaoPayat | 17.5 | 5.00 | 2.5 | 656 | 112 | 0.00 | GERES/SNV 2017 |
| ICS WS-3 | 30 | 22.50 | 2 | 361 | 112 | 0.00 | GERES/SNV |
| ICS PPT-3 | 30 | 47.00 | 2 | 308 | 112 | 0.00 | GERES/SNV |
| LPG | 60 | 80.00 | 10 | 110 | 63 | 1.65/kg | MIC Lao 2024 |
| Electricity | 85 | 55.00 | 5 | 80 | 0.0* | 0.065/kWh | EDL 2024 |
| Biogas | 60 | 450.00 | 15 | 110 | 0.0 | 0.00 | SNV 2023 |

Table 2. Technology parameters from tech_specs_lao.csv. *Electricity CO₂ intensity at point-of-use is zero; upstream grid emissions use Lao PDR generation mix (predominantly hydropower). Eff. = thermal efficiency. PM2.5 = particulate matter 2.5 emission factor. Sources: GERES/SNV Lao PDR ICS Program; IPCC 2006 Guidelines; GAINS Model (IIASA); EDL Residential Tariff 2024; Ministry of Industry and Commerce (MIC) LPG Retail 2024.

The emission profiles highlight the large gap between traditional and modern technologies. The Tripod stove emits 656 µg/m³ PM2.5 versus 0 µg/m³ for electric cooking — a ratio of more than 650:1. Even intermediate ICS technologies reduce PM2.5 exposure by approximately 45–53% compared to the Tripod. LPG achieves near-zero PM2.5 emissions (1.8 g/GJ) but incurs significant fuel costs, limiting uptake among low-income rural households.

The non-renewable biomass fraction (fnrb) of 0.15 — calibrated using the UNFCCC CDM TOOL33 methodology — reflects Lao PDR's relatively high forest cover (approximately 57% of land area), meaning that only 15% of biomass collected for cooking is assumed to come from non-renewable sources (i.e., leads to net deforestation). This relatively low fnrb reduces the attributable CO₂ emissions from biomass stoves compared with more deforested countries, but does not diminish the health burden, which is driven by combustion particle emissions regardless of biomass renewability.

# 3. Methodology and Data

## 3.1 The OnStove Modelling Framework

OnStove is a Python-based open-source geospatial model developed by KTH Royal Institute of Technology and published under a permissive open-source license. The framework operates at the level of population grid cells (typically 100m × 100m resolution) and assigns the most cost-beneficial cooking technology to each cell based on a multi-criteria net-benefit calculation. The net benefit (NB) of transitioning from a baseline technology b to a candidate technology c is defined as:

NB(b→c) = ΔHealth Benefits + ΔTime Benefits − ΔCAPEX − ΔOPEX

Where health benefits are monetised using the Value of Statistical Life (VSL) methodology, time benefits are calculated from reduced fuel collection time, and costs include both capital expenditures (CAPEX) and annual operating expenditures (OPEX). The model is fully documented at https://onstove-documentation.readthedocs.io/.

The core algorithm iterates across all populated grid cells, computing the net benefit of each candidate technology relative to the current baseline stove, and assigns the technology with the highest net benefit. This process is repeated for each projection year, allowing dynamic simulation of technology transitions as household conditions, prices, and population distributions evolve. The simulation is developed using a spatial-dynamic framework to evaluate the socio-economic and environmental impacts of transitioning to clean cooking technologies in Laos from 2025 to 2030, leveraging high-resolution spatial data to ensure localised accuracy. Unlike traditional administration-based models, this grid-based approach enables granular analysis of household distribution, capturing the nuances of energy access and population density across the diverse terrains of Lao PDR.

## 3.2 Geospatial Data Inputs

The OnStove model for Laos was built using a multi-layered GIS dataset organised into the following categories:

### 3.2.1 Administrative Boundaries

National boundaries for Lao PDR were sourced from a validated GeoJSON file (Laos_Boundary.geojson) aligned with the World Bank administrative boundary dataset.

### 3.2.2 Population Data

High-resolution population distribution was derived from the WorldPop gridded dataset (Population.tif) at 100m resolution, providing household-level spatial resolution across the country. The dataset was reprojected to WGS84 and resampled to match the analysis grid. The total national population was approximately 7.4 million people (WorldPop, 2020).

### 3.2.3 Electricity Infrastructure

Electricity access data were derived from two sources: (i) medium-voltage (MV) transmission line routing (MV_lines.geojson) from the Lao PDR Department of Energy Business; and (ii) night-time light intensity raster data (Night_time_lights.tif) from the VIIRS Day/Night Band composite, used as a proxy for electricity access probability in peri-urban and rural areas.

### 3.2.4 Biomass Availability

Solid biomass potential was assessed using: (i) forest cover extent from the Global Forest Watch (Forest.tif), indicating areas with high fuelwood availability; and (ii) a friction surface (Friction.tif) representing travel time from each grid cell to biomass sources, derived from the Malaria Atlas Project accessibility dataset. Livestock density data (Global Livestock of the World, GLW4) for cattle, buffaloes, pigs, goats, sheep, and poultry were incorporated to estimate biogas feedstock potential.

### 3.2.5 Base Layer

A binary base raster (Base.tif) was used to mask unpopulated and water areas, ensuring the model only processed inhabited grid cells.

## 3.3 Urban and Rural Classification

To simulate realistic technology adoption, the model classifies each 100m × 100m grid cell based on its population density. The classification threshold of 300 persons/km² is applied consistently across the national grid:

Urban Areas: Grid cells with a population density greater than 300 persons/km² are classified as Urban. These areas are characterised by better infrastructure and higher access to electricity and LPG markets. Consequently, the model assigns a higher probability for these households to adopt electric cooking appliances and LPG stoves.

Rural Areas: Grid cells with a population density less than or equal to 300 persons/km² are classified as Rural. These areas typically rely more on traditional biomass. The model focuses on the transition from open-fire tripod stoves to Improved Cookstoves (ICS) or advanced biomass solutions, reflecting local fuel availability and socio-economic conditions.

## 3.4 Program Workflow and Simulation Logic

The programme follows a systematic four-step process to simulate the clean cooking transition across the national grid for each projection year (2025–2030):

- Data Integration: Merging spatial population data with technical specifications of cooking technologies (efficiency, cost, and emissions).

- Stochastic Technology Mapping: For each year (2025–2030), the programme applies a stochastic (probabilistic) approach to assign cooking technologies to households within each grid cell based on the defined scenarios.

- Annual Impact Simulation: The model runs iterative loops for each year to calculate the changes in energy use, health outcomes, and economic gains.

- Results Aggregation: Localised impacts are aggregated to produce national-level trends and summary tables.

## 3.5 Core Calculation Formulas

The model’s impact assessment is driven by the following core mathematical formulas, applied iteratively for each grid cell and aggregated to the national level:

(1) Energy Consumption (TJ/year)

Total energy consumption is calculated based on the cooking energy demand and the thermal efficiency of the assigned technology for each household:

E = ∑ (Cooking Energy Demand / Efficiencytech ) × Households

(2) CO₂ Reduction (Million Tonnes)

CO₂ reductions are derived from the energy saved relative to the 2025 baseline, multiplied by the technology-specific emission factor (EF):

CO₂saved = (Ebaseline − Ecurrent ) × EF

(3) Time Saved (Hours/day)

Time saved is calculated by measuring the reduction in fuel collection time and cooking time when transitioning from traditional tripod stoves to modern technologies. This captures the welfare gain for women and girls who bear a disproportionate share of fuel collection and cooking duties.

(4) Net Economic Gains (M.USD)

Net economic gains are estimated through a Cost-Benefit Analysis (CBA) that sums up the monetised value of health improvements and time savings, minus the total programme investment cost:

Net Gains = (Valuehealth + Valuetime ) − Costinvestment

This formula is consistent with the net-benefit function described in Section 3.1, expressed here in aggregate economic terms to align with the scenario reporting in Section 5.

## 3.6 Technology Specifications

Eight cooking technologies were included in the analysis, reflecting the range of stoves observed in Lao households and those proposed for scale-up. Table 3 summarizes the technologies and their key parameters as encoded in the tech_specs_lao.csv file. These parameters include thermal efficiency, indoor emissions factor, fuel cost, appliance cost, and lifetime.

Table 3. Cooking Technologies Included in the Lao OnStove Model

| Technology | Urban % | Rural % | National % |
| --- | --- | --- | --- |
| ICS PPT-3 | 10.8 | 3.3 | 2.6 |
| ICS WS-3 | 15.2 | 6.6 | 5.9 |
| LPG | 1.2 | 0.1 | 0.1 |
| TaoCement | 34.9 | 22.3 | 21.1 |
| TaoDam | 19.3 | 15.3 | 15.2 |
| TaoPayat | 9.2 | 2.3 | 1.6 |
| Tripod stove | 0.6 | 47.6 | 51.9 |
| Electricity | 8.8 | 2.4 | 1.6 |

Source: Authors' calibration based on tech_specs_lao.csv; urban/rural shares from baseline model outputs.

Traditional technologies — Tripod stove, TaoDam, TaoCement, and TaoPayat — represent locally manufactured charcoal and biomass-burning stoves prevalent across Lao PDR. Improved Cookstoves (ICS WS-3: wood-burning; ICS PPT-3: pellet/processed fuel) represent the primary intervention technologies. LPG and electricity-based cooking represent higher-tier modern energy solutions, predominantly feasible in urban and peri-urban areas.

## 3.7 Socioeconomic Parameters

Household-level socioeconomic parameters were encoded in soc_specs_lao.csv, including household size (average 5.5 persons per household), income distribution, willingness-to-pay parameters, fuel expenditure data, and cooking time estimates. These were calibrated against the Lao Statistics Bureau (LSB) Household Income and Expenditure Survey and the World Bank Poverty Assessment.

Health impact parameters followed the WHO exposure-response functions for household air pollution and PM2.5 concentrations, with monetisation using a VSL of USD 1.1 million (adjusted for Lao per capita income).

## 3.8 Scenario Design

Following the baseline calibration, a single forward-looking scenario was developed in collaboration with researchers at NUOL and Imperial College London. The scenario represents an "accelerated clean cooking transition" in which:

- Urban households progressively transition from traditional stoves to ICS and electric cooking between 2025 and 2030, with tripod stove use eliminated in urban areas by 2026.

- Rural households receive targeted ICS WS-3 and ICS PPT-3 distribution, reflecting a subsidy-supported programme reaching approximately 3–5% of rural households annually.

- Electricity-based cooking scales in urban areas as grid reliability improves, consistent with the Lao PDR National Electrification Programme targets.

- Technology costs follow a learning-curve trajectory, with ICS costs declining by approximately 5% per year as domestic manufacturing capacity scales.

The scenarios were run over six annual timesteps (2025–2030), with each year building on the technology distribution from the previous year.

# 4. Baseline Cooking Technology Distribution

## 4.1 National Technology Mix

The baseline model captures the 2024/2025 state of cooking technology in Lao PDR. At the national level, the Tripod stove is the dominant technology, used by 51.9% of households (Figure 3). This is followed by TaoCement (21.1%), TaoDam (15.2%), ICS WS-3 (5.9%), ICS PPT-3 (2.6%), TaoPayat (1.6%), and Electricity (1.6%). LPG remains negligible at 0.1%, reflecting high cylinder costs and supply chain limitations.

This figure is consistent with the findings of the Lao PDR Household Energy Survey (LFES, 2021) and the ESMAP Multi-Tier Framework survey, which reported biomass use by approximately 80% of rural households. The slight deviation reflects the OnStove model's probabilistic assignment, which accounts for grid-cell-level variation in accessibility and income.

Figure 3. Spatial Distribution of Cooking Technologies in Lao PDR — Baseline (2024/2025). Urban threshold: 300 persons/km². National urban share: 9.24%.

## 4.2 Urban–Rural Technology Divide

The baseline reveals a pronounced urban–rural divide in cooking technology access (Table 3). In urban areas, TaoCement accounts for 34.9% of households, followed by TaoDam (19.3%), ICS WS-3 (15.2%), and ICS PPT-3 (10.8%). Electric cooking reaches 8.8% of urban households, reflecting partial grid coverage in Vientiane and provincial capitals. Tripod stove use in urban areas is only 0.6%, suggesting that urbanization drives technology upgrading even without formal programmes.

In contrast, rural areas are dominated by Tripod stoves (47.6%), TaoCement (22.3%), and TaoDam (15.3%). ICS penetration remains low, ICS WS-3 at 6.6% and ICS PPT-3 at 3.3%, and electric cooking reaches only 2.4% of rural households. This urban–rural gap underscores the need for targeted rural interventions and reflects the limited reach of market-based distribution channels into remote communities.

The spatial map (Figure 3) clearly illustrates the concentration of modern cooking technologies in the Vientiane Plain and along major road corridors in central and southern Laos, while the northern and eastern mountainous regions remain predominantly traditional. This spatial heterogeneity has important implications for intervention targeting.

# 5. Scenario Analysis: Clean Cooking Transition 2025–2030

## 5.1 Evolution of Technology Distribution

The scenario models a progressive transition across six annual timesteps. Figures 4–8 present the spatial distribution of cooking technologies for each projection year, illustrating the geographic spread of cleaner technologies.

### 5.1.1 Scenario 2026

Figure 4. Spatial Distribution of Cooking Technologies — Scenario 2026. ICS WS-3: 9.1%; ICS PPT-3: 4.0%; Electricity: 4.1%; Tripod stove: 46.6%.

In 2026, tripod stove use declines to 46.6% nationally, driven primarily by urban elimination (0.0% in urban areas) and initial rural ICS distribution. ICS WS-3 grows to 9.1% nationally, with strong urban uptake (14.8%). Electric cooking accounts for 4.1% nationally (13.3% in urban areas), reflecting grid expansion in provincial capitals. TaoCement remains the primary transitional technology in urban areas at 35.0%.

### 5.1.2 Scenario 2027

Figure 5. Spatial Distribution of Cooking Technologies — Scenario 2027. ICS WS-3: 11.1%; ICS PPT-3: 6.1%; Electricity: 4.9%; Tripod stove: 44.7%.

By 2027, ICS PPT-3 achieves significant growth, reaching 6.1% nationally as pellet supply chains establish in provincial towns. ICS WS-3 reaches 11.1% nationally and 19.9% in urban areas. Tripod stoves decline to 44.7% nationally and 44.9% in rural areas, reflecting the cumulative effect of ICS distribution programmes. TaoDam begins declining in urban areas (10.2%) as households upgrade to ICS.

### 5.1.3 Scenario 2028

Figure 6. Spatial Distribution of Cooking Technologies — Scenario 2028. ICS WS-3: 13.0%; ICS PPT-3: 8.1%; Electricity: 6.1%; Tripod stove: 42.3%.

Scenario 2028 marks a notable acceleration in ICS PPT-3 adoption (8.1% nationally, 16.2% urban), suggesting that the pellet market reaches commercial viability. Electricity use grows to 6.1% nationally. TaoPayat stoves virtually disappear in urban areas (0.0%), while TaoCement remains the dominant intermediate technology in urban areas (34.9%). Rural tripod stove use declines to 42.9%.

### 5.1.4 Scenario 2029

Figure 7. Spatial Distribution of Cooking Technologies — Scenario 2029. ICS WS-3: 15.2%; ICS PPT-3: 9.0%; Electricity: 7.1%; Tripod stove: 40.7%.

By 2029, ICS WS-3 becomes the second most common technology nationally (15.2%), surpassing TaoDam. Electric cooking reaches 17.0% in urban areas, approaching parity with ICS WS-3 (23.0%) in urban areas. Rural tripod stoves decline to 41.3%, suggesting that ICS distribution is beginning to penetrate even the most remote communities. TaoPayat is effectively eliminated across the country (0.0% nationally).

### 5.1.5 Scenario 2030

Figure 8. Spatial Distribution of Cooking Technologies — Scenario 2030. ICS WS-3: 17.0%; ICS PPT-3: 10.1%; Electricity: 8.2%; Tripod stove: 38.7%.

The 2030 scenario endpoint shows a substantially transformed technology landscape. Tripod stoves decline to 38.7% nationally and 39.1% in rural areas — a reduction of 13.2 percentage points from the baseline (51.9%). ICS WS-3 reaches 17.0% nationally and 23.6% in urban areas, making it the second largest technology category. ICS PPT-3 reaches 10.1% nationally (20.2% urban). Electric cooking achieves 8.2% nationally and 21.6% in urban areas. TaoCement declines to 16.0% nationally, reflecting household graduation to higher-tier technologies. TaoDam shrinks to 10.0% nationally and just 4.1% in urban areas.

## 5.2 Key Outcome Metrics

Table 4 presents the annual scenario outcomes across the six projection years, covering investment requirements, energy consumption, deaths avoided, CO₂ reductions, time savings, and net economic gains.

Table 4. Scenario Analysis Results 2025–2030

| Year | Investment (M.USD) | Energy Use (TJ/yr) | Deaths Avoided | CO₂ Reduction (M.Tons) | Time Saved (Hr/day) | Net Gains (M.USD) |
| --- | --- | --- | --- | --- | --- | --- |
| 2025 | 3.18 | 11988 | 2838 | 132.72 | 4.95 | 1191.31 |
| 2026 | 3.9 | 11388 | 2846 | 132.81 | 5.01 | 1214.59 |
| 2027 | 4.62 | 10789 | 2854 | 132.9 | 5.056 | 1237.87 |
| 2028 | 5.29 | 10190 | 2862 | 132.99 | 5.102 | 1261.2 |
| 2029 | 5.98 | 9590 | 2870 | 133.08 | 5.159 | 1284.51 |
| 2030 | 6.71 | 8991 | 2878 | 133.17 | 5.207 | 1307.78 |

Source: OnStove model outputs (Scenario_Analysis_Results.xlsx). Deaths avoided are cumulative annual avoided premature deaths attributed to HAP reduction.

## 5.3 Investment Requirements

Figure 9. Total Annual Investment Cost (M.USD), 2025–2030. Investment scales from USD 3.18M in 2025 to USD 6.71M in 2030 as programme coverage expands.

Total annual investment requirements scale from USD 3.18 million in 2025 to USD 6.71 million in 2030 (Figure 9), reflecting the expanding reach of ICS distribution programmes and the higher per-unit cost of electric cooking infrastructure. The cumulative investment over the six-year period totals approximately USD 29.7 million. This is primarily composed of stove CAPEX (approximately 70%), with the remainder covering distribution, awareness, and after-sales service costs.

This investment level is modest in the context of international climate finance. The Green Climate Fund (GCF) and the Clean Cooking Fund (CCF) have both channelled multi-hundred-million dollar commitments to similar programmes in Sub-Saharan Africa. The Lao scenario investment requirement is well within the scope of a single GCF project proposal, suggesting strong potential for concessional climate financing.

## 5.4 Energy Consumption Trends

Figure 10. Total Energy Consumption Trend (TJ/year), 2025–2030. Energy use declines from 11,988 TJ to 8,991 TJ — a reduction of approximately 25%.

The scenario projects a steady reduction in total cooking energy consumption, from 11,988 TJ/year in 2025 to 8,991 TJ/year in 2030 — a 25.0% reduction over five years (Figure 10). This reduction is driven by the higher thermal efficiency of ICS technologies relative to traditional stoves. Improved cookstoves typically achieve 25–40% thermal efficiency compared to 5–15% for open fires and traditional stoves (FAO, 2017).

The energy savings have direct implications for forest resources. Under the business-as-usual trajectory, the Lao forestry sector would face continuing deforestation pressure from fuelwood demand. The modelled reduction of approximately 3,000 TJ/year by 2030 translates to reduced biomass extraction of approximately 1.3 million tonnes of dry wood equivalent annually, contributing to forest conservation and biodiversity co-benefits.

## 5.5 CO₂ Emission Reductions

Figure 11. Potential CO₂ Emission Reduction (Million Tonnes), 2025–2030. Cumulative CO₂ reductions increase from 132.72 Mt in 2025 to 133.17 Mt in 2030.

The CO₂ emission reduction trajectory shows steady annual accumulation, rising from 132.72 million tonnes in 2025 to 133.17 million tonnes by 2030 (Figure 11). It should be noted that these figures represent cumulative avoided emissions relative to a business-as-usual trajectory — not annual flow reductions — reflecting the stock nature of CO₂ accumulation in the atmosphere.

The annual increment of approximately 0.09 million tonnes per year of additional avoidance is consistent with the pace of technology transition. From a carbon finance perspective, even at conservative carbon credit prices of USD 5–10 per tonne CO₂, the annual avoided emissions could generate USD 450,000–900,000 in carbon revenues per year, providing a partial subsidy stream for programme sustainability. At more ambitious carbon prices (USD 30–50/tonne, aligned with the Social Cost of Carbon), the carbon revenue potential substantially exceeds the annual programme investment cost.

## 5.6 Net Economic Gains

Figure 12. Net Economic Gains Comparison (M.USD), 2025–2030. Net gains increase from USD 1,191M in 2025 to USD 1,308M by 2030, driven by health and time savings.

The net economic gains from the clean cooking transition — calculated as the sum of avoided health costs and time savings, minus programme investment costs — are substantial, rising from USD 1,191 million in 2025 to USD 1,308 million by 2030 (Figure 12). These figures far exceed the annual investment costs (USD 3–7 million), yielding benefit-cost ratios exceeding 170:1 at the national level.

The dominant driver of economic gains is the health benefit channel. Avoided premature deaths — approximately 2,838 to 2,878 per year over the scenario period — each valued at approximately USD 400,000 (VSL adjusted for Lao income levels) account for the majority of monetised benefits. Time savings from reduced fuel collection, averaging approximately 5 hours per household per day in rural areas, add further economic value, particularly for women and girls who disproportionately bear the burden of fuel collection.

These results are consistent with findings from comparable OnStove applications. Khavari et al. (2024) reported benefit-cost ratios of 12–40 for East African clean cooking interventions; the higher ratio for Laos reflects the lower per-household investment cost of ICS relative to LPG or biogas systems.

# 6. Discussion and Policy Implications

## 6.1 Implications for National Policy

The results of this analysis carry several important implications for Lao PDR's national energy and development policy. First, the scale of the tripod stove burden — 51.9% of households at baseline — confirms that the clean cooking challenge in Laos is predominantly rural, and a remote-access challenge. Urban markets are already transitioning through market forces; the policy and investment case is strongest in rural communities where market failures are high.

Second, the ICS pathway (particularly ICS WS-3) emerges as the most scalable near-term intervention in rural Laos, given the continued abundance of biomass resources and the low capital cost of wood-burning ICS relative to LPG or electric cookstoves. A national ICS distribution programme, potentially leveraging the Lao Stove Association and local NGO networks, could deliver the majority of the modelled transition benefits at relatively modest cost.

Third, the strong urban trajectory toward electric cooking suggests that the ongoing rural electrification programme — which targets near-universal access by 2030 — can be leveraged to accelerate adoption of electric cooking in newly electrified communities. Regulatory incentives (e.g., reduced tariffs for cooking load, bundled appliance financing) could accelerate this transition.

## 6.2 Carbon Finance Opportunities

The cumulative CO₂ reductions modelled in this analysis, while modest in absolute terms relative to the global carbon market, are significant enough to warrant the development of a programmatic clean cooking carbon project under VERRA's Verified Carbon Standard (VCS) or the Gold Standard. A properly structured carbon project could mobilise 3–5 years of carbon revenues in advance, providing the working capital needed to establish ICS distribution networks and reduce the upfront subsidy requirement.

Lao PDR's NDC also identifies clean cooking as an eligible mitigation action under its REDD+ framework, providing a further financing pathway through bilateral climate agreements or REDD+ result payments.

## 6.3 Limitations and Uncertainties

This analysis is subject to several important limitations. The OnStove model uses population grid cells as the unit of analysis and assigns baseline technologies to grid cells involving probabilistic estimation based on urban/rural classification and income proxies. Actual household-level survey data at the national level would improve model accuracy, but were not available at the resolution required for this analysis.

The scenario assumes a consistent annual rate of ICS distribution without explicitly modelling supply chain constraints, household willingness-to-pay heterogeneity, or stove adoption and sustained use rates. Field evidence from similar programmes in the region suggests that actual adoption rates may be 10–30% lower than theoretical potential (World Bank, 2022). Sensitivity analysis around these parameters would be a valuable extension of this work.

The CO₂ reduction figures require careful interpretation. The values reported represent modelled avoided emissions relative to the counterfactual trajectory, calculated using standardised emission factors for each technology. Actual verified emission reductions for carbon finance purposes would require a more detailed measurement, reporting, and verification (MRV) protocol.

Finally, cultural and behavioural factors — including taste preferences associated with traditional stoves, social norms around cooking practices, and gender dynamics in technology adoption decisions — are not captured in the current model but are acknowledged as critical determinants of real-world programme success (Puzzolo et al., 2019).

# 7. Conclusions and Recommendations

This study has developed the first OnStove geospatial model for Lao PDR, providing a quantitative foundation for clean cooking policy and investment planning. The key findings are:

- The current cooking technology landscape is dominated by traditional solid-fuel stoves, with Tripod stoves accounting for 51.9% of national household use. The urban–rural technology gap is pronounced and reflects structural barriers to market penetration in rural communities.

- A moderate-ambition clean cooking transition programme, investing USD 3–7 million per year from 2025 to 2030, can reduce tripod stove prevalence to 38.7% and achieve a 25% reduction in cooking energy consumption.

- The health and time-saving co-benefits of this transition are economically overwhelming, with benefit-cost ratios exceeding 170:1 at the national level, driven by approximately 2,800–2,900 avoided premature deaths per year.

- The modelled transition generates steadily growing net economic gains, reaching USD 1,308 million per year by 2030 — approximately 5% of estimated Lao GDP — underscoring the macro-economic significance of the clean cooking challenge.

- The scenario results are directly consistent with Lao PDR’s NDC commitments and provide quantitative evidence to support the case for accessing international climate finance. Lao PDR’s updated NDC (2021) sets a specific target of distributing 100,000 electric cookstoves by 2030 as part of its household energy mitigation contribution. The modelled scenario aligns with this ambition: by 2030, electric cooking reaches 21.6% of urban households and 8.0% of rural households nationally, reflecting a trajectory consistent with progressive cookstove electrification at scale. Furthermore, the scenario demonstrates that scaling clean cooking beyond electric stoves alone — through complementary ICS WS-3 and ICS PPT-3 distribution in rural areas where grid access remains limited — substantially amplifies the health, energy, and emissions co-benefits of the NDC target. The combined avoided emissions (133.17 million tonnes cumulative by 2030) and net economic gains (USD 1,308 million per year) provide a compelling co-benefit narrative to support project proposals to the Green Climate Fund, Clean Cooking Fund, or carbon crediting under VERRA’s Verified Carbon Standard or the Gold Standard.

Based on these findings, the following recommendations are made for policy and programme design:

- Prioritise rural ICS WS-3 distribution as the core near-term intervention, targeting communities in the northern and eastern highlands where tripod stove prevalence is highest.

- Integrate clean cooking programming with the National Electrification Programme, using new grid connections as entry points for electric cooking appliance financing.

- Develop a national programmatic carbon project to mobilise climate finance and provide a sustainable revenue stream for programme operations.

- Commission a national household energy survey to validate and update the OnStove baseline, improving model precision for subnational targeting.

- Engage the Lao Women's Union and provincial health networks to address cultural and behavioural adoption barriers through participatory behaviour change communication.

The OnStove model developed in this study provides a living analytical tool that can be updated as new data become available and extended to explore additional scenario dimensions, including sensitivity to fuel price changes, carbon pricing, and targeted subsidy designs. We recommend that the model be maintained and updated annually in collaboration with the Ministry of Energy and Mines and NUOL's Clean Cooking Group.

# References

Elvidge, C.D., Baugh, K., Zhizhin, M., Hsu, F.C., & Ghosh, T. (2017). VIIRS night-time lights. International Journal of Remote Sensing, 38(21), 5860–5879. https://doi.org/10.1080/01431161.2017.1342050

FAO (2017). Wood energy. Food and Agriculture Organization of the United Nations. https://www.fao.org/forestry/energy/en/

Government of Lao PDR (2021). 9th National Socio-Economic Development Plan (NSEDP) 2021–2025. Ministry of Planning and Investment, Vientiane. https://www.mpi.gov.la

Government of Lao PDR / UNFCCC (2021). Updated Nationally Determined Contribution (NDC) of the Lao People’s Democratic Republic. UNFCCC NDC Registry. https://unfccc.int/sites/default/files/NDC/2022-06/NDC%20of%20Lao%20PDR%20Updated%202021.pdf

Gilbert, M., Nicolas, G., Cinardi, G., Van Boeckel, T.P., Vanwambeke, S.O., Wint, G.R.W., & Robinson, T.P. (2018). Global distribution data for cattle, buffaloes, horses, sheep, goats, pigs, chickens and ducks in 2010. Scientific Data, 5, 180227. https://doi.org/10.1038/sdata.2018.227

Hansen, M.C., Potapov, P.V., Moore, R., Hancher, M., Turubanova, S.A., Tyukavina, A., ... & Townshend, J.R. (2013). High-resolution global maps of 21st-century forest cover change. Science, 342(6160), 850–853. https://doi.org/10.1126/science.1244693

IEA (2023). World Energy Outlook 2023. International Energy Agency. https://www.iea.org/reports/world-energy-outlook-2023

Khavari, B., Korkovelos, A., Arderne, C., Fuso Nerini, F., & Howells, M. (2024). Quantifying the benefits of clean cooking transitions in East Africa using the OnStove framework. The Lancet Planetary Health, 8(9), e634–e645. https://doi.org/10.1016/S2542-5196(24)00209-2

Korkovelos, A., Khavari, B., Sahlberg, A., Howells, M., & Fuso Nerini, F. (2022). Open-source geospatial modelling of clean cooking transitions. Nature Sustainability, 5(12), 1041–1050. https://doi.org/10.1038/s41893-022-01039-8

Lao Statistics Bureau (LSB) (2021). Lao PDR Household Energy Survey. Ministry of Planning and Investment, Vientiane.

Malaria Atlas Project (2020). Accessibility friction surface. Wellcome Sanger Institute. https://malariaatlas.org/

OnStove Documentation (2024). OnStove model documentation. KTH Royal Institute of Technology. https://onstove-documentation.readthedocs.io/en/latest/index.html

Puzzolo, E., Zerriffi, H., Carter, E., Clemens, H., Stokes, H., Johansson, L., ... & Bailis, R. (2019). Supply considerations for scaling up clean cooking fuels for household energy in low- and middle-income countries. GeoHealth, 3(12), 370–390. https://doi.org/10.1029/2019GH000208

WHO (2023). Household air pollution. World Health Organization. https://www.who.int/news-room/fact-sheets/detail/household-air-pollution-and-health

World Bank (2022). Clean Cooking: The Challenge and the Opportunity. World Bank Group. https://www.worldbank.org/en/topic/energy/brief/clean-cooking

WorldPop (2020). Lao PDR gridded population count. University of Southampton. https://www.worldpop.org/

Weiss, D.J., Nelson, A., Gibson, H.S., Temperley, W., Peedell, S., Lieber, A., ... & Gething, P.W. (2018). A global map of travel time to cities to assess inequalities in accessibility in 2015. Nature, 553(7688), 333–336. https://doi.org/10.1038/nature25181

# Appendix A. Model File Structure

The OnStove model for Lao PDR is organized in the following directory structure:

Root Directory Structure:

├─ soc_specs_lao.csv

├─ tech_specs_lao.csv

├─ gis_data/

│   ├─ Administrative/Laos_Boundary/

│   ├─ Base/

│   ├─ Biomass/ (Forest, Friction)

│   ├─ Demographics/population/

│   ├─ Electricity/ (MV_lines, Night_time_lights)

│   └─ Livestock/ (Buffaloes, Cattle, Chicken, Goats, Pigs, Sheep)

└─ Processed_Data_Laos/

├─ Base/

├─ Biomass/ (Forest, Friction)

├─ Demographics/population/

└─ Electricity/ (MV_lines, Night_time_lights)