Table 1: Summary table

Tübingen, Germany, 48.5°N, 9.1°E, TZ: Europe/Berlin

| Overview |  |  |  |
| --- | --- | --- | --- |
| Participants | Participants | <strong>26</strong> |  |
| Participant-days | Participant-days | <strong>208</strong> (8 - 8) |  |
| Days ≥80% complete | Days ≥80% complete | <strong>156</strong> (6 - 6) |  |
| Missing/irregular | Missing/Irregular | <strong>15.0%</strong> (12.0% - 17.0%) |  |
| Photoperiod | Photoperiod | <strong>12h 32m</strong> (10h 27m - 15h 35m) | 1 |
| Metrics2 |  |  |  |
| Dose | D (lx·h) | <strong>7,475</strong> ±9,743 (173 - 68,218) |  |
| Duration above 250 lx | TAT250 | <strong>2h 44m</strong> ±2h 32m (0s - 10h 38m) |  |
| Duration within 1-10 lx | TWT1-10 | <strong>3h 45m</strong> ±2h 31m (33m - 16h) |  |
| Duration below 1 lx | TBT1 | <strong>9h 17m</strong> ±2h 24m (12m 50s - 17h 46m) |  |
| Period above 250 lx | PAT250 | <strong>35m 19s</strong> ±40m 46s (0s - 4h 14m) |  |
| Duration above 1000 lx | TAT1000 | <strong>1h 3m</strong> ±1h 18m (0s - 6h 52m) |  |
| First timing above 250 lx | FLiT250 | <strong>09:09</strong> ±02:34 (00:07 - 15:01) | 1 |
| Mean timing above 250 lx | MLiT250 | <strong>13:27</strong> ±01:41 (08:03 - 17:48) | 1 |
| Last timing above 250 lx | LLiT250 | <strong>18:31</strong> ±02:42 (11:09 - 23:59) | 1 |
| Brightest 10h midpoint | M10midpoint | <strong>13:50</strong> ±01:34 (09:32 - 18:59) | 1 |
| Darkest 5h midpoint | L5midpoint | <strong>03:21</strong> ±01:42 (02:00 - 20:54) | 1 |
| Brightest 10h mean3 | M10mean (lx) | <strong>149.7</strong> ±208.2 (2.1 - 1,161.4) |  |
| Darkest 5h mean3 | L5mean (lx) | <strong>0.0</strong> ±0.2 (0.0 - 2.1) |  |
| Interdaily stability | IS | <strong>0.330</strong> ±0.100 (0.170 - 0.508) |  |
| Intradaily variability | IV | <strong>1.159</strong> ±0.377 (0.518 - 1.871) |  |
| values show: **mean** ±sd (min - max) and are all based on measurements of melanopic EDI (lx) |  |  |  |
| 1Histogram limits are set from 00:00 to 24:00 |  |  |  |
| 2Metrics are calculated on a by-participant-day basis (n=156) with the exception of IV and IS, which are calculated on a by-participant basis (n=26). |  |  |  |
| 3Values were log 10 transformed prior to averaging, with an offset of 0.1, and backtransformed afterwards |  |  |  |