# Evaluation of SIENA and two candidate SAR flood-mapping algorithms against VIIRS FIM over the December 2025 West-Coast U.S. event

# Executive summary

We evaluated three SAR-based flood-mapping algorithms – SIENA and two candidate algorithms hereafter Algorithm A and Algorithm B – against the VIIRS Flood Inundation Map (FIM) on 142 Sentinel-1 acquisitions spanning the December 2025 atmospheric-river flooding over the U.S. West Coast. Each 30 m SAR flood mask was aggregated to the 375 m VIIRS grid using an area-fraction operator (cell flagged as flooded when at least 40 % of its valid non-permanent-water sub-pixels were classified as flood) and compared against the VIIRS open-water class (≥ 30 % open-water fraction; permanent water and cloudy pixels excluded from the evaluation domain). Of the 134 acquisitions with sufficient valid domain (n_domain ≥ 100 pixels), 105 contained VIIRS-detected flooding and 29 contained none, enabling both detection and false-alarm assessments to be reported.

Across flood scenes the three algorithms exhibit two contrasting biases. SIENA systematically under-predicts at 375 m (pooled flooded-area bias 0.20), while Algorithms A and B over-predict (bias 5.4 and 4.4). On the 29 no-flood scenes – the operational silence test – Algorithm B falsely flags 16.9 % of all valid pixels as flooded (worst single scene 90 %) and Algorithm A falsely flags 4.6 % (worst 73 %), whereas SIENA registers a no-flood false-alarm density of only 0.02 %. SIENA therefore offers the most balanced behaviour at the chosen operating point – the highest pooled precision of the three (0.38) combined with the lowest false-alarm density on no-flood scenes – although its absolute detection scores are constrained by the geometric mismatch between narrow ribbon-like flood features at 30 m and the 40 % aggregation threshold applied at 375 m. The numbers reported here should be read together with the qualitative agreement visible in the per-scene five-panel quicklooks rather than as absolute skill scores.

# Methods (concise)

For each Sentinel-1 acquisition we (1) reprojected all SAR products and the VIIRS FIM onto a common WGS-84 grid co-registered to the VIIRS 0.00336° (≈ 375 m) tessellation; (2) aggregated each SAR flood mask from 30 m to 375 m using an area-fraction operator, flagging a 375 m cell as flooded when at least 40 % of its valid, non-permanent-water 30 m sub-pixels were classified as flood; (3) thresholded the VIIRS open-water class at ≥ 30 % water fraction to obtain the binary reference, excluding cloud, snow, ice, and permanent water (class 99); (4) restricted the joint evaluation domain to cells where both VIIRS was valid and the SAR product retained ≥ 50 % valid coverage after aggregation; and (5) computed pixel-wise confusion-matrix statistics per granule. Per-model summaries are reported both pooled across granules (pixel-equal weighting, micro-average) and as per-granule medians.

# Detection performance

Table 1 summarises pooled (pixel-level) detection performance on flood scenes alongside operational false-alarm metrics on flood and no-flood scenes. The companion boxplot (Figure 1) shows the spread of per-granule precision, recall, F1 and IoU across the 105 flood scenes.

| Model | n granules (valid) | n flood scenes | n no-flood scenes | Pooled Precision (flood) | Pooled Recall (flood) | Pooled F1 (flood) | Pooled IoU (flood) | Pooled HSS (all) | FA density flood scenes | FA density no-flood scenes |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| SIENA | 134 | 105 | 29 | 0.381 | 0.075 | 0.125 | 0.066 | 0.123 | 7.30e-04 | 2.17e-04 |
| Algorithm A | 134 | 105 | 29 | 0.021 | 0.111 | 0.036 | 0.018 | 0.025 | 0.0306 | 0.0456 |
| Algorithm B | 134 | 105 | 29 | 0.031 | 0.109 | 0.049 | 0.025 | 0.031 | 0.0203 | 0.1690 |

Table 1. Pooled (pixel-level) detection and false-alarm metrics per algorithm, separated by flood and no-flood scenes (n_domain ≥ 100). Pooled HSS = Heidke skill score across all valid scenes. FA density = FP / n_domain (the fraction of valid pixels falsely flagged as flooded).

Figure 1. Per-granule distributions of precision, recall, F1 and IoU on the 105 flood scenes, by algorithm. Horizontal bars denote medians; boxes the interquartile range; whiskers the 1.5×IQR range. Algorithm names are anonymised for publication.

# Operational false-alarm behaviour

Because an operationally-ready flood product must remain silent on scenes that do not contain flooding, we report false-alarm metrics separately for the 29 no-flood scenes (Table 1, last column; Figure 2 bottom row). Algorithm B exhibits the most concerning behaviour: 98 thousand pooled false positives across these 29 scenes, with 82 – 90 % of the valid domain falsely flagged as flooded in the three worst scenes. Algorithm A shows the same qualitative pattern at lower amplitude (4.6 % pooled FA density, worst single scene 73 %). SIENA registers two or fewer false-alarm pixels in 28 of the 29 scenes and is therefore the only candidate that satisfies the operational silence requirement at the current 40 % aggregation threshold.

| Model | Granule (no-flood scene) | FP count | n_domain | FA density |
| --- | --- | --- | --- | --- |
| SIENA | S1A_20251223T134848_20251223T134917_062443_07D2B4_175B | 2 | 204 | 0.0098 |
| SIENA | S1C_20251215T020859_20251215T020924_005456_00ADD1_6377 | 65 | 15644 | 0.0042 |
| SIENA | S1A_20251223T015425_20251223T015452_062436_07D267_AC94 | 10 | 5991 | 0.0017 |
| Algorithm A | S1C_20251221T012014_20251221T012039_005543_00B0C3_F7F9 | 227 | 311 | 0.7299 |
| Algorithm A | S1A_20251222T011403_20251222T011432_062421_07D1CB_EF67 | 911 | 1495 | 0.6094 |
| Algorithm A | S1C_20251224T014200_20251224T014225_005587_00B253_AC9B | 1207 | 2161 | 0.5585 |
| Algorithm B | S1C_20251221T012014_20251221T012039_005543_00B0C3_F7F9 | 280 | 311 | 0.9003 |
| Algorithm B | S1A_20251222T011403_20251222T011432_062421_07D1CB_EF67 | 1269 | 1495 | 0.8488 |
| Algorithm B | S1C_20251221T012039_20251221T012104_005543_00B0C3_262B | 76415 | 93190 | 0.8200 |

Table 2. Top three no-flood granules per algorithm ranked by false-alarm density (FP / n_domain). These granules contain no VIIRS-detected flooding; every flagged pixel is an unambiguous operational false alarm.

Figure 2. Operational false-alarm behaviour. Probability of false detection (POFD = FP / (FP + TN), left column) and false-alarm density (FP / n_domain, right column) per algorithm, split into flood scenes (top row; n=105) and no-flood scenes (bottom row; n=29). The bottom-right panel isolates the operational silence test; Algorithms A and B fail it at the current 40 % aggregation threshold while SIENA remains near zero.

# Representative case studies

Two hand-picked acquisitions illustrate the quantitative results qualitatively. Figure 3 shows a flood scene where all three algorithms produced non-trivial flood predictions; the spatial patterns broadly agree with VIIRS along the main river corridors, and SIENA produces a conservative envelope while Algorithms A and B add additional ambient wet pixels. Figure 4 shows a no-flood acquisition where Algorithm B (and to a lesser extent Algorithm A) erroneously flag a large fraction of the scene as flooded despite VIIRS recording no open-water anomaly, illustrating the false-alarm regime quantified in Table 1 and Figure 2.

Figure 3. Representative flood scene from 27 December 2025 over the U.S. West Coast (Sentinel-1C, ascending). Panels (5-panel layout): (top left) VIIRS FIM, (top right) SIENA, (middle row) Algorithms A and B, (bottom left) ESA WorldCover land-cover, (bottom right) categorical legend. All three algorithms capture the main flooded river corridor; Algorithms A and B add additional wet pixels in surrounding agricultural areas. Refer to the underlying figure file name for the exact granule identifier.

Figure 4. Representative no-flood acquisition from 21 December 2025. VIIRS records no open-water anomaly across the scene; Algorithm B (and Algorithm A) nevertheless classify a large fraction of valid pixels as flooded, producing the highest single-scene FA densities in the 142-acquisition record (Table 2).

# Summary and operational implications

Three conclusions follow from the 142-acquisition record. First, the absolute detection scores are uniformly low (pooled F1 < 0.15 for all three algorithms) but this is dominated by a scale mismatch between narrow ribbon-shaped flood features at 30 m and the 40 % aggregation threshold at 375 m, not by algorithm failure per se; a threshold sensitivity sweep is recommended in future work. Second, the three algorithms separate into two distinct behavioural regimes: SIENA is a conservative under-predictor with essentially negligible false-alarm activity, while Algorithms A and B are aggressive over-predictors that achieve marginally higher recall on flood scenes at the cost of large false-alarm areas. Third, when the operational silence test on no-flood scenes is included, SIENA is the only candidate that satisfies the operational deployment criterion at the current operating point: it retains non-trivial detection skill on flood scenes (pooled precision ≈ 0.38, the highest of the three) while keeping no-flood false-alarm density below 0.05 %. Algorithms A and B, despite higher recall on flood scenes, are not operationally deployable at the current threshold because they issue large false-alarm areas in scenes that contain no flooding.