This is the peer-reviewed version of the following article: Selective synthesis of galactooligosaccharides containing β(1→3) linkages with - β galactosidase from Bifidobacterium bifidum (Saphera), which has been published in final form at doi: 10.1021/acs.jafc.0c00997

This article may be used for non-commercial purposes in accordance with ACS Terms and Conditions for Self-Archiving.

Selective synthesis of galactooligosaccharides containing β(1→3) linkages with - β galactosidase from Bifidobacterium bifidum (Saphera)

Vera Füredera, Barbara Rodriguez-Colinasa,b, Fadia V. Cervantesa, Lucia Fernandez-Arrojoa, Ana Povedac, Jesus Jimenez-Barberoc,d, Antonio O.

Ballesterosa, Francisco J. Ploua,*

a Instituto de Catálisis y Petroleoquímica, CSIC, 28049 Madrid, Spain

- Departamento  de  Biotecnología,  Facultad  de  Ciencias  Experimentales,  Universidad Francisco de Vitoria, 28223 Pozuelo de Alarcón, Madrid, Spain

- CIC  bioGUNE,  Center  for  Cooperative  Research  in  Biosciences,  Parque  Científico Tecnológico de Bizkaia, 48160 Derio, Biscay, Spain

d Ikerbasque, Basque Foundation for Science, Maria Diaz de Haro 3, 48012 Bilbao, Spain

*	Corresponding	author	(Tel:	+34-915854869;	Fax:	+34-915854760;	E-mail: fplou@icp.csic.es).

# ABSTRACT

- The transglycosylation activity of a novel commercial β-galactosidase from Bifidobacterium

- bifidum (Saphera) was evaluated. The optimal conditions of operation of this enzyme, measured

- with o-nitrophenyl-β-D-galactopyranoside, were pH 6.0 and 40 °C. Although at low lactose

- concentrations the character of this enzyme was basically hydrolytic, an increase of lactose

- concentration to 400 g/L resulted in a significant formation (107.2 g/L, 27% yield) of prebiotic

- galactooligosaccharides (GOS). The maximum amount of GOS was obtained at a lactose

- conversion of approximately 90%, which contrasts with other β-galactosidases, for which the

- highest  GOS  yield  is  achieved  at  40-50%  lactose  conversion.  Using  HPAEC-PAD,

- semipreparative HPLC-HILIC, MS, 1D and 2D NMR, we determined the structure of most of

- the GOS synthesized by this enzyme. The main identified products were Gal-β(13)-Gal-

- β(14)-Glc  (3´-O--galactosyl-lactose), Gal-β(1→6)-Glc (allolactose), Gal-β(13)-Glc (3-

- galactosyl-glucose), Gal-β(1→3)-Gal (3-galactobiose) and the tetrasaccharide Gal-β(13)-

- Gal-β(13)-Gal-β(14)-Glc. In general, the B. bifidum β-galactosidase showed a tendency to

- form β(13) linkages followed by β(16), and more scarcely β(14) .

15

- Keywords:	Galactooligosaccharides,	Prebiotics,	Transglycosylation,	β-Galactosidase,

- Bifidobacteria, Glycosidases.

# INTRODUCTION

  - -Galactosidases (-D-galactoside galactohydrolases, EC 3.2.1.23) catalyze the hydrolysis

  - of different oligosaccharides with a galactosyl moiety located at the nonreducing end.1,2 The

  - dairy industry is taking advantage of this reaction to remove lactose from milk to obtain

  - lactose-free products.3,4 Due to the high amount of people suffering from lactose intolerance,

  - the demand for lactose-free products is steadily increasing.5 Apart from its hydrolytic activity,

  - -galactosidases are also able to catalyze transgalactosylation reactions in which lactose ‒or

  - other carbohydrates‒ act as acceptor molecules of galactosyl units yielding  di-, tri, tetra- or

  - even higher oligomers called galactooligosaccharides (GOS).6-9 GOS chemically resemble the

  - oligosaccharides present in human milk (HMOS) ‒which are structurally more complex‒, and

  - both families exhibit prebiotic properties.10-12 More precisely, they improve the gastrointestinal

  - microbiota by promoting the growth and/or the activity of bifidobacteria and lactobacilli,13

  - which are regarded to have a beneficial effect on human health.14,15 In addition to their prebiotic

  - activity, several studies describe that GOS are non-cariogenic, diminish the serum cholesterol

  - level,  prevent  colon  cancer,  enhance  the  digestibility  of  milk  and  improve  the  lactose

  - tolerance.16,17    Therefore,  -galactosidases  from  generally  recognized  as  safe  (GRAS)

  - microorganisms have attracted the attention of the dairy industries for enriching their products

  - with GOS and developing novel functional foods.13,18

  - The yield and the composition of the synthesized GOS are influenced by the source of the

  - enzyme and several reaction parameters including lactose concentration, pH, temperature, and

  - water activity.19-21 The percentage of GOS in the mixtures may vary in a wide range between

  - 15-55 % (w/w) depending on such conditions.22-25  The health promoting properties of GOS

  - depend on their chemical composition, structure and degree of polymerization, which are highly

  - influenced by the origin of the β-galactosidase.10  The chemical structure of the synthesized

  - oligosaccharides (number of hexose units, types of linkages between them and carbohydrate

  - composition)  may  affect  their  fermentation  pattern  by  probiotic  bacteria  in  the  human

  - intestine.26,27   Apparently,  GOS  mixtures  synthesized  by  -galactosidases  from  probiotic

  - microorganisms are consumed faster by probiotic species than those produced by other -

  - galactosidases.28-30

  - Bifidobacterium species are one of the most common organisms in the human gastrointestinal

  - tract.30,31	It	has	been	demonstrated	that	GOS	produced	by	-galactosidases	from

  - Bifidobacterium species have a different structure and significantly higher prebiotic potential

  - compared with commercial mixtures of GOS,28,32 probably due to the prevalence of β(13) and

  - β(16)  linkages,25   whilst  β(14)  and  β(16)  bonds  prevail  in  commercial  GOS.6,33

  - Interestingly, GOS synthesis catalyzed by β-galactosidases from bifidobacteria is not inhibited

  - by the presence of glucose or galactose,34,35 which are commonly recognized as inhibitors of

  - other -galactosidases.36

  - In the present work, we evaluated the transgalactosylation activity of a novel commercial -

  - galactosidase  from  Bifidobacterium  bifidum  (Saphera)  developed  to  obtain  lactose-free

  - products. A detailed chemical characterization of the synthesized GOS was performed.

# MATERIALS AND METHODS

## Materials

    - The commercial β-galactosidase Saphera® (Novozym 46091) from Bifidobacterium bifidum was

    - kindly supplied by Novozymes A/S (Bagsværd, Denmark). Lactose and o-nitrophenyl--D-

    - galactopyranoside  (ONPG)  were  from  Sigma-Aldrich.  The  standards  3-O--galactosyl-

    - galactose  (3-galactobiose),  4-O--galactosyl-galactose  (4-galactobiose),  6-O--galactosyl-

    - galactose  (6-galactobiose),  6-O--galactosyl-glucose  (allolactose)  and  4’-O--galactosyl-

    - lactose  were  from  Carbosynth  (Berkshire,  UK).  The  rest  of  standards  were  synthesized

    - following previous works developed in our laboratory.22,37,38  All other reagents and solvents

    - were of the highest available purity and used as purchased.

## Determination of β-galactosidase activity

    - The	assay	of	β-galactosidase	activity	was	performed	using	o-nitrophenyl--D-

    - galactopyranoside (ONPG) as substrate. The activity was measured at 40 ºC following o-

    - nitrophenol (ONP) release at 405 nm using a microplate reader (Versamax, Molecular Devices).

    - The reaction was started by adding 10 L of the enzyme solution (properly diluted) to 190 L

    - of 15 mM ONPG in 0.1 M phosphate buffer (pH 6.8). The increase of absorbance at 405 nm

    - was followed continuously at 40 ºC during 5 min. The molar extinction coefficient of o-

    - nitrophenol at pH 6.8 was 1627 M-1  cm-1. One unit of activity (U) was defined as that

    - corresponding to the hydrolysis of 1 mol of ONPG per min under the above specified

    - conditions.

## Optimum temperature and pH

    - The optimum temperature for the β-galactosidase activity was determined by mixing 50 µL of

    - a properly diluted enzyme solution with 450 µL of 15 mM ONPG in 0.1 M phosphate buffer

    - (pH 6.8). The mixture was incubated for 10 min at different temperatures between 10 ºC and

    - 70 ºC. The reaction was stopped by adding 500 µL of 0.4 M Na2CO3 and the absorbance at 405

    - nm was measured in endpoint mode using a microplate reader (Versamax, Molecular Devices).

    - The optimum pH for the β-galactosidase activity was measured at 40 ºC by performing the

    - enzyme assay in kinetic mode at different pH values (between 2.0 and 9.0), under the conditions

    - described above. The molar extinction coefficient of o-nitrophenol for each pH was determined 89    (pH 2.0, 378.65 M-1 cm -1; pH 3.0, 383.85 M-1 cm -1; pH 4.0, 441.98 M-1 cm -1; pH 5.0, 495.38 90       M-1 cm -1; pH 6.0, 661.65 M-1 cm -1; pH 7.0, 2043.5 M-1 cm -1; pH 8.0, 4397.83 M-1 cm -1; pH

- 9.0, 4403 M-1 cm -1). To minimize the buffer effect, solutions of 15 mM ONPG were prepared

- using 0.1 M Britton & Robinson buffer,39 adjusted to the required pH. All the experiments were

- performed in triplicate and the error was expressed as the standard deviation of the three

- measurements.

## pH and thermal stability of β-galactosidase

- In order to evaluate the thermostability of the β-galactosidase, the enzyme was incubated in

- 0.1 M phosphate buffer pH 6.8 for 1 hour at different temperatures ranging from 4 ˚C to 70 ˚C.

- The remaining activity towards ONPG was determined by performing enzymatic activity

- measurements as described above. The evaluation of the stability towards pH was performed

- incubating enzyme samples at 40 ˚C for 3 hours at various pH values (between pH 2.0 and pH

- 12.0) using the Britton & Robinson buffer. The remaining activity was measured as described

- above.

## Production of galactooligosaccharides from lactose

- Lactose (400 g/L 34.7% w/w) was dissolved in 20 mL of 0.1 M phosphate buffer (pH 6.0). The

- biocatalyst was then added to adjust the -galactosidase activity in the reaction mixture to 1.5

- U/mL (ONPG units). The mixture was incubated at 40C in an orbital shaker (Vortemp 1550)

- at 200 rpm. At different times, 750 μL aliquots were harvested from the reaction vessel. The

- enzyme was inactivated by incubating the samples in a Thermomixer (Eppendorf) for 10 min at

- 96 °C. Samples were then filtered using micro-centrifuge filter tubes, with 0.45 m cellulose

- acetate filters (National Scientific) at 6000 rpm for 5 min. The samples were diluted with water

- (1:400 and 1:4000) and then analyzed using HPAEC-PAD.

## HPAEC-PAD analysis

- Products were analyzed by high-performance anion-exchange chromatography with pulsed

- amperometric detection (HPAEC-PAD) on a ICS3000 Dionex system (Dionex Corp., CA)

- consisting of a SP gradient pump, an autosampler (model AS-HV) and an electrochemical

- detector with a gold working electrode and Ag/AgCl as reference electrode. All eluents were

- degassed by flushing with helium. A pellicular anion-exchange 4 x 250 mm Carbo-Pack PA-1

- column (Dionex) connected to a 4 x 50 mm CarboPac PA-1 guard column was used at 30 °C.

- Eluents were prepared with Milli-Q water and 50% (w/v) NaOH (Sigma-Aldrich). The initial

- mobile phase was 15 mM NaOH at 1.0 mL/min for 12 min. A mobile phase gradient from 15 to

- 200 mM NaOH was performed in 15 min at 1.0 mL/min, and it was maintained for 25 min. The

- peaks  were  analyzed  by  using  Chromeleon  software.  The  identification  of  the  different

- carbohydrates was performed on the basis of commercially available standards or products

- purified in the laboratory. For those compounds whose standards were not available at sufficient

- amount, quantification was done based on the calibration curve of standards with the same

- degree of polymerization.

## Purification of GOS

- For the purification of unidentified GOS, the reaction was carried out as described above and

- stopped after 22 h by heating the solution for 10 min at 96 ºC to inactivate the biocatalyst. The

- reaction mixture was filtered using 0.45 µm paper filters and purified by semipreparative

- hydrophilic  interaction  chromatography  (HPLC-HILIC).  A  quaternary  pump  (Delta  600,

- Waters) coupled to a Lichrosorb-NH2  column (5 m, 10 x 250 mm, Merck) was used. The

- column temperature was kept at 25 ºC. The acetonitrile/water 75/25 (v/v) mobile phase was

- conditioned with helium and used as at a flow-rate of 6.25 mL/min for 43 min. Peaks were

- detected by using an evaporative light-scattering detector DDL-31 (Eurosep) equilibrated at

- 60 C. A three-way flow splitter (model Accurate, Dionex) and a fraction collector II (Waters)

- were employed. The fractions containing the main peaks were collected and the solvent was

- removed by rotary evaporation.

## Mass spectrometry

- The MS analysis of purified galactooligosaccharides was assessed using a mass spectrometer

- with hybrid QTOF analyzer (model QSTAR, Pulsar i, AB Sciex). The sample was analyzed by

- direct infusion and ionized by electrospray (with methanol containing 1% of NH4OH as ionizing

- phase) in positive reflector mode.

## Nuclear Magnetic Resonance (NMR)

- The structure of the oligosaccharides was determined by using a combination of 1D

- (1H, 13C) and 2D (COSY, NOESY, TOCSY, HSQC, HSQC-TOCSY, HMBC) NMR

- techniques.	The	samples	were	dissolved	in	D2O,	using	TSP-d4	[3-

- (trimethylsilyl)propionic-2,2,3,3-d4  acid  sodium  salt]  as  chemical  shift  reference.

- Chemical shifts were expressed in parts per million with respect to the 0 ppm point of

- TSP. The spectra were measured in a Bruker 800 AVIII spectrometer equipped with a

- 4-channels cryo TCI probe, with gradients in the Z axis, at a temperature of 298K. For

- the 1H-13C heteronuclear correlations (HSQC, HQSC-TOCSY, HMBC), values of 5 ppm

- and 1-2K points, for the 1H dimension, and 70 ppm and 384 points for the 13C

- dimension, were used. For the 1H-1H homonuclear experiments (COSY, NOESY,

- ROESY), 5 ppm windows were used in both dimensions with a 1-2K x 384 point matrix.

- For the NOESY experiments, mixing times of 500-600 ms were used. The spinlock time

- was set to 300 ms for ROESY and 60/100 ms for HSQC-TOCSY experiments. The

- standard pulse sequences used were provided by Bruker.

# RESULTS AND DISCUSSION

- Effect of pH and temperature on activity and stability of Bifidobacterium bifidum -

## galactosidase

- The hydrolytic activity of commercial Bifidobacterium bifidum β-galactosidase (Saphera),

- measured with o-nitrophenyl--D-galactopyranoside (ONPG) as substrate, was 1506 ± 0.1

- U/mL, determined at 40 ºC and pH 6.8, which are the optimal conditions reported for several -

- galactosidases from Bifidobacteria.34,35,40

- In  order  to  determine  the  optimum  conditions  of  operation  of  this  novel  enzymatic

- preparation, a series of activity assays were performed at a fixed pH value (6.8) and different

- temperatures. The optimum temperature of the -galactosidase from B. bifidum (Saphera) was

- found to be 40 ˚C (Fig. 1a). Furthermore, the thermostability was analyzed measuring the

- residual activity the enzyme in 0.1 M phosphate buffer (pH 6.8) after 1 h incubation. The β-

- galactosidase showed a rapid inactivation at temperatures above 50 ˚C (Fig. 1b). The thermal

- stability was significantly smaller than that reported for other related enzymes, such as β-

- glycosidases  from  Sulfolobus  solfataricus  and  Pyrococcus  furiosus.41   However,  the  β-

- galactosidase from B. bifidum showed a higher thermostability compared to -galactosidases

- from other Bifidobacterium species like B. breve.25

- The optimum pH of this preparation was also studied (Fig. 2a). The highest activity with

- ONPG was obtained at pH 6.0. Regarding the stability of the enzyme at different pH values, the

- results showed that the β-galactosidase displayed good stability in the pH range 5.0-10.0, after

- 3 hours of incubation at 40 ºC (Fig. 2b). At pH 3.0 and 11.0 the inactivation of the enzyme was

- very  fast.  In  this  context,  the  -galactosidase  from  Bifidobacterium  breve  was  rapidly

- inactivated at pH 9.0.25 Our results fit well with those described by Tzortis et al. using whole

- cells of B. bifidum NCIMB 41171, for which the optimum pH and temperature were 6.8-7.0 and

- 40 ˚C, respectively.34  The related β-galactosidase from B. longum showed optimal pH and

- temperature of 5.8-6.8 and 45°C, respectively.35

- GOS specificity of B. bifidum -galactosidase

- The ability of B. bifidum β-galactosidase to synthesize galactooligosaccharides (GOS) was

- assessed. First, we followed by HPAEC-PAD the hydrolysis of lactose at 160 g/L under the

- optimal conditions (pH 6.0, 40 °C). As shown in Fig. 3a, the main reaction catalyzed by this

- enzyme  under  these  conditions  was  the  hydrolysis.  However,  the  formation  of  several

- transgalactosylation products was observed.

- We further increased the lactose concentration to 400 g/L to promote transfer reaction. The

- HPAEC-PAD chromatogram (Fig. 3b) showed at least 19 peaks of which numbers 1, 2 and 5

- corresponded to galactose, glucose and lactose, respectively. The main reaction products

- corresponded to peaks 10, 15 and 19. We were able to purify peaks 15 and 19 by semipreparative

- HILIC for their characterization. For peak 15, the major signal in the mass spectrum in positive

- mode (Fig. 4A) was at m/z 527.16, corresponding to the M+[Na]+ ion of a trisaccharide. The 1D

- and 2D  NMR spectra  indicated that  the  structure  of  the compound fitted  well with  the

- trisaccharide 3’-O-β-galactosyl-lactose [Gal-β(13)-Gal-β(14)-Glc], which was previously

- identified and characterized in our laboratory studying the GOS synthesis with β-galactosidase

- from Aspergillus oryzae.22

- The main signal in the mass spectrum of peak 19 (Fig. 4B) was at m/z 689.21, corresponding

- to the M+[Na]+ ion of a tetrasaccharide. The 1H-NMR-based information for the major product

- of this peak was fairly similar to that recorded for peak 15, although it clearly displayed one

- additional anomeric proton (Figs. 5A and 5B). The analysis demonstrated that the Gal rings

- were not substituted at position 4, and only 3-Gal glycosylated moieties could be found. As for

- peak 15, the existing Glc ring was glycosylated position 4. Therefore, the structure of the

- compound was determined to be 3´-O-β-(3-galactobiosyl)-lactose [Gal-β(13)-Gal-β(13)-

- Gal-β(14)-Glc] in a non-ambiguous manner. Fig. 5 shows the similarity of the 1H-NMR

- spectra of peaks 15 and 19. Both positions α- and β- at the reducing end were unambiguously

- identified for these compounds by the HSQC spectrum.

- Peaks  8  and  9  were  identified  as  the  trisaccharide  6’-O--galactosyl-lactose  and  the

- disaccharide 3-O--galactosyl-glucose, respectively, using purified standards from K. lactis -

- galactosidase reactions.42 With commercial standards it was also possible to identify peak 12 as

- the trisaccharide 4’-O--galactosyl-lactose and several disaccharides with different glycosidic

- bonds, involving two units of galactose (peak 3, 6-galactobiose; peak 6, 3-galactobiose; peak 7,

- 4-galactobiose)  or  galactose-glucose  (peak  4,  Gal-β(16)-Glc,  allolactose;  peak  9,  Gal-

217	β(13)-Glc).

- One more reaction product (peak 13) was also partially purified by semipreparative HPLC.

- The mass spectrum showed that the main signal corresponded to a trisaccharide. Although the

- purity of this sample was not enough for a precise structure determination, the identity of the

- components of the mixture was further studied. From the NMR data, different glycosylation

- positions could be detected. For the Gal-Gal linkages, both (13) and (14) disaccharide

- fragments were found, with similar proportions. For the Gal-Glc linkages, the (14) position

- was predominant, but minor (13) as well as minute fractions of (12) linked moieties were

- also detected. Therefore, the mixture within peak 13 is composed of the Gal-β(13)-Gal-

- β(14)-Glc and Gal-β(14)-Gal-β(14)-Glc trisaccharides as major products, together with

- the	Gal-β(13)-Gal-β(13)-Glc,	Gal-β(14)-Gal-β(13)-Glc	analogues	as	minor

- components. Such assignations were performed on the basis of previous publications.43-46

- Additionally, small amounts of products with the Glc moiety glycosylated at position 2 were

- present.

- In a recent paper, using the β-galactosidase activity from several Pantoea anthophila strains,

- we were able to identify peak 14 as 3’-O-galactosyl-allolactose (Gal-β(13)-Gal-β(16)-

- Glc).47 Regarding peak 10, one of the most abundant in the GOS mixture, we believe that it

- could be composed by three galactosyl residues bound by β(13) linkages [Gal-β(13)-Gal-

- β(13)-Gal, 3-galactotriose]. This assumption is based on the enzyme specificity, the relative

- retention times and the presence of a significant amount of 3-galactobiose, which could serve as

- galactosyl acceptor. However, we were not able to isolate it with enough purity for NMR

- analysis. Peaks 11, 16, 17 and 18 remained unidentified, but all of them were minor components

- of the mixture.

- Our results fit well with those obtained using other B. bifidum strains. Depeint et al.48 and

- Goulas et al.49 reported the presence of β(13) linkages, with less participation of β(14) and

- β(16)  bonds,  in  the  GOS  mixture  obtained  with  β-galactosidase  from  B.  bifidum

- NCIMB41171. However, Rabiu et al. identified, in addition to β(13) linkages, several β(16)

- and β(14) bonds.28 This partially agrees with our findings, but it is well studied that there are

- more than one β-galactosidase in Bifidobacteria strains, and the synthesized oligosaccharides

- may vary.40,49

## Progress of GOS formation

- The progress of GOS formation was followed by HPAEC-PAD. Experimental conditions were

- 400 g/L lactose in 0.1 M sodium phosphate buffer (pH 6.0), 40 ºC and 1.5 U/mL (ONPG units).

- Table 1 shows the concentrations of the main reaction products at different times. Under the

- above conditions, the point of maximum GOS production (107.2 g/L, 27% of total sugars in the

- mixture) was obtained at 50.5 h. At this point, the rest of components in the mixture were

- glucose (38%), galactose (26%) and lactose (9%). It is worth noting that the time required to get

- the maximum GOS yield depends inversely on the amount of enzyme; however, as occurs in

- this kind of kinetically-controlled reactions,50-52  the GOS concentration at this point is not

- affected by the amount of biocatalyst. Rabiu et al. reported GOS yields between 24.7-47.6%

- employing 30% (w/w) lactose and five different species of bifidobacteria, including B. bifidum

- BB-12 (37.6% yield).28 Tzortzis et al. demonstrated that an increase in lactose concentration

- reported a higher yield of GOS using whole cells B. bifidum NCIMB41171 with β-galactosidase

- activity. Maximum GOS yield was 35% employing 55% (w/w) lactose.34

- At the point of maximum GOS concentration, the most abundant GOS corresponded to 3’-

- galactosyl-lactose (17.7 g/L), allolactose (16.2 g/L), 3-O-β-galactosyl-glucose (9.0 g/L), and 3-

- galactobiose (8.2 g/L), apart from unidentified peak 10. These results confirm that the enzyme

- has a clear tendency to form β(13) linkages followed by β(16).  However, only a small

- proportion of products containing new β(14) linkages were observed. It is worth noting that

- the GOS synthesis using B. bifidum -galactosidase resulted in a high amount of disaccharides

- (approx. 40.4 g/L), with allolactose, 3-O--galactosyl-glucose and 3-galactobiose as the main

- components of this fraction. In this context, Böger et al. recently reported that most probiotic

- strains of gut bacteria exhibited a preference for DP2 galactooligosaccharides.53

- Fig. 6 represents the profile of GOS concentration as a function of lactose conversion. It is

- remarkable that the GOS concentration increased progressively until 80% of initial lactose has

- disappeared. Then, the yield of GOS was nearly invariable until approximately 93% of lactose

- was  consumed. These results  contrast with  those  obtained  with  Bacillus  circulans37   and

- Aspergillus oryzae22  β-galactosidases, for which the maximum concentration of GOS was

- achieved at approximately 40-50% of lactose conversion. Thereafter, GOS are hydrolyzed by

- the enzyme, in particular those containing β(14) linkages. The profile obtained with B.

- bifidum -galactosidase resembles that of Kluyveromyces lactis lactase, as the maximum GOS

- yield was also achieved at approximately 95% of lactose conversion.42 In this context, Hsu et

- al., using a β-galactosidase from B. longum, reported a maximum GOS yield of 32.5%, which

- obtained at approximately 60% lactose conversion, and was followed by a hydrolysis of the

- synthesized GOS.35

- In conclusion, the commercial preparation Saphera gave rise to a moderate GOS yield (27%)

- compared with other -galactosidases. However, the predominance of β(13) linkages in the

- synthesized GOS has several advantages with respect to other commercial GOS mixtures. First,

- the production of GOS with Saphera can be easily controlled because GOS hydrolysis only takes

- place when approximately 95% of the initial lactose has disappeared. Second, it has been

- reported that GOS containing β(13) bonds are metabolized by probiotic bacteria faster than

- similar GOS containing other linkages, in particular β(14).29

## ABBREVIATIONS

- GOS, galactooligosaccharides; HMOS, human milk oligosaccharides; ONPG, o-nitrophenyl-β-

- D-galactopyranoside; GRAS, generally recognized as safe; HPAEC-PAD, high-performance

- anion-exchange	chromatography	with	pulsed	amperometric	detection;	HPLC,	high-

- performance liquid chromatography; HILIC, hydrophilic interaction liquid chromatography;

- MS, mass spectrometry; QTOF, quadrupole time of flight; NMR, nuclear magnetic resonance;

- COSY, correlation spectroscopy; NOESY, nuclear overhauser effect spectroscopy ; TOCSY,

- total correlation spectroscopy;	HSQC, heteronuclear single quantum correlation; HMBC,

- heteronuclear multiple bond correlation; TSP-d4,  3-(trimethylsilyl)propionic-2,2,3,3-d4 acid

- sodium salt; DP, degree of polymerization.

## ACKNOWLEDGEMENTS

- We thank Ramiro Martínez (Novozymes A/S, Madrid, Spain) for supplying Saphera® and for

- useful suggestions. Vera Füreder thanks the European Union for a grant of the Erasmus+

- Program. We also thank CONACYT (Mexico) for a Ph.D. grant to Fadia V. Cervantes.

## FUNDING SOURCES

- This work was supported by a grant from the Spanish Ministry of Economy and Competitiveness

- (BIO2016-76601-C3-1). The group at CIC bioGUNE thanks Agencia Estatal de Investigación

- (Spain) for the Severo Ochoa Excellence Accreditation (SEV-2016-0644).

## REFERENCES

- 1. Plou, F. J.; Polaina, J.; Sanz-Aparicio, J.; Fernandez-Lobato, M., β-Galactosidases for lactose

- hydrolysis and galactooligosaccharide synthesis. In Microbial Enzyme Technology in Food

- Applications, Ray, R. C.; Rosell, C. M., Eds. CRC Press: 2016; pp 123-146.

- 2. Husain,  Q.,  -Galactosidases  and  their  potential  applications:  A  review.  Crit.  Rev.

312	Biotechnol. 2010, 30, 41-62.

- 3. Adam,  A.  C.;  Rubio-Texeira,  M.;  Polaina,  J.,  Lactose:  The  milk  sugar  from  a

- biotechnological perspective. Crit. Rev. Food Sci. Nutr. 2004, 44, 553-557.

- 4. Dekker,  P.  J.  T.;  Koenders,  D.;  Bruins,  M.  J.,  Lactose-free  dairy  products:  Market

- developments, production, nutrition and health benefits. Nutrients 2019, 11.

- 5. Zolnere, K.; Ciprovica, I. The comparison of commercially available ß-Galactosidases for

- dairy industry: Review. Research for Rural Development 2017, 1, 215-222.

- 6. Torres, D. P.; Goncalves, M.; Teixeira, J. A.; Rodrigues, L. R., Galacto-Oligosaccharides:

- Production, properties, applications, and significance as prebiotics. Compr. Rev. Food Sci. Food

321	Saf. 2010, 9, 438-454.

- 7. Osman, A., Synthesis of Prebiotic Galacto-Oligosaccharides: Science and Technology. In

- Probiotics, Prebiotics, and Synbiotics: Bioactive Foods in Health Promotion, Watson, R.;

- Preedy, V., Eds. Academic Press: 2015; pp 135-154.

- 8. Ruiz-Matute, A. I.; Corzo-Martinez, M.; Montilla, A.; Olano, A.; Copovi, P.; Corzo, N.,

- Presence of mono-, di- and galactooligosaccharides in commercial lactose-free UHT dairy

- products. J. Food Compos. Anal. 2012, 28, 164-169.

- 9. Marin-Navarro,  J.;  Talens-Perales,  D.;  Oude-Vrielink,  A.;  Cañada,  F.  J.;  Polaina,  J.,

- Immobilization of thermostable -galactosidase on epoxy support and its use for lactose

- hydrolysis and galactooligosaccharides biosynthesis. World J. Microbiol. Biotechnol. 2014, 30,

331	989-998.

- 10. Rodriguez-Colinas, B.; Kolida, S.; Baran, M.; Ballesteros, A. O.; Rastall, R. A.; Plou, F. J.,

- Analysis of fermentation selectivity of purified galacto-oligosaccharides by in vitro human

- faecal fermentation. Appl. Microbiol. Biotechnol. 2013, 97, 5743-5752.

- 11. Corzo, N.; Alonso, J. L.; Azpiroz, F.; Calvo, M. A.; Cirici, M.; Leis, R.; Lombo, F.; Mateos-

- Aparicio, I.; Plou, F. J.; Ruas-Madiedo, P.; Ruperez, P.; Redondo-Cuenca, A.; Sanz, M. L.;

- Clemente, A., Prebiotics: Concept, properties and beneficial effects. Nutr.Hosp. 2015, 31, 99-

338	118.

- 12. Shadid,  R.;  Haarman,  M.;  Knol,  J.;  Theis,  W.; Beermann,  C.;  Rjosk-Dendorfer,  D.;

- Schendel, D. J.; Koletzko, B. V.; Krauss-Etschmann, S., Effects of galactooligosaccharide and

- long-chain fructooligosaccharide supplementation during pregnancy on maternal and neonatal

- microbiota and immunity - a randomized, double-blind, placebo-controlled study. Am. J. Clin.

343	Nutr. 2007, 86, 1426-1437.

- 13. Lamsal, B. P., Production, health aspects and potential food uses of dairy prebiotic

- galactooligosaccharides. J. Sci. Food Agric. 2012, 92, 2020-2028.

- 14. Rastall, R. A.; Gibson, G. R.; Gill, H. S.; Guarner, F.; Klaenhammer, T. R.; Pot, B.; Reid,

- G.; Rowland, I. R.; Sanders, M. E., Modulation of the microbial ecology of the human colon by

- probiotics, prebiotics and synbiotics to enhance human health: An overview of enabling science

- and potential applications. FEMS Microbiol. Ecol. 2005, 52, 145-152.

- 15. Neri-Numa, I. A.; Pastore, G. M., Novel insights into prebiotic properties on human health:

- A review. Food Res. Int. 2020, 131.

- 16. Gosling, A.; Stevens, G. W.; Barber, A. R.; Kentish, S. E.; Gras, S. L., Recent advances

- refining galactooligosaccharide production from lactose. Food Chem. 2010, 121, 307-318.

- 17. Chen, Q.; Liu, M.; Zhang, P.; Fan, S.; Huang, J.; Yu, S.; Zhang, C.; Li, H., Fucoidan and

- galactooligosaccharides ameliorate high-fat diet–induced dyslipidemia in rats by modulating the

- gut microbiota and bile acid metabolism. Nutrition 2019, 65, 50-59.

- 18. Rodriguez-Colinas, B.; L., F.-A.; de Abreu, M.; Urrutia, P.; M., F.-L.; Ballesteros, A. O.;

- Plou, F. J., On the enzyme specificity for the synthesis of prebiotic galactooligosaccharides. In

- Advances in Enzyme Biotechnology, Shukla, P.; Pletschke; B., Eds. Springer: 2013; pp 23-39.

- 19. Iqbal, S.; Nguyen, T. H.; Nguyen, T. T.; Maischberger, T.; Haltrich, D., ß-Galactosidase

- from Lactobacillus plantarum WCFS1: Biochemical characterization and formation of prebiotic

- galacto-oligosaccharides. Carbohydr. Res. 2010, 345, 1408-1416.

- 20. Panesar, P. S.; Kaur, R.; Singh, R. S.; Kennedy, J. F., Biocatalytic strategies in the production

- of galacto-oligosaccharides and its global status. Int. J. Biol. Macromol. 2018, 111, 667-679.

- 21. Santibáñez, L.; Fernández-Arrojo, L.; Guerrero, C.; Plou, F. J.; Illanes, A., Removal of

- lactose in crude galacto-oligosaccharides by β-galactosidase from Kluyveromyces lactis. J. Mol.

367	Catal. B: Enzym. 2016, 133, 85-91.

- 22. Urrutia, P.; Rodriguez-Colinas, B.; Fernandez-Arrojo, L.; Ballesteros, A. O.; Wilson, L.;

- Illanes,  A.;  Plou,  F.  J.,  Detailed  analysis  of  galactooligosaccharides  synthesis  with  -

- galactosidase from Aspergillus oryzae. J. Agric. Food Chem. 2013, 61, 1081-1087.

- 23. Yin, H.; Bultema, J. B.; Dijkhuizen, L.; van Leeuwen, S. S., Reaction kinetics and

- galactooligosaccharide  product  profiles  of  the  β-galactosidases  from  Bacillus  circulans,

- Kluyveromyces lactis and Aspergillus oryzae. Food Chem. 2017, 225, 230-238.

- 24. Vera, C.; Córdova, A.; Aburto, C.; Guerrero, C.; Suárez, S.; Illanes, A., Synthesis and

- purification of galacto-oligosaccharides: state of the art. World J. Microbiol. Biotechnol. 2016,

376	32.

  - 25. Arreola, S. L.; Intanon, M.; Suljic, J.; Kittl, R.; Pham, N. H.; Kosma, P.; Haltrich, D.;

  - Nguyen, T. H., Two β-galactosidases from the human isolate Bifidobacterium breve DSM

  - 20213:  Molecular  cloning  and  expression,  biochemical  characterization  and  synthesis  of

  - galacto-oligosaccharides. PLoS One 2014, 9.

  - 26.  Cardelle-Cobas,  A.;  Corzo,  N.;  Olano,  A.;  Pelaez,  C.;  Requena,  T.;  Avila,  M.,

  - Galactooligosaccharides  derived  from  lactose  and  lactulose:  Influence  of  structure  on

  - Lactobacillus, Streptococcus and Bifidobacterium growth. Int. J. Food Microbiol. 2011, 149,

384	81-87.

- 27. Sanz, M. L.; Gibson, G. R.; Rastall, R. A., Influence of disaccharide structure on prebiotic

- selectivity in vitro. J. Agric. Food Chem. 2005, 53, 5192-5199.

- 28. Rabiu, B. A.; Jay, A. J.; Gibson, G. R.; Rastall, R. A., Synthesis and fermentation properties

- of novel galacto-oligosaccharides by beta-galactosidases from Bifidobacterium species. Appl.

389	Environ. Microbiol. 2001, 67, 2526-2530.

- 29. Kittibunchakul, S.; Maischberger, T.; Domig, K. J.; Kneifel, W.; Nguyen, H. M.; Haltrich,

- D.; Nguyen, T. H., Fermentability of a novel galacto-oligosaccharide mixture by Lactobacillus

- spp. and Bifidobacterium spp. Molecules 2018, 23, 3352.

- 30. Viborg, A. H., Diversity in β-galactosidase specificities within bifidobacterium: Towards an

- understanding of β-galactoside metabolism in the gut niche. Trends Glycosci. Glycotechnol.

395	2015, 27, J9-J12.

- 31. Zivkovic, A. M.; German, J. B.; Lebrilla, C. B.; Mills, D. A., Human milk glycobiome and

- its impact on the infant gastrointestinal microbiota. Proc. Natl. Acad. Sci.U.S.A. 2011, 108,

398	4653-4658.

- 32.  Ferreira-Lazarte,  A.;  Gallego-Lobillo,  P.;  Moreno,  F.  J.;  Villamiel,  M.;  Hernandez-

- Hernandez, O., In vitro digestibility of galactooligosaccharides: Effect of the structural features

- on their intestinal degradation. J. Agric. Food Chem. 2019, 67, 4662-4670.

- 33. Rodriguez-Colinas, B.; Fernandez-Arrojo, L.; Santos-Moriano, P.; Ballesteros, A. O.; Plou,

- F. J., Continuous packed bed reactor with immobilized β-galactosidase for production of

- galactooligosaccharides (GOS). Catalysts 2016, 6, 189.

- 34. Tzortzis, G.; Goulas, A. K.; Gibson, G. R., Synthesis of prebiotic galactooligosaccharides

- using whole cells of a novel strain, Bifidobacterium bifidum NCIMB 41171. Appl. Microbiol.

407	Biotechnol. 2005, 68, 412-416.

408	35. Hsu, C. A.; Lee, S. L.; Chou, C. C., Enzymatic production of galactooligosaccharides by

409	beta-galactosidase from Bifidobacterium longum BCRC 15708. J. Agric. Food Chem. 2007, 55,

410	2225-2230.

- 36. Hatzinikolaou, D. G.; Katsifas, E.; Mamma, D.; Karagouni, A. D.; Christakopoulos, P.;

- Kekos, D., Modeling of the simultaneous hydrolysis-ultrafiltration of whey permeate by a

- thermostable β-galactosidase from Aspergillus niger. Biochem. Eng. J. 2005, 24, 161-172.

- 37. Rodriguez-Colinas, B.; Poveda, A.; Jimenez-Barbero, J.; Ballesteros, A. O.; Plou, F. J.,

- Galacto-oligosaccharide synthesis from lactose solution or skim milk using the -galactosidase

- from Bacillus circulans. J. Agric. Food Chem. 2012, 60, 6391-6398.

- 38. Rodriguez-Colinas, B.; De Abreu, M. A.; Fernandez-Arrojo, L.; De Beer, R.; Poveda, A.;

- Jimenez-Barbero, J.; Haltrich, D.; Ballesteros , A. O.; Fernandez-Lobato, M.; Plou, F. J.,

- Production of galacto-oligosaccharides by the -galactosidase from Kluyveromyces lactis:

- Comparative analysis of permeabilized cells versus soluble enzyme. J. Agric. Food Chem. 2011,

421	59, 10477-10484.

422	39. Britton, H. T. S.; Robinson, R. A., Universal buffer solutions and the dissociation constant

423	of veronal. J. Chem. Soc. 1931, 1456-1462.

424	40. Goulas, A.; Tzortzis, G.; Gibson, G. R., Development of a process for the production and

425	purification of alpha- and beta-galactooligosaccharides from Bifobacterium bifidum NCIMB

426	41171. Int. Dairy J. 2007, 17, 648-656.

- 41. Petzelbauer, I.; Splechtna, B.; Nidetzky, B., Development of an ultrahigh-temperature

- process for the enzymatic hydrolysis of lactose. III. Utilization of two thermostable beta-

- glycosidases in a continuous ultrafiltration membrane reactor and galacto-oligosaccharide

- formation under steady-state conditions. Biotechnol. Bioeng. 2002, 77, 394-404.

- 42.	Rodriguez-Colinas,	B.;	Fernandez-Arrojo,	L.;	Ballesteros,	A.;	Plou,	F.,

- Galactooligosaccharides formation during enzymatic hydrolysis of lactose: towards a prebiotic-

- enriched milk. Food Chem. 2014, 145, 388-394.

- 43. Yin, H.; Pijning, T.; Meng, X.; Dijkhuizen, L.; Van Leeuwen, S. S., Engineering of the

- Bacillus circulans β-galactosidase Product Specificity. Biochemistry 2017, 56, 704-711.

- 44. Van Leeuwen, S. S.; Kuipers, B. J. H.; Dijkhuizen, L.; Kamerling, J. P., Comparative

- structural	characterization	of	7	commercial	galacto-oligosaccharide	(GOS)	products.

438	Carbohydr. Res. 2016, 425, 48-58.

439	45. Van Leeuwen, S. S.; Kuipers, B. J. H.; Dijkhuizen, L.; Kamerling, J. P., 1 H NMR analysis

440	of the lactose/β-galactosidase-derived galacto-oligosaccharide components of Vivinal GOS up

441	to DP5. Carbohydr. Res. 2014, 400, 59-73.

- 46. Van Leeuwen, S. S.; Kuipers, B. J. H.; Dijkhuizen, L.; Kamerling, J. P., Development of a

- 1H-NMR	structural-reporter-group	concept	for	the	analysis	of	prebiotic	galacto-

- oligosaccharides of the [β-D-Galp-(1→x)]n-D-Glcp type. Carbohydr. Res. 2014, 400, 54-58.

- 47.  Yañez-Ñeco,  C.  V.;  Rodriguez-Colinas,  B.;  Amaya-Delgado,  L.;  Ballesteros,  A.  O.;

- Gschaedler, A.; Plou, F. J.; Arrizon, J., Galactooligosaccharide production from Pantoea

- anthophila strains isolated from “Tejuino”, a Mexican traditional fermented beverage. Catalysts

448	2017, 7, 242.

449	48. Depeint, F.; Tzortzis, G.; Vulevic, J.; I'Anson, K.; Gibson, G. R., Prebiotic evaluation of a

450	novel galactooligosaccharide mixture produced by the enzymatic activity of Bifidobacterium

- bifidum NCIMB 41171, in healthy humans: A randomized, double-blind, crossover, placebo-

- controlled intervention study. Am. J. Clin. Nutr. 2008, 87, 785-791.

- 49. Goulas, T.; Goulas, A.; Tzortzis, G.; Gibson, G. R., Expression of four β-galactosidases

- from Bifidobacterium bifidum NCIMB41171 and their contribution on the hydrolysis and

- synthesis of galactooligosaccharides. Appl. Microbiol. Biotechnol. 2009, 84, 899-907.

- 50.Plou,  F.  J.;  Gómez  de  Segura,  A.;  Ballesteros,  A.,  Application  of  glycosidases  and

- transglycosidases  for  the  synthesis  of  oligosaccharides.  In  Industrial  enzymes:  Structure,

- Function and Application, Polaina, J.; MacCabe, A. P., Eds. Springer: New York, 2007; pp 141-

459	157.

460	51. Buchholz, K.; Kasche, V.; Bornscheuer, U. T., Biocatalysts and Enzyme Technology. Wiley-

461	VCH Verlag: Weinheim, 2005.

462	52. Torres, P.; Poveda, A.; Jimenez-Barbero, J.; Parra, J. L.; Comelles, F.; Ballesteros, A. O.;

463	Plou, F. J., Enzymatic synthesis of α-glucosides of resveratrol with surfactant activity. Adv.

464	Synth. Catal. 2011, 353, 1077-1086.

465	53. Böger, M.; Van Leeuwen, S. S.; Lammerts Van Bueren, A.; Dijkhuizen, L., Structural

466	identity of galactooligosaccharide molecules selectively utilized by single cultures of probiotic

467	bacterial strains. J. Agric. Food Chem. 2019, 67, 13969-13977.

468

## FIGURE CAPTIONS

- Figure 1. Effect of temperature on a) activity and b) stability of β-galactosidase from B. bifidum.

- For the activity assay, 0.1 M phosphate buffer (pH 6.8) was used. For the thermostability assay,

- the enzyme was incubated in the same buffer at different temperatures for 1 h. Residual activity

- was referred to the initial activity of the enzyme.

- Figure 2. Effect of pH on a) activity and b) stability of β-galactosidase from B. bifidum. For the

- activity assay, 0.1 M Britton & Robinson buffer at different pH was used and the activity

- measured at 40 ˚C. For the stability assay, the enzyme solution was incubated under different

- pH values at 40 ˚C for 3 h. Residual activity was referred to the initial activity of the enzyme,

- prior to any incubation, using the standard activity assay.

- Figure 3. HPAEC-PAD analysis of the reaction of lactose with B. bifidum β-galactosidase

- (Saphera) under optimal conditions (pH 6.0, 40 °C) at two lactose concentrations: a) 160 g/L;

- b) 400 g/L. Peaks assignation: (1) Galactose; (2) Glucose; (3) 6-Galactobiose; (4) Allolactose;

- (5) Lactose; (6) 3-Galactobiose; (7) 4-Galactobiose; (8) 6’-Galactosyl-lactose; (9) 3-O-β-

- Galactosyl-glucose; (12) 4´-Galactosyl-lactose; (13) Gal-β(13)-Gal-β(13)-Glc; (14) 3’-

- Galactosyl-allolactose;   (15)   3´-Galactosyl-lactose;   (19)   3´-O-β-(3-Galactobiosyl)-lactose;

485	(10,11,16-18) unidentified.

- Figure 4. ESI-TOF mass spectra (positive mode) of: A) Peak 15; B) Peak 19.

- Figure 5. 1H-NMR spectra of the reaction products corresponding to peaks 13, 15 and 19. The

- major product for peak 19 (A) corresponds to the tetrasaccharide Gal-β(1→3)-Gal-β(1→3)-Gal-

- β(1→4)-Glc(α/β). For peak 15 (B) the major product is Gal-β(1→3)-Gal-β(1→4)-Glc(α/β). For

- peak 13 (C) a mixture of trisaccharides (according to MS) containing all the possibilities for

- Gal-β(1→3/1→4)-Gal-β(1→4/1→3/1→2)-Glc(α/β) is obtained. For the three peaks, small

- amounts of oligosaccharides with the reducing Glc residue glycosylated at position 2 have been

- found (signals at 5.4-5.5 ppm).

- Figure  6.  GOS  formation  vs.  lactose  conversion  using  β-galactosidase  from  B.  bifidum

- (Saphera). Experimental conditions: 400 g/L lactose in 0.1 M sodium phosphate buffer (pH 6.0),

- 1.5 U/ml (ONPG units), 40 ºC.

Page 25 of 32

Table 1. Carbohydrate composition (g/L) of the reaction mixture using β-galactosidase from B. bifidum (Saphera). Experimental conditions: 400 g/L lactose in 0.1 M sodium phosphate buffer (pH 6.0), 1.5 U/ml, 40 ºC.

| Reaction time Gal (min) (g/L) |  | Glc (g/L) | Lact (g/L) | Gal-6-Gal (g/L) | Gal-6-Glc (g/L) | Gal-3-Gal (g/L) | Gal-4-Gal (g/L) | Gal-3-Glc (g/L) | Gal-4-Lact (g/L) | Gal-3-Lact (g/L) | Gal-3-Gal-3-Lact (g/L) | Other GOS (g/L) | GOS TOT (g/L) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 0.0 | 0.0 | 400.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 |
| 10 | 4.1 | 4.6 | 390.4 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 0.0 | 1.0 | 0.0 | 0.0 | 1.0 |
| 20 | 6.6 | 8.0 | 382.7 | 0.0 | 0.0 | 0.0 | 0.0 | 0.1 | 0.6 | 1.6 | 0.1 | 0.3 | 2.7 |
| 30 | 9.7 | 11.6 | 374.9 | 0.0 | 0.0 | 0.0 | 0.0 | 0.3 | 0.7 | 2.3 | 0.1 | 0.4 | 3.8 |
| 60 | 16.1 | 20.8 | 355.8 | 0.0 | 0.0 | 0.0 | 0.0 | 0.8 | 1.3 | 3.8 | 0.3 | 1.2 | 7.3 |
| 120 | 25.4 | 33.0 | 326.9 | 0.0 | 0.0 | 0.1 | 0.1 | 2.2 | 2.0 | 6.6 | 0.8 | 3.0 | 14.7 |
| 180 | 30.6 | 41.7 | 306.1 | 0.0 | 0.0 | 0.0 | 0.2 | 3.8 | 2.5 | 8.7 | 1.2 | 5.2 | 21.6 |
| 240 | 34.6 | 49.1 | 285.3 | 0.0 | 0.0 | 0.2 | 0.3 | 6.0 | 3.3 | 11.2 | 1.9 | 8.1 | 31.0 |
| 300 | 38.8 | 55.3 | 272.4 | 0.0 | 0.0 | 0.2 | 0.4 | 6.5 | 3.2 | 11.8 | 2.2 | 9.2 | 33.5 |
| 390 | 40.9 | 59.6 | 241.4 | 0.0 | 0.6 | 0.4 | 0.5 | 7.6 | 3.5 | 12.2 | 2.6 | 30.7 | 58.1 |
| 1320 | 74.1 | 117.5 | 117.5 | 1.1 | 4.8 | 3.2 | 2.3 | 12.1 | 4.7 | 20.8 | 6.3 | 35.6 | 90.9 |
| 1830 | 84.6 | 131.2 | 78.2 | 1.4 | 7.5 | 4.6 | 3.2 | 13.4 | 4.1 | 20.7 | 7.1 | 44.0 | 105.9 |
| 3030 | 103.4 | 152.0 | 37.4 | 2.2 | 16.2 | 8.2 | 4.8 | 9.0 | 2.6 | 17.7 | 5.4 | 41.1 | 107.2 |
| 4200 | 109.2 | 158.8 | 27.7 | 2.8 | 23.3 | 10.9 | 5.5 | 7.9 | 1.8 | 12.2 | 3.5 | 36.3 | 104.3 |

Gal: Galactose; Glc: Glucose; Lact: Lactose; Gal-6-Gal: 6-galactobiose; Gal-6-Glc: Allolactose; Gal-3-Gal: 3-galactobiose; Gal-4-Gal: 4-galactobiose; Gal-3-Glc: 3-galactosyl-glucose; Gal-4-Lact: 4’- galactosyl-lactose; Gal-3-Lact: 3’-galactosyl-lactose; Gal-3-Gal-3-Lact: 3´-(3-galactobiosyl)-lactose; GOSTOT: Total GOS in the mixture.

ACS Paragon Plus Environment

Figure 1. Effect of temperature on a) activity and b) stability of β-galactosidase from B. bifidum. For the activity assay, 0.1 M phosphate buffer (pH 6.8) was used. For the thermostability assay, the enzyme was incubated in the same buffer at different temperatures for 1 h. Residual activity was referred to the initial activity of the enzyme.

149x228mm (300 x 300 DPI)

Figure 2. Effect of pH on a) activity and b) stability of β-galactosidase from B. bifidum. For the activity assay, 0.1 M Britton & Robinson buffer at different pH was used and the activity measured at 40 ˚C. For the stability assay, the enzyme solution was incubated under different pH values at 40 ˚C for 3 h. Residual activity was referred to the initial activity of the enzyme, prior to any incubation, using the standard activity assay.

149x225mm (300 x 300 DPI)

Figure 3. HPAEC-PAD analysis of the reaction of lactose with B. bifidum β-galactosidase (Saphera) under optimal conditions (pH 6.0, 40 °C) at two lactose concentrations: a) 160 g/L; b) 400 g/L. Peaks assignation:

  - Galactose; (2) Glucose; (3) 6-Galactobiose; (4) Allolactose; (5) Lactose; (6) 3-Galactobiose; (7) 4- Galactobiose; (8) 6’-Galactosyl-lactose; (9) 3-O-β-Galactosyl-glucose; (12) 4´-Galactosyl-lactose; (13) Gal-

β(1→3)-Gal-β(1→3)-Glc; (14) 3’-Galactosyl-allolactose; (15) 3´-Galactosyl-lactose; (19) 3´-O-β-(3-

Galactobiosyl)-lactose; (10,11,16-18) unidentified.

186x189mm (300 x 300 DPI)

Figure 4. ESI-TOF mass spectra (positive mode) of: A) Peak 15; B) Peak 19.

149x212mm (300 x 300 DPI)

Figure 5. 1H-NMR spectra of the reaction products corresponding to peaks 13, 15 and 19. The major product for peak 19 (A) corresponds to the tetrasaccharide Gal-β(1→3)-Gal-β(1→3)-Gal-β(1→4)-Glc(α/β). For peak 15 (B) the major product is Gal-β(1→3)-Gal-β(1→4)-Glc(α/β). For peak 13 (C) a mixture of trisaccharides (according to MS) containing all the possibilities for Gal-β(1→3/1→4)-Gal-β(1→4/1→3/1→2)-Glc(α/β) is obtained. For the three peaks, small amounts of oligosaccharides with the reducing Glc residue glycosylated at position 2 have been found (signals at 5.4-5.5 ppm).

169x121mm (300 x 300 DPI)

Figure 6. GOS formation vs. lactose conversion using β-galactosidase from B. bifidum (Saphera).

Experimental conditions: 400 g/L lactose in 0.1 M sodium phosphate buffer (pH 6.0), 1.5 U/ml (ONPG units), 40 ºC.

147x118mm (300 x 300 DPI)