Waveform Data File Information Sheet

Upload link: https://zenodo.org/communities/supraemi/
(Log-in to Zenodo first, otherwise the link above won’t go to the SupraEMI  site)

Details of Waveform Data

| Waveform Filename: | Desktoppowersupply_CPU40percentload_spectra_9kHz_V1.csv |
| --- | --- |
| Waveform Description: | The CPU of the desktop is loaded with 40 % of the maximum capacity. The supraharmonic spectra of the power supply is recorded up to 9 kHz according to the IEC 61000-4-7:2002+A1:2009 Annex B. In the csv file, each spectral component is labelled according the starting frequency of the 200 Hz band, i.e. V_Y2600 corresponds to the band 2.6 kHz to 2.8 kHz for voltage and I_Y8800 corresponds to the band 8.8 kHz to 9 kHz for current. |
| Date of Waveform Recording/Synthesis: | 05 Nov. 2020 |
| Author/Owner: | VSL |
| Funding Source Acknowledgement: | SupraEMI |

Details of Data file (All data files should be plain ASCII text or CSV files)

| Data file column format: |  |
| --- | --- |
| (e.g.: dd/mm/yy, hh:mm:ss:nn, VLine0, VLine1, VLine2, ILine0, Iline1, ILine2) |  |
| hh:mm:ss:nn, hh:mm:ss:nn,V_2600Hz…V__8800Hz,I_2600Hz…..I_8800Hz |  |
| Column Delimiter: (e.g. comma) | comma |
| End of line Delimiter: (if known, e.g. carriage return, line-feed) | CR LF |
| Sampling rate (samples per second): | 400000 samples/second |
| Y-axis units (e.g. kV): | V, A |
| Y-axis scaling factor (for each column): | 1, 1 |
| Y-axis resolution: | 32 bit = 1 part in 4294967296 |

Detail of Recording Instrumentation (if applicable)

| 3dB Bandwidth of recording instrument and associated transducer(s): | DC to > 150 kHz |
| --- | --- |
| Was an anti-aliasing filter used with the recording instrument?: | No |
| Manufacturer and model number(s) | NI PXIe-6356 Digitizer VSL self-made voltage divider HIOKI Current sensor CT6862-05 |

Details of public power grid or laboratory power source (if applicable)

| Supply source: (public power grid or steady frequency power amplifier etc.): | a LISN (Line impedance stabilization network) or AMN (artificial mains network) as defined in IEC 61000-4-7 is used. |
| --- | --- |
| Nominal frequency of supply source (Hz): | 50Hz |
| Nominal voltage level of supply source (V rms): | 230V |

When Uploading please note: It is the responsibility of the person uploading the data to ensure that they have full permission to publish the data.  The specific-location, times and details of the recording site should not be disclosed without the site/equipment/data-owner’s permission.  Trade-names should not be disclosed if they are likely to be detrimental to a manufacturer in any way. Ensure that all international data-protection and copyright laws are complied with.

When Downloading and using the data please note: This data is supplied for research purposes. If this data is used the support any form of public-domain publication, the source should be fully acknowledged and cited using the associated DOI and data owners details.  The publishers and data owners take no responsibility for any conclusions/decisions made from using the data.