Online supplementary information for:

A kinetic reaction model for biomass pyrolysis processes in Aspen Plus and its experimental validation

Jens F. Peters1,2, Scott W. Banks3, Anthony V. Bridgwater3, Javier Dufour4,5

1 KIT-ITAS, Karlsruhe Institute for Technology. Karlsruhe (Germany);

2 HIU, Helmholtz-Institute Ulm. Ulm (Germany)

3 EBRI European Bioenergy Research Institute, Aston University. Birmingham (UK).

4 Department of Chemical and Energy Technology, Rey Juan Carlos University. Móstoles (Spain).

5 Systems Analysis Unit, Instituto IMDEA Energía. Móstoles (Spain).

This SI document includes the complete set of kinetic reactions implemented in the reaction model for the primary pyrolysis reactions, the implemented algorithm for calculating the secondary pyrolysis reactions and a list of all chemical compounds (also intermediate compounds) and their atomic structure used for modelling the pyrolysis reactions and products.

# Decomposition reactions

The decomposition reaction is implemented in Aspen Plus in an RYield-type reactor. The yields are calculated iteratively by an embedded Excel worksheet which determines the lignin composition of the biomass according to its elemental composition. The calculation is done via an iterative distance-to-target approach by determining in each iteration step the discrepancy between the atomic composition of the biomass and the atomic composition of the decomposition products. While this discrepancy is above a certain threshold, the calculation is repeated, increasing the compounds whose elemental composition is closer to the target (the biomass feed composition), while reducing those with a higher distance. The procedure in detail is as follows:

- Determine elemental composition (wt%) of the biomass (EC_bm)

- Set equal (initial default) distribution of the different lignin monomers (equal shares)

- Calculate elemental composition (wt%) for the (sum of the) decomposition products (EC_dec; including the cellulose and hemicellulose fractions), and determine the discrepancy between input (biomass feed) and output (decomposition products)

- While EC_dec ≠ EC_bm, REPEAT (for each of the lignin model compounds):

  - Calculate elemental composition (wt%) of the compound (EC_comp)

  - Calculate the distance to target (DtT) from its H / O / C content (EC_comp) towards the H / O / C  composition  of the feed biomass (EC_bm)

  - Calculate an incremental factor (iF) for each of the elements: iF = DtT * EC_comp * multFact. / molweight (C=12, H=1, O=16)

  - Sum the incremental factors to a sum-factor for each compound

  - Normalize the sum-factors for all compounds to 1

  - Adjust the amount of each lignin compound acc. to the normalised sum-factors

The multiplication factor (multFact) can be set freely; higher values converge quicker, but increase the risk of “overshooting”. Default value: 200

Example (for lignin compound “ Lignin C”):

- Biomass feed: EC_bm =  51% C, 6.3% H, 42% O

- Initial distribution: 14 / 15% share of each lignin model compounds

  - EC_comp (Lignin C) =  69.8% C, 5.5% H, 24.8% O

  - DtT (C): 18.83%; DtT (H): -0.87%; DtT (O): -16.77%

  - iF (C): 18.83% (DtT (C)) * 69.8% (EC_comp (C)) * 200 / 12 = -0.03858717
iF (H): -0.87% (DtT (H)) * 5.5% (EC_comp (H)) * 200 / 1 = -0.004721288
iF (O): -16.77% (DtT (O)) * 24.8% (EC_comp (O)) * 200 / 16 = -0.020103408

  - sum-factor = iF(C) + iF(H) + iF(O) = 0.936588134

  - Normalised sum-factor: 0.963504343

  - Multiply yield of Lignin C with 0.963504343 for the next iteration step; and so on for all other lignin model compounds

# Primary pyrolysis reactions (kinetic reaction model)

The pyrolysis model is implemented as an interlinked kinetic model of individual decomposition reactions of cellulose, hemicellulose and lignins, according to Miller & Bellan [1] and Di Blasi [2]. A good review of kinetic model schemes for pyrolysis reactions is given by C. Gómez Díaz in her thesis [3]. The kinetic reaction schemes are implemented as Power Law type kinetic expressions with the reaction rate calculated in AspenPlus by Equation (S1).

r = k*Tn*e-E/RT 	Equation (S1)

With r being the rate of reaction,  k the pre-exponential factor, T the absolute temperature, E the activation energy and R the gas law constant

The complete set of kinetic reactions implemented in the reaction model is given in this chapter. Compounds with a name beginning with “1P” are fractional species, which are normally part (building block) of a bigger organic molecule. Species with a name beginning with “1R” are radical intermediate species. All these species are only used as intermediate products for modelling the reaction mechanisms within the pyrolysis reactor and are not part of the final pyrolysis reaction product. For them, estimation of all properties was not required therefore. All used compounds are listed with their formula and, if required, their atomic structure in Chapter S4.

## Reactions with stochiometry and kinetics taken from literature

This section contains reactions for which both the stoichiometry and the kinetic factors are taken from literature. For some of the reactions the kinetic parameters are adjusted slightly to better fit the results published in literature.

## Primary cellulose decomposition

Primary cellulose decomposition reactions are mainly taken from Ranzi et al. [4], in few cases kinetic factors are adjusted to fit the reaction mechanism to experimental data from other works, mainly Zhang et al.  [5]:

- CELLULOS -->  CELL-ACT      	k=8e13, E=52900kcal·kmol-1

- CELL-ACT -->  .95 GLYCOALD +  .25 GLYOXAL +  .2 ACETALDY +  .25 HDRMTFUR +  .16 CO2 +  .23 CO +  .9 WATER +  .1 CH4 +  .61 C +  .2 ACETONE      	k=1e9, E=30000kcal·kmol-1

- CELL-ACT  -->  LEVOGLUC  	k=4,n=1,E=10000kcal·kmol-1

- CELLULOS  -->  5 WATER +  6 C     	k=8e7, E=36800kcal·kmol-1

- HEMICELL  -->  .4 HEMCELL1  +  .6 HEMCELL2      	k=1e10, E=31000kcal·kmol-1

- HEMCELL1  -->  1.25 H2 +  .5 CO2 +  CO +  .5 FORMALDY +  .25 METHANOL +  .125 ETHANOL +  1.125 WATER +  2.3 C  +  .075 ETHENE +  .05 CH4.      	k=3e9, E=2700kcal·kmol-1

- HEMCELL1  -->  XYLOSE        	k=3,n=1,E=11000kcal·kmol-1

- HEMCELL2  -->  .75 CO2 +  .3 CH4 +  .5 CO +  .2 H2 +  .3 FORMALDY +  .25 METHANOL +  .125 ETHANOL +  1.525 WATER +  1.35 C  +  .05 ETHENE +  .2 ETYLDIOL +  .4 ACETALDY       	k=1e10, E=3300kcal·kmol-1

- HEMCELL2  -->  ACETICAC +  1.2 WATER +  .22 1PC10CHR  +  .2 FORMALDY +  .15 FORMICAC +  .15 PROPNCAC     	k=8e9, E=33000kcal·kmol-1

## Cellulose secondary reactions

Cellulose secondary reactions are based mainly on the additional information which can be found on the webpage of the CRECK modeling group [6], with some modifications of the kinetic parameters for obtaining a better fit to experimental data. The complete extense reaction model is published under www.creckmodeling.chem.polimi.it and is the basis for the work of Ranzi et al. or Faravelli et al. [7]

- LEVOGLUC -->  HDRMTFUR +  2 WATER   	k=1.25e12, E=60000kcal·kmol-1

- HDRMTFUR -->  FURFURAL +  FORMALDY   	k=5e12, E=59000kcal·kmol-1

- PROPANOL -->  ETHANE +  FORMALDY    	k=1e13, E=59000kcal·kmol-1

- GLYCOALD -->  2 FORMALDY    	k=5e12, E=60000kcal·kmol-1

- GLYCOALD -->  1PCH2OH +  1PCHO    	k=3e16, E=82000kcal·kmol-1

- PROPANOL -->  PROPENE +  WATER   	k=1e12, E=59000kcal·kmol-1

- ACETICAC -->  2 FORMALDE    	k=5e12, E=60000kcal·kmol-1

- ETYLDIOL -->  FORMALDY +  METHANOL   	k=1e13, E=59000kcal·kmol-1

- ETYLDIOL -->  WATER +  ACETALDY   	k=5e12, E=59000kcal·kmol-1

- 1ROH +  FORMALDY -->  1RH +  H2 +  CO2   	k=1e13, E=10000kcal·kmol-1

## Lignin decomposition and secondary reactions

Lignin decomposition and secondary reactions are based mainly on the work of Faravelli et al. [7]. For some of the reactions, the kinetic parameters are adjusted slightly to better fit the results published in literature, while in other cases light modifications of the stoichiometry are made in order to fit the reactions to the defined model compounds.

Initial decomposition reactions

- 1LIGH  -->  PROPENE +  1ROH +  1RLIGM2A      	k=1e13, E=163245 kJ·kmol-1

- LIGNIN-H  -->  1PRLIGH  +  1RH      	k=1e13, E=163245 kJ·kmol-1

- 1LIGM2  -->  1RPHOXM2 +  1RADIOM2      	k=1e13, E=163245 kJ·kmol-1

- LIGN-OH  -->  1PRLGM2A  +  1RH      	k=1e13, E=163245 kJ·kmol-1

- LIGN-OH  +  1RH -->  1RPHOXM2 +  1RADIOM2      	k=1e13, E=163245 kJ·kmol-1

- 1LIG  -->  1RPHENOX +  1RADIO     	k=1e13, E=184184 kJ·kmol-1

- LIGNIN-C  -->  1PRLIG-A  +  1RH      	k=1e13, E=184184 kJ·kmol-1

- LIGNIN-C  -->  1RPHENOX +  1PRADIO      	k=1e13, E=188370 kJ·kmol-1

- 1PADIOM2 -->  1RADIOM2      	k=1e13, E=171626 kJ·kmol-1

- 1PADIO -->  1PRADIO +  1RH      	k=1e13, E=179998 kJ·kmol-1

- 1PKETM2 -->  1PRKETM2 +  1RH      	k=1e13, E=167440 kJ·kmol-1

- 1PLIGC  -->  LIGNIN-C  +  KETEN-01      	k=1e8, E= 121394 kJ·kmol-1

- LIGNIN-O  -->  LIGN-OH  +  CO2      	k=1e9, E= 108836 kJ·kmol-1

Secondary decomposition reactions

- 1RPHOXM2 -->  CO +  .5 1C10H2M4 +  0.5 H2      	k=4e10, E=209300 kJ·kmol-1

- 1RPHENOX -->  .5 1C10H2 +  CO +  1.5 H2      	k=4e10, E=209300 kJ·kmol-1

- 1PRKETM2 +  1RH -->  1KETDM2 +  1ROH      	k=1e13, E=121394 kJ·kmol-1

- 1PRLIGH  -->  1PRLGM2A  +  ACETONE      	k=1e13, E=133952 kJ·kmol-1

- 1RADIOM2 -->  1RGUAI +  PROPNCAC      	k=1e13, E=133952 kJ·kmol-1

- 1RLIGM2A  -->  1KETM2 +  1RPHOXM2      	k=5e12, E=133952 kJ·kmol-1

- 1PRLGM2A  -->  1PKETM2 +  1RPHOXM2      	k=5e12, E=133952 kJ·kmol-1

- 1RLIGM2A  -->  1RGUAI +  1PFET3M      	k=1e13, E=163254 kJ·kmol-1

- 1PRFETM2 -->  1RPHOXM2 +  PROPDIAL      	k=1e13, E=133952 kJ·kmol-1

- 1RADIO -->  1RPHENOL +  PROPNCAC      	k=1e13, E=153254 kJ·kmol-1

- 1RLIG-A  -->  1KET +  1RPHENOX     	k=1e13, E=138138 kJ·kmol-1

- 1RLIG-A  -->  1RPHENOL +  1PFET3      	k=1e13, E=163254 kJ·kmol-1

- 1PRFET3 -->  1RPHENOX +  PROPDIAL      	k=1e13, E=138138 kJ·kmol-1

- 1RADIOM2 -->  1ROH +  SINPYALC      	k=3e11, E=104650 kJ·kmol-1

- 1RKETM2 -->  1ROH +  1KETDM2      	k=3e11, E=104650 kJ·kmol-1

- 1RADIO -->  1ROH +  CMRYLALC      	k=3e11, E=104650 kJ·kmol-1

- 1RKET -->  1ROH +  1KETD      	k=3e11, E=113022 kJ·kmol-1

- 1RC3H7O2 -->  1RMETNOL +  ACETALDY      	k=1e13, E=129766 kJ·kmol-1

- 0.5 1C10H2M4 -->  1RMETNOL +  1RPHENOX      	k=1e13, E=196742 kJ·kmol-1

- 1C10H2M2 -->  1RMETNOL +  0.5 1C10H2 +  1RPHENOX      	k=1e13, E=196742 kJ·kmol-1

H- abstraction reactions

- .5 1C10H2 +  1RPHENOL -->  PHENOL +  .5 1PC10CHR     	k=1e8, E= 54418 kJ·kmol-1

- 1RGUAI +  1KETM2 -->  1MGUAIAC +  1RKETM2    	k=1e12, E= 54418 kJ·kmol-1

- 1RGUAI +  1RH -->  1MGUAIAC    	k=1e12, E= 60000 kJ·kmol-1

Substitutive addition reactions

- 1ADIOM2 +  1RPHOXM2 -->  1C10H2M4 +  .5 H2 +  2 1PH2  +  2 1PCOH  +  1RC3H7O2 	k=1e9, E= 121394kJ·kmol-1

- 1KETM2 +  1RPHOXM2 -->  1C10H2M4 +  .5 H2 +  2 1PH2  +  2 1PCOH  +  1RPRPACD	k=1e9, E= 121394 kJ·kmol-1

- 1KETDM2 +  1RPHOXM2 -->  1C10H2M4 +  .5 H2 +  2 1PH2  +  2 1PCOH  +  1RC3H3O    	k=1e9, E= 121394 kJ·kmol-1

- SINPYALC +  1RPHOXM2 -->  1C10H2M4 +  H2 +  3 1PH2  +  2 1PCOH  +  1RC3H3O    	k=1e9, E= 121394 kJ·kmol-1

- 1ADIOM2 +  1RPHENOX -->  .5 1C10H2M4 +  .5 1C10H2  +  1.5 H2 +  2 1PH2  +  1PCOS  +  1PCOH  +  1RC3H7O2   	k=1e9, E= 121394 kJ·kmol-1

- 1KETM2 +  1RPHENOX -->  .5 1C10H2M4 +  .5 1C10H2  +  .5 H2 +  4 1PH2  +  2 1PCOS  +  1RPRPACD 		k=1e9, E= 121394 ·kmol-1

- 1KETDM2 +  1RPHENOX -->  .5 1C10H2M4 +  .5 1C10H2 +  .5 H2 +  2 1PH2 +  1PCOS +  1PCOH +  1RC3H3O    		k=1e9, E= 121394 kJ·kmol-1

- SINPYALC +  1RPHOXM2 -->  1C10H2M4 +  H2 +  3 1PH2  +  2 1PCOH  +  1RC3H3O    	k=1e9, E= 121394 kJ·kmol-1

- 1ADIO +  1RPHENOX -->  1C10H2 +  1.5 H2 +  4 1PH2 +  2 1PCOS +  1RC3H7O2    	k=1e9, E= 121394 kJ·kmol-1

- 1KET +  1RPHENOX -->  1C10H2 +  1.5 H2 +  4 1PH2 +  2 1PCOS +  1RPRPACD    	k=1e9, E= 121394 kJ·kmol-1

- 1KETD +  1RPHENOX -->  1C10H2 +  1.5 H2 +  4 1PH2 +  2 1PCOS +  1RC3H3O    	k=1e9, E= 121394 kJ·kmol-1

- CMRYLALC +  1RPHENOX -->  1C10H2 +  2.5 H2 +  4 1PH2 +  2 1PCOS +  1RC3H3O   	k=1e9, E= 121394 kJ·kmol-1

- 1ADIO +  1RPHOXM2 -->  .5 1C10H2M4 +  .5 1C10H2 +  1.5 H2 +  2PH2 +  1PCOS +  1PCOH +  1RC3H7O2    		k=1e9, E= 108836 kJ·kmol-1

- 1KET +  1RPHOXM2 -->  .5 1C10H2M4 +  .5 1C10H2 +  1.5 H2 +  2PH2 +  1PCOS +  1PCOH +  1RPRPACD    		k=1e9, E= 108836 kJ·kmol-1

- 1KETD +  1RPHOXM2 -->  .5 1C10H2M4 +  .5 1C10H2 +  1.5 H2 +  2PH2 +  1PCOS +  1PCOH +  1RC3H3O    		k=1e9, E= 108836 kJ·kmol-1

- CMRYLALC +  1RPHOXM2 -->  .5 1C10H2M4 +  .5 1C10H2 +  1.5 H2 +  4 1PH2 +  1PCOS +  1PCOH +  1RC3H3O    	k=1e9, E= 115115 kJ·kmol-1

- 1RPHOXM2 +  .5 1C10H2M4 -->  1C10H2M2 +  1PCHO +  1.5 H2 +  1RMETNOL +  1PCOH 	k=1e9, E= 115115 kJ·kmol-1

- .5 1C10H2M2 +  1RPHOXM2 -->  .5 1C10H2 +  .5 H2 +  1RMETNOL +  .5 1C10H2M4 +  1PCOH    			k=1e9, E= 115115 kJ·kmol-1

- .5 1C10H2M2 +  1RPHENOX -->  1C10H2 +  1.5 H2 +  1RMETNOL +  1PCOS    	k=1e9, E= 117208 kJ·kmol-1

- .5 1C10H2M4 +  1RPHENOX -->  .5 1C10H2 +  1.5 H2 +  1RMETNOL +  1PCOS +  .5 1C10H2M2    			k=1e9, E= 117208 kJ·kmol-1

Recomposition reactions

- 1RMETNOL +  1RPHENOX -->  CO2 +  .5 1C10H2  +  1.5 H2 +  1RCH3   	k=1e8, E= 54418 kJ·kmol-1

- 1RMETNOL +  1RPHOXM2 -->  1RCH3 +  CO2 +  .5 1C10H2M4 +  .5 H2   	k=1e8, E= 54418 kJ·kmol-1

- 1RPHENOX -->  1PCOH  +  .5 1C10H2  +  3 1PH2    	k=1e8, E= 48139 kJ·kmol-1

- 1RPHOXM2 -->  1PCOH  +  .5 1C10H2M4 +  1PH2    	k=1e8, E= 48139 kJ·kmol-1

- 1RMETNOL +  1RCH3 -->  ETHANOL   	k=1e8, E= 12558 kJ·kmol-1

- 2 1RMETNOL -->  ACETALDY +  WATER   	k=1e8, E= 12558 kJ·kmol-1

- 1RMETNOL +  1PRADIOM +  1PH2  -->  1PCH2OH  +  1PADIOM2   	k=1e8, E= 52325 kJ·kmol-1

- 1RMETNOL +  1PRKETM2 +  1PH2  -->  1PCH2OH  +  1PKETM2   	k=1e8, E= 52325 kJ·kmol-1

- 2 1RADIOM2 -->  2 1PCH2OH  +  2 1PCH2  +  2 1PCHOHP  +  2 1PH2  +  1C10H2M4 +  2 1PCOH    			k=3.16e7, E=83720 kJ·kmol-1

- 2 1RLIGM2A  -->  .2 1PC10CHR  +  2 1PCH2OH  +  6 1PH2  +  2 1C10H2M4 +  4 1PCOH  +  2 1PCHOHP    		k=3.16e7, E=83720 kJ·kmol-1

- 2 1RLIGM2A  -->  2 1PCHO  +  2 WATER +  6 1PH2  +  2 1C10H2M4 +  4 1PCOH  +  2 1PC2H2	k=3.16e7, E=83720 kJ·kmol-1

- 2 1RGUAI -->  4 1PH2  +  1C10H2M4 +  2 1PCOH    	k=3.16e7, E=83720 kJ·kmol-1

- 2 1RKETM2 -->  2 1PCH2OH  +  2 1PH2  +  1C10H2M4 +  4 1PCOH  +  2 1PCH2    	k=3.16e7, E=83720 kJ·kmol-1

- 2 1PRFETM2 -->  2 1PCHO  +  2 1PH2  +  1C10H2M4 +  3 1PCOH  +  2 1PCH2  +  1PCHOHP    	k=3.16e7, E=83720 kJ·kmol-1

- 2 1RC3H7O2 -->  2 WATER +  2 1PC2H2  +  2 1PCHOHP  +  2 1PH2    	k=1e8, E= 12558 kJ·kmol-1

- 2 1RPRPACD -->  2 WATER +  2 1PC2H2  +  2 1PCOH  +  2 1PH2    	k=1e8, E= 12558 kJ·kmol-1

- 2 1RC3H3O -->  2 1PCOH  +  2 1PC2H2  +  2 1PH2    	k=1e8, E= 12558 kJ·kmol-1

- 1ROH +  1RCH3 -->  METHANOL   	k=1e8, E= 12558 kJ·kmol-1

- 2 1RCH3 -->  ETHANE   	k=1e8, E= 12558 kJ·kmol-1

- 2 1PRADIO -->  1PCOS  +  1PCOH  +  1C10H2  +  H2 +  2 1PCH2  +  2 1PCH2OH  +  2 1PCHOHP  +  2 1PH2    		k=6.2e7, E= 92092 kJ·kmol-1

- 1PRADIOM +  1RH -->  1PADIOM2   	k=1e8, E= 48139 kJ·kmol-1

- 1RGUAI +  1RH -->  1MGUAIAC   	k=1e8, E= 48139 kJ·kmol-1

- 1PRKETM2 +  1RH -->  1RKETM2   	k=1e8, E= 48139 kJ·kmol-1

- 2 1RADIO -->  1PCOS  +  2 1PCHOHP  +  2 1PCH2  +  1C10H2  +  4 1PH2  +  H2 +  1PCOH  +  2 1PCH2OH    		k=3.16e7, E=92092kJ·kmol-1

- 1RPHENOX +  1RLIG-A  -->  3 1PCOS  +  1PCHOHP  +  WATER +  1PC2H2  +  1.5 1C10H2  +  6 1PH2  +  1.5 H2   		k=3.16e7, E=83720 kJ·kmol-1

- 1LIG  +  1RLIG-A  -->  4 1PCOS  +  .2 1PC10CHR  +  2 1PCH2OH  +  2 1PCHOHP  +  2 1C10H2  +  10 1PH2  +  2.5 H2	k=3.16e7, E=83720 kJ·kmol-1

- 2 1PRLIG-A  -->  2 1PCHOHP  +  2 WATER +  2 1PC2H2  +  2 1C10H2  +  4 1PCOS  +  6 1PH2  +  2 H2   			k=1e8, E= 54418 kJ·kmol-1

- 2 1PRFET3 -->  2 1PCOH  +  2 1PCH2  +  2 1PCH2OH  +  2 1PCOS  +  1C10H2  +  H2 +  2 1PH2  	k=3.16e7, E=83720 kJ·kmol-1

- 2 1RKET -->  3 1PCOH  +  2 1PCH2  +  2 1PCH2OH  +  1PCOS  +  1C10H2  +  H2 +  4 1PH2    	k=3.16e7, E=83720 kJ·kmol-1

- 2 1PRLIGH  -->  2 1PCHOHP  +  3 WATER +  3 1PC2H2  +  5 1PCOH  +  2 1C10H2M4 +  H2 +  2 1RCH3 +  1PCH2   		k=3.16e7, E=83720 kJ·kmol-1

- 1RPHENOX +  1RPHENOL -->  2 1PCOS  +  1C10H2  +  1.5 H2 +  4 1PH2    	k=5e7, E= 83720 kJ·kmol-1

- 1RPHENOX +  1RC3H3O -->  1PCOS  +  .5 1C10H2  +  H2 +  2 1PH2  +  1PC2H2  +  1PCOH    	k=1e8, E= 48139 kJ·kmol-1

- 1RPHENOX -->  1PCOS  +  .5 1C10H2  +  1.5 H2   	k=3e7, E= 104650 kJ·kmol-1

- 1RPHOXM2 -->  1PCOS  +  .5 1C10H2M4 +  .5 H2   	k=3e7, E= 104650 kJ·kmol-1

Primary char devolatization reactions

- 1PCOS  -->  CO    	k=2e8, E= 199300 kJ·kmol-1

- 1PCOH  -->  CO    	k=2e8, E= 138138 kJ·kmol-1

- 2 1PH2  -->  H2    	k=2e8, E= 209300 kJ·kmol-1

- 1PC2H2  -->  H2 +  .2 1PC10CHR     	k=5e8, E= 205114 kJ·kmol-1

- 1PCH2OH  -->  1ROH +  1PCH2     	k=2e8, E= 0 kJ·kmol-1

- 1PCHOHP  -->  1ROH +  .5 1PC2H2     	k=2e8, E= 0 kJ·kmol-1

- 1PCHO  -->  1ROH +  .1 1PC10CHR     	k=2e8, E= 0 kJ·kmol-1

- 1PCH2  -->  H2 +  .1 1PC10CHR     	k=2.5e9, E=209300 kJ·kmol-1

- 1PCH2OH  -->  .5 CH4 +  .5 CO2 +  .5 H2    	k=3e8, E= 0 kJ·kmol-1

- 1PCHOHP  -->  .5 CH4 +  .5 CO2    	k=3e8, E= 0 kJ·kmol-1

- 1PCHO  -->  .25 CO2 +  .25 CH4 +  .5 CO    	k=3e8, E= 0 kJ·kmol-1

## Reactions with estimated kinetics, stoichiometry from literature

For the following equations, the reaction stoichiometry is taken from various literature sources, while the kinetic factors are derived from independent bibliography or are adjusted manually in order to reproduce the published reaction behaviour (temperature of start of decomposition, percentage decomposed after given time, etc.). As every reaction is based on individual sources, the corresponding references are given directly with each one.

- FURAN -->  WATER +  .125 CH4 +  .125 ETHENE +  .08333333 BENZENE +  .0625 NAPHTLEN +  .25 1PC10CHR   [8]; Assumptions: CH4, ethane, benzene and naphthalene in equal mole C conversion fractions, each pathway independent, producing char.  	k =7.5e8, E=142000 kJ ·kmol-1

- PROPDIAL -->  2 WATER +  .3 1PC10CHR   -   k=2e6, E=175000kJ·kmol-1 –   [9].  	k =7.5e8, E=142000 kJ·kmol-1

- LEVOGLUC -->  .27 1PC10CHR  +  2.7 WATER +  .7 ACETALDY +  .7 CO +  .3 ACETONE +  .3 CO2    [10].   	k =250000, E=95000kJ·kmol-1

- 1KETM2 -->  1RMGUAI + 1 RC3H3O   [7].   	k =1.5e4, E= 90000  kJ ·kmol-1

- XYLOSE -->  FORMICAC +  WATER +  2 C  +  CO2 +  CH4 +  H2    [11,12].   	k =1e4, E= 90000  kJ·kmol-1

- 1KETDM2 -->  1MGUAIAC +  WATER +  .3 1PC10CHR    [7].	k =1,2e4, E=95000 kJ·kmol-1

- SINPYALC -->  1MGUAIAC +  METHANOL +  .2 1PC10CHR    [7]	k =1e4, E=95000 kJ·kmol-1

- CMRYLALC -->  PHENOL +  METHANOL +  .2 1PC10CHR    [7]  	k =1e4, E=95000 kJ·kmol-1

- FURFURAL -->  .5 CH4 +  .4 1PC10CHR  +  .5 CO2 +  WATER   [13]	k =9.5e4, E=105000 kJ ·kmol-1

- ACETICAC -->  CO2 +  CH4    [8]. 	k=5.75e5, E=43500kcal·kmol-1

- ACETICAC -->  KETEN-01 +  WATER    [8]. 	k: 5.25e12, E=70.51kcal·kmol-1

- PROPDIAL -->  ACETALDY +  CO    [10]; Assumptions: decarboxylation by CO production.  	k=8e8 and E=130000kJ·kmol-1

- FORMALDY -->  H2 +  CO   [14].    	k=2.8e6, E =33500kcal·kmol-1

- METHANOL -->  CO +  2 H2    [8].  	k= 8000, E= 195000kJ·kmol-1

- FURFURAL --> FURAN + CO   [15].  	k=2.5e8, E=155000kJ·kmol-1

- XYLOSE --> FURFURAL + 3WATER - [16]; fast route assumed.  	k=1.3e7, E= 149000 kJ·kmol-1

- 2 XYLOSE -->  CO2 +  ACETONE +  FORMALDY +  3 WATER +  .75 GLYCOALD +  .175 1PC10CHR  +  .25 ACETALDY +  .25 CO +  1PCHOHP –  [8]; slow route assumed. 	k=2.6e5, E= 102500 kJ·kmol-1

- LEVOGLUC -->  .8 GLYCOALD +  .7 ACETOL +  .45 CO2 +  .3 CO +  .055 1PC10CHR  +  .3 ETYLDIOL +  .2 ACETALDY - [10,17] and [18].    	k=5e9, E= 165000 kJ·kmol-1

- KETEN-01 -->  CO +  .085 ETHENE +  .0415 1PC10CHR  +  .415 CH4  -  [19]; Assumptions: basis:  CO 65% , ethene 5% , CH4 30% , H2 3% + carbon-like mass.    	k=9.5e9, 55500kcal·kmol-1

- PROPNCAC -->  .55 CO2 +  .4 ETHANE +  .6 WATER +  .3 PROPENE +  .45 CO +  .15 CH4 +  .15 ETHENE - [20]; Assumptions: integrated the given reaction mechanisms into one single reaction, as intermediate product is no compound of the simulation. 	k=1e8, E=49000kcal·kmol-1

- ACETONE --> KETENE + CH4 - [21]; Assumptions: at 580ºC, 15% mol conversion after 2,5min residence time.  
	k=2e7, E= 145000 kJ·kmol-1

- ACETALDY -->  .8 CO +  .7 CH4 +  .1 H2 +  .1 ETHENE +  .1 ETHANE +  .1 CO2 [8].   	k=50000, E=98000kJ·kmol-1

- 1PRADIO +  1RCH3 -->  CMRYLALC +  1PCH2OH  -  [6].  	k=1e8,  E=48139 kJ·kmol-1

- 1PRKETM2 + 1 PRKETM2 --> 2 1PCH2OH +  1C10H2M4 +  4 1PCOH +  2 1PCH2  -  [7].  	k=3.16e7, E=83720 kJ·kmol-1

## Reactions for nitrogen containing compounds

For nitrogen containing compounds, no integrated set of reaction mechanisms can be found in literature. The reaction stoichiometry and the kinetic factors are therefore taken independently from various literature sources, and adjusted manually in order to reproduce the nitrogen release profiles for pyrolysis reactions according to literature [22–24].

The reaction stoichiometrics are based on decomposition experiments of the chosen model compounds. The reaction kinetics of the decomposition reactions are then adjusted so that decomposition starts at about 250-300 ºC and decomposition behavior fits the yield and N- release behavior for pyrolysis of nitrogenous species found in various literature sources [22,23,25–29]. The implemented reactions are listed in the following.

- 2 GLUTACID -->  AMMONIA +  CO2 +  ACETICAC +  PROPNCAC +  WATER +  .3 1PC10CHR  +  CHAR-N  +  1PCHO  
acc. to [30].    	k=1e6,  E=90000 kJ·kmol-1

- GLUTACID -->  PYRROLID +  CO2 +  WATER   -  [8] 	k=9e8,  E=117000 kJ·kmol-1

- PYRROLE -->  .15 NAPHTLEN +  .5 HCN +  .15 ETHENE +  .0975 1PC10CHR  +  .2 PROPENE +  .05 ETHANE +  .025 CH4 +  
.4 CHAR-N  +  .1 AMMONIA   -  [8], Assumptions: reaction mechanism by regression based on given product components,  Naphthalene representing PCHs, ethane / propene representing propyne); and [31], Assumptions: propyne and HCN as main products; and [32,33] 	k=2,25e13, E=49500 kcal·kmol-1

- PYRROLID -->  HCN +  .5 ACETONE +  .3 CO +  .2 ETHANE +  .2 ETHENE +  .075 H2 +  .2 WATER +  .075 CH4 +  .025 1PC10CHR  +  .025 PROPENE  -  [34], with acetone introduced as intermediate product (acetone instead of propanal).    	k= 1e6, E= 120000 kJ·kmol-1

- GLUTACID -->  PYRROLE +  CO2 +  2 WATER -   [8]. 	k= 4,5e8, E= 117000 kJ·kmol-1

- PYRROLE -->  HCN +  .66666666 PROPENE +  .1 1PC10CHR  -  [35]; Assumptions: HCN and propene + C (instead of propyne) as primary decomposition products. 	k=9e13, E=62.500kcal·kmol-1

# Secondary decomposition and polymerization reactions

The reaction scheme for the secondary reactions is given in the following. Since kinetic factors for the secondary reactions are widely unknown and only a very simplified modelling approach has been published in this regard, a linear regression model based on experimental results is implemented for this purpose [3,36–38]. The results of the yield modifications are checked for thermodynamic plausibility. This is done by verifying the reaction products obtained from the linear regression model in parallel with an RGibbs-type reactor under the same conditions. RGibbs- type reactors calculate the reaction products under equilibrium conditions by minimising Gibbs free energy. Ever when the products calculated by the secondary reactions algorithm are within the range between reactant (input) and RGibbs product (minimized free energy), the reaction mechanism is considered to be thermodynamically plausible.

## Char fraction

The increase in char yield due to secondary polymerisation reactions depends basically on the alkali metal content of the biomass (which catalyses these reactions) and the residence time.  Figure DD shows the data points derived from literature [36] and the corresponding polynomial approximation. The following polynomial expression given in Equation (S2) is found to fit best the experimental data.

charnew=char*1+char+-0,4989alk2+1,0657alk+0,9966-1*(-2-5t2+0,0112t+0,0374) 	Equation (S2)

With ‘alk’ being the alkali metal content of the biomass in % (values between 0 and 1%), and t the vapour residence time (values between 0 and 200 s)

## Water content

The polymerisation reactions lead, in parallel to a higher char yield, to an increase of the water yield (Figure S1). The increase of water yield can be attributed mainly to the catalytic activity of the ash alkali metals and is approximated by Equation (S3).

waternew=water*(-0,0748 alk2+0,2512 alk+1) 	Equation (S3)

With ‘alk’ being the alkali metal content of the biomass in % (values between 0 and 1%)

Figure S1. Increase of char and water yield with increasing biomass alkali metal content [36]

## Gas fraction

The increase of gaseous components due to secondary vapour reactions depends mainly on temperature (plausible for values between 400 and 550ºC) and residence time. The experimental results could most precisely be modelled by a polynomial approach, giving Equation (S4):

Gasnew=gas+(-0,0994T3+0,6959T2-0,6133T+0,0148)*ln⁡(t5)	Equation (S4)

With T being the pyrolysis reaction temperature and t the vapour residence time

## Chlorine release

The fate of chlorine elements in the biomass is also modelled by a linear regression mechanism based on literature [39,40]. Polynomial approximation for the share of chlorine that passes into the gas phase forming HCl gives a fifth order polynom,  Equation (S5), as shown graphically in Figure S2. The remaining Cl is assumed to be retained in the char, no chlorine containing liquid compounds are modelled, so the bio-oil is assumed to be Cl- free. This is a simplification, but bio-oil generally uses to have very low Cl content [41–44], why the impact of this simplification is considered to be negligible.

Cltogas=Cl*(-9.66e-12*T5+2.71e-8* T4-2.838e-5* T3+0,013587 T2-2,829 T+211,42)

Equation (S5)

With T being the pyrolysis reaction temperature

Figure S2. Polynomial approximation of chlorine release into the gas fraction

## Sulphur release

No clear dependency of the share of sulphur released to the gas phase or retained in the char can be found in literature, values vary and no unanimous tendency could be found [40,45,46]. The type of sulphur (organic / inorganic) plays an important role, but as information about the type of sulphur contained in the biomass is normally hard to obtain a constant value is used in this thesis. 45% of the sulphur content of the biomass is therefore assumed to be released into the gas fraction, independently from the pyrolysis temperature, in line with the findings published by [46]. The ash-free bio-oil does not retain any sulphuric compounds and is therefore considered sulphur free. Nevertheless, a small amount of sulphur is contained in the actual bio-oil, as this contains a certain amount of pyrolysis char with the corresponding sulphur content.

# Chemical compounds used in the simulation

Since Aspen Plus is originally a petroleum-industry oriented software, it contains no biomass components in its database. Therefore, all non- common species are defined manually by their molecular structure and their properties estimated or by the integrated Aspen property estimation system PCES, or, when not possible, from other sources. Latter is necessary mainly for the biomass compounds. For the intermediate fractional and radical species it was not necessary to determine all properties as they do not leave the reactor and therefore the parameters required for e.g. flash or enthalpy calculations are not relevant. The property data used for the biomass components hemicellulose, cellulose and lignin is taken from [47]. All chemical compounds used in the simulation are listed in Table S3.

Table S3. Chemical compounds used in the reaction model

| Component ID | Type* | Component name | Formula | Structure (only user-defined compounds) |
| --- | --- | --- | --- | --- |
| WATER | CONV | WATER | H2O |  |
| AIR | CONV | AIR | AIR |  |
| N2 | CONV | NITROGEN | N2 |  |
| OXYGEN | CONV | OXYGEN | O2 |  |
| CO2 | CONV | CARBON-DIOXIDE | CO2 |  |
| CO | CONV | CARBON-MONOXIDE | CO |  |
| CH4 | CONV | METHANE | CH4 |  |
| ETHANE | CONV | ETHANE | C2H6 |  |
| ETHENE | CONV | ETHYLENE | C2H4 |  |
| PROPANE | CONV | PROPANE | C3H8 |  |
| PROPENE | CONV | PROPYLENE | C3H6 |  |
| BUTANE | CONV | N-BUTANE | C4H10 |  |
| H2 | CONV | HYDROGEN | H2 |  |
| H2S | CONV | HYDROGEN-SULFIDE | H2S |  |
| SO2 | CONV | SULFUR-DIOXIDE | O2S |  |
| NO2 | CONV | NITROGEN-DIOXIDE | NO2 |  |
| NO | CONV | NITRIC-OXIDE | NO |  |
| AMMONIA | CONV | AMMONIA | H3N |  |
| HCN | CONV | HYDROGEN-CYANIDE | CHN |  |
| WOOD | NC | Nonconventional compound |  |  |
| CHAR | NC | Nonconventional compound |  |  |
| ASH | NC | Nonconventional compound |  |  |
| C | SOLID | CARBON-GRAPHITE | C |  |
| S | CONV | SULFUR | S |  |
| ACETICAC | CONV | ACETIC-ACID | C2H4O2 |  |
| BTYRCACD | CONV | N-BUTYRIC-ACID | C4H8O2 |  |
| FORMICAC | CONV | FORMIC-ACID | CH2O2 |  |
| PROPNCAC | CONV | PROPIONIC-ACID | C3H6O2 |  |
| LEVLCACD | CONV | LEVULINIC-ACID | C5H8O3 |  |
| GUAIACOL | CONV | GUAIACOL | C7H8O2 |  |
| PHENOL | CONV | PHENOL | C6H6O |  |
| CRESOL | CONV | P-CRESOL | C7H8O |  |
| XYELNOL | CONV | 2,3-XYLENOL | C8H10O |  |
| ETYLPHEN | CONV | O-ETHYLPHENOL | C8H10O |  |
| PRPNYPHN | CONV | P-ISOPROPENYL-PHENOL | C9H10O |  |
| BENZENE | CONV | BENZENE | C6H6 |  |
| TOLUENE | CONV | TOLUENE | C7H8 |  |
| M-XYLENE | CONV | M-XYLENE | C8H10 |  |
| ETYLBZNE | CONV | ETHYLBENZENE | C8H10 |  |
| PROPBENZ | CONV | ISOPROPYLBENZENE | C9H12 |  |
| ACETOL | CONV | ACETOL | C3H6O2 |  |
| ACETALDY | CONV | ACETALDEHYDE | C2H4O |  |
| GLYCOALD | CONV | GLYCOL-ALDEHYDE | C2H4O2 |  |
| FORMALDY | CONV | FORMALDEHYDE | CH2O |  |
| PROPDIAL | CONV | PROPENE-DIAL | C3H4O2 |  |
| GLYOXAL | CONV | GLYOXAL | C2H2O2 |  |
| KETEN-01 | CONV | KETENE | C2H2O |  |
| ACETONE | CONV | ACETONE | C3H6O |  |
| PENTANE | CONV | N-PENTANE | C5H12 |  |
| N-HEXANE | CONV | N-HEXANE | C6H14 |  |
| N-HEPTAN | CONV | N-HEPTANE | C7H16 |  |
| N-OCTANE | CONV | N-OCTANE | C8H18 |  |
| N-NONANE | CONV | N-NONANE | C9H20 |  |
| MTYNONAN | CONV | 5-METHYLNONANE | C10H22 |  |
| UNDECAN | CONV | N-UNDECANE | C11H24 |  |
| DODECAN | CONV | N-DODECANE | C12H26 |  |
| TRIDECAN | CONV | N-TRIDECANE | C13H28 |  |
| TETDECAN | CONV | N-TETRADECANE | C14H30 |  |
| PENTDECA | CONV | N-PENTADECANE | C15H32 |  |
| OCTDECAN | CONV | N-OCTADECANE | C18H38 |  |
| CYCPNTAN | CONV | CYCLOPENTANE | C5H10 |  |
| CYCHEXEN | CONV | CYCLOHEXENE | C6H10 |  |
| CYCHEXAN | CONV | CYCLOHEXANE | C6H12 |  |
| MTHCYCPT | CONV | METHYLCYCLOPENTANE | C6H12 |  |
| MTCYCHXA | CONV | METHYLCYCLOHEXANE | C7H14 |  |
| PRCYCHXA | CONV | N-PROPYLCYCLOHEXANE | C9H18 |  |
| BICYCHEX | CONV | BICYCLOHEXYL | C12H2 |  |
| BICYPRHX | CONV | BICYCLOPROPYLHEXYL | C15H28 |  |
| NAPHTLEN | CONV | NAPHTHALENE | C10H8 |  |
| CHRYSENE | CONV | CHRYSENE | C18H12 |  |
| DEXTROSE | CONV | DEXTROSE | C6H12O6 |  |
| XYLOSE | CONV | XYLOSE | C5H10O5 |  |
| LEVOGLUC | CONV | LEVOGLUCOSAN | C6H10O5 |  |
| FURFURAL | CONV | FURFURAL | C5H4O2 |  |
| FURFYALC | CONV | FURFURYL-ALCOHOL | C5H6O2 |  |
| HDRMTFUR | CONV | HYDROXYMETHYLFURFURAL | C6H6O3 |  |
| FURAN | CONV | FURAN | C4H4O |  |
| DIMTYFUR | CONV | DIMETHYLFURAN | C6H8O |  |
| METHANOL | CONV | METHANOL | CH4O |  |
| ETHANOL | CONV | ETHANOL | C2H6O |  |
| PROPANOL | CONV | 1-PROPANOL | C3H8O |  |
| BUTANOL | CONV | N-BUTANOL | C4H10O |  |
| HEXANOL | CONV | 1-HEXANOL | C6H14O |  |
| PENTANOL | CONV | 1-PENTANOL | C5H12O |  |
| ETYLDIOL | CONV | ETHYLENE-GLYCOL | C2H6O2 |  |
| PROPDIOL | CONV | 1,3-PROPANEDIOL | C3H8O2 |  |
| CYCHXNOL | CONV | CYCLOHEXANOL | C6H12O |  |
| PRCYHXOL | CONV | 1-NONANAL | C9H18O |  |
| SINPYALC | CONV | SINAPYLALCOHOL | C11H14O4 |  |
| CMRYLALC | CONV | COUMARYLALCOHOL | C9H10O2 |  |
| LIGNIN-C | SOLID | LIGNIN MONOMER | C15H14O4 |  |
| LIGNIN-O | SOLID | LIGNIN MONOMER | C20H22O10 |  |
| LIGNIN-H | SOLID | LIGNIN MONOMER | C22H28O9 |  |
| 1LIGH | SOLID | LIGNIN MONOMER | C22H29O9 |  |
| 1LIGM2 | SOLID | LIGNIN MONOMER | C19H23O8 |  |
| 1LIG | SOLID | LIGNIN MONOMER | C15H15O4 |  |
| 1PLIGC | SOLID | LIGNIN MONOMER | C17H16O5 |  |
| LIGN-OH | SOLID | DEGRADED LIGNIN - Intermediate fractional compound | C19H22O8 |  |
| LIGN-CC | SOLID | DEGRADED LIGNIN - Intermediate fractional compound | C15H14O4 |  |
| SINAPALD | CONV | SINAPYLALDEHYDE | C11H12O4 |  |
| SINAPAL1 | CONV | SINAPYLALDEHYDE | C11H12O4 |  |
| CELLULOS | SOLID | CELLULOSE MONOMER | C6H10O5 |  |
| CELL-ACT | SOLID | CELLULOSE MONOMER | C6H10O5 |  |
| HEMICELL | SOLID | HEMI CELLULOSE MONOMER | C5H10O5 |  |
| HEMCELL1 | SOLID | HEMI CELLULOSE MONOMER | C5H10O5 |  |
| HEMCELL2 | SOLID | HEMI CELLULOSE MONOMER | C5H10O5 |  |
| GCOH2 | CONV | GAS – Radical intermediate compound | CH2O |  |
| GCO | CONV | GAS – Radical intermediate compound | CO |  |
| 1PCHO | SOLID | CHAR - Intermediate fractional compound | C1H1O1 |  |
| 1PCOH | SOLID | CHAR - Intermediate fractional compound | C1O1 |  |
| 1PCH2 | SOLID | CHAR - Intermediate fractional compound | 1PCH2 |  |
| 1PC10CHR | SOLID | CHAR - Intermediate fractional compound | C10 |  |
| 1PCHOHP | SOLID | CHAR - Intermediate fractional compound | C1H2O1 |  |
| 1PCH2OH | SOLID | CHAR - Intermediate fractional compound | C1H3O1 |  |
| 1PCOS | SOLID | CHAR - Intermediate fractional compound | C1O1 |  |
| 1PC2H2 | SOLID | CHAR - Intermediate fractional compound | RC2H2 |  |
| 1C10H2M2 | CONV | TAR - Intermediate fractional compound | C12H8O2 |  |
| 1C10H2 | SOLID | CHAR - Intermediate fractional compound | C10H2 |  |
| 1C10H2M4 | CONV | TAR - Intermediate fractional compound | C14H14O4 |  |
| 1MGUAIAC | CONV | DEGRADED LIGNIN | C8H10O3 |  |
| 1KETDM2 | CONV | DEGRADED LIGNIN MONOMER | C11H12O4 |  |
| 1KETD | CONV | CINNAMIC-ACID | C9H8O2 |  |
| 1KETM2 | CONV | DEGRADED LIGNIN MONOMER | C11H14O5 |  |
| 1PKETM2 | CONV | DEGRADED LIGNIN MONOMER | C11H13O5 |  |
| 1KET | CONV | DEGRADED LIGNIN MONOMER | C9H10O3 |  |
| 1PKET | CONV | TAR - Intermediate fractional compound | C9H9O3 |  |
| 1PFET3M | CONV | TAR - Intermediate fractional compound | C11H13O5 |  |
| 1PFET3 | CONV | TAR - Intermediate fractional compound | C9H9O3 |  |
| 1ADIO | CONV | DEGRADED LIGNIN MONOMER | C9H12O3 |  |
| 1PADIO | CONV | TAR - Intermediate fractional compound | C9H11O3 |  |
| 1PADIOM2 | CONV | TAR - Intermediate fractional compound | C11H15O5 |  |
| 1ADIOM2 | CONV | DEGRADED LIGNIN MONOMER | C11H16O5 |  |
| 1RPHENOX | CONV | TAR - Intermediate radical compound | C6H4O |  |
| 1RPHOXM2 | CONV | TAR - Intermediate radical compound | C8H8O3 |  |
| 1RPHENOL | CONV | TAR - Intermediate radical compound | C6H5O1 |  |
| 1RGUAI | CONV | TAR - Intermediate radical compound | C8H9O3 |  |
| 1PRFET3 | CONV | TAR - Intermediate radical compound | C9H8O3 |  |
| 1PRFETM2 | CONV | TAR - Intermediate radical compound | C11H12O5 |  |
| 1RKET | CONV | TAR - Intermediate radical compound | C9H9O3 |  |
| 1RKETM2 | CONV | TAR - Intermediate radical compound | C11H13O5 |  |
| 1PRKETM2 | CONV | TAR - Intermediate radical compound | C11H12O5 |  |
| 1RADIO | CONV | TAR - Intermediate radical compound | C9H11O3 |  |
| 1PRADIO | CONV | TAR - Intermediate radical compound | C9H10O3 |  |
| 1RADIOM2 | CONV | TAR - Intermediate radical compound | C11H15O5 |  |
| 1PRADIOM | CONV | TAR - Intermediate radical compound | C11H14O5 |  |
| 1RLIG-A | SOLID | DEGRADED LIGNIN - Intermediate radical compound | C15H14O4 |  |
| 1PRLIG-A | SOLID | DEGRADED LIGNIN - Intermediate radical compound | C15H13O4 |  |
| 1RLIGM2A | SOLID | DEGRADED LIGNIN - Intermediate radical compound | C19H22O8 |  |
| 1RPRPDIA | CONV | TAR - Intermediate radical compound | C3H3O2 | Propenedial radical |
| 1PRLGM2A | SOLID | DEGRADED LIGNIN - Intermediate radical compound | C19H21O8 |  |
| 1PRLIGH | SOLID | DEGRADED LIGNIN - Intermediate radical compound | C22H27O9 |  |
| 1RPRPACD | CONV | TAR - Intermediate radical compound | C3H5O2 | Propionic acid radical |
| 1RMETNOL | CONV | TAR - Intermediate radical compound | CH3O | Methanol radical |
| 1RCH3 | CONV | TAR - Intermediate radical compound | CH3 | Methane radical |
| 1RC3H3O | CONV | TAR - Intermediate radical compound | C3H3O |  |
| 1RC3H7O2 | CONV | TAR - Intermediate radical compound | C3H7O2 |  |
| 1RH | CONV | TAR - Intermediate radical compound | H | Hydrogen radical |
| 1ROH | CONV | TAR - Intermediate radical compound | OH | OH radical |

* ‘Type’ represents the implementation of the compound in Aspen Plus. CONV = defined as conventional compound; SOLID = defined as solid compound

Bibliography

[1]	Miller RS, Bellan J. A Generalized Biomass Pyrolysis Model Based on Superimposed Cellulose, Hemicellulose and Liqnin Kinetics. Combust Sci Technol 1997;126:97–137. doi:10.1080/00102209708935670.

[2]	Di Blasi C. Modeling chemical and physical processes of wood and biomass pyrolysis. Prog Energy Combust Sci 2008;34:47–90. doi:10.1016/j.pecs.2006.12.001.

[3]	Gómez Díaz CJ. Understanding Biomass Pyrolysis Kinetics: Improved Modeling Based on Comprehensive Thermokinetic Analysis. PhD Thesis; Universitat Politècnica de Catalunya, Dept. of Chemical Engineering. Barcelona, Spain, 2006.

[4]	Ranzi E, Cuoci A, Faravelli T, Frassoldati A, Migliavacca G, Pierucci S, et al. Chemical Kinetics of Biomass Pyrolysis. Energy & Fuels 2008;22:4292–300. doi:10.1021/ef800551t.

[5]	Zhang X, Li J, Yang W, Blasiak W. Formation Mechanism of Levoglucosan and Formaldehyde during Cellulose Pyrolysis. Energy & Fuels 2011;25:3739–46. doi:10.1021/ef2005139.

[6]	CRECK modeling - webpage. Chem React Eng Chem Kinet n.d. www.creckmodeling.chem.polimi.it.

[7]	Faravelli T, Frassoldati A, Migliavacca G, Ranzi E. Detailed kinetic modeling of the thermal degradation of lignins. Biomass and Bioenergy 2010;34:290–301. doi:10.1016/j.biombioe.2009.10.018.

[8]	Moldoveanu SC. Pyrolysis of Organic Molecules with Applications to Health and Environmental Issues - Techniques and Instrumentation in Analytical Chemistry 28. Elsevier B.V; 2010.

[9]	Lifshitz A, Tamburu C, Suslensky A. Decomposition of propanal at elevated temperatures: experimental and modeling study. J Phys Chem 1990;94:2966–72.

[10]	Abella L, Yamamoto K, Fukuda K, Nanbu S, Oikawa N, Morita K, et al. Ab Initio Calculations for the Reaction Paths of Levoglucosan: An Intermediate of Cellulose Pyrolysis. Mem Fac Eng Kyushu Univ 2006;66:147–68.

[11]	Jing Q, Lü X. Kinetics of Non-catalyzed Decomposition of D-xylose in High Temperature Liquid Water. Chinese J Chem Eng 2007;15:666–9. doi:10.1016/S1004-9541(07)60143-8.

[12]	Tanaka Y, Nakamura T. Kinetic study on the thermal decomposition of D-xylose. Thermochim Acta 1983;62:307–14. doi:10.1016/0040-6031(83)85049-7.

[13]	Dunlop AP, Peters FN. Thermal Stability of Furfural. Ind Eng Chem 1940;32:1639–41. doi:10.1021/ie50372a027.

[14]	Fletcher CJM. The Thermal Decomposition of Formaldehyde T. Proc R Soc London 1934;146:357–62. doi:10.1098/rspa.1934.0160.

[15]	Hoydonckx HE, van Rhijn WM, van Rhijn W, de Vos DE, Jacobs PA. Furfural and Derivatives. Ullmann’s Encycl. Ind. Chem., Weinheim, Germany: Wiley VCH; 2000. doi:10.1002/14356007.a12_119.pub2.

[16]	Lappalainen M, Pitkänen I, Heikkilä H, Nurmi J. Melting behaviour and evolved gas analysis of xylose. J Therm Anal Calorim 2006;84:367–76.

[17]	Fung DPC, Tsuchiya Y, Sumi K. Thermal degradation of cellulose and levoglucosan: The effect of inorganic stalts. Wood Sci 1972;5:38–43.

[18]	Zhu X, Lu Q. Production of Chemicals from Selective Fast Pyrolysis of Biomass. In: Momba M, Bux F, editors. Biomass, Sciyo Publishing; 2010, p. 147–64.

[19]	Williamson AT. The Polymerization and Thermal Decomposition of Ketene. J Am Chem Soc 1934;56:2216–8. doi:10.1021/ja01326a003.

[20]	Blake PG, Hole KJ. The thermal decomposition of propionic acid. J Chem Soc B Phys Org 1966:577. doi:10.1039/j29660000577.

[21]	Rice FO, Vollrath RE. The thermal decomposition of acetone in the gaseous state. Proc Natl Acad Sci U S A 1929;15:702–5.

[22]	Cao J-P, Li L-Y, Morishita K, Xiao X-B, Zhao X-Y, Wei X-Y, et al. Nitrogen transformations during fast pyrolysis of sewage sludge. Fuel 2013;104:1–6. doi:10.1016/j.fuel.2010.08.015.

[23]	Sharma RK, Chan WG, Hajaligol MR. Product compositions from pyrolysis of some aliphatic α-amino acids. J Anal Appl Pyrolysis 2006;75:69–81. doi:10.1016/j.jaap.2005.03.010.

[24]	Ren Q, Zhao C, Chen X, Duan L, Li Y, Ma C. NOx and N2O precursors (NH3 and HCN) from biomass pyrolysis: Co-pyrolysis of amino acids and cellulose, hemicellulose and lignin. Proc Combust Inst 2011;33:1715–22. doi:10.1016/j.proci.2010.06.033.

[25]	Zhai L, Zhou X, Liu R. A Theoretical Study of Pyrolysis Mechanisms of Pyrrole. J Phys Chem A 1999;103:3917–22. doi:10.1021/jp9841790.

[26]	Friebel J, Köpsel RF. The fate of nitrogen during pyrolysis of German low rank coals — a parameter study. Fuel 1999;78:923–32. doi:10.1016/S0016-2361(99)00008-3.

[27]	Pokorna E, Postelmans N, Jenicek P, Schreurs S, Carleer R, Yperman J. Study of bio-oils and solids from flash pyrolysis of sewage sludges. Fuel 2009;88:1344–50. doi:10.1016/j.fuel.2009.02.020.

[28]	Wu Z. Formation of N 2 from Pyrrolic and Pyridinic Nitrogen during Pyrolysis of Nitrogen-Containing Model Coals. Energy & Fuels 2003;17:694–8.

[29]	Mullen C a., Boateng A a. Production and Analysis of Fast Pyrolysis Oils from Proteinaceous Biomass. BioEnergy Res 2011;4:303–11. doi:10.1007/s12155-011-9130-x.

[30]	Hansson K-M, Samuelsson J, Tullin C, Åmand L-E. Formation of HNCO, HCN, and NH3 from the pyrolysis of bark and nitrogen-containing model compounds. Combust Flame 2004;137:265–77. doi:10.1016/j.combustflame.2004.01.005.

[31]	Bacskay GB, Martoprawiro M, Mackie JC. The thermal decomposition of pyrrole: An ab initio quantum chemical study of the potential energy surface associated with the hydrogen cyanide plus propyne channel. Chem Phys Lett 1999;300:321–30. doi:10.1016/S0009-2614(98)01388-8.

[32]	Zhiheng W, Sugimoto Y, Kawashima H. Formation of N2 from pyrrolic and pyridinic nitrogen during pyrolysis of nitrogen-containing model coals. Energy & Fuels n.d.;17:694–8.

[33]	Lifshitz A, Tamburu C, Suslensky A. Isomerization and decomposition of pyrrole at elevated temperatures: studies with a single-pulse shock tube. J Phys Chem 1989;93:5802–8. doi:10.1021/j100352a030.

[34]	Chin WS, Mok CY, Huang HH, Rzepa HS. Photoelectron spectroscopic study of the thermal decomposition of 2-pyrrolidinone and 2-pyrrolidinethione. J Chem Soc Perkin Trans 2 1995:427. doi:10.1039/p29950000427.

[35]	Martoprawiro M, Bacskay GB, Mackie JC. Ab Initio Quantum Chemical and Kinetic Modeling Study of the Pyrolysis Kinetics of Pyrrole. J Phys Chem 1999;103:3923–34.

[36]	Hoekstra E, Westerhof RJM, Brilman W, Van Swaaij WPM, Kersten SRA, Hogendoorn KJA, et al. Heterogeneous and homogeneous reactions of pyrolysis vapors from pine wood. AIChE J 2012;58:2830–42. doi:10.1002/aic.12799.

[37]	Wang S, Liu Q, Liao Y, Luo Z, Cen K. A study on the mechanism research on cellulose pyrolysis under catalysis of metallic salts. Korean J Chem Eng 2007;24:336–40. doi:10.1007/s11814-007-5060-x.

[38]	Aho A, DeMartini N, Pranovich A, Krogell J, Kumar N, Eränen K, et al. Pyrolysis of pine and gasification of pine chars--influence of organically bound metals. Bioresour Technol 2013;128:22–9. doi:10.1016/j.biortech.2012.10.093.

[39]	Björkman E, Strömberg B. Release of Chlorine from Biomass at Pyrolysis and Gasification Conditions 1. Energy & Fuels 1997;11:1026–32. doi:10.1021/ef970031o.

[40]	Wang X, Si J, Tan H, Ma L, Pourkashanian M, Xu T. Nitrogen, Sulfur, and Chlorine Transformations during the Pyrolysis of Straw. Energy & Fuels 2010;24:5215–21. doi:10.1021/ef1007215.

[41]	Dynamotive. The BioOil Information Book. Vancouver, Canada: DynaMotive Energy Systems Corporation: 2006.

[42]	Kechagiopoulos PN, Voutetakis SS, Lemonidou AA, Vasalos IA. Hydrogen Production via Steam Reforming of the Aqueous Phase of Bio-Oil in a Fixed Bed Reactor. Energy & Fuels 2006;20:2155–63. doi:10.1021/ef060083q.

[43]	Oasmaa A, Peacocke C. A guide to physical property characterisation of biomass-derived fast pyrolysis liquids. Espoo, Finland: VTT Technical Research Centre of Finland: 2001.

[44]	Oasmaa A, Peacocke C. Properties and fuel use of biomass-derived fast pyrolysis liquids. A guide. VTT Publication 731. Espoo, Finland: VTT Technical Research Centre of Finland: 2010.

[45]	Uzun D, Özdoğan S. The behavior of sulphur forms of three lignites exposed to pyrolysis temperatures between 350 and 950°C. Fuel 2004;83:1063–70. doi:10.1016/S0016-2361(03)00143-1.

[46]	Knudsen JN, Jensen PA, Lin W, Frandsen FJ, Dam-Johansen K. Sulfur Transformations during Thermal Conversion of Herbaceous Biomass. Energy & Fuels 2004;18:810–9.

[47]	Wooley RJ, Putsche V. Development of an ASPEN PLUS Physical Property Database for Biofuels Components. Golden, United States: National Renewable Energy Laboratory: 1996.