Waveform Data File Information Sheet

Upload link: https://zenodo.org/communities/supraemi/
(Log-in to Zenodo first, otherwise the link above won’t go to the SupraEMI  site)

Details of Waveform Data

| Waveform Filename: | ACDC converterTest3_rawCurrentL1_Data_V1.csv |
| --- | --- |
| Waveform Description: | One AC/DC converter for EV applications was tested. An electronic load (Chroma 63200A 150V 600A) was used as DC load for testing. During the Test 3, the electronic load was set in a constant resistant mode and at 5 Ω. The output voltage of the AC/DC converter is set to 100 V, the resulting current is 20 A. In the csv file, the raw current is recorded for 10 seconds at the output of the Pearson 101 current monitor on L1 phase. One column is the time, the other is the voltage value. There is -5 seconds and +5 seconds voltage sampled with 500 kSa/s resulting in a file with 5 million of samples. |
| Date of Waveform Recording/Synthesis: | 22nd of March 2021 |
| Author/Owner: | LNE, Laboratoire National de Métrologie et d’Essais |
| Funding Source Acknowledgement: | JRP 18NRM05 SupraEMI – EMPIR project |

Details of Data file (All data files should be plain ASCII text or CSV files)

| Data file column format: Time, Amplitude |  |
| --- | --- |
| (e.g.: dd/mm/yy, hh:mm:ss:nn, VLine0, VLine1, VLine2, ILine0, Iline1, ILine2) |  |
| Col A = time (s), col B = amplitude (V) |  |
| Column Delimiter: (e.g. comma) | comma |
| End of line Delimiter: (if known, e.g. carriage return, line-feed) | CR LF |
| Sampling rate (samples per second): | 500 kSa/s |
| Y-axis units (e.g. kV): | V |
| Y-axis scaling factor (for each column): | 1; 1 |
| Y-axis resolution: | 12 bit (double precision) |

Detail of Recording Instrumentation (if applicable)

| 3dB Bandwidth of recording instrument and associated transducer(s): | 0.25 Hz – 4 MHz |
| --- | --- |
| Was an anti-aliasing filter used with the recording instrument?: | No |
| Manufacturer and model number(s) | Teledyne LeCroy HDO8108A |

Details of public power grid or laboratory power source (if applicable)

| Supply source: (public power grid or steady frequency power amplifier etc.): | Pearson type 101 (10.15 mV/A) AMN (Artificial Mains Network) ESH2-Z5 |
| --- | --- |
| Nominal frequency of supply source (Hz): | 50 Hz |
| Nominal voltage level of supply source (V rms): | 230 V |

When Uploading please note: It is the responsibility of the person uploading the data to ensure that they have full permission to publish the data.  The specific-location, times and details of the recording site should not be disclosed without the site/equipment/data-owner’s permission.  Trade-names should not be disclosed if they are likely to be detrimental to a manufacturer in any way. Ensure that all international data-protection and copyright laws are complied with.

When Downloading and using the data please note: This data is supplied for research purposes. If this data is used the support any form of public-domain publication, the source should be fully acknowledged and cited using the associated DOI and data owners details.  The publishers and data owners take no responsibility for any conclusions/decisions made from using the data.