Waveform Data File Information Sheet

Upload link: https://zenodo.org/communities/supraemi/
(Log-in to Zenodo first, otherwise the link above won’t go to the SupraEMI  site)

Details of Waveform Data

| Waveform Filename: | CombinedAppliancesLab_rawVoltagePhase_Data_V1 |
| --- | --- |
| Waveform Description: | The emissions of the combined appliances were recorded. A LED lamp, an induction cooker on 1000 W position and a desktop computer with a 80% load of the CPU were connected together. The raw data are recorded with a LISN in the circuit. In the csv file, there are raw values for voltage emissions measured on the phase. The signal was recorded for 10 seconds at the output of the LISN. One column is the time, the other is the voltage value. There are -5 seconds and +5 seconds voltage sampled with 500 kSa/s resulting in a file with 5 million of samples. |
| Date of Waveform Recording/Synthesis: | 22nd of March 2021 |
| Author/Owner: | LNE, Laboratoire National de Métrologie et d’Essais |
| Funding Source Acknowledgement: | JRP 18NRM05 SupraEMI – EMPIR project |

Details of Data file (All data files should be plain ASCII text or CSV files)

| Data file column format: Time (s), Voltage amplitude (V) |  |
| --- | --- |
| (e.g.: dd/mm/yy, hh:mm:ss:nn, VLine0, VLine1, VLine2, ILine0, Iline1, ILine2) |  |
| Col.A = time; Col.B = voltage amplitude; |  |
| Column Delimiter: (e.g. comma) | comma |
| End of line Delimiter: (if known, e.g. carriage return, line-feed) | CR LF |
| Sampling rate (samples per second): | 500 kSa/s |
| Y-axis units (e.g. kV): | V |
| Y-axis scaling factor (for each column): | 1; 1; |
| Y-axis resolution: | 12 digits |

Detail of Recording Instrumentation (if applicable)

| 3dB Bandwidth of recording instrument and associated transducer(s): | 9 kHz to 150 kHz |
| --- | --- |
| Was an anti-aliasing filter used with the recording instrument?: | No |
| Manufacturer and model number(s) | Teledyne LeCroy HDO8108A |

Details of public power grid or laboratory power source (if applicable)

| Supply source: (public power grid or steady frequency power amplifier etc.): | French public power grid Insulating transformer LISN (Line impedance stabilization network) ESH3-Z5 |
| --- | --- |
| Nominal frequency of supply source (Hz): | 50 Hz |
| Nominal voltage level of supply source (V rms): | 230 V |

When Uploading please note: It is the responsibility of the person uploading the data to ensure that they have full permission to publish the data.  The specific-location, times and details of the recording site should not be disclosed without the site/equipment/data-owner’s permission.  Trade-names should not be disclosed if they are likely to be detrimental to a manufacturer in any way. Ensure that all international data-protection and copyright laws are complied with.

When Downloading and using the data please note: This data is supplied for research purposes. If this data is used the support any form of public-domain publication, the source should be fully acknowledged and cited using the associated DOI and data owners details.  The publishers and data owners take no responsibility for any conclusions/decisions made from using the data.