Spatially-explicit prediction of capacity density advances geographic characterization of wind power technical potential

Dylan Harrison-Atlas*1, Galen Maclaurin1, Eric Lantz1

1National Renewable Energy Laboratory, Colorado, USA

Supporting Information

Wind farm area characterization

In Figure S-1, we present a spatial depiction of wind farm area and associated capacity density for two sample wind farms. Here we show core components of capacity density, using wind farms in distinctive landscapes to illustrate variation in turbine layouts and wind farm area. We determine wind farm area using a method that requires only information on turbine locations and can be applied consistently to all operational wind farms. First, all turbines within a wind project are buffered by 300 meters to reflect a common setback distance. Then, buffered turbine polygons are merged into a single feature. Finally, we construct a convex hull polygon around the buffered feature, which establishes a minimum bounding area that delineates the wind project boundary. To estimate capacity density (MW/km2), we then sum the capacity of the installed turbines (MW) and divide it by the area (km2) contained within the convex hull polygon.

Figure S-1. Capacity density as visualized for two wind farms showing respective turbine configurations (red) and total wind farm area (outlined in blue). Geographic variation in wind farm capacity density occurs because of site-level differences in physical, social, and environmental characteristics. Wind farm area is derived using a convex hull geometry that describes the minimum bounding polygon based on turbine layout and assuming a 300-m setback for turbines. Wind Farm A is located on a ridgeline in the Northeast and has 33 turbines with an installed capacity of 99 MW, and occupies 38.2 km2 yielding a capacity density of 2.6 MW/km2. Wind Farm B is located in a predominately flat agricultural area in the Midwest with interspersed residential development, has 87 turbines with an installed capacity of 130.5 MW, and occupies 97.8 km2, yielding a capacity density of 1.3 MW/km2. Imagery data provided by Landsat/Copernicus, Maxar Technologies, U.S. Geological Survey, USDA Farm Service Agency via Google Earth Engine (accessed January 9, 2020).

Geospatial data

We employed a suite of geospatial layers as explanatory variables in the power density model. Data layers included derivative products of published data sets as well as novel datasets produced using geospatial modeling for the purposes of this analysis. All geospatial analyses were performed in Google Earth Engine [1]. Key data layers used as explanatory variables are shown in Table S-1. To ensure consistency, all data sets were reprocessed to 2 km x 2 km resolution to describe mean or modal conditions within each pixel depending on data type.

We selected this spatial resolution as the appropriate scale for capturing relatively fine-grained patterns in physical and social phenomena for the intended context while recognizing the coarser native resolution of particular input variables (e.g., WIND Toolkit; [2]). Categorical variables including the National Land Cover Dataset (NCLD; [3]), geophysical landform units [4], and wind exclusions were binarized for each class before computing fractional areal extent (i.e., the fraction of each pixel comprised by a particular class).

Geospatial layers were stacked to produce a multiband raster wherein each band represents a single variable. In accordance with our sampling scheme, we extracted values from the multiband raster at wind turbine locations with known power densities. After training and evaluating the boosted regression tree model using TensorFlow [5], we applied it to the multiband raster, producing spatially explicit predictions of power density for each 2x2 km resolution pixel within CONUS.

Below we describe novel geospatial layers that were produced explicitly for this study. Detailed information for the other datasets can be found via their references provided in Table S-1.

Dimensionless wind power

Wind speed alone is not a robust predictor of wind power potential, given the cubic relationship between wind speed and power output. To support broader generalizations about wind potential, we aimed to quantify wind power in ways that did not reflect a particular technological design. Dimensionless power was computed by removing the area term associated with the cross-sectional area swept by a turbine blade and using the following formula: P=12Cpv3 where Cp is the coefficient of performance (0.48) and v is the wind speed in m/s. Wind speed data were obtained from the Wind Integration National Dataset (WIND) Toolkit [2] and were used to compute dimensionless power assuming a cut-in speed of 4.0 m/s, rated speed of 12 m/s, and cut-out speed of 25 m/s.

Directional wind power equitability metric

We developed a metric to capture the evenness of energy production around the wind rose. Using the concept of equitability, we applied Shannon’s equitability index [6] from the field of ecology to determine wind energy equitability on the basis of the aforementioned dimensionless wind power outputs. The following formulas were used:

H=-i=1Spi ln(pi)

where S is the number of contributing wind directions (S=16) and pi is the relative energy contribution of direction i. The equitability index (EH) is then computed as follows:

EH=Hln⁡(S)

Values of equitability range continuously between zero and one, with one indicating complete evenness (i.e., uniform energy contributions from all directions) and low values being indicative of a dominant directional influence.

Fractional areal extent of wind exclusions

Wind exclusions refer to spatial criteria that constrain locations where wind deployment can occur. We used a standard set of wind exclusions [7] at 90-m resolution to delineate these areas deemed unsuitable for wind development. Similar to the processing of categorical variables described above, we computed the fractional areal extent of each 2x2 km pixel comprised by wind exclusions.

Contagion

We hypothesized that the spatial patterning of land suitable for wind development could help explain observed patterns of power density. Using the aforementioned set of standard wind exclusions, we assessed fragmentation using a contagion index to capture the relative degree of “clumpiness” of landscapes based on the extent and spatial distribution of developable lands. A detailed description of the contagion metric and interpretation can be found online (L115: Contagion Index,” http://www.umass.edu/landeco/research/fragstats/documents/Metrics/Contagion%20-%20Interspersion%20Metrics/Metrics/L115%20-%20CONTAG.htm). Put concisely, low values of contagion are indicative of landscapes where patches of developable lands are highly interspersed. Conversely, high values of contagion would be expected in landscapes dominated by either developable lands or by undevelopable lands and would also result in situations where similar patch types have low interspersion.

Landform regions

Landform regions describe broader scale geographic features than are captured by more localized landform assessments for which we used a landform dataset created by Theobald et al. [4]. We characterized the distribution of physical landform regions using an approached outlined by Karagulle et al. [8]. Using their methodology to characterize slope, profile, and local relief from a digital elevation model provided by the Shuttle Radar Topography Mission (SRTM) [9], we derived landform regions for the United States at 30-m spatial resolution in Google Earth Engine. A more detailed explanation of modeling methodology and description of the landform regions is available in Karagulle et al. [8].

Table S-1. Geospatial datasets used in the modeling of power density. Description and original source of variables is provided. An asterisk (*) indicates variable was selected for use in the final predictive model.

| Variable | Description | Source |
| --- | --- | --- |
| ACC* | Urban accessibility measured through travel time to nearest urban center (minutes) | [10] |
| BUI | Built-up intensity of residential and commercial buildings (m2) | Derived from [11] |
| CLF | Fractional areal extent of cliff landform (unitless) | Derived from [4] |
| CON* | Landscape metric describing contagion (i.e., the spatial clumpiness of unsuitable lands; wind exclusions) | Derived from Maclaurin et al. (unpublished) |
| CRG | Fraction of pixel containing ridge landform (unitless) | Derived from [4] |
| DIV | Fractional areal extent of divide landform (unitless) | Derived from [4] |
| GAP | GAP 1&2 status protected lands (%) | Derived from [12] |
| HUD* | Housing unit density (units/km2) | Derived from [13] |
| LBL | Wind regions defined by Lawrence Berkeley National Laboratory | [14] |
| LC1* | Fractional areal extent of water land cover class (unitless) | Derived from [3] |
| LC2 | Fractional areal extent of developed land cover class (unitless) | Derived from [3] |
| LC3 | Fractional areal extent of barren land cover class (unitless) | Derived from [3] |
| LC4* | Fractional areal extent of forest land cover class (unitless) | Derived from [3] |
| LC5 | Fractional areal extent of shrubland cover class (unitless) | Derived from [3] |
| LC7 | Fractional areal extent of herbaceous land cover class (unitless) | Derived from [3] |
| LC8* | Fractional areal extent of planted/cultivated land cover class (unitless) | Derived from [3] |
| LC9 | Fractional areal extent of wetlands land cover class (unitless) | Derived from [3] |
| LFR* | Landform regions | Harrison-Atlas (unpublished) |
| LSP | Fractional areal extent of lower slope landform (unitless) | Derived from [4] |
| LU1 | Land use class (level I) | Derived from [15] |
| MWS | Mean long-term windspeed at 80-m hub height (m/s) | Derived from [2] |
| POP | Population density (persons/km2) | Derived from [16] |
| RDG | Fractional areal extent of ridge landform (unitless) | Derived from [4] |
| RIX | Fractional areal extent exceeding critical slope threshold (unitless) | Derived from [17] |
| SLF | Fractional areal extent of suitable landforms (i.e., not cliff or valley; unitless) | Derived from [4] |
| TRC | Fractional areal extent of tree cover | Derived from [3] |
| USP | Fractional areal extent of upper slope landform (unitless) | Derived from [4] |
| VLY | Fractional areal extent of valley landform (unitless) | Derived from [4] |
| WEQ* | Power equitability metric describing the evenness of wind energy contributions at 100m hub height across compass directions | Derived from [2] |
| WEX | Fractional areal extent of wind exclusions | Derived from Maclaurin et al. (unpublished) |
| WND* | Dimensionless wind energy at 100m hub height (unitless) | Derived from [2] |

References

1.	Gorelick, N., et al., Google Earth Engine: Planetary-scale geospatial analysis for everyone. Remote Sensing of Environment, 2017. 202: p. 18-27.

2.	Draxl, C., et al., The Wind Integration National Dataset (WIND) Toolkit. Applied Energy, 2015. 151: p. 355-366.

3.	Homer, C., et al., Completion of the 2011 National Land Cover Database for the Conterminous United States – Representing a Decade of Land Cover Change Information. Photogrammetric Engineering and Remote Sensing, 2015. 81: p. 345-354.

4.	Theobald, D.M., et al., Ecologically-Relevant Maps of Landforms and Physiographic Diversity for Climate Adaptation Planning. PLoS One, 2015. 10(12): p. e0143619.

5.	Abadi, M., et al. TensorFlow: a system for large-scale machine learning. in Proceedings of the 12th USENIX conference on Operating Systems Design and Implementation. 2016. Savannah, GA, USA: USENIX Association.

6.	Shannon, C.E. and W. Weaver, The mathematical theory of communication. 1949, Urbana, Illinois: University of Illinois Press.

7.	Lopez, A., et al., U.S. Renewable Energy Technical Potentials: A GIS-based analysis. 2012, National Renewable Energy Laboratory: Golden, CO.

8.	Karagulle, D., et al., Modeling global Hammond landform regions from 250-m elevation data. Transactions in GIS, 2017. 21(5): p. 1040-1060.

9.	Farr, T.G., et al., The Shuttle Radar Topography Mission. Reviews of Geophysics, 2007. 45(2).

10.	Weiss, D.J., et al., A global map of travel time to cities to assess inequalities in accessibility in 2015. Nature, 2018. 553(7688): p. 333-336.

11.	Leyk, S. and J.H. Uhl, HISDAC-US, historical settlement data compilation for the conterminous United States over 200 years. Sci Data, 2018. 5: p. 180175.

12.	GAP, U.S.G.S., Protected areas database of the United States (PADUS), U.S.G.S. GAP, Editor. 2019.

13.	CIESIN, U.S. Census Grids (Summary File 1), C.f.I.E.S.I.N.-C.-C. University, Editor. 2017, NASA Socioeconomic Data and Applications Center (SEDAC): Palisades, NY.

14.	EERE, D., 2017 wind technologies market report. 2018, U.S. Department of Energy (DOE) Office of Energy Efficiency and Renewable Energy (EERE): United States.

15.	Theobald, D.M., Development and applications of a comprehensive land use classification and map for the US. PLoS One, 2014. 9(4): p. e94628.

16.	Pesaresi, M. and S. Freire, GHS Settlement grid following the REGIO model 2014 in application to GHSL Landsat and CIESIN GPW v4-multitemporal (1975-1990-2000-2015), J.R.C.J. European Commission, Editor. 2016.

17.	Gesch, D.B., et al., The national elevation data set. Photogrammetric Engineering and Remote Sensing, 2002. 68(1): p. 5-11.