robinCheck - a simple ROBIN check program

robinCheck is a standalone program to perform quick, basic checks of the

ROBIN card.

Running program in the standalone mode:

robinCheck [-eb] [-ncrd 0 1 2...]

Where

-eb: without -ncrd runs eBist test on all Robin cards detected in tested PC.

-ncrd selects specific cards. Example:

-eb -ncrd 0 2: runs eBist on the first and third Robin card only (cards count starts from "0")

robinCheck without options runs PCI and /proc/robin checks only (see below for the tests

description)

Program generates a logfile on the current working directory. Format of the logfile is:

robinCheckDate.log. Example: "robinCheckFri_May__4_17_28_50_2007_.log"

Tasks performed by the program:

1) Check if all the drivers needed by the program are present.

This includes:

robin, "robintty, io_rcc and cmem_rcc driver.

In case any of the drivers is missing the error message will be

printed with the list of the missing drivers.

2) PCI check: this includes running and checking the output of the

"/sbin/lspci -vvv" and "/sbin/lspci -b -vvv"

checks per

a) checks revision (should be "rev ac")

b) checks "Subsystem:" field.

c) checks all the memory regions. There should be three memory regions and none of

them disabled.

3) Program does "robin_firmware_update --mode=scan" and reads cards info: serial,

production site, type of the board.

4) checks /proc/robin

- in the case cards are not configured, program executes "robinconfig".

5) In case of any error in the above tests, program automatically

connects to the card (via ttydriver) and runs eBist test.