/** \mainpage Introduction

This document describes the API of the ROSModules package classes. This package contains all the classes responsibles for interfacing the IOManager application to its external controlled components.

The main abstract classes in the system are \c ReadoutModule and \c DataChannel.

\section motivations Motivations

The ROS IOManager was designed to handle a number of input modules

(typically an S-LINK destination or a ROBIN),

which inside the application would simply be seen as sources of data fragments.

Originally we thought that any input module would only be equipped

with one individual Read-Out Link.

This is reflected in the ROSFragmenManagement package API, where a

single base-class (FragmentManager) is foreseen to describe the

generic interface with an input module.

The FragmentManager (FM)

has both methods for handling the incoming data (requestFragment(),

getFragment() and releaseFragment() ) and methods to control the input

module (e.g. configure()/unconfigure()).

With the application of the ROS software in other contexts:

test beam, RCD and even for the new RobIn prototypes this design has

shown a major limitation.

In most of the cases the input module (being an H/W module like the

ROBIN or a S/W thread like the SWRobin) has to deal with a number of

ROLs greater than one, so that overall there is a different number of data

sources and of input modules.

\latexonly

\begin{figure}[htb]

\begin{center}

\includegraphics[width=0.8\textwidth]{FM-Module-schema.jpg}

\caption{Correspondence between FM classes and concrete inputs

or H/W modules.}

\end{center}

\end{figure}

\endlatexonly

It is immediately evident that an architecture based on a single

interface class would not cope properly with such a scenario. The

de facto approach to handle the situation has been the one of keeping the FM

as the interface with the individual data channels and

introducing some additional classes (e.g. the SWRobin or the PciRobin)

to deal with the input modules. To minimize the effects on the

high-level architecture, those classes are invisible to the IOManager

class as they are completely hidden behind the corresponding FM.

This approach has several unpleasant consequences.

First, as the

IOManager communicates run control state changes

(e.g. configure/unconfigure) to the FM instances only, the module-related classes

will be notified many times (one per linked FM, thus one per input

channel) of any such change, even though they can only access their

corresponding physical module only once (e.g. a ROBIN card should only receive

a single configuration message even though it is equipped with two

input channels). This obliges the developers to introduce complex

and possibly insecure logic in their module-related classes.

Second the module-related classes do not have implement a specific API

and it might be difficult on the long term to keep the uniformity of

the related FMs API.

More recently some even more specific problems have been seen in the area of RCD.

In RCD the role of the ROS IOM is to "handle"

a number of modules providing a functionality

which is more general than that of fragment management.

In a ROD crate there may be modules (like trigger modules) that need

to be controlled even though they are not sources of fragments.

Another problem is related to the propagation of state

transitions to the electronics interface, which again in the current view

has to go through the FM.

Starting from Online software release 00-21-00, a new state machine has been

defined for the Atlas TDAQ system.

Only 4 (instead of 5) states are exposed to the operator, but a

set of new intermediate states have been defined (see http://atlas-onlsw.web.cern.ch/Atlas-onlsw/components/RunControl/run_control_quick_ref.html).

The current FM implementation do not include methods for any of those

additional state changes, while some of them are essential for

synchronizing the different modules controlled by a RCD application.

E.g., in order to allow fragment synchronization during the run, a global trigger

enable should be released as the last operation during run start.

In this case, if trigger modules are accessed via a

FM sub-class, a startTrigger method should be added to the its API.

More in general, the configure method of the FM had always been

thought more as a method to initialize the class itself (being a

plug-in the FM can only have an empty constructor) than as a method

for executing the actual configuration of the corresponding module.

This has led to an unpleasant inconsistency:

At run control \b load transition the fragmentManager class is created

and its \b configure method is called to initialize it.

At run control \b configure transition the FM \b connect method is

called to configure the corresponding module.

At run control \b prepareForRun transition the FM \b start method is called.

The inverse transitions have been implemented consequently.

Based on these considerations, we now propose to introduce

the a ReadoutModule and a DataChannel class as a replacement of the FM.

The IOManager interacts with hardware via (instances of)

ReadoutModule which may contain zero to many DataChannel instances.

The ReadoutModule is a plug-in and has a method (with no arguments) for any possible state

change, plus a setup method (that gets a Config class as parameter) to

play the role of the constructor as the configure method in the FM

used to do.

The DataChannel instances instead play the role originally imagined for

the FM, i.e. to handle the flow of data fragments from an individual

input channel. The DataChannel do not need to be a plug-in, as its

instances are created from specific ReadoutModule sub-classes, which knows what

specific DataChannel sub-class they should use.

\section module The ReadoutModule class

The ReadoutModule class describes any possible external module that the IOManager

process is controlling.

ReadoutModule extends the Controllable abstract class, that defines an empty method

for every state transition. When notified by the run control of a state

transition, The IOManager process will call the corresponding method

on every ReadoutModule instance.

Thus if a ReadoutModule sub-class is responsible for interfacing the IOManager

with a H/W or S/W component that needs to perform any action at a

particular state transition,

the class implementation will override the definition of the

corresponding method.

As the ReadoutModule class is a plug-in for the IOManager process, any sub-class must

define only an empty constructor. Hence the setup() method is provided to

ensure a flexible configuration of the sub-class instances. This

method is expected to be called after the PluginFactory has created an instance

of a ReadoutModule sub-class. It gets as a parameter a pointer to a Config class,

to allow a specific configuration for any possible ReadoutModule sub-class.

Any ReadoutModule instance may have 0 to n input channels which are described by sub-classes of the DataChannels abstract class.

\section datachannel The DataChannel class

The DataChannel class describes the interface with individual data channels.

It is convenient to introduce

this level of abstraction, as different ReadoutModules may have different number of

input channels (e.g. one ROBIN might have 3 connected input ROLS, while another might be using only 1), and we want to avoid the extra-complexity of having to

go through the ReadoutModule instances to get the list of channels involved in a data request.

Thus the ReadoutModule sub-classes are responsible of instantiating the

DataChannel sub-class instances corresponding to their input channels,

while the IOManager and the Request classes keep a list of all available

channels and access them directly to fulfill data requests.

Only the ReadoutModule instance knows the details of the specific DataChannel

sub-class that it needs to instantiate. The IOManager and Request classes instead only rely on the fact that DataChannel sub-classes implement the specific

DataChannel API.

The DataChannel class maintain a global map of all DataChannel instances and

their ids. Every time that a DataChannel sub-class is instantiated, a pointer

to the instance is inserted in the map. A built-in automatic check ensures that

two channels with the same id cannot be created (if one attempts to do so an

exception is thrown).

Two static methods of the DataChannel class are provided to retrieve the

pointer to a channel on the basis of its id, or to get a vector containing

the complete list of all channels of the system.

\latexonly

\begin{figure}[hp]

\begin{center}

\includegraphics[width=0.8\textwidth]{ModulesAndDataChannels.jpg}

\caption{Class diagram for Module and DataChannel.}

\end{center}

\end{figure}

\endlatexonly

*/