Supplemental Data

Supplemental Table S1 GC-HRMS Parameters

| Mass Spectrometer | Thermo DFS |  |  |  |
| --- | --- | --- | --- | --- |
| GC | Thermo Trace Ultra w/ TriPlus Autosampler |  |  |  |
| Data Acquisition Software | Xcalibur 2.0 |  |  |  |
| Data Reduction Software | Waters MassLynx 4.0 |  |  |  |
| Column | Agilent CP Sil-5 - 30m x 0.25mm I.D. x 1.0µm film |  |  |  |
| Temperature Program | Rate (°C/min) | Temperature (°C) |  | Hold (min) |
|  | 0 | 50 |  | 2.0 |
|  | 20 | 70 |  | 4.5 |
|  | 4 | 80 |  | 0 |
|  | 30 | 280 |  | 5 |
| Total Run Time | 21.1 min |  |  |  |
| Carrier Gas / flow | Helium / 1.0 mL/min constant flow |  |  |  |
| Injection Mode | Splitless |  |  |  |
| Injector Temperature | 250 °C |  |  |  |
| Split Flow / Splitless Time | 30 mL/min / 1.0 min |  |  |  |
| Injection Volume | 2 µL |  |  |  |
| Autosampler Temperature | Ambient |  |  |  |
| Transfer Line Temperature | 280 °C |  |  |  |
| Ion Source | Electron Ionization (40 eV) |  |  |  |
| Polarity | Positive |  |  |  |
| Source Temperature | 250 °C |  |  |  |
| Scan Type | MID – Lock Mass |  |  |  |
| Acquisition Time | 6.5-10.0 min |  |  |  |
| Monitored Ions | Compound |  | Mass (Da) | Approximate Retention Time (minutes) |
|  | Sarin Primary |  | 99.00057 | 7.88 |
|  | Sarin Secondary |  | 125.01622 | 7.88 |
|  | PFTBA Lock Mass |  | 99.99306 | NA |
|  | PFTBA Calibration |  | 130.99147 | NA |

Supplemental Table S2 LC-MS/MS Parameters

| Tandem Mass Spectrometer | Waters Xevo TQ-XS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UPLC | Waters Acquity |  |  |  |  |  |  |  |
| Data Acquisition Software | Waters MassLynx 4.2 |  |  |  |  |  |  |  |
| MS Source | Electrospray, positive ion mode |  |  |  |  |  |  |  |
| Column | Phenomenex Prodigy ODS (3) 100Å 3µm 2.0 x 100mm |  |  |  |  |  |  |  |
| Column Temperature | 30°C |  |  |  |  |  |  |  |
| Mobile Phase A | 0.1% Formic Acid in Milli-Q Water |  |  |  |  |  |  |  |
| Mobile Phase B | 0.1% Formic Acid in Acetonitrile |  |  |  |  |  |  |  |
| Gradient Profile | Time, min |  | %B |  |  | Flow Rate, mL/min | Curve |  |
|  | 0 |  | 10 |  |  | 0.3 | - |  |
|  | 1 |  | 10 |  |  | 0.3 | 6 |  |
|  | 5 |  | 75 |  |  | 0.3 | 6 |  |
|  | 5.01 |  | 75 |  |  | 0.4 | 6 |  |
|  | 8 |  | 75 |  |  | 0.4 | 6 |  |
|  | 8.01 |  | 10 |  |  | 0.4 | 6 |  |
|  | 10 |  | 10 |  |  | 0.3 | 6 |  |
| Injection Volume | 15 µL |  |  |  |  |  |  |  |
| Capillary | 1 kV |  |  |  |  |  |  |  |
| Source Temperature | 110°C |  |  |  |  |  |  |  |
| Desolvation, nebulizer gas | Nitrogen @ 1,000 L/hr and 300°C |  |  |  |  |  |  |  |
| Collision gas | Argon @ 0.15 mL/min |  |  |  |  |  |  |  |
| Mass Resolution | Unit in both quadrupoles |  |  |  |  |  |  |  |
| Run Time | Approximately 10 minutes |  |  |  |  |  |  |  |
| Monitored Ions | Compound | Precursor Mass (Da) |  | Cone (V) | Product Mass (Da) |  |  | Collision (eV) |
|  | Sarin | 141 |  | 20 | 99 |  |  | 10 |
|  | IMPA | 139 |  | 14 | 79 |  |  | 20 |
|  |  |  |  |  | 97 |  |  | 8 |

Sequence alignment comparison of human BChE vs. BChE from various species.

AfGrn  42   MHSKVTIICIRFLFWFLLLCMLIGKSHTEDDIVIATKNGKVRGMNLTVLGGTVTAFLGIP  101

Human  1    ................................I...............F...........  60

Rhes   1    MHSKVTIICIRLLFWFLLLCMLIGKSHTEDDIVIATKNGKVRGMNLTVLGGTVTAFLGIP  60

Human  1    ...........F....................I...............F...........  60

Cyno   1    MDSKVTIICIRLLFWFLLLCMLIGKSHTEDDIVIATKNGKVRGMNLTVLGGTVTAFLGIP  60

Human  1    .H.........F....................I...............F...........  60

Pig    1    MQRKGTIMYIRYFLWFLLLWMLVGKSYAEEDIIVTTKNGKVRGMNLPVLGGTVTAFLGIP  60

Human  1    .HS.V..IC..FLF.....C..I...HT.D...IA...........T.F...........  60

AfGrn  102  YAQPPLGRLRFKKPQSLTKWSDVWNATKYANSCYQNIDQSFPGFHGSEMWNPNTDLSEDC  161

Human  61   ......................I..........C..........................  120

Rhes   61   YAQPPLGRLRFKKPQSLTKWSDIWNATKYANSCYQNIDQSFPGFHGSEMWNPNTDLSEDC  120

Human  61   .................................C..........................  120

Cyno   61   YAQPPLGRLRFKKPQSLTKWSDIWNATKYANSCYQNIDQSFPGFHGSEMWNPNTDLSEDC  120

Human  61   .................................C..........................  120

Pig    61   YAQPPLGRLRFKKPQSMTKWPDIWNATKYANSCYQNTDQSFPGFVGSEMWNPNTELSEDC  120

Human  61   ................L...S............C..I.......H.........D.....  120

AfGrn  162  LYLNVWIPAPKPKNATVMIWIYGGGFQTGTSSLHVYDGKFLARVERVIVVSMNYRVGALG  221

Human  121  .................L..........................................  180

Rhes   121  LYLNVWIPAPKPKNATVMIWIYGGGFQTGTSSLHVYDGKFLARVERVIVVSMNYRVGALG  180

Human  121  .................L..........................................  180

Cyno   121  LYLNVWIPAPKPKNATVMIWIYGGGFQTGTSSLHVYDGKFLARVERVIVVSMNYRVGALG  180

Human  121  .................L..........................................  180

Pig    121  LYLNVWIPAPKPKNATVMIWIYGGGFQTGTSSLHVYDGKFLSRVERVIVVSMNYRVGALG  180

Human  121  .................L.......................A..................  180

AfGrn  222  FLALPGNPEAPGNMGLFDQQLALQWVQKNIAAFGGNPKSVTLFGESAGAASVSLHLLSPG  281

Human  181  ............................................................  240

Rhes   181  FLALPGNPEAPGNMGLFDQQLALQWVQKNIAAFGGNPKSVTLFGESAGAASVSLHLLSPG  240

Human  181  ............................................................  240

Cyno   181  FLALPGNPEAPGNMGLFDQQLALQWVQKNIAAFGGNPKSVTLFGESAGAASVSLHLLSPG  240

Human  181  ............................................................  240

Pig    181  FLALPGNPEAPGNMGLFDQQLALQWVQKNIAAFGGNPKSVTLFGESAGAVSVSLHLLSPR  240

Human  181  .................................................A.........G  240

AfGrn  282  SHSLFTRAILQSGSSNAPWAVTSLYEARNRTLTLAKLTGCSRDNETEIVKCLRNKDPHEI  341

Human  241  ..............F.................N.........E.....I........Q..  300

Rhesu  241  SHPLFTRAILQSGSSNAPWAVTSLYEARNRTLTLAKLTGCSRDNETEIVKCLRNKDPHEI  300

Human  241  ..S...........F.................N.........E.....I........Q..  300

Cyno   241  SHSLFTRAILQSGSSNAPWAVTSLYEARNRTLTLAKLTGCSRDNETEIVKCLRNKDPHEI  300

Human  241  ..............F.................N.........E.....I........Q..  300

Pig    241  SHPLFARAILQSGSSNAPWAVTSLYEARNRTLTLAKFIGCSRENETEIIKCLRNKDPQEI  300

Human  241  ..S..T........F.................N...LT......................  300

AfGrn  342  LLNEAFVVPYGTLLSVNFGPTMDGDFLTDMPDILLELGQFKKTQILVGVNKDEGTAFLVY  401

Human  301  ............P........V......................................  360

Rhes   301  LLNEAFVVPYGTLLSVNFGPTMDGDFLTEMPDILLELGQFKKTQILVGVNKDEGTAFLVY  360

Human  301  ............P........V......D...............................  360

Cyno   301  LLNEAFVVPYGTLLSVNFGPTMDGDFLTEMPDILLELGQFKKTQILVGVNKDEGTAFLVY  360

Human  301  ............P........V......D...............................  360

Pig    301  LQNEVFVVPNHMLLSVNFGPTVDGDFLTDLPDTLLQLGQFKKTQILVGVNKDEGTAFLVY  360

Human  301  .L..A....YGTP................M..I..E........................  360

AfGrn  402  GAPGFSKDNNSIITRNEFQEGLKIFFPGVSEFGKESILFHYTDWVDDQRPENYREALDDV  461

Human  361  ...............K.........................................G..  420

Rhes   361  GAPGFSKDNDSIITRNEFQEGLKIFFPGVSEFGKESILFHYTDWVDDQRPENYREALDDV  420

Human  361  .........N.....K.........................................G..  420

Cyno   361  GAPGFSKDNDSIITRNEFQEGLKIFFPGVSEFGKESILFHYTDWVDDQRPENYREALDDV  420

Human  361  .........N.....K.........................................G..  420

Pig    361  GAPGFSKDNNSIITRKEFEEGLKIFFPGVSEFGKESILFHYMDWTDDQRAENYRDALDDV  420

Human  361  ..................Q......................T..V....P....E..G..  420

AfGrn  462  VGDYNIICPALEFTKKFSEWGNNAFFYYFEHRSSKLPWPEWMGVMHGYEIEFVFGLPLER  521

Human  421  .....F......................................................  480

Rhes   421  VGDYNIICPALEFTKKFSEWGNNAFFYYFEHRSSKLPWPEWMGVMHGYEIEFVFGLPLER  480

Human  421  .....F......................................................  480

Cyno   421  VGDYNIICPALEFTKKFSEWGNNAFFYYFEHRSSKLPWPEWMGVMHGYEIEFVFGLPLER  480

Human  421  .....F......................................................  480

Pig    421  VGDYDIICPALEFTKKFSEMGNNAFFYYFEHRSSKLPWPEWMGVMHGYEIEFVFGLPLER  480

Human  421  ....NF.............W........................................  480

AfGrn  522  RVNYTKAEEILSRSIVKRWANFAKYGNPNGTHNNSTKWPVFKSTEQKYLTLNTESSRILT  581

Human  481  .D...........................E.Q....S..................T..M.  540

Rhes   481  RVNYTKAEEILSRSIVKRWANFAKYGNPNGTHNNSTKWPVFKSTEQKYLTLNTESSRILT  540

Human  481  .D...........................E.Q....S..................T..M.  540

Cyno   481  RVNYTKAEEILSRSIVKRWANFAKYGNPNGTHNNSTKWPVFKSTEQKYLTLNTESSRILT  540

Human  481  .D...........................E.Q....S..................T..M.  540

Pig    481  RANYTKAEEILSRSIMKRWANFAKYGNPNGTQNNSTRWPVFKSNEQKYLTLNAESPRVYT  540

Human  481  .D.............V.............E......S......T........T..T.IM.  540

AfGrn  582  KLRAQQCRFWTSFFPKVLEMTGNIDEAEWEWKAGFHRWSNYMMDWKNQFNDYTSKKESCV  641

Human  541  ......................................N.....................  600

Rhes   541  KLRAQQCRFWTSFFPKVLEMTGNIDEAEWEWKAGFHRWSNYMMDWKNQFNDYTSKKESCV  600

Human  541  ......................................N.....................  600

Cyno   541  KLRAQQCRFWTSFFPKVLEMTGNIDEAEWEWKAGFHRWSNYMMDWKNQFNDYTSKKESCV  600

Human  541  ......................................N.....................  600

Pig    541  KLRAQQCRFWTLFFPKVLEMTGNIDEAEREWKAGFHRWNNYMMDWKNQFNDYTSKKESCA  600

Human  541  ...........S................W..............................V  600

AfGrn  642  GL  643

Human  601  ..  602

Rhes   601  GL  602

Human  601  ..  602

Cyno   601  GL  602

Human  601  ..  602

Pig    601  DL  602

Human  601  G.  602

Supplemental Table S3. Measured Concentrations of Sarin and IMPA using LC-MS/MS (ND = Not Detected)

| Sample ID | Time (hours) | [Sarin] (ng/mL) | [IMPA] (ng/mL) |
| --- | --- | --- | --- |
| Human (WT) BChE | 0 | ND | ND |
| Human (WT) BChE | 1 | ND | 0.17 |
| Human (WT) BChE | 2 | ND | 0.20 |
| Human (WT) BChE | 4 | ND | 0.19 |
| Human (WT) BChE | 6 | ND | 0.21 |
| Human (WT) BChE | 22 | ND | 0.26 |
| Human (Porcinated) BChE | 0 | ND | 0.31 |
| Human (Porcinated) BChE | 1 | ND | 0.44 |
| Human (Porcinated) BChE | 2 | ND | 0.45 |
| Human (Porcinated) BChE | 4 | ND | 0.57 |
| Human (Porcinated) BChE | 6 | ND | 0.66 |
| Human (Porcinated) BChE | 22 | ND | 0.68 |

Supplemental Table S4. Measured Concentrations of sarin over time using GC-HRMS

| Sample ID | Time (hours) | [Sarin] (ng/mL) |
| --- | --- | --- |
| 10 mM MOPS (-) Control | 0 | 1.61 |
| 10 mM MOPS (-) Control | 2 | 0.84 |
| 10 mM MOPS (-) Control | 5 | 0.60 |
| Porcinated-BChE | 0 | 1.57 |
| Porcinated -BChE | 2 | 0.97 |
| Porcinated -BChE | 5 | 0.42 |

For the supporting information:

Molecular Dynamics analysis of the native enzyme before and after mutation
	Molecular dynamics (MD) simulations were carried out for WT and porcinated forms of native BChE, that included each possible protonation states for the His283 in the porcinated form of the enzyme. All MD simulations were performed with the AMBER 16 molecular dynamics package 38 with the ff03 force field. The protein preparation was performed for the WT and porcinated forms of the enzyme with 1P0I human native form of BChE (Ref. 42) as previously eluded to in the computational methods.

Clustering analysis for the native enzyme

A clustering protocol using AMBER’s CPPTRAJ module was used in order to decrease the amount of structures for analysis in the native forms of the enzyme. A “representative” structure was chosen for each cluster that is color-coded by the relative populations of each cluster.

Figure S1: Cluster analysis by RMSD for the native forms of BChE color coded by relative populations of each cluster where blue is most populated, red is 2nd most populated, green is 3rd most populated, yellow is 4th most populated, pink is 5th most populated, and black is any cluster more than 5th at a 1 fs timestep.

Figure S2: Cluster analysis by RMSD for the native forms of BChE color coded by relative populations of each cluster where blue is most populated, red is 2nd most populated, green is 3rd most populated, yellow is 4th most populated, pink is 5th most populated, and black is any cluster more than 5th at a 2 fs timestep.

Root mean square fluctuation analysis for the native enzyme

The root mean square fluctuation (RMSF) was calculated using AMBER’s CPPTRAJ module for each residue in order to evaluate the flexibility of certain residues for the native enzyme. In order to compare the flexibility of the mutation site to the rest of the protein it is color-coded.

Figure S3: RMSF analysis for the native forms of BChE where the site of the mutation is color-coded red at a 1 fs timestep.

Figure S4: RMSF analysis for the native forms of BChE where the site of the mutation is color-coded red at a 2 fs timestep.

Distance analysis for the active site of the native enzyme

To observe the important interactions in the active site of the WT and porcinated forms of the native enzyme, AMBER’s PYTRAJ module was used in order to extract distances. Heat map plots were made for the WT and the HID, HIE, and HIP protonation states of the G283H mutation were made. The heat maps display the distances as a function of color over time. The interatomic distances measured are summarized below:

Interatomic distances:

- Oxyanion hole: Average distance from hydrogens from the backbone amides of Gly116, Gly117, and Ala199 to the O of Ser198.

- SerH  HisN: Distance from the H of Ser198 to the -nitrogen of the catalytic His438.

- HisH  Glu325: Distance from the -hydrogen of the catalytic His438 to the catalytic Glu325 oxyanion.

- HisH  Glu441: Distance from the -hydrogen of the catalytic His438 to the non-catalytic Glu441 oxyanion, this is an interaction that is often observed in the inhibited form of the enzyme.

- SerH  Glu197: Distance from the H of Ser198 to the adjacent Glu197 oxyanion.

- Gly115H  Glu197: Distance from the hydrogen from the backbone amide of Gly115 to the Glu325 oxyanion.

- Leu286  HisN: Distance from the closest carbon of the methyl group on Leu286 in the acyl-binding pocket to the  - Nitrogen of the catalytic His438.

- Phe329  HisN: Distance from the centroid of the benzyl group of Phe329 in the acyl-binding pocket to the   - Nitrogen of the catalytic His438.

Figure S5: Active site distances native forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S6: Active site distances native forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Distance analysis for the mutation site/acyl loop of the native enzyme

To observe the important interactions in the acyl loop and the site of the mutation of the WT and porcinated forms of the native enzyme, AMBER’s PYTRAJ module was used in order to extract distances. Heat map plots were made for the WT and the HID, HIE, and HIP protonation states of the G283H mutation were made. The heat maps display the distances as a function of color over time. The top 6 interatomic distances are shared between the WT and porcinated forms of the enzyme

Shared interatomic distances:

- Tyr332  Pro285back/Leu285back: Distance from the hydrogen of the alcohol group of Tyr332 to the backbone of Pro285 (WT) or Leu285 (porcinated) backbone.

- Tyr332  Ser79: Distance from the Distance from the hydrogen of the alcohol group of Tyr332 to the oxygen of the alcohol group of Ser79 located on the -loop.

- Tyr332  Gly75back: Distance from the hydrogen of the alcohol group of Tyr332 to the backbone of Gly75 located on the -loop.

- Tyr332  Gln71back: Distance from the hydrogen of the alcohol group of Tyr332 to the backbone of Gln71 located on the -loop.

- Tyr332  Phe329Cen: Distance from the hydrogen of the alcohol group of Tyr332 to the centroid of the benzyl group of Phe329 in the acyl-binding pocket.

- Tyr332  HisN: Distance from the hydrogen of the alcohol group of Tyr332 to the -nitrogen of the catalytic His438 located in the active site.

WT interatomic distances:

- Tyr282  Val288back: Distance from the hydrogen of the alcohol group of Tyr282 to the backbone of Val288 located in the acyl-binding pocket.

- Tyr282  Tyr332: Distance from the hydrogen of the alcohol group of Tyr282 to the hydrogen of the alcohol group of Tyr332.

- Tyr282  Tyr332back: Distance from the hydrogen of the alcohol group of Tyr282 to the backbone of Tyr332.

- Tyr282  Ile356back: Distance from the hydrogen of the alcohol group of Tyr282 to the backbone of Ile356.

Poricinated-Human BChE interatomic distances:

- His283  Tyr332: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the oxygen of the alcohol group of Tyr322.

- His283  Ser72back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Ser72 located on the -loop.

- His283  Pro230back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Pro230 located on the bottom of the acyl-binding pocket.

- His283  Asn282back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Asn282 located above the acyl-binding pocket.

- His283  Pro281back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Pro281 located above the acyl-binding pocket.

- His283  Val280back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Val280 located above the acyl-binding pocket.

Figure S7: Distances of the acyl loop and the site of the mutation for the native forms of BChE to evaluate different interactions near the mutation. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S8: Distances of the acyl loop and the site of the mutation for the native forms of BChE to evaluate different interactions near the mutation. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Distance analysis for Trp82 of the native enzyme

To visualize the stability and proximity of the omega loop to the active site of the native enzyme, AMBER’s PYTRAJ module was used in order to extract distances as well as an angle. Important distances from the  - Nitrogen of the catalytic His438 to the -carbon (C), the centroid of the amino ring (Cen1), and the centroid of the benzyl ring (Cen2) of Trp82. A summary of the distances and the angle measured are shown below:

Interatomic distances:

- C: Distance from the -nitrogen of the catalytic His438 located in the active site to the -Carbon of Trp82.

- Cen1: Distance from the -nitrogen of the catalytic His438 located in the active site to the centroid of the amino ring of Trp82.

- Cen2: Distance from the -nitrogen of the catalytic His438 located in the active site to the centroid of the benzyl ring of Trp82.

Interatomic angle:

- HisN  Cen1  Cen2: Angle formed by the -nitrogen of the catalytic His438 located in the active site to the centroid of the amino ring of Trp82 to the centroid of the benzyl ring of Trp82.

Figure S9: Distances and angle of W82 in the -loop as compared to the position and orientation of the -nitrogen of the catalytic His438 located in the active site at a 1 fs timestep.

Figure S10: Distances and angle of W82 in the -loop as compared to the position and orientation of the -nitrogen of the catalytic His438 located in the active site at a 2 fs timestep.

Figure S11: Illustration of “acyl-pocket encroachment” in the HID form of the native enzyme.

Figure S12: Illustration of active site “climbing” in the HIE form of the native enzyme.

Molecular Dynamics analysis of the inhibited enzyme before and after mutation

Moleculer dynamics (MD) simulations were carried out for WT and porcinated forms of inhibited BChE, that included each possible protonation states for the His283 in the porcinated form of the enzyme. All MD simulations were performed with the AMBER 16 molecular dynamics package with the ff03 force field.  The protein preparation was performed for the WT and porcinated forms of the enzyme with 2XQJ and 2XQK that are the (R)- and (S)- inhibited stereoisomers of VX, respectively, as previously eluded to in the computational methods.

Clustering analysis for the inhibited enzyme

A clustering protocol using AMBER’s CPPTRAJ module was used in order to decrease the amount of structures for analysis in the inhibited forms of the enzyme. A “representative” structure was chosen for each cluster that is color-coded by the relative populations of each cluster.

Figure S13: Cluster analysis by RMSD for the 2XQJ inhibited forms of BChE color coded by relative populations of each cluster where blue is most populated, red is 2nd most populated, green is 3rd most populated, yellow is 4th most populated, pink is 5th most populated, and black is any cluster more than 5th at a 1 fs timestep.

Figure S14: Cluster analysis by RMSD for the 2XQJ inhibited forms of BChE color coded by relative populations of each cluster where blue is most populated, red is 2nd most populated, green is 3rd most populated, yellow is 4th most populated, pink is 5th most populated, and black is any cluster more than 5th  at a 2 fs timestep.

Figure S15: Cluster analysis by RMSD for the 2XQK inhibited forms of BChE color coded by relative populations of each cluster where blue is most populated, red is 2nd most populated, green is 3rd most populated, yellow is 4th most populated, pink is 5th most populated, and black is any cluster more than 5th at a 1 fs timestep.

Figure S16: Cluster analysis by RMSD for the 2XQK inhibited forms of BChE color coded by relative populations of each cluster where blue is most populated, red is 2nd most populated, green is 3rd most populated, yellow is 4th most populated, pink is 5th most populated, and black is any cluster more than 5th at a 2 fs timestep.

Root mean square fluctuation analysis for the inhibited enzyme

The root mean square fluctuation (RMSF) was calculated using AMBER’s CPPTRAJ module for each residue in order to evaluate the flexibility of certain residues for the inhibited enzyme. In order to compare the flexibility of the mutation site to the rest of the protein it is color-coded.

Figure S17: RMSF analysis for the 2XQJ inhibited forms of BChE where the site of the mutation is color-coded red at a 1 fs timestep.

Figure S18: RMSF analysis for the 2XQJ inhibited forms of BChE where the site of the mutation is color-coded red at a 2 fs timestep.

Figure S19: RMSF analysis for the 2XQK inhibited forms of BChE where the site of the mutation is color-coded red at a 1 fs timestep.

Figure S20: RMSF analysis for the 2XQK inhibited forms of BChE where the site of the mutation is color-coded red at a 2 fs timestep.

Distance analysis for the active site of the inhibited enzyme

To observe the important interactions in the active site of the WT and porcinated forms of the inhibited enzyme, AMBER’s PYTRAJ module was used in order to extract distances. Heat map plots were made for the WT and the HID, HIE, and HIP protonation states of the G283H mutation for 2XQJ and 2XQK were made. The heat maps display the distances as a function of color over time. The interatomic distances measured are summarized below:

Interatomic distances:

- Oxyanion hole: Average distance from hydrogens from the backbone amides of Gly116, Gly117, and Ala199 to the O1 of phosphylated Ser198 (Sarin198) which is the oxygen that is located closest to these backbone amides.

- HisH  O1: Distance from the -hydrogen of the catalytic His438 to the O1 of phosphylated Ser198 (Sarin198) which is the oxygen that is located in the oxyanion hole.

- HisH  O2: Distance from the -hydrogen of the catalytic His438 to the O2 of phosphylated Ser198 (Sarin198) which is the oxygen connected to the isopropyl leaving group.

- HisH  O: Distance from the -hydrogen of the catalytic His438 to the O1 of phosphylated Ser198 (Sarin198) which is the oxygen connected to the protein.

- HisH  Glu325: Distance from the -hydrogen of the catalytic His438 to the catalytic Glu325 oxyanion.

- HisH  Glu441: Distance from the -hydrogen of the catalytic His438 to the non-catalytic Glu441 oxyanion, this is an interaction that is often observed in the inhibited form of the enzyme.

- HisH  Glu197: Distance from the -hydrogen of the catalytic His438 to the Glu197 oxyanion.

- Gly115H  Glu197: Distance from the hydrogen from the backbone amide of Gly115 to the Glu325 oxyanion.

- Leu286  HisN: Distance from the closest carbon of the methyl group on Leu286 in the acyl-binding pocket to the  - Nitrogen of the catalytic His438.

- Phe329  HisN: Distance from the centroid of the benzyl group of Phe329 in the acyl-binding pocket to the   - Nitrogen of the catalytic His438.

Figure S21: Active site distances for the 2XQJ inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S22: Active site distances for the 2XQJ inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Figure S23: Active site distances for the 2XQK inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S24: Active site distances for the 2XQK inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Distance analysis for the mutation site/acyl loop of the inhibited enzyme

To observe the important interactions in the acyl loop and the site of the mutation of the WT and porcinated forms of the inhibited enzyme, AMBER’s PYTRAJ module was used in order to extract distances. Heat map plots were made for the WT and the HID, HIE, and HIP protonation states of the G283H mutation for 2XQJ and 2XQK were made. Heat maps display the distances as a function of color over time. The top 6 interatomic distances are shared between the WT and porcinated forms of the enzyme

Shared interatomic distances:

- Tyr332  Pro285back/Leu285back: Distance from the hydrogen of the alcohol group of Tyr332 to the backbone of Pro285 (WT) or Leu285 (porcinated) backbone.

- Tyr332  Ser79: Distance from the Distance from the hydrogen of the alcohol group of Tyr332 to the oxygen of the alcohol group of Ser79 located on the -loop.

- Tyr332  Gly75back: Distance from the hydrogen of the alcohol group of Tyr332 to the backbone of Gly75 located on the -loop.

- Tyr332  Gln71back: Distance from the hydrogen of the alcohol group of Tyr332 to the backbone of Gln71 located on the -loop.

- Tyr332  Phe329Cen: Distance from the hydrogen of the alcohol group of Tyr332 to the centroid of the benzyl group of Phe329 in the acyl-binding pocket.

- Tyr332  HisN: Distance from the hydrogen of the alcohol group of Tyr332 to the -nitrogen of the catalytic His438 located in the active site.

WT interatomic distances:

- Tyr282  Val288back: Distance from the hydrogen of the alcohol group of Tyr282 to the backbone of Val288 located in the acyl-binding pocket.

- Tyr282  Tyr332: Distance from the hydrogen of the alcohol group of Tyr282 to the hydrogen of the alcohol group of Tyr332.

- Tyr282  Tyr332back: Distance from the hydrogen of the alcohol group of Tyr282 to the backbone of Tyr332.

- Tyr282  Ile356back: Distance from the hydrogen of the alcohol group of Tyr282 to the backbone of Ile356.

Porcinated Human BChE interatomic distances:

- His283  Tyr332: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the oxygen of the alcohol group of Tyr322.

- His283  Ser72back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Ser72 located on the -loop.

- His283  Pro230back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Pro230 located on the bottom of the acyl-binding pocket.

- His283  Asn282back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Asn282 located above the acyl-binding pocket.

- His283  Pro281back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Pro281 located above the acyl-binding pocket.

- His283  Val280back: Distance from the -(HID) or -(HIE) hydrogen or the closest of the two (HIP) of His283 to the backbone of Val280 located above the acyl-binding pocket.

Figure S25: Distances of the acyl loop and the site of the mutation for the 2XQJ inhibited forms of BChE to evaluate different interactions near the mutation. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S26: Distances of the acyl loop and the site of the mutation for the 2XQJ inhibited forms of BChE to evaluate different interactions near the mutation. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Figure S27: Distances of the acyl loop and the site of the mutation for the 2XQK inhibited forms of BChE to evaluate different interactions near the mutation. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S28: Distances of the acyl loop and the site of the mutation for the 2XQK inhibited forms of BChE to evaluate different interactions near the mutation. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Distance analysis for Trp82 of the inhibited enzyme

To visualize the stability and proximity of the omega loop to the active site of the inhibited enzyme, AMBER’s PYTRAJ module was used in order to extract distances as well as an angle. Distance plots were made for the WT and the HID, HIE, and HIP protonation states of the G283H mutation for 2XQJ and 2XQK were made. Important distances from the  - Nitrogen of the catalytic His438 to the -carbon (C), the centroid of the amino ring (Cen1), and the centroid of the benzyl ring (Cen2) of Trp82. A summary of the distances and the angle measured are shown below:

Interatomic distances:

- C: Distance from the -nitrogen of the catalytic His438 located in the active site to the -Carbon of Trp82.

- Cen1: Distance from the -nitrogen of the catalytic His438 located in the active site to the centroid of the amino ring of Trp82.

- Cen2: Distance from the -nitrogen of the catalytic His438 located in the active site to the centroid of the benzyl ring of Trp82.

Interatomic angle:

- HisN  Cen1  Cen2: Angle formed by the -nitrogen of the catalytic His438 located in the active site to the centroid of the amino ring of Trp82 to the centroid of the benzyl ring of Trp82.

Figure S29: Distances and angle of W82 in the 2XQJ inhibited form in the -loop as compared to the position and orientation of the -nitrogen of the catalytic His438 located in the active site at a 1 fs timestep.

Figure S30: Distances and angle of W82 in the 2XQJ inhibited form in the -loop as compared to the position and orientation of the -nitrogen of the catalytic His438 located in the active site at a 2 fs timestep.

Figure S31: Distances and angle of W82 in the 2XQK inhibited form in the -loop as compared to the position and orientation of the -nitrogen of the catalytic His438 located in the active site at a 1 fs timestep.

Figure S32: Distances and angle of W82 in the 2XQK inhibited form in the -loop as compared to the position and orientation of the -nitrogen of the catalytic His438 located in the active site at a 2 fs timestep.

Distance analysis for spontaneous hydrolysis of the inhibited enzyme

To observe the important water interactions in the WT and porcinated forms of the inhibited enzyme, AMBER’s PYTRAJ module was used in order to extract distances. Heat map plots were made for the WT and the HID, HIE, and HIP protonation states of the G283H mutation for 2XQJ and 2XQK were made. The heat maps display the distances as a function of color over time. The interatomic distances measured are summarized below:

Interatomic distances:

- phosph  wat1: Distance from the phosphorus of the inhibited Ser198 to the oxygen of the closest water to the phosphorous atom of the inhibited Ser198.

- phosph  wat2: Distance from the phosphorus of the inhibited Ser198 to the oxygen of the 2nd closest water to the phosphorous atom of the inhibited Ser198.

- HisN  wat1: Distance from the -nitrogen of the catalytic His438 to the center of mass of the hydrogens of the closest water to the phosphorus atom of the inhibited Ser198.

- HisN  wat2: Distance from the -nitrogen of the catalytic His438 to the center of mass of the hydrogens of the 2nd closest water to the phosphorus atom of the inhibited Ser198.

- HisH  Glu197: Distance from the -hydrogen of the catalytic His438 to the Glu197 oxyanion.

- Glu197  wat1: Distance from the Glu197 oxyanion to the center of mass of the hydrogens of the closest water to the phosphorus atom of the inhibited Ser198.

- Glu197  wat2: Distance from the Glu197 oxyanion to the center of mass of the hydrogens of the closest water to the phosphorus atom of the inhibited Ser198.

Figure S33: Water interaction distances for the 2XQJ inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S34: Water interaction distances for the 2XQJ inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Figure S35: Water interaction distances for the 2XQK inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 1 fs timestep.

Figure S36: Water interaction distances for the 2XQK inhibited forms of BChE color-coded by the different interactions in the active site. Distances are displayed as heat maps and are shown as a function of color over time at a 2 fs timestep.

Figure S37: Example of an “active” water for Pathway 2 that sits in between the phosphylated S198 and E197. This is taken from the most populated cluster of the HID form of 2XQK inhibition at a 2 fs timestep.

Figure S38: Example of an “active” water for Pathways 1 and 2 that sits in between the phosphylated S198 and E197. This is taken from the third most populated cluster of the HID form of 2XQK at a 2 fs timestep.

Figure S39: Illustration of the water pocket formed in between E197 and phosphylated S198 for the top 3 clusters of the HID form of 2XQK. Each of the individual clusters is color coded by population: blue, most populated; red, next most populated; and green, lowest population at a 2 fs timestep.