/**

* \file        index.doc

* \author      Cyrille Bonamy and Julien Chauchat

* \date        June 03, 2021

* \brief       index (doc)

*/

/*!

\mainpage

<!--sedFOAM Manual-->

@image html "cylinder3d.png" width=800px

@image latex "cylinder3d.png"

<h2>Introduction</h2>

This documentation describes the three-dimensional two-phase flow solver for sediment transport applications:  SedFoam. The solver is based on twoPhaseEulerFoam available in the official release of the open-source CFD software OpenFOAM (based on OF2.4). In this approach the sediment phase is modeled as a continuum, and constitutive laws have to be prescribed for the sediment stresses. In the proposed solver, two different inter-granular stress models are implemented: the kinetic theory of granular flows and the dense granular flow rheology &mu(I). For the fluid stress, laminar or turbulent flow regimes can be simulated and three different turbulence models are available for sediment transport: a simple mixing length model (one-dimensional configuration only), a &k-&epsilon and a &k-&omega model.

Different easy benchmarks/tutorials_1d are provided to illustrate the model capabilities: a sedimentation of spherical partiles at low Reynolds number, the laminar bed-load problem, the turbulent sheet-flow regime. Moreover some advanced multi-dimensional cases/tutorials_2d3d are proposed. The source code and tutorial cases are disseminated at the public website: https://github.com/sedfoam/sedfoam and detailed tutorial description.

This manual is written as the documentation of the open-source solver sedFoam, it includes the installation procedure and a description of the model input/output files.

The sedfoam solver should be referenced using the GMD paper :

Chauchat, J., Cheng, Z., Nagel, T., Bonamy, C., and Hsu, T.-J. (2017) SedFoam-2.0: a 3-D two-phase flow numerical model for sediment transport, Geosci. Model Dev., 10, 4367-4392, https://doi.org/10.5194/gmd-10-4367-2017.

and the reference in which a specific configuration and/or feature has been first presented (please see list of references in the documentation).

If you are using a sedFoam version without modification, you should also refer to the Zenodo DOI of the version, current version is https://doi.org/10.5281/zenodo.836642

The documentation is divided into two parts, the first one is the user manual:

<ul>

<li>Section \ref install explains how to download, compile and install SedFoam.

<li>Section \ref howtodocker provides a survival guide on how to use SedFoam docker

<li>Section \ref governing describes sedFoam governing equations.

<li>Section \ref algoSedFoam describes sedFoam algorithm.

<li>Section \ref tutorials_1d describes the 1D tutorials provided with SedFoam sources including postprocessing using a python toolbox also provided with the source and the expected results.

<li>Section \ref tutorials_2d3d describes the 2D and 3D tutorials provided with SedFoam sources

<li>Section \ref publicationList list of publications using SedFoam

<li>Section \ref inputs describes the input parameters for SedFoam

<li>Section \ref howtogit provides a survival guide on how to use git to use/develop sedFoam

<li>Section \ref faq provides answers to typical Frequently Asked Questions about SedFoam

</ul>

The second part provides informations for developers.

\n<h2>sedFoam license</h2>

Copyright &copy; 2021 by

<a href="mailto:cyrille.bonamy@univ-grenoble-alpes.fr">Cyrille Bonamy</a>,

<a href="mailto:julien.chauchat@univ-grenoble-alpes.fr">Julien Chauchat</a>,

<a href="https://sites.udel.edu/thsu/group-members">Tian-Jian Hsu</a>,

and <a href="credits-contributors.html">contributors.</a><p>

Permission to use, copy, modify, and distribute this software and its

documentation under the terms of the GNU General Public License is hereby

granted. No representations are made about the suitability of this software

for any purpose. It is provided "as is" without express or implied warranty.

See the

<a href="http://www.gnu.org/licenses/old-licenses/gpl-2.0.html">

GNU General Public License</a>

for more details.

<p>

<h2>Acknowledgments</h2>

This project is supported by many organisations.

<a href="credits-contributors.html">Please find the list here.</a>

\htmlonly

Go to the <a href="install.html">next</a> section.

\endhtmlonly

*/