/**

* \file        input.doc

* \author      Cyrille Bonamy and Julien Chauchat

* \date        June 03, 2021

* \brief       Input parameters (doc)

*/

/*! \page inputs Input Parameters

@brief Description of the sedFoam input parameters

\tableofcontents

The input parameters for sedFoam are numerous and are modifiable through different files in the current working directory. Some of the files are identical to other openFoam solvers and won't be described in details herein. This is the case for the files:

\verbatim

system/blockMesh

system/controlDict

system/fvSchemes

system/fvSolution

\endverbatim

The file 'blockMesh' contains the description of the mesh (see \ref meshgen). The file 'controlDict' contains the runtime parameters such as the timestep and the output interval, etc... The files 'fvScheme' describes the numerical shemes for the different differential operators (temporal, divergens, laplacian) and the file 'fvSolution' describes the algebraic solvers and the associated tolerance for each equation as well as the PIMPLE algorithm controls such as the number of inner and outer iterations ...

The files that contain the specific input parameters for sedFoam are:

\verbatim

constant/g

constant/transportProperties

constant/interfacialProperties

constant/forceProperties

constant/ppProperties

constant/granularRheologyProperties

constant/kineticTheoryProperties

constant/turbulenceProperties

constant/twophaseRASProperties

\endverbatim

The file 'g' contains the gravity vector. The user shall be careful when using a 1D/2D configuration that 'g' has to be carried by the 'y' direction.

\section inputParams Physical processes

sedFoam includes a variety of physical processes: fluid-particle forces such as drag froce, fluid and particle phase rheology, elastic pressure for the solid phase, turbulence modeling. Each process can be switched on or off. The various tutorials available with the source code give a good overview of different combination of physical processes. The table below lists the keywords used to switch on or off physical processes in sedFoam and the key input parameters.

In the following the input parameters will be described file-by-file.

\subsection transportProperties File transportProperties

In the file 'transportProperties' the user can specify the input parameters listed in the table below:

parameter     | description            |    default   |     range    |    units     |

--------------|------------------------|--------------|--------------|--------------|

phasea        | solid phase            |              |              |              |

phased        | fluid phase            |              |              |              |

rho           | phase density          |      1000    | 1-2650       |  kg/m^3      |

nu            | phase viscosity        |      1e-6    | 0-1e0        |  m^2/s       |

d             | particle diameter      |      200e-6  | 62e-6-1      |  m           |

sF            | shape factor           |      1       | 0.1-2        |  -           |

hExp          | Hindrance exponent     |      2.65    | 1.5-3.15     |  -           |

Ws0           | settling velocity      | 1.15e-4      | 1e-6-1e-4    |  m/s         |

WsGel         | set. vel. gel point    | 8.53e-6      | 1e-5-1e-3    |  m/s         |

dimFrac       | fractal dimension      | 2.748        | 2-3          |  -           |

Xi            | empirical coef.        | 1.155        | 1-10         |  -           |

alphaGel      | gel point              | 0.28         | 0.05-0.4     |  -           |

phiMax        | floc concentration     | 0.85         | 0.5-0.9      |  -           |

rhoFloc       | floc density           | 1566.59      | 1050-2000    | kg/m^3       |

alphaSmall    | Min. solid vol. frac.  | 1e-6         | 1e-9-1e-5    |  -           |

alphaDiffusion| articifical Diffusivity| 0            | 0-1e-6       | m^2/s        |

nuMax         | maximium viscosity     | 1            | 1e-2-1e6     | m^2/s        |

nuFraMax      | maximium sed viscosity | 1e3          | 1e-2-1e6     | m^2/s        |

nuMaxExp      | idem for Explicit terms| 1            | 1e-2-1e6     | m^2/s        |

\subsection interfacialProperties File interfacialProperties

In the file 'transportProperties' the user can specified the input parameters listed in the table below:

dragModela/b           |   description                                                      |

-----------------------|--------------------------------------------------------------------|

Ergun                  | for dense packing of spheres (consistent with Darcy's law)         |

Gibilaro               |                                                                    |

GidaspowErgunWenYu     |                                                                    |

GidaspowSchillerNaumann|                                                                    |

SchillerNaumann        | for dilute suspensions at any Re_p                                 |

SyamlalOBrien          |                                                                    |

WenYu                  |                                                                    |

Camenen                | for mud flocs (see Chauchat et al 2013)                            |

\subsection ppProperties File ppProperties

In the file 'ppProperties' the user can specify the input parameters for the elastic pressure model for 'pff':

ppModel                  |   description                                                      |

-------------------------|--------------------------------------------------------------------|

JohnsonJackson           | default model (for spheres)                                        |

Hsu                      | model for spheres, Fr represents an elastic modulus                |

MerckelbachKranenburg    | model for mud (based on fractal dimension)                         |

Chauchat                 | model for mud (see Chauchat et al., 2013)                          |

Depending on the ppModel, the following parameters need to be specified:

parameter       | description               |    default   |     range    |    units     |

----------------|---------------------------|--------------|--------------|--------------|

alphaMax        | max. solid vol. fraction  |    0.635     | 0.57-0.635   |  -           |

alphaMinFriction| random loose packing frac.|    0.57      | 0.53-0.57    |  -           |

Fr              | elastic modulus           |    0.05      | 0.001-1e8    |  Pa          |

eta0            | empirical exponent        |    3         | 1-3          |  -           |

eta1            | empirical exponent        |    5         | 5            |  -           |

\subsection granularRheologyProperties File granularRheologyProperties

In the file 'granularRheologyProperties' the user can specify the input parameters for the dense granular flow rheology, in particular the user decides if he/she wants to use the dense granular flow rheology or not by setting the switch 'granularRheology' to on/off. If it's switched on, the user has to decide which rheological model to use for the different components, _i.e._ granular shear stresss, granular normal stress and fluid shear stress, amongst:

FrictionModel      |   description                                                     |

-------------------|-------------------------------------------------------------------|

none               |     no friction                                                   |

Coulomb            | Coulomb friction with constant friction coef mus                  |

MuI                | mu(I) friction model in the inertial regime                       |

MuIv               | mu(Iv) friction model in the viscous regime                       |

PPressureModel     |   description                                                     |

-------------------|-------------------------------------------------------------------|

none               | no particle phase shear induced pressure                          |

MuI                | mu(I) friction model in the inertial regime                       |

MuIv               | mu(Iv) friction model in the viscous regime                       |

FluidViscosityModel|   description                                                     |

-------------------|-------------------------------------------------------------------|

none               | effective viscosity = pure fluid viscosity                        |

Einstein           | Einstein (1906) viscosity model for dilute supensions             |

KriegerDougherty   | Krieger and Dougherty (1959) viscosity model for dense supensions |

BoyerEtAl          | Boyet et al. (2011) viscosity model for dense supensions          |

The following list of parameters can be modified from the 'granularRheologyProperties' file:

parameter       | description               |    default   |     range    |    units     |

----------------|---------------------------|--------------|--------------|--------------|

alphaMaxG       | max. solid vol. fraction  |    0.6       | 0.55-0.6     |  -           |

mus             | static friction coeff.    |    0.38      | 0.3-0.7      |  -           |

mu2             | dynamic friction coeff.   |    0.64      | 0.6-1.2      |  -           |

I0              | empirical coeff. mu(I)    |    0.3       | 0.005-0.6    |  -           |

Bphi            | empirical coeff. phi(I)   |    0.31      | 0.3-1        |  -           |

n               | exponent. effective visc. |    2.5       | 1-3          |  -           |

BulkFactor      | coef for 2nd viscosity    |    0         | 0-10         |  -           |

Dsmall          | regularisation param.     |    1e-6      | 1e-4-1e-9    |  1/s         |

relaxPa         | relaxation factor for Pa  |    1.0      | 1e-9-1       |  -           |

Additionally, the switch 'granularDilatancy' found in the file 'granularRheologyProperties' is used to incoporate dilatancy effects in the model. This feature is important to capture the coupling between the pore

pressure feedback mechanism and the initial dynamics of a granular medium subjected to shear stress.

\subsection forceProperties File forceProperties

In the file 'forceProperties' the user can specify the input parameters for the external pressure gradient and select the fluid-particle forces to be added in the model, _e.g._ lift force, added mass force, Ct model:

parameter       | description                      |    default   |     range    |    units     |

----------------|----------------------------------|--------------|--------------|--------------|

gradPMEAN       | mean pressure gradient           |    0         | 0-           |  kg/m^2/s^2  |

gradPAMP1       | 1st order Stokes wave contrib.   |    0         | 0-           |  kg/m^2/s^2  |

gradPAMP2       | 2nd order Stokes wave contrib.   |    0         | 0-           |  kg/m^2/s^2  |

oscpT           | wave period                      |    0         | 0-           |  s           |

initTheta       | initial wave phase               |    0         | 0-359        |  deg         |

tilt            | to mimic bedslope with gradPMEAN |    0         | 0 or 1       |  -           |

Cvm             | virtual mass force coef.         |    0         | 0-10         | -            |

Cl              | Lift force coef.                 |    0         | 0-10         | -            |

Ct              | Ct model coef. nuEffa=Ct^2 nutb  |    0         | 0-1          | -            |

InitFreeze      | Freeze solid phase               |    0         | 0, 1 or 2    | -            |

fixbeddepth     | depth below which to freeze      |    0         | 0 to max(Y)  | m            |

ClipUa          | impose Ua=Ub for alpha<alphaSmall|    0         | 0 or 1       | -            |

writeTau        | write shear stress tensors       |    false     | true or false| boolean      |

debugInfo       | show debugging information       |    false     | true or false| boolean      |

spongeLayer     | sponge layer                     |    false     | true or false| boolean      |

xSmin           | min. X position for sponge layer |    0         | min(X)-max(X)| m            |

xSmax           | max. X position for sponge layer |    0         | min(X)-max(X)| m            |

\subsection turbulenceProperties File turbulenceProperties

In the file 'turbulenceProperties' the user can specify the turbulence model to be used:

simulationType     |   description                                                     |

-------------------|-------------------------------------------------------------------|

laminar            | laminar flow - no turbulence model (nutb=0)                       |

RAS                | Reynolds Averaged turbulence model                                |

If simulationType=RAS is selected, the user needs to specify a turbulence model using the keyword 'RASModel' in the 'RAS' dictionnary. All singlephase turbulence model can be selected and specific two-phase flow turbulence models are provided with sedFoam:

RASModel                |   description                                                     |

------------------------|-------------------------------------------------------------------|

twophaseMixingLength    | two-phase flow mixing length model (see chauchat et al. 2017, GMD)|

twophasekEpsilon        | two-phase flow k-epsilon model (see chauchat et al. 2017, GMD)    |

twophaseLaunderSharmaKE | two-phase flow k-epsilon model from Launder and Sharma            |

twophasekOmega          | two-phase flow k-omega model (see chauchat et al. 2017, GMD)      |

twophasekOmega2006      | two-phase flow k-omega model from Wilcox (2006)                   |

\subsection twophaseRASProperties File twophaseRASProperties

In the file 'twophaseRASProperties' the user can specified the input parameters for the turbulence model:

parameter       | description                      |    default   |     range    |    units     |

----------------|----------------------------------|--------------|--------------|--------------|

SUSread         | turbulent Schmidt number         |    0         | 0-3          |  -           |

SUSlocal        | switch to turn on local SUS model|    false     | true or false|  boolean     |

Usmall          | min. u* value for local SUS      |  1e-6        | 1e-9-1e-4    | m/s          |

Cmu             | Cmu constant for k-eps or k-omega|  0.09        | 0.09         | -            |

B               | empirical parameter for turb drag|  0.25        | 0.1-1        | -            |

kSmall          | min. Tp value for turb. drag     |  1e-6        | 1e-9-1e-4    | m^2/s^2      |

TpSmall         | min. Tp value for turb. drag     |  1e-6        | 1e-9-1e-4    |  kg/m^3/s    |

nutMax          | max. eddy viscosity value        |  -           | 5e-3-1e1     | m^2/s        |

KE1             | coef. for density stratif (Uf-Us)|   0          | 0 or 1       | -            |

KE2             | coef. for turbulence modulation  |   0          | 0 or 1       | -            |

KE3             | coef. for turbulence generation  |   0          | 0 or 1       | -            |

KE4             | coef. for density stratif g      |   0          | 0 or 1       | -            |

*/