This document includes a plan for developing modules within the new FAST modularization framework for the example problems from: http://www.nrel.gov/docs/fy13osti/57298.pdf. While these example problems are quite trivial, this document can be used as a “how-to” reference when developing a plan for more sophisticated modules.

As this example demonstrates, a good plan for a module within the new FAST modularization framework will follow the definitions and nomenclature used by: http://www.nrel.gov/docs/fy13osti/57228.pdf, and should clearly:

- State whether the module is intended for loose coupling, tight coupling for time marching, and/or tight coupling for linearization.

- Define the module’s inputs (including initialization), outputs (including initialization), states (continous, discrete, and constraint), and parameters, including units.

- Lay out an example input file for the module.

- Explain the module’s mathemetical formulation, including Jacobians (for tight coupling), in the form required of the framework.

- Prescribe how the module’s inputs are derived from the outputs of other specific modules.

- Identify any potential numerical problems and how to avoid them in the code.

- Lay out the module’s subroutines using pseudocode (as opposed to actual code), including identifying which mathematical formulas are used by which subroutines, and describing the algorithms used in the solution process.

The plan should be written before writing source code and discussed with the tool’s primary owner.  When written well, the plan will greatly aid in the programming effort and is a useful jumping off point for writing the module’s user’s guide and theory manual.

Module 1

# Description

This module represents System 1 from: http://www.nrel.gov/docs/fy13osti/57298.pdf.

The module is given the module name ModuleName = Module1 and the abbreviated name ModName = Mod1.

The mathematical formulation of this module is a subset of the most general form permitted by the FAST modularization framework in tight coupling, thus, the module is developed to support both loose and tight coupling (tight coupling for both time marching and linearization).

# Inputs, Outputs, States, and Parameters

These are the nonlocal variables that must be defined in the module’s Registry.

## Initialization Input

None

Normally an input file name would be placed here (among other variables).  But in this example, for simplicity, the parameters and initial states are set within the initialization subroutine instead of being read from an input file.

## Initialization Output

None

## Inputs ()

=	coupling force (N)

## Outputs ()

=	displacement of mass 1 (m)

=	velocity of mass 1 (m/s)

## States

### Continuous States ()

=	displacement of mass 1 (m)

=	velocity of mass 1 (m/s)

### Discrete States ()

None

### Constraint States ()

None

### Other States ()

These variables are needed when using a multistep integrator in loose coupling.

=	time-step counter (-)

=	first time derivative of the continuous states  at time  ([]/s)

=	first time derivative of the continuous states  at time  ([]/s)

=	first time derivative of the continuous states  at time  ([]/s)

=	first time derivative of the continuous states  at time  ([]/s)

## Parameters ()

=	mass of mass 1 (kg)

=	damping of dashpot 1 (N/(m/s))

=	stiffness of spring 1 (N/m)

=	external force applied to mass 1 (constant or dependent only on time, ) (N)

=	module time step (increment) (s)

=	integration method {1: RK4, 2: AB4, 3: ABM4} (switch)

# Mathematical Formulation

The mathematical formulation of this module is a subset of the most general form permitted by the FAST modularization framework in tight coupling.  Because there are no discrete states, the module represents a pure continuous-time system.  Because there are no constraint states, the equations of motion form ODEs.  The system is naturally linear, so, the equivalent linear state-space system is also identified below.

## Continuous-State Functions ()

The continuous-state equations are  with

, or equivalently, 	(1)

To avoid a division-by-zero error, the condition  should be checked when it is set at module initialization in Mod1_Init.

## Discrete-State Functions ()

None

## Constraint-State Functions ()

None

## Output Functions ()

The output equations are  with

, or equivalently, 	(2)

This module does not have direct feedthrough of input to output.

# Jacobians

The absence of discrete and constraint states means that all Jacobians are blank except for the Jacobians identified below.  All Jacobians can be calculated analytically (as opposed to numerically through a perturbation technique).  Because the system is naturally linear, the Jacobians are independent of the operating point.

## Jacobians with respect to Continous States

(3)

(4)

## Jacobians with respect to Inputs

(5)

(6)

# Input-Output Transformation

When system (module) 1 is coupled to system (module) 2, the input to this module exactly equals the output of module 2.  The input-output transformation equation for this module is  with

(7)

The Jacobians of the input-output transformation equation for this module are

and 	(8)

When system (module) 1 is coupled to system (module) 3, the input to this module exactly equals the output of module 3.  The input-output transformation equation for this module is  with

(9)

The Jacobians of the input-output transformation equation for this module are

and 	(10)

# Subroutines

Because the module supports both loose and tight coupling (tight coupling for both time marching and linearization), all of the subroutines in the template are used.  The absence of discrete and constraint states means that the Mod1_CalcConstrStateResidual, Mod1_UpdateDiscState, Mod1_JacobianPDiscState, and Mod1_JacobianPConstrState routines are blank.

## Mod1_Init

In this example, for simplicity, the parameters and initial states are set within the initialization subroutine instead of being read from an input file.This initialization subroutine is used to:

- Set the parameters, .  The condition  is checked to avoid a future division-by-zero error.

- Set an initial guess for the input, .

- Set initial values of the continous states, .

- Set the module time step, .

## Mod1_End

This end subroutine is used to release memory.

## Mod1_UpdateStates

This time-stepping subroutine is used to advance the continuous states from the current time step to the next time step using one of three different integrators: RK4, AB4, and ABM4.  Each integration method is documented separately below.  The nomenclature used below is as follows: , , and  are some of the input arguments to Mod1_UpdateStates;  is the output argument from Mod1_UpdateStates;  represents a CALL to Mod1_CalcContStateDeriv; and  represents a CALL to Mod1_Input_ExtrapInterp.

### RK4 ()

The RK4 method is an explicit ODE integrator that uses midpoints between time steps.

### AB4 ()

This AB4 method is an explicit multistep ODE integrator that is initialized with 3 steps of RK4.   is used to store previously saved values.  Backing up in time is not supported with multistep methods.

### ABM4 ():

This ABM4 method is a predictor-corrector multistep ODE integrotor that is initialized with 3 steps of RK4 and uses AB4 for the predictor.   is used to store previously saved values.  Backing up in time is not supported with multistep methods.

## Mod1_CalcOutput

This time-stepping subroutine is used to compute the outputs per Eq. (2).

## Mod1_CalcContStateDeriv

This time-stepping subroutine is used to compute the first time derivative of the continuous states per Eq. (1).

## Mod1_UpdateDiscState

This time-stepping subroutine is blank due to the absence of discrete states.

## Mod1_CalcConstrStateResidual

This time-stepping subroutine is blank due to the absence of constraint states.

## Mod1_JacobianPContState

This Jacobian subroutine is used to compute the Jacobians of Eqs (3)-(4).

## Mod1_JacobianPInput

This Jacobian subroutine is used to compute the Jacobians of Eqs (5)-(6).

## Mod1_JacobianPDiscState

This Jacobian subroutine is blank due to the absence of discrete states.

## Mod1_JacobianPConstrState

This Jacobian subroutine is blank due to the absence of constraint states.

Module 2

# Description

This module represents System 2 from: http://www.nrel.gov/docs/fy13osti/57298.pdf.

The module is given the module name ModuleName = Module2 and the abbreviated name ModName = Mod2.

The mathematical formulation of this module is a subset of the most general form permitted by the FAST modularization framework in tight coupling, thus, the module is developed to support both loose and tight coupling (tight coupling for both time marching and linearization).

# Inputs, Outputs, States, and Parameters

These are the nonlocal variables that must be defined in the module’s Registry.

## Initialization Input

None

Normally an input file name would be placed here (among other variables).  But in this example, for simplicity, the parameters and initial states are set within the initialization subroutine instead of being read from an input file.

## Initialization Output

None

## Inputs ()

=	displacement of virtual interaction node (m)

=	velocity of virtual interaction node (m/s)

## Outputs ()

=	coupling force (N)

## States

### Continuous States ()

=	displacement of mass 2 (m)

=	velocity of mass 2 (m/s)

### Discrete States ()

None

### Constraint States ()

None

### Other States ()

These variables are needed when using a multistep integrator in loose coupling.

=	time-step counter (-)

=	first time derivative of the continuous states  at time  ([]/s)

=	first time derivative of the continuous states  at time  ([]/s)

=	first time derivative of the continuous states  at time  ([]/s)

=	first time derivative of the continuous states  at time  ([]/s)

## Parameters ()

=	mass of mass 2 (kg)

=	damping of dashpot 2 (N/(m/s))

=	stiffness of spring 2 (N/m)

=	external force applied to mass 2 (constant or dependent only on time, ) (N)

=	damping of coupling dashpot (N/(m/s))

=	stiffness of coupling spring (N/m)

=	module time step (increment) (s)

=	integration method {1: RK4, 2: AB4, 3: ABM4} (switch)

# Mathematical Formulation

The mathematical formulation of this module is a subset of the most general form permitted by the FAST modularization framework in tight coupling.  Because there are no discrete states, the module represents a pure continuous-time system.  Because there are no constraint states, the equations of motion form ODEs.  The system is naturally linear, so, the equivalent linear state-space system is also identified below.

## Continuous-State Functions ()

The continuous-state equations are  with

, or equivalently, 	(11)

To avoid a division-by-zero error, the condition  should be checked when it is set at module initialization in Mod2_Init.

## Discrete-State Functions ()

None

## Constraint-State Functions ()

None

## Output Functions ()

The output equations are  with

, or equivalently, 	(12)

This module has direct feedthrough of input to output.

# Jacobians

The absence of discrete and constraint states means that all Jacobians are blank except for the Jacobians identified below.  All Jacobians can be calculated analytically (as opposed to numerically through a perturbation technique).  Because the system is naturally linear, the Jacobians are independent of the operating point.

## Jacobians with respect to Continous States

(13)

(14)

## Jacobians with respect to Inputs

(15)

(16)

# Input-Output Transformation

The input to this module exactly equals the output of system (module) 1.  The input-output transformation equation for this module is  with

(17)

The Jacobians of the input-output transformation equation for this module are

and 	(18)

# Subroutines

Because the module supports both loose and tight coupling (tight coupling for both time marching and linearization), all of the subroutines in the template are used.  The absence of discrete and constraint states means that the Mod2_CalcConstrStateResidual, Mod2_UpdateDiscState, Mod2_JacobianPDiscState, and Mod2_JacobianPConstrState routines are blank.

## Mod2_Init

In this example, for simplicity, the parameters and initial states are set within the initialization subroutine instead of being read from an input file.This initialization subroutine is used to:

- Set the parameters, .  The condition  is checked to avoid a future division-by-zero error.

- Set an initial guess for the input, .

- Set initial values of the continous states, .

- Set the module time step, .

## Mod2_End

This end subroutine is used to release memory.

## Mod2_UpdateStates

This time-stepping subroutine is used to advance the continuous states from the current time step to the next time step using one of three different integrators: RK4, AB4, and ABM4.  Each integration method is documented separately below.  The nomenclature used below is as follows: , , and  are some of the input arguments to Mod2_UpdateStates;  is the output argument from Mod2_UpdateStates;  represents a CALL to Mod2_CalcContStateDeriv; and  represents a CALL to Mod2_Input_ExtrapInterp.

### RK4 ()

The RK4 method is an explicit ODE integrator that uses midpoints between time steps.

### AB4 ()

This AB4 method is an explicit multistep ODE integrator that is initialized with 3 steps of RK4.   is used to store previously saved values.  Backing up in time is not supported with multistep methods.

### ABM4 ():

This ABM4 method is a predictor-corrector multistep ODE integrotor that is initialized with 3 steps of RK4 and uses AB4 for the predictor.   is used to store previously saved values.  Backing up in time is not supported with multistep methods.

## Mod2_CalcOutput

This time-stepping subroutine is used to compute the outputs per Eq. (12).

## Mod2_CalcContStateDeriv

This time-stepping subroutine is used to compute the first time derivative of the continuous states per Eq.  (11).

## Mod2_UpdateDiscState

This time-stepping subroutine is blank due to the absence of discrete states.

## Mod2_CalcConstrStateResidual

This time-stepping subroutine is blank due to the absence of constraint states.

## Mod2_JacobianPContState

This Jacobian subroutine is used to compute the Jacobians of Eqs (13)-(14).

## Mod2_JacobianPInput

This Jacobian subroutine is used to compute the Jacobians of Eqs (15)-(16).

## Mod2_JacobianPDiscState

This Jacobian subroutine is blank due to the absence of discrete states.

## Mod2_JacobianPConstrState

This Jacobian subroutine is blank due to the absence of constraint states.

Module 3

# Description

This module represents System 3 from: http://www.nrel.gov/docs/fy13osti/57298.pdf.

The module is given the module name ModuleName = Module3 and the abbreviated name ModName = Mod3.

The mathematical formulation of this module is a subset of the most general form permitted by the FAST modularization framework in tight coupling, thus, the module is developed to support both loose and tight coupling (tight coupling for both time marching and linearization).

# Inputs, Outputs, States, and Parameters

These are the nonlocal variables that must be defined in the module’s Registry.

## Initialization Input

None

Normally an input file name would be placed here (among other variables).  But in this example, for simplicity, the parameters and initial states are set within the initialization subroutine instead of being read from an input file.

## Initialization Output

None

## Inputs ()

=	displacement of virtual interaction node (m)

## Outputs ()

=	coupling force (N)

## States

### Continuous States ()

None

### Discrete States ()

None

### Constraint States ()

=	horizontal tension in the cable (N)

### Other States ()

None

## Parameters ()

=	horizontal span of the cable when  is zero (m)

=	unstretched cable length (m)

=	cable weight per unit length (N/m)

=	cable cross-section extensional stiffness (N)

=	tolerance used in Newton-Raphson iteration and to avoid ill-conditioning of the module equations (N)

=	maximum number of iterations used in Newton-Raphson iteration (-)

=	module time step (increment) (s)

# Mathematical Formulation

The mathematical formulation of this module is a subset of the most general form permitted by the FAST modularization framework in tight coupling.  Because there are no discrete states, the module represents a pure continuous-time system.  Because there are no continuous states, the equations are algebraic only.  The system is naturally nonlinear.

## Continuous-State Functions ()

None

## Discrete-State Functions ()

None

## Constraint-State Functions ()

The constraint-state equation is  with

(19)

To avoid division-by-zero errors, the conditions  and  should be checked when they are set at module initialization in Mod3_Init.  There is no closed-form solution of Eq. (19) for , so the constraint-state equation must be solved iteratively.  To ensure  such that a solution always exists, the condition  should also be checked when it is set at module initialization in Mod3_Init.  (The Jacobian of the constraint-state function with respect to the constraint state, , is given in Eq. (21) below.)  When  approaches ,  approaches zero and Eq. (19) becomes ill-conditioned; when , the correct solution is ; otherwise, .  Unless the limiting case of  () is explicitly implemented,  should be kept from equaling zero in the iterative solution process.  This can be achieved by replacing  with  and  with .

## Output Functions ()

The output equations are  with

(20)

Even though the sign of  depends on , the module does not have direct feedthrough of input to output because , as given in Eq. (24) below.

# Jacobians

The absence of continuous and discrete states means that all Jacobians are blank except for the Jacobians identified below.  All Jacobians can be calculated analytically (as opposed to numerically through a perturbation technique).  Because the system is naturally nonlinear, the Jacobians depend on the operating point.  The Jacobians are implemented such that the Jacobians are accurate even when the constraint state doesn’t satisfy the constraint-state equation exactly (so as to be useful in the iterative solution procedure).  Although not all explicitly shown below, the Jacobians are undefined at  (), but again, this limiting case can be avoided.

## Jacobians with respect to Constraint States

(21)

(22)

In Eq. (21),  on the right-hand side comes from the right-hand side of Eq. (19) and would equal zero if the constraint state satisfies the constraint-state equation exactly.  Just like Eq. (19), Eq. (21) becomes ill-conditioned as  approaches zero, which can be avoided by replacing  with  and  with .  In Eq. (22),  is arbitrarily replaced with unity when .

## Jacobians with respect to Inputs

(23)

(24)

In Eq. (24),  is arbitrarily replaced with unity when .

# Input-Output Transformation

The input to this module exactly equals the displacement output of system (module) 1.  The input-output transformation equation for this module is  with

(25)

The Jacobians of the input-output transformation equation for this module are

and 	(26)

# Subroutines

Because the module supports both loose and tight coupling (tight coupling for both time marching and linearization), all of the subroutines in the template are used.  The absence of continuous and discrete states means that the Mod3_CalcContStateDeriv, Mod3_UpdateDiscState, Mod3_JacobianPContState, and Mod3_JacobianPDiscState routines are blank.

## Mod3_Init

In this example, for simplicity, the parameters and initial states are set within the initialization subroutine instead of being read from an input file.This initialization subroutine is used to:

- Set the parameters, .  The conditions , , and  should be checked when they are set to avoid a future division-by-zero error and to ensure that there is always a solution.

- Set an initial guess for the input, .

- Set initial guess of the constraint states, .

- Set the module time step, .

## Mod3_End

This end subroutine is used to release memory.

## Mod3_UpdateStates

This time-stepping subroutine is used to solve the constraint-state equation for the constraint state using Newton-Raphson iteration, based on the specified convergence tolerance.  A fatal error should be set if  is reached before the convergence tolerance is met.  The nomenclature used below is as follows:  and  are some of the input arguments to Mod3_UpdateStates;  is the output argument from Mod3_UpdateStates;  represents a CALL to Mod3_CalcConstrStateResidual;  represents a CALL to Mod3_JacobianPConstrState; and  represents a CALL to Mod3_Input_ExtrapInterp.

## Mod3_CalcOutput

This time-stepping subroutine is used to compute the outputs per Eq. (20).  Even though there are no potential numerical problems with Eq. (20), to ensure consistency with the remaining routines, the constraint state is kept from equaling zero by arbitrarily replacing  with unity when and  with .

## Mod3_CalcContStateDeriv

This time-stepping subroutine is blank due to the absence of continuous states.

## Mod3_UpdateDiscState

This time-stepping subroutine is blank due to the absence of discrete states.

## Mod3_CalcConstrStateResidual

This time-stepping subroutine is used to compute the residual of the constraint-state function per Eq. (19).  To avoid numerical problems, the constraint state is kept from equaling zero by replacing  with  and  with .

## Mod3_JacobianPContState

This Jacobian subroutine is blank due to the absence of continuous states.

## Mod3_JacobianPInput

This Jacobian subroutine is used to compute the Jacobians of Eqs (23)-(24).  In Eq. (24),  is arbitrarily replaced with unity when .

## Mod3_JacobianPDiscState

This Jacobian subroutine is blank due to the absence of discrete states.

## Mod3_JacobianPConstrState

This Jacobian subroutine is used to compute the Jacobians of Eqs (21)-(22).  In Eq. (21),  represents a call to Mod3_CalcConstrStateResidual, and to avoid numerical problems, the constraint state is kept from equaling zero by replacing  with  and  with .  In Eq. (22),  is arbitrarily replaced with unity when .