Bio-oil production from biogenic wastes, the hydrothermal conversion step

Geert Haarlemmer 1, Anne Roubaud 1, Bruno Lacaze 1

1 CEA-LITEN, Université Grenoble Alpes, 17, rue des martyrs, 38054 GRENOBLE Cedex 9, France

Abstract

Food wastes are an abundant resource that can be effectively valorised by hydrothermal liquefaction to produce bio-fuels. The objective of the European project Waste2Road is to demonstrate the complete value chain from waste collection to engine tests. The principle of hydrothermal liquefaction is well known but there are still many factors that make the science very empirical. Most experiments in the literature are performed on batch reactors. Comparison of results from batch reactors with experiments with continuous reactors are rare in the literature.

This dataset presents fully documented experiments, performed in this project, on food wastes, with different compositions, conditions and solvents. The data set is extended with data from the literature. This data set also includes bio-oil and aqueous phase analysis by gas chromatography coupled with mass spectrometry.

# Chromatography of the bio-oil.

GCMS analysis of the bio-oil solubilised in ethyl acetate was performed for samples issues of W2R 29 run. A comparison was made for two different preparation procedures: the biocrude was in a case only dried at room temperature and another sample was analysed after azeotropic distillation with toluene to remove water.

Figure 1: Gas chromatography of bio-oil after azeotropic distillation from continuous experiment W2R-29

Figure 2: Gas chromatography of air dried bio-oil from continuous experiment W2R-29

Table 1 Percentage area of the different identified peaks are given in the table below for W2R 29.

| Retention time | Identification | Bio-oil distilled | Bio-oil air dried |
| --- | --- | --- | --- |
| min |  | area % | area % |
| 16.126 | Pyrazine, methyl- | 2.31 | 1.83 |
| 19.505 | Pyrazine, 2,5-dimethyl- | 2.17 | 1.62 |
| 20.689 | 2-Cyclopenten-1-one, 2-methyl- | 3.96 | 4.76 |
| 22.148 | 1-Pentanone, 1-(4-methylphenyl)- | 7.90 | 9.23 |
| 23.589 | Cyclopentene, 3-propyl- | 1.81 | 1.50 |
| 25.422 | Cyclopentene, 3-propyl- | 1.86 | 1.78 |
| 26.414 | Phenol | 9.20 | 9.50 |
| 26.799 | Phenol, 2-methoxy- | 3.79 | 3.01 |
| 27.633 | 1H-Pyrrole, 2,3,4,5-tetramethyl- | 2.04 | 2.71 |
| 28.51 | Pyrazine, ethyl- | 3.54 | 4.92 |
| 29.05 | Cysteine | 0.72 | 1.02 |
| 29.09 | p-Amino-DL-phenylalanine | 0.88 | 0.90 |
| 30.569 | Paredrine | 7.14 | 8.90 |
| 31.316 | Phenol, 4-ethyl-2-methoxy- | 8.83 | 8.43 |
| 33.189 | Dopamine | 4.25 | 3.50 |
| 39.281 | Citronellyl isobutyrate | 1.57 | 1.56 |
| 41.866 | Tridecanoic acid, methyl ester | 5.15 | 2.09 |
| 45.857 | 9-Dodecenoic acid, methyl ester, (E)- | 15.41 | 2.09 |
| 46.003 | 1-Tridecyne | 1.00 | 0.88 |
| 53.414 | Hexanamide | 1.41 | 13.20 |
| 53.68 | Hexane, 2,5-dimethoxy-2,5-dimethyl- |  | 1.26 |
| 59.845 | Dodecanamide | 9.46 | 7.33 |
| 60.119 | 2-Tridecyne | 5.62 | 5.28 |

Figure 2: Oil composition by family from continuous experiment W2R-29

# Chromatography of the aqueous phases.

GCMS analysis was performed after extraction 50/50 with ethyl acetate for the different continuous runs. An example of chromatograms are given below with identification of the main species. Calibration for quantification was not performed. Area percentage are provided for information. Organic species in the aqueous phase are either oxygenates like alcohol, ketones, cyclic ethers, phenolic species or nitrogen containing species like pyrazine and derivatives

A comparison of chromatograms for three continuous experiments W2R 29, 28 and 30 is given below with identifications obtained for W2R 29 of the main responsive species indicated with the relative peak area (%) of the total area of identified peaks in the analysis. The detected peaks are comparable for the three analyses. The abundance are sometimes different but the major ones are similar in their relative abundance. The heaviest species have 13 carbons and include nitrogen and oxygen atoms.

Figure 2: Chromatography after extraction of the aqueous phase of the continuous experiments W2R-29, 28 and 30.

Table 1 Percentage area of the different identified peaks in the water phase are given in the table below for W2R-29.

| Retention Time (min) | Area (%) | Identification | Formula | Mole weight |
| --- | --- | --- | --- | --- |
| 10.232 | 0.68 | Acetic acid | C2H4O2 | 60 |
| 11.277 | 15.00 | 2-Propanone, 1-hydroxy- | C3H6O2 | 74 |
| 11.907 | 3.29 | Pyrazine | C4H4N2 | 80 |
| 13.249 | 1.15 | 2-Butanone, 3-hydroxy- | C4H8O2 | 88 |
| 15.35 | 1.61 | Propanoic acid | C3H6O2 | 74 |
| 15.752 | 1.96 | Cyclopentanone | C5H8O | 84 |
| 16.05 | 1.28 | Pyrazine, methyl- | C5H6N2 | 94 |
| 17.112 | 6.28 | 3-Pentanol | C5H12O | 88 |
| 17.544 | 1.06 | 4-Methyl-2-hexanol | C7H16O | 116 |
| 18.559 | 1.32 | 2-Cyclopenten-1-one | C5H6O | 82 |
| 19.423 | 2.62 | Pyrazine, 2,5-dimethyl- | C6H8N2 | 108 |
| 19.621 | 0.82 | Pyrazine, ethyl- | C6H8N2 | 108 |
| 19.872 | 0.24 | Pyrazine, 2,3-dimethyl- | C6H8N2 | 108 |
| 20.654 | 5.85 | 2-Cyclopenten-1-one, 2-methyl- | C6H8O | 96 |
| 21.215 | 0.42 | 1-Butanone, 1-(2-furanyl)- | C8H10O2 | 138 |
| 22.843 | 0.62 | 2,5-Hexanedione | C6H10O2 | 114 |
| 23.578 | 0.75 | 2-Cyclopenten-1-one, 2,3-dimethyl- | C7H10O | 110 |
| 23.794 | 1.82 | 2-Cyclopenten-1-one, 3-methyl- | C6H8O | 96 |
| 23.934 | 1.72 | Butyrolactone | C4H6O2 | 86 |
| 24.191 | 0.32 | trans,trans-3,5-Heptadien-2-one | C7H10O | 110 |
| 25.404 | 0.73 | 2-Cyclopenten-1-one, 2,3-dimethyl- | C7H10O | 110 |
| 25.48 | 2.53 | 2-Cyclopenten-1-one, 2-hydroxy-3-methyl- | C6H8O2 | 112 |
| 26.385 | 2.03 | Phenol | C6H6O | 94 |
| 26.776 | 1.05 | Phenol, 2-methoxy- | C7H8O2 | 124 |
| 28.497 | 0.99 | 2-Amino-5,6-dihydro-4,4,6-trimethyl-4H 1,3-oxazine | C7H14N2O | 142 |
| 28.672 | 0.14 | 2.5-Pyrrolidinedione,1-methyl- | C5H7NO2 | 113 |
| 29.087 | 0.36 | 2,6-Piperazinedione, 4,4'-(1-methyl-1,2-ethanediyl) bis-, (S)- | C11H16N4O4 | 268 |
| 30.33 | 1.06 | 2H-Pyran, 3,4-dihydro-6-methyl- | C6H10O | 98 |
| 31.38 | 4.11 | 3-Pyridinol | C5H5NO | 95 |
| 35.949 | 1.06 | Hydroquinone | C6H6O2 | 110 |
| 36.79 | 0.63 | 2,5-Furandicarboxaldehyde | C6H4O3 | 124 |
| 44.825 | 0.39 | dl-Alanyl-l-leucine | C9H18N2O3 | 202 |
| 45.414 | 0.43 | 1H-Purine-2,6-dione, 3,7-dihydro-1,3,7-trimethyl- | C8H10N4O2 | 194 |
| 49.097 | 0.24 | Pyrrolo[1,2-a]pyrazine-1,4-dione, hexahydro-3-(2-methylpropyl)- | C11H18N2O2 | 210 |
| 50.363 | 0.20 | 1,2,3-Trimethyl-5-(2-thia-n-hexyl)piperid-4-one | C13H25NOS | 243 |
| 50.52 | 0.45 | Pyrrolo[1,2-a]pyrazine-1,4-dione, hexahydro-3-(2-methylpropyl)- | C11H18N2O2 | 210 |