14.10.2022

This README file describes the dataset files made available in open access on Zenodo.

For any further queries, please contact Hadeel S. Obaid (hadeel.obaid@tuni.fi).

How to cite this package:

H. Obaid, B. Tan, C. Morlaas, and E.S. Lohan, “I/Q measurements with 5G SRS signals and receiver 4-ports 3D Vector Antenna for positioning studies”

This dataset contains the I\Q data of four received signals from a 4-port 3D Vector Antenna (3D VA) as well as *fig and *png examples of angle of arrival (AoA)/azimuth angle estimation using the MUSIC algorithm based on the raw data. The data was collected from four ports (p5, p6, p7, p8) of a 3D VA provided by ENAC. A single Yagi antenna has been used as a transmitter at 2.1GHz carrier frequency and horizontal polarization.

The 3D VA is described in our publication:

B. Tan, E. S. Lohan, B. Sun, W. Wang, T. Yesilyurt, C. Morlaas, C. D. Morales Pena, K. Abdo, F.  Ben Slama, A. Simonin, and  M. Ellejmi,  “Improved Sensing and Positioning via 5G and mmWave radar for Airport Surveillance”, in Proceedings  of SESAR 3 Joint Undertaking, 2021, available  at https://www.sesarju.eu/sites/default/files/documents/sid/2021/papers/SIDs_2021_paper_4.pdf and at https://arxiv.org/abs/2202.13650

Five pairs of the incident azimuth angle 𝝋 and incident Elevation angle θ were used: [(45°,45°), (170°,90°), (190°,80°), (210°,70°), (230°,60°)].

The measurements were collected using four different SRS 5G signals (stored in folders SRS1 to SRS4). Each SRS 5G signal was used in both Line of Sight (LoS) scenario and in Non-Line of Sight (NLoS) scenario.

In-Lab setup

The in-lab measurement set-up is depicted in the following block diagram

Three USRP (two NI USRP-2954R and one NI USRP-2953R) are set in a synchronized setup using an OctoClock CDA-2990. One USRP as a transmitter (TX) and the other two as a receiver (RXs).  Two USRP receivers are needed in our configuration because each receiver has two channels for receiving and we need four channels receiving to use the four ports of the 3D VA.

Pictures of the set-up in LOS and NLOS conditions are shown below:

| LOS | NLOS |
| --- | --- |

File structure and Data description:

- The dataset file is in WinRAR ZIP format which contains six folders:

- Examples of AoA Estimation of the Azimuth angle in LOS

- Examples of AoA Estimation of the Azimuth angle in NLOS

- SRS1

- SRS2

- SRS3

- SRS4

- The folders Examples of AoA Estimation of the Azimuth angle in LOS and Examples of AoA Estimation of the Azimuth angle in NLOS contain figures of some of our AoA estimators (in *fig and *png formats), based on MUSIC algorithm, in LOS and NLOS respectively. The MUSIC algorithm is not provided, as it was implemented in LabView for our USRPs, but these results can be used as benchmark results.

- Each of the folders SRS1, SRS2, SRS3, and SRS4 contains:

- LOS: contains five folders of different elevation and azimuth pairs angles used in LOS scenarios and each folder contains two *csv files of the AoA Estimation and the I\Q data of 4Channels Receivers. The AoA Estimation *csv files contain the raw data collected, where column B shows the angle estimation indexes and column C shows the maximum values.  The columns from D to MZ are the MUSIC algorithm spectrum.

- NLOS: contains five folders of different elevation and azimuth pairs angles used in NLOS scenarios and each folder contains two *csv files of the AoA Estimation and the I\Q data of 4Channels Receivers. The AoA Estimation *csv files contain the raw data collected, where column B shows the angle estimation indexes and column C shows the maximum values.  The columns from D to MZ are the MUSIC algorithm spectrum.

- TX _SRS_Signal: it’s the I/Q data of the corresponding transmitted (SRS) signal in a *csv file.

This dataset was generated as part of the experimental activities of the project “Evaluation of 5G Network and mmWave Radar Sensors to Enhance Surveillance of the Airport Surface (NewSense)” funded by EU H2020 SESAR Joint Undertake, under the grant 893917.