Minimal Working Manual

The k-means code is written by Chelsea Mahn. This minimal working manual is written by Pengning Xu based on his experiences from the k-means kinetic analysis of trinucleotide repeat DNA hairpin smFRET data. Sample traces are included in the same folder.

Before starting the steps, add the ‘classDefinitionsAndFunctions’ folder to the MATLAB search path or put the folder in the same folder with the ‘kmeans.mlapp’ program. Run the ‘kmean.mlapp’ program. A very helpful note: if you mess up any of the steps below, there is an ‘Undo!’ button that restore the analysis back to the last finished step.

- Load traces

Click the ‘Load’ button in the top left corner and choose the file type of your smFRET data. For example, we use ‘itx’ format in the TNR hairpin sample traces. select one trace in the data folder then the program will load all the traces files in the same folder. You can see the traces showing up in the figure panel.

- Data filtering

Click the leftmost ‘Data filtering’ tab below the figure panel. All the steps in data filtering are completely optional in generating the k-means clusters from the program.

  - Remove trace head and tail sections if you use the red/green laser alternative mode for imaging

Find the ‘Ren Green Alt Mode’ section in the ‘Data filtering’ panel. Input frame number in the ‘Green Start Frame’ and ‘Green End Frame’ boxes, which indicate the start and end frame of the trace section you want to analyze. Click the ‘Remove Red Frames’ button and the unwanted part of trace will be removed in the figure panel.

  - Remove photoblinking and photobleaching events and smooth the traces

Find the ‘Threshold Filter’ section in the ‘Data filtering’ panel. In the ‘Date to use’ line, choose the types of signal intensity (donor, acceptor, donor+acceptor, or FRET) to perform the filtering. Input the minimal threshold value in the ‘min’ box and you can see the data points below the threshold turn gray in the traces. The default action is to remove the data points instead of replacing them, as indicated in the ‘Action To Take’ line.

To smooth the FRET traces, check the ‘Smooth Data’ box and input smoothing parameters in the same line. The ‘Padding’ box removes the suggested number of data points at the beginning and end of traces based on the average window size.

  - Confirm all the data filtering choices

When choices are finalized, click the ‘Apply’ button at the bottom of the ‘Threshold Filter’ section.

- k-means analysis, post-analysis, and results export

Click ‘K-means Options’ tab below the figure panel.

  - Pick k-means parameters and run the analysis

Finish all the pre-analysis setting in the ‘K-means Settings’ panel. Input the number of clusters for k-means analysis. Choose the types of signal intensity (donor, acceptor, donor+acceptor, or FRET) for k-means analysis. Choose the criteria for measuring distance between data points and cluster center points. Choose the method to calculate the cluster center points. To perform k-means analysis on the smoothed traces, check the ‘Smooth Data’ box and input smoothing parameters in the same line. When choices are finalized, click the ‘Run K-means’ button on the bottom. The program will analyze all the traces in the data folder.

  - Remove single frame transitions after k-means analysis

In the ‘Single Frame Analysis’ panel, pick the parameters and click the ‘Analyze Single Frames’ button.

  - Remove first and last dwells from each trace after k-means analysis

In the ‘Remove First and Last’ panel, click the ‘Remove First and Last’ button.

  - Export generated clusters.

Input the frame rate of time traces in msec unit and click the ‘Export Results’ button. A new folder containing all the results will be created in the same trace folder. Text file named ‘analysisDetails’ records all the analysis steps from the MATLAB APP to obtains the clusters. Most importantly, text file named ‘dwellTimeResults’ records all the individual dwells (not individual cluster!) including the dwell’s original trace name, starting time, dwelling time, average FRET efficiency value, average FRET efficiency value of next dwell, standard deviation of FRET efficiencies, and affiliated cluster central value.

To generate the kinetic rates. I use the custom MATLAB script to import the dwell information from the ‘dwellTimeResults’ text files and perform the exponential fitting. The custom script named ‘AllstateKineticsKmeans.m’ is also attached.