Minimal Working Manual

This code for implementing a k-means algorithm for detection of transition edges in single molecule FRET data was written by Chelsea Mahn. This minimal working manual was written by Pengning Xu based on his experiences using the k-means kinetic analysis program for analysis of trinucleotide repeat DNA hairpin smFRET data. Sample data are included with this distribution in the folder ‘(CAG)15 Sample Trace Clean’. The results of an example analysis of these tis data is in the folder ‘(CAG)15 Sample Trace Analyzed’, and the set of parameters used for this analysis is in the file ‘analysisDetails.txt’ that is in the subfolder \(CAG)15 Sample Trace Analyzed\20220513KmeansAnalysisResults\Analysis05.

The program is a Matlab app written under version 2020a. Before running the app, add with subfolders the ‘classDefinitionsAndFunctions’ folder to the MATLAB search path Then run the program ‘kmeans.mlapp’. It is very helpful to note the following: if you make a mistake in any of the steps below, there is an ‘Undo!’ button that will restore the analysis back to the last completed step.

- Load traces

Click to open the ‘Load’ menu on the menu bar in the top left corner and choose the file type of your smFRET data. For the example data provided in the TNR hairpin sample traces, select the ‘itx’ format. Select one trace in the data folder then the program will load all the traces files in the same folder. The first trace will appear in the figure panel where acceptor, donor, sum of acceptor and donor, and FRET efficiency are plotted. Directly below the traces graphs is the file name of the currently displayed file. The two boxes to the right of the name will step through displaying different traces in the folder.

- Data filtering

Click the leftmost tab ‘Data filtering’, which is in the lower part of the window, below the trajectory figures. All the steps in data filtering are completely optional in generating the k-means clusters from the program.

  - Remove trace head and tail sections of traces. Our experiments use red laser to directly excite the acceptor for an interval at the beginning and end of the acquisition interval. This step can remove these sections and retain only the green laser illuminated middle interval where FRET is possible. We call this protocol, red-green-alt-mode.

Find the ‘Red Green Alt Mode’ section in the ‘Data filtering’ panel. Input the first and last frame number of the trace section you want to analyze (the middle interval where green laser excited FRET) in the ‘Green Start Frame’ and ‘Green End Frame’ boxes. For the provided example data, the start frame is 20 and the end frame is 4000 (and use 0 in the ‘num shutter switch frames’ box). After entering values, click the ‘Remove Red Frames’ button and the unwanted part of trace will be removed in the figure panel and will not be included in further analysis.

  - Remove photoblinking and photobleaching events and smooth the traces

Find the ‘Threshold Filter’ section in the ‘Data filtering’ panel. In the ‘Date to use’ line, choose the types of signal intensity (donor, acceptor, donor+acceptor, donor AND acceptor, donor OR acceptor, or FRET) to perform the filtering. Input the minimal threshold value in the ‘min’ box and you can see the data points below the threshold turn gray in the traces. The default action is to remove the data points instead of replacing them, as indicated in the ‘Action To Take’ line.

To smooth the FRET traces, check the ‘Smooth Data’ box and input smoothing parameters in the same line. The ‘Padding’ box removes the suggested number of data points at the beginning and end of traces based on the average window size.

  - Confirm all the data filtering choices

When choices are finalized, click the ‘Apply’ button at the bottom of the ‘Threshold Filter’ section.

- k-means analysis, post-analysis, and results export

Click the ‘K-means Options’ tab below the figure panel, beside the ‘Data Filtering’ tab.

  - Choose k-means parameters and run the analysis

Finish all the pre-analysis setting in the ‘K-means Settings’ panel. Input the number of clusters for k-means analysis. Choose the types of signal intensity (donor, acceptor, donor+acceptor, or FRET) for k-means analysis. Choose the criteria for measuring distance between data points and cluster center points. Choose the method to calculate the cluster center points. To perform k-means analysis on the smoothed traces, check the ‘Smooth Data’ box and input smoothing parameters in the same line. When choices are finalized, click the ‘Run K-means’ button on the bottom. The program will analyze all the traces in the data folder. You may change the parameters and repeat the k-means run as many times as you like. Note, the parameters used in our example analysis (in the folder \(CAG)15 Sample Trace Analyzed\20220513KmeansAnalysisResults\Analysis05) is available in the file analysisDetails.txt.

  - Remove single frame transitions after k-means analysis

In the ‘Single Frame Analysis’ panel, pick the parameters and click the ‘Analyze Single Frames’ button.

  - Remove first and last dwells from each trace after k-means analysis

For kinetic studies, the first and last states have uncertain duration because they began before observation (first state) and end after termination of the observation (last state). We remove them for kinetic analyses. In the ‘Remove First and Last’ panel, click the ‘Remove First and Last’ button.

  - Export generated clusters.

Input the frame integration time used to acquire the time traces in milliseconds and click the ‘Export Results’ button. (Note the provided example traces were acquired with 10 msec frame integration time).  A new folder containing all the results will be created within the folder that contains the data. The text file created that is named ‘analysisDetails’ records all the analysis steps from the MATLAB APP to obtains the clusters. Most importantly, the text file named ‘dwellTimeResults’ records all the individual dwells (not individual cluster!) including the dwell’s original trace name, starting time, dwelling time, average FRET efficiency value, average FRET efficiency value of next dwell, standard deviation of FRET efficiencies, and affiliated cluster central value.

Details of the k-means algorithm are available published papers (1-2).

To generate the kinetic rates, you can perform your own analysis on the information in the ‘dwellTimeResults’ text file. This will require pooling dwell times from each FRET state (cluster) and perform the exponential fitting. The sequence of transitions is also available in the ‘dwellTimeResults’ text file, which is needed to calculate the ‘branching ratios’ for systems with more than 2 states (3). Included with the current distribution are the custom MATLAB script we use to import the dwell information from the ‘dwellTimeResults’ text files and perform the kinetic analysis. The custom script is named ‘AllstateKineticsKmeans.m’. Note, it requires additional files to run, ObsWindowLength.txt and Settings.mat.  These files are generated from our related CK edge detection code, which is available at the same location as this k-means package.

References

1 M. Götz, et al., A blind benchmark of analysis tools to infer kinetic rate constants from single-molecule FRET trajectories. Nat Commun 13, 5402 (2022).

2 Pengning Xu, Jiahui Zhang, Feng Pan, Chelsea Mahn, Christopher Roland, Celeste Sagui*, Keith Weninger, Frustration between preferred states of complementary trinucleotide repeat DNA hairpins correlates with expansion disease propensity, preprint submitted.

3. C. D. Kinz-Thompson, N. A. Bailey, R. L. Gonzalez, Precisely and Accurately Inferring Single-Molecule Rate Constants. Methods Enzymol 581, 187–225 (2016).