- COAWST

- User’s Manual

- Version 3.3.1

http://woodshole.er.usgs.gov/operations/modeling/COAWST/index.html

July 22, 2018

"Although this program has been used by the USGS, no warranty, expressed or implied, is made by the USGS or the United States Government as to the accuracy and functioning of the program and related program material nor shall the fact of distribution constitute any such warranty, and no responsibility is assumed by the USGS in connection therewith."

- Table of Contents.

1.   Introduction

2.   Accessing COAWST

3.   Required libraries for model installation

4.   Setting up and running applications

5.   Compiling and Running

6.   Distributed Projects/examples

7.   Visualization Tools

8.   Setting up a WRF application

9.   Setting up a ROMS application

10. Setting up a SWAN application

11. Setting up a WAVEWATCH III application

12. Setting up an InWave application

13. MATLAB (.m) scripts for pre/post processing

14. Tracking model development changes log

15. Previous COAWST training workshops and Data

16. List of References and Acknowledgements

- COAWST User’s Manual

- 1. Introduction

The Coupled-Ocean-Atmosphere-Wave-Sediment Transport (COAWST) Modeling System is an agglomeration of open-source modeling components that has been tailored to investigate coupled processes of the atmosphere, ocean, and waves in the coastal ocean. The modeling system currently contains:

- Coupler: 		- Model Coupling Toolkit (MCT) v 2.6.0

Ocean:			- Regional Ocean Modeling System (ROMS) svn 904

Atmosphere:		- Weather Research and Forecasting Model (WRF) v 3.9.1.1

Wave(s):		- Simulating Waves Nearshore (SWAN) v 41.20

- WAVEWATCH III (WW3) v 5.16

- Infragravity wave model (InWave) v 1.0

Sediment Transport:	- Community Sediment Transport Modeling Systems (CSTMS)

Sea Ice:		- Sea Ice model

Here are some model specific user forums that can provide useful information:

- MCT

http://www.mcs.anl.gov/research/projects/mct/

- ROMS

https://www.myroms.org/forum/index.php

- WRF

http://forum.wrfforum.com/

- SWAN

http://swanmodel.sourceforge.net/

WAVEWATCH III

http://polar.ncep.noaa.gov/waves/wavewatch/

The main reference for the COAWST modeling system is:

Warner, J.C., Armstrong, B., He, R., and Zambon, J.B., 2010, Development of a Coupled Ocean-Atmosphere-Wave-Sediment Transport (COAWST) modeling system: Ocean Modeling, v. 35, no. 3, p. 230-244.

The main website is:

http://woodshole.er.usgs.gov/operations/modeling/COAWST/index.html

For bug reports and Discussion, we are currently using the Trac site:

https://coawstmodel-trac.sourcerepo.com/coawstmodel_COAWST/

Please log onto that site to post questions or submit a bug report.

- 2. Accessing COAWST

The code is maintained in a svn repository and is distributed thru subversion checkout. Please contact John Warner for code access at jcwarner@usgs.gov You will then receive an email with your username (myusrname) and password. To obtain the source code via the svn repository, I suggest you make a directory called COAWST, cd to that dir, and use the following command (all one line):

svn checkout --username myusrname https://coawstmodel.sourcerepo.com/coawstmodel/COAWST .

Notice the “.” at the end of the command to place the code in that location. Alternatively instead of the “." you can add any path of where you want it to go.

- 3. Required libraries for model installation

The following libraries are required to run the coupled modeling system. These are:

- Fortran Compiler

- NetCDF

- MPI

- MCT

- SCRIP_COAWST

These libraries/programs only need to be installed once, typically by an administrator, and set to be available for all users.

- Fortran Compiler: The system has been tested with several different fortran compilers, including ifort, pgi, and gfortran. Each compiler is different and the same compiler will have multiple versions. We cannot test them all, but have tried many configurations. You need to use the same fortran compiler to install/compile NetCDF, MPI, MCT, SCRIP_COAWST, and then to compile COAWST itself. Some compilers are free, and some have a cost.

- NetCDF is a common data format that can be freely acquired from

http://www.unidata.ucar.edu/software/netcdf/    The models WRF, ROMS, and SWAN can each individually use NetCDF v3.x with the fortran interface. However, as of COAWST release v3.2, we require NetCDF v4.x for the SCRIP_COAWST interpolation weights package. We therefore require NetCDF v4.x for full model support. You need to set your environment variables so the system knows where to locate the netcdf files. The way to set environment variables is different for each system, so you may need to use “setenv” or “export” or some other command, but an example is listed here:

setenv  NETCDF_INCDIR       /PATH_TO_YOUR_NETCDF/include

setenv  NETCDF_LIBDIR        /PATH_TO_YOUR_NETCDF/lib

setenv  NETCDF                       /PATH_TO_YOUR_NETCDF/

setenv  NETCDF_CONFIG      /PATH_TO_YOUR_NETCDF/nc-config

The settings of NETCDF_INCDIR and NETCDF_LIBDIR are for ROMS. The setting of NETCDF is for WRF. The setting of NETCDF_CONFIG is for WAVEWATCH III.

- MPI is a message passing interface software to allow the models to utilize multiple processors. There are many types and versions and we have tested mvapich, openmpi, and MPICH2. MPI is required to run any coupled (more than 1 model) simulation. Some of these are free.

- MCT is the Model Coupling Toolkit, and is freely available from http://www.mcs.anl.gov/research/projects/mct/. We recommend you use the version provided with this distribution in COAWST/Lib/MCT, as it has been slightly modified and tested to be compatible with COAWST.

- MCT installation

1) Copy the MCT package from COAWST/Lib/MCT and place into a folder that can be shared to all users. It is recommended to only install this once, and just let all users link to this for all their compilations.

2) ./configure

This will create a file Makfile.conf.  You should edit this file and see if the paths are correct. Alos, make sure the compiler flags are the same as needed for your system. For example, if your flags need to use –assume-byterecl (ifort) or -frecord-marker=4 -fconvert=big-endian (gfortran) then add these compiler flags. I included the file Makefile.conf_jcw as an example of how my setup was on a Windows machine.

3) make

This will build the package in both the mct and mpeu directories. I modified the Makefiles in both of those directories so that they can also work in a Windows world. The Windows compilers typically build *.obj files, instead of just *.o files. So I modified the mct/Makefile and mpeu/Makefile for the $(F90RULECPP). You can edit those files and see the changes.

4) make install

This will place the MCT libraries and include files into the locations listed at the bottom of the Makefile.conf file. You can change these locations if you want to, but they typically are something like /usr/lib and /usr/include.

5) Set your environment variables so the system knows where to locate required files. The way to set environment variables is different for each system, so you may need to use “setenv” or “export” or some other command, but an example is listed here:

setenv   MCT_INCDIR   /usr/include

setenv   MCT_LIBDIR   /usr/lib	[or whatever the actual paths are]

- - SCRIP_COAWST

The spherical coordinate remapping interpolation package (SCRIP) is freely available from http://oceans11.lanl.gov/trac/SCRIP. This package is required when you have an application using more than one model, and the models are using different grids (different spatial extents and/or different number of mesh points). This SCRIP code will generate interpolation weights that are used to conservatively remap data fields between the model grids. This original SCRIP package has been modified for COAWST v3.2, and we now require users to compile SCRIP_COAWST as a Fortran executable.  When you run the executable it will generate a single netcdf file the holds all the interpolation weights for any number of grids.  To work with SCRIP_COAWST package, the user needs NetCDF.4.x to have the netcdf group feature available.

SCRIP_COAWST installation

1) Copy the COAWST/Lib/SCRIP_COAWST package and place into a folder that can be shared to all users. It is recommended to only install this once, and just let all users link to this for all their compilations.

2) Edit the makefile to enter the fortran version that is installed, for example, set

FORT = pgi (or whatever compiler you use)

3) type “make” at the command prompt.

4) This should compile to create an executable called scrip_coawst[.exe]

- Section 4. Setting up and running applications

- The COAWST system is designed to allow the user to select any combination of the main models (WRF, ROMS, SWAN, or WAVEWATCH III). All of these main models compile in a different way, but we need to have one consistent approach to compile the complete coupled system. This procedure is based on the method to build the ROMS code, with additional options for each model and the coupled system. The user needs to list:

- - cpp options to select the model(s)  		(Section 4.1)

- - cpp options for each model 			(Section 4.2)

- - cpp options for the coupled system 		(Section 4.3)

This is accomplished by listing the options in a header control file, let’s call it project.h. This project.h file will list cpp (c pre-processor) choices that determine which options in the code are compiled. These options are listed below and some are described further in the test examples section.

- 4.1 CPP options to select the model(s)

These are REQUIRED cpp option(s). This allows the user to activate a single model or multiple models. Specify which model(s) you want to use:

#define ROMS_MODEL    /* if you want to use the ROMS model */

#define SWAN_MODEL   /* if you also want to use the SWAN model */

#define WRF_MODEL      /* if you also want to use the WRF model */

#define WW3_MODEL      /* if you also want to use the WW3 model */

As of COAWST v3.2, the system can run any set of model choices:

WRF only, ROMS only, SWAN only, WW3 only,

WRF+ROMS, WRF+SWAN, ROMS+SWAN, WRF+ROMS+SWAN

WRF+WW3, ROMS+WW3, WRF+ROMS+WW3

We can also run:

- SWAN only with or without grid refinement,

- ROMS only with or without grid refinement,

- WRF only with or without grid refinement (static + moving nest)

- ROMS+SWAN+WRF with grid refinement in all 3 (WRF with static or moving nest). To run with a WRF moving nest, that moving nest needs to be the last child grid of the WRF system.  We currently only support coupling to 1 moving WRF nest.

- ROMS+WW3+WRF: For WW3, we currnently only allow one wave grid. However that WW3 grid can be a different size than any other grids. Also ROMS and WRF can have multiple grids, but only 1 WAVCEWATCHIII grid (for now). You then need to use SCRIP to compute interpolation weights (just as you would for SWAN or any other model).

4.2 CPP options for each model

Each model may have individual options that can be set for compilation. These are described below.

- For WRF: there are no additional cpp options that need to be listed in the project.h file at this time. Most options are set in the namelist.input and determined during the build.

- For WAVEWATCH III: there are no additional cpp options that need to be listed in the project.h file at this time. Most options are set in the WW3 switch file for the build.

- For SWAN: there are no additional options needed to be listed in the project.h file at this time. Most options are set in the INPUT file. However, for completeness, we already preset several options in SWAN and used:

perl switch.pl -unix -impi -mpi -f95  -netcdf *.ftn *.ftn90

This activated MPI routines, unix environments, and netcdf functionality. Users do not need to list anything at this time.

- For ROMS: The user need to become familiar with the vast number of ROMS cpp options that are available. Please look at the ROMS/Inlcude/cppdefs.h and the ROMS wiki pages https://www.myroms.org/wiki/Documentation_Portal that has a lot of documentation for ROMS cpp options. The user needs to select the ROMS cpp options and list them in the project.h file.

4.3 CPP options for the coupled system.

Below are cpp options specific to COAWST.

4.3.1) To activate model coupling:

#define MCT_LIB           /* if you have more than one model selected and you want to couple them.*/

The following cpp options are activated internally. The user should NOT list these in their project.h file.

ROMS_COUPLING – roms is being used and is coupled to another model.

SWAN_COUPLING  – swan is being used and is coupled to another model.

WW3_COUPLING  – wavewatch3 is being used and is coupled to another model.

WRF_COUPLING – wrf is being used and is coupled to another model.

AIR_OCEAN – wrf and roms are active (other models could also be active).

WAVES_OCEAN – swan-or-wavewatch3 and roms are active (other models could also be active).

AIR_WAVES – swan-or-wavewatch3 and wrf are active (other models could also be active).

4.3.2) Some wave-current cpp options that are available for the coupling include:

#define UV_KIRBY        /* compute "surface-average" current based on Hwave that will

be sent from the ocn to the wav model for coupling*/

#define UV_CONST        /* send vel = 0 from the ocn to wave model */

#define ZETA_CONST   /* send zeta = 0 from the ocn to wave model */

#define SST_CONST       /* do not send sst data from roms to wrf  */

4.3.3) Atmosphere coupling cpp option:

- #define ATM2OCN_FLUXES

This option specifies that the heat and momentum fluxes as computed by the atmosphere model will be used in the ocean model. This will allow both models to be using the identically same fluxes at the interface. When using this option, you should also use

- #undef BULK_FLUXES

because the fluxes will be computed by wrf depending on the surface scheme you select.

4.3.4) Methods for grid interpolation.

#define MCT_INTERP_WV2AT   /* this allows grid interpolation between the wave and atmosphere models */

#define MCT_INTERP_OC2AT    /* this allows grid interpolation between the ocean and atmosphere models */

#define MCT_INTERP_OC2WV    /* this allows grid interpolation between the ocean and wave  models */

- If you use different grids for the ocean, atmosphere, or wave models, then you need to activate the appropriate option above so that the data fields are interpolated between grids.

- If you have a coupled application with NESTING in ROMS or SWAN or refinement in WRF, then you need to use the appropriate interpolation flag because all the grids talk to all the other grids (for example  all WRF grids would communicate to all the ocean grids).

- If you need any of these _INTERP_ cpp options, then you need to create interpolation weights using SCRIP_COAWST.  This is described in Section 5 below.

4.3.5) Methods for grid refinement.

#define NESTING  /* this activates grid refinement in both roms and in swan.*/

- ROMS has one-way, two-way, and composed grids nesting.

- SWAN has one-way grid refinement.

- WRF has two way nesting and composed grids.

- As of COAWST v3.2, if you are running a simulation with nesting, the models can all have different number of grids. The test case INLET_TEST_REFINED (explained below) is an example where both roms and swan are coupled, with grid refinement, and coupling on the refined grids.  The test case SANDY is an example with 2 grids for roms, 2 for swan, and 2 for wrf (2 static – or -- 1 static + 1 moving).

- If the grids for ROMS, SWAN, and WRF are different sizes, then the user needs to provide interpolation weights using SCRIP_COAWST for those grid combinations. The number of grids for ROMS is set in the ocean.in file, the number of grids for SWAN set in the swan.in file, and for WRF set in the namelist.input file.

If your simulations have more than 1 model, and at least one of those models has multiple grids, then you need to create the interpolation weights file using the SCRIP_COAWST library. This creates the netcdf weight files that need to be listed in the coupling.in file.

4.3.6) SWAN –or- WAVEWATCH III wave interactions to ROMS or to WRF:

The following 3 options are available to allow exchange of wave data to ROMS for use in bulk_fluxes for computation of ocean surface stress, and to allow exchange of wave data to WRF for use in MYJSFC and MYNN surface layer schemes to allow increased bottom roughness of the atmosphere over the ocean:

#define COARE_TAYLOR_YELLAND

Taylor, P. K., and M. A. Yelland, 2001: The dependence of sea surface roughness on the height and steepness of the waves. J. Phys. Oceanogr., 31, 572-590.

#define COARE_OOST

Oost, W. A., G. J. Komen, C. M. J. Jacobs, and C. van Oort, 2002:New evidence for a relation between wind stress and wave age from measurements during ASGAMAGE. Bound.-Layer Meteor., 103, 409-438.

#define DRENNAN

Drennan, W.M., P.K. Taylor and M.J. Yelland, 2005: Parameterizing the sea surface roughness. J. Phys. Oceanogr. 35, 835-848.

4.3.7) Implementation of wave-current interaction formulation.

We added a new method based on the vortex force approach. The method is described in detail Kumar et al (2012). The new cpp options for this are:

#define WEC_MELLOR                 radiation stress terms from Mellor 08

#define WEC_VF                             wave-current stresses from Uchiyama et al.

#define WDISS_THORGUZA        wave dissipation from Thorton/Guza

#define WDISS_CHURTHOR        wave dissipation from Church/Thorton

#define WDISS_WAVEMOD         wave dissipation from a wave model

#define WDISS_INWAVE              wave dissipation from a InWave model

#define ROLLER_SVENDSEN      wave roller based on Svendsen

#define ROLLER_MONO               wave roller for monchromatic waves

#define ROLLER_RENIERS          wave roller based on Reniers

#define BOTTOM_STREAMING  wave enhanced bottom streaming

#define SURFACE_STREAMING wave enhanced surface streaming

Interested users should read the Kumar et al. (2012) paper.

4.3.8) Drag limiter option.

#define DRAGLIM_DAVIS is a feature added to WRF and SWAN to limit the ocean roughness drag to be a maximum of 2.85E-3, as detailed in:

Davis et al, Prediction of Landfall Hurricanes with the Advanced Hurricane WRF Model, Monthly Weather Review, 136, pp 1990-2005.

In SWAN, this can be activated when using the Komen wind input. For WRF, it can be activated when using myjsfc or mynn surface layer options.

4.3.9) Vegetation options.

Vegetation module was added to get the 3-D effect of vegetation on wave and current fields. Details of the implementation are in this paper:

Beudin, A., Ganju, N., K., Warner, J.C., and Kalra, T. S., “Development of a Coupled Wave-Current-Vegetation Interaction”, Computers and Geosciences Journal – Elsevier (In preparation)

The following flags can be used to work with the vegetation module.

# define VEGETATION                     Switch on vegetation module

# define VEG_DRAG                          Drag terms Luhar M. et.al (2011)

# define VEG_FLEX                           Flexible vegetation terms

# define VEG_TURB                          Turbulence terms, Uittenbogaard R. (2003)

# define VEG_SWAN_COUPLING  Exchange of VEG data btwn. ROMS and SWAN

# define VEG_STREAMING             Wave streaming effects

In addition of calculating the effect of vegetation, the vegetation module can compute wave thrust on the marshes.

# define MARSH_WAVE_THRUST  Wave thrust on marshes, Tonelli, M. et al. (2010)

This code is based on this reference:

“Tonelli, M., Fagherazzi, Sergio., and Petti., M., 2010: Modeling  wave impact on salt marsh boundaries, Journal of Geophysical Research, 115, 0148-0227”

4.3.10) InWave options.

The following cpp options are available to run the InWave model:

# define INWAVE_MODEL          	  activate InWave model

# define INWAVE_SWAN_COUPLING  activate reading of a SWAN 2D spec file

# define DOPPLER                        use to turn ON or OFF the effect of currents on the dispersion relation

# define ACX_ADVECTION       use to turn ON or OFF advection of Ac in the xi direction

# define ACY_ADVECTION       use to turn ON or OFF advection of Ac in the etai direction

# define ACT_ADVECTION       use to turn ON or OFF advection of Ac in the directional direction

# define WDISS_ROELVINK     use to turn ON or OFF Roelvink energy dissipation

# define WDISS_GAMMA           use to turn ON or OFF gamma based energy dissipation

The InWave model is described more in Section 11. For now you can run InWave forced with a SWAN 2d spec file, or you can impose the wave action density along an open boundary. InWave runs coupled to ROMS for wave-current feedbacks. It can also be run with the WEC options, wetting drying, sediment, and morphology.

Section 5. Compiling and Running

This section describes how to:

5.1- How to Compile and Run COAWST

5.2- How to Compile and Run SCRIP_COAWST

5.3- Some general helpful tips and a few new functionalities.

5.1 To Compile and Run COAWST

First, make sure that you read Section 3 and you have selected a Fortran Compiler and then used that compiler to build and install all the required libraries of: NetCDF, MPI, and MCT. Additionally, if your application uses models on different grids and you need to create interpolation weights, then you also need to compile SCRIP_COAWST and run that program to create weights. Section 5.2 describes how to run that program to create the interpolation weights.

To compile COAWST, you need to use the coawst.bash file. This file is at the COAWST root level. You need to edit the coawst.bash to set the options specific to your system and to choose the location of the projects.h file (described in Section 4).  Some important parts are:

export COAWST_APPLICATION=JOE_TC

Use capital letters to list the application name (in this example the application name is JOE_TC). This needs to be the same name as your project.h file that contains all your cpp options (See Section 4). The project.h file itself can be named in lower case (for example, the COAWST_APPLICATION is JOE_TC but the file name is joe_tc.h).

export     MY_ROOT_DIR=/raid1/jcwarner/Models/COAWST

export     MY_PROJECT_DIR=${MY_ROOT_DIR}

The rootdir is the location of the source code. You can have a project dir that you work in. I typically make a new copy of the code each time in case I change something and this means I have multiple rootdirs. Then I can always go back and see exactly the version of code that was used for a particular run. Therefore I set the project dir as the same location as the rootdir.

export        MY_ROMS_SRC=${MY_ROOT_DIR}/

Keep this the same as listed here.

For WAVEWATCH III we now have 5 environment variables:

export   COAWST_WW3_DIR=${MY_ROOT_DIR}/WW3

The COAWST_WW3_DIR is a pointer to root WW3 code, do not change.

export   WWATCH3_NETCDF=NC4

This flag tells WAVEWATCH III to use netcdf 4. do not change.

export   WWATCH_ENV=${COAWST_WW3_DIR}/wwatch.env

This WWATCH_ENV points to WW3 environment listing. do not change.

export   NETCDF_CONFIG=/share/apps/netcdf-4.1.3_intel-2011.4.191/bin/nc-config

(or wherever your nc-config is.).  This is needed to be set by you.

export   WW3_SWITCH_FILE=sandy_coupled

This switch file is used by WAVEWATCHIII.  It is like the roms project.h file. You need to create and set the name of the file (see Section 11 for more information).

There are several compiler selections:

export           USE_MPI=on

export        USE_MPIF90=on

The COAWST system was designed to work with MPI for parallel processor applications. If you set the modeling system to build a coupled application, then it will always produce an executable “coawstM”. If you set to build an individual model and also set MPI=on, then you will get a “coawstM.”

export              FORT=pgi

You can set this to be ifort, pgi, or gfortran. Other choices may work.

Header and other source directories:

export  MY_HEADER_DIR=${MY_PROJECT_DIR}/Projects/JOE_TC/Coupled

export MY_ANALYTICAL_DIR=${MY_PROJECT_DIR}/Projects/JOE_TC/Coupled

Use these to set the locations for you application. Header dir is where your project.h file is located.

End of user defined coawst.bash features.

After you edit the coawst.bash file you need to run it. To (re)build everything you use:

./coawst.bash –j X

(the –j says to use ‘X’ number of processors, uise a number for X, like 2 or 4).

This command will just rebuild any changes to roms or swan, and will rebuild all of wrf and all of ww3:

./coawst.bash –noclean

This command will rebuild all of roms and swan and just the changes that have been made to wrf or to ww3:

./coawst.bash –nocleanwrf –nocleanww3

This will rebuild roms, rebuild swan, rebuild ww3, and rebuild only the wrf files that have been changed.

./coawst.bash –nocleanwrf

If you need to make modifications to the WRF configuration file, here are some tips. you could do this:

cd WRF

./clean -a

./configure

then select the options that you want. It will create the configure.wrf file and you can edit that file. then cd .. (to the root dir) and use

- ./coawst.bash -nocleanwrf

This way it will not re-create the configure.wrf file and it will build all of the system.

- Another handy tool is the script command

http://www-users.cs.umn.edu/~skim/cs1901/script.html

You could use:

script coawst.log

./coawst.bash

exit

and then save that coawst.log file to see how the system was built.

The system may take a while to build. If it was successful, then you will get a coawstM or coawstM.exe file. If you get that file then the build was successful.  In some instances you may see an error or warning “undefined wrf.main.” That is the only error that is ok. We can not allow a wrf.exe to be built, nor an ocean.exe, and/or swan.exe to be created. Only one executable is created, and that is called coawstM[.exe].

To Run COAWST, you need to use a shell script that is dependent on your system. One example of a job script used on a PBS system is the file run_nemo:

#!/bin/bash

### Job name

#PBS -N cwstv3

### Number of nodes

#PBS -l nodes=2:ppn=8

### Mail to user

#PBS -m ae

#PBS -M jcwarner@usgs.gov

#PBS -q standard

umask 0002

echo "this job is running on:"

cat $PBS_NODEFILE

- NPROCS=`wc -l < $PBS_NODEFILE`

cd /raid1/jcwarner/Models/COAWST_regress/COAWST_v3.3

mpirun -np 16 -machinefile $PBS_NODEFILE ./coawstM Projects/JOE_TC/Couped/coupling_joe_tc.in > cwstv3.out

- You need to talk to your system administrator to get a run script for your computer system. Not that the mpirun command calls the coawstM, and the input file is the coupling.in file for this application. This coupling.in file is described below.

- To run/submit the the job, you would use the command

qsub run_nemo

- To check job status

qstat –an

qstat –f

To kill the job use

qdel ‘pid’   where pid is the job number.

On a SLURM system, the commands are squeue, and sbatch.  Please check with your system administrator for specific commands on your system.

Command Run File: coupling.in

To Run COAWST, your run script points to an input file. For example, in the run script above it pointed to coupling_joe_tc.in.  The details of this file are:

Step1)  Set the nodes to allocate for each model. This will depend on the application, number of processors you have access to, etc.

! Number of parallel nodes assigned to each model in the coupled system.

! Their sum must be equal to the total number of processors.

NnodesATM =  1                    ! atmospheric model

NnodesWAV =  1                    ! wave model

NnodesOCN =  1                    ! ocean model

Step 2) Set the coupling interval in seconds.

! Time interval (seconds) between coupling of models.

TI_ATM2WAV =   600.0d0              ! atmosphere to wave coupling interval

TI_ATM2OCN =   600.0d0              ! atmosphere to ocean coupling interval

TI_WAV2ATM =   600.0d0              ! wave to atmosphere coupling interval

TI_WAV2OCN =   600.0d0              ! wave to ocean coupling interval

TI_OCN2WAV =   600.0d0              ! ocean to wave coupling interval

TI_OCN2ATM =   600.0d0              ! ocean to atmosphere coupling interval

Step 3) Enter names of the input files for ROMS, WRF, and SWAN –or- WW3.

! Enter names of Atm, Wav, and Ocn input files.

! The Wav program needs multiple input files, one for each grid.

ATM_name = namelist.input                                                     ! atmospheric model

WAV_name = Projects/JOE_TC/Coupled/INPUT_JOE_TC    ! wave model

OCN_name = Projects/JOE_TC/Coupled/ocean_joe_tc.in        ! ocean model

Step 4) This is evolving, and we now suggest users to use the SCRIP_COAWST Fortran code to create a single weights file. We still allow the older approach of multiple files, but this will go away in future releases.

! Sparse matrix interpolation weights files. You have 2 options:

! Enter "1" for option 1, or "2" for option 2, and then list the

! weight file(s) for that option.

SCRIP_WEIGHT_OPTION = 1

!

! Option 1: IF you set "SCRIP_WEIGHT_OPTION = 1", then enter name

!           of the single netcdf file containing all the exchange

!           weights. This file is created using the code in

!           Lib/SCRIP_COAWST/scrip_coawst[.exe]

SCRIP_COAWST_NAME = Projects/JOE_TC/Coupled/scrip_weights_joe_tc.nc

! Option 2: THIS OPTION WILL BE REMOVED IN FUTURE VERSIONS.

!           IF you set "SCRIP_WEIGHT_OPTION = 2", then enter

!           the names of the separate files. The file names

!           must be provided in a specific order. For example:

! W2ONAME == wav1 to ocn1

!            wav1 to ocn2

!            wav1 to ocn3 ....for all the ocean models.

!            wav2 to ocn1

!            wav2 to ocn2

!            wav2 to ocn3 ....for all the ocean models.

W2ONAME == wav2ocn_weights.nc

W2ANAME == wav2atm_weights.nc

A2ONAME == atm2ocn_weights.nc

A2WNAME == atm2wav_weights.nc

O2ANAME == ocn2atm_weights.nc

O2WNAME == ocn2wav_weights.nc

!

That completes the coupling.in file.  You point to that file to run a coupled application.

5.2- How to Compile and Run SCRIP_COAWST

SCRIP_COAWST is required to create a netcdf file that contains interpolation weights. These weights are only needed if you have a coupled application (more than 1 model) and the models are on different grids. Then you need to compile and run the SCRIP_COAWST and create the weights netcdf file. The SCRIP_COAWST needs to be built first, as described in Section 3. To run the program, you need to edit one of the scrip_coawst*.in files. Lets use

COAWST/Lib/SCRIP_COAWST/scrip_coawst_sandy.in as an example.

This Example uses 2 roms grids, 2 swan grids, and 2 wrf grids with the last wrf grid as a moving grid.

Step 1) Enter name of output netcdf4 file

OUTPUT_NCFILE='scrip_sandy_moving.nc'

!OUTPUT_NCFILE='scrip_sandy_static.nc'    ( this line is a comment)

Step 2) Enter total number of ROMS, SWAN, and WRF (max_dom) grids:

NGRIDS_ROMS=2,

NGRIDS_SWAN=2,

NGRIDS_WRF=2,

NGRIDS_WW3=0,

Step 3) Enter name of the ROMS grid file(s):

ROMS_GRIDS(1)='../../Projects/Sandy/Sandy_roms_grid.nc',

ROMS_GRIDS(2)='../../Projects/Sandy/Sandy_roms_grid_ref3.nc',

Step 4) Enter SWAN information:

!    -the name(s) of the SWAN grid file(s) for coords and bathy.

!    -the size of the SWAN grids, and

!    -if the swan grids are Spherical(set cartesian=0) or

!                           Cartesian(set cartesian=1).

SWAN_COORD(1)='../../Projects/Sandy/Sandy_swan_coord.grd',

SWAN_COORD(2)='../../Projects/Sandy/Sandy_swan_coord_ref3.grd',

SWAN_BATH(1)='../../Projects/Sandy/Sandy_swan_bathy.bot',

SWAN_BATH(2)='../../Projects/Sandy/Sandy_swan_bathy_ref3.bot',

SWAN_NUMX(1)=84,

SWAN_NUMX(2)=116,

SWAN_NUMY(1)=64,

SWAN_NUMY(2)=86,

CARTESIAN(1)=0,

CARTESIAN(2)=0,

! 5) Enter WW3 information

!    -the name(s) of the WW3 grid file(s) for x- y- coords and bathy.

!    -the size of the WW3 grids (full number of grid center points).

!

WW3_XCOORD(1)='../../Projects/Sandy/ww3_sandy_xcoord.dat',

WW3_YCOORD(1)='../../Projects/Sandy/ww3_sandy_ycoord.dat',

WW3_BATH(1)='../../Projects/Sandy/ww3_sandy_bathy.bot',

WW3_NUMX(1)=84,

WW3_NUMY(1)=64,

Step 6) Enter the name of the WRF input grid(s). If the grid is a

!    moving child nest then enter that grid name as 'moving'.

!    Also provide the grid ratio, this is used for a moving nest.

WRF_GRIDS(1)='../../Projects/Sandy/wrfinput_d01',

!WRF_GRIDS(2)='../../Projects/Sandy/wrfinput_d02',    !this is a comment line

WRF_GRIDS(2)='moving',

PARENT_GRID_RATIO(1)=1,

PARENT_GRID_RATIO(2)=3,

PARENT_ID(1)=0

PARENT_ID(2)=1

Step 7): at the command prompt, run the program as:

./scrip_coawst[.exe] scrip_coawst_sandy.in

This should run and write out information about the weights as it creates the file.

- 5.3 Some general helpful tips and a few new functionalities.

5.3.1) ROMS is now very sensitive to the vertical coordinate system. The user needs to make sure that the values set in the roms init file are the same as the values set in the ocean.in file. For example the following values are checked for consistency:

Vtransform=1;

Vstretching=1;

theta_s=5.0;

theta_b=0.4;

Tcline=50.0;

- N=16;

User should check that these values are the same in their netcdf files as the *.in file. More info is provided at: https://www.myroms.org/wiki/index.php/Vertical_S-coordinate.

5.3.2) ROMS grid files should not have fill values for mask arrays. A command to remove these are:

ncatted -O -a _FillValue,mask_rho,d,, -a _FillValue,mask_u,d,, -a _FillValue,mask_v,d,, -a _FillValue,mask_psi,d,, USeast_grd17.nc

5.3.3) For SWAN, you do not need to specify any OUTPUT commands for the coupling. The coupling and OUTPUT are now completely separate. SWAN now offers writing out of files in netcdf. This option is available with the COAWST format.

5.3.4) For WRF, you can have a 2-way nest in WRF, and have this coupled to roms and /or swan. As of COAWST v3.2, we can now couple ROMS and SWAN to a moving WRF nest. There can only be 1 moving WRF nest, and it needs to be the last WRF child grid.

5.3.5) For WRF-ROMS coupling, you really should set

sst_update = 1

in namelist.input and use the appropriate io_form_auxinput4 settings. This is correct in svn 876. The sst_update computes the correct TSK in WRF and should be activated for ocn-atm coupling.

5.3.6) Some information about heat fluxes for WRF-ROMS.

If WRF_MODEL is defined:

- you still have to define EMINUSP to activate exchange of rain and evap.

- SOLAR_SOURCE is needed, otherwise all the heat goes into the surface layer only.

- longwave outgoing component is estimated in Master/mct_roms_wrf.f so there is no need to define LONGWAVE_OUT in ROMS

If WRF_MODEL is not defined or you are going to use BULK FLUXES:

BULK_FLUXES (in bulk_flux.F) computes turbulent heat fluxes (latent and sensible heat), momentum stress and evaporation (used in the fresh water flux if EMINUPS is also defined -used as salinity surface boundary condition-). Radiative fluxes (i.e., shortwave and longwave radiation flux) are not calculated, nor is the rain component of the EMINUSP calculation. The surface scheme (COARE)  implemented in bulk_flux.F requires:

- air temperature (usually at 2m)

- relative humidity (usually at 2m)

- mean sea level pressure

- u-wind component (positive east), usually at 10m v-wind component (positive north), usually at 10m.

With these parameters bulk_flux will estimate latent heat, sensible heat, u-momentum stress, v-momentum stress and evaporation. Note that in the ocean.in, you have to specify:

BLK_ZQ (usually 2m)

BLK_ZT (usually 2m)

BLK_ZW (usually 10m)

these numbers should be consistent with the height of the levels of the surface variables (as said usually wind is at 10m, air temp at 2m, humidity at 2m, but this might be different depending on your surface forcing dataset).

Net shortwave should be provided by you meteo forcing. This is not calculated in bulk_flux.f, but is necessary to compute the full heat flux term.

Net longwave: you have several choices:

- provide net longwave in the forcing file

- provide INCOMING longwave in the forcing file and define  LONGWAVE_OUT (ROMS then will estimate the outgoing component based on its SST)

- do not provide the longwave but instead total cloud cover (in the forcing file) and ROMS will estimate the net longwave. You do not need to define CLOUD, as it is defined internally by ROMS if def LONGWAVE

If you want the E-P flux, define EMINUSP and provide in the forcing file the variable rain, while, as said, evaporation is estimated in bulk_flux.F.

So, in the end:

- #define BULK_FLUXES

#define LONGWAVE or #define LONGWAVE_OUT or provide the net longwave

in the forcing file

#define EMINUSP is needed, otherwise set to zero the surface salinity fux (#define ANA_SSFLUX and set zero  stflx(itrc==isalt) in stflux.h)

#define ATM_PRESS if you want the inverted barometric effect (mean sea level pressure must be in the forcing file)

5.3.7)  SWAN restart files.

SWAN has the command 'HOTFILE' that is used to write out the model state of two-dimensional spectra to a file at the end of the simulation. This file can be used as a restart for a future simulation. We have recently added the option to have SWAN create restart files during the simulation. This can be useful for a coupled simulation (for example a roms + swan run). If the modeling system reaches a fault during the run, the ROMS model can be restarted with the ROMS rst or his file. Now you can also restart the SWAN model with a restart file for that same instance in time.  You need to add the command "RESTART'. For example, the swan.in file would need:

- RESTART 'swan_intest_rst.dat' FREE 1 HR

- COMPUTE NONSTATIONARY 20000101.000000 60 SEC 20000101.120000

STOP

With this set of commands, swan is going to run from Jan 1, 2000, at hour 0 to Jan 1, 2000 to hour 12, with a 60 sec time step.  The 'RESTART' command will update a file called swan_inlet_rst.dat every 1 hour during that simulation.  You can make this be any time you require (1 HR or 30 MIN or 12 HR or whatever). It is recommended that the user set the restart times to be the same for WRF, ROMS, and SWAN so that a consistent set of restart files are available.

- 6. Distributed Projects/examples.

| Application |  |
| --- | --- |
| 6.1 Ducknc | ROMS only. Wave-current interaction (essentially x-z) for cross-shore flows at Duck, NC. |
| 6.2 Estuary_test2 | ROMS only to test estuarine dynamics, prismatic channel. |
| 6.3 Inlet_test/Coupled | Idealized inlet with wave and tidal driven flows. ROMS+SWAN same grids. |
| 6.4 Inlet_test/DiffGrid | Idealized inlet with wave and tidal driven flows. ROMS+SWAN different grids. |
| 6.5 Inlet_test/InWave | Idealized inlet with wave and tidal driven flows. ROMS+InWave same grids, inlet configuration with infragravity waves. |
| 6.6 Inlet_test/Refined | Idealized inlet with wave and tidal driven flows. ROMS+SWAN have the same parent grids for roms and swan and the same child grids for roms and swan. |
| 6.7 Inlet_test/Swanonly | Idealized inlet with waves only. SWAN only one grid, or with grid refinement. |
| 6.8 InWave_Shoreface | ROMS+InWave same grid, sloping beach. |
| 6.9 JOE_TC/Coupled | Idealized tropical cyclone that travels west from a deep ocean basin onto a shelf that slopes landward to the west. WRF+ROMS+SWAN all 3 on same grid. Also can be used for WRF-ROMS, WRF-SWAN, or WRF only. |
| 6.10 JOE_TC/DiffGrid | Idealized tropical cyclone that travels west from a deep ocean basin onto a shelf that slopes landward to the west. WRF+ROMS+SWAN with ROMS and SWAN on same grid, WRF on different grid. |
| 6.11 Rip_current | ROMS+SWAN same grid, idealized rip current. |
| 6.12 Sandy | WRF + ROMS + SWAN each with 2 grids, WRF can be static or moving nest, coarse resolution of realistic Hurricane Sandy simulation. Also distributed as WRF + ROMS + WW3 each with one grid. |
| 6.13 Sed_floc_toy | ROMS only, tests sediment module with flocculation. |
| 6.14 Sedbed_toy | ROMS only, tests sediment module. |
| 6.15 Trench | ROMS only, test sediment morphology. |
| 6.16 Veg_test | ROMS+SWAN, test vegetation module. |
| 6.17 Wetdry | ROMS only, test wetting/drying algorithms. |

All of the test cases are described below, some in more detail than others. The cases of JOE_TC, Inlet_test, Rip current and Vegetation are examples of how to set up different idealized configurations. The test case Sandy is provided for a more realistic application. The references for the test cases are provided in Section 15.

- 6.1 Ducknc = ROMS only, Wave-current interaction (essentially x-z) for cross-shore flows at Duck, NC

- This test case validates the implementation of the Wave effect on Currents (WEC) module in the COAWST framework. The case is based upon experriemnts from the Duck (94) experiment. The experiment includes oblique waves that are incident on a synthetic planar beach and a natural barred beach. The simulations show the capability of the WEC module in COAWST to capture nonlinear processes of wave roller generation and wave-induced mixing. The details of the test case are presented in Kumar et al. (2013). To setup the case modify the coawst.bash to include this application:

export   ROMS_APPLICATION=DUCKNC

- Figures 6.1(a) and 6.1(b) show the vertical velocity and vertical stokes velocity respectively varying with cross-shore width (x) at a middle along-shore plane (y=4) at the end of 8400 seconds.

- (a) Vertical velocity                          (b) Vertical stokes velocity

- Figure 6.1: Results from Ducknc test case

- 6.2 Estuary_test2 = ROMS only, to test estuarine dynamics, prismatic channel

- This application tests the ROMS for a prismatic shaped estuary that is 110 km long with the ocean end being 60 kms wide. The width of the estuary decreases exponentially from 60 km to 2 kms until 80 kms and stays uniform beyond 80 kms until the river end. The vertical layers follow a gaussian shape and consist of 40 vertical levels.  The simulation is run for 5 days.

- The application is run by editing the coawst.bash file to set

export   ROMS_APPLICATION=ESTUARY_TEST2

This case requires mpi and the MCT libraries because it is a coupled application. And the run files are at:

export     MY_HEADER_DIR=${MY_PROJECT_DIR}/Projects/Estuary_test2

export MY_ANALYTICAL_DIR=${MY_PROJECT_DIR}/Projects/Estuary_test2

- Estuary grid

- b) Salt intrusion length after 5 days

- Figure 6.2: Results from Estuary_test2 test case

- The above figures (6.2a) and (6.2b) show the horizontal grid describing the shape of the estuary along with the salt intrusion at the end of 30 days.

- 6.3.  INLET_TEST/Coupled = ROMS+SWAN same grid

This application tests the MCT coupling between ROMS and SWAN for an idealized inlet. The application has an enclosed basin in the southern part of the domain with a small opening near the center of the grid. The model is driven by a sinusoidal water level on the northern edge and is coupled to SWAN which develops waves from the north with a 1m height propagating towards the south.  Flow pattern that developed are shown below:

Wave height. 					depth integrated current to wave model

(Hwave)					(vWave)

- Figure 6.3. Results for Inlet_test coupled.

All of the grids and input files have already been created for this application and are available in Projects/Inlet_test/Coupled. Here is a basic set of steps required to make these files and run this application.

Step 1: Use create_roms_xygrid.m to create the roms and swan grids.  These are simple rectangular grids and can be created easily with these tools. User sets the x/y locations, depth, and masking. This m file calls mat2roms_mw to create the roms grid file called inlet_test_grid.nc and calls roms2swan to create the swan grid files. The swan grid files were renamed to inlet_test_grid_coord.grd and inlet_test_bathy.bot.

Step 2: For this case, we are initializing the roms model from rest, so we are relying on ANA_INITIAL default values.  The m file create_roms_init could be used to create a more complicated set of init conditions. We are also using ana_sediment to create a uniform field of 10m thick bed with porosity=0.5 and 1 grain size. This could have been created with the create_roms_init file as well. The file sediment_inlet_test.in lists some sediment properties for this application.

Step 3: For this case we are initializing SWAN from rest so there are no swan init files. we could have run swan for with some wind field, but this case has no winds.

Step 4: create the header file: Edit inlet_test.h to see the options selected for this application. we needed to define

#define ROMS_MODEL

#define SWAN_MODEL

#define MCT_LIB

, as well as several boundary conditions and wave current interaction (WEC) options, the wave-enhanced bottom boundary layer option (SSW_BBL), GLS_MIXING, and some sediment options of suspended load and bed morphology updating.

Step 5: Determine settings for roms and swan in the ocean_inlet_test.in and swan_inlet_test.in. These are some standard options in these files.

Step 6: Create the coupling file called coupling_inlet_test.in to enter the roms and swan input file names and determine the coupling interval.

- Step 7: edit the coawst.bash file to build this application.

- Step 8: compile and run.

- 6.4. INLET_TEST/DiffGrid = ROMS+SWAN different grids

This test case has a grid for ROMS and a larger grid for SWAN. This is useful for applications where the lateral shadow effects of the wave model can influence the ocean grid. So the wave model can be simulated on a larger grid and the ocean model on a smaller inset grid. This application is very similar to case 4 above i.e. inlet_test/coupled. All of the files generated for this test case are in Projects/Inlet_test/DiffGrid.

Step 1: ROMS and SWAN are on different grids. You need to create a roms grid and a swan grid. This is achieved using create_roms_xygrid.m.

Step 2:  Create the interpolation weights file – (The interpolation weight file for this case is already provided right now in the folder as “scrip_inlet_test_diffgrid.nc”). Follow the steps below to create the interpolation weight file yourself.

Build SCRIP_COAWST

We will use the SCRIP_COAWST package provided in “Lib” folder to create the interpolation weights. The steps for this are described in Section 3 above.

- Create the interpolation weights file -

The interpolation weight file for this case is already provided right now in the folder as “scrip_inlet_test_diffgrid.nc”.  But here are the steps to create it.

- Edit the flie Lib/SRIP_COAWST/scrip_coawst_inlet_test_diffgrid.in. This is the input file and has already been created for this application. The information has already been set for the output_ncfile name, the number of grids for each model, and the roms and swan grid information.

- Execute the scrip_coawst by going to the path of the SCRIP_COAWST folder and then running the program using

./scrip_coawst scrip_coawst_inlet_test_diffgrid.in

This will create the weights file scrip_inlet_test_diffgrid.nc

You can then copy that weights nc file to the Project folder

cp scrip_inlet_test_diffgrid.nc ../../Projects/Inlet_test/Diffgrid

- Step 3: Edit the Projects/Inlet_test/Diffgrid/coupling_inlet_diffgrid.in and use SCRIP_WEIGHT_OPTION=1

Add the lines:

- SCRIP_COAWST_NAME=Projects/Inlet_test/DiffGrid/scrip_inlet_test_diffgrid.nc

Step 4: Edit the ocean model configuration file.

Edit Projects/Inlet_test/DiffGrid/inlet_test.h and add the option

#define MCT_INTERP_OC2WV

This will allow mct interpolation between the ocean and wave models.

Step 5: Change the ocean and wave input file names to be

WAV_name = Projects/Inlet_test/DiffGrid/swan_inlet_test_diff.in ! wave model

OCN_name = Projects/Inlet_test/DiffGrid/ocean_inlet_test.in     ! ocean model

- Step 6: make, run the program

- 6.5. INLET_TEST/InWave = ROMS+InWave, Inlet test case.

This case is an example to use the InWave infragravity wave model. The application is an inlet with an enclosed back basin, with offshore open boundaries, similar to the other inlet tests but with a smaller domain size. The offshore northern boundary is driven by a 2Dspec file from SWAN. The model reads that 2d spec file, generates the envelope of the infragravity waves, and propagates them into the domain. (The only other way to drive InWave is to prescribe the wave action density along the open boundary, and that is described with the Projects/InWave_shoreface test case.)

To generate the files for this inlet_test case, you need to edit Tools/mfiles/inwave_tools/master_InWave_create_files.m. This is the main driver to create files for the InWave applications. All of the files generated for this test case are placed in Projects/Inlet_test/InWave.

Step 1: Edit Tools/mfiles/inwave_tools/master_InWave_create_files.m and select to setup the Inlet_test case

%1) SET THE CASE TO = 1.

INWAVE_SHOREFACE=0;

INLET_TEST=1;

This will tell the program to look at a configuration file called

elseif (INLET_TEST)

inwave_gen_file='InWave_inlet_test_param';

So the goal here is to create the InWave_inlet_test_param file, then just run the master m file.

Step 2:

Create (edit) the file Tools/mfiles/inwave_tools/InWave_inlet_test_param.m. Follow the instructions in that m file. We need 3 netcdf files: grid, initial, and boundary files. For this test case we need a grid and an initial file. We do not need to create a boundary file because this case reads in a 2dspec file and will internally compute the wave energy. So we selected:

make_InWave_grd=1;

make_InWave_ini=1;

make_InWave_bry=0;

To make the grid, we set up some basic paramters for dx, dy, x, y, depth, angle, masking, f, and spherical. See the m file.

For the initial conditions, we need to set the action density (Ac) and wave celerities (cx, cy, ct). We just start them all at 0. You also need to specify the number of computational bins (Nbins == ND in the ocean.in file), the directions of these bins (direction the wave is coming from), and a representative period (Ta).

Step 3:

Edit the inlet_test.h file, to add InWave cpp options:

#ifdef  INWAVE_MODEL

# define INWAVE_SWAN_COUPLING

# define ACX_ADVECTION

# define ACY_ADVECTION

# define ACT_ADVECTION

# undef  DOPPLER

# define WDISS_GAMMA

# undef  WDISS_ROELVINK

#endif

The “INWAVE_SWAN_COUPLING” says that it will read a 2d spec file from swan. The AC*_ADVECTION activate the advective terms (like UV_ADV for ROMS). The DOPPLER could be activated, and adds the effects of currents to the celerities. Then there are two choics for wave dissipation – a gamma approach and that of Roelvink.

Step 4:

Edit the Projects/Inlet_test/InWave/ocean_inlet_test.in

The key new options here would be to set

-1) The number of wave directional bins for the computation. This needs to be the same as in the init file

ND == 20            ! Number of wave directional bins

- 2) Inwave boundary conditions:

! InWave boundary conditions

!                                west    south    east    north

LBC(isAC3d) ==   Gra     Clo     Gra     Cla         ! 3D wave action density

LBC(isCT3d) ==   Gra     Clo     Gra     Gra         ! 3D wave theta celerity

LBC(isCX3d) ==   Gra     Clo     Gra     Gra         ! 3D wave x-dir celerity

LBC(isCY3d) ==   Gra     Clo     Gra     Gra         ! 3D wave y-dir celerity

For this case, we want the 2dspec to be applied on the north (isAC3d is Clamped).

The others can be gradient, or closed (on the south as that is a wall).

- 3) Set output or parameters

Hout(idACen) == T       ! AC                 3D wave action

Hout(idACcx) == T       ! Cx                 2D Group velocity in xi dimension

Hout(idACcy) == T       ! Cy                 2D Group velocity in etai dimension

Hout(idACct) == T       ! Ct                 2D Group velocity in the directional dimension

Hout(idACtp) == T       ! Tp                 2D Group peak period

- 4) enter the grid and ini files:

GRDNAME == Projects/Inlet_test/InWave/InWave_inlet_test_grd.nc

(this is the same as the roms grid)

IWININAME == Projects/Inlet_test/InWave/InWave_inlet_test_ini.nc

( this is the ini file created with AC, ct, cx, cy, Ta).

Step 5: edit coawst.bash and build this as any other application.  Run as any other application.  An example of the output is shown here for the wave height.

Figure 6.5. Significant wave hegith at 180s for the InWave inlet test.

- 6.6. INLET_TEST/Refined = ROMS+SWAN same grid + grid refinement in each.

This test case has a coarse and fine grid for ROMS, and a coarse and fine grid for SWAN, run coupled on both grids. All of the files generated for this test case are in Projects/Inlet_test/Refined.

Step1: You need to create a roms grid for the coarse model, roms grid for fine model, swan grid for coarse model, and swan grid for fine model. Suggest you do this:

-  First create a roms grid for the coarse model. This was done using Tools/mfiles/create_roms_grid and selecting the inlet_test case.

-  To create the refined grid, use the following procedure:

F=coarse2fine('inlet_test_grid.nc','inlet_test_grid_ref5.nc',3,24,54,40,56);

Gnames={'inlet_test_grid.nc','inlet_test_grid_ref3.nc'}

[S,G]=contact(Gnames,'inlet_test_contact_ref3.nc');

This uses m files from Rutgers and they are in the Tools directory. These calls set a refinement ratio of 3 and set the starting and ending indices of the child grid to range from Istr=24 to Iend =54, and Jstr = 40 to Jend =56. When you run those 3 commands above a series of figures are created that allow you to see the parent and child grids. The inlet test grids and the contact files need to be listed in the ocean.in file ocean_inlet_test_ref3.in.

-  To create the two swan grids, you can use Tools/mfiles/roms2swan.m from the 2 roms grids.

Step 2: Create input files for the ocean and the swan grids. You need one ocean.in file, with the parameter values repeated for the 2 grids. See Projects/Inlet_test/Refined/ocean_inlet_test_ref3.in.  Edit this ocean*.in and set values for all Numgrids (ie dt, file names, etc.)

- For Lm and Mm the values are  -2 of the total number of rho points.

Step 3: You need to create to separate SWAN INPUT files. See that same folder for the 2 swan files (swan_inlet_test.in and swan_inlet_test_ref3.in).

- Step 4: Need to create init and bndry files for the largest scale ocean and wave grids.

Step 5: Need to create init file for each ocean and wave refined child grids. The child grids do not need boundary files.

Step 6: Edit the Projects/Inlet_test/Refined/coupling_inlet_test_refined3.in file and list the input files: swan has two, just one for roms.

Step 7:  Create the interpolation weights file – (The interpolation weight file for this case is already provided right now in the folder as “scrip_inlet_test_refined.nc”). Follow the steps below to create the interpolation weight file yourself.

Build SCRIP_COAWST

We will use the SCRIP_COAWST package provided in “Lib” folder to create the interpolation weights. The steps for this are described in Section 3 above.

- Create the interpolation weights file -

The interpolation weight file for this case is already provided right now in the folder as “scrip_inlet_test_refined.nc”.  But here are the steps to create it.

- Edit the flie Lib/SRIP_COAWST/scrip_coawst_inlet_test_refined.in. This is the input file and has already been created for this application. The information has already been set for the output_ncfile name, the number of grids for each model, and the roms and swan grid information.

- Execute the scrip_coawst by going to the path of the SCRIP_COAWST folder and then running the program using

./scrip_coawst scrip_coawst_inlet_test_refined.in

This will create the weights file scrip_inlet_test_refined.nc

You can then copy that weights nc file to the Project folder

cp scrip_inlet_test_refined.nc ../../Projects/Inlet_test/Diffgrid

- Step 8: Edit the Projects/Inlet_test/Refined/coupling_inlet_test_ref3.in and use SCRIP_WEIGHT_OPTION=1

Add the lines:

- SCRIP_COAWST_NAME=Projects/Inlet_test/Refined/scrip_inlet_test_refined.nc

Step 9: Build the model

./coawst.bash –j

and run it using

mpiexec –np 2 ./coawstM.exe Projects/Inlet_test/Refined/coupling_inlet_test_refined3.in

- 6.7. INLET_TEST/Swanonly = SWAN by itself, also with grid refinement.

This test case is basically the swan files from case 7 (mentioned above), but allows you to run swan by itself, and with a child grid.

- To run SWAN by itself, with only 1 grid, the header file should have

#define SWAN_MODEL

#undef NESTING

./coawst.bash to compile.

One note is that this needs to be run with the command pointing to the swan input file(s). So to run the swan only with just one grid, run the model with (X can be any number of processors)

mpiexec –np X ./coawstM.exe Projects/Inlet_test/Swanonly/swan_inlet_test.in

- To run SWAN by itself and with a child grid, the header file should have

#define SWAN_MODEL

#define NESTING

./coawst.bash to compile.

The command line needs to point to the swan input files. So to run the swan only with a parent and a child use (X can be any number of processors)

mpiexec –np 1 ./coawstM.exe Projects/Inlet_test/Swanonly/swan_inlet_test.in

Projects/Inlet_test/Swanonly/swan_inlet_test_refined5.in

(all that is on one line, the text is just word wrapping here).

- 6.8 Inwave_shoreface = ROMS+InWave (Sloping beach).

This case is an example to use the InWave infragravity wave model. The application is an inlet with a simple sloping beach in the north-south direction, eastern wall, open western coast. The south, north, and western boundaries are driven by wave action density from a netcdf file. This test case is advanced in that the user needs to understand the physics to create the wave actions time series. The other way to drive InWave is to provide a SWAN 2d spec file and that is described with the Projects/Inlet_test/InWave test case.)

To generate the files for this inlet_test case, you need to edit Tools/mfiles/inwave_tools/master_InWave_create_files.m. This is the main driver to create files for the InWave applications. All of the files generated for this test case are placed in Projects/Inlet_test/InWave.

Step 1: Edit Tools/mfiles/inwave_tools/master_InWave_create_files.m and select to setup the Inlet_test case

%1) SET THE CASE TO = 1.

INWAVE_SHOREFACE=1;

INLET_TEST=0;

This will tell the program to look at a configuration file called

if (INWAVE_SHOREFACE)

inwave_gen_file='InWave_shoreface_param';

So the goal here is to create the InWave_shoreface_param file, then just run the master m file.

Step 2:

Create (edit) the file Tools/mfiles/inwave_tools/InWave_shoreface_param.m. Follow the instructions in that m file. We need 3 netcdf files: grid, initial, and boundary files. For this test case we need all 3 – grid, initial, and boundary files so we selected:

make_InWave_grd=1;

make_InWave_ini=1;

make_InWave_bry=1;

To make the grid, we set up some basic paramters for dx, dy, x, y, depth, angle, masking, f, and spherical. See the m file.

For the initial conditions, we need to set the action density (Ac) and wave celerities (cx, cy, ct). We just start them all at 0. You also need to specify the number of computational bins (Nbins), the directions of these bins (direction the wave is coming from), and a representative period (Ta).

To create the boundary forincg file, the user needs to create the time series of wave action density. This is a user defined option.

Step 3:

Edit the Projects/inwave_shoreface.h file, to add InWave cpp options:

#ifdef  INWAVE_MODEL

# undef  INWAVE_SWAN_COUPLING

# define ACX_ADVECTION

# define ACY_ADVECTION

# define ACT_ADVECTION

# undef  DOPPLER

# define WDISS_GAMMA

# undef  WDISS_ROELVINK

#endif

The “INWAVE_SWAN_COUPLING” is to read s 2d spec file from swan, and we are not doing that for this application. The AC*_ADVECTION activate the advective terms (like UV_ADV for ROMS). The DOPPLER could be activated, and adds the effects of currents to the celerities. Then there are two choics for wave dissipation – a gamma approach and that of Roelvink.

Step 4:

Edit the Projects/InWave_shoreface/ocean_inwave_shoreface.in

The key new options here would be to set

-1) The number of wave directional bins for the computation. This needs to be the same as in the init file

ND == 11            ! Number of wave directional bins

- 2) Inwave boundary conditions:

! InWave boundary conditions

!                                west    south    east    north

LBC(isAC3d) ==   Cla     Gra     Clo     Gra         ! 3D wave action density

LBC(isCT3d) ==   Gra     Gra     Clo     Gra         ! 3D wave theta celerity

LBC(isCX3d) ==   Gra     Gra     Clo     Gra         ! 3D wave x-dir celerity

LBC(isCY3d) ==   Gra     Gra     Clo     Gra         ! 3D wave y-dir celerity

For this case, we want the wave action to be applied on the west (isAC3d is Clamped).

The others can be gradient, or closed (on the east as that is a wall).

- 3) Set output or parameters

Hout(idACen) == T       ! AC                 3D wave action

Hout(idACcx) == T       ! Cx                 2D Group velocity in xi dimension

Hout(idACcy) == T       ! Cy                 2D Group velocity in etai dimension

Hout(idACct) == T       ! Ct                 2D Group velocity in the directional dimension

Hout(idACtp) == T       ! Tp                 2D Group peak period

- 4) enter the grid and ini files:

GRDNAME == Projects/InWave_shoreface/InWave_shoreface_grd.nc

(this is the same as the roms grid)

IWININAME == Projects/InWave_shoreface/InWave_shoreface_ini.nc

(this is the ini file created with AC, ct, cx, cy, Ta).

Step 5: edit coawst.bash and build this as any other application.  Run as any other application.  An example of the output is shown here for the wave height.

Figure 6.8. Significant wave hegith at 200s for the InWave_shoreface test.

- 6.9 JOE_TC/Coupled = WRF-ROMS-SWAN all 3 on same grid.

Step 1: Create a folder to hold all the project files. For this application a folder is already made and is called COAWST/Projects/JOE_TC/Coupled.  This will be referred to as the ‘project folder.’

Step 2: Create WRF grid, init file, and input files:

wrfbdy_d01

wrfinput_d01

namelist.input

These files need to be placed in the root_dir.

Copies of these files have already been created and are located in the Projects/JOE_TC folder. Copy these 3 files from that folder to the root dir.

Edit namelist.input and set in the &domains section the number of processors for WRF to use for the simulation (total WRF = nproc_x*nproc_y).

nproc_x                         = M

nproc_y                         = N

step 3: Create a project  ‘header’ file.

The application control file has already been created for this setup.  For this case, we can use the file in the project directory

COAWST/Projects/JOE_TC/Coupled/joe_tc.h

We have preselected to use a certain case G, which has the 3 way coupling.

Some key parameters in here are ….

# define ROMS_MODEL

# define SWAN_MODEL

# define WRF_MODEL

If you want to create your own, the best choice is to start out by copying an ‘application.h’ file from some other project folder which is similar to your configuration, and then tailor it to the new application.

Step 4: Build the system.

Edit coawst.bash and provide appropriate file locations and run the build command:

./coawst.bash

During the build if wrf asks for the location of the netcdf files, you can provide them. If you set NETCDF=___  to the correct location in your environment, then WRF will know where the files are.

During the buld wrf will ask for the system you are using such as:

Select option 3, PC Linux x86_64, PGI compiler 5.2 and higher (RSL_LITE)

And select 1 for normal. Compiling wrf can take up to 20 minutes or longer.

Step 5: Create the ocean grid. The project folder already contains the roms grid. This grid was made from a version of :

COAWST/Tools/mfiles/create_roms_grid.m.

Alternatively, we now distribute wrf2roms_mw.m that can be used to create a roms grid from a wrf grid. After creating the grid, copy the grid file (in this case joe_tc_grd.nc) to the project directory.

Step 6: Create the initial file for the ocean model. The project folder already contains the roms init file but if you want to see how it was made you can use: COAWST/Tools/mfiles/create_roms_init.m.

After creating the init file copy the file (in this case joe_tc_ocean_init.nc) to the project directory.

Step 7: Create an ocean input file.

The application ocean input file has already been created and we can use the file in the project directory

COAWST/Projects/JOE_TC/Coupled/ocean_joe_tc.in

Set the total number of processors to be allocated to roms as:

NtileI == 4                              ! I-direction partition

NtileJ == 3                              ! J-direction partition

Step 8: Make the swan bathy + grid files.

When you use create_roms_grid, there were other files created:

roms_bathy.bot

grid_coord.grd

These were created by roms2swan.m and these are the swan files needed. Rename these (such as joe_tc_roms_bathy.bot and joe_tc_grid_coord.grd) and copy these 2 files to the project directory. (This has already been done.)

- Step 9: create swan input file

The best step here is to copy an existing file and tailor it to your application. For now we can use:

COAWST/Projects/JOE_TC/Coupled/INPUT_JOE_TC

Step 10: modify the coupling.in file COAWST/Projects/JOE_TC/Coupled /coupling_joe_tc.in to:

- allocate processors for all 3 models

- set coupling time interval

- list input file names

NnodesATM =  4                    ! atmospheric model

NnodesWAV = 4                    ! wave model

NnodesOCN =  4                    ! ocean model

! Time interval (seconds) between coupling of models.

TI_ATM_WAV =   600.0d0              ! atmosphere-wave coupling interval

TI_ATM_OCN =    600.0d0              ! atmosphere-ocean coupling interval

TI_WAV_OCN =   600.0d0              ! wave-ocean coupling interval

! Coupled model standard input file name.

ATM_name = namelist.input                           ! atmospheric model

WAV_name = Projects/JOE_TCs/INPUT_JOE_TC             ! wave model

OCN_name = Projects/JOE_TCs/ocean_joe_tc.in          ! ocean model

- Step 11: run the system using a PBS run script or whatever is for your system

/usr/local/mpi/bin/mpirun -np 12 -machinefile $PBS_NODEFILE ./coawstM Projects/JOE_TC/coupling_joe_tc.in > joe_tc.out

- A) PSFC from WRF at hour 40.

- B) SST from ROMS in the WRF output at hour 40.

- C)  Hwave from SWAN in the WRF output at hour 40.

Figure 6.9. Results at hour 40 for JOE_TCs simulation for: A) PSFC from WRF; B) SST from ROMS; and C) HWAVE from SWAN.

Additional notes for JOE_TCS:

This test case is distributed with "ExpG" defined. That case has ROMS+WRF+SWAN all coupled. You can select to activate a different case in the Projects/JOE_TC/Coupled /joe_tc.h file, such as "ExpA1" which is just WRF+SWAN, or "ExpA" which is just WRF+ROMS.

- 6.10 JOE_TC/DiffGrid simulation using ROMS + SWAN same grid, WRF different grid.

This application provides information on how to run the models on different grids. Each model could be on a separate grid, but for this example roms and swan are on the same grid, and wrf is on a different grid. This will be very close to the setup in 1 but now we will just create new grids for roms and swan. The roms and swan grids will be decreased resolution to have 100 cells in the x-direction and 75 cells in the y-direction (instead of 200x150).

- Step 1: Create a new projects folder

Projects/JOE_TC/DiffGrid. (this has already been created).

- Step 2: Create roms + swan grids. This can be accomplished using

COAWST/Tools/mfiles/create_roms_xygrid.m or wrf2roms_mw.

We renamed grid_coord.grd and roms_bathy.bot to

joe_tc_coarse_grid_coord.grd and joe_tc_coarse_roms_bathy.bot. Copy these 2 files and the joe_tc_coarse_grd.nc file to the project folder.

- Step 3: create roms init files using

COAWST/Tools/mfiles/mtools/create_roms_init.m

Step 4: Create wrf files: we will use the same wrf files from before of:

wrfbdy_d01

wrfinput_d01

namelist.input

These files need to be placed in the root_dir (I also make a copy and put into the project folder).

Step 5: Create the interpolation weights file – (The interpolation weight file for this case is already provided right now in the folder as “scrip_joe_tc_diffgrid.nc”). Follow the steps below to create the interpolation weight file yourself.

Build SCRIP_COAWST

We will use the SCRIP_COAWST package provided in “Lib” folder to create the interpolation weights. The steps for this are described in Section 3 above.

- Create the interpolation weights file -

- Edit the “scrip_coawst_joetc_diffgrid.in”. This is the input file and needs to be edited. (The input file for this case is provided). The name of the output interpolation weight file can be added at the beginning of this file. Let us call it “scrip_joe_tc_diffgrid.nc”

- Execute the scrip_coawst by going to the path of the SCRIP_COAWST folder and then running the program using

./scrip_coawst scrip_coawst_joe_tc_diffgrid.in

This will create the weights file scrip_joe_tc_diffgrid.nc

You can then copy that weights nc file to the Project folder

cp scrip_ scrip_joe_tc_diffgrid.nc  ../../Projects/JOE_TC/Diffgrid

- Step 6: Edit the Projects/JOE_TCd/coupling_joe_tc.in and use SCRIP_WEIGHT_OPTION=1

Add the lines:

- SCRIP_COAWST_NAME=Projects/JOE_TC/DiffGrid/scrip_ joetc_diffgrid.nc

Step 7: change the ocean and wave input file names to be

WAV_name = Projects/JOE_TC/DiffGrid/INPUT_JOE_TC_COARSE  !    wave model

OCN_name = Projects/JOE_TC/DiffGrid /ocean_joe_tc_coarse.in   ! ocean model

Step 8: Edit the ocean model configuration file.

Edit Projects/JOE_TC/DiffGrid /joe_tc.h and add the option

#define MCT_INTERP_WV2AT

#define MCT_INTERP_OC2AT

This will allow mct interpolation between the wave – atm models, and between the ocean – atm models.

This is set already to define case H which has those cpp options set.

- Step 9: make, run the program

- 6.11. Rip_current test case.

This test case is provided as an example of how to setup a coupled simulation to study a rip current. As of version 750, it supersedes the previous distribution of coupling to RefDif, as described in Haas and Warner (2009). The current test case is coupling of ROMS+SWAN, and the application is described in detail in Kumar et al. (2012) in section 4.3 of that paper. Users can adapt the test case for other types of similar investigations.

- The application is run by editing the coawst.bash file to set

export   ROMS_APPLICATION=RIP_CURRENT

This case requires mpi and the MCT libraries because it is a coupled application. And the run files are at:

export     MY_HEADER_DIR=${MY_PROJECT_DIR}/Projects/Rip_current

export MY_ANALYTICAL_DIR=${MY_PROJECT_DIR}/Projects/Rip_current

This is built like the other tests cases, using ./coawst.bash. The user needs to set the total number of processors for each of roms and swan in the coupling_rip_current.in file. Then you need to set the processor allocation in the ocean_rip_current.in. The model is set up to run for 1 hour of simulation time, with swan on a 5s time step and roms on a 1s time step. Coupling is every 5s. The model is run using:

mpiexec -np 4 ./coawstM.exe Projects/Rip_current/coupling_rip_current.in

You can certainly change the number of processors for your own system. Upon completion of the test case, you can use the included plot_rip_current.m matlab file to create the plot below.

- Figure 4. Rip current test case bathy and currents.

- 6.12  Hurricane Sandy = 2 grids for WRF, 2 grids for ROMS, 2 grids for SWAN.

The notes for this application are in the mfile in Projects/Sandy/create_sandy_application.m

This case was onriginally distributed as WRF (2 grids), ROMS (2 grids), and SWAN (2 grids). All of these capabiiltes still exist. But we have changed this case to show coupling to WAVEWATCH III. The m file create_sandy_application has been updated to discuss WW3 coupling. Please also read Section 11 on the WW3 coupling.

Figure 2: fPSFC of Hurricane Sandy along the US east coast.

- Figure 3: SST combined GFS and ROMS in WRF output.

6.13. Sed_floc_toy test case - ROMS only, tests sediment module with flocculation.
The test case is added to test the additional sediment module routines in COAWST that account for the aggregation, disaggregation of the cohesive sediment class in the water column. These processes are desribed as flocculation dynamics. This test case is still under development.

6.14 Sedbed_toy test case - ROMS only, tests sediment module.

- This test case is formed to highlight the capabilities of the sediment module in COAWST framework to simulate various sediment transport processes (for instance sediment erosion, deposition) etc. It can incorporate both cohesive and non-cohesive sediment types.  The details of the sediment module and related processes that can be handled are described in Warner et. al(2008).  The application is run by editing the coawst.bash file to set:

export   ROMS_APPLICATION=SEDBED_TOY

Figures 6.14 (a) and (b) illustrate the results at the end of simulation ().  The results show the concentration for two non-cohesive sediment classes varying with the number of vertical levels.

(a) Non-cohesive sediment class 1   		 (b) Non-cohesive sediment class 2

Figure 6.14 Results from Sedbed_toy test case

6.15 Trench test case - ROMS only, test sediment morphology.12

- The test case is based on the morphological evolution of bedload by simulating the experiemental work of Van Rijn (1993). The setup consists of a 30-m straight channel with a vertical trench in a mobile sand bed. More details are present in Warner et al. (2008). The application is run by editing the coawst.bash file:

export   ROMS_APPLICATION=TRENCH

Figure 6.15 shows the bed thickness varying with along channel distance at the start and end of the simulation at 500 secs. The results are extracted from a cross channel plane of y=4.

Figure 6.15 Results from Trench test case

6.16. Vegetation test case - ROMS+SWAN, test vegetation module.

This test case presents an idealized tidal and wave basin inhabited by a square patch of submerged aquatic vegetation (SAV) along with a marsh face. It accounts for the following hydrodynamic effects of vegetation in ROMS: sink of momentum, flexible plant reconfiguration, production and dissipation of turbulent kinetic energy. It also addresses wave damping by vegetation in SWAN and wave-current interactions around a vegetation patch (ROMS-SWAN). The details of this implementation are presented in Beudin et al. Wave thrust is computed on the marsh face following Tonelli et al.

Users can adapt the test case for other types of similar investigations. All the steps described below have already been undertaken to setup a vegetation test case in the Veg_test folder distributed with COAWST. The description of this test case is provided in the veg_test_case.docx file.

Figure 5: Schematic showing vegetation and marsh domains

- Step 1: Create roms grid and init files using

- COAWST/Tools/mfiles/mtools/create_roms_netcdf_grid_file.m

COAWST/Tools/mfiles/mtools/create_roms_init.m - To setup vegetation properties in the intial file, enter NVEG. Currently plant density, height and diameter and the marsh_mask arrays are set to zero. The user can provide these values depending on the presence of the vegetation. Copy the grid and init file to the vegetation test folder.

Step 2: Create the SWAN grid file and bathymetry file and copy them to the vegetation folder.

Step 3: Edit coupling_veg_test.in.

NnodesWAV and NnodesOCN provide the number of processors for Swan and Roms codes respectively. Provide the path of Swan and Roms input file in WAV_NAME and OCN_name inputs

Step 4: Edit the ocean_veg_test.in for providing number of interior points in I and J directions (Lm, Mm), number of vertical layers. Enter the tiling inputs NtileI and NtileJ accordingly. Provide the name of roms grid and input file as generated in Step 1 above in GRDNAME and ININAME options.

Step 5: Edit the swan input file to enter the swan grid file and bathymetry file. Make sure to have the Keyword “NPLANTS” and “VEGETATION” properties correctly. The Vegetation properties should be consistent with the ROMS input file.

Step 6: Edit the “vegetation.in” to specify sptatially fixed vegetation properties along with the specifying logical options such as Hout(pdens)==T to output vegetation density in the output “.nc” file.

Step 7: Edit ana_fsobc.in for tidal inputs.

Step 8. Compile the code using ./coawst.bash (Check that the application is set to veg_test). Run the code by using the input file “coupling_veg_test.in”).

- Figure 5: Figure generated from Veg_test case simulation showing depth averaged velocity (cm/s). Vegetation patch is in the outlined red box

6.17 Wetdry test case - ROMS only, test wetting/drying algorithms.

- This case is provided to test the ability of COAWST framework to let the computational cells to get indundated/wet or dewatered/dry when total depth in a cell is less than a user defined critical value. This test case is consisting of a basin of rectangular water basin. The test case allows for water to spill through an opening to flood in an initially dry flood plain. More details of the methodology and test case is provided in Warner et. al (2013). Figures 6.17(a) shows the free surface level at 4 secs. Figure 6.17(b) shows the water levels at x=1.54m (40th plane), y=1.0m (14th plane) varying with time.

- Water level at 4 secs

- Figure 6.17 Wetdry test case

- Section 7. Visualization Tools.

- NCVIEW

Source code at:

http://meteora.ucsd.edu/~pierce/ncview_home_page.html

Some new colormaps at:

https://www.myroms.org/forum/viewtopic.php?f=31&t=1930&p=6926#p6926

- VAPORGUI

this from the WRF visualization world.

http://www.vapor.ucar.edu/docs/install/index.php?id=unixbin

- Section 8. Setting up a WRF application.

This section describes how to set up a WRF (only) application. Some of this was taken from the WRF User’s manual. We use the ARW core. It is recommended for you to do the online wrf tutorial (http://www.mmm.ucar.edu/wrf/OnLineTutorial/index.htm)

- Building the WRF Code

Starting with COAWST svn version 469 (June 2, 2011), you can use the distributed WRF and WPS programs to create the WRF forcings and/or to run WRF by itself.

• Make a new directory and copy the COAWST code into that dir, and cd into that dir.

• You will always need a Projects header file. I recommend that you create a directory called "My_Project" (create your own name, dont actually use 'My_Project').  The directory could be: COAWST/Projects/My_Project.  In that directory, create a header file such as: my_project.h.  For a wrf only application, you would need to have in that file:

- #define WRF_MODEL

• edit coawst.bash and set the Project name, header file location, and build location.

• build the model with

./coawst.bash

choose one of the options

o usually, option "1" is for a serial build, that is the best for an initial test ( we use option 3 for pgi ), then 1 again for basic nesting (if needed).

• When it is done, you can list the contents:  ls WRF/main/*.exe

o if you built a real-data case, you should see ndown.exe, real.exe, and

wrf.exe

o if you built an ideal-data case, you should see ideal.exe and wrf.exe

- Building the WPS Code

Building WPS requires that WRF is already built. WPS is distributed with the code.

• cd COAWST/WPS

• ./configure

o choose one of the options

o usually, option "1" is for a serial build, that is the best for an initial test  (we used 5)

• ./compile

• ls -ls *.exe

o you should see geogrid.exe, ungrib.exe, and metgrid.exe

o if you built an ideal-data case, you should see ideal.exe and wrf.exe

• ls -ls util/*.exe

o you should see a number of utility executables: avg_tsfc.exe, g1print.exe,

g2print.exe, mod_levs.exe, plotfmt.exe, plotgrid.exe, and

rd_intermediate.exe

geogrid:  Creating a Grid

- 1) edit WRF/WPS/namelist.wps

namelist.wps

&share

wrf_core = 'ARW',

max_dom = 1,

start_date = '2003-12-01_00:00:00',

end_date   = '2003-12-19_00:00:00',

interval_seconds = 21600

io_form_geogrid = 2,

/

&geogrid

parent_id         =   1,

parent_grid_ratio =   1,

i_parent_start    =   1,

j_parent_start    =   1,

e_we              =  395,

e_sn              =  360,

geog_data_res     = '10m',

dx = 12000,

dy = 12000,

map_proj = 'mercator',

ref_lat   =  32.0381,

ref_lon   = -77.45058,

truelat1  =  31.5,

truelat2  =  0.0,

stand_lon = -78.4,

geog_data_path = '/raid1/jcwarner/Models/WRF/geog'

/

&ungrib

out_format = 'WPS',

prefix = 'FILE',

/

&metgrid

fg_name = 'FILE'

io_form_metgrid = 2,

opt_output_from_metgrid_path = './',

opt_metgrid_tbl_path         = 'metgrid/',

/

&mod_levs

press_pa = 201300 , 200100 , 100000 ,

95000 ,  90000 ,

85000 ,  80000 ,

75000 ,  70000 ,

65000 ,  60000 ,

55000 ,  50000 ,

45000 ,  40000 ,

35000 ,  30000 ,

25000 ,  20000 ,

15000 ,  10000 ,

5000 ,   1000

/

2) get the geogrid data http://www.mmm.ucar.edu/wrf/src/wps_files/geog.tar.gz

untar this to a directory called geog

gzip -cd geog.tar.gz | tar -xf -

Set the geog_data_path in the namelist.wps file to this dir.

- 3) make sure there is a link to the correct geogrid table.

> ls –ltr WPS/geogrid/*.TBL

should return

> geogrid/GEOGRID.TBL -> GEOGRID.TBL.ARW

- 4) run geogrid

>WPS  ./geogrid.exe

and get back a complete successful information.

- 5) ls –ltr should show a geo_em.d01.nc file. Use ncview or some other viewer to check it out.

ungrib: Getting IC and BC data

- 1) Get grib data here:

http://dss.ucar.edu/datasets/ds083.2/

You will need to register first and get a username/passwd.

Then you can use wget (v1.9 or higher) to get the data. Here is an example of a wget script:

##############   file get_gfs_data  #########################

#! /bin/csh -f

#

# c-shell script to download selected files from dss.ucar.edu

# using wget

# NOTE: if you want to run under a different shell, make sure you

# change the 'set' commands according to your shell's syntax

# after you save the file, don't forget to make it executable

# i.e. - "chmod 755 <name_of_script>"

#

# Experienced Wget Users: add additional command-line flags here

#   Use the -r (--recursive) option with care

#   Do NOT use the -b (--background) option - simultaneous file

#   downloads can cause your data access to be blocked.

set opts = "-N"

#

# Replace "xxxxxx" with your password

# IMPORTANT NOTE:  If your password uses a special character that has

# special meaning to csh, you should escape it with a backslash

#                  Example:  set passwd = "my\!password"

set passwd = "MYPASSWD"

#

#set cert_opt = ""

# If you get a certificate verification error (version 1.10 or higher),

# uncomment the following line

set cert_opt = "--no-check-certificate"

#

# authenticate

wget $cert_opt -O /dev/null --save-cookies auth.dss_ucar_edu --post-data="email=jcwarner@usgs.gov&passwd=$passwd&action=login" https://dss.ucar.edu/cgi-bin/login

#

# download the file(s)

wget $cert_opt $opts --load-cookies auth.dss_ucar_edu http://dss.ucar.edu/datasets/ds083.2/data/grib1/2003/2003.09/fnl_20030909_00_00

wget $cert_opt $opts --load-cookies auth.dss_ucar_edu http://dss.ucar.edu/datasets/ds083.2/data/grib1/2003/2003.09/fnl_20030909_06_00

wget $cert_opt $opts --load-cookies auth.dss_ucar_edu http://dss.ucar.edu/datasets/ds083.2/data/grib1/2003/2003.09/fnl_20030909_12_00

wget $cert_opt $opts --load-cookies auth.dss_ucar_edu http://dss.ucar.edu/datasets/ds083.2/data/grib1/2003/2003.09/fnl_20030909_18_00

wget $cert_opt $opts --load-cookies auth.dss_ucar_edu http://dss.ucar.edu/datasets/ds083.2/data/grib1/2003/2003.09/fnl_20030910_00_00

#

# clean up

rm auth.dss_ucar_edu

##############  end of  file get_gfs_data  #########################

make the file executable and run it : ./get_gfs_data

cp the data, I used /raid1/jcwarner/Models/WRF/gfs_data/fnl_0309_gfs

- 2) The gfs data is in grib format. We want to convert that format to something that WRF can read so we use the ungrib.exe program. cd to COAWST/WPS

link a Vtable to the GFS data with:

>ln –sf ungrib/Variable_Tables/Vtable.GFS Vtable

- 3) link the GFS files to common names that WPS will recognize. Use:

( I moved all the fnl files to the gfs_data/fnl_0309 dir).

> ./link_grid.csh     /raid1/jcwarner/Models/WRF/gfs_data/fnl_0309_gfs/fnl_0309

Do an ls -ltr and see all the GRIBFILE.AAA etc ….linking to all the gfs data files.

- 4) Run ungrib

> ./ungrib.exe >& ungrib.out &

Edit the .out file and see if there were any problems. do an ls and see FILE:YYYY-MM-DD-HH.

- 5) Get SST data

- Go here for SST data:  ftp://polar.ncep.noaa.gov/pub/history/sst

for the times that you want, and put into a folder.

I used: /WRF/gfs_data/fnl_0309_sst

5a) WPS/ ln -sf ungrib/Variable_Tables/Vtable.SST Vtable

5b) WPS/ rm GRIBFILE*

WPS/ ./link_grib.csh   /raid1/jcwarner/Models/WRF/gfs_data/fnl_0309_sst/rtg_sst_grb_0.5

5c) edit COAWST/WPS/namelist.wps and change

prefix = ‘FILE’

to

prefix = ‘SST’

5d) COAWST/WPS/  ./ungrib.exe >& ungrib_sst.out &

edit the .out file and see that all went well.

metgrid: Interpolate ungribbed met data to the grid.

1a) Check the the metgrid table is pointing to the right place.

> cd metgrid

> ls –ltr

should show METGRID.TBL -> METGRID.TBL.ARW

1b) if you acquired sst data, then edit

COAWST/WPS/namelist.wps and change the fg_name from

fg_name = ‘FILE’

to

fg_name = ‘FILE’, ‘SST’

1c) run metgrid

COAWST/WPS/ ./metgrid.exe

As it runs you see …. processing FILE, SST, …

When done check that the met files were made. Do an ls –ltr and see the met_em.d01** files.

real.exe to create the init and BC files.

Now we need to run the real program.

- 1) >cd WRF/test/em_real

edit the namelist file. I have:

&time_control

run_days                            = 18,

run_hours                           = 0,

run_minutes                         = 0,

run_seconds                         = 0,

start_year                          = 2003, 2000, 2000,

start_month                         = 12,   01,   01,

start_day                           = 01,   24,   24,

start_hour                          = 00,   12,   12,

start_minute                        = 00,   00,   00,

start_second                        = 00,   00,   00,

end_year                            = 2003, 2000, 2000,

end_month                           = 12,   01,   01,

end_day                             = 19,   25,   25,

end_hour                            = 00,   12,   12,

end_minute                          = 00,   00,   00,

end_second                          = 00,   00,   00,

interval_seconds                    = 21600

input_from_file                     = .true.,.true.,.true.,

history_interval                    = 60,  60,   60,

frames_per_outfile                  = 12, 1000, 1000,

restart                             = .false.,

restart_interval                    = 720,

io_form_history                     = 2,

io_form_restart                     = 2,

io_form_input                       = 2,

io_form_boundary                    = 2,

debug_level                         = 0,

auxinput4_inname = "wrflowinp_d<domain>",
 auxinput4_interval = 360,
 io_form_auxinput4 = 2

/

&domains

time_step                           = 60,

time_step_fract_num                 = 0,

time_step_fract_den                 = 1,

max_dom                             = 1,

s_we                                = 1,     1,     1,

e_we                                = 395,    112,   94,

s_sn                                = 1,     1,     1,

e_sn                                = 360,    97,    91,

s_vert                              = 1,     1,     1,

e_vert                              = 28,    28,    28,

num_metgrid_levels                  = 27

dx                                  = 12000, 10000,  3333.33,

dy                                  = 12000, 10000,  3333.33,

grid_id                             = 1,     2,     3,

parent_id                           = 0,     1,     2,

i_parent_start                      = 1,     31,    30,

j_parent_start                      = 1,     17,    30,

parent_grid_ratio                   = 1,     3,     3,

parent_time_step_ratio              = 1,     3,     3,

feedback                            = 1,

smooth_option                       = 0,

nproc_x                             =8,

nproc_y                             =8,

/

&physics

mp_physics                          = 6,     3,     3,

ra_lw_physics                       = 1,     1,     1,

ra_sw_physics                       = 1,     1,     1,

radt                                = 6,    30,    30,

sf_sfclay_physics                   = 2,     1,     1,

sf_surface_physics                  = 2,     2,     2,

bl_pbl_physics                      = 2,     1,     1,

bldt                                = 0,     0,     0,

cu_physics                          = 1,     1,     0,

cudt                                = 6,     5,     5,

isfflx                              = 1,

ifsnow                              = 0,

icloud                              = 1,

surface_input_source                = 1,

num_soil_layers                     = 4,

ucmcall                             = 0,

maxiens                             = 1,

maxens                              = 3,

maxens2                             = 3,

maxens3                             = 16,

ensdim                              = 144,

sst_update                          = 1

/

&fdda

/

&dynamics

w_damping                           = 0,

diff_opt                            = 1,

km_opt                              = 4,

diff_6th_opt                        = 0,

diff_6th_factor                     = 0.12,

base_temp                           = 290.

damp_opt                            = 0,

zdamp                               = 5000.,  5000.,  5000.,

dampcoef                            = 0.2,    0.2,    0.2

khdif                               = 0,      0,      0,

kvdif                               = 0,      0,      0,

non_hydrostatic                     = .true., .true., .true.,

pd_moist                            = .true., .true., .true.,

pd_scalar                           = .true., .true., .true.,

/

&bdy_control

spec_bdy_width                      = 5,

spec_zone                           = 1,

relax_zone                          = 4,

specified                           = .true., .false.,.false.,

nested                              = .false., .true., .true.,

/

&grib2

/

&namelist_quilt

nio_tasks_per_group = 0,

nio_groups = 1,

/

- Pay attention to time and e_* settings.

- 2) cp or link metfiles to the WRF/test/em_real directory.

> ln –sf /raid1/jcwarner/Models/WRF/WPS/met_em.d01.2003-09* .

- 3) If you do not want bogus vtex, go to step 4.

If you want to include a Bogus Vortex

This is sort of painful, but may be worth it.

3a) reconfigure WRF to work in serial mode 1, option 0.

3b) edit WRFV3/test_em_real/namelist.input to include

&tc

insert_bogus_storm                  = .true.

remove_storm                        = .false.

latc_loc                            = 21.2

lonc_loc                            = -52.3

vmax_meters_per_second              = 62.5

rmax                                = 100000.0

vmax_ratio                          = 0.85

/

Namelist var descriptions here =>

http://www.mmm.ucar.edu/wrf/users/docs/user_guide_V3.1/users_guide_chap10.htm#tc_bogus

3c) Run tc.exe with

COAWST/WRF/test_em_real/mpirun –np 1 –hostfile nodes.list ./tc.exe

3d) cp the outfile to replace the first met_em file. for example:

cp auxinput1_d01_2003_09_11:00:00:00 met_e.d01.2003_09_11:00:00

3e) reconfigure wrf:

>WRF  ./configure  (3,1)

>WRF ./compile em_real >& compile.out &

- 4) run the real program

4a) edit namelist.input and set nproc_x=1, nproc_y=1

4b) Make a file called nodes.list, and just list one node

nemo3

Then run real (in mpi) with this command:

> mpirun –np 1 –hostfile nodes.list ./real.exe

When done, check to see that it made wrfinput_d01 and wrfbdy_d01 netcdf files.

Edit the rsl.error* and rsl.out* files to see that all went well.

I then removed the met* files.

run WRF.

Now we need to run the wrf program. For a wrf only application, I soft linked coawstM with WRF/test/em_real/wrf.exe.  Edit namelist.input and reset nproc_x=NX, nproc_y=NY

> qsub run_nemo with :

#!/bin/bash

### Job name

#PBS -N wrf1

### Number of nodes

#PBS -l nodes=5:ppn=8,walltime=120:00:00

### Mail to user

#PBS -m ae

#PBS -M jcwarner@usgs.gov

### Out files

###PBS -e isabel_105.err

###PBS -o isabel_105.out

### PBS queue

#PBS -q standard

echo "this job is running on:"

cat $PBS_NODEFILE

- NPROCS=`wc -l < $PBS_NODEFILE`

cd /raid1/jcwarner/Models/COAWST

mpirun -np 40 -machinefile $PBS_NODEFILE ./coawstM > wrf_run1.out

look at the output.

ncview wrfout_d01_***

Section 9. Setting up a ROMS application.

This section is under development. Users are suggested to view the online tutorials from the workshops.

The main driver is Tools/mfiles/roms_clm/roms_master_climatology.m. User needs to edit this file to identify grid and directory names. This set of tools will interpolate HYCOM data to the user grid for the requested time period. Users will need to edit the irg.mat and jrg.mat files to set the HYCOM grid indices from which to interpolate the data from.

- Section 10. Setting up a SWAN application.

Wind Forcing:

If not coupled to wrf, you need to download wind data from somewhere and create an ascii wind file. See the SWAN manual as to their format.  It needs to be written out in an ascii text file. We typically use format number 4. We typically obtain wind data from NARR at http://nomads.ncdc.noaa.gov/.  You can use narr2romsnc.m to do this.

Boundary data:

There are several sources and methods to obtain boundary data from swan. We typically obtain data from Wave Watch 3 and use this to create TPAR files (see swan manual).  Here are some helpful methods/tools.

1. To run in hindcast mode, you will need to download historical ww3 data in grib2 format from NOAA's ftp server. First, decide what noaa grid you want the data from

ftp://polar.ncep.noaa.gov/pub/history/waves/README

2. Get the data. Go here to get ftp data from the grid and time period you need.

This is the global data set:

ftp://polar.ncep.noaa.gov/pub/history/waves/

(scroll down to, for example,  multi_1.glo_30m.xx.YYYYMM.grb2)

get the three files for each month: xx = hs, tp, dp files (Hsig, Period, and Direction).

Do not change the file names.

3. The file Tools/mfiles/swan_forc/ww3_swan_input.m is the main driver. This is used to create SWAN TPAR boundary forcing files. For hindcast mode, the TPAR files will be converted from the grib2 data using nctoolbox. Edit Tools/mfiles/swan_forc/ww3_swan_input.m

and enter the user required info. Run the m file.

5. Edit the SWAN INPUT file and add the information from the Bound_spec_command file.

6. Place the TPAR files in the ../forcings folder.

Init files:

Method 1: (try this first)

In the swan input file, use the command

INITIAL DEFAULT

(need to use linear wind growth terms)

Method 2:

To create an init file for swan, you can run SWAN in stationary mode and create a ‘hot start’ file(s).

- Section 11. Setting up a WAVEWATCH III application.

In January 2018 we added WaveWatch III to the coupled system.

******************* However:

We can not distribute the wave watch code right now. This will change when WaveWatch releases version 6.0, planned for November 2018. But for now, YOU have to go get the wavewatch tar file. To do this, you need to register to get the Wave Watch code. Go here:

http://polar.ncep.noaa.gov/waves/wavewatch/

and register to get the code. They will send to you a username and pwd. Follow their instructions and get the tar file. Below we describe how to incorporate the wavewatch tar file into the COAWST system.

WaveWatch can be run alone or coupled to ROMS, to WRF, or to both. For now we can only run one WW3 grid (ww3_shel), and if WW3 is coupled to another model(s), the other model(s) can run multiple grids. We are looking to incorporate the multi domain capability of WW3 in the future.

As with every other model, we recommend that the user become familiar with that model first. The WW3 user manual is provided in COAWST/WW3 directory. To run WW3, we will go through the Sandy test as an example to run WW3 by itself. This will provide the steps necessary.

1) You need to get the wavewatch tar file. Register to get the Wave Watch code. Go here:

http://polar.ncep.noaa.gov/waves/wavewatch/

and register to get the code. They will send to you a username and pwd. Follow their instructions and get the tar file.

2) Copy that tar file to this directory:

COAWST/WW3

In that directory you will see your tar file (wwatch3.v5.16.tar), a file called add_ww3_to_coawst, and a directory called coawst_ww3_files.

3) run the add_ww3_to_coawst bash file, by typing

./add_ww3_to_coawst

This will perform the unpacking of the tar file and install of wave watch. You will need to answer these questions:

Select ‘y’ to continue.

Select ‘L’ for a local install.

Select ‘y’ to install in whatever directory you are in (it will be different than shown here).

At this point, you can just say ‘n’ to not update any settings. These will change when you actually compile a test case or application.

One last time to enter ‘y’ to continue with the actual install.

The install will then go through many screens to extract the files. When it is done, you should see:

This will tell you that the program installed correctly.

4) edit coawst.bash and set:

export   COAWST_APPLICATION=SANDY

and set the root location of your code, such as:

export   MY_ROOT_DIR=/sand/usgs/users/jcwarner/tests/coawst_v3.3

5) We then need to set 5 WW3 environement settings.

#1) COAWST_WW3_DIR is a pointer to root WW3 code, do not change.

export   COAWST_WW3_DIR=${MY_ROOT_DIR}/WW3

#2) WWATCH3_NETCDF can be NC3 or NC4. We need NC4 for COAWST. do not change.

export   WWATCH3_NETCDF=NC4

#3) WWATCH_ENV points to WW3 environment listing. do not change.

export   WWATCH_ENV=${COAWST_WW3_DIR}/wwatch.env

#4) NETCDF_CONFIG is needed by WW3. You need to set this:

export   NETCDF_CONFIG=/usr/bin/nc-config

This may require

export   NETCDF_CONFIG=/usr/bin/nf-config

depending on your system.

#5) WW3_SWITCH_FILE is like cpp options for WW3. You need to create it and

#        list the full name here. This is described below.

export  WW3_SWITCH_FILE=switch_sandy

6) As with all applications, set the MPI and compiler information (this will depend on your system):

export           USE_MPI=on            # distributed-memory parallelism

export        USE_MPIF90=on            # compile with mpif90 script

export         which_MPI=openmpi       # compile with OpenMPI library

export              FORT=ifort

7) Set the location of the header and analytical files:

export     MY_HEADER_DIR=${MY_PROJECT_DIR}/Projects/Sandy

export MY_ANALYTICAL_DIR=${MY_PROJECT_DIR}/Projects/Sandy

8) For this description, we are saying to run WW3 by itself so edit Projects/Sandy/sandy.h and only list

#define WW3_MODEL

9) The compiler flags for the WW3 build are taken from the Compilers/* file. For example, if you work on Linux with ifort, then the Compilers/Linux-ifort.mk file will be used for the WW3 build.  You may also need to edit some WW3 specific build files.  So if you selected ifort (for example), then we need to look at

COAWST/WW3/bin/comp.Intel

COAWST/WW3/bin/link.Intel

Please look at these files (or the files needed for your compiler) to see if the flags are correct.

10) Create a switch file.

The switch file is like the cppdefs file for roms. It tells WW3 what features to compile. We have a file COAWST/WW3/bin/switch_sandy. List the full switch file name (new feature) in the coawst.bash file. The file should start with ‘switch_’ Here is the switch_sandy:

F90 NOGRB COAWST LRB4 NC4 TRKNC DIST MPI PR3 UQ FLX0 LN1 ST4 STAB0 NL1 BT4 DB1 MLIM TR0 BS0 IC2 IS2 REF1 IG1 XX0 WNT2 WNX1 RWND CRT1 CRX1 TIDE O0 O1 O2 O2a O2b O2c O3 O4 O5 O6 O7

Please read the WW3 manual in the WW3 directory. Some important options are:

NOPA	DO NOT USE THIS.  This is for stand alone WW3, and I have modified the build structure to be part of COAWST. So do not ever use it.

PALM	Do NOT USE PALM – this is a different coupler. I may try to use some of these features in the future, but for now do not use it.

COAWST	ALWAYS list this. I have modified the build strucutrre to be part of COAWST (this is new as of June 2018.).

SCRIP, SCRIPNC	I don’t use the WW3 scrip but we may in the future. So these are not really needed.

DIST, MPI	These are to use mpi, needed.

11) compile the code by running ./coawst.bash at the command prompt.

If it compiles correctly, you will get a coawstM.

12) Now we need to create some WW3 grids files. Here is a way to do that. There are probalbly many other ways to do this.

You can use Tools/mfiles/mtools/create_ww3_grid_files to create an x, y, bath, and mask files for WW3. This m file uses the roms grid to create the WW3 files of:

ww3_sandy_xcoord.dat, ww3_sandy_ycoord.dat, ww3_sandy_bathy.bot, and ww3_sandy_mapsta.inp.

13) Forcinng files for WW3: again, there are probably many ways to do this, but here is one way.

You can use Tools/mfiles/mtools/create_ww3_wind_forcing to create a wind forcing file ww3_sandy_winnd_forc.dat.

14) We then need to create some WW3 run files using 4 of their utilities:

- edit WW3/work/ww3_grid.inp. Here is where you enter number of bins, advection choices, time steps (we chose 180), and the grid settings

'CURV'  T  'NONE'

84     64

40 1. 0. 1 1 'FREE' 'NAME' 'ww3_sandy_xcoord.dat'

40 1. 0. 1 1 'FREE' 'NAME' 'ww3_sandy_ycoord.dat'

-1.0  -1.0  40 -1. 1 1 '(....)' 'NAME' 'ww3_sandy_bathy.bot'

Then cd to WW3/work and run ../exe/ww3_grid to create mask.ww3, mapsta.ww3, mod_def.ww3.

- edit WW3/work/ww3_strt.inp. Here is where you enter init information.

Then cd to WW3/work and run ../exe/ww3_strt to create restart.ww3.

- edit WW3/work/ww3_prep.inp. Here is where you enter forcing data. We have:

$

'WND' 'LL' T T

$ Additional input format type 'LL' ---------------------------------- $

$ Grid range (degr. or m) and number of points for axes, respectively.

$ Example for longitude-latitude grid.

-105. -50. 551   10. 50. 401

This tells WW3 that the wind is on a grid with these lon/lat values.

$ Define data files -------------------------------------------------- $

$ The first input line identifies the file format with FROM, IDLA and

$ IDFM, the second (third) lines give the file unit number and name.

'NAME' 1 1 '(....)' '(....)'

40 'ww3_sandy_wind_forc.dat'

This tells WW3 the name of the wind forcing file.

Then cd to WW3/work and run ../exe/ww3_prep to create wind.ww3.

- edit WW3/work/ww3_shel.inp. Here is where you enter run information.

For this test we are just looking to run WW3 alone so we choose:

F F     Water levels

F F     Currents

T F     Winds

The first letter is to set if the field is to be not used (F), used as read in from a file (T), or used and read in from a coupled model ( C). So those settings are to read winds from a file.

If we had coupled WW3 to WRF we could use

F F     Water levels

F F     Currents

C F     Winds

And that would get coupled winds from WRF.

If we had coupled WW3 to ROMS, then we could use

C F     Water levels

C F     Currents

T F     Winds

To get coupled fields from roms to ww3 and read a wind file.

If we had coupled WW3 to ROMS and to WRF, then we could use

C F     Water levels

C F     Currents

C F     Winds

You need to set the run times here:

$ Type 1 : Fields of mean wave parameters

20121028 120000   1800  20121030 120000

and set to have the fields of Hwave, Dwave, and Lwave written out:

$ HS  LM  T02 T0M1 T01 FP DIR SPR DP

T   T   T   T   T   T   T   T   T

We do NOT need to run ../exe/ww3_shel because that would run the wave model. We will run the wave model as coawstM.

15) Now we are ready to run WW3.  Cd to the root dir and use run_* script to run the code on your system. We have run_coawst as an example of a run sciprt, but you need to get a script that works on your system. You do not need to set the number of processors in any of the ww3 files.

mpirun –np X ./coawstM WW3/work/ww3_grid.inp

16) When the model is done you can visualize the output by using

cd WW3/work and type

../exe/ww3_ounf

This will create a ww3.*.nc file called ww3.201210.nc.

Figure 11.1 Ncview of WW3 wave heigth for Sandy simulation.

- Section 12. Setting up an InWave application.

InWave solves the wave action balance conservation equation, which is given by:

where A stands for the time varying energy action or density for each directional bin (); , , and  represent the wave group celerity in the x, y, and directions respectively (Olabarrieta, et al., in prep).  is the energy dissipation due to the wave breaking and is the intrinsic wave frequency.

There are two ways to run InWave: 1) forcing the model using a 2D spec file from SWAN and 2) imposing time series of wave action density along the open boundaries. Option 1 has an example for the Inlet_test/InWave (Example 6.5) and Option 2 example is Shoreface_InWave (Example 6.8). The cpp options for InWave are described in 4.3.10.  More information will be provided soon.

- Section 13. MATLAB (.m) scripts for pre/post processing.

- As of release 567, February 8, 2012, we have rewritten the m files to use native matlab netcdf interface.  This will allow a more uniform practice amongst users. The caveat of this is that the matlab interface uses the Fortran convention, ie the variables are read and writen as Var(x,y,z,t).  Many of the previous toolboxes used the C convention of Var(t,z,y,x).  Please be careful when using the native matlab. You can use Tools/mfiles/mtools/netcdf_load to use native matlab to load a netcdf file into the workspace.

- The only other toolbox that is currently required is the nctoolbox found at: http://code.google.com/p/nctoolbox/

Follow instructions on that website to install that toolbox. It is relatively painless. These nctools are used in the roms_clm routines to read HYCOM data via opendap. They are also used in the swan_forc m files to read the grib2 WW3 data.

This nctoolbox is not required to run the COAWST system. We are just offering this as a method to obtain forcings, bry, and init data. Users can obtain this data by many other methods.

mfile listing:

- inwave_tools:  under development.  Not supported at this time.

- m_map: set of utilities used in the grid generation routines, used to convert lat/lon to meters, and to support different projections.

- mtools: set of utilites mostly for ROMS to create grids, load a netcdf file, converting grids for scrip, creating wrf grid from roms grid, etc.

- roms_clm:  main driver is roms_master_climatology_coawst_mw.m, used to create boundary, init, and climatology files for roms grids using opendap to read HYCOM data.

- rutgers:  contains m files from Rutgers for grid, landmask, netcdf, and utility folders.

- swan_forc: main driver is ww3_swan_input.m to read WW3 grib2 data and create SWAN TPAR forcing files

- tides:   Use this set of m files to create tidal forcings for roms.

- Section 14. Tracking model development changes log.

"svn log" gives the whole listing. Here are some key updates:

Rev #	Date			Description

--------------------------    COAWST V1.0 ------------------------------------

0	01Jan2009	First upload.

17	23Feb2009	Merge grid refinement with atm coupling methods.

83	16Apr2009	Update time stepping for refined grids.

99	24Apr2009	Add SWAN forcing package.

107	28Apr2009	Add InWave.

147	06Jul2009	Add Inlet tests (3).

185	07Aug2009	Update to WRFV3.1.1

196	03Sep2009	Add BLOCK_JONSSON.

203	14Sep2009	Add UV_KIRBY vel avg for wave coupling.

207	16Sept2009	Added hwave/lwave from SWAN to WRF.

--------------------------    COAWST V2.0 ------------------------------------

250-262	01May2010	Update to ROMS svn 455, Update SWAN 4072ABCDE

This was a major update. Complete build is now handled thru coawst.bash script.

--------------------------    COAWST V3.0 ------------------------------------

300-315

330	15Jun2010	Removed old Project files.

331-338	07Jul2010	Add ATM2OCN_FLUXES to allow consistent wrf-roms fluxes and to update scrip interpolation to use conservative fluxes.

351	19Jul2010	Update Inlet tests to work with consv fluxes, and be more consistent. Update manual to include Inlet Tests descriptions, added m files to create the refined grids.

393	07Dec2010	SWAN uses urms instead of ubot to compute Madsen bottom stress. Also pass urms to roms for bbl models instead of ubot. Rename Ub_swan to Uwave_rms. If user does not activate a bbl, then swan fric will be based on user supplied value in swan.in. If user does activate bbl, then swan fric is based on ZoNik.

426	27Feb2011	Update to WRF 3.2.1

430	05Mar2011	Incorporate wec-vortex force method.

433	09Mar2011	Update to SWAN 40.81

469	02Jun2011	Add WPS and WRF to run separately

476-477	12Jul2011	Add WRF_CPL_GRID to choose which wrf grid to couple to. Modifications to work for ifort on cx2. Create mct_couple_params file to hold coupled vars. Modify WRF to write to stdout, not to rsl files. modify coupling.in files.

500-516	27Oct2011	Update to conservative 2-way grid refinement for ROMS.

528-531	30Nov2011	Incorporate Seaice module.

561	08Feb2012	Update m files to use native matlab netcdf interface. Go thru the distributed swan_forc, roms_clm, and mtools to ensure functionality. Update /cleanup manual.

599	11Jul2012	last distribution on hosted-projects.com Moved to Source repo.

602	11Jul2012	Allow SWAN only simulations.

603	11Jul2012	Allow WRF only simulations.

607-635	24Jul2012	Update to WRF 3.4.

669	02Oct2012	Add #define ROMS_MODEL to all roms apps.

672	11Oct2012	Change mct_couper_params to mct_wrf_coupler_params in wrf side, update wrf time step counter.

673	11Oct2012	Update ROMS grid refinement to allow refined region to vary spatially in the parent grid (interp2 w/ spline).

680	22Oct2012	Edit Compilers to not use LIBS=netcdf calls for swan only compilations.

681	22Oct2012	Allow SWAN to create its own hot file.

682	24Oct2012	Allow SWAN to run in STAT mode.

686	01Nov2012	Update mask_io to not overwrite point sources in his files.

701-708	04Feb2013	Update SWAN to 4091A.

725	26Apr2013	Update JOE_TC to use sf_surface_physics = 2.

751-752	06Sept2013	Update Rip_current test case to Kumar et al 2012.

758	16Sept2013	Update some roms input files so they all have Ltracer src

762-767	26Sept2013	Allow WRF-SWAN coupling, add DRAGLIM_DAVIS

771	23Oct2013	Add headland test case

789	19Dec2013	Update wrf-swan cpling for diff grids.

801	07Mar2014	Update Compilers and mct_coupler for gfortran with wrf.

813	17Apr2014	Update Compiles and wrf makefile, -I more wrf paths, gfortran flags for recl=4

866	07Aug2014	COAWST v3.1 released.

914	17Sept2014	Add new src weights for atm land points to ocn wet pts.

919	25Sept2014	Reorganize JOE TC and other test cases.

933	20Oct2014	Update wec stokes computations.

936	01Nov2014	Correct tile MPDATA ranges.

943	17Nov2014	Add spectral light and sea grass.

951	10Dec2014	Correct Longwave flux bug for roms from multiple wrf grids. Correct restart for coupled wrf runs, and modify wrf determination of grid number for cpling.

953	17Dec2014	Add some SCRIP as matlab routines

956	19Dec2014	Update to Rutgers nesting and wet/dry masking.

959	06Jan2015	Update way to call SCRIP in weights computation mfiles.

964	05Feb2015	Rutgers bug 658 to use LallocateClima in mod_arrays.F and correct gasdev.F for randum number; Rutgers bug # 659 for set_contact.F for wet_dry weights and mod_mixing.F for FORWARD_MIXING spelling.

965	10Feb2015	Correct swan compute bound locations for child, enforce no-removal of grid rows for tiling during all swan apps

973	04Mar2015	Update SCRIP routines to check for coincident grids.

974	04Mar2015	Add NPLANTS to SWAN read.

982	19Mar2015	Allow SWAN to create RESTART files during time stepping. Add gridnum to write out during SWAN STATIONARY runs.

985	24Mar2015	Add wetdry_psi to wrt_his, add swan restart file to Inlet_test, update inlet test sediment input files to include bcs, update init m files to read a his file and to add bedload.

986	02Apr2015	Updates for ROMS src bug #s 662, 663, ad 664 for t3dmix, lmd mixing, and 3dfldr routines.

990	14Apr2015	update ww3_swan_input to use nctoolbox v1.1.1

994	27Apr2015	update def his for pmask_wet

996	12May2015	correct east west sums in nesting.F

1000	26May2015	add disclaimer

1001	29May2015	change waves in WRF/Registry from i0124rhd to i012rhd,

only have one namelist.input file for JOE_TC examples.

1008	14Jul2015	updates to ROMS to include src#665: fennel.h; src#666: nesting.F checksums; src#667 was wetdry mask write for his but we already had this; src#668: check_multifile EXIT  removed; src#669: add limit_stflux_coolig to set_vbc; src#670: ad_v2dbc typo; src#671: mod_boundary tl typo

1010	04Aug2015	updates to ROMS to include src#674 MEAN_AGE plus inp_par changes to load_s1d and _s2d; src#675 for ANA_SSFLUX and ANA_PASSIVE; and src#676 nesting.F deallocation. Update Adjoint, Representer, and Tangent folders, User, and Functionals.

1014	20Aug2015	updates to Rutgers: src#677 npzd powell; src#673 dogbone ngc files; src#678 ntimestep.F for >3 telescoped nested grids; src#679 inimetricval in mod_grid.F; src#680 masking in refined and read_stapar.F; update Projects Sedbed_toy and Sed_floc_toy

1015	31Aug2015	make sstupdate read i1, update joetc scrip master

1023	15Sep2015	update maplev to use nearest not average.

1032	08Oct2015	update to Rutgers: src#682 for timers.F; src#684 for nesting.F check of massflux for debugging only; src#685 for nesting call from main2d and main3d.

1033	08Oct2015	correct fac2 computation in wec_wave_mix.F.

1072-1077	20May2016	Release of COAWST v3.2

1084	24June2016	correct point source indices.

1086	07July2016	correct WRF moving nest GLW to ROMS.

1087	12July2016	update to Rutgers: src#697 output z_* to his files; src#698; src#699 gmake version; src#700 Output switches for sediment wrt to his and station files; src#701 SGRID conventions; src#702 4DV update

1111	22Sep2016	ROMS src updates 705, 706, 707, 709, 710, WRF Registry.EM_COMMON update, Correct WRF rotations to ROMS for U10, V10, USTRESS, and VSTRESS for non-mercator grids, add nomads_2roms.m, add offset in SCRIP-COAWST for coincident grids.

1122	06Dec2016	InWave read of 2d spec files

1130	01Jan2017	Update SCRIP for WRF – ROMS land sea masking mismatches.

1132-1136	15Feb2017	ROMS src updates including 704 and 708 Quick Files, 711-720 for bug in nesting tracer flux, adjoint updates, write water points, nc_config replace nf_config, Copyright to 2017, and reading Qbar.

1142	02Mar2017	set surface dissip props from swan to 0 for initial exchange to roms because they are not computed yet

1143	06Mar2017	update SCRIP for swan-roms coindent points.

1151	13Apr2017	add line contunations to SCRIP

1165	09Jun2017	update InWave Lwave and zeta_filter

1179	31July2017	add sedslump

1197-1200	12Jan2018	COAWST v3.3: Update ROMS to svn 885, WRF to v3.9.1.1, SWAN to v41.20, add WW3 v 5.16

1237-1256	04Jun2018	Update to COAWST v3.3.1 to include ROMS tickets 757-769

- Section 15.  Previous COAWST training workshops and Data.

We have held several COAWST Modeling System Trainings. The first was at the USGS Woods Hole Science Center from July 23-27, 2012. It was attended by 46 scientists from over 8 countries onsite. The second Training was held on the Woods Hole Oceanographic Institution (WHOI) campus from August 25-28, 2014. It was attended by 78 scientists from over 15 countries. The third was also on the WHOI campus from August 15-19, 2016 and was attended by over 85 scientists onsite and via webex worldwide. All the Trainnigs were recorded and the presentations, photos, agenda, and webex recordings have been posted to the svn site. To view the webex presentations we also distribute the nbr2player.msi to install the viewer.

To access all of the 2012 and 2014 Trainings, some data, and some test cases I suggest you make a folder called COAWST_data, cd to that folder and use:

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data .

- To access the 2012 meeting use

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/training_23jul2012 .

To access all the 2014 meeting presentations and webex use:

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/training_25aug2014/ .

- To access only the 2014 meeting presentations use

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/training_25aug2014/presentations .

- To access only the 2014 meeting webex use

svn checkout

https://coawstmodel.sourcerepo.com/coawstmodel/data/training_25aug2014/webex .

To access all the 2016 meeting presentations and webex use:

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/training_15aug2016/ .

- To access only the 2016 meeting presentations use

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/training_15aug2016/presentations .

- To access only the 2016 meeting webex use

svn checkout

https://coawstmodel.sourcerepo.com/coawstmodel/data/training_15aug2016/webex .

We have some data files that are distributed to create tidal boundary conditions. That data is available using

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/tide .

- We will be adding some of the larger applications to be distributed using

svn checkout https://coawstmodel.sourcerepo.com/coawstmodel/data/apps .

16. List of References and Acknowledgements.

Warner, J.C., Armstrong, B., He, R., and Zambon, J.B., (2010). Development of a Coupled Ocean-Atmosphere-Wave-Sediment Transport (COAWST) modeling system: Ocean Modeling, v. 35, no. 3, p. 230-244.

Kumar, N., Voulgaris, G., and Warner, J.C. (2011). Implementation and modification of a three-dimensional radiation stress formulation for surf zone and rip-current applications, Coastal Engineering, 58, 1097-1117, doi:10.1016/j.coastaleng.2011.06.009.

Olabarrieta, M., J. C. Warner, and N. Kumar (2011), Wave-current interaction in Willapa Bay, J. Geophys. Res., 116, C12014, doi:10.1029/2011JC007387.

Olabarrieta, M., Warner, J.C., and Armstrong, B. (2012). “Ocean-atmosphere dynamics during Hurricane Ida and Nor'Ida: an atmosphere-ocean-wave coupled modeling system application.” Ocean Modelling, 43-44, pp 112-137.

Kumar, N., Voulgaris, G., Warner, J.C., and M., Olabarrieta (2012). Implementation of a vortex force formalism in the coupled ocean-atmosphere-wave-sediment transport (COAWST) modeling system for inner-shelf and surf-zone applications. Ocean Modeling 47, pp 65-95.

Nelson, J. and R. He, (2012), Effect of the Gulf Stream on winter extratropical cyclone outbreaks, Atmosphere Research Letters, doi: 10.1002/asl.400.

Renault, L., J. Chiggiato, J. C. Warner, M. Gomez, G. Vizoso, and J. Tintoré (2012), Coupled atmosphere-ocean-wave simulations of a storm event over the Gulf of Lion and Balearic Sea, J. Geophys. Res., 117, C09019, doi:10.1029/2012JC007924.

Benetazzo, A., Carniel, S., Sclavo, M., and Bergamasco, A. (2013). Wave-current interaction: effect on the wave field in a semi-enclosed basin. Ocean Modeling, 70, 152-165.

Nelson, J. He, R., and Warner, J.C., Bane, J. (2014). Air-Sea Interactions during Strong Winter Extratropical Storms, Ocean Dynamics. doi:10.1007/s10236-014-0745-2.

Grifoll, M., A. L. Aretxabaleta, J. L. Pelegrı, M. Espino, J. C. Warner, and A. Sanchez-Arcilla (2014), Seasonal circulation over the Catalan inner-shelf (northwest Mediterranean Sea), J. Geophys. Res. Oceans, doi:10.1002/2014JC010187.

Zambon, J.B., He, R., and Warner, J.C. (2014).  Investigation of Hurricane Ivan using the Coupled Ocean-Atmosphere-Wave-Sediment Transport (COAWST) Model, Ocean Dynamics, DOI 10.1007/s10236-014-0777-7.

Grifoll, M., Gracia, V., Aretxabaleta, A., Guillen, J., Espino, M., Warner, J.C. (2014). Formation of fine sediment deposit from a flash-flood river in the Mediterranean Sea. Journal of Geophysical Research, Oceans, 119, 5837-5853, doi:10.1002/2014JC010187.

Rong, Z., Hetland, R., Zhang, W., and Zhang, X. (2014). Current-wave interaction in the Mississippi-Atchafalaya river plume on the Texas-Louisiana shelf. Ocean Modelling, 84, 67-83.

Zambon, J.B., He, R., and Warner, J.C. (2014). Tropical to Extratropical:  Marine Environmental Changes Associated with Superstorm Sandy Prior to its Landfall, Geophysical Research Letters, 41, doi:10.1002/2014GL061357.

M.J. Lewis, S.P. Neill, M.R. Hashemi, M. Reza (2014). Realistic wave conditions and their influence on quantifying the tidal stream energy resource, Applied Energy, Vol 136, pp 495-508, ISSN 0306-2619, http://dx.doi.org/10.1016/j.apenergy.2014.09.061.

Spydell, M. S., F. Feddersen, M. Olabarrieta, J. Chen, R. T. Guza, B. Raubenheimer, and S. Elgar (2015), Observed and modeled drifters at a tidal inlet, J. Geophys. Res. Oceans, 120, 4825–4844, doi:10.1002/2014JC010541.

M. Reza Hashemi, Simon P. Neill & Alan G. Davies (2015) A coupled tidewave model for the NW European shelf seas, Geophysical & Astrophysical Fluid Dynamics, 109:3, 234-253, DOI: 10.1080/03091929.2014.944909.

Danqin Ren, Jiantin Du, Feng Hua, Yongzeng Yang, Lei Han (2016). Analysis of different atmospheric physical parameterizations in COAWST modeling system for the Tropical Storm Nock-ten application. Natural Hazards. 1-18, 10.1007/s11069-016-2225-0.    http://www.tandfonline.com/doi/abs/10.1080/03091929.2014.944909

Carniel, S., Benetazzo, A., Bonaldo, D., Falcieri, F.M., Miglietta, M.M., Ricchi, A., and Sclavo, M. (2016). Scratching beneath the surface while coupling atmosphere, ocean and waves: Analysis of a dense water formation event. Ocean Modeling, 101, 101-112.

Ricchi, A., Miglietta, M., Falco, P., Benetazzo, A., Bonaldo, D., Bergamasco, A., Sclavo, M., Carniel, S. (2016). On the use of a coupled ocean–atmosphere-wave model during an extreme Cold Air Outbreak over the Adriatic Sea. Atmospheric Research. 172. 10.1016/j.atmosres.2015.12.023.

Beudin, A., Kalra, T. S., Ganju, N. K., and Warner, J.C. (2016). Development of a coupled wave-flow-vegetation interaction module. Computers and Geosciences, http://dx.doi.org/10.1016/j.cageo.2016.12.010.

Warner, J.C. (2016). “Advanced Model Training for Predicting Coastal Storm Impacts.”

http://soundwaves.usgs.gov/2016/09/meetings.html

Bruneau, N., and Toumi, R. (2016). A fully-coupled atmosphere-ocean-wave model of the Caspian Sea. Ocean Modeling, 107, 97-111.

Feddersen, F., M. Olabarrieta, R. T. Guza, D. Winters, B. Raubenheimer, and S. Elgar (2016), Observations and modeling of a tidal inlet dye tracer plume, J. Geophys. Res. Oceans, 121, 7819–7844, doi:10.1002/2016JC011922.

Safak, I., List, J.H., and Warner, J.C. (2016). Barrier island breach evolution: Alongshore transport and bay-ocean pressure gradient interactions. J. Geophys. Res. Oceans, 121,doi:10.1002/2016JC012029.

Safak, I., List, J.H., Warner, J.C., and Kumar, N. (2017.) Observations and 3D hydrodynamics-based modeling of decadal-scale shoreline change along the Outer Banks, North Carolina, Coastal Engineering, 120, 78-92.

Warner, J.C., Schwab, W.C., List, J.H., Safak, I., Liste, M., and Baldwin, W. (2017).

Inner-shelf ocean dynamics and seafloor morphologic changes during Hurricane Sandy, Continental Shelf Research, 138, 1-18.

Beudin, A., Ganju, N. K., Defne, Z. and Aretxabaleta, A. L. (2017), Physical response of a back-barrier estuary to a post-tropical cyclone. J. Geophys. Res. Oceans. doi:10.1002/2016JC012344

Akan, Ç., S. Moghimi, H. T. Özkan-Haller, J. Osborne, and A. Kurapov (2017), On the dynamics of the Mouth of the Columbia River: Results from a three-dimensional fully coupled wave-current interaction model, J. Geophys. Res. Oceans, 122, 5218–5236, doi:10.1002/2016JC012307.

Beudin, A., N. K. Ganju, Z. Defne, and A. L. Aretxabaleta (2017), Physical response of a back-barrier estuary to a post-tropical cyclone, J. Geophys. Res. Oceans, 122, 5888–5904, doi:10.1002/2016JC012344.

Ricchi, A.; Miglietta, M.M.; Barbariol, F.; Benetazzo, A.; Bergamasco, A.; Bonaldo, D.; Cassardo, C.; Falcieri, F.M.; Modugno, G.; Russo, A.; Sclavo, M.; Carniel, S. Sensitivity of a Mediterranean Tropical-Like Cyclone to Different Model Configurations and Coupling Strategies. Atmosphere 2017, 8, 92.

Zhao, X. and Chan, J. C. L. (2017), Effect of the Initial Vortex Size on Intensity Change in the WRF-ROMS Coupled Model. J. Geophys. Res. Oceans. Accepted Author Manuscript. doi:10.1002/2017JC013283.

Kukulka, T., Jenkins, R. L., Kirby, J. T., Shi, F., & Scarborough, R. W. (2017). Surface wave dynamics in Delaware Bay and its adjacent coastal shelf. Journal of Geophysical Research: Oceans, 122, 8683–8706. https://doi.org/10.1002/2017JC013370.

Safak, I., List, J. H., Warner, J. C., & Schwab, W. C. (2017). Persistent shoreline shape induced from offshore geologic framework: Effects of shoreface connected ridges. Journal of Geophysical Research: Oceans, 122, 8721–8738. https://doi.org/10.1002/2017JC012808.

Wandres, M., Wijeratne, E. M. S., Cosoli, S., & Pattiaratchi, C. (2017). The effect of the Leeuwin Current on offshore surface gravity waves in southwest western Australia. Journal of Geophysical Research: Oceans, 122, 9047–9067. https://doi.org/10.1002/2017JC013006.

Çiğdem Akan, James C. McWilliams, Saeed Moghimi, H. Tuba Özkan-Haller. (2018)  Frontal dynamics at the edge of the Columbia River plume, Ocean Modelling, Volume 122, pp 1-12, ISSN 1463-5003, https://doi.org/10.1016/j.ocemod.2017.12.001.

J.M.R. Alves, A. Peliz, R.M.A. Caldeira, P.M.A. Miranda. (2018). Atmosphere-ocean feedbacks in a coastal upwelling system, Ocean Modelling, 123, pp 55-65,

https://doi.org/10.1016/j.ocemod.2018.01.004.

Zhengchen Zang, Z. George Xue, Shaowu Bao, Qin Chen, Nan D. Walker, Alaric S. Haag, Qian Ge, Zhigang Yao, (2018). Numerical study of sediment dynamics during hurricane Gustav, Ocean Modelling, Volume 126, p 29-42.

https://doi.org/10.1016/j.ocemod.2018.04.002.

Dufois, F., Lowe, R., Rayson, M., & Branson, P. (2018). A numerical study of tropical cyclone‐induced sediment dynamics on the Australian North West Shelf. Journal of Geophysical Research: Oceans, 123. https://doi.org/10.1029/2018JC013939.

Wenping Gong, Zhongyuan Lin, Yunzhen Chen, Zhaoyun Chen, and Heng Zhang, (2018). Effect of winds and waves on salt intrusion in the Pearl River estuary, Ocean Science, 14, 139-159.

- Forecast systems:

- Hydro and Agro Informatics Institute

http://www.thaiwater.net/v3/wrfroms/rain_forecast_pre/tab1/image1

- USGS Woods Hole

http://woodshole.er.usgs.gov/project-pages/cccp/public/COAWST.htm

- North Carolina State University

http://omgsrv1.meas.ncsu.edu:8080/ocean-circulation-useast2/

Madeira Island Oceanic Observatory forecasting

https://oom.arditi.pt/mission072018/

- Acknowledgements

We thank all the modeling and tool systems for open access to their codes, and to the Integration and Application Network (ian.umces.edu/symbols), University of Maryland Center for Environmental Science, for the courtesy use of their symbols and diagrams.