# Incomplete cocrystalization of ibuprofen and nicotinamide and its interplay with formation of ibuprofen dimer and/or nicotinamide dimer: a thermodynamic analysis based on DFT data

Milad Asgarpour Khansary1, Gavin Walker2, Saeed Shirazian3

1-Confirm Smart Manufacturing, University of Limerick, Limerick, Ireland; 2-Synthesis & Solid-State Pharmaceutical Centre (SSPC), Bernal Institute, University of Limerick, Limerick, Ireland; 3-Department of Chemical Sciences, University of Limerick, Limerick, Ireland. * correspondence: Milad.Asgarpour@ul.ie

Abstract

Cocrystallization of ibuprofen and nicotinamide in hot melt extrusion process has been subject of many studies addressing low ibuprofen bioavailability. However, it is observed that the process of cocrystal formation of ibuprofen and nicotinamide might be incomplete. We hypothesized that formation of dimers of ibuprofen–ibuprofen or dimers nicotinamide– nicotinamide might be the cause of such poor cocrystalization process by altering the phase behaviour of the mixture. This paper addresses the molecular thermodynamics of mixtures of ibuprofen and nicotinamide, with special focus on the possibility of formation of these dimers and their corresponding interplay with mixture phase behaviour. For this purpose, density functional theory calculations are used to calculate electron donor-acceptor sites on each molecule and accordingly possible dimers of each molecule are analysed. The free energies and phase diagram are determined for (1) when a dimer is formed or (2) no dimer is formed, over a wide operating temperature range of 273.15 K – 390 K. The binding and solvation energies are calculated to identify/rank dimers. Calculations showed that formation of dimers requires an energy input which can be accessible noting to the external heating in hot melt extrusion process. The calculated solvation energies of the dimers suggest that addition of liquid binder (water) can mitigate the risk of dimer formations. Addition of proper binder/excipient is an easy route to compensate such dimer formation and to engineer ibuprofen and nicotinamide cocrystallization behaviour.

Keywords: pharmaceutical; molecular interactions; dimer formation; phase behaviour; twin-screw granulator

# Introduction

The common use of ibuprofen (IBF) is to treat pain associated with rheumatoid arthritis and osteoarthritis, and fever [1], mostly through oral delivery route. Quick availability of IBF in the blood is important in order to achieve fast relief [2]. However, the IBF has a poor solubility in an aqueous media, particularly in the acidic environment of the stomach [3, 4]. The solubility of ibuprofen and its dissolution rate affect IBF absorption rate into the blood [1]. In order to enhance the IBF bioavailability [5], nicotinamide (NCTA) has been used as a coformer in hot melt extrusion (HME) process to produce IBF–NCTA cocrystals. However, an undesirable incomplete cocrystal formation of IBF–NCTA has been observed [6, 7]. To mitigate such incomplete cocrystal formation, addition of an inert excipient with low melting point temperature is sought in literature [6, 7].

We noted that despite such attempts, the underlying cause of incomplete IBF–NCTA cocrystal formation has not been explored before. The molecular chemical thermodynamics and phase equilibria play an important role in the phase behaver of mixtures [8]. Improper of altered interaction of molecules may lead to different phase behaviour and mixture properties [9]. The possible of interaction/contact of same molecules may decrease the desirable contact between two molecules toward cocrystal formation. As in the hot melt extrusion process, the IBF and NCTA mixture is exposed to external heating [10, 11], in order to study the molecular interactions and contacts, a nonequilibrium thermodynamic analysis is needed [12-14]. Indeed, to get an insight on such interactions and synthesize useful information, an atomistic level study of mixture behaviour is of great interest [15-17].

This analysis is important as we have gone through literature and noted that IBF has a melting temperature of 353.15 K [18, 19] and NCTA has a melting temperature of 398.5 K [20, 21]. Literature suggested that IBF may form dimers in temperatures higher than its melting temperature [18]. Noting to the common operating temperature range of HME process i.e. 273.15 K – 390 K, it readily concludes that such interactions might be possible and present. The temperature of 390 K is considered as upper temperature boundary as in hot melt extrusion process, temperatures above coformer melting temperature are usually avoided.

Therefore, in this work, we hypothesized that formation of IBF dimers and/or NCTA dimers are possible within HME process. We analysed dimers contribution to incomplete cocrystal formation of IBF-NCTA. IBF dimers may consume IBF toward dimer formation and lowering the availability of IBF toward its interaction with NCTA and formation of IBF-NCTA cocrystals. We determined free energies and phase diagram for when either a dimer is formed or not. The binding energies for all possible dimers are calculated to identify/rank the most possible form of dimers as detailed in following paragraphs.

# Methods

The molecular structure of IBF and NCTA were retrieved from National Institute of Standards and Technology (NIST) Reference Database Number 69 (https://webbook.nist.gov/chemistry). The molecules were optimized using the DMol3 DFT package. We employed the Generalized Gradient Approximations (GGA) with Perdew-Becke-Ernzerhof (PBE) functional [22] including implicit solvent [23] as described by COnductor-like Screening MOdel (COSMO) [24]. The choice of PBE in current DFT calculations is based on (1) our analysis of computational works reported in literature investigating properties of IBF [25-29] and NCTA [28-32] and (2) recommendation in Ref. [33]. To control the convergence behaviour for an enhanced self-consistent field calculations [34, 35], thermal smearing [36] was also applied including double numerical basis including d-polarization function [37] level of theory. The convergence tolerances are energy: 2.0×10-5 kcal/mol, force: 10-3 kcal/mol/Å, max iterations: 104, displacement: 10-5 Å.

The surface donor/acceptor sites on both IBF and NCTA molecules were identified based on the surface site charge densities [38]  by employing Hirshfeld partitioning scheme [39] as implemented in DMol3 DFT package. All possible dimers of IBF or NCTA were considered by putting molecules in a close contact (shorter than the corresponding Van der Waals distance). All model dimer structures were optimized employing GGA-PBE using the DMol3 DFT package. The thermodynamic properties (solvation, enthalpy, entropy, free energy and heat capacity) were extracted following Hirano’s formulation [40] as implemented in DMol3 DFT package. The binding energies (Eb) are calculated as Eb = Eii – (Ei + Ei), where Ei is energy of relaxed molecule i and Eii is the dimer.

# Results and discussion

## Molecular surface properties

In Fig. 1, the relaxed structure of ibuprofen and nicotinamide are illustrated. The calculated solvation energy (reference solvent: water) for IBF is -60.18 kJ/mol and -66.96 kJ/mol for NCTA, meaning that NCTA have much higher solubility than IBF which is in agree with experimental observations [41, 42], and the spirit of NCTA as the coformer [43]. IBF is practically insoluble in water (very low solubility ~ 0.021 mg/ml at 20 °C) while NCTA showed excellent solubility in water (~ 24 mg/ml).

| NCTA | IBF |
| --- | --- |
| Fig. 1. Relaxed structure of ibuprofen (IBF) and nicotinamide (NCTA) calculated using DFT. |  |

The calculated surface charge densities of IBF and NCTA are reported in Fig. 2. For both IBF and NCTA, significant peaks have been observed within the nonpolar region i.e. -0.01<σ<0.01 e/Å2. This means that most of the surface of these molecules has nonpolar characteristics. In NCTA, the peak around σ = -0.014 e/Å2 is due to the two polarized hydrogen atoms bonded to nitrogen atom. The peak around σ = -0.006 e/Å2 is due to hydrogens available in ring. The peak around σ = -0.003 e/Å2 is due carbon atom attached to oxygen and nitrogen. The peak around σ = 0.002 e/Å2 is due to exposed surfaces of carbon atoms. The peak around σ = 0.009 e/Å2 is due to lone electron on nitrogen attached to nitrogen. The peak around σ = 0.014 e/Å2 is due to the lone electrons on oxygen in C=O group. The peak around σ = -0.018 e/Å2 is due to hydrogen in CH3 group adjacent to carboxyl group. The wide peak around -0.0155<σ<-0.0125 e/Å2 is due to hydrogen in hydroxyl (OH) group. The peak around σ = -0.01 e/Å2 is due to hydrogen in CH group adjacent to carboxyl group. The peaks within -0.0035<σ<0.001 e/Å2 are due to hydrogens in CH2 (around σ = -0.0025 e/Å2), CH3 (around σ = -0.0015 e/Å2) and CH in ring (around σ = -0.0035 e/Å2). The peak around σ = 0.0012 e/Å2 is due exposed surfaces of carbon atoms in ring. The peak σ = 0.006 e/Å2 is due to lone electron in hydroxyl group (OH). The two peaks at σ = 0.009 e/Å2 and σ = -0.0125 e/Å2 are due to lone electrons on oxygen in C=O. Thus, in a NCTA–IBF contact, it can be assumed that due to peaks match, the hydrogen in OH group as well as hydrogens in CH3 group adjacent to carboxyl group of IBF molecule would interact with lone-electron on nitrogen in ring as well as lone-electrons in oxygen of C=O group in NCTA. Repelling interactions can be assumed between IBF and NCTA due to repulsive interactions among lone-electron on nitrogen in ring as well as lone-electrons in oxygen of C=O group in NCTA and lone-electrons in oxygen of C=O group in IBF. Also, repelling interactions can be assumed between IBF and NCTA due to repulsive interactions among hydrogen in OH group as well as hydrogens in CH3 group adjacent to carboxyl group of IBF and hydrogens attached to NH2 group in NCTA. Weak π–π attractive interactions between IBF and NCTA can be assumed due to peak match where exposed surface of carbon atoms in IBF tends to interact with carbon atoms in ring of NCTA. Also, IBF molecules are expected to establish weak π–π interactions with each where exposed surface of carbon atoms in one IBF molecule interacts with CH, CH2 and CH3 groups in another IBF molecule. It can be assumed that lone electron in OH group of IBF would establish some nonpolar interactions with carbon atoms attached to oxygen and nitrogen in NCTA as well as hydrogen atoms in NCTA. It can be expected that lone electrons in oxygen group of C=O of NCTA involve in attractive interactions with hydrogen attached to nitrogen of NH2 group in another NCTA molecule. Also, the lone electron on oxygen of C=O in one IBF molecule may attract hydrogen in OH group as well as hydrogen in CH3 group adjacent to carboxyl group in another IBF molecule.

Fig. 2. Sigma charge densities of ibuprofen (IBF) and nicotinamide (NCTA) calculated using DFT/cosmo.

The surface charges in ibuprofen and nicotinamide calculated using DFT/cosmo considering solvation effect (reference solvent: water) are also shown in Fig. 3 where a blue colour indicates an electrophobe site (electron donor) and a red colour shows electrophile site (electron acceptor). In ibuprofen, the oxygen in C=O of carboxyl group shows significant electrophobic characteristics (electron donor). Also, the oxygen in OH group shows electrophobic characteristics (electron donor), but the first is much stronger. The most electrophilic sites (electron acceptor) are the adjacent CH3 groups, mainly due to electron polarization established because of the presence of carboxyl group. One may assume that there might be possibility of π–π interactions between ibuprofen and ibuprofen due to the carbon rings. Such π–π interactions between ibuprofen and nicotinamide would be rare. This is because such π–π interactions are less competitive when compared to the interactions that the nitrogen of central ring in nicotinamide tends to engage. In nicotinamide, the oxygen in C=O as well as nitrogen in the ring showed electrophobic characteristics (electron donor). While electrophobic characteristics (electron donor) are observed for nitrogen in ring, due to low electron availability in nitrogen as a result of its engagement with adjacent carbon atoms, it is assumed that electrophilic attacks (from an electron acceptor site) will mainly aim the oxygen in C=O group. The two hydrogens attached to terminal nitrogen as well as the hydrogen attached to the ring and adjacent to nitrogen in ring are the most susceptive sites for a nucleophilic attack (from an electron donor site).

| NTCA | IBF-view 1 |
| --- | --- |
|  | IBF-view 2 |
| Fig. 3. Surface charges in ibuprofen (IBF) and nicotinamide (NCTA) calculated using DFT/cosmo considering solvation effect (reference solvent: water). For IBF, two plots are provided for conciseness. |  |

The electrostatic potentials of ibuprofen and nicotinamide are shown in Fig. 4, at an isosurface of 0.05. Here, a blue colour shows an electrophobic site (electron donor) while a red colour shows an electrophilic site (electron acceptor). It is worthy to mention that a negative electrostatic potential indicates sites for attraction of the proton by the concentrated electron density in the molecules while a positive electrostatic potential indicates a repulsion of the proton by the atomic nuclei due to low electron density (as it causes an incompletely shielded nuclear charge). For ibuprofen, it is evidence that a proton or positively charge surface group would approach it through the oxygen in C=O as the most of IBF’s surface is repelling positively charged particles. On the other hand, such a large proton repelling surface, itself, shows its potential attraction to surface groups with negative electrostatic potentials. However, as noted in solvation properties analysis, all the IBF molecular surface won't be available in intra/inter–molecular interactions. The higher positive electrostatic potential around the hydrogen in OH unit of carboxyl group is in agree with our previously noted results of solvation properties indicating significant role of this hydrogen when IBF is exposed to nucleophilic attacks. Also, for nicotinamide, two negative electrostatic potential regions have been observed around (1) the oxygen of C=O and (2) the nitrogen in the ring. This means that NCTA can be exposed to electrophilic attacks within both these regions. However, it is obvious that less negative electrostatic potential is available around the nitrogen in ring while for the oxygen of C=O, a significant population of negative electrostatic potential can be seen. This implies that the oxygen in C=O is the main role–player when NCTA is subject to electrophilic attacks and would establish much stronger interactions.

| NCTA | IBF |
| --- | --- |
| Fig. 4. The electrostatic potential in ibuprofen (IBF) and nicotinamide (NCTA) calculated using DFT. Isosurface at 0.05. |  |

## Details of dimers structure

For all surface sites identified in previous section, each pair of two molecules i.e. IBF–IBF and NCTA–NCTA were considered. In Fig. 5.A, two IBF molecules (IBF-d1) contact each other at their carboxyl groups though hydrogen in OH and oxygen in C=O and establish an electrostatic interaction of 1.733 Å in length and a C–O–H angel of 118.1°. A perfect lone electron sharing from oxygen is observed according to electrostatic potential map (Fig. 5.B). This distance is 66.1% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting strong intramolecular hydrogen bonding. It also suggests possibility of IBF dimer formation. The C=O experienced a 1.5% elongation from 1.21 Å in isolated IBF molecule to 1.23 Å. The OH experienced a 2.5% elongation from 0.97 Å in isolated IBF molecule to 1.00 Å.

A B

Fig. 5. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in IBF-d1

In Fig. 6.A, two IBF molecules (IBF-d2) contact through oxygen in C=O of carboxyl group and hydrogens in CH3 group adjacent to carboxyl group over a distance of 3.150 Å and 108.5° C–O–H angel, where no practical electron sharing is observed (Fig. 6.B). This distance is 120.2% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45] and no intramolecular hydrogen bonding can be realized. The CO experienced a 0.6% elongation from 1.21 Å in isolated IBF molecule to 1.22 Å. The CH experienced a 0.2% length–reduction from 1.098 Å in isolated IBF to 1.095 Å.

A B

Fig. 6. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in IBF-d2

In Fig. 7.A, two IBF molecules (IBF-d3)  were initially positioned to bring into contact: (1) the oxygen in C=O of carboxyl group and hydrogens in CH3 group adjacent to carboxyl group and (2) oxygen in OH of carboxyl group and nearest hydrogen in main ring. The relaxed structure established a distance of 2.486 Å with C–O–H angle of 107.7° with no practical electron sharing from lone electron on oxygen (Fig. 7.B). This distance is 94.88% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting very weak intermolecular hydrogen bonding. The CH experienced a length– reduction of 0.01% from 1.0962 Å in isolated molecule to 1.0962 Å. The CO experienced an elongation of 0.2% from 1.21 Å in isolated molecule to 1.22 Å.

A B

Fig. 7. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in IBF-d3

In Fig. 8.A, two NCTA molecules (NCTA-d1) are in contact through (1) oxygen in C=O of carboxyl group and hydrogen in NH2 as well as hydrogen attached to main ring located adjacent to double–bonded nitrogen and (2) nitrogen in main ring and hydrogen attached to main ring. The two molecules establish a distance of 2.607 Å with a C–H–N angle of 164.7° between hydrogen attached to main ring and nitrogen in main ring, a distance of 2.423 Å with a C–O–H angle of 135.1° between oxygen in C=O group and hydrogen attached to main ring adjacent to nitrogen, and a distance of 2.044 Å with C–O–H angle of 159.5°. No proper electron sharing can be observed in this contact (Fig. 8.B). The two C–O–H distances are 92.4% and 78.01% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting very weak and moderate intramolecular hydrogen bonding. The C–H–N distance is 98.3% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45], suggesting too weak intramolecular hydrogen bonding. The C=O experienced a 0.5% elongation from 1.24 Å in isolated NCTA molecule to 1.25 Å. The NH experienced a 0.8% elongation from 1.011 Å in isolated NCTA molecule to 1.011 Å. The single–bond CN in NCTA main ring experienced a 0.1% elongation from 1.343 Å in isolated molecule to 1.344 Å. The double–bond CN in NCTA main ring experienced a 0.1% elongation from 1.340 Å in isolated molecule to 1.342 Å.

A B

Fig. 8. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d1

In Fig. 9.A, a contamination of the two electrostatic interaction between nitrogen in NCTA main ring and the hydrogen atom adjacent to nitrogen in main ring of another NCTA can be observed (NCTA-d2). Distances of (1) 2.386 Å and C–H–N angle of 165.1° and (2) 2.425 Å and C–H–N angle of 163.1° were obtained, which are 90% and 91.5% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45], suggesting weak intramolecular hydrogen bonding (Fig. 9.B). The CH experienced a length–reduction of 0.2% from 1.092 Å in isolated molecule to 1.09 Å. The double–bond CN in ring experienced a 0.15% elongation from 1.34 Å in isolated molecule to 1.342 Å. The double–bond CN in ring experienced a 0.19% elongation from 1.343 Å in isolated molecule to 1.345 Å.

B A

Fig. 9. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d2

In Fig. 10.A, two NCTA molecules (NCTA-d3) are in contact through nitrogen in main ring and (1) hydrogen in NH2 group and (2) hydrogen attached to main ring adjacent to nitrogen at double bonded carbon–nitrogen. Distances of (1) 1.963 Å with N–H–N angle of 166.7°, (2) 2.910 Å with C–H–N of 123.03°, (3) 3.057 Å with C–H–N angle of 148.7° and (4) 3.502 Å with N–H–N angle of 158.6° were established. These are 74.07% (1), 109.8% (2), 115.35% (3) and 132.15% (4) of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45], where only the first suggests weak intramolecular hydrogen bonding (Fig. 10.B). The NH experienced a 0.17–2.1% elongation from 1.011 Å in isolated molecule to 1.013–1.03 Å. The CH experienced a length–reduction of 0.10–0.16% from 1.092 Å in isolated molecule to 1.090–1.091 Å. The single–bond CN in main ring experienced an elongation of 0.13–0.2% from 1.343 Å in isolated molecule to 1.345–1.346 Å. The double–bond CN in main ring experienced an elongation of 0.15–0.17% from 1.340 Å in isolated molecule to 1.342 Å.

B A

Fig. 10. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d3

In Fig. 11.A (NCTA-d4), the lone electron on nitrogen in main ring and the hydrogens in NH2 group are established distances of (1) 2.531 Å with a N–H–N angle of 105.6° and (2) 2.622 Å with a N–H–N angle of 99.8°. These are 95.5% and 98.9% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45], suggesting very weak hydrogen bonding interaction (Fig. 11.B). The NH experienced an elongation of 0.02% from 1.0115 Å in isolated molecule to 1.0118 Å and experienced a length–reduction of 0.05% from 1.0137 Å in isolated molecule to 1.0132 Å. The single–bond CN in main ring experienced an elongation of 0.07% from 1.343 Å from isolated molecule to 1.344 Å. The double–bond CN in main ring experienced a length–reduction of 0.02% from 1.3405 Å from isolated molecule to 1.3402 Å.

B A

Fig. 11. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d4

In Fig. 12.A (NCTA-d5), the lone electron on oxygen and the hydrogens in NH2 group have established distances of (1) 2.537 Å with a N–H–O angle of 100.1° and (2) 2.484 Å with a N–H–O angle of 103.5°, which are 95.7% and 93.7% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45]. This suggests very weak intramolecular hydrogen bonding with improper electron sharing from nitrogen atom (Fig. 12.B). The OH experienced an elongation of 0.29% from 1.243 Å in isolated molecule to 1.247 Å. The NH experienced elongation of 0.11–0.16% from 1.011–1.013 Å in isolated molecule to 1.013–1.014 Å.

A B

Fig. 12. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d5

In Fig. 13.A (NCTA-d6), partial electron sharing from lone electrons on oxygen in C=O groups with the hydrogen in NH2 group is established over a distance of (1) 1.823 Å with N–H–O angle of 118.1° and (2) 1.845 Å with N–H–O angle of 119.3°, which are 68.7% and 69.6% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45]. This suggests moderate intramolecular hydrogen bonding (Fig. 13.B). The NH experienced an elongation of 2.17–2.18% from 1.01 Å in isolated molecule to 1.03 Å. The C=O experienced an elongation of 1.12–1.13% from 1.24 Å in isolated molecule to 1.25 Å.

A B

Fig. 13. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d6

In Fig. 14.A (NCTA-d7), the hydrogen in NH on NCTA resides at a distance of 1.937 Å of oxygen in C=O with a N–H–O angle of 171.6°, which is 73.9% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting moderate intermolecular hydrogen bonding together with partial oxygen lone electron sharing (Fig. 14.B). The NH experiences an elongation of 0.9% from 1.01 Å in isolated molecule to 1.02 Å. The CO experiences an elongation of 0.5% from 1.24 Å in isolated molecule to 1.25 Å. The CH in main ring adjacent to double–bond nitrogen resides at a distance of 2.283 Å to oxygen in C=O with a C–H–O angle of 164.4°with impractical oxygen lone electron sharing. This distance is 87.1% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting a slight moderate intermolecular hydrogen bonding. The CH experiences a length–reduction of 0.1% from 1.092 Å in isolated molecule in 1.091 Å. The CH in main ring resides at a distance of 3.180 Å to nitrogen in main ring with a C–H–N angle of 157.4°, which is 120% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45], and no intermolecular hydrogen bonding can be realized in this case. The CH experiences a length–reduction of 0.02% from 1.0886 Å in isolated molecule to 1.0883 Å. The single–bond CN in main ring experiences an elongation of 0.13% from 1.343 Å in isolated molecule to 1.345 Å. The double–bond CN in main ring experiences an elongation of 0.21% from 1.34 Å in isolated molecule to 1.343 Å.

A B

Fig. 14. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d7

In Fig. 15.A (NCTA-d8), the oxygen in C=O group resides at a distance of 1.931 Å to hydrogen in NH2 group with a C–O–H angle of 132.4°, which is 73.7% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting slightly moderate intermolecular hydrogen bonding showing proper oxygen lone electron sharing (Fig. 15.B). The NH experiences an elongation of 1.3% from 1.01 Å in isolated molecule to 1.02 Å. The CO undergoes an elongation of 0.67% from 1.24 Å in isolated molecule to 1.25 Å. The nitrogen in NH2 group resides at a distance of 3.465 Å to hydrogen in NH2 group with a C–N–H angle of 55.7° which is 130.7% of the summation of van der Waals Radii for hydrogen and nitrogen [44, 45], suggesting no intermolecular hydrogen bonding in this case. The CN experiences a length–reduction of 0.19% from 1.35 Å in isolated molecule to 1.34 Å. The CO resides at a distance of 2.379 Å to hydrogen attached to main ring adjacent to double–bond nitrogen with a C–O–H angle of 148.9°, which is 90.8% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting a slightly moderate intermolecular hydrogen bonding. The CH experiences a length–reduction of 0.2% from 1.092 Å in isolated molecule to 1.09 Å.

A B

Fig. 15. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d8

In Fig. 16.A (NCTA-d9), oxygen in C=O resides at a distance of 2.432 Å to hydrogen attached to ring adjacent to nitrogen with a C–O–H angle of 127.4°, which is 98.2% of the summation of van der Waals Radii for hydrogen and oxygen [44, 45], suggesting very weak intermolecular hydrogen bonding (Fig. 16.B). The CH experiences an elongation of 0.05% from 1.092 Å in isolated molecule to 1.093 Å. The CO experience an elongation of 0.2% from 1.243 Å in isolated molecule to 1.247 Å.

A B

Fig. 16. Calculated surface electron density (A), electrostatic potential (B) for identified donor-acceptor interactions in NCTA-d9

## Thermodynamics of mixture

Calculated standard thermodynamic properties of IBF and NCTA including enthalpy, Gibbs free energy, entropy and heat capacity over a temperature range of 273.15 K – 390 K are shown in Fig. 17. Increasing temperature increases the molecular vibration intensities, the effect of which is obvious as reflected in Fig. 17 by decreasing Gibbs free energy and increasing entropy and enthalpy.

| Fig. 17. Calculated standard thermodynamic functions including enthalpy (H0), Gibbs free energy (G0), and entropy (S0) using vibrational analysis and statistical thermodynamics as well as heat capacity (Cp) over a wide temperature range of 273.15 K – 390 K for ibuprofen (IBF) and nicotinamide (NCTA). |  |
| --- | --- |

For IBF–NCTA mixture, the free energy as well as its first and second derivatives are reported in Fig. 18 for two scenarios: (1) IBF dimer is formed and (2) no IBF dimer is formed. The phase diagram (Fig. 19) enables us to study the thermodynamic behaviour of mixture [8, 9].

| a | d |
| --- | --- |
| Fig. 18. Free energy and its first and second derivatives for ibuprofen (IBF) and nicotinamide (NCTA) mixtures at various temperatures. (a) – (c) indicates free energy, its first and second derivatives when no IBF dimer exists, (d) – (f) indicates free energy and its first and second derivatives when IBF forms dimer |  |

It was found that, when IBF does not form dimers, the mixture has a lower critical solution temperature (LCST) of around 350 K as shown in Fig. 19. It is worthy to mention that, in this case, ibuprofen and nicotinamide mixture also have an upper critical solution temperature (UCST) of ≈ 215 K. When IBF forms dimers, the LCST drops to 314 K and UCST drops to 194 K. For temperatures between LCST and UCST, according to the thermodynamic properties reported in Fig. 18 and Fig. 19, it can be concluded that the mixture remains unchanged and shows no tendency in changing its thermodynamic behaviour [9] in absence of external fluctuations/excitations. Increasing mixture temperature to cross binodal, mixture either enter directly into unstable region though LCST point or it enters the metastable region i.e. the region between binodal and spinodal curves [8]. In metastable region, the second derivative of free energy is positive the mixture tends to remain unchanged and resists against small composition fluctuations [8, 9]. In this case, a phase separation cannot be expected unless some nucleus for nucleation appear.

T = 314.29 K T = 351.23 K

Fig. 19. Phase diagram of ibuprofen (IBF) and nicotinamide (NCTA) mixtures including when IBF forms dimers. Thin lines correspond to when IBF exists in form of dimers.

It should be worthy to get an estimate of the relative composition of such nucleus. For this, let’s assume showing largest and lowest roots of second derivative of free energy as A and B respectively, and largest and lowest roots of first derivative of free energy as C and D respectively, then for a mixture with a composition w where D < w < B, these nucleus, normally, should have a composition close to C and for a mixture with a composition w where A < w < C, these nucleus, normally, should have a composition close to D. It is obvious to reader that D and C are on binodal curve. At points A and B, the second derivative of free energy is zero and in region within these points, the second derivative of free energy is negative. This means that within this region, the mixture is thermodynamically unstable and undergoes spinodal decomposition into two new phases with compositions of A and B [8, 9]. This suggests that, according to thermodynamics – and ignoring kinetics and in absence of external excitations – if compositions of these new phases i.e. compositions at point A and B can get closer and nearly the same, the final product should be of uniform distribution of these pairs. This immediately reads that a tight phase boundary is more feasible than a wide one. For instance, when IBF dimer is formed, the binodal and spinodal boundaries are expanded and the difference between compositions A and B would be larger. This means that the final product would contain two phases with large difference in compositions, losing the uniformity. As the wide phase diagram of IBF dimer–formed is not of interest, then temperatures higher than 380 K should be avoided. A hypothetical pure thermodynamic based strategy to get uniform particles/phases requires heating of an equimolar mixture of IBF–NCTA until it crosses the plaint point – critical point where binodal and spinodal boundaries meet. Here, the mixture directly enters the unstable region. Temperature should be maintained slightly higher than LCST to allow the mixture to undergo phase separation and to form new phases and then gradually temperature should be decreased. Based on the phase diagram, two phases of ~ 0.42 – 0.56, NCTA mole fraction, can be achieved, which are relatively uniform in terms of composition.

For identified three possible dimers for ibuprofen and nine possible dimers for nicotinamide, the corresponding binding and solvation energies are given in Fig. 20. The IBF-d3, d2 and d1 in Fig. 20 are reflecting the possible mechanism of IBF dimer formation ranked by their binding energies [18, 46]. NCTA-d6 is an excellent example of possible NCTA dimer formation as noted elsewhere [47]. Formation of these dimers requires an energy input. In course of hot melt extrusion in twin-screw granulator, such energy can be provided to initiate these contacts through external heating. To mitigate the risk of such dimer formation, addition of liquid binder (water) can be considered as an efficient rout noting to the solvation energies of the dimers as reported in Fig. 20.

Fig. 20. Calculated binding and solvation energies corresponding to IBF and NCTA dimers

# Concluding remarks

We performed computational experiments based on density functional theory calculations for phase behaviour of ibuprofen and nicotinamide when there is possibility/risk of dimer formation. We identified three and nine possible dimers for ibuprofen and nicotinamide respectively, ranked according to their binding energies. We concluded that the required energy for formation of these dimers can be available to them due to the energy input within twin-screw granulator as external heating are sufficiently enough to initiate such dimer formations. It is concluded that the use of a liquid binder (water) is sufficient to mitigate the risk of dimers formation.

# Funding Acknowledgment

This project has received funding from the European Union's Horizon 2020 research and innovation programme under the Marie Sklodowska-Curie grant agreement No. 801165 with co-funding through Confirm smart manufacturing centre by its funding body, Science Foundation Ireland (SFI), grant no. 16/rc/3918.

# Data availability

The associated data of this paper can be found at: https://sites.google.com/view/makhansary, under Downloads page using tag ID = CoCryM.Thermo.

# Conflict of interest

There is no conflict of interest to declare.

References

1.	Janus, E., et al., Enhancement of ibuprofen solubility and skin permeation by conjugation with l-valine alkyl esters. RSC Advances, 2020. 10(13): p. 7570-7584.

2.	Laska, E.M., et al., The correlation between blood levels of ibuprofen and clinical analgesic response. Clinical Pharmacology & Therapeutics, 1986. 40(1): p. 1-7.

3.	Levis, K.A., M.E. Lane, and O.I. Corrigan, Effect of buffer media composition on the solubility and effective permeability coefficient of ibuprofen. International Journal of Pharmaceutics, 2003. 253(1): p. 49-59.

4.	Potthast, H., et al., Biowaiver Monographs for Immediate Release Solid Oral Dosage Forms: Ibuprofen**This paper reflects the scientific opinion of the authors and not the policies of regulating agencies. Journal of Pharmaceutical Sciences, 2005. 94(10): p. 2121-2131.

5.	Dhumal, R.S., et al., Cocrystalization and Simultaneous Agglomeration Using Hot Melt Extrusion. Pharmaceutical Research, 2010. 27(12): p. 2725-2733.

6.	Li, S., et al., Mechanochemical Synthesis of Pharmaceutical Cocrystal Suspensions via Hot Melt Extrusion: Feasibility Studies and Physicochemical Characterization. Molecular Pharmaceutics, 2016. 13(9): p. 3054-3068.

7.	Walsh, D., et al., Engineering of pharmaceutical cocrystals in an excipient matrix: Spray drying versus hot melt extrusion. International Journal of Pharmaceutics, 2018. 551(1): p. 241-256.

8.	Keshavarz, L., M.A. Khansary, and S. Shirazian, Phase diagram of ternary polymeric solutions containing nonsolvent/solvent/polymer: Theoretical calculation and experimental validation. Polymer, 2015. 73: p. 1-8.

9.	Ejraei, A., et al., Lower and upper critical solution temperatures of binary polymeric solutions. Fluid Phase Equilibria, 2016. 425: p. 465-484.

10.	Moseson, D.E., et al., Application and limitations of thermogravimetric analysis to delineate the hot melt extrusion chemical stability processing window. International Journal of Pharmaceutics, 2020. 590: p. 119916.

11.	Thakkar, R., et al., Systematic screening of pharmaceutical polymers for hot melt extrusion processing: a comprehensive review. International Journal of Pharmaceutics, 2020. 576: p. 118989.

12.	Khansary, M.A., A. Marjani, and S. Shirazian, On the search of rigorous thermo-kinetic model for wet phase inversion technique. Journal of Membrane Science, 2017. 538: p. 18-33.

13.	Arsalan, F., et al., Binary Mutual Diffusion Coefficients of Polymer/Solvent Systems Using Compressible Regular Solutions Theory and Free Volume Theory. Journal of Non-Equilibrium Thermodynamics, 2016. 41(3): p. 215-223.

14.	Asgarpour Khansary, M., et al., A priority supposition for estimation of time-dependent changes in thickness and weight of polymeric flat sheet membranes fabricated by the nonsolvent induced phase separation (NIPS) technique. Advances in Polymer Technology, 2018. 37(6): p. 1963-1969.

15.	Ghasemi, A., et al., Using quantum chemical modeling and calculations for evaluation of cellulose potential for estrogen micropollutants removal from water effluents. Chemosphere, 2017. 178: p. 411-423.

16.	Karezani, E., A. Hallajisani, and M. Asgarpour Khansary, A quantum mechanics/molecular mechanics (QM/MM) investigation on the mechanism of adsorptive removal of heavy metal ions by lignin: single and competitive ion adsorption. Cellulose, 2017. 24(8): p. 3131-3143.

17.	Asgarpour Khansary, M., M.A. Aroon, and S. Shirazian, Physical adsorption of CO2 in biomass at atmospheric pressure and ambient temperature. Environmental Chemistry Letters, 2020. 18(4): p. 1423-1431.

18.	Lerdkanchanaporn, S. and D. Dollimore, A thermal analysis study of Ibuprofen. Journal of thermal analysis, 1997. 49(2): p. 879-886.

19.	Yuliandra, Y., et al., Cocrystal of Ibuprofen-Nicotinamide: Solid-State Characterization and In Vivo Analgesic Activity Evaluation. Sci Pharm, 2018. 86(2).

20.	Ouyang, J., et al., Solubility Determination of Nicotinamide and Its Application for the Cocrystallization with Benzoic Acid. Journal of Chemical & Engineering Data, 2018. 63(11): p. 4157-4165.

21.	Hino, T., J.L. Ford, and M.W. Powell, Assessment of nicotinamide polymorphs by differential scanning calorimetry. Thermochimica Acta, 2001. 374(1): p. 85-92.

22.	Perdew, J.P., K. Burke, and M. Ernzerhof, Generalized Gradient Approximation Made Simple. Physical Review Letters, 1996. 77(18): p. 3865-3868.

23.	Zhou, Y., et al., Effect of a Hydrogen Bond on Molecular Probing Properties in the Solvent. J Phys Chem A, 2020. 124(3): p. 520-528.

24.	Klamt, A., COSMO-RS From Quantum Chemistry to Fluid Phase Thermodynamics and Drug Design. 2005: Elsevier

25.	Vueba, M.L., M.E. Pina, and L.A. Batista de Carvalho, Conformational stability of ibuprofen: assessed by DFT calculations and optical vibrational spectroscopy. J Pharm Sci, 2008. 97(2): p. 845-59.

26.	Jubert, A., et al., Vibrational and theoretical studies of non-steroidal anti-inflammatory drugs Ibuprofen [2-(4-isobutylphenyl)propionic acid]; Naproxen [6-methoxy-α-methyl-2-naphthalene acetic acid] and Tolmetin acids [1-methyl-5-(4-methylbenzoyl)-1H-pyrrole-2-acetic acid]. Journal of Molecular Structure, 2006. 783(1-3): p. 34-51.

27.	Rossi, B., et al., Vibrational properties of ibuprofen–cyclodextrin inclusion complexes investigated by Raman scattering and numerical simulation. Journal of Raman Spectroscopy, 2009. 40(4): p. 453-458.

28.	Soares, F.L. and R.L. Carneiro, Evaluation of analytical tools and multivariate methods for quantification of co-former crystals in ibuprofen-nicotinamide co-crystals. J Pharm Biomed Anal, 2014. 89: p. 166-75.

29.	Oberoi, L.M., K.S. Alexander, and A.T. Riga, Study of interaction between ibuprofen and nicotinamide using differential scanning calorimetry, spectroscopy, and microscopy and formulation of a fast-acting and possibly better ibuprofen suspension for osteoarthritis patients. J Pharm Sci, 2005. 94(1): p. 93-101.

30.	Ramalingam, S., et al., FT-IR and FT-Raman vibrational spectra and molecular structure investigation of nicotinamide: A combined experimental and theoretical study. Spectrochimica Acta Part A: Molecular and Biomolecular Spectroscopy, 2010. 75(5): p. 1552-1558.

31.	Jaworska, A., et al., Nicotinamide and trigonelline studied with surface-enhanced FT-Raman spectroscopy. Vibrational Spectroscopy, 2012. 63: p. 469-476.

32.	Sallum, L.F., et al., Optimization of SERS scattering by Ag-NPs-coated filter paper for quantification of nicotinamide in a cosmetic formulation. Talanta, 2014. 118: p. 353-8.

33.	Dal Corso, A., et al., Generalized-gradient approximations to density-functional theory: A comparative study for atoms and solids. Physical Review B, 1996. 53(3): p. 1180-1185.

34.	Rabuck, A.D. and G.E. Scuseria, Improving self-consistent field convergence by varying occupation numbers. The Journal of Chemical Physics, 1999. 110(2): p. 695-700.

35.	Pulay, P., Improved SCF convergence acceleration. Journal of Computational Chemistry, 1982. 3(4): p. 556-560.

36.	Leavens, C.R., Effect of thermal smearing on the electron-phonon spectral function obtained by inversion of normal metal tunneling data. Solid State Communications, 1985. 54(7): p. 625-628.

37.	Delley, B., An all‐electron numerical method for solving the local density functional for polyatomic molecules. The Journal of Chemical Physics, 1990. 92(1): p. 508-517.

38.	Brinck, T. and J.H. Stenlid, The Molecular Surface Property Approach: A Guide to Chemical Interactions in Chemistry, Medicine, and Material Science. Advanced Theory and Simulations, 2019. 2(1).

39.	Hirshfeld, F.L., Bonded-atom fragments for describing molecular charge densities. Theoretica chimica acta, 1977. 44(2): p. 129-138.

40.	Hirano, T., A note on thermochemistry, in MOPAC Manual, J.J.P. Stewart, Editor. 1993.

41.	Filippa, M.A. and E.I. Gasull, Ibuprofen solubility in pure organic solvents and aqueous mixtures of cosolvents: Interactions and thermodynamic parameters relating to the solvation process. Fluid Phase Equilibria, 2013. 354: p. 185-190.

42.	Wu, H., L. Dang, and H. Wei, Solid–Liquid Phase Equilibrium of Nicotinamide in Different Pure Solvents: Measurements and Thermodynamic Modeling. Industrial & Engineering Chemistry Research, 2014. 53(4): p. 1707-1711.

43.	Zhao, J., J. Yang, and Y. Xie, Improvement strategies for the oral bioavailability of poorly water-soluble flavonoids: An overview. International Journal of Pharmaceutics, 2019. 570: p. 118642.

44.	Chernyshov, I.Y., I.V. Ananyev, and E.A. Pidko, Revisiting van der Waals Radii: From Comprehensive Structural Analysis to Knowledge-Based Classification of Interatomic Contacts. Chemphyschem, 2020. 21(5): p. 370-376.

45.	Mantina, M., et al., Consistent van der Waals radii for the whole main group. J Phys Chem A, 2009. 113(19): p. 5806-12.

46.	Hédoux, A., et al., Raman spectroscopy of racemic ibuprofen: Evidence of molecular disorder in phase II. International Journal of Pharmaceutics, 2011. 421(1): p. 45-52.

47.	Li, S., et al., Mechanochemical Synthesis of Pharmaceutical Cocrystal Suspensions via Hot Melt Extrusion: Feasibility Studies and Physicochemical Characterization. Mol Pharm, 2016. 13(9): p. 3054-68.