# Flow Cytometry Data Reader and Plotting Example

Flow cytometry is a method for quantitating components or structural features of cells primarily by optical means. Although it makes measurements on one cell at a time, it can process thousands of cells in a few seconds. Since different cell types can be distinquished by quantitating structural features, flow cytometry can be used to count cells of different types in a mixture[1].

Importing data	1

Visualization	1

References	5

## Importing data

The standard output of a flow cytometry machine is an FCS file. This is somehow related to FACS (Fluorescence-Activated Cell Sorter) machines, which are made by Becton-Dickinson. These are binary files with a fairly complex format. The function fcsread can read what seems to be the most common flavour of FCS files. The output is an array with each column representing a different parameter. Some of these are measured on a linear scale, others are measured on a log scale. The second output is a structure giving information about the parameters.

[data,params] = fcsread('SampleFACS.fcs');

## Visualization

A histogram of one parameter is one way to look at the data.

hist(data(:,1),max(data(:,1))-min(data(:,1)));

title(params(1).LongName);

A more interesting way to look at the data is to create a scatter plot of two of the parameters. In this case the second paramter is measured on a log scale.

numDecades = params(2).Amplification(1);

theRange = params(2).Range;

semilogy(data(:,1),10.^(data(:,2)/(theRange/numDecades)),'+');

xlabel(params(1).Name); ylabel(params(2).Name);

An even better visualization is to look at the scatter plot coloured by density of the scatter. The function dscatter does this.

dscatter(data(:,1),10.^(data(:,2)/256),'log',true)

xlabel(params(1).LongName); ylabel(params(2).LongName);

A second call to dscatter adds contours.

hold on

dscatter(data(:,1),10.^(data(:,2)/256),'log',true,'plottype','contour')

hold off

## References

[1] http://www.bio.umass.edu/mcbfacs/intro.htm