Auger Hole data (August 21, 2021)

* depths estimated and rounded to nearest 5 cm interval, uncertainty due to minor sloughing of auger walls during excavation

Auger hole at 22.2H (trench grid), 50cm east of east wall

0-70 cm: sand, few pebbles

70-75 cm: rounded pebbles and sand (base pink on yellow flags)

75-115 cm: gray and brown mottled clay, stiff (base orange flags)

115-120 cm: dark brown organic rich soil (grass mat) at top of clean gray/blue sand

120-140 cm: clean gray/blue sand (base teal flags)

140-170 cm: dark brown clay with abundant woody organic debris (base red/yellow stripe flags)

170 cm: subtle change to gray fine grained sediment

185 cm: more gray fine grained sediment with minor root/wood; one cm diam round pebble observed in gray silt/clay

180-190 cm: gray silt/clay

195-205 cm: gray silt/clay

205-230 cm: gray silt/clay, mottled with orangish brown (brecciated?)

230-260 cm: gray silt/clay, moderate orange mottling

256 cm: same

270-278 cm: mottling is the same in gray silt/clay

300 cm: 316 mostly the same

320 cm: brown with grey mottled

327 cm: brown

332 cm: more brown very muddy

350 cm: maybe slightly sandier

360- 365 cm: transition to upper gravel with more pebbles (base blue flags)

367 cm: refusal in sand and gravel

Bottom 367cm

Second hole at 30.0H, 55 cm east of east trench wall side of trench 55 cm (going through thicker sand)

0-62 cm: sand, minor gravel

62- 74 cm: increase in cobbles

74-130 cm: sand, minor gravel (base pink on yellow flags?)

130 cm: root mat (2-4 cm thick)

135 cm: go back into sand (base teal flags?)

145 cm: blue grey sand

148 cm: blue grey sand

150-155 cm: mud with abundant wood debris

175 cm: gray siltstone with blobs of brown weathered material

190 cm: silt w wood chunk (wood could be root or fragment from mud with abundant wood debris unit that fell down from side wall of auger hole?)

210 cm: gray siltstone, stiff

Bottom 210 cm: no need to continue augering

Auger holes on trench floor (August 23, 2021)

Hole in floor at 15H (trench grid)

Top of hole is at -0.10V (10 cm below 0V, trench grid)

0-35 cm: orangish brown siltstone

35 cm: observe first 3 cm pebble

38 cm: stiff brown siltstone

50 cm: stiff brown gray siltstone

68 cm: top of sand gravel

Bottom 75cm: refusal in sand/gravel

Hole on trench floor at 16H (trench grid)

Top of hole is at 0.55V (45cm below 1V, trench grid)

0-20 cm: brown siltstone

20 cm: siltstone slight increase in sand

45-55 cm: stiff siltstone; slow digging

65-85 cm: orangish brown siltstone; easier digging

85 cm: brown tan siltstone; stiff, harder digging (top of fat varve section?)

100 cm: brown siltstone, stiff

115 cm: siltstone, minor increase in sand, rare pebble

128 cm: siltstone, stiff

140-150 cm: siltstone, slight increase in sand

157 -172 cm: sand gravel

Bottom  172 cm