# Automation of Airport Luggage Check in System

Ashmita Choudhary

Department of Mathematics, University of Sciences,

Chandigarh University, Punjab, India.

ashmita404@gmail.com

Himank Aggarwal

Department of Electronics and Communication Engineering,

Jaypee Institute of Information Technology, Noida-62, (U.P.), India. himankaggarwal22@gmail.com

Deval Verma

School of Computer Science, Engineering and Technology,

Bennett University, Greater Noida, India.

deval09msc@gmail.com

Abstract

The airplane is currently the fastest mode of transportation and the first choice for those who want to make a long journey. Even after take-off you can quickly reach your destination, but the pre-start process often takes a long time. As a rule, we have problems with baggage weight measurement at check-in at the airport as the airlines have set a fixed weight limit for our luggage. Regardless of the terminal's modernity and impressive architecture, a fast, efficient and fluid baggage handling system still plays an important role in providing exceptional service. If we have an automatic baggage check-in system, all you have to do is plunder your ticket and your baggage will be registered under your name and the extra charge for your baggage will be automatically posted, if any. In this article, we introduce a system model designed to automate the flight baggage registration system by attempting to create a user-friendly, cost-effective, and efficient self-checking system.

Keywords:  Arduino Uno Board, LCD, RFID, Wheat Stone Bridge, UART.

1. Introduction

Aviation is not what it used to be. Far away are the wonder and the excitement of yesterday and in its place is the desire for speed, simplicity and freedom of worry. Not only the journey itself, but also the entire check-in process at the airport must be made more efficient and user-friendly in today's world of automation. This begins with the registration process, which is usually the passenger's first interface with the terminal. Passengers today expect a form of self-registration, but it must be easy to use, even for beginners or those who fly once a year. In most new terminals, the ideal solution is to go to the kiosk, register, get a boarding pass, pick up and attach your luggage, and then weigh, scan, and send your baggage. Now there are many ways to register instead of waiting in long queues. One of these self-registration methods is online, in which you can definitely skip the queue, but what is the benefit of checking online for a flight, when you need to check baggage at the airport. This requires a system whereby luggage can be automatically measured and recorded without delay.

2. Literature Review

In [1] the article is devoted to the development of the information system for the airport activity automation. This system will be useful in an airline ticket choice. For detection and identification purpose, every time only one baggage is placed by the passengers in check-in self-service at the Airport [2]. This paper uses dual laser range finders, to know the number of baggage that is placed on the conveyor belt by scanning luggage. In [3] for check-in and check-out information updation a smart RF reader was developed. RF reader is required for computing and logging operations. In [4] A tag is attached with the luggage and each time it checked by the RFID reader. To track the airline database status its updation is necessary. In this system passenger checks the location of luggage. The central idea of [5] this work is to design a vending machine that can be installed at airport.

3. Methodology

We made a model for automatic baggage control. To measure the weight, we used the load cell and IC HX711. The load cell is a type of transducer that performs the function of converting power into electrical energy that can be measured. It works on the principle of the Wheatstone bridge as shown in Fig. 1. You can find the load cell in the center of each ladder or ladder. This type of converter is very accurate, providing the user with the necessary information that is difficult to obtain due to other business factors due to other technologies. This electrical output is amplified by the HX711 IC load sensor module [6]. Conversion from changed resistance value into electrical energy is done with the help of HX711 weighing module. The main purpose of HX711 is to convert resistance into electrical energy. Because of TTL232 there is a communication between HX711 module and host computer. This amplified signal is sent to the Arduino Uno board (Fig. 2). This card contains everything that is needed to support the microcontroller [7]. This is the brain of our project. All other modules and hardware components depend directly or indirectly on it [8]. We also use the RFID module. MFRC522 contain a RFID module [9]. It works only when UART interface sends command to make operation easy. In this Embedded MCU plays a major role by doing most of the work. Board also has a SPI interface that is MFRC’s SPI. MFRC is used easily with this interface. It is used to scan the ticket given to the passenger in whom he/she has her details. It has a fixed pin configuration and a code is given through Arduino to display the content on LCD.

Figure 1. Four Strain Gauges in a Wheat-Stone Bridge Formation

Figure 2. Arduino Uno Board

# 4. Results

The content we want to show is displayed on LCD (Fig. 3). In most of the devices LCD is used because it is a very simple module. There are 2 lines in LCD and we can display a total of 16 characters in each line. In LCD, there are two register, Command and Data. The control instructions which are going to display are saved in control register. To initialize, clean, adjust the cursor position, command statement is used in LCD. The data we want to display is saved in data register. What this model does so is scanning the passenger tickets into RFID and measuring the weight of the luggage by the load cell, the additional cost if the weight exceeds the threshold given by the airlines and displays it to the passenger via LCD. This automates the process of baggage registration. The prototype of assembled system is shown in Fig. 4.

Figure 3. Pin Diagram of LCD

Figure 4. Prototype of Assembled System

# 5. Conclusion

It can be concluded that in today’s world of automation, every aspect of live has been affected and travelling has been simplified and made more efficient with time. The process of luggage check-in is what this model aims to automate. This Project gives a design which has many benefits like low cost, portability, small size. Due to its affordable cost, the airlines can place this system at every counter for automatic luggage check-in which would turn out to be a lot more convenient for the passengers. Thus the fastest mode of transport can be made even more efficient and less time consuming than before. This system would be used for check-ins and due to its cost effectiveness, number of systems can be increased at the airport so there won’t be long queues and it will also tackle the weight measurement issue which is faced in online check-in system.

References

- V. Lytvyn and Z. Rybchak, "Design of Airport Service Automation System," Xth International Scientific and Technical Conference "Computer Sciences and Information Technologies" (CSIT), Lviv, Ukraine, 2015, pp. 195-197, 2015.

- G. Qingji, L. Taiwen and L. Qijun, "An Algorithm for Inspecting the Number of Self Check-In Airline Luggage Based on Hierarchical Clustering," 2014 Sixth International Conference on Measuring Technology and Mechatronics Automation, Zhangjiajie, China, 2014, pp. 71-74, 2014.

- N. Koteswaramma, P. A. H. Vardhini, K. Divya, “FPGA based Luggage Tracking System in Airport”,  International Journal of Innovative Research in Science and Engineering, vol. 3, no. 1, pp. 430-435, 2017.

- A. Kavianpour, R. Monterrosa, R. Badran, “Automated Luggage Tracking System”, American Society for Engineering Education, pp. 1-10, 2010.

- G. Verma, A.Papreja, S. Shekhar, S. Maheshwari, S. K. Virdi. “Low Power Implementation of FSM based Vending Machine on FGPA”, 3rd International Conference on Computing for Sustainable Global Development (INDIACom), IEEE, pp. 2054-2058, 2016.

- Arduino Project Hub, projecthub.arduino.cc. Available at: https://create.arduino.cc/projecthub/mitov/arduino-and-visuino-measure-weight-with-hx711-and-load-cell-76de80 (Accessed: 17 June 2023).

- Team, T.A., Liquid Crystal Displays (LCD) with Arduino, Arduino Documentation. Available at: https://www.arduino.cc/en/Tutorial/HelloWorld (Accessed: 17 June 2023).

- Arduino Project Hub, projecthub.arduino.cc. Available at: https://create.arduino.cc/projecthub/team- arduinotronics/arduino-scale-b821ae (Accessed: 17 June 2023).

- Arduino Project Hub, projecthub.arduino.cc. Available at: https://create.arduino.cc/projecthub/Aritro/security-access-using-rfid-reader-f7c746 (Accessed: 17 June 2023).

Authors

Ashmita Choudhary received her Bachelors degree from Deenbandhu Chottu Ram University with distinction. She is pursuing her masters in Data Science from Chandigarh University.

Dr. Deval Verma is currently working as Assistant Professor in School of Computer Science at Bennett University (Times Group), Greater Noida. Her current research focuses on Digital Watermarking, Palmprint Matching and Optical Character Recognition (OCR). She is an active researcher having publications in reputed SCI & SCOPUS journals and in conferences of IEEE & Springer repute. She has also served as a TPC member, session chair and keynote speaker in some of the reputed conferences in India and abroad. She has also been an editor, an advisor, and a reviewer of several well-known international journals published by IEEE, Taylor & Francis, Springer, and Elsevier.