Medical Physics: Research Article 10-page limit

# Pediatric-Specific Evaluations for Deep Learning CT Denoising

Brandon Nelson, Prabhat Kc, Andreu Badal, Lu Jiang, Shane Masters, Rongping Zeng

# Abstract

Background: Deep learning (DL) CT denoising models are generally trained with large quantities of adult patient image data and have the potential to improve image quality with less radiation dose. However, CT is used in both adult and pediatric populations. Pediatric body habitus and size vary dramatically from newborns to adolescents and can significantly differ from adults. Ensuring that pediatric subgroups of different body sizes are not disadvantaged by DL methods not explicitly trained for them requires evaluations capable of assessing performance in each subgroup.

Purpose: To assess DL CT denoising in pediatric and adult-sized patients we built a framework of computer simulated image quality (IQ) control phantoms and evaluation methodology.

Methods:  The computer simulated IQ phantoms in the framework featured pediatric-sized versions of standard CatPhan 600 and MITA-LCD phantoms with a range of diameters matching the mean effective diameters of pediatric patients ranging from newborns to 18 years old. These phantoms were used in simulating CT images that were then inputs for a DL denoiser to evaluate performance in different sized patients. Adult CT test images were simulated using standard-sized phantoms scanned with adult scan protocols. Pediatric CT test images were simulated with pediatric-sized phantoms and adjusted pediatric protocols. The framework’s evaluation methodology consisted of denoising both adult and pediatric test images then assessing changes in image quality, including noise, image sharpness, CT number accuracy, and low contrast detectability. To demonstrate the use of the framework, a REDCNN denoising model trained on adult patient images was evaluated. To validate that the DL model performance measured with the proposed pediatric IQ phantoms was representative of performance in more realistic patient anatomy, anthropomorphic pediatric XCAT phantoms of the same age range were also used to compare noise reduction performance.

Results: Using the proposed pediatric-sized IQ phantom framework, size differences between adult and pediatric-sized phantoms were observed to substantially influence the adult trained DL denoising model’s performance. When applied to adult images, the DL model achieved a 60% reduction in noise standard deviation without substantial loss in sharpness in mid or high spatial frequencies. However, in smaller phantoms the denoising performance dropped due to field of view (FOV) differences between adult and pediatric protocols that influence image noise texture. In the validation study, noise reduction trends in the pediatric-sized IQ phantoms were found to be consistent with those found in anthropomorphic phantoms.

Conclusion: We developed a framework of using pediatric-sized IQ phantoms for pediatric subgroup evaluation of DL denoising models. Using the framework, we found the performance of an adult trained DL denoiser did not generalize well in the smaller diameter phantoms corresponding to younger pediatric patient sizes. This suggests FOV differences between adult and pediatric protocols can contribute to poor generalizability and must be accounted for in model training and evaluation to ensure pediatric populations benefit from DL denoising.

# Introduction

The use of x-ray computed tomography (CT) in pediatric patients presents benefits of improved diagnosis and treatment as well as risks associated with radiation exposure. This radiation exposure is particularly concerning for pediatric patients because they are more radiosensitive per unit dose than adults and have a longer expected lifetime to accumulate associated cancer risks.1 Therefore, appropriate pediatric CT radiation dose reduction techniques are important to minimize risks from radiation while maintaining the diagnostic utility of exams.

Image reconstruction can contribute to CT dose reduction by using computer algorithms to better separate useful information from noise in low dose CT data. A new paradigm in CT image reconstruction has been the introduction of deep learning (DL)-based techniques. DL can be used to generate CT images directly from low dose x-ray projections. Alternatively, DL can be used to remove noise from low dose CT images reconstructed with conventional methods such as filtered backprojection (FBP).2 Hybrid approaches have also been proposed that utilize DL in iterative reconstruction routines.3

Among the wide variety of DL implementations for image reconstruction, image-based DL denoising in CT has been most intensively studied and is available in commercial CT scanners.4 These image-based DL denoisers are typically trained on many examples of paired low and high noise patient CT images. The performance of DL denoisers has been shown to be advantageous to other advanced reconstruction methods like model based iterative reconstruction (MBIR) due to faster computation times, preferable noise texture, along with comparable noise reduction and sharpness preservation.5

When using image reconstruction to reduce dose, performance assessments are essential to ensure that diagnostic quality is not impaired. Earlier reconstruction methods, like FBP, are unable to reduce dose, but understanding their performance and technical characteristics is simpler. Measures of noise standard deviation, high contrast sharpness, and Hounsfield unit (HU) accuracy are often sufficient characterizations. Nonlinear iterative reconstruction and denoising algorithms have greater noise reduction potential by assuming local smoothness in the imaged object. These nonlinear algorithms require further evaluations of noise power spectrum (NPS) and low contrast detectability to ensure diagnostic value is not lost due to over-smoothing in low contrast regions. DL methods are also highly nonlinear and add further complexity as they are trained on a fixed dataset of patient images. As a result, DL algorithm performance decays when encountering different patient populations, scanners, clinical protocols, or other situations or image features outside their training data distribution. Therefore, assessing the generalizability of DL reconstruction and denoising models to different patient characteristics and populations has become a growing concern.

Generalizability assessments of DL reconstruction are especially important for pediatric patients since DL models are typically trained on primarily adult patient data. One aspect where pediatric patients characteristically differ from adults is body habitus and size (e.g., waist diameter). Patient size is a key characteristic influencing image quality in CT as it factors into x-ray path length, impacting noise and image artifacts. Furthermore, the size range of pediatric patients supersedes that of adult patients, from less than 100 mm in newborns to over 350 mm in adolescents.6 This is illustrated in Figure 1 by plotting mean waist effective diameter with age using data from AAPM Task Group report 204 CDC Vital and Health Statistics data.7,8 Due to this wide range of waist sizes, the FDA recommends dividing pediatric patients into the following age-subgroups when evaluating abdominal x-ray imaging devices: newborn, infant, child, and adolescent.9 These subgroups, overlaid in Figure 1, each have a narrower range of waist sizes. Thus, for a DL model to generalize to pediatrics it must perform well across these pediatric subgroups.

Figure 1 Mean waist effective diameters by age compiled from AAPM Task Group report 204 and the CDC Vital and Health Statistics data.7,8 Shaded regions indicate age ranges of pediatric subgroups recommended by FDA for evaluating abdominal x-ray imaging devices.9

However, pediatric patients are an underrepresented group in healthcare and accumulating sufficient pediatric image data to assess DL generalizability in each subgroup is challenging. Initial studies of using DL denoising models on pediatric image data have demonstrated DL denoising models to be effective in clinical settings, but only with small patient numbers and a limited range of body sizes.5,10,11 Without subgroup analyses12 as well as comparisons against adult patients, it is unclear whether disparities in DL denoising performance exist among subgroups.

To address this lack of appropriate evaluations for pediatric generalizability for DL models, this work introduces a computational framework featuring a set of pediatric sized IQ phantoms and appropriate evaluations. The pediatric-sized phantoms are used as inputs to CT simulations generating realistic noisy images. These images are then processed by the DL denoising model being evaluated. A series of image quality assessments compare performance between pediatric subgroups and adults both before and after denoising. To demonstrate the use of this framework, a study was performed on an adult-trained DL denoising model. The results of this study show how such framework can be used to complement existing scarce pediatric data in assessing the performance of a given DL denoiser in pediatric patients.

# Methods

This investigation of pediatric generalizability of DL denoising models for CT starts by introducing the developed pediatric-sized image quality assessment framework. A case study using the framework is then shown to assess the performance of an adult trained REDCNN DL denoising model on different pediatric subgroups.

## Pediatric-sized Image Quality Phantom Assessment Framework

The proposed pediatric-sized image quality (IQ) phantom assessment framework consists of two parts: 1) a set of pediatric-sized digital phantoms and 2) a set of image quality assessments. The digital phantoms are virtually imaged with a CT simulation framework to provide test images for a DL denoising model. These denoised test images are then assessed by the evaluation framework to identify performance disparities based on patient size.

### Pediatric-Sized Digital Phantom Design

To assess different aspects of image quality performance, three sets of phantoms were simulated. These phantoms were modeled after standard CT image quality evaluation phantoms but with varying cross-sectional diameters. The diameters were simulated to match the mean effective diameters of pediatric subgroups ranging from newborns to adults. Figure 1 illustrates how these effective diameters relate to patient age and subgroup. A subset of these phantoms is shown in Figure 2 with their diameters, associated ages, and subgroup classification.

The first computer modeled IQ phantom is a modified version of the sensitometry module CPT404 from the Catphan 600 phantom (The Phantom Laboratory, Salem, NY). This cylindrical phantom has eight unique contrast inserts ranging from -1000 to +900 HU in a uniform background of 0 HU. In its standard size, CTP404 has a diameter of 150 mm with 12 mm diameter inserts. Due to the sharp intersection between the phantom background and multi-contrast inserts, this module was used to evaluate contrast-dependent image sharpness using the contrast-dependent modulation transfer function.13

The second IQ phantom is the MITA-LCD (Catphan CCT189) phantom used to evaluate low contrast detectability. The standard MITA-LCD phantom is 200 mm in diameter and includes four cylindrical low contrast inserts of 14, 7, 5, and 3 HU, which at its standard adult size have diameters of 3, 5, 7, and 10 mm respectively.

Finally, a set of uniform cylinder water-equivalent phantoms was created to characterize noise properties.

These three sets of phantoms were defined analytically such that they can be generated at any desired size and resolution. For this study, eight phantom sizes were generated that span the range of pediatric effective diameters: 112, 131, 151, 185, 200, 216, 292, and 350 mm. While scaling the CTP404 sensitometry and MITA-LCD phantom diameters, their inserts can either be kept constant at their standard diameter or scaled proportionately. In this study the insert diameters were proportionately scaled to reflect the varying size of pediatric anatomy. For example, in the standard 200 mm diameter MITA-LCD phantom the largest insert is 10 mm in diameter, but when scaled down to 112 mm in phantom diameter, the largest insert diameter became 5.6 mm.

Figure 2 Computer modeled phantoms representing different ages and pediatric subgroups.

### Image Quality Assessments

Image quality assessments were divided into task-independent physical image quality assessments and task-based assessments of low contrast detectability.

Physical image quality assessments such as CT number accuracy, sharpness, and noise properties are standard in the evaluation of image reconstruction and denoising. These assessments are the most basic yet very important tests in the domain of CT imaging.

When evaluating sharpness, nonlinear denoisers, including those utilizing DL, are known to perform variably under different contrast conditions. Thus, for each age-based phantom size investigated, contrast-dependent modulation transfer functions (MTF)13 were calculated to assess sharpness at each contrast difference. The MTF curves were derived from radially averaged line profiles extending from the center of different contrast disks for each size of the CTP404 module.

The eight unique contrast inserts of the CTP404 module were also used in assessing CT number accuracy.

To assess noise properties, both noise magnitude and noise power spectra (NPS) were calculated. Noise magnitude measurements were calculated as the standard deviation (std or σ) of pixels in a central circular region of interest (ROI) in the uniform water phantom for each phantom size with an ROI diameter equal to 1/3 the phantom diameter. NPS measurements were taken from these same ROIs and calculated as the complex modulus of the 2D Fourier Transform of the ROI yielding the 2D NPS. These 2D NPS images were then averaged across 200 repeat noise instances to get an average 2D NPS. 1D radially averaged NPS curves were then extracted from these 2D NPS images. CT number accuracy was calculated with each of the eight unique contrast inserts of the CTP404 module.

Task-based image quality refers to the imaging performance required to accomplish a predefined clinical task.  Well-designed image reconstruction and denoising methods should be able to allow a reduction of x-ray dose while maintaining or improving task performance for a fixed dose compared to the standard FBP reconstruction method.

To assess the task-based performance of a DL denoiser in several pediatric subgroups, a low contrast detectability study was performed with mathematical model observers using the simulated MITA-LCD phantom with low contrast inserts of 14, 7, 5, and 3 HU. For each investigated diameter, 200 repeat simulated CT scans of the MITA-LCD phantom were performed to generate signal-present images. Another 200 repeat scans of the equal-size uniform water phantom were performed to generate a matched set of signal-absent images. These paired signal-present, signal-absent images were further cropped to ROIs around each low contrast insert to evaluate detectability as a function of lesion size and contrast level. From this dataset of paired signal-present and signal absent ROIs, 40% were used to train model observers and the remaining 60% were used for testing.

Different model observers can yield different results, thus two model observers were selected, a Laguerre-Gauss Channelized Hotelling observer (LG-CHO)14 and a non-prewhitening observer (NPW)15. The LG-CHO is a linear approximation of an ideal observer capable of prewhitening, while the NPW is shown to be more responsive to noise reduction strategies.5 Together this pair of efficient and inefficient model observers aim to establish bounds of detectability performance to be expected by human readers.

## DL Denoising Model and Training

To demonstrate the use of this framework in identifying DL image denoising (DLID) sized-based performance, an initial investigation was performed using the image-based REDCNN DL denoising model.2 This DLID model takes as inputs standard filtered backprojection (FBP) reconstructed images yielding processed lower noise outputs. The model was trained on patient data consisting using low dose inputs and routine dose target images from the Low Dose Grand Challenge (LDGC) Dataset.16 The dataset is composed of abdominal non-contrast CT exams from 10 adult patients with reconstructed fields of view ranging from 340 – 420 mm in diameter. The dataset includes full- and quarter-dose image pairs for model training and validation. Of the 10 patients included, 7 were used for model training. The implementation details of the network optimization and parameter tuning process were described previously.17

## CT Simulation Parameters for Test Data Generation

The Michigan Image Reconstruction Toolbox (MIRT) was used to simulate acquiring x-ray projection data, the sinogram. Noise was simulated in projections as the Poisson distribution with mean equal to the number of photons reaching the detector in the noise-free case.  MIRT was also used for FBP reconstruction of the digital phantom CT scans to create input images for the DL denoising model. The source to isocenter distance and source to detector distance, detector size, reconstruction kernel, and mAs of the MIRT simulations were updated to match the noise and image resolution characteristics of the LDGC dataset used in the DL denoising model training.18 CT simulations were performed at dose levels such that at full dose and quarter dose the noise magnitude measured at the center of the 200 mm uniform water digital phantom matched the LDGC dataset. As a result, both the CT simulations and LDGC datasets had an average isocenter noise magnitude of 23 HU in water at full dose and 47 HU at quarter dose. To account for differences in x-ray pathlength caused by imaging phantoms of different diameters, the simulated tube output I0 was set to scale exponentially from its reference value (I0,ref) in the  d= 200 mm uniform water phantom to yield constant noise magnitude relative to patient size:

I0d=I0,ref* exp⁡(μwater*d)/exp⁡(μwater*dref). 			(1)

Thus, in the reference 200 mm water phantom an I0,ref=3×105 photons/pixel yielded a noise magnitude of 23 HU. In phantoms with different diameters d the tube output I0 varied according to Eq. 1 to maintain a full dose noise magnitude of 23 HU at the center. This is demonstrated by the constant standard deviation noise measurements made in images from an adult and newborn-sized IQ phantom in Figure 3.

Where applicable, both pediatric and adult acquisition protocols followed abdomen/pelvis protocols recommended by the AAPM Alliance for Quality Computed Tomography.19 Adult and pediatric scans were both reconstructed with the same body reconstruction kernel. Adult acquisition protocols were defined as having a scanning and reconstructed field of view (FOV) of 340 mm in diameter or 110% the patient effective diameter depending on which was larger. Body fitting FOVs are routinely used in pediatric CT to make efficient use of dose and system spatial resolution. To reflect this practice, pediatric protocols had scanning and reconstructed FOVs defined as 110% the patient effective diameter. While all images were reconstructed in a 512×512 image matrix, this change in FOV size changes the reconstructed voxel size influencing noise texture and image sharpness. This is demonstrated in Figure 3 by comparing reconstructed images from an adult and newborn-sized IQ phantom. Images reconstructed in a smaller FOV have a finer voxel size and appear as a zoomed view. Features can thus appear blurrier if the pixel size is beyond the intrinsic image resolution, as observed in the Newborn phantom image on top right of Figure 3. Additionally, while the noise magnitude is constant across phantom size due to the exposure control defined in Eq. 1, this zoomed view gives an apparent lower frequency noise texture. This can be seen in the bottom row of Figure 3 where the noise in inset image has a larger-grained appearance.

Figure 3 Representative full dose filtered backprojection (FBP) image quality after CT simulation of in silico pediatric IQ phantoms. When reconstructed in a field of view (FOV) 110% the body diameter, the image reconstructed in a smaller FOV with finer pixel size is similar to a zoomed view and can appear blurrier if the pixel size is beyond the intrinsic image resolution, as observed in the Newborn phantom image on top right of the figure.

## Anthropomorphic Phantom Validation

Since the DL denoising models are trained on patient data rather than phantoms, the pediatric XCAT cohort was included in the study to ensure that results found on the pediatric-sized IQ phantoms are representative of those to be expected in patients.  In this simulation study, ground truth is available, which is the FBP reconstructed image from a noiseless sinogram. Noiseless FBP images were chosen as the ground truth because they contain the same streak and blurring artifacts induced by the deterministic imaging system transfer function as in the noisy test images. This choice better reflects the noise removal task of these denoisers which cannot be expected to remove other artifacts that they were not trained to remove. Thus, in this validation experiment, noise reduction was defined as reduction in root mean square error (RMSE) between the noisy reconstructed images x and the ground truth noiseless FBP y:

RMSEx,y=iNxi-yi2N   .					(2)

RMSE reductions measured on the pediatric-sized phantoms were compared against RMSE reduction values determined from the XCAT anthropomorphic phantoms. This demonstrated that noise reduction trends with phantom size measured on IQ phantoms were representative of the noise reduction expected in pediatric subgroups of matching effective diameter.

# Results

The pediatric-sized IQ phantom assessment framework was used to assess the pediatric generalizability of an adult-trained REDCNN DL denoising model. Below are the physical image quality assessments and task-based low contrast detectability results each reported as a function of patient size, followed by the validation experiment results using anthropomorphic phantoms.

## Physical Image Quality Assessment

The physical image quality assessment results include noise, image sharpness and CT number accuracy assessments.

Figure 4 assesses the influence of patient size on noise image quality performance. Figure 4a features an ROI comparison between the FBP and DL denoised images. Despite having the same 24 HU noise standard deviation across phantom sizes in the FBP images, the noise has a finer texture in the larger diameter phantom due to differences in FOV size. Additionally, between the small newborn-sized and large adult-sized phantoms, only the adult-sized DLID processed image saw a reduction in standard deviation noise.

Figure 4b presents noise in spatial frequencies as 2D NPS. The finer noise textures in the adult-phantom images correspond to more pronounced higher frequency noise components towards the periphery of the 2D NPS. Following denoising of the adult-phantom images, the noise power is lower at all frequencies compared to its FBP input. As for the newborn-sized phantom, there is little change in ROI image or 2D NPS following denoising.

In Figure 4c 1D radially averaged NPS curves are plotted for several phantom sizes to show NPS trends versus phantom size. NPS is reported here with spatial frequencies in units of cycles per pixel (cyc/pix), namely pixel-level NPS, rather than line pairs per cm (lp/cm). This better reflects the changes in local noise texture as seen by the denoising model, which is trained to operate on pixelized images and is oblivious to the absolute value of the pixel size, when imaging with different FOVs that change with patient size. For example, in small phantom sizes NPS curves appear to peak at primarily lower frequency before denoising, while larger phantom sizes have higher frequency noise components. In the smaller phantom sizes with lower mean frequency NPS there is little difference following DL denoising. However, in the larger diameter phantoms the gap between FBP and DLID NPS curves increases as REDCNN primarily removes higher frequency noise components.

This size-dependent noise reduction is further demonstrated by plotting the measured noise reduction as a percent of FBP vs phantom diameter in Figure 4d. Age groups with equivalent waist mean effective diameters are overlaid to demonstrate which pediatric subgroups are represented with each phantom diameter.  These results suggest that while larger phantoms, corresponding to larger and older subgroups benefit from noise reductions over 60%, smaller patients under 5 years old see less benefit, with almost no noise reduction expected for newborns and infants based upon their size.

Figure 4 Noise assessment. a) Region of interest (ROI) comparison of uniform phantom images with standard deviation noise measurements. b) 2D noise power spectra images from a), note: brightness has been increased for improved visualization. c) 1D radial averaged NPS profiles show apparent increasing NPS, in cycles per pixel, at larger phantom diameter. DL models are oblivious to pixel size and are sensitive to these apparent changes in NPS. This can be observed also in d), which compares noise reduction performance versus phantom diameter, and by extension, fields of view (FOV). The pediatric reconstruction protocols have reconstructed FOV 110% their effective diameter while the plotted adult protocol has a fixed 340 mm reconstructed FOV. Noise reduction is calculated as the percent difference following denoising measured in the uniform water phantom.

Nonlinear denoising methods are known to better preserve sharpness at high contrasts. To assess the influence of patient size on a DL denoiser’s image sharpness performance, MTF curves were calculated at six different contrast levels of the CTP404 phantom and compared between the original FBP image and the DL denoised image. These MTF curves were measured from 20 repeat scans at 100% relative dose level. Sharpness was assessed as 50% and 10% MTF, the spatial frequencies where 50% and 10% of signal are retained in an ideal edge measurement. The differences in 50% MTF and 10% MTF frequencies between FBP and the DL denoised images (ΔMTF50 and ΔMTF10) were then used to assess the change in sharpness. These changes in sharpness were then plotted against phantom diameters in Figure 5. At high contrasts the DL denoised images showed little loss in sharpness at 50% or 10% MTF, but sharpness was observed to degrade in lower contrasts. These trends are consistent with other nonlinear reconstruction and denoising methods4 that tend to smooth lower-contrast features. Furthermore, these trends of reduced sharpness at low contrast were exacerbated in larger phantom sizes. This indicates a greater reduction in sharpness in larger patients following DL denoising. This finding is consistent with Figure 4c that shows denoising performance also increasing with phantom diameter. Together these suggest that at phantom diameters and FOVs far from the DL training distribution the DL denoiser acts more like an identity operator, leaving the image unchanged. However, at larger diameters and FOVs the denoiser has greater influence on both the noise and sharpness of the resulting image.

Figure 5 Evaluation of image sharpness as a function of phantom diameter and contrast level as measured with contrast dependent Modulation Transfer Function (MTF). Relative difference in sharpness is defined as the difference in 50% MTF frequency of DLID relative to FBP (ΔMTF50 [lp/cm]) and is plotted against phantom diameter for different contrast levels. Both ΔMTF50 and higher frequency ΔMTF15 results are shown. Bands indicate 95% confidence interval following 20 repeat measurements at 100% dose.

CT number accuracy was also evaluated as a function of contrast level and phantom diameter at 25% dose level. Across contrast levels and diameters, mean HU differences between FBP and DLID were observed to be within 2 HU and not found to be substantially influenced by contrast level or phantom size.

## Task-Based Image Quality Assessment

To assess the impact of the DL denoiser on a low contrast detectability task, two different model observers were used: a Laguerre-Gauss Channelized Hotelling Observer (CHO) and a nonprewhitening (NPW) observer. Task performance was calculated as the model observer’s area under the receiver operating characteristic curve (AUC). To show potential improvements in the task following DL denoising, the difference in AUCs between the DL denoised images and standard FBP reconstruction (ΔAUC) is reported in Figure 6 at 25% dose level. Using this measure of performance, task-based performance was shown to decrease progressively in smaller phantom diameters when measured with the less efficient NPW model observer. Similar trends were observed across the four inserts so only the 7 HU, 5 mm insert is shown in Figure 6.

When low contrast detectability was measured using the Laguerre-Gauss CHO observer, no change in detectability was observed following DL denoising in any phantom diameter. This suggests that the evaluated REDCNN DL denoiser only improves performance in this detectability task in larger patients when the observer is not able to decorrelate noise from signal. This is the case for the nonprewhitening observer.

Figure 6 Low contrast detectability task-based image quality assessment measured as a function of diameter of the pediatric-sized standard QA phantom (MITA-LCD), shown in a). Low contrast detectability performance is reported in b) is the difference in the area under the receiver operating characteristic curve (ΔAUC) following DLID compared to the standard filtered backprojection (FBP) result. Different age groups in years (yrs) are overlaid on the plot where the phantom diameter matches the mean effective diameter of each subgroup.

## Anthropomorphic Phantom Validation

Virtual anthropomorphic phantoms were used to validate the noise reduction estimates given by the pediatric IQ phantoms. Example images of adult and pediatric anthropomorphic phantoms both before and after DL denoising are shown in  Figure 7a. When applied to the quarter-dose FBP adult images (Figure 7a top), the DL denoising model reduces noise across the image. Noise is both visibly reduced and has a lower measured voxel standard deviation in the homogenous liver, from 49 HU before denoising to 11 HU after. On the contrary, the DL denoising model has no perceptible denoising effect on the newborn patient image (Figure 7a bottom). There is visually no change in noise compared to the low-dose FBP input image and the measured standard deviation is the same. This outcome is consistent with the uniform phantom-based findings in Figure 4a that the adult-trained REDCNN model was more effective in larger patients. Figure 7b confirms these findings quantitatively by plotting the percent decrease in RMSE against the anthropomorphic patient diameter. RMSE reductions from the uniform pediatric IQ phantoms of matching diameter are overlaid on Figure 7b and trend consistently with the anthropomorphic phantom results. This suggests that even in a simple uniform phantom, size and reconstructed FOV have an outsized influence on DLID denoising performance and can reasonably estimate noise reduction in an anthropomorphic phantom or patient of matched size. However, patient size does not determine all the noise reduction performance as there is still some underestimation of RMSE reduction in the uniform phantom relative to the anthropomorphic phantom.

Figure 7 Adult and pediatric XCAT phantoms were imaged in our simulation framework at 25% dose level to validate our use of pediatric-sized image quality (IQ) phantoms to characterize DLID model performance in different pediatric subgroups. a) shows abdominal cross sections from a small newborn pediatric phantom and a large adult phantom. b) plots noise reduction, calculated as root mean square error (RMSE) reduction following DLID denoising as a function of diameter when applied to uniform water IQ phantoms and anthropomorphic phantoms.

# Discussion

Deep learning image reconstruction and denoising techniques are potentially valuable CT dose reduction tools for pediatric patients. However, pediatric patients are underrepresented in healthcare so evaluating the safety and effectiveness of deep learning-enabled tools is challenging. To address this gap, this work developed a framework to assess the efficacy of DL CT denoising algorithms on pediatric patients based on one simple yet critical difference from adult patients: body size. Using virtual pediatric-sized versions of standard image quality phantoms and associated image quality assessments, the proposed pediatric IQ phantom framework can assess pediatric performance on size-based subgroups.

The present study performed an initial investigation into the use of the proposed pediatric IQ phantom assessment framework to evaluate the performance of a REDCNN denoising model trained on adult CT image data and applied on pediatric images. Using the framework, we observed the REDCNN model to have superior noise reduction in large patient diameters and FOVs. These large sizes are consistent with the adult patients in the training dataset (FOV ≥ 340 mm).  Noise reduction performance was observed to be >60% reduction in pixel standard deviation in larger patients corresponding to adolescents and adults. However, an inflection in performance was observed in smaller phantoms, starting at around 220 mm in effective diameter, equal to the mean-size of a 10-year-old based on reference data. Here performance dropped from >60% to around 40%. Performance was found to be worst, at less than 20%, for the smallest investigated diameters representing subgroups 5 years-old and younger. At these sizes using body fitting reconstructed FOVs the DL model was observed to have minimal influence on the input image.

These findings of DL denoising performance being sensitive to changes in FOV are consistent with other studies of image-based DL denoising models that concluded convolutional neural network (CNN)-based denoising performance decays under imaging factors that change the NPS of the image. These factors include FOV and recon kernel.18,20 The present study further investigated the implications of this local noise texture and FOV sensitivity to pediatric imaging.

The use of body fitting FOVs is common practice in pediatric abdominal imaging. An example where DL denoising models are potentially brittle is when the smaller reconstructed FOV decreases the effective voxel size. This causes the influence of geometric blurring to become more apparent and spread across more voxels in the reconstructed matrix. This can be visualized by comparing the visual edge sharpness and extent of noise correlation (noise texture size) between images of the adult-sized and newborn-sized phantoms in Figure 3 reconstructed with the same FBP reconstruction kernel. This longer scale noise correlation (larger noise grain size) in the smaller FOV reconstructed newborn appears to the CNN, which has no understanding of pixel size, as lower spatial frequency noise components (smaller diameter solid curves in Figure 4c). The investigated REDCNN was trained exclusively on adult patient image data with noise power extending to higher spatial frequencies (in cycles per pixel), like the larger diameter solid curves in Figure 4c). This REDCNN then generalized poorly to small phantom diameter FBP inputs with lower mean apparent NPS frequencies. These findings suggest that the smaller reconstructed FOVs used in scanning smaller patients is one such key image characteristic that can cause CNN-based models to generalize poorly.

While a larger adult FOV could be used to reconstruct data from smaller patients for more consistent DLID performance, this would come at the cost of resolving small spatial features necessary to image the smaller patient anatomy. A better solution would be to utilize DLID models specifically designed for smaller pediatric patients or utilize data augmentation strategies that incorporate lower frequency noise textures in training data as would be found in smaller FOV pediatric images.

The pediatric IQ phantom framework makes use of geometric IQ phantoms which are routinely used in bench testing-based CT image quality evaluation and still provide the fundamental imaging performances of DLID methods. The findings from our validation study utilizing anthropomorphic phantoms indicate that performance trends are similar in both uniform IQ and anthropomorphic phantoms. This suggests that a set of small, medium, and large IQ phantoms covering the desired patient size range could be used to estimate or at least flag for poor performance in different patient subgroup sizes for a given DLID model. Based on this finding, some existing physical IQ phantoms, such as the CATPHAN600 insert without the housing, about 12 cm in diameter, can be utilized for the purpose of testing the generalizability of DLID on small-size pediatric CT. Similar to the MURCURY4.0 phantom (Sun Nuclear, Melbourne, FL) that consists of five sections of diameter from 16 and 36 cm, a pediatric IQ phantom consisting of multiple sections of diameter representing age groups from newborn to 12 years-old may be manufactured to allow the evaluation of the DLID performance across a broader size range of pediatric populations using physical scans.

The current study and framework have limitations. First, only a single image-based DL denoising model was evaluated. The principles behind the proposed framework are not exclusive to image-based methods and future work aims to conduct comparisons between different DL reconstruction methods including projection-based denoising models and MBIR with deep priors. This will allow us to determine the DL methods or training methods such as data augmentation that have better generalizability in pediatric and small patients.

A second limitation is that the CT simulations utilizing MIRT made several simplifying assumptions in favor of reduced simulation computation time. These include monoenergetic source and infinitesimally small x-ray focal spot, and not incorporating detector blur and the effects of scatter. Further planned iterations of the framework’s simulations will leverage existing CT simulation frameworks such as XCIST21 that include these factors.

Additionally, the use of standard image quality phantoms enables measurement of essential CT image performances including MTF and NPS, however their simple uniform background is not representative of the textures and features observed in real patients. This could be responsible for the remaining differences of REDCNN’s noise reduction rate between the uniform and anthropomorphic phantoms observed in Figure 7. Later versions of the framework aim to further leverage the virtual patient cohorts, like XCAT,22 to utilize more realistic patient anatomy and local textures for more representative assessments beyond body size.

A final limitation of our framework is on the use of model observers to estimate low-contrast lesion detectability. Model observers are useful for estimating image performance without the need of expensive and time-consuming reader studies with real human readers. A limitation of using model observers is that the conclusions drawn can vary widely based on the choice of model observer. In this relatively simple task of detecting a disk signal in flat background containing correlated noise, a nonprewhitening (NPW) model observer was shown to correlate well with human performance23,24 while Channelized Hotelling Observers (CHO) such as the Laguerre-Gauss CHO estimate the linear ideal observer and their performance more closely relates to the underlying information in the image.14 Both observers were included in this study to see how their assessment of task performance varies with phantom size. The NPW has been used in other DLID studies to assess task performance5,25. Our findings with the NPW observer are consistent with those previous works in that task performance as measured by a NPW observer benefits from noise reduction. However, NPW models have also been reported as overestimating detection AUC following nonlinear noise reduction such as IR and DLID.26 Meanwhile, when task-performance was measured with a Laguerre-Gauss CHO in this study, no task-benefit was measured, indicating that there was no improvement in the underlying information from the perspective of the estimated linear ideal observer. Thus, while NPW task results are a likely overestimate of the expected task-benefit of DLID and LG-CHO likely underestimates, actual human reader performance may be somewhere in between as humans have been shown to be able to prewhiten noise but not as efficiently as ideal model observers.23,27

Medical devices are generally not initially designed for children yet at some point most medical devices will be used on children regardless of whether that was their designed population or not.28 For some applications the safety and efficacy of a device might not be sensitive to patient age or size. However, as medical devices increasingly incorporate more data-driven methods, it is crucial to understand the safety and efficacy in pediatric populations that are not thoroughly represented in training datasets. This work aims to support a culture of informed development and use of devices to better serve these patients. While evaluations relying on simulation reduce the burden of implementation, they should be used to supplement existing patient evaluations when pediatric data is scarce rather than replace it entirely. Simulations can produce large amounts of data representing key aspects of pediatric patients, but they still lack in realism and diversity available in real patient data. The development of large public repositories of anonymized pediatric data and improved generative methods in time may help address this shortcoming of current simulation methods. However, constant vigilance of device safety and performance in pediatric and other vulnerable populations should continue to be a priority in the development and use of medical devices.

# Conclusion

A framework was developed for pediatric evaluation of deep learning image reconstruction and denoising models. Using this framework, the generalizability of an adult-data trained REDCNN model image performance was evaluated in different pediatric subgroups by size. The results showed that the DL denoising model’s noise reduction capability decreased with the simulated pediatric phantom size, in both the uniform and anthropomorphic backgrounds. FOV differences between adult and pediatric protocols were identified as contributing to reduced noise reduction and task performance in the evaluated model. These differences must be accounted for in training DLID models that inevitably will be used on pediatric patients to ensure they benefit equally from DL denoising.

# References

1.	Hall EJ. Lessons we have learned from our children: cancer risks from diagnostic radiology. Pediatr Radiol. 2002;32(10):700-706. doi:10.1007/s00247-002-0774-8

2.	Chen H, Zhang Y, Kalra MK, et al. Low-Dose CT With a Residual Encoder-Decoder Convolutional Neural Network. IEEE Transactions on Medical Imaging. 2017;36(12):2524-2535. doi:10.1109/TMI.2017.2715284

3.	H C, Y Z, Y C, et al. LEARN: Learned Experts’ Assessment-Based Reconstruction Network for Sparse-Data CT. IEEE transactions on medical imaging. 2018;37(6). doi:10.1109/TMI.2018.2805692

4.	Szczykutowicz TP, Toia GV, Dhanantwari A, Nett B. A Review of Deep Learning CT Reconstruction: Concepts, Limitations, and Promise in Clinical Practice. Curr Radiol Rep. 2022;10(9):101-115. doi:10.1007/s40134-022-00399-5

5.	Brady SL, Trout AT, Somasundaram E, Anton CG, Li Y, Dillman JR. Improving Image Quality and Reducing Radiation Dose for Pediatric CT by Using Deep Learning Reconstruction. Radiology. 2021;298(1):180-188. doi:10.1148/radiol.2020202317

6.	McDowell MA, Fryar CD, Ogden CL, Flegal KM. Anthropometric Reference Data for Children and Adults: United States, 2003-2006: (623932009-001). Published online 2008. doi:10.1037/e623932009-001

7.	Boone J, Strauss K, Cody D, et al. Size-Specific Dose Estimates (SSDE) in Pediatric and Adult Body CT Examinations. AAPM; 2011. doi:10.37206/143

8.	Vital and Health Statistics, Series 3, Number 46. :44.

9.	Pediatric Information for X-ray Imaging Device Premarket Notifications - Guidance for Industry and Food and Drug Administration Staff. Published online March 2017.

10.	Yoon H, Kim J, Lim HJ, Lee MJ. Image quality assessment of pediatric chest and abdomen CT by deep learning reconstruction. BMC Medical Imaging. 2021;21(1):146. doi:10.1186/s12880-021-00677-2

11.	Sun J, Li H, Wang B, et al. Application of a deep learning image reconstruction (DLIR) algorithm in head CT imaging for children to improve image quality and lesion detection. BMC Medical Imaging. 2021;21(1):108. doi:10.1186/s12880-021-00637-w

12.	Brendlin AS, Schmid U, Plajer D, et al. AI Denoising Improves Image Quality and Radiological Workflows in Pediatric Ultra-Low-Dose Thorax Computed Tomography Scans. Tomography. 2022;8(4):1678-1689. doi:10.3390/tomography8040140

13.	Richard S, Husarik DB, Yadava G, Murphy SN, Samei E. Towards task-based assessment of CT performance: system and object MTF across different reconstruction algorithms. Med Phys. 2012;39(7):4115-4122. doi:10.1118/1.4725171

14.	Gallas BD, Barrett HH. Validating the use of channels to estimate the ideal linear observer. J Opt Soc Am A. 2003;20(9):1725. doi:10.1364/JOSAA.20.001725

15.	Abbey CK, Barrett HH. Human- and model-observer performance in ramp-spectrum noise: effects of regularization and object variability. J Opt Soc Am A, JOSAA. 2001;18(3):473-488. doi:10.1364/JOSAA.18.000473

16.	McCollough CH, Bartley AC, Carter RE, et al. Low-dose CT for the detection and classification of metastatic liver lesions: Results of the 2016 Low Dose CT Grand Challenge. Med Phys. 2017;44(10):e339-e352. doi:10.1002/mp.12345

17.	Kc P, Zeng R, Farhangi MM, Myers KJ. Deep neural networks-based denoising models for CT imaging and their efficacy. In: Bosmans H, Zhao W, Yu L, eds. Medical Imaging 2021: Physics of Medical Imaging. SPIE; 2021:16. doi:10.1117/12.2581418

18.	Zeng R, Lin CY, Li Q, et al. Performance of a deep learning-based CT image denoising method: Generalizability over dose, reconstruction kernel, and slice thickness. Med Phys. 2022;49(2):836-853. doi:10.1002/mp.15430

19.	AAPM CT Scan Protocols - The Alliance for Quality Computed Tomography. Accessed June 29, 2023. https://www.aapm.org/pubs/ctprotocols/default.asp?tab=5#CTabbedPanels

20.	Huber NR, Missert AD, Yu L, Leng S, McCollough CH. Evaluating a Convolutional Neural Network Noise Reduction Method When Applied to CT Images Reconstructed Differently Than Training Data. Journal of Computer Assisted Tomography. 2021;Publish Ahead of Print. doi:10.1097/RCT.0000000000001150

21.	Wu M, FitzGerald P, Zhang J, et al. XCIST-an open access x-ray/CT simulation toolkit. Phys Med Biol. 2022;67(19). doi:10.1088/1361-6560/ac9174

22.	Segars WP, Norris H, Sturgeon GM, et al. The development of a population of 4D pediatric XCAT phantoms for imaging research and optimization. Med Phys. 2015;42(8):4719-4726. doi:10.1118/1.4926847

23.	Barrett HH, Yao J, Rolland JP, Myers KJ. Model observers for assessment of image quality. Proc Natl Acad Sci U S A. 1993;90(21):9758-9765. doi:10.1073/pnas.90.21.9758

24.	Myers KJ, Barrett HH, Borgstrom MC, Patton DD, Seeley GW. Effect of noise correlation on detectability of disk signals in medical imaging. J Opt Soc Am A, JOSAA. 1985;2(10):1752-1759. doi:10.1364/JOSAA.2.001752

25.	Nagayama Y, Goto M, Sakabe D, et al. Radiation Dose Reduction for 80-kVp Pediatric CT Using Deep Learning–Based Reconstruction: A Clinical and Phantom Study. American Journal of Roentgenology. 2022;219(2):315-324. doi:10.2214/AJR.21.27255

26.	Chen B, Yu L, Leng S, et al. Predicting detection performance with model observers: Fourier domain or spatial domain? Proc SPIE Int Soc Opt Eng. 2016;9783:978326. doi:10.1117/12.2216962

27.	Abbey CK, Samuelson FW, Zeng R, Boone JM, Eckstein MP, Myers K. Classification images for localization performance in ramp-spectrum noise. Medical Physics. 2018;45(5):1970-1984. doi:10.1002/mp.12857

28.	SECTION ON CARDIOLOGY AND CARDIAC SURGERY, SECTION ON ORTHOPAEDICS, Jenkins KJ, et al. Off-Label Use of Medical Devices in Children. Pediatrics. 2017;139(1):e20163439. doi:10.1542/peds.2016-3439