- TOXIC  METALS REMOVAL  BY  BIOSORPTION   2019

- J of Research in Applied chemistry  vol 2 {1}15-34  Mantech publications

ABSTRACT

Toxicity of Flouride and Chromate ions is well known,causing Fluorosis and Cancer respectively.  Arsenic , mercury, lead and cadmium  are highly toxic even at low levels. Zn( ll) is substituted in metallo enzymes by cadmium. Toxic As ( lll) inhibits enzyme action. Mercury is  responsible for Minimata disease   while lead poisoning occurs due to its presence at high levels in food and water.Developing country like. India needs cheap water treatment methods which are sludge free and adsorption has an edge over all other techniques Like  .Biosorption and  Low cost adsorbents prepared from waste organic matter , Tree leaves, Bark,  Sawdust ,Rice husk , coal ,Flyash have been tested for removal of  these toxic anions and cations from water It  has  long been  known  that   many   biological  materials  will  accumulate  metals     This   phenomenon   tends  to exacerberate  the   environmental    impact  of  various  metals   that   are  released  in    the  environment  because  they   become  concentrated in the flora  and \or fauna   and   ultimately get  into  the  food chain .   How ever , the  same  phenomena   potentially can be   used   in   a  controlled  environment   to  remove  dissolved  metal  from     processing   streams   either  for  recovery  of  valuable  resources  or  to   remove  pollutants .This  paper explores  Toxic  metal removal by  Biosorption

Dr   Jyotsna  Lal       Professor,  Department of  Chemistry

Christ Church College . Kanpur 208001 UP

INDIA

- TOXIC  METALS REMOVAL  BY  BIOSORPTION

- Dr   Jyotsna  Lal

Professor,  Department of  Chemistry 
  Christ Church College . Kanpur 208001 UP,INDIA

# INTRODUCTION

It  has  long been  known  that   many   biological  materials  will  accumulate  metals     This   phenomenon   tends  to exacerberate  the   environmental    impact  of  various  metals   that   are  released  in    the  environment  because  they   become  concentrated in the flora  and \or fauna   and   ultimately get  into  the  food chain .   How ever , the  same  phenomena   potentially can be   used   in   a  controlled  environment   to  remove  dissolved  metal  from     processing   streams   either  for  recovery  of  valuable  resources  or  to   remove  pollutants . The interaction   between   microbes and metals   has   been  studied  by  scientists  of  different  disciplines.  Agricultural  scientists are primarily   interested in the microbicidal action of metals ,   bio inorganic  chemists  basically  look  at  the  metal   coordinating geometry and   their environment   in metallo proteins   and    biological   functions    Scientists  in  life  sciences  study  the  toxicological  effects ,    its     bio accumulation and    bio magnification  while  the  environmental  scientists and  engineers  try  to utilize  this   property  of   bio accumulation  for  monitoring   metal  pollution as  well  as   removal \ recovery  of   metals  from   natural  and   waste waters .  There  is   an  increasing  interest   in  investigating   such    biological   adsorbents  for  variety   of   applications , as   biologically   derived   material  can  be  produced  at  low  cost , yet   have  adsorbtion  capacities   near   those   of  commercial  ion-exchange   resins .   Adsorbtion  of   the  metal   ions   on  to   materials of   biological origin is   now   regarded   as  one of    the  most   promising   technologies  for  treating   wastes  economically . In  this  paper  the author  wishes  to  direct  attention  towards  the  application  of  micro organisms   for   metal   reclaimation   and   recovery       It  has   been observed  that  many  microbial  species  exhibit  metal  binding properties   the   adsorption   capacities   of   microbial   biomass   are   comparable  to  or  can   exceed   those  of   inorganic adsorbents .  The  use  of   bio adsorbents .for  removal   and  recovery   of   dissolved  metals  has   been  demonstrated  on   a    laboratory  scale  or   with   small  pilot  plants   Biosorption    has  emerged as a   viable   alternative   to  conventional  metal  scavenging   techniques .   To make  this  technology   economically viable,   however   the   dynamics  of  this  reaction  must  be  enhanced  significantly   Some   mechanisms  have  been  proposed  by  reseachers  for   metal  accumulation  by  micro organisms

# BIO SORPTION  BY  MICROBIAL    BIOMASS

Use of   yeast  cells   Saccharomyces sp  for starch  fermentation  in  ethanol   production   is  well  known    Research  in  use  of   mixed   culture  of  Saccharomyces  diastaticus and  Zymomonas  mobilis   in  ethanol   production   has  been   reported. Biosorption ,  is   the  process  in  which  live or dead  microbes are  used  for  removal  of  metals  from  their  dilute solutions   In  recent   years  investigation  of  various  types  of  micro organisms  for  metal  adsorption  has  been  reported   Heavy   metals ,  which can  be  toxic  to  living  organisms  are   present  in  many  industrial aqueous   waste  streams .  These  metals  may  originate  from  a  variety  of   sources including   the nuclear   power  defence   and   fuel  reprocessing  industries ; surface finishing   processes in   the  aerospace industry ; and silk –screening processes in the computer industry.Many  microbial  species  exhibit  metal  binding properties    Most   research  in   this  area  has  been   associated  with   heavy  metals ,  such as    Pb,   Cd,  Cu ,  Cr   ,and   radio  nuclide   accumulation.      Intra cellular  accumulation  of  many  metals  has  been  observed  in   bacteria ,  fungi and  algae  .It  has  been  shown    [Table 1   ]  by  several  researchers  that  for  some  types  of  microbial  biomass  ,most  of  the  metal adsorption  occurs   in  the  cell   or    membrane  wall ,this  included   microbial  biomass   from  fungi  such  as  Ganoderma lucidum    which   adsorbs   Cu  ,   bacteria  such  as    Micrococcus  luteus  for  adsorption of  Sr and  yeast    Saccharomyces  cerevisae  for  U , Zn , Mg , Co . Bioaccumulation   of   Ni, Cu   by    non pathogenic    yeast  Candica sp  Copper  was accumulated  by   Pseudomonas  sp Artificial  ponds  and  streams  contain   filamentatous  algae   and  cyanobacteria  can    greatly   reduce levels   Pb  ,Cu ,  Zn and   Mn  from  waste  waters   The  live  cyano bacteria   Anabena  cylindrica , Anabena  flos-aquae   and   Nostoc  sp    accumulates    Ni .Arthrobacter    has   high  Ni  uptake Chlorella vulgaris  has  affinity  for  Au  ,Ag   Hg Oscillatoria  perornata  var  attenate  and   Scenedesmus  quadricauda  var  longispuia   are   able  to  accumulate   Fe  Zn , Cu   and  also  affected  by  high  concentrations  of  heavy   metals

| TABLE 1 MICRO ORGANISMS | # WHICH CAN BIND METALS | # IN THEIR BIOMASS |
| --- | --- | --- |
| MICRO ORGANISM | METAL | ### SITE OF BINDING |
| Rhiszopus arrhizus | U , [180 mg more than biomass ] | Cell wall [ Chitin ] |
| Pencillum sp | U ,[ 80- 120 \ kg biomass] | Cell wall |
| Bacillus subtilus | Fe ,Mn ,Cu , Zn , Mg , Ni Pb | Cell wall [Peptidoglyan] |
| Mucor flavus | Fe (OH )2 | Chitosan |
| Bacillus lichencforms | Cu, Fe III , Ni ,Mn , Au . | Cell wall [Teichoic acids ] |
| Denitrifying bacteria | U [140 mg \ g biomass ] |  |
| Pseudomonas aeruginosa | U | Li poly saccharides |
| Sacchromyces cerevisiae | U, Zn , Mg | Cell wall |
| Cyano bacteria | Ni |  |
| Entero bacter cloaca | U, Cr (IV) - Cr (III) - |  |
| Ganoderm lucidum | Cu | Cell wall |
| Micro coccus luteus | Sr | Peptidoglyan content [ Glutamic acid ] |
| Ascophyllum nodosum | Co |  |
| Arthro bacter ,Klebsilla Consortia of bacteria , Staphylococcus aureous P .maltophilia | Ag 30 % of biomass |  |
| Citro bacter sp | Cd | Envelope bound Phosphatase at cell surface |
| Aspergillus niger | Cd |  |

| TABLE 2 TOXICITY AND | UPTAKE OF HEAVY | METALS TO ALGAE |
| --- | --- | --- |
| ALGAE | METAL ION | MAXIMUM [ug\ml] CONCENTRATIONS THAT FAVOURED GROWTH |
| Oscillatoria perornata var attennate | Fe (III), Zn (II), Cu (II) | 27.56 0.833 0.0207 |
| Scenedesmus quadricauda var longispuia | Fe (III), Zn (II), Cu (II) | 20.67 0.595 0.0188 |
| ALGAE | METAL ION | MAXIMUM UPTAKE ug\ml |
| Oscillatoria perornata var attennate | Fe (III), Zn (II), Cu (II) | 07.506 0.1086 0.0111 |
| Scenedesmus quadricauda var longispuia | Fe (III), Zn (II), Cu (II) | 06.449 0.1086 0.0076 |

In  Table 2    the  toxicity  and  uptake  of   heavy metals  on  Algae are given . Blue green   algae  Westiellopsis  prolifica  has  significantly    reduced  high  levels  of    Na , K , Ca  , Cl ,  SO42-  ,   PO 43-    from  Paper  mill  waste  waters .   Thiobacillus   species   are  involved  in  oxidation  of  sulphide minerals    Aspergillus  niger  and   Trichoderma harzianum  oxidises  sulphides   of  Cu  , Pb , Zn  , Co ,  Mg  .      Bacillus sp  and  Vibrio  sp   reduce  Ferromanganese  nodules.Escheria  coli  can  be  used  to accumulate   Ni    10Bacterium  such as  Enterobacter cloacae    will   reduce   chromate to   trivalent   chromium   which   can   then   be   removed  using  other   bioadsorption technology.

Zooglea ramigera ,   common  in  natural   water  have   been  used  to accumulate   Ni  , Fe , Cr    from   waste   waters .Micro organisms   involved  in  bio reduction   and   bio  oxidation   of minerals   are given in  Table   3  &  4     Citrobactor  sp    immobilised  in poly acrylamide  gel    scavenged  large   quantities  of   Cd

Most   research  in  this  area  has  been  associated  with  heavy  metals  such  as  of   Cd  Cu ,  Pb , Cr,    but   a     broad   range  of    micro organisms  has   been   shown  to   effectively    remove  metals   like  Sr    from   a   typical  waste  water  containing   radioactive   pollutants    As  seen  in  Table  5  the  distribution  coefficients  for  Sr adsorbtion  (quantity of Sr adsorbed per unit mass   of  dry   biosorbent  divided   by  equilibrium  concentration  of   the  metal    within   the bulk solution )

| TABLE 3 MICRO-ORGANISMS |  | INVOLVED IN BIO-REDUCTION |  |
| --- | --- | --- | --- |
| ORGANISMS |  | ORE |  |
| Vibrio sp |  | Ferro manganese nodules |  |
| Clostridium sp |  | Goethite , Haematite |  |
| Entero bactor sp |  | Goethite , Haematite, Cr(IV) to Cr (III) |  |
| Bacillus sp |  | Ferro manganese nodules |  |
| Pseudomonas sp |  | “ “ |  |
| Chromatiphilia sp |  | Chromium ore |  |
| Sulphate reducers |  | Sulphates of Fe(II),Ni(II) ,Cu(II) ,Co(II) |  |
| Bacteria and Fungi |  | SeO , SeO |  |

|  | ORGANISM | ORE |
| --- | --- | --- |
| 1 | Thio bacillus ferro oxidans 2- 40 C pH 1.4 –4.8 | Metal sulphides, Uranite Rock phosphate, |
| 2 | T . thiopains | Sulphide minerals |
| 3 | T .cupriums | “ “ |
| 4 | T .acidophilus | Pyrite Chalcopyrites |
| 5 | Sulfolobus acido caldarium Optimum Temp 80 - 85 C | Sulphide minerals |
| 6 | S .brierleyi Optimum Temp 70 C | Sulphide minerals |
| 7 | Sulphobacillus thermo sulkfido oxidans 20 –60 C | Sulphide minerals |
| 8 | Archae bacteria [un identified] | Mn ( II ) minerals |
| 9 | Thio microspria crunogen | Sulphur oxidizing |
| 10 | Arthro bacter Optimum Temp 88 C | Mn ( II ) |
| 11 | Hypho microbium sp | Mn ( II ) |
| 12 | Gallionella | Fe 2+ -----> FeO OH |
| 13 | Aspergillus niger | Sulphides of Cu Pb , Zn ,Co , Mg |
| 14 | Trichoderma harzianum | “ “ |
| 15 | Leptothrix discophora | Mn (II) |

TABLE  4          MICRO- ORGANISMS    INVOLVED   IN   BIO-OXIDATION

|  | TABLE 5 ADSORPTION OF STRONTIUM BY | VARIOUS MICRO-ORGANISMS |  |
| --- | --- | --- | --- |
|  | ## Micro organism | Distribution 2 Days | Coefficient 7 Days |
|  | ## Rhizopus arrhizus | 1,470 | 85,533 |
|  | Micro coccus lutes | 24,943 | 12,010 |
|  | Anaebaena | 5.554 | 10,304 |
|  | Streptomyces viridochromogenes | 4,147 | 3,949 |
|  | Penicillium chrysogenum | 7,181 | 3,625 |
|  | Escherichia coli | 967 | 1,961 |
|  | Chlorella pyrenoidosa | 734 | 1,915 |
|  | Zooglea ramigera | 493 | 1,858 |
|  | Chlamydomonas reinhardtii | 1,639 | 1,785 |
|  | Candida sp | 764 | 1,139 |
|  | Caulobactor fusiformis | 619 | 1,055 |
|  | Pseudomonas aeruginosa | 6,321 | 1,003 |
|  | Paecilomyces marquandii | 1,808 | 656 |

## IMMOBILIZED   MICRO ORGANISMS

The  immobilization  of  micro organisms   can   be    defined  as  any   technique  which     limits  the  free  migration  of  cells   There  are  two    types   of   immobilization

(i )  attachement  , micro organisms   and  other  cellular  materials  have  a  natural

inclination  to  adhere  to surfaces  and  in this  way  become immobilised

(ii)   entrapment  where  micro organisms  are  caught  in  the  interstices   of   fiberous \porous  material  or  physically  restrained  within  solid  or  porous  matrix  such  as  a stabilized  gel  or  a  membrane  To  concentrate  large  quantities  of   bio  mass  , the  microbial  biomass  is  entraped  into  gel  matrix   like   carragenan , alginate , modified  bone  gel  in the  form  of  beads   .The  gel  is   generally   formed   into  useful

biocatalyst  beads  by   adding   the  cells  as a  sus pension  to   the   aqueous   solution

of    the   gelling   material   .  This  material     is  formed   into   droplets   either   by   forcing    it  dropwise   through  a  nozzle  or   orifice   or   by   dispersing  into  a   non

interacting   liquid   medium  .  Immobilization  or   entrapment  of micro   organisms  is   done    for     various   applications     like   ethanol   fermentation   ,  production   of   antibiotics   and  therapeutics .  Micro organism   Cephalosporium  acremonium  was   immobilized  in    poly acrylamide  gel ,  used   for  the    production  of   Cephalosporin   ß  lactam  antibiotic . Citrobactor  sp    immobilised  in   poly acrylamide  gel    scavenged  large   quantities  of   Cd.L.K  Jang   et   al   dispensed   green    algae   Microcystis   in   sodium   alginate   to  form   a  biopolymer   gel  for   the    recovery    of       Cu2+  and    Co 2+    ions  .Researchers  have   discussed     the    use  of    various    types  of  bioadsorbent    materials  plant  tissue  and   microbial  mass   for   immobilization  in   gel  matrix  for  removal  of  dissolved  metals   A  unique  columnar  system  has  been  proposed  for  the  use of  crosslinked  bone  gel  beads      Significant    quantities  of  heavy  metal   ions   will  adsorb  on    bone  gel  beads  in  a   Fluidized - bed  contactor. The   loaded     beads  can   easily   be   removed   and  regenerated

# MECHANISM   OF    BIO ACCUMULATION

Mechanisms  by  which  micro organisms  remove  metals  from  solution  are

(i) extracellular accumulation \ precipitation  (ii)  cell – surface  sorption \complexation

(iii) Intracellular accumulation  ;   process  (ii)   can  occur  whether  the  micro organism    is   living  or dead    (i)   may  be  facilitated  by  micro bial  viabilty (iii)  process  requires  micro bial  activity

Both  active  and  passive  mechanisms  are   employed  by   living  cells to  accumulate dissolved  metals    Since ,  high  concentration   of   heavy  metals   tend  to  adversely affect   phsiological    processes     living  cells  that   adsorb  such  materials  must  use  [Table  2]     mechanisms  that  ensure   that  only     a  limited  amount  of  the  metal  actually  reaches  in the interior of  the cell  where  it  is  most  toxic .  Many   cellular   mechanisms  result  in    depositing   the  metals  in  cell wall or  membrane   Citrobactor  sp  in   presence  of  a  suitable  phosphate  donor  is  able  to  immobilise  large   quantities  of   Cd , Cu  ,Pb ,U  by    forming  insoluble  metal  phosphates on the  cell surface .Sulphate  reducing  bacteria  may  produce  H 2S  gas  which  will  react  with metal  ions  to  form  an  insoluble  precipitate .Phytochelatins ,  a  class of   peptides  found   in  plants  are   apparently  effective in    metal   binding    These   peptides  are  in  the range of  5-17  amino  acids  in  length  with  a   carboxyl  terminal  glycine   that    apparently   binds  metals  by   thiolate  coordination .   The   Phytochelatins   are  generated in   the  living   plant   tissue  that  is  exposed  to   metal ions   thus   providing  a  means  of  enhancing  the  metal   adsorption  capacity    of the   biomass ,

Living cells  are affected  by   metal  toxicty and they   require the  addition of necessary nutrients ,  maintenance of   specific  operating  conditions .  On  the other hand nonliving or inactive microbial  biomass  does   not  have  these  requirements  and  in  many  cases can  be  a  very  effective  bioadsorbent  for  dissolved   metals . The  cell  walls   and   internal  materials   within   biomass  offer   abundant  sites for  metal  complexation  or  micro precipitation . Biological  components  have  amounts  of   poly saccharides , protiens and  lipids  that  are  replete  with  metal  binding  functional  groups  including  carboxylate ,  hydroxy ,  sulfate ,  phosphate   and   amino  groups

It  has been shown    [Table 1  ]  by several researchers that for some types of  microbial  biomass  ,most  of  the  metal  adsorption  occurs   in  the cell or    membrane  wall ,    this  included   microbial  biomass   from  fungi  such  as     Ganoderma lucidum   which adsorbs Cu ,   bacteria  such  as  Micrococcus  luteus  for  adsorption of  Sr  and   yeast  Saccharomyces  cerevisae  for  U , Zn , Mg , Co  .

Strandberg  et al  studied Saccharomyces  cerevisae  for  U  suggested  the  role  of   phosphomannans  carboxylic  groups  of  cell  wall  protein  of  yeasts  in   metal  binding   Yeast  cells   have  been  shown  to  accumulate  heavy  metals  such  as Cu , Ni  via  two distinct  process  .There  is  an   initial  rapid   accumulation  step  that  is  metabolism  and   temperature  independent  ,involves   cation   binding  to  negetively  charged  sites  on the  cell wall .The  outer  mannan  protein  layer  of  the  yeast  cell wall  as well as  the inner  glucan  -chitin  layer are  important   for  heavy  metal  accumulation Second  process  is  metabolism  dependent  much   slower  can  accumulate  large quantities of  cations .

Researchers  have  shown  that  nine  species  of  genus   Penicillum  had  no significant difference in their ability to accumulate metal ions , this  accumulation   property  is associated   with  a  microbial  species  rather  than  genus .

Chitin  , an   amino  polysaccaride  with   a   molecular  structure  shown   in  Figure  1 has  been  postulated  as the  cell  wall  component  responsible   for   metal  coordination

Tsezos and Volesky  have  reported  results  of a    systematic study   carried  out  to delineate the mechanism  of  biosorption  of  uranium  by  Rhizopus arrhizus  .Based on electron  microscopy ,X-ray energy dispersion  analysis ,and  infrared spectrascopic studies , they  proposed  three  processes   responsible  for metal  scavenging  :

(i)  The first  process is  complex  formation  between  dissolved  uranium  ionic  species   and  chitin  present  in  the  cell  wall  of   Rhizopus  arrhizus .    Uranium coordinates   to  the   amino nitrogen  of the chitin   crystallates  and  is   retained  within  the  cell wall of the mycellium .

(ii)   The  second  process is  adsorption  of  additional  uranium by  chitin   network close   to  that  complexed  by  chitin  nitrogen .

(ii) The  third   process  is  hydrolysis  of   uranium  chitin  complex  formed  by  the  first process  and  the   precipitation  of   the  hydrolysis   product  on  the  cell  wall  upon which  the    chitin   nitrogen  relieved  will   re engage  further   uranium   until accumulation   of   the   hydrolysis   product   inhibits    the   cycle .

Free  radical s  are highly  unstable   species   which  are  trapped  in  the  very  stable  cell  wall  matrix , this  free radical  has  been  attributed  to  chitin  nitrogen

Muraleedharan et al  have  concluded  that  the   free  radical   present  consistently in the biosorbents is  not  taking  part  in the  metal  uptake . However  the  cell  wall  matrix   which has  encampassed  and  trapped  this  free  radical opens  up  upon  metal  uptake   indicating  its  role  in  metal  uptake  .  The  exposed  cell  wall  matrix  thus  freely   interacts  with   the  metal  resulting  in   high  removal  rates ,   the   free radical  can  be used  to  probe  for   further  identification  of  the  stable  cell  wall  components  responsible  for   metal  sorption .

Tsezos  observed  that  at   pH  4 , uranly   uptake  by   Rhizopus arrhizus  was  inhibited  by  competing  cations ,while  pH  2   no  effect  was  evident     J.M..Tobin et al    24 studied  the  effects  of  cation   competition on  the  uptake  of   a range  of  different metals  on  the  fungus  Rhizopus arrhizus ,  They   have  reported  ,the displacement of  Ag  + ions  by  UO 2 +2    ,  indicates  that  the  sites  of   uptake  of  these  metals  are  the  same  .  In  studies   where  Ag + was  in  excess  the  results  support the  presence  of  multiplicity  of  binding  sites in the biomass .In  the  UO 2 +2 - Ag  +  system  it appears  that  at  less  than   saturation  concentrations  UO 2 +2   ions   bind   preferentially  to  certain  sites,   leaving   others   partially   unoccupied  and  available  for  Ag +  binding   even   at  low   [Ag  + ] \  [UO 2 +2   ]  ratios .As  this  ratio  increases  ,Ag  +   ions  may   increasingly   displace  the    UO 2 +2   ions    from  the  latter  type  sites  .Conversely  , at  higher  UO 2 +2 concentrations  UO 2 +2  ions  occupy  all  the  sites  and  Ag+ is  not  absorbed

L.K  Jang   and   coworkers   dispensed   green    algae   Microcystis   in   sodium   alginate   to  form   a  biopolymer   gel  for   the    recovery    of       Cu2+  and    Co 2+    ions  .The    algin  contained  4.356  mmol\g   uronate  residues   or  repeating monomer  units    According   to   the   egg  box   model   of   gelatin  mechanism   ,  each  cavity   formed  in  the   alginate  gel to   trap  a    divalent   metal  ion   has  to  involve  two  carboxyl    groups   from  participating  uronate  residues  .   Bone  gelatin   based  gel   beads  are  able  to   accumulate  large  quantities  of  Cu  and  Sr and  thus  enhance  capacity  of  immobilized  biosorbent

FIGURE    2    A     DIAGRAMMATIC    MODEL   OF THE   MULTIPLICITY   OF

UPTAKE  OF  UPTAKE SITES   ON   Rhizopus  arrhizus

[UO represents UO 2 2+ Suggested  by   J.M .Tobin  and  coworkers ]

# CONCLUSION

Water pollution has emerged as one of the most alarming public health problems in india in recent years.  Based upon the above literature survey the following conclusion can be drawn (i) factors responsible for spread and severity of health problems

population over growth ,consumption of more water for daily use;unplanned and indiscrimination in digging of wells;increase in rate of industrialization;high level of As , Cr.Ni, Pb, F- intake through drinking water and food stuff

(2) It has been observed that, there is a pressing need  for development and introduction of simple and inexpensive methods of  water treatment.(iv) To deal with the problem an awareness should be created among villagers about the ill effects of drinking contaminated water.  There should be transfer of water treatment techniques to the villages.  There is a need for water quality surveillance.  If villagers are trained to use water testing kits to monitor the purity of water independently, they will realise the gravity of the problem.The water pollution menance can be overcome by   community  participation.

Rice straw , Baggase and  Sawdust  are  natural inexpensive  materials ,which are  easily available and do not  require chemical pretreatment  .Exhibit  exceptionly high  degree of Cr(VI) adsorption , thus can be utilized for treatment of  industrial waste effluents containing Cr(VI).Red mud may serve as economic and effective fixing agent for Cr(III) removal from chromium laden wastewater.On the  other hand Tea leaves carbon TLC  and  Groundnut husk carbon GHC  are excellent  cheap  carbonaceous  adsorbents showing high  degree of Cr(VI) adsorption. Iron hexamine modified sawdust  is another  cheap adsorbent These adsorbents can easily prepared  in any  laboratory   of  a  tannery  , chrome plating  or  paint  industry Use  of  adsorbents  derived from plant  biomass sawdust , mustard husk, tealeaves  are cheap and  effective for  lead removal similarly  coal based adsorbent CFe  and CMnO2  can be  used  for  lead though the  kinetics  need to  be  studied .

Phytoremediation of lead  by Eichhornia crassipes from water  and Brassica juncea , B.rapa ,Helianthus annus , Vicia faba, Pissum sativum , Phaseolus vulgaris Nicotiana glauca (shrub tobacco)  from   soil offer   good  alternatives.

All these adsorbents offer  a  good alternative to  expensive activated carbons and  other water treatment  technologies .

ii) Several technologies have been studied and the process of absorption on alumina has an edge other methods due to its simplicity and sludge free operation. Activated alumina  and  Alum  based  defluoridation  technologies  should  be avoided Water  treated  by   Nalgonda  technology   contains  residual  aluminium  in the  range  of  2.1 -  6. 8  mg\ l   under various  operating  conditions . Al  in  concentration  of  80  μ g \  l or  more  results  in  Dialysis   dementia  and risk of   Alzheimer’ s  disease  [JyotsnaLal.,2002;Gupta 1999

This research has been limited  to a study of adsorption capacities at different pH for some low cost asorbents . There is a need to describe the mechanisms responsible for removal of  metal ions Cr , Ni ,Pb ad anion F-   from  aqueous  solution by low cost asorbents .

The  preceeding  discussion shows  that  many  microbial  species  exhibit  metal  binding

properties   The  major  potential  use  for  immobilized  cells  is  in   biomedical  area  for  production   of   antibiotics  and    therapeutics     Waste  water   treatment  is one  of  the  significant  application  for  immobilized  cells     To  evaluate  the  suitability  of  a biosorbent   for   field  application  in   the   treatment of  metal   rich  waste   waters ,  it  is  neccessary  to  determine  the   maximum  sorption   capacity  ,  kinetics  of  sorption , recovery  of  the metal  and   physical  state  of the  biosorbent .

However , not   all  of  them  may  be  suitable , it  is better to select  the  microbial  mass  which  can  fulfil  the  following  criteria : (i)  the  bio sorbent  should  be  produced at  low  cost  and  be  reusable (ii)  uptake and release of the metal should  be efficient  and  rapid (iii)  regeneration  of  the biosorbent  should  be  possible

An   understanding  of  the  mechanisms  by  which  organisms  accumulate  metals  is important to  the  developement  of  microbial processes for  the  concentration , removal  and  recovery  of  the metal  from  aqueous  solutions  .A   knowledge  of  the  chemical and  physiological  reactions  during  metal  uptake  might  enable  the  specification  and  control  of  process  parameters to increase  the rate  of  metal  accumulation

REFERENCES

Shumate .S.E ; Strandberg GW [1985 ] “Comprehensive Biotechnology”  ed

Robinson CW  ;Howell  J  A  Pergmon Press

.  J .A .Brierly  ;  G .M.  Goyak  [1986]    Fundamentals  and   Applied   io  hydrometallurgy   eds

R W Lawrence, R M R Bronien ,H G Ebner  Elsevier        New York

.  Amutha   . R  ;  Gunasekaran .P  [2000]   Improved  Ethanol  production  by   mixed  culture  of

Saccaromyces  diasticus  and  Zymomonas  mobilis   from      liquified  casava  starch  .  Ind .  J .

Microbiology  ,  40 , 103- 107

.  Sawhney  .A.L  et  .al   [2000]  Degradation  of  ground water quality due to  heavy   metals  in

industrial areas   of  India  -A  review , Ind . J.  Environ .   Protection       20 :  174

BIS  [1996]     Indian Standard :  Drinking Water Specification [IS 10500:1991]

Bureau  of Indian Standard ,New Delhi ,8

.  Sharad Mittal .; R.M.S .Sengar  ;  B.D .Kaushik  [1992] Uptake  and Toxicity  of   Heavy  metals

to Algae  Ind . J. Microbiology  32, 51-55

Pawan Sharma  ; Ajit Verma   [1991]  Microbial reclaimation  of  metals  from   ores  and

industrial  waste  waters .  Ind . J. Microbiology  31 1-26

.   K. Komori  ;  A .Rivas ; K . Toda ; H . Ohtake  [1990 ]  Biological  removal  of   toxic Cr  using

Enterobacter  cloaco  strain  Biotech  &  Bioengg  , 35, 951 – 54

Aksu .Z  ;  Kutsal  .T  [1990  ]  A  comparitive  study  for  biosorption  characteristics  of  heavy

metal   ions   with   Chlorella.  vulgaris    Environ  Tech nol   11, 979-987

.  Ranul Krihna swamy  ;  David  B Wilson    [2000]  Construction  and  characterization    of  an

Escheria  coli    genetically   eng inered  for   Ni    accumulation

Appl .Environ  Microb  66 ,   5383 - 5386

.  Corder S.L ; Reeves .M  [1994]  Biosorption  of  Ni   in  complex  aqueous  streams  by

Cyano bacteria     Appl. Biochem.Biotechnol  45\46  , 847  -  859

.  Veglio . F ;  Beol/ F;  Gasbarro. A  [1998]   Biosorption  of  toxic heavy    metal   ......using   free

Arthrobacter sp    Process  Biochem  32,  99-105

L .E .Macaskie and  A.C.R .Dean  [1989 ]  Advances  in     Biotechnological   processes  Vol 12

eds  A . Mizrani , AR Liss New York

. Charles .D .Scott and  James N.  Petersen   [1992]  Immobilized  biosobents  for dissolved metals

Separation and  Purification Technology  eds  Norman  Li , Joseph M Calo  ,

Marcel Dekker  Inc  New York. Basel .Hong Kong

## . Charles .D .Scott   [1987] Immobilised  cells  A  review  of   recent   literature       Enzyme .Micro .

## Techno .   ,  9 ,  66-73

.Macaskie , M.  ; Dean .A.C.R. , [1986 ]   Cd  accumulation   by   immobilised   cells    of

Citrobacter  sp   Biotech  &  Bioengg  , 28 ,1358 –65

. Muraleedharan ,T.R ; C. Venkobacchar [1990]  Mechanism of biosorption  of   Cu II    by

Ganoderm  luciderm  . Biotech  &  Bioengg  [1989 ], 35, 320 – 325

. B .D. Faison ;  C. A . Cancel ;  S.N.  Lewis ; H.I. Adler [1990]

Appl .  Environ  Microbiol  56 ,36-49

.Norberg  .  A B ;  Pearson  . H ;  [1984 ] Accumulation   of   heavy  metals  by  Zooglea  ramigera

Biotech  &  Bioengg  26 , 239

. Strand berg G.W. ; Shumate S. E  ;  J. R  . Parrot  [1981]   Microbial  cells  as  Biosorbents  for

Heavy  metals   :  Accumulation   by  Saccharomyces  cerevisiae  ;  Pseudomonas  aeruginosa

Appl .  Environ  Microbiol    , 41  ,237  .

. Niu H ;  Xu . X .S ;Wang  J .H     [1993  ]  Removal   of   Lead  by   Penicillium biomass

Biotech  &  Bioengg  42 ,785-787

.Tsezos  ; Volesky  [1981]  Mechanism  of  biosorption  of  Uranium  by  Rhizopus arrhizus

Biotech  &  Bioengg  , 23 , 583

. G. Donmez ;  Z .Aksu  [2001]  Bio accumulation  of  Cu  and  Ni  by   the  non  adapted  and

adapted growing   Candida   sp  Wat  Res  ,35  ,  1425 – 1434

. Tobin  .  J.M  ; Cooper  D .G ;  Neufeld .R .J .  [1988]  The  effects  of  cation  competition  on

metal  adsorption  by   Rhizopus arrhizus    Biotech  &  Bioengg  31,  282 – 286

.Jang  L  K  ; Lopez  S L;  Eastman  ;Pryfogie  .P  [1991]   Recovery  of    copper and   cobalt  by

Biopolymer  Gels   Bio technol  & Bioengg  ,37  ,266-73

.Wulf .C  [1984 ]   Biotechnology : A  Textbook  of  Industrial  Microbiology  [ English   trans ]

.Saurabh  Gupta ;  Vinod  Bihari ;  S. C. Agarwal   et. al [2000]  studies on  free and immobilised

cells  of   Cephalosporium  acremonium  gel   for  the  production  of   Cephalosporin [ ß  lactam

antibiotic ]  Ind  J   . Microbiology        40 , 141 -144

. Macaskie , M ;  J. M . Wates ;  Dean .A.C.R. , [1987 ]   , Cadmium   Accumulation by Citrobacter

immobilised on  gel  and solid  supports       Biotech  &  Bioengg    30 , 66

.  Good man G.T  ; Roberts T . M  ; [1971 ]    Plants  and   Soils   as   indicators   of   metals    in

air  .    Nature   231 , 287

.    Watson  J.S ;Scott  C.D ; Fasison  B D   ;  [1990]   Emerging  Technologies  in  Hazardous

Waste  Management  ACS Symposium   Series no  422  [DW Tedder  ;  FG Pohland]

American   Chemical  Society  Washington

.  Petersen ,J.N  ;Scott  C.D ; et  al  [1990]    Accumulation  of  Cu  on   to Modified  bone   gelatin

beads    Bio technol  .Techniqes  ,4  ,435 –440

. Yi-Tin  Wang ;  Evans ,M.  Chirwa  [2000]  Cr  (VI)  reduction  in  continuous  flow  coculture

Bio reactor  .  J .  Environ  .  Engg  .  126  ,  300-306

.  Frencessa  Podda ; Paola  Zudda  et  al  [2000]  Heavy  metal  co precipitation  with

Hydro zincite from  mine  waters  caused  by  photosynthetic  micro  organisms

App .  Environ .  Micro biology  ,11, 5092 -5098

.  M . Vasanthy  ;  P .  Lakshmana  perumala samy ;  [2000]    Copper  accumulation  by

Pseudomonas  sp  ;  IJEP  20 , 280-283

A .K  .Dash  ;  P .C.  Misra  [1999]  Role  of  Blue green  algae    Westiellopsis  prolifica

in  reducing  pollution  load  from  paper  mill  waste  water  IJEP  ,19 ,1-5

J.N. Petersen ; B.H . Davidson ;C.D. Scott ;  SL Blankinship  [19991] Size  changes  associated

with   metal  adsorbtion  onto  Modified  bone  gelatin  beads

Biotech  &  Bioengg    38 , 923-928