Notes on Crampton QA/QC  for GLEON metab/catchment group

Jake Zwart; 2016-02-16

QA/QC overview

METADATA

Crampton DO sensor depth = 0.5 m.

Ave Elevation = 511 m

windHeight <- 2 m   #height above lake at which wind speed is meaured

timeStep <- 10      #number of minutes between DO measurements

lat =  46.21046 #latitude of Crampton

DO file

No NA’s / missing data;

Range = 7.26- 10.47 mg L-1

Wind speed file

No NA’s / missing data; no obvious anomalies

Range = 0 – 6.49 m s-1

PAR file

No NA’s / missing data; no obvious anomalies

Range = 1.2 – 2553.7 umol m-2 s-1 (1.2 is the minimum measurement from Onset light sensors – occurs at night)

Thermistor data file

Some missing data (filled in as NA’s) at 7 m from May –July

No obvious anomalies

Heat map of thermistor profile