Notes on EastLong QA/QC  for GLEON metab/catchment group

Jake Zwart; 2016-02-16

QA/QC overview

METADATA

EastLong DO sensor depth = 0.7 m.

Ave Elevation = 514 m

windHeight <- 2 m   #height above lake at which wind speed is meaured

timeStep <- 10      #number of minutes between DO measurements

lat = 46.23657 #latitude of EastLong

DO file

No NA’s / missing data; no obvious anomalies

Range = 4- 8.6 mg L-1

Wind speed file

No NA’s / missing data; no obvious anomalies

Range = 0 – 6.49 m s-1

PAR file

No NA’s / missing data; no obvious anomalies

Range = 1.2 – 2553.7 umol m-2 s-1 (1.2 is the minimum measurement from Onset light sensors – occurs at night)

Thermistor data file

Some missing data (filled in as NA’s) at the beginning and end of the time series, but thermistor data is complete on dates for which there is DO obs.

No obvious anomalies

Heat map of thermistor profile