Data description:

- This data set contains data from 8 tables which are described below.

- The dump typically contains SQL statements to create the table, populate it, or both. However, mariadb-dump can also be used to generate files in CSV, other delimited text, or XML format.

logging:

Time series of all measured values (historical data) for the current, voltage and power curves. The Data  are described in Table 1below.

scn_values:

Time series of all SCN (Smart Charging Network) measurements (60 Seconds cycle). The table contains the maximum current value per phase valid at the time of data collection and the actual current per phase in the SCN. The Data  are described in Table 2 below.

conncetor_status,

contains status messages that are reported during a charging process. (60 Seconds cycle). A status message can be that a charging process is active, has been stopped or ended, but also a pure error message. All status messages with the statuses are important for the charging processes:

Available, Charging, SuspendedEV and Finishing.

connector_info

The current error message and/or the current SCN setting can be seen in the connector_info table (60 Seconds cycle)

connector_meter_value

The meter value during the charging process (60 Seconds cycle)

transaction_start
Contain the starting time of a charging process, meter value at the start of the charging process and an anonymous ID of the user. (60 Seconds cycle)

transaction_stop
Contain the stop time of a charging process, meter value at the End of the charging process and an anonymous ID of the user. (60 Seconds cycle)

view_values
The view_values table shows the forecast data and current data on the power parameters of the microgrid (20 Seconds cycle). This table hat 4 column (id, Description, Value, and timestamp). The Description column is shown in the table 3. The Discription “Gesamtprognose Heute” and “Gesamtprognose Morgen” Shows the forecast for the current day and the next day.

| Column Name | Description | Unit |
| --- | --- | --- |
| U_1 | Line-to-ground voltage in the microgrid for phase 1 | 1 × 10^-2 V |
| U_2 | Line-to-ground voltage in the microgrid for phase 2 | 1 × 10^-2 V |
| U_3 | Line-to-ground voltage in the microgrid for phase 3 | 1 × 10^-2 V |
| I_1 | Current in the microgrid for phase 1 (excluding reactive currents) | 1 × 10^-3 A |
| I_2 | Current in the microgrid for phase 2 (excluding reactive currents) | 1 × 10^-3 A |
| I_3 | Current in the microgrid for phase 3 (excluding reactive currents) | 1 × 10^-3 A |
| I_n | Neutral conductor current in the microgrid | 1 × 10^-3 A |
| P_sum | Summed power across all three phases | 1 × 10^1 W |
| P_1 | Power on phase 1 (with sign) | 1 × 10^1 W |
| P_2 | Power on phase 2 (with sign) | 1 × 10^1 W |
| P_3 | Power on phase 3 (with sign) | 1 × 10^1 W |
| bat_cur | DC current of the battery (with sign) | 1 × 10^-3 A |
| bat_soc | State of charge of the battery | % |
| bat_temp | Battery temperature | 1 × 10^-1 °C |
| cur_L1 | Current in the microgrid for phase 1 (including reactive currents) | 1 × 10^-3 A |
| cur_L2 | Current in the microgrid for phase 2 (including reactive currents) | 1 × 10^-3 A |
| cur_L3 | Current in the microgrid for phase 3 (including reactive currents) | 1 × 10^-3 A |
| E_sum | Total energy fed into all phases | Wh |
| p_actual | Current PV feed-in active power across all phases | W |

Table1: Description of the table logging

| Column Name | Description | Unit |
| --- | --- | --- |
| Max_L1 | Maximum current in the SCN for phase 1 | A |
| Max_L2 | Maximum current in the SCN for phase 2 | A |
| Max_L3 | Maximum current in the SCN for phase 3 | A |
| SCN_L1 | Current in the SCN for phase 1 | A |
| SCN_L2 | Current in the SCN for phase 2 | A |
| SCN_L3 | Current in the SCN for phase 3 | A |

Table2: Description of the table scn_values

| Description Column | Unit |
| --- | --- |
| SCN_Current_L1 | 1 × 10^-3 A |
| SCN_Current_L2 | 1 × 10^-3 A |
| SCN_Current_L3 | 1 × 10^-3 A |
| Max_Current_L1 | 1 × 10^-3 A |
| Max_Current_L2 | 1 × 10^-3 A |
| Max_Current_L3 | 1 × 10^-3 A |
| SMA_battery_temp | 1 × 10^-1 °C |
| SMA_battery_soc | % |
| SMA_Current_L1 | 1 × 10^-3 A |
| SMA_Current_L2 | 1 × 10^-3 A |
| SMA_Current_L3 | 1 × 10^-3 A |
| SMA_actual_PV_power | 1 × 10^1 W |
| Smart_Meter_P_sum | 1 × 10^1 W |
| Smart_Meter_P_1 | 1 × 10^-1 °C |
| Smart_Meter_P_2 | 1 × 10^-3 A |
| Smart_Meter_P_3 | 1 × 10^-3 A |
| Smart_Meter_I_1 | 1 × 10^-3 A |
| Smart_Meter_I_2 | 1 × 10^-3 A |
| Smart_Meter_I_3 | 1 × 10^-3 A |
| Smart_Meter_U_1 | 1 × 10^-2 V |
| Smart_Meter_U_2 | 1 × 10^-2 V |
| Smart_Meter_U_3 | 1 × 10^-2 V |
| SMA_E_sum | Wh × 10^-5 |
| Gesamtprognose Heute | kWh |
| Gesamtprognose Morgen | kWh |

Table3: Description of the description coloum in the table view_values