Bill of Materials

Parts required for both conventional and AutoFSP

| Category | Item | Detail | Manufacturer | Est. cost unit-1 [CHF] | Units Used | Total Cost [CHF] |
| --- | --- | --- | --- | --- | --- | --- |
| Inlet and exhaust-gas manage-ment | Vacuum pump | MINK 0080D, (80Nm3 h-1) | Busch Vacuum Solutions, Maulburg, DE | 4600 | 1 | 4600 |
|  | Heat exchanger for exhaust gas | GPLK 30-30-H-30 | Funke, Gronau/Leine, DE | 220 | 1 | 220 |
|  | Stainless steel exhaust pipe | 316L, KF40, with connectors | VACOM, Grosslöbichau, DE | 330 | 3 m | 1000 |
|  | Pressure sensor ∆pfilter | PE5 (-1 to 0 MPa-g and 0 to 5 MPa-g) | Emerson Electric Co., USA | 150 | 1 | 150 |
|  | Thermo element | 1.5*300 |  | 60 | 4 | 240 |
|  | Inlet airflow hose | Foxi Garant Pur, 45mm | Peter Hefti AG, Frauenfeld, CH | 35 | 1m | 35 |
|  | Flowmeter inlet airflow | VA520 | CS Instruments, Harrislee, DE | 1270 | 1 | 1270 |
|  | HEPA inlet air filter + housing | Megalam MG14 & CamSeal, CSL-3P3-TS-C 160 | CAMFIL AG, Oberägeri, CH | 500 | 1 | 500 |
| Process gas manage-ment | Mass Flow Controllers | Red-y-smart series | Vögtlin Instruments, Muttenz, CH | 1483 | 3 | 4450 |
|  | Push-in fittings & PU-hosing for gases and liquids |  | Parker Legris | - | - | 500 |
|  | Pressure sensors, pO2,disp | PE5 (0 to 5 MPa-g) | Emerson Electric Co., USA | 150 | 1 | 150 |
| Micro-annular gear pump | Pump for liquid precursors | mzr-2905 | hnp-Mikrosysteme, Schwerin, DE | 5400 | 1 | 1 |
| Setup build materials | Nuts, bolts, plates, flanges, strut profile |  | stocks from inhouse workshop | - | - | 3150 |
| Parts for HMI | Computer w/ housing | Raspberry Pi 4 | Raspberry Inc. | 100 | 1 | 100 |
|  | 12.5” Touch Screen |  |  | 180 | 1 | 180 |
| PLC Parts | PLC controller | PFC100, 750-8100 | WAGO, Minden, DE | 382 | 1 | 382 |
|  | Power supply 24VCD | 750-602 |  | 14 | 2 | 28 |
|  | 8-channel Analog In: Thermocouple | 750-458 |  | 243 | 1 | 243 |
|  | 4-channel analog In, 0-10VDC | 750-459 |  | 140 | 1 | 140 |
|  | RS232/485 Interface | 750-652 |  | 231 | 2 | 462 |
|  | 4-channel analog Out, 0-10VDC | 750-559 |  | 140 | 1 | 140 |
|  | Housing |  |  | 140 | 1 | 140 |
|  | Emergency off switch |  | Eaton | 65 | 1 | 65 |
|  | Power supply | 24VDC, 5A |  | 219 | 1 | 219 |
| Various electronics | Plugs, cables, adapters |  | Various | - | - | 680 |
| Module Fabrication | Machining Cost | Electronic Workshop | D-CHAB, ETH Zürich |  | 90 h | 2700 |
|  | Machining Cost | Mechanic Workshop |  |  | 50 h | 1500 |
| Total |  |  |  |  |  | 23245 |

Parts required for automation

| Category | Item | Detail | Manufacturer | Est. cost unit-1 [CHF] | Units Used | Total Cost [CHF] |
| --- | --- | --- | --- | --- | --- | --- |
| Microfluidic liquid valves w/ FFKM seals | Type 2/2NC | SCS067A031 | ASCO Emerson Electric Co., USA | 235 | 10 | 2350 |
|  | Type 3/2 | SCS067A111 |  | 235 | 3 | 705 |
|  | Type 3/2, ¼-28UNF | 6724 | Bürkert, Ingelfingen, DE | 190 | 3 | 570 |
|  | Type 2/2 NC, ¼-28UNF |  |  | 143 | 6 | 860 |
|  | Type 2/2NC, R1/8” | 6011 |  | 157 | 3 | 470 |
| Pneumatic Liquid handling | Rotary Valve | PTFE Body, 10-port, 1000µm Orifice | Advanced Microfluidics, Ecublens, CH. | 980 | 1 | 980 |
|  | Hosing, manifolds & Ferrules | Tubing 1/8”, Ferrules: ¼”-28UNF | Darwin Microfluidics, Paris, FR | - | - | 1000 |
|  | Pressure Controller 0.05-5 MPa-g, | ITV1030-31F1N3 | SMC Corp. Japan. | 450 | 1 | 450 |
|  | Type 3/2 gas valves used for vessel pressurization | VUVS | Festo, Esslingen, DE | 67 | 3 | 200 |
|  | Optical bubble sensor | BE-A301P | Panasonic Electronics, Japan | 87 | 4 | 348 |
| Dosing and Mixing Vessels | Magnetic stirring plates | Rotilabo M3 | Roth AG, Arlesheim, CH | 100 | 2 | 200 |
|  | Manufacturing of D&M vessels | Material Cost & Machining | Inhouse workshop | - | - | 600 |
|  | Coating for D&M vessels | PTFE | Buser Oberflächentechnik, Wiler, CH. | 150 | 3 | 450 |
|  | PTFE Gear Pump | A500S-4NP | Oriwen Fluidics Technology, Shanghai, PRC | 400 | 1 | 400 |
|  | Cleaning Nozzles | 1.35-80° H-LE | Danfoss, Nordborg, DK | 33 | 3 | 100 |
|  | Analytical Scale | JJBC-1523 | G&G, Kaarst, DE | 690 | 1 | 690 |
|  | Various small parts | Cables, Plugs, Connectors |  | - | - | 500 |
| PLC | Power supply 24VCD w/ fuse | 750-601 | WAGO, Minden, DE | 17 | 1 | 17 |
|  | 16-channel digital out, 24VDC | 750-1504 |  | 100 | 2 | 200 |
|  | 8-channel digital in/out, 24VDC | 750-1506 |  | 78 | 1 | 78 |
|  | 2-channel relay, NO | 750-512 |  | 85 | 1 | 85 |
|  | RS232/485 Interface | 750-652 |  | 231 | 1 | 231 |
|  | Power supply, 12VDC, 5A | RND315-00001 | RND Power | 8 | 1 | 8 |
| Manufacturing of Mixing unit housing | Materials, Machining, Anodizing (Al) | Mechanic Workshop | D-CHAB, ETH Zürich | - | - | 2800 |
| Total |  |  |  |  |  | 14292 |