Algorithm 1: DPGADP

Start from an initial Q function and controller Q0(xk,u) and C0xk and select LC. Alternate the next two steps:

- Evaluate the Q function of the current controller Cj(∙) by issuing the full-back operation

| Qj+1xk, uk=rxk, uk+γQj+1xk+1, Cj(xk+1). | (10) |
| --- | --- |

- Issue the controller improvement step. This is achieved by doing LC gradient steps, with the index ic=0:LC, LC>0. Initializing Cj0xk=Cj(xk) the controller update is

| CjiC+1 xk=CjiC xk-α∂Qj+1xk,u∂CjiCxku=Cj, | (11) |
| --- | --- |

where α∈R+m represents the learning rate. After LC steps, the controller from iteration j+1 is Cj+1xk=CjLC xk.

- If no more changes happen between Qj+1xk, uk and Qjxk, uk, stop the iterative process and go to Step 4. Else, issue j=j+1 and go to Step 1.

4.     Get Cj(xk).