Data Source for

Synergizing Resource Recovery and Net-Zero Emissions in China’s Wastewater Sector

Wei Yang 1,2,3,*, Haoyuan Liu 3, Tianfu Yao 3, Qilin Cao 3, Wufei Zhang 3, Junnian Song 1,2,3,*

1 Key Laboratory of Groundwater Resources and Environment, Ministry of Education, Jilin University, Changchun, 130021, China.

2 Jilin Provincial Key Laboratory of Water Resources and Environment, Jilin University, Changchun, 130021, China.

3 College of New Energy and Environment, Jilin University, Changchun, 130012, China.

*Corresponding author

Wei Yang, yangwea@jlu.edu.cn, C128 Tang Aoqing Bldg., 2699 Qianjin St., Changchun, 130012, China;

Junnian Song, songjunnian@jlu.edu.cn, C440 Tang Aoqing Bldg., 2699 Qianjin St., Changchun, 130012, China

Table 1. Data source of GHG emission factors for conventional wastewater treatment technologies.

| Technology | Emission factor | Reference |
| --- | --- | --- |
| Anaerobic-anoxic-oxic | CO2 emission factor | Bao et al., 2015; Yan et al., 2014 |
|  | CH4 emission factor | Li et al., 2017; Ren et al., 2013; Rodriguez-Caballero et al., 2014; Wang et al., 2011 |
|  | N2O emission factor | Li et al., 2017; Ren et al., 2013; Rodriguez-Caballero et al., 2014; Yan et al., 2014 |
| Oxidation ditch | CO2 emission factor | Bao et al., 2015; Yan et al., 2014 |
|  | CH4 emission factor | Delre et al., 2017; Li et al., 2016; Li et al., 2017; Ren et al., 2013; Yan et al., 2014 |
|  | N2O emission factor | Delre et al., 2017; Foley et al., 2010; Li et al., 2017; Ren et al., 2013; Yan et al., 2014 |
| Conventional activated sludge | CO2 emission factor | Hu et al., 2019; Yoshida et al., 2014 |
|  | CH4 emission factor | Aboobakar et al., 2014; Hwang et al., 2016; Masuda et al., 2018 |
|  | N2O emission factor | Brotto et al., 2015; Hwang et al., 2016; Masuda et al., 2018 |
| Sequencing batch reactor | CO2 emission factor | Bao et al., 2015; Bao et al., 2016 |
|  | CH4 emission factor | Bao et al., 2016; Delre et al., 2017; Kong et al., 2013 |
|  | N2O emission factor | Bao et al., 2016; Foley et al., 2010; Kong et al., 2013; Lo et al., 2010 |
| Anoxic-oxic | CO2 emission factor | Bao et al., 2015; Bao et al., 2016 |
|  | CH4 emission factor | Bao et al., 2016 |
|  | N2O emission factor | Aboobakar et al., 2013; Bao et al., 2016 |

Table 2. Data source of pollutant removal efficiencies for wastewater treatment technologies for enhanced energy recovery.

| Technology | Removal rate (%) | Reference |
| --- | --- | --- |
| EGSB | COD removal rate | Dai et al., 2011; He et al., 2023; Sheldon et al., 2016; Xing et al., 2024; Yang et al., 2020 |
|  | TN removal rate | Lee et al., 2015; Liu et al., 2021; Justo et al., 2016; Yang et al., 2020 |
|  | TP removal rate | Dai et al., 2011; He et al., 2023; Liu et al., 2021; Sheldon et al., 2016 |
| CETP | COD removal rate | Checa-Fernández et al., 2024; Chun et al., 2020; Hu et al., 2023 |
|  | TN removal rate | Chakraborty et al., 2020; Chun et al., 2020; Rahman et al., 2016 |
|  | TP removal rate | Chen et al., 2019; Hu et al., 2023; Shewa et al., 2020; Wu et al., 2023 |
| HRAS | COD removal rate | Abdelrahman et al., 2023; Guven et al., 2019; Ji et al., 2023 |
|  | TN removal rate | Abdelrahman et al., 2023; Canals et al., 2024; Ji et al., 2023; Rahman et al., 2019 |
|  | TP removal rate | Abdelrahman et al., 2023; Canals et al., 2024; Ji et al., 2023 |
| AnMBR | COD removal rate | Dong et al., 2016; Evans et al., 2019; Gouveia et al., 2015; Kong et al., 2021a |
|  | TN removal rate | Kong et al., 2021b; Rong et al., 2021; Wimalaweera et al., 2024; Wimalaweera et al., 2025 |
|  | TP removal rate | Liu et al., 2020; Wimalaweera et al., 2024 |

Table 3. Data source of additional parameters for wastewater treatment technologies for enhanced energy recovery.

| Technology | Parameter | Reference |
| --- | --- | --- |
| EGSB | Electricity intensity | Xu et al., 2020 |
|  | Sludge yield | MEE, 2012 |
|  | CH4 yield | MEE, 2012 |
|  | Biogas leakage rate | Song et al., 2023 |
|  | N2O conversion rate | Fang et al., 2020 |
| CEPT | Iron salts consumption | Chun et al., 2020; Hu et al., 2023; Ye et al., 2024 |
|  | Electricity intensity | Ye et al., 2024 |
|  | Sludge yield | Chun et al., 2020; Hu et al., 2023; |
| HRAS | Electricity intensity | Abdelrahman et al., 2023 |
|  | Sludge yield | Carrera et al., 2022; Liang et al., 2022; Shao et al., 2020 |
|  | CO2 emission factors | Ji et al., 2023 |
|  | CH4 emission factors | Ji et al., 2023 |
|  | N2O conversion rate | Ji et al., 2023 |
| AnMBR | Electricity intensity | Evans et al., 2019 |
|  | Sludge yield | Dong et al., 2016; Evans et al., 2019; Kong et al., 2021a |
|  | CH4 yield | Gouveia et al., 2015; Kong et al., 2021a; Liu et al., 2024 |
|  | Biogas leakage rate | Kong et al., 2021a |
| PND | Electricity intensity | Chai et al., 2014; Wang et al., 2021 |
|  | N2O conversion rate | Chai et al., 2014; Wang et al., 2021 |
| PN/A | Electricity intensity | Chai et al., 2014 |
|  | N2O conversion rate | Chen et al., 2020; Christensson et al., 2013; Hausherr et al., 2022; Joss et al., 2009 |

Table 4. Data source of wastewater reclamation technologies.

| Technology | Parameter | Reference |
| --- | --- | --- |
| UF | Electricity intensity | Chew et al., 2016; Li et al., 2018 |
|  | Water recovery rate | Stachurski et al., 2024; Verhuelsdonk et al., 2021 |
| RO | Electricity intensity | Lee et al., 2012 |
|  | Water recovery rate | Lee et al., 2012; Stachurski et al., 2024 |
| DDBF | Electricity intensity | Huang et al., 2019; Zhang et al., 2024 |
|  | Polyaluminum chloride consumption | Zhang et al., 2024 |
|  | Methanol consumption | Zhang et al., 2024 |
|  | N2O conversion rate | Du et al., 2023 |
| UV | Electricity intensity | Longo et al., 2016 |

Table 5. Data source of energy recovery and conversion technologies.

| Technology | Parameter | Reference |
| --- | --- | --- |
| AD | CH4 yield | Balasundaram et al., 2024 |
|  | Electricity intensity | Balasundaram et al., 2024 |
|  | Heat intensity | Balasundaram et al., 2024 |
|  | Biogas leakage rate | O'Shea et al., 2020 |
| Biogas purification | Electricity intensity | Angelidaki et al., 2018; Lombardi et al., 2020 |
|  | Iron salts consumption | Angelidaki et al., 2018 |
|  | Activated carbon consumption | Lombardi et al., 2020 |
|  | Sodium hydroxide consumption | Lombardi et al., 2020 |
| Water scrubber (for biogas upgrading) | Electricity intensity | DGTC, 2020 |
|  | Piped water consumption | Baena-Moreno et al., 2019; Kapoor et al., 2019 |
| WSHP (for HOCIW heat recovery) | Coefficient of performance | Wang et al., 2024; Yu et al., 2014 |
| CHP | Heat generation efficiency | Akbulut et al., 2021 |
|  | Power generation efficiency | Akbulut et al., 2021 |

Table 6. Data source of carbon capture technologies.

| Technology | Parameter | Unit | Reference |
| --- | --- | --- | --- |
| Carbon capture | CO2 capture rate (for biogas upgrading) | % | Varling et al., 2023 |
|  | CO2 capture rate (after CHP) | % | Beiron et al., 2023; Varling et al., 2023 |
|  | Electricity intensity | kWh/t CO2 captured | CCGGWG, 2023; Christensen et al., 2021; DGTC, 2020 |
|  | Heat intensity | MJ/t CO2 captured | CCGGWG, 2023; Christensen et al., 2021; DGTC, 2020 |
|  | Piped water consumption | kg/t CO2 captured | CCGGWG, 2023; Christensen et al., 2021; DGTC, 2020 |
|  | Organic amines consumption | kg/t CO2 captured | CCGGWG, 2023; Christensen et al., 2021; DGTC, 2020 |

Table 7. Data source of biofertilizer production and application processes.

| Process | Parameter | Unit | Reference |
| --- | --- | --- | --- |
| Dewatering | Electricity intensity | kWh/kg digestate | Yang et al., 2023 |
| Biofertilizer production | Nutrient recovery rate | % | Du et al., 2023 |
|  | Electricity intensity | kWh/kg digestate | Yang et al., 2023 |
| Applied to fields | Direct GHG emission factors | kg CO2eq/kg digestate | Yang et al., 2023 |

Table 8. Data source of nutrient contents in digestate.

| Nutrients content | Unit | Reference |
| --- | --- | --- |
| Total nitrogen (as N) | % w/w | Peccia et al., 2015; Yang et al., 2023 |
| Total phosphorus (as P2O5) | % w/w | Peccia et al., 2015; Yang et al., 2023 |

Table 9. Emission factors for chemicals production.

| Chemical | Emission factor | Unit | Reference |
| --- | --- | --- | --- |
| Ferric chloride | 0.243 | kg CO2eq/kg | Wernet et al., 2016 |
| Activated carbon | 1.0 | kg CO2eq/kg | DGTC, 2020 |
| Sodium hydroxide | 1.59 | kg CO2eq/kg | DGTC, 2020 |
| Piped water | 0.168 | kg CO2eq/t | MHURD, 2019 |
| Softened water | 0.636 | kg CO2eq/t | NDRC, 2019; NEA, 2020; SETC, 2002 |
| Organic amines | 0.0912 | kg CO2eq/kg | Wernet et al., 2016 |
| Polyaluminum chloride | 1.828 | kg CO2eq/kg | Wernet et al., 2016 |
| Polyacrylamide | 1.5 | kg CO2eq/kg | Wernet et al., 2016 |
| Methanol | 0.659 | kg CO2eq/kg | Wernet et al., 2016 |
| Nitrogen fertilizer (as N) | 7.76 | kg CO2eq/kg | CCGGWG, 2023 |
| Phosphorus fertilizer (as P2O5) | 5.34 | kg CO2eq/kg | CCGGWG, 2023 |

Table 10. Changes in emission factors of grid electricity and heat in China with increasing share of renewable energy in the energy mix (ERICAMR, 2023; GONDRC, 2015; Jia et al., 2024; MEE, 2020; SGETRI, 2024).

| Factor | Region | Share of renewable energy |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 28% | 44% | 52% | 66% |
| Emission factors of electricity production (kg CO2/kWh) | Beijing | 0.942 | 0.828 | 0.441 | 0.361 |
|  | Tianjin | 0.942 | 0.646 | 0.412 | 0.316 |
|  | Hebei | 0.942 | 0.558 | 0.438 | 0.334 |
|  | Shanxi | 0.942 | 0.725 | 0.599 | 0.449 |
|  | Inner Mongolia | 0.942 | 0.684 | 0.589 | 0.439 |
|  | Liaoning | 1.083 | 0.591 | 0.401 | 0.297 |
|  | Jilin | 1.083 | 0.622 | 0.332 | 0.263 |
|  | Heilongjiang | 1.083 | 0.822 | 0.642 | 0.471 |
|  | Shanghai | 0.792 | 0.433 | 0.330 | 0.260 |
|  | Jiangsu | 0.792 | 0.577 | 0.459 | 0.340 |
|  | Zhejiang | 0.792 | 0.570 | 0.445 | 0.331 |
|  | Anhui | 0.792 | 0.712 | 0.608 | 0.443 |
|  | Fujian | 0.792 | 0.513 | 0.384 | 0.296 |
|  | Jiangxi | 0.859 | 0.594 | 0.421 | 0.326 |
|  | Shandong | 0.942 | 0.620 | 0.473 | 0.361 |
|  | Henan | 0.859 | 0.558 | 0.401 | 0.290 |
|  | Hubei | 0.859 | 0.755 | 0.598 | 0.447 |
|  | Hunan | 0.859 | 0.737 | 0.520 | 0.384 |
|  | Guangdong | 0.804 | 0.591 | 0.484 | 0.350 |
|  | Guangxi | 0.804 | 0.452 | 0.369 | 0.288 |
|  | Hainan | 0.804 | 0.523 | 0.296 | 0.227 |
|  | Chongqing | 0.859 | 0.537 | 0.333 | 0.274 |
|  | Sichuan | 0.859 | 0.718 | 0.337 | 0.237 |
|  | Guizhou | 0.804 | 0.633 | 0.519 | 0.392 |
|  | Yunnan | 0.804 | 0.508 | 0.420 | 0.290 |
|  | Tibet | 0.848 | 0.644 | 0.488 | 0.360 |
|  | Shaanxi | 0.892 | 0.780 | 0.600 | 0.453 |
|  | Gansu | 0.892 | 0.826 | 0.601 | 0.449 |
|  | Qinghai | 0.892 | 0.585 | 0.417 | 0.186 |
|  | Ningxia | 0.892 | 0.660 | 0.487 | 0.369 |
|  | Xinjiang | 0.892 | 0.816 | 0.625 | 0.395 |
| Emission factors of heat production (kg CO2/MJ) | China | 0.11 | 0.10 | 0.09 | 0.078 |

Table 11. Decline rate of heat intensity due to carbon capture technologies advancement (Li et al., 2024).

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
| Decline rate of heat intensity (%) | 0% | 24% | 37% | 55% |
| Heat intensity of CO2 capture (MJ/t CO2 captured) | 2998.1 | 2271.3 | 1892.7 | 1350.2 |

Table 12. Relative change in GHG emission factors of chemicals with increasing share of renewable energy in the energy mix, relative to 2019 (baseline = 100%) (Sun et al., 2025).

| Parameter | Share of renewable energy |  |  |  |
| --- | --- | --- | --- | --- |
|  | 28% | 44% | 52% | 66% |
| Relative GHG emission factors for chemicals (%) | 100.0% | 92.7% | 85.6% | 78.7% |

Reference

Abdelrahman, A.M., Kosar, S., Gulhan, H., Cicekalan, B., Ucas, G., Atli, E., Guven, H., Ozgun, H., Ozturk, I., Koyuncu, I., van Lier J.B., Volcke, E.I.P., Ersahin, M.E., 2023. Impact of primary treatment methods on sludge characteristics and digestibility, and wastewater treatment plant-wide economics. Water Res. 235, 119920.

Aboobakar, A., Cartmell, E., Stephenson, T., Jones, M., Vale, P., Dotro, G., 2013. Nitrous oxide emissions and dissolved oxygen profiling in a full-scale nitrifying activated sludge treatment plant. Water Res. 47 (2), 524-534.

Aboobakar, A., Jones, M., Vale, P., Cartmell, E., Dotro, G., 2014. Methane emissions from aerated zones in a full-scale nitrifying activated sludge treatment plant. Water, Air, Soil Pollut. 225, 1-10.

Akbulut, A., Arslan, O., Arat, H., Erbaş, O., 2021. Important aspects for the planning of biogas energy plants: Malatya case study. Case Stud. Therm. Eng. 26, 101076.

Angelidaki, I., Treu, L., Tsapekos, P., Luo, G., Campanaro, S., Wenzel, H., Kougias, P.G., 2018. Biogas upgrading and utilization: Current status and perspectives. Biotechnol. Adv. 36 (2), 452-466.

Baena-Moreno, F.M., Rodríguez-Galán, M., Vega, F., Vilches, L.F., Navarrete, B., 2019. Review: Recent advances in biogas purifying technologies. Int. J. Green Energy 16 (5), 401-412.

Balasundaram, G., Gahlot, P., Kumar Tyagi, V., Kannah, Y., Rajesh Banu, J., Kazmi, A.A., 2024. Mesophilic, thermophilic and thermal hydrolysis process coupled anaerobic digestion of sewage sludge: Biomethane potential, pathogen removal and energy feasibility. Sustain. Chem. Pharm. 37, 101397.

Bao, Z., Sun, S., Sun, D., 2015. Characteristics of direct CO2 emissions in four full-scale wastewater treatment plants. Desalin. Water Treat. 54 (4), 1070-1079.

Bao, Z., Sun, S., Sun, D., 2016. Assessment of greenhouse gas emission from A/O and SBR wastewater treatment plants in Beijing, China. Int. Biodeterior. Biodegrad. 108, 108-114.

Beiron, J., Normann, F., Johnsson, F., 2023. Carbon capture from combined heat and power plants – Impact on the supply and cost of electricity and district heating in cities. Int. J. Greenh. Gas Control 129, 103973.

Brotto, A.C., Kligerman, D.C., Andrade, S.A., Ribeiro, R.P., Oliveira, J.L., Chandran, K., de Mello, W.Z., 2015. Factors controlling nitrous oxide emissions from a full-scale activated sludge system in the tropics. Environ. Sci. Pollut. Res. 22, 11840-11849.

Canals, J., Cabrera-Codony, A., Carbó, O., Baldi, M., Gutiérrez, B., Ordoñez, A., Martín, M.J., Poch, M., Monclús, H., 2024. Nutrients removal by high-rate activated sludge and its effects on the mainstream wastewater treatment. Chem. Eng. J. 479, 147871.

Carrera, J., Carbó, O., Doñate, S., Suárez-Ojeda, M.E., Pérez, J., 2022. Increasing the energy production in an urban wastewater treatment plant using a high-rate activated sludge: Pilot plant demonstration and energy balance. J. Clean Prod. 354, 131734.

CCGGWG (China City Greenhouse Gas Working Group), 2023. China products carbon footprint factors database. http://lca.cityghg.com (accessed May 15, 2024) (In Chinese).

Chai, H., Yang, S., He, Q., Li, G., Hu, X., 2014. Comparison of greenhouse gas emissions from biological wastewater treatment processes for nitrogen removal. Water Wastewater Eng. 50 (07), 129-134 (In Chinese).

Chakraborty, T., Balusani, D., Smith, S., Santoro, D., Walton, J., Nakhla, G., Ray, M.B., 2020. Reusability of recovered iron coagulant from primary municipal sludge and its impact on chemically enhanced primary treatment. Sep. Purif. Technol. 231, 115894.

Checa-Fernández, A., Ruiz, L.M., Torre-Marín, J.M., Muñoz-Ubina, A., Pérez, J.I., Gómez, M.A., 2024. Direct application of chemically enhanced primary treatment in a municipal wastewater treatment plant: A case study. Chem. Eng. Res. Des. 204, 183-192.

Chen, Y., Chen, H., Li, J., Xiao, L., 2019. Rapid and efficient activated sludge treatment by electro-Fenton oxidation. Water Res. 152, 181-190.

Chen, G., Zhang, Y., Wang, X., Chen, F., Lin, L., Ruan, Q., Wang, Y., Wang, F., Cao, W., Chiang, P., 2020. Optimizing of operation strategies of the single-stage partial nitrification-anammox process. J. Clean Prod. 256, 120667.

Chew, C.M., Aroua, M.K., Hussain, M.A., Ismail, W.M.Z.W., 2016. Evaluation of ultrafiltration and conventional water treatment systems for sustainable development: An industrial scale case study. J. Clean Prod. 112, 3152-3163.

Christensen, T.H., Bisinella, V., 2021. Climate change impacts of introducing carbon capture and utilisation (CCU) in waste incineration. Waste Manage. 126, 754-770.

Christensson, M., Ekström, S., Chan, A., Le Vaillant, E., Lemaire, R., 2013. Experience from start-ups of the first ANITA Mox Plants. Water Sci. Technol. 67 (12), 2677-2684.

Chun, M., Kim, H., Bae, H., 2020. Biochemical methane potential of chemically enhanced primary treatment sludge for energy-independence of sewage treatment plants. J. Korean Soc. Water Environ. 36 (4), 322-331.

Dai, R., Liu, Y., Liu, X., Zhang, X., Zeng, C., Li, L., 2011. Investigation of a sewage-integrated technology combining an expanded granular sludge bed (EGSB) and an electrochemical reactor in a pilot-scale plant. J. Hazard. Mater. 192 (3), 1161-1170.

Delre, A., Mønster, J., Scheutz, C., 2017. Greenhouse gas emission quantification from wastewater treatment plants, using a tracer gas dispersion method. Sci. Total Environ. 605-606, 258-268.

DGTC (Danish Gas Technology Centre), 2020. Mapping the production chain for upgraded biogas. https://www.dgc.dk/media/kwzolimf/kortlagning_produktionskade.pdf (accessed April, 2024).

Dong, Q., Parker, W., Dagnew, M., 2016. Influence of SRT and HRT on bioprocess performance in anaerobic membrane bioreactors treating municipal wastewater. Water Environ. Res. 88 (2), 158-167.

Du, W., Lu, J., Hu, Y., Xiao, J., Yang, C., Wu, J., Huang, B., Cui, S., Wang, Y., Li, W., 2023. Spatiotemporal pattern of greenhouse gas emissions in China’s wastewater sector and pathways towards carbon neutrality. Nat. Water 1 (2), 166-175.

ERICAMR (Energy Research Institute of China Academy of Macroeconomic Research), 2023. China's Energy Transition Outlook. https://usercontent.one/wp/www.cet.energy/wp-content/uploads/2023/12/CET_CETO2023_Full-report-20231203.pdf (accessed October 10, 2023) (In Chinese).

Evans, P.J., Parameswaran, P., Lim, K., Bae, J., Shin, C., Ho, J., McCarty, P.L., 2019. A comparative pilot-scale evaluation of gas-sparged and granular activated carbon-fluidized anaerobic membrane bioreactors for domestic wastewater treatment. Bioresour. Technol. 288, 120949.

Fang, F., Li, H., Jiang, X., Deng, X., Yan, P., Guo, J., Chen, Y., Yang, J., 2020. Significant N2O emission from a high rate granular reactor for completely autotrophic nitrogen removal over nitrite. J. Environ. Manage. 266, 110586.

Foley, J., de Haas, D., Yuan, Z., Lant, P., 2010. Nitrous oxide generation in full-scale biological nutrient removal wastewater treatment plants. Water Res. 44 (3), 831-844.

GONDRC (General Office of the National Development and Reform Commission of China), 2015. Guidelines for greenhouse gas emission calculation and reporting for other industrial enterprises. https://www.ndrc.gov.cn/xxgk/zcfb/tz/201511/W020190905506438889540.pdf (accessed October 10, 2022) (In Chinese).

Gouveia, J., Plaza, F., Garralon, G., Fdz-Polanco, F., Peña, M., 2015. Long-term operation of a pilot scale anaerobic membrane bioreactor (AnMBR) for the treatment of municipal wastewater under psychrophilic conditions. Bioresour. Technol. 185, 225-233.

Guven, H., Ersahin, M.E., Dereli, R.K., Ozgun, H., Isik, I., Ozturk, I., 2019. Energy recovery potential of anaerobic digestion of excess sludge from high-rate activated sludge systems co-treating municipal wastewater and food waste. Energy 172, 1027-1036.

Hausherr, D., Niederdorfer, R., Bürgmann, H., Lehmann, M.F., Magyar, P., Mohn, J., Morgenroth, E., Joss, A., 2022. Successful year-round mainstream partial nitritation anammox: Assessment of effluent quality, performance and N2O emissions. Water Res. X 16, 100145.

He, C., Liu, X., Guan, T., Zhang, A., Liu, Y., 2023. Performance of a pilot-scale EGSB-Bardenpho process treating fresh leachate from municipal solid waste incineration plant. J. Environ. Chem. Eng. 11 (3), 110051.

Hu, W., Tian, J., Chen, L., 2019. Greenhouse gas emission by centralized wastewater treatment plants in Chinese industrial parks: Inventory and mitigation measures. J. Clean Prod. 225, 883-897.

Hu, Z., Liu, T., Wang, Z., Meng, J., Zheng, M., 2023. Toward energy neutrality: Novel wastewater treatment incorporating acidophilic ammonia oxidation. Environ. Sci. Technol. 57 (11), 4522-4532.

Huang, H., Peng, C., Li, K., Wang, Q., Ren, H., 2019. Optimized design of a novel filter brick in denitrification deep-bed filter. Bioprocess. Biosyst. Eng. 42, 1701-1710.

Hwang, K., Bang, C., Zoh, K., 2016. Characteristics of methane and nitrous oxide emissions from the wastewater treatment plant. Bioresour. Technol. 214, 881-884.

Ji, M., Wang, J., Khanal, S.K., Wang, S., Zhang, J., Liang, S., Xie, H., Wu, H., Hu, Z., 2023. Water-energy-greenhouse gas nexus of a novel high-rate activated sludge-two-stage vertical up-flow constructed wetland system for low-carbon wastewater treatment. Water Res. 229, 119491.

Jia, M., Zhang, Z., Zhang, L., Zhao, L., Lu, X., Li, L., Ruan, J., Wu, Y., He, Z., Liu, M., Jiang, L., Gao, Y., Wu, P., Zhu, S., Niu, M., Zheng, H., Cai, B., Tang, L., Shu, Y., Wang,J., 2024. Optimization of electricity generation and assessment of provincial grid emission factors from 2020 to 2060 in China. Appl. Energy 373, 123838.

Jin, L., Zhang, G., Tian, H., 2014. Current state of sewage treatment in China. Water Res. 66, 85-98.

Joss, A., Salzgeber, D., Eugster, J., König, R., Rottermann, K., Burger, S., Fabijan, P., Leumann, S., Mohn, J., Siegrist, H., 2009. Full-scale nitrogen removal from digester liquid with partial nitritation and anammox in one SBR. Environ. Sci. Technol. 43 (14), 5301-5306.

Justo, A.J., Junfeng, L., Lili, S., Haiman, W., Lorivi, M.R., Mohammed, M.O.A., Zhou, X., Feng, Y., 2016. Integrated expanded granular sludge bed and sequential batch reactor treating beet sugar industrial wastewater and recovering bioenergy. Environ. Sci. Pollut. Res. 23 (20), 21032-21040.

Kapoor, R., Ghosh, P., Kumar, M., Vijay, V.K., 2019. Evaluation of biogas upgrading technologies and future perspectives: A review. Environ. Sci. Pollut. Res. 26, 11631-11661.

Kong, Q., Zhang, J., Miao, M., Tian, L., Guo, N., Liang, S., 2013. Partial nitrification and nitrous oxide emission in an intermittently aerated sequencing batch biofilm reactor. Chem. Eng. J. 217, 435-441.

Kong, Z., Wu, J., Rong, C., Wang, T., Li, L., Luo, Z., Ji, J., Hanaoka, T., Sakemi, S., Ito, M., Kobayashi, S., Kobayashi, M., Qin, Y., Li, Y., 2021a. Sludge yield and degradation of suspended solids by a large pilot-scale anaerobic membrane bioreactor for the treatment of real municipal wastewater at 25°C. Sci. Total Environ. 759, 143526.

Kong, Z., Wu, J., Rong, C., Wang, T., Li, L., Luo, Z., Ji, J., Hanaoka, T., Sakemi, S., Ito, M., Kobayashi, S., Kobayashi, M., Qin, Y., Li, Y., 2021b. Large pilot-scale submerged anaerobic membrane bioreactor for the treatment of municipal wastewater and biogas production at 25°C. Bioresour. Technol. 319, 124123.

Lee, C.O., Howe, K.J., Thomson, B.M., 2012. Ozone and biofiltration as an alternative to reverse osmosis for removing PPCPs and micropollutants from treated wastewater. Water Res. 46 (4), 1005-1014.

Lee, Y., Han, G., 2015. Waste treatment with the pilot scale ATAD and EGSB pig slurry management system followed by sequencing batch treatment. Environ. Eng. Res. 20, 277-284.

Li, H., Peng, D., Liu, W., Wei, J., Wang, Z., Wang, B., 2016. N2O generation and emission from two biological nitrogen removal plants in China. Desalin. Water Treat. 57 (25), 11800-11806.

Li, H., Peng, D., Liu, W., Yao, Q., Zhuo, Y., 2017. Non-CO2 greenhouse gas release from different biological wastewater treatment processes. Environ. Sci. 38 (4), 1640-1646 (In Chinese).

Li, X., Jiang, L., Li, H., 2018. Application of ultrafiltration technology in water treatment. IOP Conf. Ser.: Earth Environ. Sci. 186 (3), 012009.

Li, Y., Wang, R., Zhao, Q., Zhou, Y., Fang, X., Xue, Z., 2024. CO2-enhanced oil recovery with CO2 utilization and storage: Progress and practical applications in China. Unconv. Resour. 4, 100096.

Liang, M., Lu, X., Liu, P., Wu, X., Zan, F., 2022. Tapping the energy potential from wastewater by integrating high-rate activated sludge process with anaerobic membrane bioreactor. J. Clean Prod. 333, 130071.

Liu, J., Kang, X., Luan, X., Gao, L., Tian, H., Liu, X., 2020.Performance and membrane fouling behaviors analysis with SVR-LibSVM model in a submerged anaerobic membrane bioreactor treating low-strength domestic sewage. Environ. Technol. Innov. 19, 100844.

Liu, Z., Gao, Z., Lu, X., 2021. Advanced treatment of pharmaceutical wastewater with a combined Fe-C micro-electrolysis/EGSB system assisted by microalgae. Sep. Sci. Technol. 56 (16), 2826-2837.

Liu, Z., Ding, H., Huang, X., 2024. Anaerobic membrane bioreactor treating municipal wastewater: Mainstream or sidestream? Environ. Sci. Technol. 58 (46), 20329-20332.

Lo, I., Lo, K., Mavinic, D.S., Shiskowski, D., Ramey, W., 2010. Contributions of biofilm and suspended sludge to nitrogen transformation and nitrous oxide emission in hybrid sequencing batch system. J. Environ. Sci. 22 (7), 953-960.

Lombardi, L., Francini, G., 2020. Techno-economic and environmental assessment of the main biogas upgrading technologies. Renew. Energy 156, 440-458.

Longo, S., d’Antoni, B.M., Bongards, M., Chaparro, A., Cronrath, A., Fatone, F., Lema, J.M., 2016. Mauricio-Iglesias, M., Soares, A., Hospido, A., Monitoring and diagnosis of energy consumption in wastewater treatment plants. A state of the art and proposals for improvement. Appl. Energy 179, 1251-1268.

Masuda, S., Sano, I., Hojo, T., Li, Y., Nishimura, O., 2018. The comparison of greenhouse gas emissions in sewage treatment plants with different treatment processes. Chemosphere 193, 581-590.

MEE (Ministry of Ecology and Environment of China), 2012. Technical specifications of expanded granular sludge bed (EGSB) reactor for wastewater treatment. China Environmental Science Press, Beijing. (In Chinese).

MEE (Ministry of Ecology and Environment of China), 2020 Baseline emission factors of regional power grids in china for the 2019 emission reduction programme. https://www.mee.gov.cn/ywgz/ydqhbh/wsqtkz/202012/W020201229610353340851.pdf (accessed July 10, 2023) (In Chinese).

MHURD (Ministry of Housing and Urban-Rural Development of China), 2019. Standard for building carbon emission calculation. Standards Press of China, Beijing (In Chinese).

NDRC (National Development and Reform Commission of China), 2019. CO₂ emissions accounting methodology and data verification checklist. https://www.ndrc.gov.cn/xxgk/zcfb/tz/201605/W020190905516997176703.pdf (accessed October 10, 2023) (In Chinese).

NEA (National Energy Administration of China), 2020. 2019 National monitoring and evaluation report on renewable electricity development. https://www.gov.cn/zhengce/zhengceku/2020-05/16/content_5512148.htm (accessed May 10, 2025) (In Chinese).

O'Shea, R., Lin, R., Wall, D.M., Browne, J.D., Murphy, J.D., 2020. Using biogas to reduce natural gas consumption and greenhouse gas emissions at a large distillery. Appl. Energy 279, 115812.

Peccia, J., Westerhoff, P., 2015. We should expect more out of our sewage sludge. Environ. Sci. Technol. 49 (14), 8271-8276.

Rahman, A., Meerburg, F.A., Ravadagundhi, S., Wett, B., Jimenez, J., Bott, C., Al-Omari, A., Riffat, R., Murthy, S., De Clippeleir, H., 2016. Bioflocculation management through high-rate contact-stabilization: A promising technology to recover organic carbon from low-strength wastewater. Water Res. 104, 485-496.

Rahman, A., De Clippeleir, H., Thomas, W., Jimenez, J.A., Wett, B., Al-Omari, A., Murthy, S., Riffat, R., Bott, C., 2019. A-stage and high-rate contact-stabilization performance comparison for carbon and nutrient redirection from high-strength municipal wastewater. Chem. Eng. J. 357, 737-749.

Ren, Y., Wang, J., Li, H., Zhang, J., Qi, P., Hu, Z., 2013. Nitrous oxide and methane emissions from different treatment processes in full-scale municipal wastewater treatment plants. Environ. Technol. 34 (21), 2917-2927.

Rodriguez-Caballero, A., Aymerich, I., Poch, M., Pijuan, M., 2014. Evaluation of process conditions triggering emissions of green-house gases from a biological wastewater treatment system. Sci. Total Environ. 493, 384-391.

Rong, C., Luo, Z., Wang, T., Guo, Y., Kong, Z., Wu, J., Ji, J., Qin, Y., Hanaoka, T., Sakemi, S., Ito, M., Kobayashi, S., Kobayashi, M., Qin, Y., Li, Y., 2021. Chemical oxygen demand and nitrogen transformation in a large pilot-scale plant with a combined submerged anaerobic membrane bioreactor and one-stage partial nitritation-anammox for treating mainstream wastewater at 25°C. Bioresour. Technol. 341, 125840.

SETC (State Economic and Trade Commission of China), 2002. Calculation method for energy consumption inpetrochemical engineering design. Standards Press of China, Beijing (In Chinese).

SGETRI (Sinopec Group Economic and Technical Research Institute), 2024. China Energy Outlook 2060 (2024 version). https://www.cpnn.com.cn/news/baogao2023/202401/W020240105322424121296.pdf (accessed October 15, 2024) (In Chinese).

Shao, Y., Liu, G., Wang, Y., Zhang, Y., Wang, H., Qi, L., Xu, X., Wang, J., He, Y., Li, Q., Xu, X., Wang, J., He, Y., Li, Q., Fan, H., Zhang, J., 2020. Sludge characteristics, system performance and microbial kinetics of ultra-short-SRT activated sludge processes. Environ. Int. 143, 105973.

Sheldon, M.S., Erdogan, I.G., 2016. Multi-stage EGSB/MBR treatment of soft drink industry wastewater. Chem. Eng. J. 2016. 285, 368-377.

Shewa, W.A., Dong, T., Mu, W., Murray, K., Dagnew, M., 2020. The impact of chemically enhanced primary treatment on the downstream liquid and solid train processes. Water Environ. Res. 92 (3), 359-368.

Song, J., Dong, X., Yang, W., Xing, J., Wang, X., 2023. Environmental co-benefits of energy recovery from wastewater of typical industrial sectors from life cycle perspective: Regional potentials in China. Energy Conv. Manag. 293, 117450.

Stachurski, A., Delgorge, F., Muszyński, A., 2024. Water reuse from wastewater: Experiences and challenges of implementing a full-scale water reuse system in a food industry plant. J. Water Process. Eng. 59, 104994.

Sun, H., Li, B., Wang, W., 2025. Research on low-carbon transformation pathways for China's chemical industry under ‘carbon dioxide emission peaking and carbon neutrality’ goals. Mod. Chem. Ind. 45 (07), 1-18 (In Chinese).

Varling, A.S., Christensen, T.H., Bisinella, V., 2023. Life cycle assessment of alternative biogas utilisations, including carbon capture and storage or utilisation. Waste Manage. 157, 168-179.

Verhuelsdonk, M., Glas, K., Parlar, H., 2021. Economic evaluation of the reuse of brewery wastewater. J. Environ. Manage. 281, 111804.

Wang, J., Zhang, J., Xie, H., Qi, P., Ren, Y., Hu, Z., 2011. Methane emissions from a full-scale A/A/O wastewater treatment plant. Bioresour. Technol. 102 (9), 5479-5485.

Wang, J., Li, L., Liu, Y., Li, W., 2021. A review of partial nitrification in biological nitrogen removal processes: From development to application. Biodegradation 32, 229-249.

Wang, R., Yan, H., Wu, D., Jiang, J., Dong, Y., 2024. High temperature heat pumps for industrial heating processes using water as refrigerant. Energy 313, 133847.

Wernet, G., Bauer, C., Steubing, B., Reinhard, J., Moreno-Ruiz, E., Weidema, B., 2016. The ecoinvent database version 3 (part I): Overview and methodology. Int. J. Life Cycle Assess. 21, 1218-1230.

Wimalaweera, I.P., Wei, Y., Zuo, F., Tang, Q., Ritigala, T., Wang, Y., Zhong, H., 2024. Weerasooriya, R., Jinadasa, S., Weragoda, S., Enhancing rubber industry wastewater treatment through an integrated AnMBR and A/O MBR system: Performance, membrane fouling analysis, and microbial community evolution. Membranes 14 (6), 130.

Wimalaweera, I., Zuo, F., Tang, Q., Sui, Q., Jinadasa, S., Weragoda, S., Ritigala, T., Weerasooriya, R., Wang, Y., Zhong, H., Makehelwala, M., Wei, Y., 2025. Synchronised removal of nitrogen and sulphate from rubber industrial wastewater by coupling of Sulfammox and sulphide-driven autotrophic denitrification in anaerobic membrane bioreactor. Bioresour. Technol. 416, 131785.

Wu, Z., Ji, S., Li, Y., Liu, J., 2023. A review of iron use and recycling in municipal wastewater treatment plants and a novel applicable integrated process. Bioresour. Technol. 379, 129037.

Xing, X., Wang, X., Zhao, L., Wang, W., Xing, D., Ren, N., Lee, D., Chen, C., 2024. Anaerobic treatment of fruit and vegetable wastewater using EGSB: From strategies for regulating over-acidification to microbial community. Renew. Energy 230, 120882.

Xu, F., Shao, J., Zhang, C., Guan, G., 2020. Engineering application of EGSB anaerobic reactor in dyeing wastewater treatment. Water Wastewater Eng. 56 (11), 65-68 (In Chinese).

Yan, X., Li, L., Liu, J., 2014. Characteristics of greenhouse gas emission in three full-scale wastewater treatment processes. J. Environ. Sci. 26 (2), 256-263.

Yang, L., Liu, W., Wang, H., Bian, X., Liu, F., 2020. Performance of antibiotic pharmaceutical wastewater treatment by expanded granular sludge reactor. Ind. Water Treat. 40 (05), 52-56 (In Chinese).

Yang, H., Guo, Y., Fang, N., Dong, B., 2023. Life cycle assessment of sludge anaerobic digestion combined with land application treatment route: Greenhouse gas emission and reduction potential. J. Environ. Chem. Eng. 11 (6), 111255.

Ye, M., Zhu, A., Liu, J., Li, Y., 2024. Iron recycle-driven organic capture and sidestream anaerobic membrane bioreactor for revolutionizing bioenergy generation in municipal wastewater treatment. Environ. Sci. Technol. 58 (21), 9350-9360.

Yoshida, H., Mønster, J., Scheutz, C., 2014. Plant-integrated measurement of greenhouse gas emissions from a municipal wastewater treatment plant. Water Res. 61, 108-118.

Yu, X., Zhang, Y., Deng, N., Chen, C., Ma, l., Dong, l., Zhang, Y., 2014. Experimental performance of high temperature heat pump with near-azeotropic refrigerant mixture. Energy Build. 78, 43-49.

Zhang, X., Liu, J., Zhang, Y., Ma, L., 2024. Deep denitrification of domestic sewage by sulfur-based mixotrophic denitrification filter. Teh. Vjesn. 31 (2), 412-419.