Pore-Doublet Model Code:

Define constants:

uw = 0.001  // dynamic viscosity

Lh0 = 0.0005092  // initial liquid layer thickness

h0 = -0.0005  // initial height

r = 0.0006  // radius

RHO = 1000  // fluid density

A = 0.62  // excitation amplitude

L = 0.04  // length

PI = 3.1415  // pi

fr = 5  // excitation frequency

OMEGA = 2.0 * PI * fr  // angular frequency

Define function `time_varying_pressure`:

Calculate current time t_sim = current time

Compute numerator and denominator:

numerator = 8.0 * uw * Lh0^2 * h0 * OMEGA

denominator = r^4

Compute terms in the formula:

term1 = (numerator * L / denominator)^2

term2 = (RHO * A * L)^2

A1 = sqrt(term1 + term2)

phi = atan((denominator * RHO * A) / numerator)

Output values of A1 and phi:

Print "A1: " and A1

Print "phi: " and phi

For each face f on boundary t:

Compute pressure:

pressure = A1 * sin(OMEGA * t_sim + phi) + 143

Output pressure on face f:

Print "Pressure at face f: " and pressure

Assign pressure value to the face:

Set pressure of face f to pressure

Image Recognition Code:

Set logging configuration:

Configure logging level as INFO

Set logging format to '%(asctime)s - %(levelname)s - %(message)s'

Obtain logger instance

Define function equation(x, y):

Compute term1 = 1.25 - 0.75 * cos(π / 10 * x)

Compute term2 = cos(arctan(-0.075 * π * sin(π / 10 * x)))

Compute term3 = 0.075 * π * sin(π / 10 * x)

Return y - (x + term1 / term2 + term1 * term3)

Define function solve_for_x(y_value):

Set initial guess x_initial_guess = 0

Use fsolve to solve the equation and return the first value of the solution

Define function find_shape_width(img_array, red_mask, blue_mask):

Compute color_mask = red_mask OR blue_mask

Obtain indices of non-zero columns as nonzero_cols

If there are non-zero columns:

Return index of the last non-zero column minus the index of the first

non-zero column

Else return None

Define function find_contact_point(img):

Get image width and height

Convert image to array img_array

Define red and blue masks:

Blue mask: blue channel > 200 and red channel < 100

Red mask: red channel > 200 and blue channel < 100

Call find_shape_width to calculate the shape width

If width is None:

Return None, None, None

Initialize contact_points list

Find the first left boundary

Iterate over each pixel and check for intersections of blue and red masks,

recording contact points

If contact points exist:

Find the contact point with the maximum X coordinate

Return contact point, shape width, left boundary

Else return None, None, None

Define function extract_number(filename):

Use regular expressions to extract numeric part from filename

If match is successful, return the number; else return infinity

Define function process_images_in_folder(folder_path):

Obtain all PNG files in the folder and sort them by numeric part

For each file:

Open the image

Log the filename being processed

Call find_contact_point to get contact point, shape width, and left boundary

If a contact point is found:

Calculate pixel-to-mm conversion factor

Calculate contact point value in mm

Call solve_for_x to compute X value

Output the result

Else log a warning message

Handle exceptions: if an error occurs, log the error

Close the image file

Main program:

Get the path of the current folder

Call process_images_in_folder to process all images in the folder