Calculation procedure supporting the manuscript

Spatio-temporal analysis for identifying hotspots and trends of nitrogen and potential energy valorisation from livestock manure in Italy

# Purpose of the document

This document outlines the calculation process employed to estimate livestock manure production, nitrogen (N) loads, methane (CH4) production, and electricity generation potential (EP) at a municipal level in Italy. This procedure was developed to ensure the transparency and reproducibility of the dataset uploaded to Zenodo. All calculations were performed using municipal livestock data and standardised coefficients derived from official Italian guidelines and literature sources.

# Input data

The main input data consists of livestock statistics at the municipality level, including the number of animals by species and category. The livestock categories considered in the calculations are cattle, buffalo, and pigs. The National Zootechnical Register statistics also provide disaggregation by age class, enabling animals to be categorised by production type, age group, and average live weight for each species. Tables 1 - 5 report the coefficients used to estimate manure production and N excretion for each livestock category

Table 1. Manure production and nitrogen content for livestock categories (meat production)

| Category | Weight (kg) | Slurry (m³ t⁻¹ live weight yr⁻¹) | Manure (m³ t⁻¹ live weight yr⁻¹) | N from slurry (kg t⁻¹ live weight yr⁻¹) | N from manure (kg t⁻¹ live weight yr⁻¹) |
| --- | --- | --- | --- | --- | --- |
| Calves (0 - 6 months) | 100 | 15.8 | 7.6 | 86.8 | 17.2 |
| Growing calves (6 months - 1 year) | 130 | 50.0 | 0.0 | 67.0 | 0.0 |
| Fattening buffalo | 400 | 20.4 | 2.7 | 69.1 | 5.9 |

Table 2. Manure production and nitrogen content for livestock categories (milk production)

| Category | Weight (kg) | Slurry (m³ t⁻¹ live weight yr⁻¹) | Manure (m³ t⁻¹ live weight yr⁻¹) | N from slurry (kg t⁻¹ live weight yr⁻¹) | N from manure (kg t⁻¹ live weight yr⁻¹) |
| --- | --- | --- | --- | --- | --- |
| Calves (0 - 6 months) | 100 | 15.8 | 7.6 | 86.8 | 17.2 |
| Growing stock (6 months - 1 year) | 300 | 19.9 | 3.79 | 91.7 | 11.3 |
| Dairy buffalo | 650 | 20.1 | 5.75 | 71.2 | 10.3 |

Table 3. Manure production and nitrogen content for cattle categories (meat production)

| Category | Weight (kg) | Slurry (m³ t⁻¹ live weight yr⁻¹) | Manure (m³ t⁻¹ live weight yr⁻¹) | N from slurry (kg t⁻¹ live weight yr⁻¹) | N from manure (kg t⁻¹ live weight yr⁻¹) |
| --- | --- | --- | --- | --- | --- |
| Calves (0 - 6 months) | 100 | 17.9 | 4.8 | 98.4 | 21.6 |
| Growing calves (6 months - 1 year) | 130 | 50.0 | 0.0 | 67.0 | 0.0 |
| Fattening cattle | 350 | 24.0 | 3.1 | 77.4 | 6.6 |

Table 4. Manure production and nitrogen content for cattle categories (milk production)

| Category | Weight (kg) | Slurry (m³ t⁻¹ live weight yr⁻¹) | Manure (m³ t⁻¹ live weight yr⁻¹) | N from slurry (kg t⁻¹ live weight yr⁻¹) | N from manure (kg t⁻¹ live weight yr⁻¹) |
| --- | --- | --- | --- | --- | --- |
| Calves (0 - 6 months) | 100 | 17.9 | 4.8 | 98.4 | 21.6 |
| Growing stock (6 months - 1 year) | 300 | 23.1 | 3.7 | 106.9 | 13.1 |
| Dairy cattle | 600 | 28.8 | 7.6 | 120.2 | 17.9 |

Table 5. Manure production and nitrogen content for pig categories

| Category | Weight (kg) | Slurry (m³ t⁻¹ live weight yr⁻¹) | Manure (m³ t⁻¹ live weight yr⁻¹) | N from slurry (kg t⁻¹ live weight yr⁻¹) | N from manure (kg t⁻¹ live weight yr⁻¹) |
| --- | --- | --- | --- | --- | --- |
| Boars | 250 | 29.7 | 6.2 | 88.4 | 21.6 |
| Piglets | 18 | 47.2 | 6.2 | 103.9 | 21.9 |
| Gilts | 107.5 | 42.4 | 5.6 | 90.9 | 19.1 |
| Growing pigs | 40 | 42.4 | 5.6 | 90.9 | 19.1 |
| Finishing pigs | 70 | 42.4 | 5.6 | 90.9 | 19.1 |
| Sows | 180 | 47.5 | 5.5 | 86.9 | 6.6 |
| Heavy pigs | 90 | 42.4 | 5.6 | 17.9 | 5.6 |

The parameters reported in Table X were used to estimate methane production and electricity generation potential from livestock manure and slurry. The calculations were based on volatile solids (VS), biochemical methane potential (BMP), methane yield coefficients for fresh manure/slurry, and conversion efficiencies reported in the literature. These parameters were applied according to effluent type to estimate the theoretical methane production and the associated electricity generation potential within the proposed framework.

Table 6. Parameters used for methane and electricity potential estimation from livestock manure and slurry

| Effluent | VS (g/kg) | BMP (mL CH4/g VS) | Yslurry/manure (Nm3 CH4/kg fresh manure) | (Gómez et al., 2010) | Reference literature |
| --- | --- | --- | --- | --- | --- |
| Pig manure | 359,00 | 128,00 | 0,0460 | 0,8 | (Garcia et al., 2019) |
| Pig slurry | 29,00 | 187,00 | 0,0054 | 0,8 | (Garcia et al., 2019) |
| Bovine manure | 215,33 | 153,00 | 0,0329 | 0,45 | (Kouas et al., 2017) |
| Bovine slurry | 62,77 | 200,46 | 0,0126 | 0,45 | (Allen et al., 2016) |
| Buffalo | 81,4 | 313,3 | 0,0255 | 0,45 | (Esposito et al., 2012) |

# Estimation of manure production

Manure production was estimated separately for slurry and solid manure. For each municipality and livestock category, manure production was calculated as:

Gi=ni×LWi×gi

where:

- Gi= manure or slurry production for livestock category i(m³ yr⁻¹);

- ni= number of animals in category i;

- LWi= average live weight of category i(t live weight);

- gi= manure or slurry production coefficient (m³ t⁻¹ live weight yr⁻¹)

Total manure production at the municipal level was obtained by summing slurry and solid manure production across all livestock categories.

# Nitrogen load estimation

For each livestock category, N production was calculated as:

Nt= a,ina,i×LWa,i×Na,i

where:

- Nᵢ = nitrogen load for livestock category i (kg N yr⁻¹)

- nᵢ = number of animals in livestock category i

- Wᵢ = average live weight of livestock category i (t live weight)

- NCᵢ = nitrogen coefficient (kg N t⁻¹ live weight yr⁻¹)

# Methane production and electricity generation potential

Methane production and electricity generation potential were estimated from the amount of manure produced and from literature-based conversion parameters. Methane yield was calculated from volatile solids and biochemical methane potential as:

Yslurry/manure=VS×BMP

where:

- Y = methane yield (Nm³ CH₄ kg⁻¹ fresh manure)

- VS = volatile solids content (g kg-1)

- BMP = biochemical methane potential (mL CH4 g-1 VS)

Methane production was then estimated as:

GMslurry/manure=Yslurry/manure × Gslurry/manure × η

where:

- GM = methane production (Nm³ CH₄ yr⁻¹)

- Y = methane yield (Nm³ CH₄ kg⁻¹ fresh manure)

- G = manure or slurry quantity produced annually (m³ yr⁻¹)

- η = availability factor, representing the fraction of manure that can be collected and treated

Electricity generation potential was calculated from methane production as:

EP=GMslurry/manure × LHV × α × D

where:

- EP = electricity generation potential (kWh yr⁻¹)

- GM = methane production (Nm³ CH₄ yr⁻¹)

- LHV = lower heating value of methane (13.69 kWh kg⁻¹)

- α = electricity conversion efficiency (0.28)

- D = methane density (0.657 kg m⁻³)

# Output variables

The final dataset includes the following calculated variables at the municipality-year level:

- manure production

- nitrogen load from livestock manure

- methane production

- electricity generation potential

# Bibliography

Allen, E., Wall, D.M., Herrmann, C., Murphy, J.D., 2016. A detailed assessment of resource of biomethane from first, second and third generation substrates. Renew. Energy 87, 656–665. https://doi.org/10.1016/j.renene.2015.10.060

Esposito, G., Frunzo, L., Panico, A., Pirozzi, F., 2012. Enhanced bio-methane production from co-digestion of different organic wastes. Environmental Technology (United Kingdom) 33, 2733–2740. https://doi.org/10.1080/09593330.2012.676077

Garcia, N.H., Mattioli, A., Gil, A., Frison, N., Battista, F., Bolzonella, D., 2019. Evaluation of the methane potential of different agricultural and food processing substrates for improved biogas production in rural areas. Renewable and Sustainable Energy Reviews 112, 1–10. https://doi.org/10.1016/j.rser.2019.05.040

Gómez, A., Zubizarreta, J., Rodrigues, M., Dopazo, C., Fueyo, N., 2010. Potential and cost of electricity generation from human and animal waste in Spain. Renew. Energy 35, 498–505. https://doi.org/10.1016/j.renene.2009.07.027

Kouas, M., Torrijos, M., Sousbie, P., Steyer, J.P., Sayadi, S., Harmand, J., 2017. Robust assessment of both biochemical methane potential and degradation kinetics of solid residues in successive batches. Waste Management 70, 59–70. https://doi.org/10.1016/j.wasman.2017.09.001